BMI088

6-axis Motion Tracking for

High-performance Applications

BMI088: Data Sheet

Document revision

1.8

Document release date

January 2022

Document number

BST-BMI088-DS000-18

Technical reference code

0 273 141 365

Notes

Data and descriptions in this document are subject to change without

notice. Product photos and pictures are for illustration purposes only

and may differ from the real product appearance

2 |

62

Bosch Sensortec | BMI088 Data sheet

Basic Description

BMI088 is an inertial measurement unit (IMU) for the detection of movements and rotations in 6 degrees

of freedom (6DoF). It combines the functionality of two inertial sensors in one device: an advanced

triaxial 16-bit gyroscope and a versatile, leading-edge triaxial 16-bit accelerometer.

BMI088  is  designed  to  meet  all  requirements  for  high  performance  consumer  applications  in  harsh

vibration  environments  such  as  those  encountered  in  drones  and  robotics  applications. The  IMU  is

designed to effectively suppress vibrations above a few hundred Hz that could occasionally occur due

to resonances on the pcb or the structure of the total system.

The sensor has an extended measurement range of up to ±24g to avoid signal clipping under strong

signal exposure.

An evaluation circuitry (ASIC) converts the output of the micro-electro-mechanical sensing structures

(MEMS), which are developed, produced and tested in BOSCH facilities. The corresponding chip-sets

are packed into one single LGA 3.0mm x 4.5mm x 0.95mm housing. For optimum system integration,

BMI088 is fitted with digital interfaces (SPI or I2C), offering a wide VDDIO voltage range from 1.2V to

3.6V.  To  provide  maximum  performance  and  reliability,  each  device  is  tested  and  is  ready-to-use

calibrated.

To increase flexibility, both gyroscope and accelerometer can not only be operated individually, but tied

together for data synchronization purposes. The on-chip features comprise FIFOs for acceleration and

gyroscope data and interrupt controllers.

The BMI088 has an excellent temperature behavior with an outstanding low temperature coefficient of

the offset (TCO) and temperature coefficient of the sensitivity (TCS).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

3 |

62

Bosch Sensortec | BMI088 Data sheet

Index of Contents

Basic Description .................................................................................................................................. 2

1.

Specification ..................................................................................................................................... 7

1.1

Electrical Specifications ............................................................................................................. 7

Electrical Specifications: Accelerometer/Gyroscope ................................................ 8

1.2

Accelerometer Specifications .................................................................................................... 9

1.3

Gyroscope Specifications ........................................................................................................ 10

1.4

Temperature Sensor Specifications ......................................................................................... 11

1.5

Absolute Maximum Ratings ..................................................................................................... 12

2.

Block Diagram ................................................................................................................................ 13

3.

Quick Start Guide – Device Initialization ..................................................................................... 13

4.

Functional Description .................................................................................................................. 14

4.1

Power Management and Power Modes .................................................................................. 14

4.1.1

Power Modes: Accelerometer ................................................................................. 14

4.1.2

Power Modes: Gyroscope ...................................................................................... 15

4.2

Sensor Data ............................................................................................................................. 15

4.3

Sensor Time............................................................................................................................. 16

4.4

Output Data Rate (ODR) and Low-pass Filter ......................................................................... 16

4.4.1

Accelerometer ......................................................................................................... 16

4.4.2

Gyroscope .............................................................................................................. 16

4.5

Range Settings ........................................................................................................................ 16

4.6

Self-test .................................................................................................................................... 16

4.6.1

Accelerometer ......................................................................................................... 17

4.6.2

Gyroscope .............................................................................................................. 17

4.7

New Data Interrupt .................................................................................................................. 18

4.7.1

Accelerometer ......................................................................................................... 18

4.7.2

Gyroscope .............................................................................................................. 18

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

4 |

62

Bosch Sensortec | BMI088 Data sheet

4.8

Soft-Reset ................................................................................................................................ 18

4.8.1

Soft-Reset Accelerometer ....................................................................................... 18

4.9

FIFO ......................................................................................................................................... 19

4.9.1

FIFO operating modes ............................................................................................ 19

4.9.2

FIFO interrupts ........................................................................................................ 19

4.9.3

Accelerometer sensor FIFO buffer ......................................................................... 19

4.9.4

Gyroscope sensor FIFO buffer ............................................................................... 22

5.

Register Maps ................................................................................................................................. 24

5.1

Communication with the sensor .............................................................................................. 24

5.2

Register Map: Accelerometer .................................................................................................. 25

5.3

Register Description: Accelerometer ....................................................................................... 26

5.3.1

Register 0x00: ACC_CHIP_ID ................................................................................ 26

5.3.2

Register 0x02: ACC_ERR_REG ............................................................................. 26

5.3.3

Register 0x03: ACC_STATUS ................................................................................ 26

5.3.4

Register 0x12 – 0x17: ACC data ............................................................................ 27

5.3.5

Register 0x18 – 0x1A: Sensortime data ................................................................. 27

5.3.6

Register 0x1D: ACC_INT_STAT_1 ......................................................................... 27

5.3.7

Register 0x22 – 0x23: Temperature sensor data.................................................... 28

5.3.8

Register 0x24-0x25: FIFO_LENGTH ...................................................................... 28

5.3.9

Register 0x26: FIFO_DATA .................................................................................... 28

5.3.10

Register 0x40: ACC_CONF .................................................................................... 29

5.3.11

Register 0x41: ACC_RANGE ................................................................................. 30

5.3.12

Register 0x45: FIFO_DOWNS ............................................................................... 30

5.3.13

Register 0x46 – 0x47: FIFO_WTM ......................................................................... 30

5.3.14

Register 0x48: FIFO_CONFIG_0 ........................................................................... 31

5.3.15

Register 0x49: FIFO_CONFIG_1 ........................................................................... 31

5.3.16

Register 0x53: INT1_IO_CONF .............................................................................. 32

5.3.17

Register 0x54: INT2_IO_CONF .............................................................................. 33

5.3.18

Register 0x58: INT1_INT2_MAP_DATA ................................................................. 33

5.3.19

Register 0x6D: ACC_SELF_TEST ......................................................................... 34

5.3.20

Register 0x7C: ACC_PWR_CONF ......................................................................... 34

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

5 |

62

Bosch Sensortec | BMI088 Data sheet

5.3.21

Register 0x7D: ACC_PWR_CTRL .......................................................................... 34

5.3.22

Register 0x7E: ACC_SOFTRESET ........................................................................ 35

5.4

Register Map: Gyroscope ........................................................................................................ 36

5.5

Register Description: Gyroscope ............................................................................................. 37

5.5.1

Register 0x00: GYRO_CHIP_ID ............................................................................. 37

5.5.2

Register 0x02 – 0x07: Rate data ............................................................................ 37

5.5.3

Register 0x0A: GYRO_INT_STAT_1 ...................................................................... 38

5.5.4

Register 0x0E: FIFO_STATUS ............................................................................... 38

5.5.5

Register 0x0F: GYRO_RANGE .............................................................................. 39

5.5.6

Register 0x10: GYRO_BANDWIDTH ..................................................................... 39

5.5.7

Register 0x11: GYRO_LPM1 .................................................................................. 40

5.5.8

Register 0x14: GYRO_SOFTRESET ..................................................................... 40

5.5.9

Register 0x15: GYRO_INT_CTRL .......................................................................... 40

5.5.10

Register 0x16: INT3_INT4_IO_CONF .................................................................... 41

5.5.11

Register 0x18: INT3_INT4_IO_MAP ...................................................................... 41

5.5.12

Register 0x1E: FIFO_WM_ENABLE ...................................................................... 42

5.5.13

Register 0x34: FIFO_EXT_INT_S .......................................................................... 42

5.5.14

Register 0x3C: GYRO_SELF_TEST ...................................................................... 42

5.5.15

Register 0x3D: FIFO_CONFIG_0 ........................................................................... 43

5.5.16

Register 0x3E: FIFO_CONFIG_1 ........................................................................... 43

5.5.17

Register 0x3F: FIFO_DATA .................................................................................... 43

6.

Digital Interface .............................................................................................................................. 44

6.1

Serial Peripheral Interface (SPI) .............................................................................................. 45

6.1.1

SPI interface of gyroscope part .............................................................................. 47

6.1.2

SPI interface of accelerometer part ........................................................................ 48

6.2

Inter-Integrated Circuit (I²C) ..................................................................................................... 48

7.

Pin-out and Connection Diagram ................................................................................................. 52

7.1

Pin-out ..................................................................................................................................... 52

7.2

Connection diagram SPI .......................................................................................................... 53

2

7.3

Connection diagram I

C .......................................................................................................... 53

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

6 |

62

Bosch Sensortec | BMI088 Data sheet

8.

Package ........................................................................................................................................... 54

8.1

Outline Dimensions ................................................................................................................. 54

8.2

Landing pattern ........................................................................................................................ 55

8.3

Sensing axes orientation ......................................................................................................... 56

8.4

Marking .................................................................................................................................... 57

8.4.1

Mass production samples ....................................................................................... 57

8.4.2

Engineering samples .............................................................................................. 57

8.5

PCB layout and soldering guidelines ....................................................................................... 57

8.6

Handling instructions ............................................................................................................... 58

8.7

Tape and Reel specification ..................................................................................................... 58

8.7.1

Orientation within the reel ....................................................................................... 59

8.8

Environmental safety ............................................................................................................... 59

8.8.1

Halogen content ...................................................................................................... 59

9.

Legal Disclaimer ............................................................................................................................. 60

9.1

Engineering samples ............................................................................................................... 60

9.2

Product use .............................................................................................................................. 60

9.3

Application examples and hints ............................................................................................... 60

10.

Document History and Modification ............................................................................................ 61

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

7 |

62

Bosch Sensortec | BMI088 Data sheet

1.  Specification

If not stated otherwise, the given values are over lifetime and full performance temperature and voltage

σ

ranges, minimum/maximum values are ±3

.

1.1

Electrical Specifications

Table 1: Electrical parameter specification

Parameter

Symbol

Condition

Min

Max

Unit

Supply Voltage

VDD

2.4

V

Internal Domains

3.6

Supply Voltage

VDDIO

1.2

V

I/O Domain

3.6

Voltage Input

V

SPI & I²C

-

IL,a

Low Level

0.3VDDIO

Voltage Input

V

SPI & I²C

-

IH,a

0.7VDDIO

High Level

Voltage Output

V

I

<= 2mA, SPI

-

OL,a

OL

0.23VDDIO

Low Level

Voltage Output

V

OH

High Level

I

<= 2mA, SPI

-

OH

0.8VDDIO

Operating

T

A

Temperature

-40

+85

°C

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

8 |

62

Bosch Sensortec | BMI088 Data sheet

Electrical Specifications: Accelerometer/Gyroscope

Table 2: Electrical parameter specification accelerometer

Parameter

Symbol

Condition

Min

Typ

Max

Units

Total Supply

=3.0V,

µA

DD

I

Current in

Normal mode

VDD = VDDIO

150

25°C, g

FS4g

Total Supply

VDD = VDDIO

=3.0V,

3

µA

I

DDsum

Current in

Suspend Mode

25°C

Power-up time

t

Time to first valid

ms

s_up

sample from suspend

mode

1

Table 3: Electrical parameter specification gyroscope

Parameter

Symbol

Condition

Min

Typ

Max

Unit

Supply Current in

= 3.0V,

5

mA

I

VDD = VDDIO

DD

Normal Mode

25°C, ODR =1kHz

Supply Current in

µA

I

DDsum

25

Suspend Mode

VDD = VDDIO

= 3.0V,

25°C

Supply Current in

I

VDD = VDDIO

= 3.0V,

DDdsum

Deep Suspend

Mode

25°C

<5

µA

Start-up time

t

to ±1º/s of final value;

30

ms

su

from power-off

Wake-up time

t

From suspend- and

30

ms

wusm

deep suspend-modes

Wake-up time

t

From fast power-up

10

ms

wufpm

mode

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

9 |

62

Bosch Sensortec | BMI088 Data sheet

1.2

Accelerometer Specifications

Table 4: Accelerometer specifications

Parameter

Symbol

Condition

Min

Typ

Max

Units

Acceleration Range

g

g

FS3g

±3

Selectable

g

g

FS6g

via serial digital interface

±6

g

FS12g

±12

g

g

±24

g

FS24g

Sensitivity

S

g

, T

=25°C

10920

LSB/g

3g

FS3g

A

S

g

, T

=25°C

5460

LSB/g

6g

FS6g

A

S

g

, T

=25°C

LSB/g

12g

FS12g

A

2730

S

, T

=25°C

1365

24g

g

FS24g

A

LSB/g

Sensitivity

TCS

%/K

Temperature Drift

0.002

Zero-g Offset

Off

Nominal VDD and VDDIO,

mg

20

25°C, g

FS6g

Zero-g Offset

mg/K

TCO

Temperature Drift

<0.2

Output Data Rate

ODR

12.5

1600

Hz

Bandwidth range

BW

3dB cut-off frequency of

5

Hz

the accelerometer

depends on ODR and

OSR

280

(245 for

Z axis)

Nonlinearity

NL

best fit straight line, g

%FS

FS3g

0.5

Output Noise

g

, T

√

=25°C

Hz

rms

n

Density

FS3g

A

Nominal VDD supplies

Normal mode

190

(Z-axis)

160

µg/

(X- & Y-

axis)

Cross Axis

S

relative contribution

0.5

%

Sensitivity

between any two of the

three axes

Alignment Error

E

relative to package outline

A

0.5

°

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

10 |

62

Bosch Sensortec | BMI088 Data sheet

1.3

Gyroscope Specifications

Table 5: Gyroscope specifications

Parameter

Symbol

Condition

Min

Typ

Max

Unit

Range

R

°/s

FS125

R

FS250

125

Selectable

°/s

250

via serial digital interface

R

°/s

FS500

500

R

°/s

FS1000

1000

R

°/s

FS2000

2000

LSB/°/s

Ta=25°C, R

FS125

262.144

Sensitivity

Ta=25°C, R

LSB/°/s

FS250

131.072

Ta=25°C, R

65.536

LSB/°/s

FS500

Ta=25°C, R

LSB/°/s

FS1000

32.768

Ta=25°C, R

16.384

FS2000

LSB/°/s

Sensitivity

Ta=25°C, R

±1

%

FS2000

tolerance

Sensitivity Change

Nominal VDD supplies

TCS

over Temperature

-40°C ≤ T

≤ +85°C

±0.03

%/K

A

R

FS2000

Sensitivity

T

=25°C,

S

<0.4

A

VDD

Supply Volt. Drift

VDD

≤ VDD ≤ VDD

min

max

%/V

best fit straight line

Nonlinearity

NL

±0.05

%FS

R

R

FS1000,

FS2000

Sensitivity to

g-Sensitivity

°/s/g

acceleration stimuli in

all three axis

(frequency <20kHz)

0.1

Ω

Ω

Off

Zero-rate Offset

±1

°/s

x

y

Nominal VDD supplies

Ω

=25°C, slow and fast

and

z

T

A

offset cancellation off

Zero-rate Offset

Nominal VDD supplies

TCO

°/s per K

Change over

±0.015

-40°C ≤ T

≤ +85°C

A

Temperature

R

FS2000

T

=25°C,

Zero-rate Offset

<0.1

A

Ω

Off

VDD

Supply Volt. Drift

VDD

≤ VDD ≤ VDD

min

max

°/s /V

rms, BW=47Hz

Output Noise

n

0.1

°/s

rms

(@ 0.014°/s/√Hz)

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

11 |

62

Bosch Sensortec | BMI088 Data sheet

523

Bandwidth BW

f

Hz

-3dB

230

116

64

47

32

23

12

Data rate

2000

Hz

(set of x,y,z rate)

1000

400

200

100

Data rate tolerance

%

(set of x,y,z rate)

±0.3

Cross Axis

Sensitivity to stimuli in

±1

%

Sensitivity

non-sense-direction

1.4

Temperature Sensor Specifications

Table 6: Temperature sensor specifications

Parameter

Symbol

Condition

Min

Typ

Max

Units

Temperature Sensor

150

°C

T

S

Measurement Range

-104

Temperature Sensor

dT

K/LSB

S

Slope

0.125

Temperature Sensor

at 25°C

K

S

OT

Offset error

±1

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

12 |

62

Bosch Sensortec | BMI088 Data sheet

1.5

Absolute Maximum Ratings

Table 7: Absolute maximum ratings

Parameter

Condition

Min

Max

Units

Voltage at Supply Pin

VDD Pin

V

-0.3

4

VDDIO

Pin

-0.3

4

V

Voltage at any Logic Pin

Non-Supply Pin

-0.3

VDDIO+0.3

V

Passive Storage Temp. Range

≤ 65% rel. H.

-50

+150

°C

Duration ≤ 200µs

10,000

g

Mechanical Shock

Duration ≤ 1.0ms

2,000

g

Free fall

1.8

m

onto hard surfaces

ESD

HBM, at any Pin

2

kV

CDM

500

V

MM

200

V

Note: Stress above these limits may cause damage to the device. Exceeding the specified electrical

limits may affect the device reliability or cause malfunction.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

13 |

62

Bosch Sensortec | BMI088 Data sheet

2.

Block Diagram

Figure 1 shows the basic building blocks of the BMI088:

Figure 1: Block diagram of BMI088

3.

Quick Start Guide – Device Initialization

For a proper device initialization, the following steps should be considered:

•

The user must decide on the interface (I2C or SPI) already during HW design: with the PS pin

the user determines which interface the sensor should listen to (see chapter 6).

•

The gyroscope part of the BMI088 initializes its I/O pins according to the selection given by the

PS pin.

•

The accelerometer part starts in I2C mode. It will stay in I2C mode until it detects a rising edge

on the CSB1 pin (chip select of the accelerometer), on which the accelerometer part switches

to SPI mode and stays in this mode until the next power-up-reset.

•

To change the accelerometer to SPI mode in the initialization phase, the user could perform a

dummy SPI read operation, e.g. of register ACC_CHIP_ID (the obtained value will be invalid).

After the POR the gyroscope is in normal mode, while the accelerometer is in suspend mode. To switch

the accelerometer into normal mode, the user must perform the following steps:

a.

Power up the sensor

b.

Wait 1 ms

c.

Enter normal mode by writing ‘4’ to ACC_PWR_CTRL

d.

Wait for 450 microseconds

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

14 |

62

Bosch Sensortec | BMI088 Data sheet

4.

Functional Description

4.1

Power Management and Power Modes

The BMI088 has two distinct power supply pins:

•

VDD is the main power supply for the internal blocks

•

VDDIO is a separate power supply pin mainly used for the supply of the interface

There are no limitations on the voltage levels of both pins relative to each other, as long as each of them

lies within its operating range.  Furthermore, the device can be completely switched off (VDD = 0V) while

keeping the VDDIO supply on (VDDIO > 0V) or vice versa.

When the VDDIO supply is switched off, all interface pins (CSB, SDI, SCK, PS) must be kept close to

GND

potential.

IO

The device contains a power-on reset (POR) generator. It resets the logic part and the register values

after powering-on VDD and VDDIO. This means that all application specific settings which are not equal

to the default settings (refer to 6.2 register map accelerometer and to 8.2 register map gyroscope), must

be changed back to their designated values after POR.

Please note: the POR resets also the interface. For the gyroscope part, the interface is defined by the

voltage level on the PS pin. The interface of the accelerometer part is defined by the voltage level of the

CSB1 pin at the moment when the POR is initiated (see chapter 3).

Power Modes: Accelerometer

4.1.1

The power state of the BMI088 accelerometer is controlled through the register ACC_PWR_CTRL. The

register ACC_PWR_CTRL enables and disables the accelerometer and the temperature sensor.

To enter normal mode, the value 0x04 must be written to ACC_PWR_CTRL.

To enter suspend mode, register ACC_PWR_CTRL must be cleared.

Note: The sensor is in suspend mode after reset (POR or soft-reset), thus the user actively needs to

enter normal mode in order to obtain acceleration values.

Note: After POR or soft-reset, the acceleration sensor needs up to 1ms boot time (t

)

s_up

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

15 |

62

Bosch Sensortec | BMI088 Data sheet

Power Modes: Gyroscope

4.1.2

The  gyroscope  has  3  different  power  modes.  Besides  normal  mode,  which  represents  the  fully

operational state of the device, there are 2 energy saving modes: suspend mode and deep-suspend

mode.

After power-up gyro is in normal mode so that all parts of the device are held powered-up and data

acquisition is performed continuously.

In suspend mode the whole analog part is powered down. No data acquisition is performed. While in

suspend mode the latest rate data and the content of all configuration registers are kept. The registers

can still be read (though they are not updated).

Suspend mode is entered by writing 0x80 to the register GYRO_LPM1. It can be left by writing 0x00 to

GYRO_LPM1 or by a soft reset (see 4.8).

Although write access to registers is supported at the full interface clock speed (SCL or SCK), a waiting

period must be inserted between two consecutive write cycles (please refer also to section 9.2.1).

In deep suspend mode the device reaches the lowest possible power consumption. Only the interface

section is kept alive. No data acquisition is performed and the content of the configuration registers is

lost.

Deep suspend mode is entered by writing 0x20 to the register GYRO_LPM1. It can be left by writing

0x00 to GYRO_LPM1 or by a soft reset (see 4.8).

Please note, that all application specific settings, which are not equal to the default settings, must be re-

set to its designated values after leaving deep-suspend mode.

Note: after POR or soft-reset, or when switching between the different power modes, the gyroscope

sensor needs up to 30ms time to reach the new state. Any communication with the sensor during this

time should be avoided.

4.2

Sensor Data

The width of the gyroscope and accelerometer sensor data is 16 bits (11 bits for the temperature sensor)

given in two’s complement representation.

The bits for each axis are split into an MSB upper part and an LSB lower part. Reading the sensor data

registers shall always start with the LSB part. In order to ensure the integrity of the sensor data, the

content of an MSB register is locked by reading the corresponding LSB register (shadowing procedure).

For details regarding the registers and the interpretation of the data found in these registers see:

•

chapter 5.5.2 for the gyroscope part

•

chapter 5.3.4 or the accelerometer part

•

chapter 5.3.7 for the temperature sensor

2

The burst-access mechanism provides an efficient way to read out the angular rate data in I

C or SPI

mode. During a burst-access, the sensor automatically increments the starting read address after each

2

C bus with an up to 50% reduced data

byte. The burst-access allows data to be transferred over the I

density. The sensor data (angular rate or acceleration data) in all read-out registers is locked as long as

the burst read access is active. Reading the sensor data registers of each gyroscope and accelerometer

part in burst read access mode ensures that the sensor values in all readout registers belong to the

same sample.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

16 |

62

Bosch Sensortec | BMI088 Data sheet

4.3

Sensor Time

The accelerometer part of BMI088 has a built-in counter with a width of 24 bits. It increments periodically

with a resolution of 39.0625µs. Details can be found in chapter 5.3.5.

4.4

Output Data Rate (ODR) and Low-pass Filter

The  sensor  signals  from  the  acceleration  sensor  and  gyroscope  analog  front-end  are  each  routed

through a low-pass filter.

Accelerometer

4.4.1

The 3db cut-off frequency of the digital low-pass filter depends on the chosen ODR as well as on the

over-sampling-ratio (OSR). Both can be configured in register ACC_CONF. The following table lists the

possible options:

Table 8: 3dB cutoff frequency of the accelerometer according to ODR and OSR settings in

ACC_CONF register

Accelerometer

Normal

OSR2

OSR4

ODR [Hz]

(acc_bwp = 0xA)

(acc_bwp = 0x9)

(acc_bwp = 0x8)

12.5

5 Hz

2 Hz

1 Hz

25

10 Hz

5 Hz

3 Hz

50

20 Hz

9 Hz

5 Hz

100

40 Hz

19 Hz

10 Hz

200

80 Hz

38 Hz

20 Hz

400

145 Hz

75 Hz

40 Hz

800

230 Hz

140 Hz

80 Hz

(200 Hz for z channel)

1600

280 Hz

234 Hz

145 Hz

(245 Hz for z channel)

(215 Hz for z channel)

Gyroscope

4.4.2

The user can choose between 8 different ODR and low pass filter bandwidth settings (see section 5.5.6).

4.5

Range Settings

The  measurement  range  can  be  set  through  the  registers  described  in  section  5.3.11  for  the

accelerometer and in section 5.5.5 for the gyroscope.

4.6

Self-test

The BMI088 incorporates a self-test feature for both the accelerometer and the gyroscope, indicating

whether the sensor is still ok.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

17 |

62

Bosch Sensortec | BMI088 Data sheet

Accelerometer

4.6.1

The self-test feature allows for checking the sensor functionality by applying electrostatic forces to the

sensor core instead of external accelerations. By physically deflecting the seismic mass, the entire signal

path of the sensor is tested. Activation of the self-test results in a static offset in the acceleration data.

Any external acceleration or gravitational force, which is applied to the sensor during a self-test, will be

observed in the sensor output as a superposition of the acceleration and the self-test signal. This means

that the self-test signal depends on the orientation of the sensor. To overcome this, the full self-test

procedure should be performed under static circumstances, e.g. when the part is not excited to any

acceleration except gravity.

The recommended self-test procedure is as follows:

1)

Set ±24g range by writing 0x03 to register ACC_RANGE (0x41)

2)

Set  ODR=1.6kHz,  continuous  sampling  mode,  “normal  mode”  (norm_avg4)  by  writing  0xA7  to

register ACC_CONF (0x40)

•

Continuous filter function: set bit7 in ACC_CONF

•

“normal avg4 mode”: ACC_CONF |= 0x02<<4

•

ODR=1.6kHz: ACC_CONF |= 0x0C

3)

Wait for > 2 ms

4)

Enable the positive self-test polarity (i.e. write 0x0D to register ACC_SELF_TEST (0x6D))

5)

Wait for > 50ms

6)

Read the accelerometer offset values for each axis (positive self-test response)

7)

Enable the negative self-test polarity (i.e. write 0x09 to register ACC_SELF_TEST (0x6D))

8)

Wait for > 50ms

9)

Read the accelerometer offset values for each axis (negative self-test response)

10) Disable the self-test (i.e. write 0x00 to register ACC_SELF_TEST (0x6D))

11) Calculate difference of positive and negative self-test response and compare with the expected

values (see table below)

12) Wait for > 50ms to let the sensor settle to normal mode steady state operation

Table 9: Accelerometer self-test: resulting minimum difference signal between positive and negative

self-test signal

x-axis signal

y-axis signal

z-axis signal

≥1000 mg

≥1000 mg

≥500 mg

It is recommended to perform a reset of the device after a self-test has been performed, since the self-

test  response  also  affects  the  interrupt  generation.  If  the  reset  cannot  be  performed,  the  following

sequence must be kept to prevent unwanted interrupt generation: disable interrupts, change parameters

of interrupts, wait for at least 50ms, and enable desired interrupts.

Gyroscope

4.6.2

A  built-in  self-test  facility  of  the  gyro  does  not  deflect  the  mechanical  MEMS  structure  (as  the

accelerometer self-test does), but this test also provides a quick way to determine if the gyroscope is

operational within the specified conditions.

To trigger the self-test, bit #0 (‘bite_trig’) in address GYRO_SELF_TEST must be set. When the test is

finished, bit #1 (‘bist_rdy’) will be set by the gyro and the test result can then be found in bit #2 (‘bist_fail’).

A ‘0’ indicates that the test was passed without issues. If a failure occurred, the bit ‘bist_fail’ will be set

to ‘1’.

A further test which is running continuously in the background can be checked by reading bit #4 in

address GYRO_SELF_TEST. Proper sensor function is indicated if the bit is set to ‘1’.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

18 |

62

Bosch Sensortec | BMI088 Data sheet

4.7

New Data Interrupt

Both accelerometer and gyroscope part offer a new data ready interrupt, which fires whenever a new

data sample set is complete and made available in the corresponding sensor data registers. This allows

a low latency data readout.

Accelerometer

4.7.1

The new data interrupt flag can be found in the register ACC_INT_STAT_1 (bit #7). It is set whenever

new data is available in the data registers and cleared automatically.

The interrupt can be mapped to the interrupt pins INT1 and/or INT2 in register INT1_INT2_MAP_DATA.

Both

interrupt

pins

INT1

and

INT2

can

be

configured

regarding

their

electrical

behavior

(see

INT1_IO_CONF and INT2_IO_CONF).

Gyroscope

4.7.2

The gyroscope provides a new data interrupt, which will generate an interrupt every time after storing a

new value of z-axis angular rate data in the data register. The interrupt is cleared automatically after

280-400 µs.

In contrast to the accelerometer part, for the gyro the new data interrupt must be explicitly enabled by

writing 0x80 to the register GYRO_INT_CTRL.

The interrupt can be mapped to the interrupt pins INT3 and/or INT4 in register INT3_INT4_IO_MAP.

Both

interrupt

pins

INT3

and

INT4

can

be

configured

regarding

their

electrical

behavior

(see

INT3_INT4_IO_CONF).

4.8

Soft-Reset

A soft-reset can be initiated at any time

•

for

the

accelerometer

part

by

writing

the

command

soft-reset

(0xB6)

to

register

ACC_SOFTRESET (see 5.3.22)

•

for

the

gyroscope

part

by

writing

the

command

soft-reset

(0xB6)

to

register

GYRO_SOFTRESET (see 5.5.8)

The soft-reset performs a fundamental reset to the device, which is largely equivalent to a power cycle.

Following a delay, all user configuration settings are overwritten with their default state (setting stored

in the NVM) wherever applicable. This command is functional in all operation modes but must not be

performed while NVM writing operation is in progress.

Soft-Reset Accelerometer

4.8.1

Soft reset timing:

In I2C mode, the rising edge of the SCL line for the ACK bit of the soft reset transfer determines the

reception of soft reset. The execution of the soft rest is delayed by 0.5 – 1us if the sensor is not in

advanced power save mode i.e. PWR_CONF.adv_power_save=0b0.

The execution of the soft reset causes that the SDA line to be released immediately by the device,

independent of the state of the SCL line.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

19 |

62

Bosch Sensortec | BMI088 Data sheet

If the host requires an explicit I2C ACK for the soft reset (i.e. SDA line remains low while the SCL line

is

high)

the

host

must

put

the

sensor

in

advance

power

save

mode

(PWR_CONF.adv_power_save=0b1), disable the sensor (PWR_CTRL.acc_en=0b0) and the auxillary

interface (PWR_CTRL.aux_en=0b0) before issuing a soft reset.

4.9

FIFO

BMI088 offers two integrated FIFO (First In, First Out) buffers for accelerometer and gyroscope sensor

signals, helping the user to reduce or even omit time critical read access to the sensor in order to obtain

data with a high timing precision.

FIFO operating modes

4.9.1

The FIFO can be operated in different modes: FIFO (or stop-at-full) mode and STREAM mode.

•

FIFO or stop-at-full mode: In FIFO or stop-at-full mode, the sensor values are stored in the

FIFO buffer subsequently until it is full.

•

STREAM mode: The STREAM mode works like the FIFO mode with the difference  that once

the buffer is full, the oldest data in the FIFO will be overwritten with the newest data from the

sensor.

FIFO interrupts

4.9.2

The FIFO buffers support two different types of interrupts:

•

Watermark interrupt: Triggered, when the fill level of the FIFO buffer reaches a user-defined

level.

•

FIFO-full interrupt: Triggered, when the FIFO is full.

Accelerometer sensor FIFO buffer

4.9.3

The accelerometer part of BMI088 has an integrated 1024 byte data FIFO. The FIFO captures data

from the data registers in frames, and each frame contains only one sample of a sensor.

4.9.3.1  Enabling FIFO and selecting the mode

The FIFO for accelerometer sensor data is enabled by setting bit #6 in register 0x49 (see 5.3.15).

4.9.3.1.1

Mode selection

When STREAM mode is desired, then the bit #0 in register 0x48 has to be cleared (set to ‘0’) (default

value on power up reset, see 5.3.14).

For FIFO or stop-at-full mode, bit #0 has to be set to ‘1’ in register 0x48.

4.9.3.1.2

FIFO data sampling rate

The input data rate to the FIFO is the same as the configured ODR of the sensor. However, it can be

k

reduced selecting a down-sampling factor of 2

with k=[0, 1, … 7]. The factor k must be written to bits

#4-6 of register 0x45 (see 5.3.12).

4.9.3.1.3

FIFO synchronization with external interrupts (tag application) for the accel

If the INT1 and/or INT2 pin is configured as input pin (by setting int2_io in register INT2_IO_CTRL

and/or setting int1_io in register INT1_IO_CTRL), signals on these pins can also be recorded in the

FIFO, and the frames are “tagged” accordingly. Therefore the pins need to be activated for FIFO

recording in register FIFO_CONFIG_1 (see 5.3.15).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

20 |

62

Bosch Sensortec | BMI088 Data sheet

4.9.3.2  Data format in FIFO

The FIFO captures data in frames. The first byte is a header byte, defining the type of frame. From

this, the number of consecutive bytes and their content can be derived.

The header byte consists of the header signature (first 6 bits) and two bits indicating the status of the

interrupt pins INT1 and INT2 if configured accordingly (see 4.9.3.1.3).

4.9.3.2.1

Acceleration sensor data frame

•

Frame length: 7 bytes (1 byte header + 6 bytes payload)

•

Header:

Bit

7

6

5

4

3

2

1

0

1

0

0

0

0

1

[INT2 tag]

[INT1 tag]

•

Payload: the next bytes contain the sensor data in the same order as defined in the register

map (addresses 0x12 – 0x17).

4.9.3.2.2

Skip Frame

In the case of FIFO overflows, in both FIFO and STREAM mode, a skip_frame is prepended to the

FIFO content, when read out next time. A skip frame does not consume memory in the FIFO.

•

Frame length: 2 bytes (1 byte header + 1 byte payload)

•

Header:

Bit

7

6

5

4

3

2

1

0

0

1

0

0

0

0

reserved

reserved

•

Payload: one byte containing the number of skipped frames. When more than 0xFF frames

have been skipped, 0xFF is returned.

4.9.3.2.3

Sensortime Frame

A sensortime frame is only sent if the FIFO becomes empty during the burst read. A sensortime frame

does not consume memory in the FIFO.

•

Frame length: 4 bytes (1 byte header + 3 bytes payload)

•

Header:

Bit

7

6

5

4

3

2

1

0

0

1

0

0

0

1

reserved

reserved

•

Payload: Sensortime (content of registers 0x18 – 0x1A), taken when the last byte of the last

frame is read.

4.9.3.2.4

FIFO input config Frame

Whenever the filter configuration or the range of the accelerometer sensor is changed, a FIFO input

config frame is inserted into the FIFO, before the configuration change becomes active. E.g. when the

bandwidth for the accelerometer filter is changed in Register ACC_CONF, a FIFO input config frame is

inserted before the first frame with accelerometer data with the new bandwidth configuration.

•

Frame length: 2 bytes (1 byte header + 1 byte payload)

•

Header:

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

21 |

62

Bosch Sensortec | BMI088 Data sheet

Bit

7

6

5

4

3

2

1

0

0

1

0

0

1

0

reserved

reserved

•

Payload: The FIFO input config frame contains one byte of data, of which the following bits have

a meaning (the content of the other bits can safely be ignored):

Bit #1: indicates that a configuration change through register ACC_RANGE becomes

o

active (means for example that the range of the accelerometer was changed).

Bit #0: indicates that a configuration change through the registers ACC_CONF or

o

FIFO_DOWNS becomes active (means of example that the filter settings where

changed or the FIFO sampling rate was modified).

4.9.3.2.5

Sample drop Frame

After a reconfiguration, indicated by the fifo_Input_Config frame, the next sample may be dropped,

until the sensor delivers valid data again. Instead, a drop frame is inserted at the ODR tick at which a

sample was to be expected without reconfiguration.

•

Frame length: 2 bytes (1 byte header + 1 byte payload)

•

Header:

Bit

7

6

5

4

3

2

1

0

0

1

0

1

0

0

reserved

reserved

•

Payload: The sample drop frame contains one byte of data, whose content can be ignored.

4.9.3.2.6

FIFO partial frame reads and overreads

When a frame is only partially (uncompletely) read through the register FIFO_DATA it will be repeated

completely with the next access. In the case of a FIFO overflow between the first partial read and the

second read attempt, the frame may be deleted.

When more data is read from the FIFO than it contains valid data, 0x8000 is returned.

4.9.3.3   FIFO Interrupts

The FIFO supports two interrupts, a FIFO full interrupt and a watermark interrupt:

•

The FIFO full interrupt is issued when the FIFO fill level is above the full threshold. The full

threshold is reached just before the last two frames are stored in the FIFO.

•

The FIFO watermark is issued when the FIFO fill level is superior or equal to the watermark

level defined in register FIFO_WTM (see 5.3.13).

In order to enable/use the FIFO full or watermark interrupts they need to be mapped on the desired

interrupt pin via INT1_INT2_MAP_DATA (see 5.3.18).

Both interrupts are suppressed when a read operation on the register FIFO_DATA is ongoing.

Latched FIFO interrupts will only get cleared, if the status register gets read and the fill level is below

the corresponding FIFO interrupt (full or watermark).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

22 |

62

Bosch Sensortec | BMI088 Data sheet

4.9.3.4  FIFO Reset

The user can trigger a FIFO reset by writing 0xB0 to ACC_SOFTRESET (register 0x7E).

Gyroscope sensor FIFO buffer

4.9.4

The gyroscope part of BMI088 features an integrated FIFO memory capable of storing up to 100

frames of data in FIFO mode. Each frame consists of three 16-bit rate_x,y,z data words, and 16 bits of

interrupt data sampled at the same point in time.

4.9.4.1

Enabling FIFO and selecting the mode

The FIFO for gyroscope sensor data is enabled by setting the appropriate FIFO mode in Register 0x3E:

FIFO_CONFIG_1.

4.9.4.1.1

FIFO data sampling rate

The input data rate to the FIFO is the same as the configured ODR of the sensor.

4.9.4.1.2

FIFO sync with external interrupts (tag application) for the gyroscope

The FIFO of the gyroscope features a mode that allows the precise synchronization of external events

with the gyroscope angular rate saved in the FIFO. This synchronization can be used for example for

image and video stabilization applications.

Any of the gyroscope interrupt pins (INT3 or INT4) can be reconfigured to act as input pin, but not

both. In addition, the tag mode has to be enabled. The so configured interrupt pin will then behave as

an input pin and not as an interrupt pin. The working principle is shown in below figure:

Timing diagram for external FIFO synchronization. EFS-pin is the Interrupt pin configured to capture

external events. FIFO z(0) is the least significant bit of the z-axis gyro data stored in the FIFO.

In order to enable the tag mode, bit 5 must be set in register 0x34 (see 5.5.13). The pin can be chosen

in the same register, bit 4.

In this mode, the least significant bit of the z-axis is used as tag-bit, therefore losing its meaning as

gyroscope data bit. The remaining 15 bits of the z-axis gyroscope data keep the same meaning as in

standard mode.

Once the pin, which is configured for the tag mode, is set to high level, the next FIFO word will be

marked with a tag (z-axis LSB = 1). While pin is kept at a high level, the corresponding FIFO words will

continuously be tagged. After the pin is reset to low level, the immediate next FIFO word could still be

tagged, and only after this word, the next tag will be reset (z-axis LSB=0). This is shown in the above

diagram.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

23 |

62

Bosch Sensortec | BMI088 Data sheet

The tag synchronizes external events with the same time precision as the FIFO update rate. Therefore

update rate of the tag is determined by the output data rate.

4.9.4.2

FIFO Data Readout

The FIFO stores the data that are also available at the read-out registers 0x02-0x07. Thus, all

configuration settings apply to the FIFO data as well as the data readout registers. The FIFO read out

is possible through register 0x3F (FIFO_DATA). The readout can be performed using burst mode. A

single burst can read out one or more frames at a time. If a frame is not read completely due to an

incomplete read operation, the remaining part of the frame is lost. In this case the FIFO aligns to the

next frame during the next read operation.

The data format is described in 5.5.17.

4.9.4.2.1

Interface speed requirements for Gyroscope FIFO use

In order to use the FIFO effectively, larger blocks of data need to be read out quickly. Depending on

the output data rate of the sensor, this can impose requirements on the interface.

The output data rate of the gyroscope is determined by the filter configuration (see the data sheet of

the sensor). What interface speed is required depends on the selected rate.

•

2

For an I

C speed of 400 kHz, every filter mode can be used.

•

2

C speed of 200 kHz, only modes with an output data rate of 1 KHz and below are

For an I

recommended.

•

2

C speed of 100 kHz, only modes with an output data rate of 400 Hz and below are

For an I

recommended.

4.9.4.3

FIFO Frame Counter and Overrun Flag

The frame counter (address 0x0E bits<6:0>, see 5.5.4) indicates the current fill level of the buffer. If

additional frames are written to the buffer although the FIFO is full, the overrun flag (register 0x0E bit 7)

is set. If the FIFO is reset, the FIFO fill level indicated in the frame_counter<6:0> is set to ‘0’ and the

overrun flag is reset each time a write operation happens to the FIFO configuration registers.

Note: the overrun bit is not reset when the FIFO fill level frame_counter<6:0> has decremented to ‘0’

due to reading from the FIFO_DATA register, but only when a write operation is performed on FIFO

configuration registers.

4.9.4.4

FIFO Interrupts

The FIFO supports two interrupts, a FIFO full interrupt and a watermark interrupt:

•

The FIFO full interrupt is issued when the buffer has been fully filled with samples. In FIFO

mode this occurs after 100 samples, and in STREAM mode after 99 samples, have been

stored in a previously empty FIFO.

The status of the FIFO-full interrupt may be read back through the status bit in

INT_STATUS_1 register 0x0A.

•

The watermark interrupt is issued when the fill level in the buffer has reached the frame

number defined by the water mark level trigger in 0x3D. The status of the watermark may be

read back through the address 0x0A bit 4 (fifo_int) status bit. Writing to water mark level

trigger in register 0x3D clears the FIFO buffer.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

24 |

62

Bosch Sensortec | BMI088 Data sheet

5.

Register Maps

5.1

Communication with the sensor

The  entire  communication  with  the  device  is  performed  by  reading  from  and  writing  to  registers.

Registers  have  a  width  of  8  bits;  they  are  mapped  to  an  8-bit  address  space. Accelerometer  and

gyroscope have individual register maps. The selection of the appropriate register map is done on digital

2

2

interface level by either selecting the corresponding chip select pin (SPI mode) or I

C address (I

C

mode). For details regarding the digital interface, see chapter 0.

The functional registers and the register addresses containing functional bits are marked in the following

register maps. All non-functional registers are marked as reserved and should be completely ignored by

the user.

It is recommended to mask out (logical and with zero) non-functional bits (marked with ‘-‘) of registers

which partially contain functional bits (i.e. read the register content first, changing bit by means of bit-

wise operations, and write the modified byte back to the register).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

25 |

62

Bosch Sensortec | BMI088 Data sheet

5.2

Register Map: Accelerometer

Legend

Read-only

Read/Write

Write-only

Reserved

Reset

Addr

Name

bit7

bit6

bit5

bit4

bit3

bit2

bit1

bit0

value

0x7E

ACC_SOFTRESET

0x00

softreset_cmd (0xb6)

0x7D

ACC_PWR_CTRL

0x00

acc_enable

0x7C

ACC_PWR_CONF

0x03

pwr_save_mode

0x7B - 0x6E: reserved

-

0x6D

ACC_SELF_TEST

0x00

acc_self_test

0x6B - 0x59: reserved

-

0x58

INT_MAP_DATA

0x00

-

int2_drdy

int2_fwm

int2_ffull

-

int1_drdy

Int1_fwm

Int1_ffull

0x57 - 0x55: reserved

-

0x54

INT2_IO_CTRL

0x00

-

int2_in

int2_out

int2_od

int2_lvl

-

0x53

INT1_IO_CTRL

0x00

-

int1_in

int1_out

int1_od

int1_lvl

-

0x52 - 0x4A: reserved

-

FIFO_CONFIG_1

0x10

-

acc_en

-

1

int1_en

int2_en

0x49

-

0x48

FIFO_CONFIG_0

0x02

1

mode

-

fifo_water_mark[12:8]

0x47

FIFO_WTM_1

0x02

fifo_water_mark[7:0]

0x46

FIFO_WTM_0

0x00

fifo_downs

-

0x45

FIFO_DOWNS

0x80

1

0x44 - 0x42: reserved

-

0x41

ACC_RANGE

0x01

-

acc_range

acc_odr

0x40

ACC_CONF

0xA8

acc_bwp

0x3F - 0x27: reserved

-

fifo_ data

0x00

0x26

FIFO_DATA

fifo_byte_counter[13:8]

0x25

FIFO_LENGTH_1

0x00

-

fifo_byte_counter[7:0]

0x24

FIFO_LENGTH_0

0x00

temperature[2:0]

-

0x23

TEMP_LSB

0x00

temperature[10:3]

0x22

TEMP_MSB

0x00

0x21 - 0x1E: reserved

-

0x1D

ACC_INT_STAT_1

0x00

-

acc_drdy

0x1C - 0x1B: reserved

-

sensor_time[23:16]

0x1A

SENSORTIME_2

0x00

sensor_time[15:8]

0x19

SENSORTIME_1

0x00

sensor_time[7:0]

0x18

SENSORTIME_0

0x00

acc_z[15:8]

0x17

ACC_Z_MSB

0x00

acc_z[7:0]

0x16

ACC_Z_LSB

0x00

acc_y[15:8]

0x15

ACC_Y_MSB

0x00

acc_y[7:0]

0x14

ACC_Y_LSB

0x00

acc_x[15:8]

0x13

ACC_X_MSB

0x00

acc_x[7:0]

0x12

ACC_X_LSB

0x00

0x11 - 0x04: reserved

-

0x03

ACC_STATUS

0x10

-

drdy_acc

error_code

0x02

ACC_ERR_REG

0x00

-

-

fatal_ err

0x01

-

-

-

acc_chip_id

0x00

ACC_CHIP_ID

0x1E

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

26 |

62

Bosch Sensortec | BMI088 Data sheet

5.3

Register Description: Accelerometer

Register 0x00: ACC_CHIP_ID

5.3.1

Bit

Access

Reset

Description

value

[7:0]

Contains identifier code of acceleration sensor

RO

0x1E

Register 0x02: ACC_ERR_REG

5.3.2

Reports sensor error conditions.

Bit

Name

Access

Reset

Description

value

[7:5]

reserved

[4:2]

error_code

RO

0x00

Error codes for persistent errors:

0x00: no error

0x01: error occurred in accelerometer configuration

(unvalid data in register ACC_CONF)

[1]

reserved

[0]

fatal_err

RO

0x0

Fatal Error, chip is not in operational state (Boot-,

power-system). This flag will be reset only by

power-on-reset or soft-reset.

Register 0x03: ACC_STATUS

5.3.3

Sensor status flag.

Bit

Name

Access

Reset

Description

value

[7]

acc_drdy

RO

0x0

Data ready for Accelerometer. Reset when one

acceleration data register is read out.

[6:0]

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

27 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x12 – 0x17: ACC data

5.3.4

Registers containing the acceleration sensor output. The sensor output is stored as signed 16 bit number

in  2’s  complement  format  in  each  2  registers.  From  the  registers,  the  acceleration  values  can  be

calculated as follows:

Accel_X_int16 = ACC_X_MSB * 256 + ACC_X_LSB

Accel_Y_int16 = ACC_Y_MSB * 256 + ACC_Y_LSB

Accel_Z_int16 = ACC_Z_MSB * 256 + ACC_Z_LSB

When a register is read containing the LSB value of an acceleration value, the corresponding MSB

register is locked internally, until it is read. By this mechanism, it is ensured that both LSB and MSB

values belong to the same acceleration value and are not updated between the readouts of the individual

registers.

The unit is in LSB. The conversion from LSB to acceleration (mg) is based on the range settings and

can be calculated as follows (<0x41>: content of the ACC_RANGE register):

Accel_X_in_mg = Accel_X_int16 / 32768 * 1000 * 2^(<0x41> + 1) * 1.5

Accel_Y_in_mg = Accel_Y_int16 / 32768 * 1000 * 2^(<0x41> + 1) * 1.5

Accel_Z_in_mg = Accel_Z_int16 / 32768 * 1000 * 2^(<0x41> + 1) * 1.5

5.3.5

Register 0x18 – 0x1A: Sensortime data

Registers containing the value of the internal 24-bit counter.

•

Register  0x18  (SENSORTIME_0)  contains  the  lower  8  bits  of  the  counter.  This  register  is

incremented every 39.0625 µs.

•

Register  0x19  (SENSORTIME_1)  contains  the  middle  8  bits  of  the  counter.  This  register  is

incremented on SENSORTIME_0 overflow, which is every 10 ms.

•

Register  0x1A  (SENSORTIME_2)  contains  the  higher  8  bits  of  the  counter.  This  register  is

incremented on SENSORTIME_1 overflow, which is every 2.56 s.

The complete 24-bit counter overflows after 655.36 s or almost 11 minutes.

Register 0x1D: ACC_INT_STAT_1

5.3.6

Interrupt status register.

Bit

Name

Access

Reset

Description

value

[7]

acc_drdy

RO

0x00

Acceleration data ready interrupt. Cleared on read

of this register

[6:0]

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

28 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x22 – 0x23: Temperature sensor data

5.3.7

Registers containing the temperature sensor data output. The data is stored in an 11-bit value in 2’s

complement format. The resolution is 0.125°C/LSB, thus the temperature can be obtained as follows:

Temp_uint11 = (TEMP_MSB * 8) + (TEMP_LSB / 32)

if Temp_uint11 > 1023:

Temp_int11 = Temp_uint11 – 2048

else:

Temp_int11 = Temp_uint11

Temperature = Temp_int11 * 0,125°C/LSB + 23°C

TEMP_MSB

TEMP_LSB

Temp_int11

Temperature

0x3E

0x00

496

85 °C

…

…

…

…

0x00

0x60

3

23.375 °C

0x00

0x40

2

23.250 °C

0x00

0x20

1

23.125 °C

0x00

0x00

0

23.0°C

…

…

…

…

0xC1

0x00

-504

-40 °C

0x80

Invalid

The temperature sensor data is updated every 1.28 s.

Register 0x24-0x25: FIFO_LENGTH

5.3.8

The FIFO length registers FIFO_LENGTH_1 and FIFO_LENGTH_0 contain the 14 bit FIFO byte

counter. The counter represents the current fill level of the FIFO buffer.

An empty FIFO corresponds to 0x8000. A FIFO content reset can be triggered by reading out all

frames from the FIFO buffer or by writing 0xB0 into register 0x7E. The byte counter is updated when a

complete frame is read or written.

Register 0x26: FIFO_DATA

5.3.9

When reading out data from FIFO, burst read access must be used. The address will not increase

when burst read at the address of FIFO_DATA. The FIFO data is organized in frames as described in

section 4.9.3.2.

When a frame is partially read through FIFO Data Register 0x26, it will be repeated completely with the

next access. However, in case of a FIFO overflow between the first partial read and the second read

attempt, the frame may be deleted.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

29 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x40: ACC_CONF

5.3.10

Accelerometer configuration register.

Bit

Name

Access

Reset

Description

value

[7:4]

acc_bwp

RW

0x0A

This parameter influences the bandwidth of the

accelerometer low pass filter. For details, see

section 4.4.1. Possible values:

acc_bwp

Filter setting

0x08

OSR4 (4-fold oversampling)

0x09

OSR2 (2-fold oversampling)

0x0A

Normal

others

reserved

[3:0]

acc_odr

RW

0x08

This parameter sets the output data rate ODR.

Possible values:

acc_odr

ODR in Hz

0x00 – 0x04

reserved

0x05

12.5

0x06

25

0x07

50

0x08

100

0x09

200

0x0A

400

0x0B

800

0x0C

1600

0x0D – 0X0F

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

30 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x41: ACC_RANGE

5.3.11

Accelerometer range setting register.

Bit

Name

Access

Reset

Description

value

[7:2]

reserved

[1:0]

acc_range

RW

0x01

This parameter sets the measurement range.

Possible values:

acc_range

Range setting

0x00

±3g

0x01

±6g

0x02

±12g

0x03

±24g

Register 0x45: FIFO_DOWNS

5.3.12

Reduction of sample rate.

Bit

Name

Access

Reset

Description

value

[7]

Reserved

RW

0x01

This bit must always be ‘1’.

[6:4]

fifo_downs

RO

0x00

Reduction of sample rate by a factor

2**fifo_downs.

Example: fifo_downs=5 will reduce the FIFO

sampling rate by a factor of 2**5 = 32 in relation to

the chosen ODR of the sensor signal.

[3:0]

reserved

Register 0x46 – 0x47: FIFO_WTM

5.3.13

Registers containing the 13 bit FIFO watermark level value. A FIFO water mark interrupt signal is

active if the FIFO fill level is equal or greater than fifo_water_mark [12:0] (unit of the fifo water mark is

one byte).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

31 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x48: FIFO_CONFIG_0

5.3.14

Sets the FIFO mode.

Bit

Name

Access

Reset

Description

value

[7:2]

reserved

[1]

Reserved

RW

0x01

This bit must always be ‘1’.

[0]

mode

RW

0x00

This parameter sets the FIFO mode.

Possible values:

value

mode

0x00

STREAM mode

0x01

FIFO mode

Register 0x49: FIFO_CONFIG_1

5.3.15

Selects sources for the FIFO buffer.

Bit

Name

Access

Reset

Description

value

[7]

reserved

[6]

Acc_en

RW

0x00

Enables storing of accelerometer sensor data

[5]

Reserved

RW

0x00

[4]

Reserved

RW

0x01

This bit must always be ‘1’.

[3]

Enables storing of captured interrupt events at pin

Int1_en

RW

0x00

INT1 (pin needs to be configured as input pin

accordingly)

[2]

Enables storing of captured interrupt events at pin

Int2_en

RW

0x00

INT2 (pin needs to be configured as input pin

accordingly)

[1:0]

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

32 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x53: INT1_IO_CONF

5.3.16

Configures the input/output pin INT1.

Bit

Name

Access

Reset

Description

value

[7:5]

reserved

[4]

int1_in

RW

0x00

Enable INT1 as input pin.

[3]

int1_out

RW

0x00

Enable INT1 as output pin.

[2]

int1_od

RW

0x00

int1_od

Pin behavior

0x00

Push-pull

0x01

Open-drain

[1]

int1_lvl

RW

0x00

int1_lvl

Active state

0x00

Active low

0x01

Active high

[0]

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

33 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x54: INT2_IO_CONF

5.3.17

Configures the input/output pin INT2.

Bit

Name

Access

Reset

Description

value

[7:5]

reserved

[4]

int2_io

RW

0x00

Enable INT2 as input pin.

[3]

int2_out

RW

0x00

Enable INT2 as output pin.

[2]

int2_od

RW

0x00

int2_od

Pin behavior

0x00

Push-pull

0x01

Open-drain

[1]

int2_lvl

RW

0x00

int2_lvl

Active state

0x00

Active low

0x01

Active high

[0]

reserved

Register 0x58: INT1_INT2_MAP_DATA

5.3.18

Map data ready interrupt to output pin INT1 and/or INT2.

Bit

Name

Access

Reset

Description

value

[7]

reserved

[6]

Int2_drdy

RW

0x00

Map data ready interrupt to pin INT2

[5]

int2_fwm

RW

0x00

Map FIFO watermark interrupt to pin INT2

[4]

Map FIFO full interrupt to pin INT2

int2_ffull

RW

0x00

[5:3]

reserved

[2]

Int1_drdy

RW

0x00

Map data ready interrupt to pin INT1

[1]

int1_fwm

RW

0x00

Map FIFO watermark interrupt to pin INT1

[0]

int1_ffull

RW

0x00

Map FIFO full interrupt to pin INT1

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

34 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x6D: ACC_SELF_TEST

5.3.19

Enables the sensor self-test signal, occurring as a steady offset to the sensor output. Note that the

self-test needs to be switched off actively by the user (details see 4.6.1).

Bit

Access

Reset

Description

value

[7:0]

RW

0x00

self_test

Behaviour

0x00

Self-test is switched off.

0x0D

Enable positive self-test signal.

0x09

Enable negative self-test signal.

Register 0x7C: ACC_PWR_CONF

5.3.20

Switches accelerometer into suspend mode for saving power. In this mode the data acquisition is

stopped.

Bit

Name

Access

Reset

Description

value

[7:0]

acc_pwr_save

RW

0x03

acc_pwr_save

Filter setting

0x03

Suspend mode

0x00

Active mode

Register 0x7D: ACC_PWR_CTRL

5.3.21

Switches accelerometer ON or OFF. Required to do after every reset in order to obtain acceleration

values.

Bit

Name

Access

Reset

Description

value

[7:0]

acc_enable

RW

0x00

acc_enable

Filter setting

0x00

Accelerometer off

0x04

Accelerometer on

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

35 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x7E: ACC_SOFTRESET

5.3.22

Bit

Access

Reset

Description

value

Writing a value of 0xB6 to this register resets the sensor. (Do not write

[7:0]

W

N/A

any other content to this register.)

Following a delay of 1 ms, all configuration settings are overwritten with

their reset value.

The soft-reset can be triggered from any operation mode.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

36 |

62

Bosch Sensortec | BMI088 Data sheet

5.4

Register Map: Gyroscope

Reserved

Legend

Read-only

Read/Write

Write-only

Reset

bit4

bit3

bit2

bit1

bit0

bit5

Addr

Name

bit7

bit6

value

FIFO_DATA

N/A

0x3F

fifo_data_output_register

0x3E

FIFO_CONFIG_1

0x00

fifo_mode

-

FIFO_CONFIG_0

0x00

0x3D

fifo_water_mark_level_trigger_retain

0x3C

GYRO_SELF_TEST

N/A

bist_rdy

trig_ bist

-

bist_ fail

-

rate_ok

0x3B - 0x35: reserved

-

-

0x00

0x34

FIFO_EXT_INT_S

ext_fifo_s_en ext_fifo_s_sel

0x33 - 0x1F: reserved

-

0x00

0x1E

FIFO_WM_EN

fifo_watermark_enable

0x1D - 0x19: reserved

-

INT3_INT4_IO_MAP

0x00

0x18

Int3_ fifo

-

Int3_data

-

Int4_ fifo

Int4_data

-

0x17: reserved

-

INT3_INT4_IO_CONF

0x0F

0x16

Int3_ lvl

Int3_od

Int4_ lvl

-

Int4_od

0x00

0x15

GYRO_INT_CTRL

data_en

fifo_ en

-

0x14

GYRO_SOFTRESET

N/A

softreset

0x13 - 0x12: reserved

-

0x11

GYRO_LPM1

0x00

gyro_pm

0x10

GYRO_BANDWIDTH

0x80

gyro_bw

0x0F

GYRO_RANGE

0x00

gyro_range

0x0E

FIFO_STATUS

N/A

fifo_overrun

fifo_frame_counter

0x0D - 0x0B: reserved

-

N/A

0x0A

GYRO_INT_STAT_1

fifo_ int

gyro_drdy

-

-

0x09 - 0x08: reserved

-

N/A

0x07

RATE_Z_MSB

rate_z[15:8]

0x06

RATE_Z_LSB

N/A

rate_ z[7:0]

0x05

RATE_Y_MSB

N/A

rate_y[15:8]

0x04

RATE_Y_LSB

N/A

rate_ y[7:0]

0x03

RATE_X_MSB

N/A

rate_x[15:8]

0x02

RATE_X_LSB

N/A

rate_ x[7:0]

0x01

Reserved

N/A

-

0x00

GYRO_CHIP_ID

0x0F

gyro_chip_id

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

37 |

62

Bosch Sensortec | BMI088 Data sheet

5.5

Register Description: Gyroscope

Register 0x00: GYRO_CHIP_ID

5.5.1

Bit

Access

Reset

Description

value

[7:0]

RO

Contains identifier code of gyroscope.

0x0F

Register 0x02 – 0x07: Rate data

5.5.2

Registers containing the angular velocity sensor output. The sensor output is stored as signed 16-bit

number  in  2’s  complement  format  in  each  2  registers.  From  the  registers,  the  gyro  values  can  be

calculated as follows:

Rate_X: RATE_X_MSB * 256 + RATE_X_LSB

Rate_Y: RATE_Y_MSB * 256 + RATE_Y_LSB

Rate_Z: RATE_Z_MSB * 256 + RATE_Z_LSB

When a register is read containing the LSB value of a rate value, the corresponding MSB register is

locked internally, until it is read. By this mechanism, it is ensured that both LSB and MSB values belong

to the same rate range value and are not updated between the readouts of the individual registers.

The unit is in LSB. The conversion from LSB to angular velocity (degree per second) is based on the

range settings (see 5.5.5). For example, for the default range setting of 0x00 in register 0x0F, the

following conversion table applies:

Sensor output [LSB]

Angular rate (in 2000°/s range mode)

+32767

+ 2000°/s

…

…

0

0°/s

…

…

-32767

- 2000°/s

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

38 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x0A: GYRO_INT_STAT_1

5.5.3

Bit

Name

Access

Reset

Description

value

[7]

gyro_drdy

RO

N/A

Data ready interrupt status. The interrupt is cleared

automatically after 280-400 µs.

reserved

[6:5]

[4]

FIFO interrupt status

fifo_int

RO

N/A

reserved

[3:0]

Register 0x0E: FIFO_STATUS

5.5.4

The register contains FIFO status information.

Bit

Name

Access

Reset

Description

value

[7]

Fifo_overrun

RO

If set, FIFO overrun condition has occurred.

N/A

Note: flag can only be cleared by writing to

the FIFO configuration register

FIFO_CONFIG_1

[6:0]

Fifo_frame_counter

RO

N/A

Current fill level of FIFO buffer. An empty

FIFO corresponds to 0x00. The frame counter

can be cleared by reading out all frames from

the FIFO buffer or writing to the FIFO

configuration register FIFO_CONFIG_1.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

39 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x0F: GYRO_RANGE

5.5.5

Bit

Access

Reset

Description

value

[7:0]

RW

0x00

Angular rate range and resolution. Possible values:

gyro_range

Full scale

Resolution

[°/s]

0x00

±2000

16.384 LSB/°/s



61.0 m°/s / LSB

0x01

±1000

32.768 LSB/°/s



30.5 m°/s / LSB

0x02

±500

65.536 LSB/°/s



15.3 m°/s / LSB

0x03

±250

131.072 LSB/°/s



7.6 m°/s / LSB

0x04

±125

262.144 LSB/°/s



3.8m°/s / LSB

Register 0x10: GYRO_BANDWIDTH

5.5.6

Bit

Access

Reset

Description

value

[7:0]

1

RW

0x80

The register allows the selection of the rate data filter bandwidth and

output data rate (ODR). Possible values:

gyro_bw

ODR [Hz]

Filter bandwidth [Hz]

0x00

2000

532

0x01

2000

230

0x02

1000

116

0x03

400

47

0x04

200

23

0x05

100

12

0x06

200

64

0x07

100

32

1

Note: bit #7 is read-only and always ‚1‘, but has no function and can safely be ignored.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

40 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x11: GYRO_LPM1

5.5.7

Selection of the main power modes. Please note that only switching between normal mode and the

suspend modes is allowed, it is not possible to switch between suspend and deep suspend and vice

versa.

Bit

Access

Reset

Description

value

[7:0]

RW

0x00

Switch to the main power modes.

gyro_pm

Power mode

0x00

normal

0x80

suspend

0x20

deep suspend

Register 0x14: GYRO_SOFTRESET

5.5.8

Bit

Access

Reset

Description

value

Writing  a  value  of  0xB6  to  this  register  resets  the  sensor.  (Other

[7:0]

W

N/A

values are ignored.)

Following a delay of 30 ms, all configuration settings are overwritten

with their reset value.

The soft reset can be triggered from any operation mode.

Register 0x15: GYRO_INT_CTRL

5.5.9

Bit

Access

Reset

Description

value

[7]

RW

0x0

Enables the new data interrupt to be triggered on new data.

[6]

RW

0x0

Enables the FIFO interrupt.

[5:0]

reserved

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

41 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x16: INT3_INT4_IO_CONF

5.5.10

Sets electrical and logical properties of the interrupt pins.

Bit

Name

Access

Reset

Description

value

[3]

Int4_od

RW

‘1’

Int4_od

Pin INT4 output configuration

‘0’

Push-pull

‘1’

Open-drain

[2]

Int4_lvl

RW

‘1’

Int4_lvl

Pin INT4 active state

‘0’

Active low

‘1’

Active high

[1]

Int3_od

RW

‘1’

Int3_od

Pin INT3 output configuration

‘0’

Push-pull

‘1’

Open-drain

[0]

Int3_lvl

RW

‘1’

Int3_lvl

Pin INT3 active state

‘0’

Active low

‘1’

Active high

Register 0x18: INT3_INT4_IO_MAP

5.5.11

Map the data ready interrupt pin to one of the interrupt pins INT3 and/or INT4.

Bit

Access

Reset

Description

value

Data ready interrupt is mapped to INT4 pin.

[7]

RW

0x0

reserved

[6]

[5]

RW

0x0

FIFO interrupt is mapped to INT4.

[4:3]

reserved

[2]

RW

FIFO interrupt is mapped to INT3.

0x0

reserved

[1]

[0]

RW

0x0

Data ready interrupt is mapped to INT3 pin.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

42 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x1E: FIFO_WM_ENABLE

5.5.12

Enables FIFO watermark level interrupt.

Bit

Access

Reset

Description

value

[7:0]

RW

0x08

Value

description

0x08

FIFO watermark level interrupt disabled

0x88

FIFO watermark level interrupt enabled

Register 0x34: FIFO_EXT_INT_S

5.5.13

Bit

Access

Reset

Description

value

reserved

[7:6]

[5]

RW

0x00

It set, enables external FIFO synchronization mode

Selects source for external FIFO synchronization

[4]

RW

0x00

ext_fifo

Behavior

_s_sel

0x0

Source is pin INT3

0x1

Source is pin INT4

reserved

[3:0]

Register 0x3C: GYRO_SELF_TEST

5.5.14

Built-in self-test of gyroscope.

Bit

Access

Name

Reset

Description

value

[4]

R

rate_ok

‘0’

A value of ‘1’ indicates proper sensor function.

If ‘0’ and bist_rdy = ‘1’: built-in self-test is ok, sensor is

[2]

R

bist_fail

‘0’

ok

If ‘1’ and bist_rdy = ‘1’: built-in self-test is not ok,

sensor values may not be in expected range

If bit is ‘1’, built-in self-test has been performed and

[1]

R

bist_rdy

‘0’

finished

[0]

W

trig_bist

Setting this bit to ‘1’ (i.e. writing 0x01 to this register)

N/A

starts the built-in self-test.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

43 |

62

Bosch Sensortec | BMI088 Data sheet

Register 0x3D: FIFO_CONFIG_0

5.5.15

Bit

Access

Reset

Description

value

[7]

Reserved

[6:0]

RW

0x00

fifo_water_mark_level_trigger_retain<6:0> defines the FIFO

watermark level. An interrupt will be generated, when the number of

entries in the FIFO exceeds

fifo_water_mark_level_trigger_retain<6:0>.

Writing to this register clears the FIFO buffer.

Register 0x3E: FIFO_CONFIG_1

5.5.16

Contains FIFO configuration settings. The FIFO buffer memory is cleared and the fifo-full flag is

cleared when writing to FIFO_CONFIG_1 register. In addition, the FIFO overrun flag (see 5.5.4) is

cleared (it overrun occurred before).

Bit

Access

Reset

Description

value

[7:0]

RW

0x08

fifo_

mode

description

mode

0x40

data collection stops once buffer is full (i.e.

FIFO

filled with 100 frames)

sampling continues when buffer is full (i.e.

0x80

STREAM

filled with 99 frames); old is discarded

else

reserved

Register 0x3F: FIFO_DATA

5.5.17

FIFO data readout register. The format of the LSB and MSB components corresponds to that of the

angular rate data readout registers. Read burst access may be used since the address counter will not

increment when the read burst is started at the address of FIFO_DATA. The entire frame is discarded

when a fame is only partially read out.

The format of the data read-out from register 0x3F is as follows:

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

44 |

62

Bosch Sensortec | BMI088 Data sheet

6.

Digital Interface

The BMI088 supports two serial digital interface protocols for communication as a slave with a host

device: SPI and I²C. The active interface is selected by the state of the Pin#07 (PS) ‘protocol select’ pin:

•

PS = ´VDDIO´ selects I²C

•

PS = ´GND´ selects SPI

Important:

•

Please note that in case of SPI protocol the initialization process for the accelerometer part of

BMI088 requires some additional steps (see chapter 3).

•

Please  also  note  that  as  the  pins  of  the  package  are  shared  between  accelerometer  and

gyroscope part, it is not advisable to configure different interfaces for the two parts.

Both digital interfaces share partly the same pins. Additionally each inertial sensor (accelerometer and

gyroscope)  provides  specific  interface  pins,  which  allow  the  user  to  operate  the  inertial  sensors

independently of each other. The mapping for each interface and each inertial sensor is given in the

following table:

Table 10: Mapping of the interface pins

Pin#

Name

use w/

Description

use w/

SPI

I²C

15

SDO1

SDO1

address

SPI: Accel Data Output

I²C:  Used to set LSB of Accel I²C address

10

SDO2

SDO2

address

SPI: Gyro Data Output

I²C:  Used to set LSB of Gyro I²C address

9

SDA /

SDA

SDI

SDI

SPI: Accel and Gyro Data In

I²C:  Serial Data

14

CSB1

CSB1

unused

SPI: Accel Chip Select (enable)

5

CSB2

CSB2

unused

SPI: Gyro Chip Select (enable)

8

SCL /

SCL

SCK

SPI: Serial Clock SCK

SCK

I²C:  Serial Clock SCL

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

45 |

62

Bosch Sensortec | BMI088 Data sheet

The following table shows the electrical specifications of the interface pins:

Table 11: Electrical specification of the interface pins

Parameter

Symbol

Condition

Min

Typ

Max

Units

Pull-up Resistance,

75

kΩ

R

up

CSB pin

Internal Pull-up

Resistance to

VDDIO

100

125

Input Capacitance

C

5

in

10

pF

I²C Bus Load

pF

I2C_Load

C

Capacitance (max.

drive capability)

400

In order to allow for the correct internal synchronisation of data written to the BMI088, a wait time of at

least 2 µs (normal mode) or 1000 µs (suspend mode) must be followed.

6.1

Serial Peripheral Interface (SPI)

The behavior of the SPI interface is slightly different between gyroscope part and accelerometer part:

•

Initialization phase: as described in chapter 3, the interface of the gyroscope part is selected by

2

the level of the PS pin. In contrast to this, the accelerometer part starts always in I

C mode

(regardless of the level of the PS pin) and needs to be changed to SPI mode actively by sending

a rising edge on the CSB1 pin (chip select of the accelerometer), on which the accelerometer

part

switches

to

SPI

mode

and

stays

in

this

mode

until

the

next

power-up-reset.

To change the sensor to SPI mode in the initialization phase, the user could perfom a dummy

SPI read operation, e.g. of register ACC_CHIP_ID (the obtained value will be invalid).

•

In  case  of  read  operations,  the  SPI  interface  of  the  accelerometer  part  does  not  send  the

requested information directly after the master has send the corresponding register address,

but sends a dummy byte first, whose content is not predictable. Only after this dummy byte the

desired content is sent. (This dummy byte procedure does not apply to the gyroscope part.)

Please find more details below in section 6.1.2.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

46 |

62

Bosch Sensortec | BMI088 Data sheet

The timing specification for SPI of the BMI088 is given in the following table:

Table 12: SPI timing

Parameter

Symbol

Condition

Min

Max

Units

Clock Frequency

f

10

MHz

SPI

Max. Load on SDI or

SDO = 25pF

SCK Low Pulse

t

45

ns

SCKL

SCK High Pulse

t

ns

SCKH

45

SDI Setup Time

t

20

ns

SDI_setup

SDI Hold Time

t

20

ns

SDI_hold

SDO Output Delay

t

Load = 25pF

ns

SDO_OD

30

Load = 250pF,

ns

40

VDDIO > 2.4V

CSB Setup Time

t

40

ns

CSB_setup

CSB Hold Time

t

ns

CSB_hold

40

Idle time between write

normal mode

2

µs

IDLE_wacc

t

accesses

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

47 |

62

Bosch Sensortec | BMI088 Data sheet

The following figure shows the definition of the SPI timings:

t

t

CSB_hold

CSB_setup

CSB

SCK

t

t

SCKH

SCKL

SDI

t

t

SDI_hold

SDI_setup

SDO

t

SDO_OD

Figure 2: SPI timing diagram

The SPI interface of the BMI088 is compatible with two modes, ´00´ and ´11´. The automatic selection

between [CPOL = ´0´ and CPHA = ´0´] and [CPOL = ´1´ and CPHA = ´1´] is controlled based on the

value of SCK after a falling edge of CSB (1 or 2).

SPI interface of gyroscope part

6.1.1

For  single  byte  read  as  well  as  write  operations,  16-bit  protocols  are  used. The  SPI  interface  also

supports multiple-byte read operations (burst-read).

The communication starts when the CSB (1 or 2) is pulled low by the SPI master and stops when CSB

(1 or 2) is pulled high. SCK is also controlled by SPI master. SDI and SDO (1 or 2) are driven at the

falling edge of SCK and should be captured at the rising edge of SCK.

The data bits are used as follows:

•

Bit #0: Read/Write bit. When 0, the data SDI is written into the chip. When 1, the data SDO from

the chip is read.

•

Bit #1-7: Address AD(6:0).

•

Bit #8-15: when in write mode, these are the data SDI, which will be written into the address.

When in read mode, these are the data SDO, which are read from the address.

Multiple read operations (burst-read) are possible by keeping CSB low and continuing the data transfer

(i.e.  continuing  to  toggle  SCK).  Only  the  first  register  address  has  to  be  written.  Addresses  are

automatically incremented after each read access as long as CSB stays active low.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

48 |

62

Bosch Sensortec | BMI088 Data sheet

SPI interface of accelerometer part

6.1.2

In case of read operations of the accelerometer part, the requested data is not sent immediately, but

instead first a dummy byte is sent, and after this dummy byte the actual reqested register content is

transmitted.

This means that – in contrast to the description in section 6.1.1 – a single byte read operation requires

to read 2 bytes in burst mode, of which the first received byte can be discarded, while the second byte

contains the desired data.

The same applies to burst-read operations. For example, to read the accelerometer values in SPI mode,

the user has to read 7 bytes, starting from address 0x12 (ACC data). From these bytes the user must

discard the first byte and finds the acceleration information in byte #2 – #7 (corresponding to the content

of the addresses 0x12 – 0x17).

The data bits are used as follows:

•

Bit #0: Read/Write bit. When 0, the data SDI is written into the chip. When 1, the data SDO from

the chip is read.

•

Bit #1-7: Address AD(6:0).

•

Bit #8-15:

When in write mode, these are the data SDI, which will be written into the address.

o

When in read mode, these bits contain unpredictable values, and the user has to read

o

Bit #16-23 to get the actual data from the reading address.

6.2

Inter-Integrated Circuit (I²C)

The I²C bus uses SCL (= SCx pin, serial clock) and SDA (= SDx pin, serial data input and output) signal

lines. Both lines are connected to VDDIO externally via pull-up resistors so that they are pulled high

when the bus is free.

The I²C interface of the BMI088 is compatible with the I²C Specification UM10204 Rev. 03 (19 June

2007), available at http://www.nxp.com. The BMI088 supports I²C standard mode and fast mode, only

7-bit address mode is supported.

The default I²C addresses are:



Accelerometer:



SDO1 pin pulled to ‘GND’: 0011000b (0x18)



SDO1 pin pulled to ‘VDDIO’: 0011001b (0x19)



Gyroscope:



SDO2 pin pulled to ‘GND’: 1101000b (0x68)



SDO2 pin pulled to ‘VDDIO’: 1101001b (0x69)

The timing specification for I²C of the BMI088 is given in table 13:

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

49 |

62

Bosch Sensortec | BMI088 Data sheet

Table 13: I²C timings

Parameter

Symbol

Min

Max

Units

Clock Frequency

f

400

kHz

SCL

SCL Low Period

t

s

LOW

1.3

SCL High Period

t

µ

0.6

HIGH

SDA Setup Time

t

0.1

SUDAT

SDA Hold Time

t

HDDAT

0.0

Setup Time for a repeated Start

0.6

t

SUSTA

Condition

Hold Time for a Start Condition

t

0.6

HDSTA

Setup Time for a Stop Condition

t

SUSTO

0.6

Time before a new Transmission can

t

BUF

1.3

start

Idle time between write accesses,

µs

t

IDLE_wacc_nm

2

normal mode

Idle time between write accesses,

t

450

µs

IDLE_wacc_sum

suspend mode

Figure 3 shows the definition of the I²C timings given in table 13:

SDA

t

BUF

t

LOW

t

f

SCL

t

t

HIGH

t

HDDAT

HDSTA

t

r

t

SUDAT

SDA

t

SUSTA

t

SUSTO

Figure 3: I²C timing diagram

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

50 |

62

Bosch Sensortec | BMI088 Data sheet

The I²C protocol works as follows:

START: Data transmission on the bus begins with a high to low transition on the SDA line while SCL is

held high (start condition (S) indicated by I²C bus master). Once the START signal is transferred by the

master, the bus is considered busy.

STOP: Each data transfer should be terminated by a Stop signal (P) generated by master. The STOP

condition is a low to HIGH transition on SDA line while SCL is held high.

ACK: Each byte of data transferred must be acknowledged. It is indicated by an acknowledge bit sent

by the receiver. The transmitter must release the SDA line (no pull down) during the acknowledge pulse

while the receiver must then pull the SDA line low so that it remains stable low during the high period of

the acknowledge clock cycle.

In the following diagrams, these abbreviations are used:

S

Start

P

Stop

ACKS

Acknowledge by slave

ACKM

Acknowledge by master

NACKM

Not acknowledge by master

RW

Read / Write

A  START  immediately  followed  by  a  STOP  (without  SCL  toggling  from  ´VDDIO´  to  ´GND´)  is  not

supported. If such a combination occurs, the STOP is not recognized by the device.

I²C write access:

I²C write access can be used to write a data byte in one sequence.

The sequence begins with start condition generated by the master, followed by 7 bits slave address and

a write bit (RW = 0). The slave sends an acknowledge bit (ACK = 0) and releases the bus. Then the

master sends the one byte register address. The slave again acknowledges the transmission and waits

for  the  8  bits  of  data,  which  shall  be  written  to  the  specified  register  address.  After  the  slave

acknowledges the data byte, the master generates a stop signal and terminates the writing protocol.

Example  of  an  I²C  write  access  to  the  accelerometer,  writing  0xA8  to  address  ox40  (i.e.  setting

continuous filter function, averaging to 4 samples, ODR to 100Hz):

Slave address (0x18)

RW

Register address (0x40)

Data (0xA8)

Start

Stop

ACKS

ACKS

ACKS

dummy

S

0

0

1

0

0

0

A

0

0

0

0

0

0

A

1

0

1

0

1

0

0

0

A

P

1

0

0

1

Figure 4: I²C write

I²C read access:

I²C read access also can be used to read one or multiple data bytes in one sequence.

A read sequence consists of a one-byte I²C write phase followed by the I²C read phase. The two parts

of the transmission must be separated by a repeated start condition (Sr). The I²C write phase addresses

the slave and sends the register address to be read. After slave acknowledges the transmission, the

master generates again a start condition and sends the slave address together with a read bit (RW =

1). Then the master releases the bus and waits for the data bytes to be read out from slave. After each

data byte the master has to generate an acknowledge bit (ACK = 0) to enable further data transfer. A

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

51 |

62

Bosch Sensortec | BMI088 Data sheet

NACKM (ACK = 1) from the master stops the data being transferred from the slave. The slave releases

the bus so that the master can generate a STOP condition and terminate the transmission.

The  register  address  is  automatically  incremented  and,  therefore,  more  than  one  byte  can  be

sequentially read out. Once a new data read transmission starts, the start address will be set to the

register address specified in the latest I²C write command. By default, the start address is set at 0x00.

In this way, repetitive multi-bytes reads from the same starting address are possible.

Example of an I²C read access to the accelerometer, reading all 6 bytes containing acceleration data

(0x12-0x17):

Slave address (0x18)

Register address (0x12)

RW

Start

ACKS

ACKS

dummy

S

0

0

1

1

0

0

0

0

A

x

0

0

1

0

0

1

0

A

Slave address (0x18)

Read data (0x13)

Read data (0x12)

RW

Start

ACKS

ACKM

ACKM

Sr

0

0

1

1

0

0

0

1

A

x

x

x

x

x

x

x

x

A

x

x

x

x

x

x

x

x

A

…

Read data (0x14)

Read data (0x15)

ACKS

ACKM

ACKM

… A

x

x

x

x

x

x

x

x

A

x

x

x

x

x

x

x

x

A

…

Read data (0x16)

Read data (0x17)

Stop

ACKS

ACKM

NACKM

… A

x

x

x

x

x

x

x

x

A

x

x

x

x

x

x

x

x

NA

P

Figure 5: I²C multiple read

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

52 |

62

Bosch Sensortec | BMI088 Data sheet

7.

Pin-out and Connection Diagram

Figure 6: Pin-out top view                           Figure 7: Pin-out bottom view

7.1

Pin-out

Table 14: Pin description

1

2

Pin#

Name

I/O Type

Description

SPI mode

I

C mode

1*

INT2

Digital I/O

Interrupt pin 2 (accel int #2)

INT2

INT2

2

NC

--

--

GND

GND

3

VDD

Supply

Power supply analog & digital

VDD

VDD

domain (2.4 – 3.6V)

4

GNDA

Ground

Ground for analog domain

GND

GND

5

CSB2

Digital in

SPI Chip select Gyro

CSB2

DNC (float)

6

GNDIO

Ground

Ground for I/O

GND

GND

7

PS

Digital in

Protocol select gyroscope

GND

VDDIO

(GND = SPI, VDDIO = I²C)

8

SCL /

SCL

Digital in

SPI: serial clock SCK

SCK

SCK

I²C: serial clock SCL

9

SDA /

Digital I/O

SDA

I²C: SDA serial data I/O

SDI

SDI

SPI 4W: SDI serial data I

SPI 3W: SDA serial data I/O

10

SDO2

Digital out

SPI Serial data out Gyro

SDO2

GND

Address select in I²C mode

for default addr.

see chapter 9.2

11

VDDIO

Supply

Digital I/O supply voltage

VDDIO

VDDIO

(1.2V … 3.6V)

12*

INT3

Digital I/O

Interrupt pin 3 (gyro int #1)

INT3

INT3

13*

INT4

Digital I/O

Interrupt pin 4 (gyro int #2)

INT4

INT4

14

CSB1

Digital in

SPI Chip select Accel

CSB1

VDDIO or DNC

(float)

15

SDO1

Digital out

SPI Serial data out Accel

SDO1

GND

Address select in I²C mode

for default addr.

see chapter 9.2

16*

INT1

Digital I/O

Interrupt pin 1 (accel int #1)

INT1

INT1

* If INT are not used, do not connect them (DNC)!

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

53 |

62

Bosch Sensortec | BMI088 Data sheet

7.2

Connection diagram SPI

Figure 8: SPI connection

2

7.3

Connection diagram I

C

BMI088

2

Figure 9: I

C connection

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

54 |

62

Bosch Sensortec | BMI088 Data sheet

8.

Package

8.1

Outline Dimensions

The sensor housing is a standard LGA package. Its dimensions are the following.

Unit is mm. Note: Unless otherwise specified tolerance = decimal ± 0.05

Top view

Figure 10: Package outline dimensions

Bottom view

Figure 10: Package dimensions

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

55 |

62

Bosch Sensortec | BMI088 Data sheet

8.2

Landing pattern

For the design of the landing patterns, we recommend the following dimensioning:

0.675

16

15

1

0.25

14

2

0.5

13

3

12

4

4.5

11

5

1.675

10

6

7

9

8

0.925

3.0

Figure 11: Landing pattern recommendation

Same tolerances as given for the outline dimensions in 8.1 should be assumed. A wiring no-go area in

the top layer of the PCB below the sensor is strongly recommended (e.g. no vias, wires or other metal

structures).

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

56 |

62

Bosch Sensortec | BMI088 Data sheet

8.3

Sensing axes orientation

If the sensor is accelerated and/or rotated in the indicated directions, the corresponding channels of the

device will deliver a positive acceleration and/or yaw rate signal (dynamic acceleration). If the sensor is

at rest without any rotation and the force of gravity is acting contrary to the indicated directions, the

output  of  the  corresponding  acceleration  channel  will  be  positive  and  the  corresponding  gyroscope

channel will be “zero” (static acceleration).

Example: If the sensor is at rest or at uniform motion in a gravity field according to the figure given below,

the output signals are:

GYR channel

•

0g for the X ACC channel

and

0°/sec for the Ω

X

GYR channel

•

0g for the Y ACC channel

and

0°/sec for the Ω

Y

GYR channel

•

+ 1g for the Z ACC channel

and

0°/sec for the Ω

Z

force

of gravity

Figure 12: Orientation of sensing axis

The following table lists all corresponding output signals on X, Y, Z while the sensor is at rest or at

uniform motion in a gravity field under assumption of a top down gravity vector as shown above.

, Ω

, Ω

show 0dps output under these static conditions.

The gyroscope signals Ω

X

Y

Z

Table 15: Output signals depending on device orientation

Sensor orientation

(gravity vector    )

o

o

upright

upright

o

o

Output Signal X

0g

+1g

0g

-1g

0g

0g

Output Signal Y

-1g

0g

+1g

0g

0g

0g

Output Signal Z

0g

0g

0g

0g

+1g

-1g

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

57 |

62

Bosch Sensortec | BMI088 Data sheet

8.4

Marking

Mass production samples

8.4.1

Table 16: Marking of mass production parts

Labeling

Name

Symbol

Remark

Product number

365

3 numeric digits, fixed to identify

product type

365

LYYWW

Sub-con ID

L

1 alphanumeric digit,

variable to identify sub-con

CCC

Date-Code

YYWW

4 numeric digits, fixed to identify YY = “year”

WW = “working week

Lot counter

CCC

3 alphanumeric digits, variable

to generate mass production trace-code

•

Pin 1 identifier

--

Engineering samples

8.4.2

Table 17: Marking of engineering samples

Labeling

Name

Symbol

Remark

Eng. sample ID

N

1 alphanumeric digit, fixed to identify

engineering sample, N = “+” or “e” or “E”

088N

PYYWW

Sample ID

PYYWW

P: assembly house

YYWW: Year (last 2 digits)/Work week

Counter ID

CC

C-samples; lot number

th

(e.g.C5: C-samples, 5

lot)

CC

•

Pin 1 identifier

--

8.5

PCB layout and soldering guidelines

The following general layout rules are recommended

•

PCB land width = LGA solder pin width

•

PCB land length = LGA solder pin length + 0.1 mm on each side

•

Solder mask opening width = PCB land width + 0.05 mm on each side

•

Solder mask opening length = PCB land length + 0.05 mm on each side

Recommendation about stencil design and solder paste application

•

It is recommended to keep the openings of the stencil mask for the signal pads between 70%

and 90% of the PCB pad area.

•

An  accurate  alignment  of  the  stencil  and  the  printed  circuit  board  (within  0.025mm)  is

recommended.

•

A stencil thickness of 80 – 150 μm is recommended for screen printing

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

58 |

62

Bosch Sensortec | BMI088 Data sheet

The moisture sensitivity level (MSL) of the BMI088 sensors corresponds to JEDEC Level 1. See also:

•

IPC/JEDEC J-STD-020E  “Joint Industry Standard: Moisture/Reflow Sensitivity Classification

for non-hermetic Solid State Surface Mount Devices”

•

IPC/JEDEC  J-STD-033D  “Joint  Industry  Standard:  Handling,  Packing,  Shipping  and  Use  of

Moisture/Reflow Sensitive Surface Mount Devices”

The sensor fulfils the lead-free soldering requirements of the above-mentioned IPC/JEDEC standard,

i.e. reflow soldering with a peak temperature up to 260°C.

For more details, refer the Handling, Soldering and Mounting Instructions document available at

https://www.bosch-sensortec.com/bst/support_tools/downloads/overview_downloads

8.6

Handling instructions

Micromechanical sensors are designed to sense acceleration with high accuracy even at low amplitudes

and  contain  highly  sensitive  structures  inside  the  sensor  element.  The  MEMS  sensor  can  tolerate

mechanical shocks up to several thousand g’s. However, these limits might be exceeded in conditions

with extreme shock loads such as e.g. hammer blow on or next to the sensor, dropping of the sensor

onto hard surfaces etc.

We recommend to avoid g-forces beyond the specified limits during transport, handling and mounting

of the sensors in a defined and qualified installation process.

This  device  has  built-in  protections  against  high  electrostatic  discharges  or  electric  fields  (e.g.  2kV

HBM); however, anti-static precautions should be taken as for any other CMOS component. Unless

otherwise  specified,  proper  operation  can  only  occur  when  all  terminal  voltages  are  kept  within  the

supply voltage range. Unused inputs must always be tied to a defined logic voltage level.

8.7

Tape and Reel specification

BMI088 is shipped in a standard cardboard box. The box dimension for each reel is L x W x H = 35cm

x 35cm x 5cm. Each reel contains 5,000pcs of BMI088.

A

= 4.85; B

= 3.35; K

= 1.20

0

0

0

Tape and reel dimensions in mm

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

59 |

62

Bosch Sensortec | BMI088 Data sheet

Orientation within the reel

8.7.1

Orientation of the BMI088 devices relative to the tape

8.8

Environmental safety

The BMI088 sensor meets the requirements of the EC restriction of hazardous substances (RoHS)

directive, see also:

RoHS–Directive 2011/65/EU and its amendments, including the amendment 2015/863/EU on the

restriction of the use of certain hazardous substances in electrical and electronic equipment.

Halogen content

8.8.1

The  BMI088  is  halogen-free.  For  more  details  on  the  analysis  results  please  contact  your  Bosch

Sensortec representative.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

60 |

62

Bosch Sensortec | BMI088 Data sheet

9.

Legal Disclaimer

9.1

Engineering samples

Engineering  Samples  are  marked  with  an  asterisk  (*),  (E)  or  (e).  Samples  may  vary  from  the  valid

technical  specifications  of  the  product  series  contained  in  this  data  sheet.  They  are  therefore  not

intended or fit for resale to third parties or for use in end products. Their sole purpose is internal client

testing. The testing of an engineering sample may in no way replace the testing of a product series.

Bosch Sensortec assumes no liability for the use of engineering samples. The Purchaser shall indemnify

Bosch Sensortec from all claims arising from the use of engineering samples.

9.2

Product use

Bosch Sensortec products are developed for the consumer goods industry. They may only be used

within the parameters of this product data sheet. They are not fit for use in life-sustaining or safety-

critical systems. Safety-critical systems are those for which a malfunction is expected to lead to bodily

harm, death or severe property damage. In addition, they shall not be used directly or indirectly for

military purposes (including but not limited to nuclear, chemical or biological proliferation of weapons or

development of missile technology), nuclear power, deep sea or space applications (including but not

limited to satellite technology).

Bosch Sensortec products are released on the basis of the legal and normative requirements relevant

to the Bosch Sensortec product for use in the following geographical target market: BE, BG, DK, DE,

EE, FI, FR, GR, IE, IT, HR, LV, LT, LU, MT, NL, AT, PL, PT, RO, SE, SK, SI, ES, CZ, HU, CY, US, CN,

JP, KR, TW. If you need further information or have further requirements, please contact your local sales

contact.

The  resale  and/or  use  of  Bosch  Sensortec  products  are  at  the  purchaser’s  own  risk  and  his  own

responsibility. The examination of fitness for the intended use is the sole responsibility of the purchaser.

The purchaser shall indemnify Bosch Sensortec from all third party claims arising from any product use

not covered by the parameters of this product data sheet or not approved by Bosch Sensortec and

reimburse Bosch Sensortec for all costs in connection with such claims.

The purchaser accepts the responsibility to monitor the market for the purchased products, particularly

with regard to product safety, and to inform Bosch Sensortec without delay of all safety-critical incidents.

9.3

Application examples and hints

With  respect  to  any  examples  or  hints  given  herein,  any  typical  values  stated  herein  and/or  any

information  regarding  the  application  of  the  device,  Bosch  Sensortec  hereby  disclaims  any  and  all

warranties  and  liabilities  of  any  kind,  including  without  limitation  warranties  of  non-infringement  of

intellectual property rights or copyrights of any third party. The information given in this document shall

in no event be regarded as a guarantee of conditions or characteristics. They are provided for illustrative

purposes only and no evaluation regarding infringement of intellectual property rights or copyrights or

regarding functionality, performance or error has been made.

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

61 |

62

Bosch Sensortec | BMI088 Data sheet

10. Document History and Modification

Rev. No

Chapter

Description of modification/changes

Date

1.0

-

Initial release

03-Dec-2017

1.1

All

Typos

30-Jan-2018

3 + 4.1.1

5.2

1.2

5.3.6

23-Mar-2018

5.2+5.3.10+5.3.11

Updated switching between power modes

Fixed typo in register range

Added clearing condition of data ready int bit

Corrected INT pin settings bits

5.4

Fixed typo in bit naming in register 0x16

1.3

7.1

Updated pin 14 assignment in I2C mode

27-Apr-2018

4.4.1 + 5.2 + 5.3.8

Clarified how to set Accel bandwidth setting

4.9

Included info on FIFO

5.3.4

1.4

Corrected conversion from LSB to mg

7.2 + 7.3

16-Sep-2018

Updated pin 4 and 6 assignment in SPI/I2C mode

7.2

Clarified connection to SPI host interface

8.3

Updated marking

8.6

Included tape & reel specifications

4.9.1 – 4.9.4

Updated FIFO details

5.2, 5.4

Updated Register Maps with FIFO content

5.5.10

1.5

Updated reset values of Gyro Reg 0x16

8.2

07-Feb-2020

Included landing pattern

8.4.1

Corrected lot counter typo

8.5, 8.8

Updated content

9

Updated product use disclaimer

1.6

9

Disclaimer update

18-Nov-2020

8.8

Environmental safety – RoHS directive update

1.7

3

Device Initialization chapter update

08-Nov-2021

6.2

Timing t

update

IDLE_wacc_sum

1.8

4.8

Soft Reset chapter updated

17-Jan-2022

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

62 |

62

Bosch Sensortec | BMI088 Data sheet

Bosch Sensortec GmbH

Gerhard-Kindler-Straße 9

72770 Reutlingen / Germany

contact@bosch-sensortec.com

www.bosch-sensortec.com

Modifications reserved

Specifications subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022

Modifications reserved | Data subject to change without notice

Document number: BST-BMI088-DS000-18

Revision_1.8_012022