IST8310

IST8310

3D Magnetometer

Datasheet

IST8310 Datasheet, Version 1.2                                                                                                                              1

IST8310

Table of Contents

................................................................................... 4

1 GENERAL DESCRIPTION

... 5

2 BLOCK DIAGRAM, PACKAGE DIMENSION AND APPLICATION CIRCUIT

............................................................................................................ 5

2.1 Block diagram

....................................................................... 5

2.2 Package Dimensions and Pin Description

...................................................................................................... 7

2.3 Application Circuit

......................... 8

3 OPERATIONAL MODES AND FUNCTIONAL DESCRIPTIONS

......................................................................................................... 8

3.1 Operation modes

........................................................................................................ 8

3.1.1 Stand-By Mode

........................................................................................ 8

3.1.2 Single Measurement Mode

........................................................................................................ 8

3.1.3 Self-Test Mode

....................................................................................................... 9

3.2 Interrupt Function

.......................................................................................................... 9

3.3 DRDY Function

................................................................................................. 9

3.4 IST8310 Read Process

...................................................................... 10

4 ELECTRICAL SPECIFICATIONS

......................................................................................................... 10

4.1 Extreme Rating

.......................................................................... 10

4.2 Recommended Operating Conditions

............................................................................................. 10

4.3 Electrical Specifications

.................................................................................. 11

4.4 Magnetic Sensor Specifications

......................................................................... 12

4.5 Power On Reset (POR) Specifications

.............................................................................. 13

5 TECHNOLOGY OVERVIEW

...................................................................................................... 13

5.1 AMR Technology

............................................................................. 13

5.2 High Reliability Planarized Design

....................................................................................... 13

5.3 Ultra-low Hysteresis Design

..................................................................................... 13

5.4 Magnetic Setting Mechanism

.......................................................... 14

6 DIGITAL INTERFACE AND REGISTERS

IST8310 Datasheet, Version 1.2                                                                                                                              2

IST8310

2

............................................................................................................. 14

6.1 I

C Interface

2

................................................................................................... 14

6.2 I

C Read Operation

2

................................................................................................... 15

6.3 I

C Write Operation

.................................................................................................................. 15

6.4 Registers

............................................................................. 21

7 ORDERING INFORMATION

IST8310 Datasheet, Version 1.2                                                                                                                              3

IST8310

1 General Description

3

iSentek IST8310 is a 3-axis digital magnetometer with 3.0x3.0x1.0mm

, 16-pin LGA package. It is an

integrated chip with 3-axis magnetic sensors, digital control logic, built-in temperature compensation

2

circuit and self-test function. IST8310 provides an I

C digital output with fast mode up to 400kHz.

The high output data rate, ultra-low hysteresis, excellent temperature drift and low noise performance

feathers make it a perfect candidate for high accuracy applications.

Features

Single chip 3-axis magnetic sensor



3

3.0x3.0x1.0mm

, 16-pin LGA package



2

I

C slave, Fast Mode up to 400kHz



14 bits data output



Wide dynamic range of ±1600uT (x, y-axis) and ±2500uT (z-axis)



High output data rate of maximum 200Hz



Ultra-low hysteresis (<0.1%FS)



o

Ultra-low sensitivity temperature drift (±0.016 %/

K)



o

Ultra-low offset temperature drift (0.024uT/

K)



Wide operating temperature range



High precision temperature compensation



Built-in self-test function



Software and algorithm support available (For tilt compensation, cross-axis compensation and



noise suppression)

Applications

Quadcopter/Drone Applications

Augmented Reality Applications

Virtual Reality Applications

Location Based Services

Navigation Applications

Industrial Applications

Magnetometry

IOT devices

Heading

Gaming

IST8310 Datasheet, Version 1.2                                                                                                                              4

IST8310

2 Block Diagram, Package Dimension and Application Circuit

2.1 Block diagram

AVDD

VSS

CAD0 CAD1

Regulator

SDA

3-Axis

Low

SCL

Digital

Control

MUX

Noise

ADC

AMR

DRDY

AMP

Logic

Sensing

Elements

RSTN

Set/Reset

Clock

Trimming

Controller

Generator

Logic

Temperature

OTP

VPP

Compensation

Circuit

C1

DVDD

Figure 1. Block Diagram

2.2 Package Dimensions and Pin Description

IST8310 LGA Top View (Looking Through)

0.325

0.625

3.0

0.625

0.25

3.0

0.5

0.5

Unit:  mm

Tolerance:

0.1mm

±

IST8310 Datasheet, Version 1.2                                                                                                                              5

IST8310

IST8310 LGA Side View

1.0

IST8310 3D Top View

Z+

X+

Y+

Unit:  mm

Tolerance:

0.1mm

±

Pin

Name

Function

2

1

SCL

I

C serial clock

2

AVDD

Analog supply voltage, 1.72~3.6V

3

NC

Not use

4

NC

Not use

2

5

CAD0

I

C slave address

2

6

CAD1

I

C slave address

7

VPP

Test pin, floating connection is suggested

8

NC

Not use

9

VSS

GND

10

C1

Set/Reset function, 4.7uF

11

VSS

GND

12

NC

Not use

13

DVDD

Digital supply voltage, 1.72~3.6V

14

RSTN

Reset  pin,  resets  registers  by  setting  it  to  “Low”.

Internally pulled to “High” for floating connection.

MCU connection is suggested (but not necessary).

15

DRDY

Data ready indication, output pin only

2

16

SDA

I

C serial data

*please refer to Figure 2.

IST8310 Datasheet, Version 1.2                                                                                                                              6

IST8310

2.3 Application Circuit

DRDY

RSTN

DVDD

0.1uF

4.7K

4.7K

SDA

DRDY

RSTN

DVDD

SDA

SCL

16

15

14

13

AVDD

NC

1

SCL

12

GND

AVDD

2

11

0.1uF

(Top View)

3

4.7uF

C1

10

NC

GND

4

9

NC

5

6

7

8

CAD0

CAD1

VPP

NC

Slave Address Select

CAD1     CAD0      Address (7-bit)   Address (8-bit)

VSS        VSS

0CH

18H

VSS        VDD

0DH                    1AH

VDD       VSS

0EH                     1CH

VDD       VDD

0FH                     1EH

2

*if CAD1 and CAD0 are floating, I

C address will be 0EH/1CH.

Figure 2. Application Circuit

IST8310 Datasheet, Version 1.2                                                                                                                              7

IST8310

3 Operational Modes and Functional Descriptions

3.1 Operation modes

IST8310 has following operation modes:

(1) Stand-By Mode

(2) Single Measurement Mode

(3) Self-Test Mode

3.1.1 Stand-By Mode

The  initial  mode  (after  power  on)  of  IST8310  is  Stand-By  Mode.  In  Stand-By  Mode,  all  internal

circuits are off (except oscillator and regulator). All registers can be accessed in Stand-By Mode. Data

stored in Read/Write registers remains as last state. Registers can be reset by soft reset or hard reset

(through RSTN pin).

As initial setting, please set Pulse Duration Control Register, PDCTNL(0x42) = 11000000b (C0H) for

performance

optimization.

For

low

noise

performance,

please

set

Average

Control

Register,

AVGCNTL(0x41) = 00100100b (24H) for more internal average times. The minimum waiting time

between two measurements under low noise performance setup is 6ms (166Hz).

3.1.2 Single Measurement Mode

In Single Measurement Mode, the measured data is stored in data registers then IST8310 transits to

Stand-By Mode automatically. On transition to Stand-By Mode, Control register 1(CNTL1[3:0]) turns

to “0000”. At the same time, DRDY bit in STAT1 register turns to “1”. This is called “data ready”.

When any of the measurement data registers or STAT2 register is read, DRDY bit turns to “0”. For the

next measurement, user needs to set Control register 1(CNTL1[3:0]) to “0001” again. The minimum

waiting time between two measurements in default setup is 5ms (200Hz).

Please  noted  in  IST8310,  ultra-low  noise  performance  is  obtained  through  soft-averaging  in

driver. Please contact iSentek for technical details.

3.1.3 Self-Test Mode

Self-Test mode is used to check if the 3-axis outputs read in Single Measurement Mode are correct. It

is  activated  by  setting  Self-Test  Register  (0x0C)  to  40h;  then  all  3-axis  outputs  will  change  their

polarity.  User  can check  the  3-axis  output  values  before and  after  activating  Self-Test Mode;  if  the

absolute values are the same, then the IC is working correctly. It can be turned off by setting Self-Test

Register (0x0C) to 00h.

IST8310 Datasheet, Version 1.2                                                                                                                              8

IST8310

3.2 Interrupt Function

Interrupt function is used when there is a huge external magnetic field in the surrounding. When the

absolute sum of measured 3-axis output value exceeds 1600 uT, the INT flag is activated. The INT flag

can be found in STAT2 register.

3.3 DRDY Function

DRDY function is used when the output data is updated. The DRDY pin is used to monitor the data

ready output. DRDY is changed to low after reading data from the output register.

3.4 IST8310 Read Process

(1) Read STAT1 register:

‐

Polling STAT1 register bit 0

‐

DRDY: shows if the data is ready or not

0: no data ready

1: data ready

‐

DOR: shows if any data has been skipped before the current data

0: no skipped data

1: data skipped.

(2) Read Measurement Data:

Read Register 0x03h~0x08h for X, Y and Z axis data. When data reading starts, DRDY bit and

DOR bit turn to “0”.

IST8310 Datasheet, Version 1.2                                                                                                                              9

IST8310

4 Electrical Specifications

4.1 Absolute Maximum Ratings

Parameter

Symbol

Limits

Unit

o

Storage Temperature

TSTG

-40 to +125

C

o

Operating Temperature

TA

-40 to +85

C

Analog Supply Voltage

AVDD

-0.5 to +3.6

V

Digital Supply Voltage

DVDD

-0.5 to +3.6

V

Digital Input Voltage

VIN

-0.3 to DVDD+0.3

V

1

Electrostatic Discharge Voltage*

VESD_HBM

-4000 to 4000

V

2

Electrostatic Discharge Voltage*

VESD_MM

-350 to 350

V

o

Reflow Classification

JESD22-A113 with 260

C Peak Temperature

1. Human Body Model (HBM)

2. Machine Model (MM)

4.2 Recommended Operating Conditions

Parameter

Symbol

Min.

Typ.

Max.

Unit

o

Operating Temperature

TA

-40

+85

C

Analog Supply Voltage

AVDD

1.72

2.8

3.6

V

Digital Supply Voltage

DVDD

1.72

1.8

3.6

V

4.3 Electrical Specifications

℃

(Operating conditions: TA=+25

; AVDD=2.8V; DVDD=1.8V; 0.1µF ceramic capacitors tied closely

to AVDD/DVDD and GND respectively.)

Parameter

Symbol

Conditions

Min.

Typ.

Max

Unit

Operating Current

IDD3A

Full operation, at

uA

1 sps

20

8 sps

72

10 sps

80

20 sps

140

50 sps

320

100 sps

600

200 sps

1200

Standby Current

ISTB

10

uA

IST8310 Datasheet, Version 1.2                                                                                                                              10

IST8310

Output Data Rate

ODR

1

200

Hz

(ODR)

Input Low Voltage

VIL

0

DVDD

V

*30%

Input High Voltage

VIH

DVDD

DVDD

V

*70%

Output Low Voltage

VOL

IOL= +4 mA

0

DVDD

V

*20%

Output High Voltage

VOH

IOH= -100 uA

DVDD

DVDD

V

(Except SCL and

*80%

SDA)

4.4 Magnetic Sensor Specifications

℃

(Operating conditions: Ta=+25

; AVDD=2.8V; DVDD=1.8V; 4.7µF ceramic capacitors tied closely

to C1 and GND respectively. )

Parameter

Symbol

Condition

Min.

Typ.

Max

Unit

o

MDR_XY

TA=25

C

±1600

Dynamic Range

uT

o

MDR_Z

TA=25

C

±2500

X-axis

1

1.5

Linearity

LIN

%FS

Y, Z-axis

0.1

0.5

Resolution

RESO

0.3

uT/LSB

Sensitivity

SEN

3.3

LSB/uT

Zero Gauss Offset

ZGD

RMS value

±0.3

uT

Hysteresis

HS

0.1

%FS

Sensitivity

o

o

TD_S

-40 ~ 85

C

±0.016

%/

K

Temperature Drift

Zero-B Offset

o

o

TD_O

-40 ~ 85

C

0.024

uT/

K

Temperature Drift

IST8310 Datasheet, Version 1.2                                                                                                                              11

IST8310

4.5 Power On Reset (POR) Specifications

AVDD

PSTO

0v

PSOI

POR

POR

PSTO: max=0.1volt

PSTO: Power Supply Turn Off voltage

PSOI: min=10ms

PSOI: Power Supply Turn Off Interval

POR: max:50ms

POR: Power On Reset

When POR circuit detects the rise of AVDD voltage, it resets all internal circuits and initializes all

registers. After reset, IST8310 transits to Stand-By mode.

IST8310 Datasheet, Version 1.2                                                                                                                              12

IST8310

5 Technology Overview

5.1 AMR Technology

IST8310,  an  iSentek  patented  magnetometer  is  designed  based  on  Anisotropy  Magneto-Resistance

(AMR) technology. The output is generated from the resistance change of the AMR resistors while

external magnetic field changes. The sensitivity is about 50 to 200 times larger than traditional Hall

element.  The  high  sensitivity  allows  higher  output  data  rate  (ODR),  lower  noise  and  lower  power

consumption.

5.2 High Reliability Planarized Structure Design

IST8310  consists  of  three  full  Whetstone  Bridge  of  AMR  resistors.  The  three  bridges  detecting

magnetic component in three directions orthogonal to each other are located on one chip, wire-bonded

to  a  control  ASIC.  This  planarized  structure  design  enables  outstanding  stability  to  thermal  shock,

making our device highly reliable, immune from thermal reflow-induced failure. While other known

AMR

magnetometers

placing

z-axis

sensor

vertical

to

the

substrate

using

90-degree

flip-chip

packaging suffers from reliability issues

5.3 Ultra-low Hysteresis Design

iSentek has developed a specialized high permeability (μ) material for magnetic field detection. This

high-μ material has ultra-low residual magnetization below 0.1 %FS in the field range as large as +/-

500

G.

The

ultra-low

hysteresis

design

prevents

the

magnetometer

from

dynamic

offset

after

encountering

a

strong

external

magnetic

field

impact;

that

is,

the

angular

accuracy

restores

automatically  without  calibration  after  the  removal  of  interference  field.  This  feature  fulfills  the

requirements for applications when real time calibration is not available. No calibration is required in

general conditions.

5.4 Magnetic Setting Mechanism

AMR sensing resistors consist of Permalloy thin film and metallization. Permalloy is soft magnetic,

irreversible magnetic rotation may occur after the strength of external magnetic field exceeds half of

the anisotropy field of the sensing resistor, resulting in angular error induced by offset. To solve this

issue, a magnetic setting mechanism is introduced in IST8310. A magnetic field is generated within

IST8310  to  align  the  magnetization  of  AMR  sensing  resistors  before  every  measurement.  This

auto-zeroing mechanism ensures the stability of angular accuracy of IST8310 during whole operation.

IST8310 Datasheet, Version 1.2                                                                                                                              13

IST8310

6 Digital Interface and Registers

2

6.1 I

C Interface

2

The interface of IST8310 follows the standard I

C definition guidelines with some additional protocol

definitions. IST8310 supports standard speed (100kHz) and fast speed (400kHz). Pull-up resistors of

4.7kohm for both SDA and SCL lines should be used.

2

Figure 3. I

C Operation

6.1.1 Slave Address

MSB                                                                                                                            LSB

Bit7

Bit6

Bit5

Bit4

Bit3

Bit2

Bit1

Bit0

0

R/W

0

0

1

1

selectable

selectable

2

IST8310 has 4 different I

C slave addresses, which can be chosen through Pin 5 (CAD0) and Pin 6

2

(CAD1), the corresponding 7-bit and 8-bit defined I

C slave addresses are listed below:

2

2

I

C Slave Address

I

C Slave Address

CAD1

CAD0

(7-bit)

(8-bit)

VSS

VSS

0CH

18H

VSS

VDD

0DH

1AH

VDD

VSS

0EH

1CH

VDD

VDD

0FH

1EH

Floating (default)

Floating (default)

0EH

1CH

2

6.2 I

C Read Operation

Single Byte Read:

SA

Slave

ACK

Reg

ACK

SP

Slave Address+RW

ACK

DATA

NA

ST

Address+ RW

Address

2

Figure 4. I

C Single Byte Read Operation

IST8310 Datasheet, Version 1.2                                                                                                                              14

IST8310

ACK: Acknowledge, NA: Not Acknowledge, SA: START Condition, SP: Repeat Start Condition, ST: STOP Condition

■: Slave to Master

: Master to Slave

Multiple Byte Read:

SA

Slave

ACK

Reg

ACK

SP

Slave

ACK

DATA

ACK

DATA

NA

ST

Address+

Address

Address+RW

RW

2

Figure 5. I

C Multiple Byte Read Operation

ACK: Acknowledge, NA: Not Acknowledge, SA: START Condition, SP: Repeat Start Condition, ST: STOP Condition

■: Slave to Master

: Master to Slave

2

6.3 I

C Write Operation

Single Byte Write:

SA

Slave Address +RW

ACK

Reg Address

ACK

DATA

ST

ACK

2

Figure 6. I

C Single Byte Write Operation

ACK: Acknowledge, NA: Not Acknowledge, SA: START Condition, SP: Repeat Start Condition, ST: STOP Condition

■: Slave to Master

: Master to Slave

Multiple Byte Write:

SA

Slave Address

ACK

Reg Adderss

ACK

DATA

ACK

DATA

ST

+RW

NA

2

Figure 7. I

C Multiple Byte Write Operation

ACK: Acknowledge, NA: Not Acknowledge, SA: START Condition, SP: Repeat Start Condition, ST: STOP Condition

■: Slave to Master

: Master to Slave

6.4 Registers

6.4.1 Customer Defined Registers

2

Register name

C

Size

Info

Typ

I

Addr

(bit)

Who am I

R

00h

8

Device ID

Status Register 1

R

02h

8

Data Status

Output Value X_L

R

03h

8

Low byte for X-axis data

Output Value X_H

R

04h

8

High byte for X-axis data

Output Value Y_L

R

05h

8

Low byte for Y-axis data

Output Value Y_H

R

06h

8

High byte for Y-axis data

Output Value Z_L

R

07h

8

Low byte for Z-axis data

IST8310 Datasheet, Version 1.2                                                                                                                              15

IST8310

Output Value Z_H

R

08h

8

High byte for Z-axis data

Status Register 2

R

09h

8

Data Status

Control Register 1

R/W

0Ah

8

Chip Control setting 1

Control Register 2

R/W

0Bh

8

Chip Control setting 2

Output Value T_L

R

1Ch

8

Low byte for Temperature data

Output Value T_H

R

1Dh

8

High byte for Temperature data

6.4.2 Who Am I Register

This Register provides device ID information

WAI (0x00)

Bit

Description

Attr

Default

7:0

Device ID

R

10

6.4.3 Status Register 1

This Register provides status information of IST8310

STAT1 (0x02)

Bit

Description

Attr

Default

7:2

Reserved

1

DOR: Turns to 1 when data has been skipped. Bit is released

R

0

after any output data register read

0: no data overrun

1: data overrun

0

DRDY: Data ready pin

R

0

0: data not ready

1: data ready

This status bit follows physical signal appearance, expect the

polarity control (refer to DRP bit in CNTL2 register)

If data ready function enable bit is not set (refer to DREN bit in

CNTL2 register), this bit is zero

IST8310 Datasheet, Version 1.2                                                                                                                              16

IST8310

6.4.4 Output Data Registers

The Output Registers (from 0x03h~0x08h) contain X, Y and Z axis measurement data. Measurement

data are stored in 2’s complement format.

DATAXL(0x03)

Bit

Description

Attr

Default

7:0

Low Byte of X-axis data

R

0

DATAXH(0x04)

Bit

Description

Attr

Default

7:0

High Byte of X-axis data

R

0

DATAYL(0x05)

Bit

Description

Attr

Default

7:0

Low Byte of Y-axis data

R

0

DATAYH(0x06)

Bit

Description

Attr

Default

7:0

High Byte of Y-axis data

R

0

DATAZL(0x07)

Bit

Description

Attr

Default

7:0

Low Byte of Z-axis data

R

0

DATAZH(0x08)

Bit

Description

Attr

Default

7:0

High Byte of Z-axis data

R

0

6.4.5 Status Register 2

In this register, there is an INT flag for customer use.

STAT2(0x09)

Bit

Description

Attr

Default

IST8310 Datasheet, Version 1.2                                                                                                                              17

IST8310

7:4

Reserved

3

INT : Interrupt bit. When interrupt event occurs, this bit will be

R

0

set to 1

2:0

Reserved

6.4.6 Control Setting Register 1

This Register controls and adjusts the main parameters.

CNTL1(0x0A)

Bit

Description

Attr

Default

7:4

Reserved

3:0

Mode: Operating mode setting

R/W

0

0000: Stand-By mode

0001: Single Measurement Mode

Others: Reserved

6.4.7 Control Setting Register 2

This Register controls and adjusts the main parameters.

CNTL2(0x0B)

Bit

Description

Attr

Default

7:4

Reserved

3

DREN : Data ready enable control:

R/W

1

0: disable

1: enable

Master switch for DRDY output pin

2

DRP: DRDY pin polarity control

R/W

1

0: active low

1: active high

1

Reserved

0

SRST: Soft reset, perform Power On Reset (POR) routine

R/W

0

0: no action

1: start immediately POR routine

This bit will be set to zero after POR routine

IST8310 Datasheet, Version 1.2                                                                                                                              18

IST8310

6.4.8 Self-Test Register

STR(0x0C)

Bit

Description

Attr

Default

7

Reserved

6

SELF_TEST : When this bit set to 1, chip enters self-test mode

R/W

0

5:0

Reserved

6.4.9 Temperature Sensor Output Registers

The Output Data Registers use 2’s complement format.

TEMPL(0x1C)

Bit

Description

Attr

Default

7:0

Low Byte of Temperature data

R

0

TEMPH(0x1D)

Bit

Description

Attr

Default

7:0

High Byte of Temperature data

R

0

6.4.10 Average Control Register

This register controls the times of average done in the circuit to lower the noise. Higher average times

leads to lower noise.

AVGCNTL(0x41)

Bit

Description

Attr

Default

7:6

Reserved.

R/W

0

5:3

Average times for y sensor data. Times of average will be done

R/W

0

before switch to next channel

3’b000      no average

3’b001      average by 2 times

3’b010      average by 4 times (Default)

3’b011      average by 8 times

3’b100      average by 16 times

IST8310 Datasheet, Version 1.2                                                                                                                              19

IST8310

Others: no average

2:0

Average times for x & z sensor data. Times of average will be

R/W

0

done before switch to next channel

3’b000      no average

3’b001      average by 2 times

3’b010      average by 4 times (Default)

3’b011      average by 8 times

3’b100      average by 16 times

Others: no average

6.4.11 Pulse Duration Control Register

This register controls the pulse duration for set/reset function of AMR sensors.

PDCNTL(0x42)

Bit

Description

Attr

Default

7:6

Pulse duration:

R/W

0

2’b01      Long

2’b11      Normal (please use this setting)

Others: only for extreme cases

5:0

Reserved.

R/W

0

IST8310 Datasheet, Version 1.2                                                                                                                              20

IST8310

7 Ordering Information

Order Number

Package Type

Packaging

Marking Information

IST8310

LGA – 16 pin

Tape and Reel: 5k

X

X

X

0

1

2

3

pieces per reel

010●

X

: Last number of the year

1

X

X

: Week number

2

3

010: Product code of IST8310

For

more

information

on

iSentek’s

Magnetic

Sensors,

please

contact

us

by

phone

at

+86-132-6706-8686 (China), +86-755-3337-0168 (China) or +886-2-2698-3306 ext:110 (Taiwan); via

e-mail: sales@isentek.com or visit us online at www.isentek.com.

The application circuits herein constitute typical usage and interface of iSentek’s product. iSentek does

not  warranty  or  assume  liability  of  customer-designed  circuits  derived  from  this  description  or

depiction.

iSentek reserves the right to make changes to improve reliability, function or design. iSentek does not

assume any liability arising out of the application or use of any product or circuit described herein;

neither does it convey any license under its patent rights nor the rights of others.

US Patent 9,297,863, Taiwanese Patents I437249, I420128 and I463160 apply to our magnetic sensor

technology described.

IST8310 Datasheet, Version 1.2                                                                                                                              21