STM32F427xx STM32F429xx

®

®

32b Arm

Cortex

-M4 MCU+FPU, 225DMIPS, up to 2MB flash/256+4KB RAM, USB

OTG HS/FS, Ethernet, 17 TIMs, 3 ADCs, 20 com. interfaces, camera and LCD-TFT

-

Datasheet

production data

Features

•

Includes ST state-of-the-art patented

technology.

®

®

•

Core: Arm

32-bit Cortex

-M4 CPU with FPU,

Adaptive real-time accelerator (ART

LQFP100 (14 × 14 mm)

Accelerator™) allowing 0-wait state execution

UFBGA176 (10 x 10 mm)

WLCSP143

LQFP144 (20 × 20 mm)

UFBGA169 (7 × 7 mm)

from flash memory, frequency up to 180 MHz,

LQFP176 (24 × 24 mm)

TFBGA216 (13 x 13 mm)

MPU, 225 DMIPS/1.25 DMIPS/MHz

LQFP208 (28 x 28 mm)

(Dhrystone 2.1), and DSP instructions

bit timers up to 180 MHz, each with up to

•

Memories

four IC/OC/PWM or pulse counter and

–

512 bytes of OTP memory

quadrature (incremental) encoder input

–

Up to 2 MB of flash memory organized into

•

Debug mode

two banks allowing read-while-write

–

SWD & JTAG interfaces

–

Up to 256+4 KB of SRAM including 64 KB

–

Cortex®-M4 Trace Macrocell™

of CCM (core coupled memory) data RAM

•

–

Flexible external memory controller with up

Up to 168 I/O ports with interrupt capability

to 32-bit data bus: SRAM, PSRAM,

–

Up to 164 fast I/Os up to 90 MHz

SDRAM/LPSDR SDRAM, compact

–

Up to 166 5 V-tolerant I/Os

flash/NOR/NAND memories

•

Up to 21 communication interfaces

•

LCD parallel interface, 8080/6800 modes

2

–

Up to 3 × I

C interfaces (SMBus/PMBus)

•

LCD-TFT controller with fully programmable

–

Up to four USARTs/4 UARTs (11.25 Mbit/s,

resolution (total width up to 4096 pixels, total

ISO7816 interface, LIN, IrDA, modem

height up to 2048 lines and pixel clock up to

control)

83 MHz)

–

Up to 6 SPIs (45 Mbit/s), 2 with muxed full-

2

•

S for audio class accuracy via

duplex I

Chrom-ART Accelerator™ for enhanced

internal audio PLL or external clock

graphic content creation (DMA2D)

–

1 x SAI (serial audio interface)

•

Clock, reset, and supply management

–

2 × CAN (2.0B active) and SDIO interface

–

1.7 V to 3.6 V application supply and I/Os

•

Advanced connectivity

–

POR, PDR, PVD, and BOR

–

USB 2.0 full-speed device/host/OTG

–

4-to-26 MHz crystal oscillator

controller with on-chip PHY

–

Internal 16 MHz factory-trimmed RC (1%

–

USB 2.0 high-speed/full-speed

accuracy)

device/host/OTG controller with dedicated

–

32 kHz oscillator for RTC with calibration

DMA, on-chip full-speed PHY and ULPI

–

Internal 32 kHz RC with calibration

–

10/100 Ethernet MAC with dedicated DMA:

•

Low power

supports IEEE 1588v2 hardware, MII/RMII

•

8- to 14-bit parallel camera interface up to

–

Sleep, Stop, and Standby modes

54 Mbytes/s

supply for RTC, 20×32-bit backup

–

V

BAT

registers + optional 4 KB backup SRAM

•

True random number generator

•

3×12-bit, 2.4 MSPS ADC: up to 24 channels

•

CRC calculation unit

and 7.2 MSPS in triple interleaved mode

•

RTC: subsecond accuracy, hardware calendar

•

2×12-bit D/A converters

•

96-bit unique ID.

•

General-purpose DMA: 16-stream DMA

•

ECOPACK2 compliant packages.

controller with FIFOs and burst support

•

Up to 17 timers: up to twelve 16-bit and two 32-

February 2026

DS9405 Rev 13

1/240

This is information on a product in full production.

www.st.com

STM32F427xx STM32F429xx

•

Table 1. Device summary

Reference

Part number

STM32F427VG, STM32F427ZG, STM32F427IG, STM32F427AG, STM32F427VI, STM32F427ZI,

STM32F427xx

STM32F427II, STM32F427AI

STM32F429VG, STM32F429ZG, STM32F429IG, STM32F429BG, STM32F429NG,

STM32F429xx

STM32F429AG, STM32F429VI, STM32F429ZI, STM32F429II,, STM32F429BI, STM32F429NI,

STM32F429AI, STM32F429VE, STM32F429ZE, STM32F429IE, STM32F429BE, STM32F429NE

2/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Contents

Contents

1

Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

2

Description . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

2.1

Full compatibility throughout the family   . . . . . . . . . . . . . . . . . . . . . . . . . .  18

3

Functional overview  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

®

3.1

Arm

Cortex®-M4 with FPU and embedded flash and SRAM . . . . . . . . .  21

3.2

Adaptive real-time memory accelerator (ART Accelerator™)   . . . . . . . . .  21

3.3

Memory protection unit . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  21

3.4

Embedded flash memory  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  22

3.5

CRC (cyclic redundancy check) calculation unit   . . . . . . . . . . . . . . . . . . .  22

3.6

Embedded SRAM . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  22

3.7

Multi-AHB bus matrix  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  22

3.8

DMA controller (DMA)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  23

3.9

Flexible memory controller (FMC)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  24

3.10

LCD-TFT controller (available only on STM32F429xx)  . . . . . . . . . . . . . .  24

3.11

Chrom-ART Accelerator™ (DMA2D)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  25

3.12

Nested vectored interrupt controller (NVIC)  . . . . . . . . . . . . . . . . . . . . . . .  25

3.13

External interrupt/event controller (EXTI) . . . . . . . . . . . . . . . . . . . . . . . . .  25

3.14

Clocks and startup  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  25

3.15

Boot modes   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  26

3.16

Power supply schemes   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  26

3.17

Power supply supervisor   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  26

3.17.1

Internal reset ON  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 26

3.17.2

Internal reset OFF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

3.18

Voltage regulator   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  28

3.18.1

Regulator ON . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 28

3.18.2

Regulator OFF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 29

3.18.3

Regulator ON/OFF and internal reset ON/OFF availability  . . . . . . . . . . 32

3.19

Real-time clock (RTC), backup SRAM, and backup registers  . . . . . . . . .  32

3.20

Low-power modes  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  33

3.21

V

operation  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  34

BAT

DS9405 Rev 13

3/240

6

Contents

STM32F427xx STM32F429xx

3.22

Timers and watchdogs  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  34

3.22.1

Advanced-control timers (TIM1, TIM8)   . . . . . . . . . . . . . . . . . . . . . . . . . 36

3.22.2

General-purpose timers (TIMx)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

3.22.3

Basic timers TIM6 and TIM7  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

3.22.4

Independent watchdog   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

3.22.5

Window watchdog  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

3.22.6

SysTick timer  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

2

3.23

Inter-integrated circuit interface ( I

C)   . . . . . . . . . . . . . . . . . . . . . . . . . . .  37

3.24

Universal synchronous/asynchronous receiver transmitters (USART)   . .  37

3.25

Serial peripheral interface (SPI)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  38

2

3.26

Inter-integrated sound (I

S)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  39

3.27

Serial Audio interface (SAI1)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  39

3.28

Audio PLL (PLLI2S)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  39

3.29

Audio and LCD PLL(PLLSAI) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  39

3.30

Secure digital input/output interface (SDIO) . . . . . . . . . . . . . . . . . . . . . . .  40

3.31

Ethernet MAC interface with dedicated DMA and IEEE 1588 support . . .  40

3.32

Controller area network (bxCAN)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  40

3.33

Universal serial bus on-the-go full-speed (OTG_FS)  . . . . . . . . . . . . . . . .  41

3.34

Universal serial bus on-the-go high-speed (OTG_HS) . . . . . . . . . . . . . . .  41

3.35

Digital camera interface (DCMI) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  42

3.36

True random number generator (RNG)  . . . . . . . . . . . . . . . . . . . . . . . . . .  42

3.37

General-purpose input/outputs (GPIOs) . . . . . . . . . . . . . . . . . . . . . . . . . .  42

3.38

Analog-to-digital converters (ADCs) . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  42

3.39

Temperature sensor  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  43

3.40

Digital-to-analog converter (DAC)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  43

3.41

Serial wire JTAG debug port (SWJ-DP)  . . . . . . . . . . . . . . . . . . . . . . . . . .  43

3.42

Embedded Trace Macrocell™  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  44

4

Pinouts and pin description . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 45

5

Memory mapping . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 86

6

Electrical characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

6.1

Parameter conditions  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  91

6.1.1

Minimum and maximum values  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

4/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Contents

6.1.2

Typical values   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

6.1.3

Typical curves   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

6.1.4

Loading capacitor   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

6.1.5

Pin input voltage   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

6.1.6

Power supply scheme  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 92

6.1.7

Current consumption measurement   . . . . . . . . . . . . . . . . . . . . . . . . . . . 93

6.2

Absolute maximum ratings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  93

6.3

Operating conditions   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  95

6.3.1

General operating conditions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 95

6.3.2

VCAP1/VCAP2 external capacitor . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 97

6.3.3

Operating conditions at power-up / power-down (regulator ON)  . . . . . . 98

6.3.4

Operating conditions at power-up / power-down (regulator OFF)  . . . . . 98

6.3.5

Reset and power control block characteristics   . . . . . . . . . . . . . . . . . . . 99

6.3.6

Overdrive switching characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . 100

6.3.7

Supply current characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 101

6.3.8

Wake-up time from low-power modes  . . . . . . . . . . . . . . . . . . . . . . . . . 117

6.3.9

External clock source characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . 118

6.3.10

Internal clock source characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . 122

6.3.11

PLL characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 124

6.3.12

PLL spread spectrum clock generation (SSCG) characteristics   . . . . . 127

6.3.13

Memory characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 129

6.3.14

EMC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 131

6.3.15

Absolute maximum ratings (electrical sensitivity)   . . . . . . . . . . . . . . . . 133

6.3.16

I/O current injection characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . 134

6.3.17

I/O port characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 135

6.3.18

NRST pin characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 141

6.3.19

TIM timer characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 142

6.3.20

Communications interfaces  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 142

6.3.21

12-bit ADC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 158

6.3.22

Temperature sensor characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . 164

6.3.23

V

monitoring characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

BAT

6.3.24

Reference voltage  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

6.3.25

DAC electrical characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 166

6.3.26

FMC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 169

6.3.27

Camera interface (DCMI) timing specifications  . . . . . . . . . . . . . . . . . . 193

6.3.28

LCD-TFT controller (LTDC) characteristics  . . . . . . . . . . . . . . . . . . . . . 194

6.3.29

SD/SDIO MMC card host interface (SDIO) characteristics  . . . . . . . . . 196

DS9405 Rev 13

5/240

6

Contents

STM32F427xx STM32F429xx

6.3.30

RTC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 197

7

Package information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 198

7.1

Device marking . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  198

7.2

LQFP100 package information (1L)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  199

7.3

WLCSP143 package information  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  202

7.3.1

Device marking for WLCSP143  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 204

7.4

LQFP144 package information (1A) . . . . . . . . . . . . . . . . . . . . . . . . . . . .  205

7.5

LQFP176 package information (1T) . . . . . . . . . . . . . . . . . . . . . . . . . . . .  209

7.6

LQFP208 package information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  213

7.7

UFBGA169 package information (A0YV) . . . . . . . . . . . . . . . . . . . . . . . .  216

7.8

UFBGA(176+25) package information (A0E7) . . . . . . . . . . . . . . . . . . . .  219

7.9

TFBGA216 package information (A0L2)   . . . . . . . . . . . . . . . . . . . . . . . .  221

7.10

Thermal characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  224

8

Ordering information   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 225

Appendix A

Recommendations when using internal reset OFF  . . . . . . . . . . . 226

A.1

Operating conditions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  226

Appendix B

Application block diagrams . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 227

B.1

USB OTG full speed (FS) interface solutions . . . . . . . . . . . . . . . . . . . . .  227

B.2

USB OTG high speed (HS) interface solutions . . . . . . . . . . . . . . . . . . . .  229

B.3

Ethernet interface solutions. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  230

9

Important security notice  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 232

10

Revision history   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 233

6/240

DS9405 Rev 13

STM32F427xx STM32F429xx

List of tables

List of tables

Table 1.

Device summary . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Table 2.

STM32F427xx and STM32F429xx features and peripheral counts . . . . . . . . . . . . . . . . . . 16

Table 3.

Voltage regulator configuration mode versus device operating mode . . . . . . . . . . . . . . . . 29

Table 4.

Regulator ON/OFF and internal reset ON/OFF availability. . . . . . . . . . . . . . . . . . . . . . . . . 32

Table 5.

Voltage regulator modes in stop mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 33

Table 6.

Timer feature comparison. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

Table 7.

Comparison of I2C analog and digital filters . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

Table 8.

USART feature comparison . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

Table 9.

Legend/abbreviations used in the pinout table . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 53

Table 10.

STM32F427xx and STM32F429xx pin and ball definitions  . . . . . . . . . . . . . . . . . . . . . . . . 53

Table 11.

FMC pin definition. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 72

Table 12.

STM32F427xx and STM32F429xx alternate function mapping . . . . . . . . . . . . . . . . . . . . . 75

Table 13.

STM32F427xx and STM32F429xx register boundary addresses. . . . . . . . . . . . . . . . . . . . 87

Table 14.

Voltage characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 93

Table 15.

Current characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 94

Table 16.

Thermal characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 94

Table 17.

General operating conditions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 95

Table 18.

Limitations depending on the operating power supply range . . . . . . . . . . . . . . . . . . . . . . . 97

Table 19.

VCAP1/VCAP2 operating conditions  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 97

Table 20.

Operating conditions at power-up / power-down (regulator ON)  . . . . . . . . . . . . . . . . . . . . 98

Table 21.

Operating conditions at power-up / power-down (regulator OFF). . . . . . . . . . . . . . . . . . . . 98

Table 22.

Reset and power control block characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 99

Table 23.

Over-drive switching characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 100

Table 24.

Typical and maximum current consumption in Run mode, code with data processing

running from Flash memory (ART accelerator enabled except prefetch) or RAM. . . . . . 102

Table 25.

Typical and maximum current consumption in Run mode, code with data processing

running from Flash memory (ART accelerator disabled) . . . . . . . . . . . . . . . . . . . . . . . . . 103

Table 26.

Typical and maximum current consumption in Sleep mode . . . . . . . . . . . . . . . . . . . . . . . 104

Table 27.

Typical and maximum current consumptions in Stop mode . . . . . . . . . . . . . . . . . . . . . . . 105

Table 28.

Typical and maximum current consumptions in Standby mode . . . . . . . . . . . . . . . . . . . . 106

Table 29.

Typical and maximum current consumptions in V

mode. . . . . . . . . . . . . . . . . . . . . . . 106

BAT

Table 30.

Typical current consumption in Run mode, code with data processing running from

Flash memory or RAM, regulator ON (ART accelerator enabled except prefetch),

VDD=1.7 V . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 108

Table 31.

Typical current consumption in Run mode, code with data processing running

from Flash memory, regulator OFF (ART accelerator enabled except prefetch). . . . . . . 109

Table 32.

Typical current consumption in Sleep mode, regulator ON, VDD=1.7 V  . . . . . . . . . . . . . 110

Table 33.

Tyical current consumption in Sleep mode, regulator OFF. . . . . . . . . . . . . . . . . . . . . . . . 111

Table 34.

Switching output I/O current consumption  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 113

Table 35.

Peripheral current consumption . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 114

Table 36.

Low-power mode wakeup timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 117

Table 37.

High-speed external user clock characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 118

Table 38.

Low-speed external user clock characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 119

Table 39.

HSE 4-26 MHz oscillator characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

Table 40.

LSE oscillator characteristics (f

= 32.768 kHz) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 121

LSE

Table 41.

HSI oscillator characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 122

Table 42.

LSI oscillator characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 123

Table 43.

Main PLL characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 124

DS9405 Rev 13

7/240

9

List of tables

STM32F427xx STM32F429xx

Table 44.

PLLI2S (audio PLL) characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 125

Table 45.

PLLISAI (audio and LCD-TFT PLL) characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 126

Table 46.

SSCG parameters constraint . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 127

Table 47.

Flash memory characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 129

Table 48.

Flash memory programming. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 129

Table 49.

Flash memory programming with V

PP  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 130

Table 50.

Flash memory endurance and data retention . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 131

Table 51.

EMS characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 131

Table 52.

EMI characteristics for fHSE= 25 MHz and fCPU= 168 MHz . . . . . . . . . . . . . . . . . . . . . . 132

Table 53.

EMI characteristics for HSE= 25 MHz and fCPU= 180 MHz  . . . . . . . . . . . . . . . . . . . . . . 133

Table 54.

ESD absolute maximum ratings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 133

Table 55.

Electrical sensitivities . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 134

Table 56.

I/O current injection susceptibility . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 134

Table 57.

I/O static characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 135

Table 58.

Output voltage characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 138

Table 59.

I/O AC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

Table 60.

NRST pin characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 141

Table 61.

TIMx characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 142

Table 62.

I2C analog filter characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143

Table 63.

SPI dynamic characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143

2

Table 64.

I

S dynamic characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 146

Table 65.

SAI characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 149

Table 66.

USB OTG full speed startup time . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 151

Table 67.

USB OTG full speed DC electrical characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 151

Table 68.

USB OTG full speed electrical characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 152

Table 69.

USB HS DC electrical characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 152

Table 70.

USB HS clock timing parameters . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 153

Table 71.

Dynamic characteristics: USB ULPI . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 154

Table 72.

Dynamics characteristics: Ethernet MAC signals for SMI. . . . . . . . . . . . . . . . . . . . . . . . . 155

Table 73.

Dynamics characteristics: Ethernet MAC signals for RMII . . . . . . . . . . . . . . . . . . . . . . . . 156

Table 74.

Dynamics characteristics: Ethernet MAC signals for MII  . . . . . . . . . . . . . . . . . . . . . . . . . 157

Table 75.

ADC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 158

Table 76.

ADC static accuracy at f

= 18 MHz. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 159

ADC

Table 77.

ADC static accuracy at f

= 30 MHz. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 160

ADC

Table 78.

ADC static accuracy at f

= 36 MHz. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 160

ADC

Table 79.

ADC dynamic accuracy at f

= 18 MHz - limited test conditions  . . . . . . . . . . . . . . . . . 160

ADC

Table 80.

ADC dynamic accuracy at f

= 36 MHz - limited test conditions  . . . . . . . . . . . . . . . . . 160

ADC

Table 81.

Temperature sensor characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 164

Table 82.

Temperature sensor calibration values. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 164

Table 83.

V

monitoring characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

BAT

Table 84.

internal reference voltage  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

Table 85.

Internal reference voltage calibration values  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

Table 86.

DAC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 166

Table 87.

Asynchronous non-multiplexed SRAM/PSRAM/NOR -

read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 170

Table 88.

Asynchronous non-multiplexed SRAM/PSRAM/NOR read -

NWAIT timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 171

Table 89.

Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings . . . . . . . . . . . . . . . . . 172

Table 90.

Asynchronous non-multiplexed SRAM/PSRAM/NOR write -

NWAIT timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 173

Table 91.

Asynchronous multiplexed PSRAM/NOR read timings. . . . . . . . . . . . . . . . . . . . . . . . . . . 174

Table 92.

Asynchronous multiplexed PSRAM/NOR read-NWAIT timings . . . . . . . . . . . . . . . . . . . . 174

8/240

DS9405 Rev 13

STM32F427xx STM32F429xx

List of tables

Table 93.

Asynchronous multiplexed PSRAM/NOR write timings  . . . . . . . . . . . . . . . . . . . . . . . . . . 175

Table 94.

Asynchronous multiplexed PSRAM/NOR write-NWAIT timings . . . . . . . . . . . . . . . . . . . . 176

Table 95.

Synchronous multiplexed NOR/PSRAM read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . 177

Table 96.

Synchronous multiplexed PSRAM write timings. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 179

Table 97.

Synchronous non-multiplexed NOR/PSRAM read timings . . . . . . . . . . . . . . . . . . . . . . . . 180

Table 98.

Synchronous non-multiplexed PSRAM write timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . 181

Table 99.

Switching characteristics for PC Card/CF read and write cycles

in attribute/common space. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 186

Table 100.

Switching characteristics for PC Card/CF read and write cycles

in I/O space . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 187

Table 101.

Switching characteristics for NAND Flash read cycles . . . . . . . . . . . . . . . . . . . . . . . . . . . 188

Table 102.

Switching characteristics for NAND Flash write cycles. . . . . . . . . . . . . . . . . . . . . . . . . . . 189

Table 103.

SDRAM read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 190

Table 104.

LPSDR SDRAM read timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 190

Table 105.

SDRAM write timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 192

Table 106.

LPSDR SDRAM write timings. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 192

Table 107.

DCMI characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 193

Table 108.

LTDC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 194

Table 109.

Dynamic characteristics: SD / MMC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 197

Table 110.

RTC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 197

Table 111.

LQFP100 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 200

Table 112.

WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package mechanical data  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 202

Table 113.

WLCSP143 recommended PCB design rules  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 204

Table 114.

LQFP144 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 206

Table 115.

LQFP176 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 210

Table 116.

LQFP208 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 214

Table 117.

UFBGA169 - Mechanical data  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 217

Table 118.

UFBGA169 - Example of PCB design rules (0.5 mm pitch BGA). . . . . . . . . . . . . . . . . . . 218

Table 119.

UFBGA(176+25) - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 219

Table 120.

UFBGA(176+25) - Example of PCB design rules (0.65 mm pitch BGA)  . . . . . . . . . . . . . 220

Table 121.

TFBGA216 - Mechanical data  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 222

Table 122.

TFBGA216 - Example of PCB design rules (0.8 mm pitch) . . . . . . . . . . . . . . . . . . . . . . . 223

Table 123.

Package thermal characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 224

Table 124.

Limitations depending on the operating power supply range . . . . . . . . . . . . . . . . . . . . . . 226

Table 125.

Document revision history  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 233

DS9405 Rev 13

9/240

9

List of figures

STM32F427xx STM32F429xx

List of figures

Figure 1.

Compatible board design STM32F10xx/STM32F2xx/STM32F4xx

for LQFP100 package. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

Figure 2.

Compatible board design between STM32F10xx/STM32F2xx/STM32F4xx

for LQFP144 package. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

Figure 3.

Compatible board design between STM32F2xx and STM32F4xx

for LQFP176 and UFBGA176 packages . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

Figure 4.

STM32F427xx and STM32F429xx block diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

Figure 5.

STM32F427xx and STM32F429xx Multi-AHB matrix . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

Figure 6.

Power supply supervisor interconnection with internal reset OFF . . . . . . . . . . . . . . . . . . . 27

Figure 7.

PDR_ON control with internal reset OFF . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 28

Figure 8.

Regulator OFF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 30

Figure 9.

Startup in regulator OFF: slow V

slope

DD

- power-down reset risen after V

/V

stabilization . . . . . . . . . . . . . . . . . . . . . . . . 31

CAP_1

CAP_2

Figure 10.

Startup in regulator OFF mode: fast V

slope

DD

- power-down reset risen before V

/V

stabilization  . . . . . . . . . . . . . . . . . . . . . . 31

CAP_1

CAP_2

Figure 11.

STM32F42x LQFP100 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 45

Figure 12.

STM32F42x WLCSP143 ballout. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

Figure 13.

STM32F42x LQFP144 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

Figure 14.

STM32F42x LQFP176 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

Figure 15.

STM32F42x LQFP208 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 49

Figure 16.

STM32F42x UFBGA169 ballout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 50

Figure 17.

STM32F42x UFBGA176 ballout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 51

Figure 18.

STM32F42x TFBGA216 ballout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 52

Figure 19.

Memory map. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 86

Figure 20.

Pin loading conditions. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

Figure 21.

Pin input voltage . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 91

Figure 22.

Power supply scheme  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 92

Figure 23.

Current consumption measurement scheme . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 93

Figure 24.

External capacitor C

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 97

EXT

Figure 25.

Typical V

current consumption (LSE and RTC ON/backup RAM OFF)  . . . . . . . . . . . 107

BAT

Figure 26.

Typical V

current consumption (LSE and RTC ON/backup RAM ON) . . . . . . . . . . . . 107

BAT

Figure 27.

High-speed external clock source AC timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . 119

Figure 28.

Low-speed external clock source AC timing diagram. . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

Figure 29.

Typical application with an 8 MHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 121

Figure 30.

Typical application with a 32.768 kHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 122

Figure 31.

ACCHSI accuracy versus temperature. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 123

Figure 32.

ACC

versus temperature . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 124

LSI

Figure 33.

PLL output clock waveforms in center spread mode  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 128

Figure 34.

PLL output clock waveforms in down spread mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 128

Figure 35.

FT I/O input characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 137

Figure 36.

I/O AC characteristics definition . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

Figure 37.

Recommended NRST pin protection  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 141

Figure 38.

SPI timing diagram - slave mode and CPHA = 0 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 145

Figure 39.

SPI timing diagram - slave mode and CPHA = 1 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 145

Figure 40.

SPI timing diagram - master mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 146

2

(1)

Figure 41.

I

S slave timing diagram (Philips protocol)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 148

2

(1)

Figure 42.

I

S master timing diagram (Philips protocol)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 148

Figure 43.

SAI master timing waveforms . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 150

10/240

DS9405 Rev 13

STM32F427xx STM32F429xx

List of figures

Figure 44.

SAI slave timing waveforms . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 150

Figure 45.

USB OTG full speed timings: definition of data signal rise and fall time. . . . . . . . . . . . . . 152

Figure 46.

ULPI timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 153

Figure 47.

Ethernet SMI timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 155

Figure 48.

Ethernet RMII timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 156

Figure 49.

Ethernet MII timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 157

Figure 50.

ADC accuracy characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 161

Figure 51.

Typical connection diagram when using the ADC with FT/TT pins

featuring the analog switch function . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 162

Figure 52.

Power supply and reference decoupling (V

not connected to V

). . . . . . . . . . . . . 163

REF+

DDA

Figure 53.

Power supply and reference decoupling (V

connected to V

). . . . . . . . . . . . . . . . 164

REF+

DDA

Figure 54.

12-bit buffered /non-buffered DAC . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 168

Figure 55.

Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms . . . . . . . . . . . . . . 170

Figure 56.

Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms . . . . . . . . . . . . . . 172

Figure 57.

Asynchronous multiplexed PSRAM/NOR read waveforms. . . . . . . . . . . . . . . . . . . . . . . . 173

Figure 58.

Asynchronous multiplexed PSRAM/NOR write waveforms  . . . . . . . . . . . . . . . . . . . . . . . 175

Figure 59.

Synchronous multiplexed NOR/PSRAM read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . 177

Figure 60.

Synchronous multiplexed PSRAM write timings. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 178

Figure 61.

Synchronous non-multiplexed NOR/PSRAM read timings . . . . . . . . . . . . . . . . . . . . . . . . 180

Figure 62.

Synchronous non-multiplexed PSRAM write timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . 181

Figure 63.

PC Card/CompactFlash controller waveforms for common memory read access . . . . . . 183

Figure 64.

PC Card/CompactFlash controller waveforms for common memory write access. . . . . . 183

Figure 65.

PC Card/CompactFlash controller waveforms for attribute memory

read access  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 184

Figure 66.

PC Card/CompactFlash controller waveforms for attribute memory

write access . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 185

Figure 67.

PC Card/CompactFlash controller waveforms for I/O space read access . . . . . . . . . . . . 185

Figure 68.

PC Card/CompactFlash controller waveforms for I/O space write access . . . . . . . . . . . . 186

Figure 69.

NAND controller waveforms for read access . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 188

Figure 70.

NAND controller waveforms for write access . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 188

Figure 71.

SDRAM read access waveforms (CL = 1)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 189

Figure 72.

SDRAM write access waveforms . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 191

Figure 73.

DCMI timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 193

Figure 74.

LCD-TFT horizontal timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 195

Figure 75.

LCD-TFT vertical timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 195

Figure 76.

SDIO high-speed mode  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 196

Figure 77.

SD default mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 196

(15)

Figure 78.

LQFP100 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 199

Figure 79.

LQFP100 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 201

Figure 80.

WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package outline . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 202

Figure 81.

WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package recommended footprint  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 203

Figure 82.

WLCSP143 marking example (package top view) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 204

(15)

Figure 83.

LQFP144 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 205

Figure 84.

LQFP144 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 208

(15)

Figure 85.

LQFP176 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 209

Figure 86.

LQFP176 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 212

(15)

Figure 87.

LQFP208 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 213

Figure 88.

LQFP208 - footprint example . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 215

Figure 89.

UFBGA169 - Outline. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 216

Figure 90.

UFBGA169 - Footprint example . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 218

DS9405 Rev 13

11/240

12

List of figures

STM32F427xx STM32F429xx

Figure 91.

UFBGA(176+25) - Outline  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 219

Figure 92.

UFBGA(176+25) - Footprint example. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 220

Figure 93.

TFBGA216 - Outline . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 221

Figure 94.

TFBGA216 - Footprint example . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 223

Figure 95.

USB controller configured as peripheral-only and used

in Full speed mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 227

Figure 96.

USB controller configured as host-only and used in full speed mode. . . . . . . . . . . . . . . . 227

Figure 97.

USB controller configured in dual mode and used in full speed mode . . . . . . . . . . . . . . . 228

Figure 98.

USB controller configured as peripheral, host, or dual-mode

and used in high speed mode. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 229

Figure 99.

MII mode using a 25 MHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 230

Figure 100.

RMII with a 50 MHz oscillator . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 230

Figure 101.

RMII with a 25 MHz crystal and PHY with PLL. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 231

12/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Introduction

1

Introduction

This datasheet provides the description of the STM32F427xx and STM32F429xx line of

microcontrollers. For more details on the whole STMicroelectronics STM32 family, refer to

Section 2.1: Full compatibility throughout the family.

The STM32F427xx and STM32F429xx datasheet should be read with the STM32F4xx

reference manual.

®

®

For information on the Cortex

-M4 core, refer to the Cortex

-M4 programming manual

(PM0214), available from www.st.com.

For information on the device errata with respect to the datasheet and reference manual,

refer to the STM32F427/437xx and STM32F429/439xx errata sheet (ES0206), available

from www.st.com.

Note:

Arm and Cortex are registered trademarks of Arm Limited (or its subsidiaries or affiliates) in

the US and/or elsewhere.

The Arm word and logo are trademarks of Arm Limited (or its subsidiaries) in the US and/or

elsewhere. All rights reserved.

DS9405 Rev 13

13/240

44

Description

STM32F427xx STM32F429xx

2

Description

®

The STM32F427xx and STM32F429xx devices are based on the high-performance Arm

®

®

Cortex

-M4 32-bit RISC core operating at a frequency of up to 180 MHz. The Cortex

-M4

®

core features a floating-point unit (FPU) single precision, which supports all Arm

single-

precision data-processing instructions and data types. It also implements a full set of DSP

instructions and a memory protection unit (MPU) which enhances application security.

The STM32F427xx and STM32F429xx devices incorporate high-speed embedded

memories (Flash memory up to 2 Mbyte, up to 256 Kbytes of SRAM), up to 4 Kbytes of

backup SRAM, and an extensive range of enhanced I/Os and peripherals connected to two

APB buses, two AHB buses and a 32-bit multi-AHB bus matrix.

All devices offer three 12-bit ADCs, two DACs, a low-power RTC, 12 general-purpose 16-bit

timers including two PWM timers for motor control, two general-purpose 32-bit timers. They

also feature standard and advanced communication interfaces.

2

•

Up to three I

Cs

2

2

•

Six SPIs, two I

Ss full duplex. To achieve audio class accuracy, the I

S peripherals can

be clocked via a dedicated internal audio PLL or via an external clock to allow

synchronization.

•

Four USARTs plus four UARTs

•

An USB OTG full-speed and a USB OTG high-speed with full-speed capability (with the

ULPI),

•

Two CANs

•

One SAI serial audio interface

•

An SDIO/MMC interface

•

Ethernet and camera interface

•

LCD-TFT display controller

•

Chrom-ART Accelerator™.

Advanced peripherals include an SDIO, a flexible memory control (FMC) interface, a

camera interface for CMOS image sensors. Refer to Table 2: STM32F427xx and

STM32F429xx features and peripheral counts for the list of peripherals available on each

part number.

The STM32F427xx and STM32F429xx devices operates in the –40 to +105 °C temperature

range from a 1.7 to 3.6 V power supply.

The supply voltage can drop to 1.7 V with the use of an external power supply supervisor

(refer to Section 3.17.2: Internal reset OFF). A comprehensive set of power-saving mode

allows the design of low-power applications.

The STM32F427xx and STM32F429xx devices offer devices in 8 packages ranging from

100 pins to 216 pins. The set of included peripherals changes with the device chosen.

14/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Description

These features make the STM32F427xx and STM32F429xx microcontrollers suitable for a

wide range of applications:

•

Motor drive and application control

•

Medical equipment

•

Industrial applications: PLC, inverters, circuit breakers

•

Printers, and scanners

•

Alarm systems, video intercom, and HVAC

•

Home audio appliances

Figure 4 shows the general block diagram of the device family.

DS9405 Rev 13

15/240

44

Description

16/240

Table 2. STM32F427xx and STM32F429xx features and peripheral counts

STM32F427

STM32F427

STM32F427

STM32F429

STM32F427

Peripherals

STM32F429Vx

STM32F429Zx

STM32F429Ix

STM32F429Bx

STM32F429Nx

Vx

Zx

Ax

Ax

Ix

Flash memory in Kbytes

1024

2048

512

1024

2048

1024

2048

512

1024

2048

1024

2048

1024

2048

1024

2048

512

1024

2048

512

1024

2048

512

1024

2048

System

256(112+16+64+64)

SRAM in

Kbytes

Backup

4

(1)

FMC memory controller

Yes

Ethernet

Yes

General-

10

purpose

Timers

Advanced

2

-control

Basic

2

Random number generator

Yes

DS9405 Rev 13

2

(2)

(2)

S

4/2 (full duplex)

6/2 (full duplex)

SPI / I

2

I

C

3

USART/

4/4

UART

USB OTG

Yes

Communication

FS

interfaces

USB OTG

Yes

HS

CAN

2

SAI

1

STM32F427xx STM32F429xx

SDIO

Yes

Camera interface

Yes

LCD-TFT (STM32F429xx

No

Yes

No

Yes

No

Yes

No

Yes

only)

Chrom-ART Accelerator™

Yes

GPIOs

82

114

130

140

168

3

12-bit ADC

Number of channels

16

24

Table 2. STM32F427xx and STM32F429xx features and peripheral counts (continued)

STM32F427xx STM32F429xx

STM32F427

STM32F427

STM32F427

STM32F429

STM32F427

Peripherals

STM32F429Vx

STM32F429Zx

STM32F429Ix

STM32F429Bx

STM32F429Nx

Vx

Zx

Ax

Ax

Ix

12-bit DAC

Yes

Number of channels

2

Maximum CPU frequency

180 MHz

(3)

Operating voltage

1.8 to 3.6 V

Ambient temperatures: –40 to +85 °C /–40 to +105 °C

Operating temperatures

Junction temperature: –40 to + 125 °C

WLCSP143

UFBGA176

Packages

LQFP100

UFBGA169

LQFP208

TFBGA216

LQFP144

LQFP176

1.

For the LQFP100 package, only FMC Bank1 or Bank2 are available. Bank1 can only support a multiplexed NOR/PSRAM memory using the NE1 Chip Select. Bank2 can only support a 16- or 8-bit

NAND Flash memory using the NCE2 Chip Select. The interrupt line cannot be used since Port G is not available in this package. For UFBGA169 package, only SDRAM, NAND and multiplexed

static memories are supported.

2.

The SPI2 and SPI3 interfaces give the flexibility to work in an exclusive way in either the SPI mode or the I2S audio mode.

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use of an external power supply supervisor (refer to Section 3.17.2: Internal reset

3.

V

DD

DDA

OFF

).

DS9405 Rev 13

Description

17/240

Description

STM32F427xx STM32F429xx

2.1

Full compatibility throughout the family

The STM32F427xx and STM32F429xx devices are part of the STM32F4 family. They are

fully pin-to-pin, software and feature compatible with the STM32F2xx devices, allowing the

user to try different memory densities, peripherals, and performances (FPU, higher

frequency) for a greater degree of freedom during the development cycle.

The STM32F427xx and STM32F429xx devices maintain a close compatibility with the

whole STM32F10xx family. All functional pins are pin-to-pin compatible. The STM32F427xx

and STM32F429xx, however, are not drop-in replacements for the STM32F10xx devices:

the two families do not have the same power scheme, and so their power pins are different.

Nonetheless, the transition from the STM32F10xx to the STM32F42x family remains simple

as only a few pins are impacted.

Figure 1, Figure 2, and Figure 3, give compatible board designs between the STM32F4xx,

STM32F2xx, and STM32F10xx families.

Figure 1. Compatible board design STM32F10xx/STM32F2xx/STM32F4xx

for LQFP100 package

V

SS

75

51

50

76

73

49

V

SS

V

SS

UHVLVWRURUVROGHULQJEULGJH

SUHVHQWIRUWKH670

32F10xxx

FRQILJXUDWLRQQRWSUHVHQWLQWKH

99 (VSS)

670)[[FRQILJXUDWLRQ

100

26

19

20

1

25

V

SS

V

SS

966IRU670)[[

7ZRUHVLVWRUVFRQQHFWHGWR

9''IRU670)[[

V

V

DD

SS

966IRUWKH670)[[

-

966IRUWKH670)[[

-

966RU1&IRUWKH670)[[

-

ai18488d

18/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Description

Figure 2. Compatible board design between STM32F10xx/STM32F2xx/STM32F4xx

for LQFP144 package

UHVLVWRURUVROGHULQJEULGJH

SUHVHQWIRUWKH670

32F10xx

V

FRQILJXUDWLRQQRWSUHVHQWLQWKH

SS

73

108

670)[[FRQILJXUDWLRQ

106

109

72

71

V

SS

1RWSRSXODWHGZKHQ

V

SS

UHVLVWRURUVROGHULQJ

EULGJHSUHVHQW

6LJQDOIURP

H[WHUQDOSRZHU

3'5B21

VXSSO\

144

VXSHUYLVRU

37

30

31

1

36

V

V

V

SS

DD

SS

1RWSRSXODWHGIRU670

32F10xx

7ZRUHVLVWRUVFRQQHFWHGWR

IRU670)[[

V

SS

IRUWKH670)[[

- V

V

V

SS

IRU670)[[

V

DD

SS

DD

RU1&IRUWKH670)[[

, V

- V

DD

SS

RUVLJQDOIURPH[WHUQDOSRZHUVXSSO\VXSHUYLVRUIRUWKH670)[[

- V

DD

ai18487d

Figure 3. Compatible board design between STM32F2xx and STM32F4xx

for LQFP176 and UFBGA176 packages

132

89

88

133

*1'IRU670)[[

48

%<3$66B5(*IRU670)[[

6LJQDOIURPH[WHUQDO

SRZHUVXSSO\

3'5B21

VXSHUYLV

U

o

176

45

1

44

V

V

DD

SS

7ZRUHVLVWRUVFRQQHFWHGWR

RU1&IRUWKH670)[[

, V

- V

DD

SS

RUVLJQDOIURPH[WHUQDOSRZHUVXSSO\VXSHUYLV

UIRUWKH670)[[

o

- V

DD

MS31835V1

DS9405 Rev 13

19/240

44

Description

STM32F427xx STM32F429xx

Figure 4. STM32F427xx and STM32F429xx block diagram

CLK, NE [3:0], A[23:0],

External memory controller (FMC)

CCM data RAM 64 KB

D[31:0], NOEN, NWEN,

SRAM, SDRAM, PSRAM,

NJTRST, JTDI,

NBL[3:0], SDCLKE[1:0], SDNE[1:0],

JTAG & SW

MPU

AHB3

JTCK/SWCLK

NOR Flash, PC Card,

SDNWE, NL

JTDO/SWD, JTDO

NAND Flash

NRAS, NCAS, NADV

ETM

NVIC

NWAIT/NIORD, NREG, CD

TRACECLK

D-BUS

INTR

TRACED[3:0]

Arm Cortex-M4

180 MHz

I-BUS

FPU

1MB Flash

S-BUS

RNG

1MB Flash

Ethernet MAC

DMA/

CACHE

MII or RMII as AF

ART ACCEL/

FIFO

MDIO as AF

10/100

SRAM 112 KB

USB

DMA/

DP, DM

HSYNC, VSYNC

Camera

PUIXCLK, D[13:0]

interface

FIFO

OTG HS

FIFO

ULPI:CK, D[7:0], DIR, STP, NXT

PHY

ID, VBUS, SOF

AHB bus-matrix 8S7M

SRAM 64 KB

8 Streams

DMA2

SRAM 16 KB

AHB2 180 MHz

FIFO

DP

USB

DM

OTG FS

ID, VBUS, SOF

PHY

FIFO

8 Streams

DMA1

AHB1 180 MHz

FIFO

LCD_R[7:0], LCD_G[7:0], LCD_B[7:0],

LCD_HSYNC, LCD_VSYNC, LCD_DE,

LCD_CLK

LCD-TFT

FIFO

Power managmt

VDD

Voltage

VDD = 1.8 to 3.6 V

regulator

VSS

3.3 to 1.2 V

VCAP1, VCAP2

@ V DD

@ V DDA

CHROM-ART

POR

FIFO

DMA2D

RC

HS

reset

PA[15:0]

GPIO PORT A

RC

L S

VDDA, VSSA

PLL1,2,3

NRST

PB[15:0]

GPIO PORT B

Supply

supervision

POR/PDR

Int

BOR

PVD

@ V DD

@ V DDA

PC[15:0]

GPIO PORT C

OSC_IN

XTAL OSC

OSC_OUT

PD[15:0]

4- 26MHz

GPIO PORT D

PE[15:0]

GPIO PORT E

Reset &

clock

M AN A G T

IWDG

control

Standby

GPIO PORT F

PF[15:0]

VBAT = 1.65 to 3.6 V

interface

@ V

PG[15:0]

BAT

GPIO PORT G

OSC32_IN

XTAL 32 kHz

PH[15:0]

GPIO PORT H

PCLKx

LS

HCLKx

RTC

AWU

PI[15:0]

GPIO PORT I

OSC32_OUT

RTC_AF1

RTC_AF1

Backup register

RTC_50HZ

PJ[15:0]

LS

4 KB BKPSRAM

GPIO PORT J

PK[7:0]

GPIO PORT K

4 channels, ETR as AF

32b

16b

4 channels, ETR as AF

168 AF

DMA1

DMA2

EXT IT. WKUP

16b

4 channels, ETR as AF

D[7:0]

AHB/APB2

CMD, CK as AF

SDIO / MMC

32b

4 channels

AHB/APB1

FIFO

4 compl. chan. (TIM1_CH1[1:4]N),

4 chan. (TIM1_CH1[1:4]ETR,

16b

16b

TIM1 / PWM

2 channels as AF

BKIN as AF

TIM13

16b

1 channel as AF

4 compl. chan.(TIM8_CH1[1:4]N),

TIM8 / PWM

16b

4 chan. (TIM8_CH1[1:4], ETR,

16b

1 channel as AF

BKIN as AF

16b

RX, TX as AF

smcard

USART2

CTS, RTS as AF

irDA

WWDG

1 channel as AF

TIM10

16b

RX, TX as AF

smcard

USART3

CTS, RTS as AF

irDA

1 channel as AF

16b

TIM11

UART4

RX, TX, CK,

smcard

RX, TX as AF

USART1

CTS, RTS as AF

irDA

RX, TX as AF

UART5

RX, TX, CK,

smcard

USART6

RX, TX as AF

CTS, RTS as AF

UART7

irDA

MOSI, MISO,

SPI1

SCK, NSS as AF

RX, TX as AF

UART8

16b

MOSI, MISO,

MOSI/SD, MISO/SD_ext, SCK/CK

SPI4

A P B 1 3 0 M Hz

SP2/I2S2

SCK, NSS as AF

16b

NSS/WS, MCK as AF

MOSI, MISO,

MOSI/SD, MISO/SD_ext, SCK/CK

SP3/I2S3

SCK, NSS as AF

SPI5

NSS/WS, MCK as AF

APB1 45 MHz (max)

MOSI, MISO,

A P B 2 60 M Hz

SPI6

I2C1/SMBUS

SCK, NSS as AF

SCL, SDA, SMBA as AF

SAI1

SD, SCK, FS, MCLK as AF

@ VDDA

I2C2/SMBUS

SCL, SDA, SMBA as AF

FIFO

TIM2

TIM3

TIM4

TIM5

TIM12

TIM14

TIM9

2 channels as AF

APB2 90 MHz

TIM6

TIM7

VDDREF_ADC

Temperature sensor

DAC1

Digital filter

U S AR T 2 M B p s

ADC1

DAC2

TX, RX

bxCAN1

8 analog inputs common

ADC2

SCL, SDA, SMBA as AF

I2C3/SMBUS

ITF

8 analog inputs common

to the 3 ADCs

I F

IF

to the ADC1 & 2

bxCAN2

TX, RX

FIFO

ADC3

8 analog inputs for ADC3

DAC1_OUT

DAC2_OUT

@ VDDA

as AF

as AF

MSv30420V5.svg

1.

The timers connected to APB2 are clocked from TIMxCLK up to 180 MHz, while the timers connected to APB1 are clocked

from TIMxCLK either up to 90 MHz or 180 MHz depending on TIMPRE bit configuration in the RCC_DCKCFGR register.

2.

The LCD-TFT is available only on STM32F429xx devices.

20/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

3

Functional overview

®

®

3.1

Arm

Cortex

-M4 with FPU and embedded flash and SRAM

®

®

The Arm

Cortex

-M4 with FPU processor is the latest generation of Arm® processors for

embedded systems. It was developed to provide a low-cost platform that meets the needs of

MCU implementation, with a reduced pin count and low-power consumption, while

delivering outstanding computational performance and an advanced response to interrupts.

®

®

The Arm

Cortex

-M4 with FPU core is a 32-bit RISC processor that features exceptional

code-efficiency, delivering the high-performance expected from an Arm® core in the

memory size usually associated with 8- and 16-bit devices.

The processor supports a set of DSP instructions, which allow efficient signal processing

and complex algorithm execution.

Its single-precision FPU (floating-point unit) speeds up software development by using

metalanguage development tools, while avoiding saturation.

The STM32F42x family is compatible with all Arm tools and software.

Figure 4 shows the general block diagram of the STM32F42x family.

Note:

Cortex®-M4 with FPU core is binary compatible with the Cortex®-M3 core.

3.2

Adaptive real-time memory accelerator (ART Accelerator™)

The ART Accelerator™ is a memory accelerator, which is optimized for STM32 industry-

®

®

standard Arm

Cortex

-M4 with FPU processors. It balances the inherent performance

®

®

advantage of the Arm

Cortex

-M4 with FPU over flash memory at higher frequencies.

To release the processor full 225 DMIPS performance at this frequency, the accelerator

implements an instruction prefetch queue and branch cache, which increases program

execution speed from the 128-bit flash memory. Based on the CoreMark benchmark, the

performance achieved thanks to the ART Accelerator is equivalent to 0 wait state program

execution from the flash memory at a CPU frequency up to 180 MHz.

3.3

Memory protection unit

The memory protection unit (MPU) is used to manage the CPU accesses to memory to

prevent one task to accidentally corrupt the memory or resources used by any other active

task. This memory area is organized into up to 8 protected areas that can in turn be divided

up into 8 subareas. The protection area sizes are between 32 bytes and the whole 4

gigabytes of addressable memory.

The MPU is especially helpful for applications where some critical or certified code has to be

protected against the misbehavior of other tasks. It is usually managed by an RTOS (real-

time operating system). If a program accesses a memory location that is prohibited by the

MPU, the RTOS can detect it and act. In an RTOS environment, the kernel can dynamically

update the MPU area setting, based on the process to be executed.

The MPU is optional and can be bypassed for applications that do not need it.

DS9405 Rev 13

21/240

44

Functional overview

STM32F427xx STM32F429xx

3.4

Embedded flash memory

The devices embed 512 bytes of OTP memory, and a flash memory of up to 2 Mbytes

available for storing programs and data.

3.5

CRC (cyclic redundancy check) calculation unit

The CRC (cyclic redundancy check) calculation unit is used to get a CRC code from a 32-bit

data word and a fixed generator polynomial.

Among other applications, CRC-based techniques are used to verify data transmission or

storage integrity. In the scope of the EN/IEC 60335-1 standard, they offer a means of

verifying the flash memory integrity. The CRC calculation unit helps compute a software

signature during runtime, to be compared with a reference signature generated at link-time

and stored at a given memory location.

3.6

Embedded SRAM

All devices embed:

•

Up to 256Kbytes of system SRAM including 64 Kbytes of CCM (core coupled memory)

data RAM

RAM memory is accessed (read/write) at CPU clock speed with 0 wait states.

•

4 Kbytes of backup SRAM

This area is accessible only from the CPU. Its content is protected against possible

unwanted write accesses, and is retained in Standby or V

mode.

BAT

3.7

Multi-AHB bus matrix

The 32-bit multi-AHB bus matrix interconnects all the masters (CPU, DMAs, Ethernet, USB

HS, LCD-TFT, and DMA2D) and the slaves (Flash memory, RAM, FMC, AHB, and APB

peripherals) and ensures a seamless and efficient operation even when several high-speed

peripherals work simultaneously.

22/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

Figure 5. STM32F427xx and STM32F429xx Multi-AHB matrix

64-Kbyte

CCM data RAM

ARM

GP

MAC

GP

USB OTG

DMA1

Cortex-M4

DMA2

Ethernet

(DMA2D)

HS

I-bus

S-bus

D-bus

DMA_PI

DMA_P2

Chrom ART Accelerator

LCD-TFT

DMA2D

USB_HS_M

LCD-TFT_M

DMA_MEM1

DMA_MEM2

ETHERNET_M

ICODE

Flash

memory

DCODE

ACCEL

SRAM1

112 Kbyte

SRAM2

16 Kbyte

SRAM3

64 Kbyte

AHB2

APB1

peripherals

AHB1

peripherals

FMC external

APB2

MemCtl

Bus matrix-S

MS30421V6

3.8

DMA controller (DMA)

The devices feature two general-purpose dual-port DMAs (DMA1 and DMA2) with 8

streams each. They are able to manage memory-to-memory, peripheral-to-memory, and

memory-to-peripheral transfers. They feature dedicated FIFOs for APB/AHB peripherals,

support burst transfer and are designed to provide the maximum peripheral bandwidth

(AHB/APB).

The two DMA controllers support circular buffer management, so that no specific code is

needed when the controller reaches the end of the buffer. The two DMA controllers also

have a double buffering feature, which automates the use and switching of two memory

buffers without requiring any special code.

Each stream is connected to dedicated hardware DMA requests, with support for software

trigger on each stream. Configuration is made by software and transfer sizes between

source and destination are independent.

DS9405 Rev 13

23/240

44

Functional overview

STM32F427xx STM32F429xx

The DMA can be used with the main peripherals:

2

•

SPI and I

S

2

•

I

C

•

USART

•

General-purpose, basic, and advanced-control timers TIMx

•

DAC

•

SDIO

•

Camera interface (DCMI)

•

ADC

•

SAI1.

3.9

Flexible memory controller (FMC)

All devices embed an FMC. It has four Chip Select outputs supporting the following modes:

PCCard/Compact flash, SDRAM/LPSDR SDRAM, SRAM, PSRAM, NOR flash and NAND

flash.

Functionality overview:

•

8-,16-, 32-bit data bus width

•

Read FIFO for SDRAM controller

•

Write FIFO

•

Maximum FMC_CLK/FMC_SDCLK frequency for synchronous accesses is 90 MHz.

LCD parallel interface

The FMC can be configured to interface seamlessly with most graphic LCD controllers. It

supports the Intel 8080 and Motorola 6800 modes, and is flexible enough to adapt to

specific LCD interfaces. This LCD parallel interface capability makes it easy to build cost-

effective graphic applications using LCD modules with embedded controllers or high-

performance solutions using external controllers with dedicated acceleration.

3.10

LCD-TFT controller (available only on STM32F429xx)

The LCD-TFT display controller provides a 24-bit parallel digital RGB (Red, Green, Blue)

and delivers all signals to interface directly to a broad range of LCD and TFT panels up to

XGA (1024x768) resolution with the following features:

•

Two display layers with dedicated FIFO (64x32-bit)

•

Color look-up table (CLUT) up to 256 colors (256x24-bit) per layer

•

Up to eight input color formats selectable per layer

•

Flexible blending between two layers using alpha value (per pixel or constant)

•

Flexible programmable parameters for each layer

•

Color keying (transparency color)

•

Up to four programmable interrupt events.

24/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

3.11

Chrom-ART Accelerator™ (DMA2D)

The Chrom-Art Accelerator™ (DMA2D) is a graphic accelerator, which offers advanced bit

blitting, row data copy and pixel format conversion. It supports the following functions:

•

Rectangle filling with a fixed color

•

Rectangle copy

•

Rectangle copy with pixel format conversion

•

Rectangle composition with blending and pixel format conversion.

Various image format coding are supported, from indirect 4 bpp color mode up to 32 bpp

direct color. It embeds dedicated memory to store color lookup tables.

An interrupt can be generated when an operation is complete or at a programmed

watermark.

All the operations are fully automatized and are running independently from the CPU or the

DMAs.

3.12

Nested vectored interrupt controller (NVIC)

The devices embed a nested vectored interrupt controller able to manage 16 priority levels,

®

and handle up to 91 maskable interrupt channels plus the 16 interrupt lines of the Cortex

-

M4 with FPU core.

•

Closely coupled NVIC gives low-latency interrupt processing

•

Interrupt entry vector table address passed directly to the core

•

Allows early processing of interrupts

•

Processing of late arriving, higher-priority interrupts

•

Support tail chaining

•

Processor state automatically saved

•

Interrupt entry restored on interrupt exit with no instruction overhead

This hardware block provides flexible interrupt management features with minimum interrupt

latency.

3.13

External interrupt/event controller (EXTI)

The external interrupt/event controller consists of 23 edge-detector lines used to generate

interrupt/event requests. Each line can be independently configured to select the trigger

event (rising edge, falling edge, both) and can be masked independently. A pending register

maintains the status of the interrupt requests. The EXTI can detect an external line with a

pulse width shorter than the Internal APB2 clock period. Up to 168 GPIOs can be connected

to the 16 external interrupt lines.

3.14

Clocks and startup

On reset the 16 MHz internal RC oscillator is selected as the default CPU clock. The

16 MHz internal RC oscillator is factory-trimmed to offer 1% accuracy over the full

temperature range. The application can then select as system clock either the RC oscillator

or an external 4-26 MHz clock source. This clock can be monitored for failure. If a failure is

DS9405 Rev 13

25/240

44

Functional overview

STM32F427xx STM32F429xx

detected, the system automatically switches back to the internal RC oscillator and a

software interrupt is generated (if enabled). This clock source is input to a PLL thus allowing

to increase the frequency up to 180 MHz. Similarly, full interrupt management of the PLL

clock entry is available when necessary (for example if an indirectly used external oscillator

fails).

Several prescalers allow the configuration of the two AHB buses, the high-speed APB

(APB2) and the low-speed APB (APB1) domains. The maximum frequency of the two AHB

buses is 180 MHz while the maximum frequency of the high-speed APB domains is

90 MHz. The maximum allowed frequency of the low-speed APB domain is 45 MHz.

The devices embed a dedicated PLL (PLLI2S) and PLLSAI, which allows to achieve audio

2

class performance. In this case, the I

S master clock can generate all standard sampling

frequencies from 8 kHz to 192 kHz.

3.15

Boot modes

At startup, boot pins are used to select one out of three boot options:

•

Boot from user flash

•

Boot from system memory

•

Boot from embedded SRAM

The bootloader is located in system memory. It is used to reprogram the flash memory

through a serial interface. Refer to application note AN2606 for details.

3.16

Power supply schemes

•

V

= 1.7 to 3.6 V: external power supply for I/Os and the internal regulator (when

DD

enabled), provided externally through V

pins.

DD

•

V

, V

= 1.7 to 3.6 V: external analog power supplies for ADC, DAC, reset blocks,

SSA

DDA

RCs, and PLL. V

and V

must be connected to V

and V

, respectively.

DDA

SSA

DD

SS

•

V

= 1.65 to 3.6 V: power supply for RTC, external clock 32 kHz oscillator and

BAT

backup registers (through power switch) when V

is not present.

DD

Note:

The V

/V

minimum value of 1.7 V is obtained with the use of an external power supply

DD

DDA

supervisor (refer to Section 3.17.2: Internal reset OFF). Refer to Table 3: Voltage regulator

configuration mode versus device operating mode to identify the packages supporting this

option.

3.17

Power supply supervisor

3.17.1

Internal reset ON

On packages embedding the PDR_ON pin, the power supply supervisor is enabled by

holding PDR_ON high. On the other package, the power supply supervisor is always

enabled.

The device has an integrated power-on reset (POR)/ power-down reset (PDR) circuitry

coupled with a brownout reset (BOR) circuitry. At power-on, POR/PDR is always active and

ensures proper operation starting from 1.8 V. After the 1.8 V POR threshold level is

26/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

reached, the option byte loading process starts, either to confirm or modify default BOR

thresholds, or to disable BOR permanently. Three BOR thresholds are available through

option bytes. The device remains in reset mode when V

is below a specified threshold,

DD

V

, or V

, without the need for an external reset circuit.

POR/PDR

BOR

The device also features an embedded programmable voltage detector (PVD) that monitors

the V

/V

power supply and compares it to the V

threshold. An interrupt can be

DD

DDA

PVD

generated when V

/V

drops below the V

threshold and/or when V

/V

is

DD

DDA

PVD

DD

DDA

higher than the V

threshold. The interrupt service routine can then generate a warning

PVD

message and/or put the MCU into a safe state. The PVD is enabled by software.

3.17.2

Internal reset OFF

This feature is available only on packages featuring the PDR_ON pin. The internal power-on

reset (POR) / power-down reset (PDR) circuitry is disabled through the PDR_ON pin.

An external power supply supervisor should monitor V

and should maintain the device in

DD

reset mode as long as V

is below a specified threshold. PDR_ON should be connected to

DD

this external power supply supervisor. Refer to Figure 6: Power supply supervisor

interconnection with internal reset OFF.

Figure 6. Power supply supervisor interconnection with internal reset OFF

V

DD

power supply supervisor

External V

DD

Ext. reset controller active when

< 1.7 V

V

DD

PDR_ON

Application reset

NRST

signal (optional)

V

DD

MS31383V3

The V

specified threshold, below which the device must be maintained under reset, is

DD

1.7 V (see Figure 7).

A comprehensive set of power-saving mode allows to design low-power applications.

When the internal reset is OFF, the following integrated features are no more supported:

•

The integrated power-on reset (POR) / power-down reset (PDR) circuitry is disabled

•

The brownout reset (BOR) circuitry must be disabled

•

The embedded programmable voltage detector (PVD) is disabled

•

V

functionality is no more available and V

pin should be connected to V

.

BAT

BAT

DD

All packages, except for the LQFP100, allow to disable the internal reset through the

PDR_ON signal.

DS9405 Rev 13

27/240

44

Functional overview

STM32F427xx STM32F429xx

Figure 7. PDR_ON control with internal reset OFF

V

DD

PDR = 1.7 V

time

Reset by other source than

power supply supervisor

NRST

PDR_ON

PDR_ON

time

MS19009V6

3.18

Voltage regulator

The regulator has four operating modes:

•

Regulator ON

–

Main regulator mode (MR)

–

Low-power regulator (LPR)

–

Power-down

•

Regulator OFF

3.18.1

Regulator ON

On packages embedding the BYPASS_REG pin, the regulator is enabled by holding

BYPASS_REG low. On all other packages, the regulator is always enabled.

There are three power modes configured by software when the regulator is ON:

•

MR mode used in Run/sleep modes or in Stop modes

–

In Run/Sleep mode

The MR mode is used either in the normal mode (default mode) or the over-drive

mode (enabled by software). Different voltages scaling are provided to reach the

best compromise between maximum frequency and dynamic power consumption.

28/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

The overdrive mode allows operating at a higher frequency than the normal mode

for a given voltage scaling.

–

In Stop modes

The MR can be configured in two ways during stop mode:

MR operates in normal mode (default mode of MR in stop mode)

MR operates in underdrive mode (reduced leakage mode).

•

LPR is used in the Stop modes:

The LP regulator mode is configured by software when entering Stop mode.

Like the MR mode, the LPR can be configured in two ways during stop mode:

–

LPR operates in normal mode (default mode when LPR is ON)

–

LPR operates in underdrive mode (reduced leakage mode).

•

Power-down is used in Standby mode.

The Power-down mode is activated only when entering in Standby mode. The regulator

output is in high impedance and the kernel circuitry is powered down, inducing zero

consumption. The contents of the registers and SRAM are lost.

Refer to Table 3 for a summary of voltage regulator modes versus device operating modes.

Two external ceramic capacitors should be connected on V

and V

pin. Refer to

CAP_1

CAP_2

Figure 22: Power supply scheme and Table 19: VCAP1/VCAP2 operating conditions.

All packages have the regulator ON feature.

(1)

Table 3. Voltage regulator configuration mode versus device operating mode

Voltage regulator

Run mode

Sleep mode

Stop mode

Standby mode

configuration

Normal mode

MR

MR

MR or LPR

-

Over-drive

MR

MR

-

-

(2)

mode

Under-drive mode

-

-

MR or LPR

-

Power-down

-

-

-

Yes

mode

1.

‘-’ means that the corresponding configuration is not available.

2.

The over-drive mode is not available when V

= 1.7 to 2.1 V.

DD

3.18.2

Regulator OFF

This feature is available only on packages featuring the BYPASS_REG pin. The regulator is

disabled by holding BYPASS_REG high. The regulator OFF mode allows to supply

externally a V

voltage source through V

and V

pins.

12

CAP_1

CAP_2

Since the internal voltage scaling is not managed internally, the external voltage value must

be aligned with the targeted maximum frequency. Refer to Table 17: General operating

conditions.The two 2.2 µF ceramic capacitors should be replaced by two 100 nF decoupling

capacitors. Refer to Figure 22: Power supply scheme.

When the regulator is OFF, there is no more internal monitoring on V

. An external power

12

supply supervisor should be used to monitor the V

of the logic power domain. PA0 pin

12

should be used for this purpose, and act as a power-on reset on V

power domain.

12

DS9405 Rev 13

29/240

44

Functional overview

STM32F427xx STM32F429xx

In regulator OFF mode, the following features are no more supported:

•

PA0 cannot be used as a GPIO pin since it allows to reset a part of the V

logic power

12

domain, which is not reset by the NRST pin.

•

As long as PA0 is kept low, the debug mode cannot be used under power-on reset. As

a consequence, PA0 and NRST pins must be managed separately if the debug

connection under reset or prereset is required.

•

The overdrive and underdrive modes are not available.

•

The Standby mode is not available.

Figure 8. Regulator OFF

V12

Application reset

power

External V

CAP_1/2

supply supervisor

signal (optional)

Ext. reset controller active

< Min V

when V

12

CAP_1/2

V

DD

PA0

NRST

V

DD

BYPASS_REG

V12

V

CAP_1

V

CAP_2

ai18498V3

The following conditions must be respected:

•

V

should always be higher than V

and V

to avoid current injection

DD

CAP_1

CAP_2

between power domains.

•

If the time for V

and V

to reach V

minimum value is faster than the time for

CAP_1

CAP_2

12

V

to reach 1.7 V, then PA0 should be kept low to cover both conditions: until V

DD

CAP_1

and V

reach V

minimum value and until V

reaches 1.7 V (see Figure 9).

CAP_2

12

DD

•

Otherwise, if the time for V

and V

to reach V

minimum value is slower

CAP_1

CAP_2

12

than the time for V

to reach 1.7 V, then PA0 could be asserted low externally (see

DD

Figure 10).

•

If V

and V

go below V

minimum value and V

is higher than 1.7 V, then a

CAP_1

CAP_2

12

DD

reset must be asserted on PA0 pin.

Note:

The minimum value of V

depends on the maximum frequency targeted in the application

12

(see Table 17: General operating conditions).

30/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

Figure 9. Startup in regulator OFF: slow V

slope

DD

- power-down reset risen after V

/V

stabilization

CAP_1

CAP_2

V

DD

PDR = 1.7 V or 1.8 V

/ V

V

CAP_2

CAP_1

V

12

Min V

12

time

NRST

time

ai18491f

1.

This figure is valid whatever the internal reset mode (ON or OFF).

Figure 10. Startup in regulator OFF mode: fast V

slope

DD

- power-down reset risen before V

/V

stabilization

CAP_1

CAP_2

V

DD

PDR = 1.7 V or 1.8 V

/ V

V

CAP_2

CAP_1

V

12

Min V

12

time

NRST

PA0 asserted externally

time

ai18492e

1.

This figure is valid whatever the internal reset mode (ON or OFF).

DS9405 Rev 13

31/240

44

Functional overview

STM32F427xx STM32F429xx

3.18.3

Regulator ON/OFF and internal reset ON/OFF availability

Table 4. Regulator ON/OFF and internal reset ON/OFF availability

Package

Regulator ON

Regulator OFF

Internal reset ON

Internal reset OFF

LQFP100

Yes

No

Yes

No

LQFP144,

LQFP208

Yes

Yes

PDR_ON

WLCSP143,

PDR_ON set to

connected to an

Yes

Yes

LQFP176,

V

external power

UFBGA169,

DD

BYPASS_REG set

BYPASS_REG set

supply supervisor

UFBGA176,

to V

to V

SS

DD

TFBGA216

3.19

Real-time clock (RTC), backup SRAM, and backup registers

The backup domain includes:

•

The real-time clock (RTC)

•

4 Kbytes of backup SRAM

•

20 backup registers

The real-time clock (RTC) is an independent BCD timer/counter. Dedicated registers contain

the second, minute, hour (in 12/24 hour), weekday, date, month, year, in BCD (binary-coded

decimal) format. Correction for 28, 29 (leap year), 30, and 31 day of the month are

performed automatically. The RTC provides a programmable alarm and programmable

periodic interrupts with wake-up from Stop and Standby modes. The subseconds value is

also available in binary format.

It is clocked by a 32.768 kHz external crystal, resonator or oscillator, the internal low-power

RC oscillator or the high-speed external clock divided by 128. The internal low-speed RC

has a typical frequency of 32 kHz. The RTC can be calibrated using an external 512 Hz

output to compensate for any natural quartz deviation.

Two alarm registers are used to generate an alarm at a specific time and calendar fields can

be independently masked for alarm comparison. To generate a periodic interrupt, a 16-bit

programmable binary autoreload downcounter with programmable resolution is available

and allows automatic wake-up and periodic alarms from every 120 µs to every 36 hours.

A 20-bit prescaler is used for the time base clock. It is by default configured to generate a

time base of 1 second from a clock at 32.768 kHz.

The 4-Kbyte backup SRAM is an EEPROM-like memory area. It can be used to store data,

which need to be retained in VBAT and standby mode. This memory area is disabled by

default to minimize power consumption (see Section 3.20: Low-power modes). It can be

enabled by software.

The backup registers are 32-bit registers used to store 80 bytes of user application data

when V

power is not present. Backup registers are not reset by a system, a power reset,

DD

or when the device wakes up from the Standby mode (see Section 3.20: Low-power

modes).

32/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

Additional 32-bit registers contain the programmable alarm subseconds, seconds, minutes,

hours, day, and date.

Like backup SRAM, the RTC and backup registers are supplied through a switch that is

powered either from the V

supply when present or from the V

pin.

DD

BAT

3.20

Low-power modes

The devices support three low-power modes to achieve the best compromise between low-

power consumption, short startup time and available wake-up sources:

•

Sleep mode

In Sleep mode, only the CPU is stopped. All peripherals continue to operate and can

wake up the CPU when an interrupt/event occurs.

•

Stop mode

The Stop mode achieves the lowest power consumption while retaining the contents of

SRAM and registers. All clocks in the 1.2 V domain are stopped, the PLL, the HSI RC

and the HSE crystal oscillators are disabled.

The voltage regulator can be put either in main regulator mode (MR) or in low-power

mode (LPR). Both modes can be configured as follows (see Table 5: Voltage regulator

modes in stop mode):

–

Normal mode (default mode when MR or LPR is enabled)

–

Underdrive mode.

The device can be woken up from the Stop mode by any of the EXTI lines (the EXTI

line source can be one of the 16 external lines, the PVD output, the RTC alarm / wake-

up / tamper / time stamp events, the USB OTG FS/HS wake-up or the Ethernet wake-

up).

Table 5. Voltage regulator modes in stop mode

Voltage regulator

Main regulator (MR)

Low-power regulator (LPR)

configuration

Normal mode

MR ON

LPR ON

Under-drive mode

MR in under-drive mode

LPR in under-drive mode

•

Standby mode

The Standby mode is used to achieve the lowest power consumption. The internal

voltage regulator is switched off so that the entire 1.2 V domain is powered off. The

PLL, the HSI RC and the HSE crystal oscillators are also switched off. After entering

Standby mode, the SRAM and register contents are lost except for registers in the

backup domain and the backup SRAM when selected.

The device exits the Standby mode when an external reset (NRST pin), an IWDG reset,

a rising edge on the WKUP pin, or an RTC alarm / wake-up / tamper /time stamp event

occurs.

The standby mode is not supported when the embedded voltage regulator is bypassed,

and an external power controls the 1.2 V domain.

DS9405 Rev 13

33/240

44

Functional overview

STM32F427xx STM32F429xx

3.21

V

operation

BAT

The V

pin allows to power the device V

domain from an external battery, an external

BAT

BAT

supercapacitor, or from V

when no external battery and an external supercapacitor are

DD

present.

V

operation is activated when V

is not present.

BAT

DD

The V

pin supplies the RTC, the backup registers, and the backup SRAM.

BAT

Note:

When the microcontroller is supplied from V

, external interrupts and RTC alarm/events

BAT

do not exit it from V

operation.

BAT

When PDR_ON pin is not connected to V

(Internal Reset OFF), the V

functionality is

DD

BAT

no more available and V

pin should be connected to V

.

DD

BAT

3.22

Timers and watchdogs

The devices include two advanced-control timers, eight general-purpose timers, two basic

timers and two watchdog timers.

All timer counters can be frozen in debug mode.

Table 6 compares the features of the advanced-control, general-purpose and basic timers.

34/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

Table 6. Timer feature comparison

Max

Max

DMA

Capture/

timer

Timer

Counter

Counter

Prescaler

Complementary

interface

Timer

request

compare

clock

type

resolution

type

factor

output

clock

generation

channels

(MHz)

(MHz)

(1)

Any

Up,

integer

Advanced

TIM1,

16-bit

Down,

between 1

Yes

4

Yes

90

180

-control

TIM8

Up/down

and

65536

Any

Up,

integer

TIM2,

32-bit

Down,

between 1

Yes

4

No

45

90/180

TIM5

Up/down

and

65536

Any

Up,

integer

TIM3,

16-bit

Down,

between 1

Yes

4

No

45

90/180

TIM4

Up/down

and

65536

Any

integer

TIM9

16-bit

Up

between 1

No

2

No

90

180

and

65536

General

purpose

Any

TIM10

integer

,

16-bit

Up

between 1

No

1

No

90

180

TIM11

and

65536

Any

integer

TIM12

16-bit

Up

between 1

No

2

No

45

90/180

and

65536

Any

TIM13

integer

,

16-bit

Up

between 1

No

1

No

45

90/180

TIM14

and

65536

Any

integer

TIM6,

Basic

16-bit

Up

between 1

Yes

0

No

45

90/180

TIM7

and

65536

1.

The maximum timer clock is either 90 or 180 MHz depending on TIMPRE bit configuration in the RCC_DCKCFGR register.

DS9405 Rev 13

35/240

44

Functional overview

STM32F427xx STM32F429xx

3.22.1

Advanced-control timers (TIM1, TIM8)

The advanced-control timers (TIM1, TIM8) can be seen as three-phase PWM generators

multiplexed on six channels. They have complementary PWM outputs with programmable

inserted dead times. They can also be considered as complete general-purpose timers.

Their four independent channels can be used for:

•

Input capture

•

Output compare

•

PWM generation (edge- or center-aligned modes)

•

One-pulse mode output

If configured as standard 16-bit timers, they have the same features as the general-purpose

TIMx timers. If configured as 16-bit PWM generators, they have full modulation capability (0-

100%).

The advanced-control timer can work together with the TIMx timers via the Timer Link

feature for synchronization or event chaining.

TIM1 and TIM8 support independent DMA request generation.

3.22.2

General-purpose timers (TIMx)

There are ten synchronizable general-purpose timers embedded in the STM32F42x devices

(see Table 6 for differences).

•

TIM2, TIM3, TIM4, TIM5

The STM32F42x include 4 full-featured general-purpose timers: TIM2, TIM5, TIM3,

and TIM4. The TIM2 and TIM5 timers are based on a 32-bit autoreload

up/downcounter and a 16-bit prescaler. The TIM3 and TIM4 timers are based on a 16-

bit auto-reload up/downcounter and a 16-bit prescaler. They all feature 4 independent

channels for input capture/output compare, PWM, or one-pulse mode output. This

gives up to 16 input capture/output compare/PWMs on the largest packages.

The TIM2, TIM3, TIM4, TIM5 general-purpose timers can work together, or with the

other general-purpose timers and the advanced-control timers TIM1 and TIM8 via the

Timer Link feature for synchronization or event chaining.

Any of these general-purpose timers can be used to generate PWM outputs.

TIM2, TIM3, TIM4, TIM5 all have independent DMA request generation. They can

handle quadrature (incremental) encoder signals and the digital outputs from 1 to 4

hall-effect sensors.

•

TIM9, TIM10, TIM11, TIM12, TIM13, and TIM14

These timers are based on a 16-bit auto-reload upcounter and a 16-bit prescaler.

TIM10, TIM11, TIM13, and TIM14 feature one independent channel, whereas TIM9

and TIM12 have two independent channels for input capture/output compare, PWM or

one-pulse mode output. They can be synchronized with the TIM2, TIM3, TIM4, TIM5

full-featured general-purpose timers. They can also be used as simple time bases.

3.22.3

Basic timers TIM6 and TIM7

These timers are mainly used for DAC trigger and waveform generation. They can also be

used as a generic 16-bit time base.

TIM6 and TIM7 support independent DMA request generation.

36/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

3.22.4

Independent watchdog

The independent watchdog is based on a 12-bit downcounter and 8-bit prescaler. It is

clocked from an independent 32 kHz internal RC and as it operates independently from the

main clock, it can operate in Stop and Standby modes. It can be used either as a watchdog

to reset the device when a problem occurs, or as a free-running timer for application timeout

management. It is hardware- or software-configurable through the option bytes.

3.22.5

Window watchdog

The window watchdog is based on a 7-bit downcounter that can be set as free-running. It

can be used as a watchdog to reset the device when a problem occurs. It is clocked from

the main clock. It has an early warning interrupt capability and the counter can be frozen in

debug mode.

3.22.6

SysTick timer

This timer is dedicated to real-time operating systems, but could also be used as a standard

downcounter. It features:

•

A 24-bit downcounter

•

Autoreload capability

•

Maskable system interrupt generation when the counter reaches 0

•

Programmable clock source.

2

3.23

Inter-integrated circuit interface ( I

C)

Up to three I²C bus interfaces can operate in multimaster and slave modes. They can

support the standard (up to 100 kHz), and fast (up to 400 kHz) modes. They support the

7/10-bit addressing mode and the 7-bit dual addressing mode (as slave). A hardware CRC

generation/verification is embedded.

They can be served by DMA and they support SMBus 2.0/PMBus.

The devices also include programmable analog and digital noise filters (see Table 7).

Table 7. Comparison of I2C analog and digital filters

Analog filter

Digital filter

Pulse width of

Programmable length from 1 to 15

≥

50 ns

suppressed spikes

I2C peripheral clocks

3.24

Universal synchronous/asynchronous receiver transmitters

(USART)

The devices embed four universal synchronous/asynchronous receiver transmitters

(USART1, USART2, USART3, and USART6) and four universal asynchronous receiver

transmitters (UART4, UART5, UART7, and UART8).

These six interfaces provide asynchronous communication, IrDA SIR ENDEC support,

multiprocessor communication mode, single-wire half-duplex communication mode and

have LIN Master/Slave capability. The USART1 and USART6 interfaces are able to

DS9405 Rev 13

37/240

44

Functional overview

STM32F427xx STM32F429xx

communicate at speeds of up to 11.25 Mbit/s. The other available interfaces communicate

at up to 5.62 bit/s.

USART1, USART2, USART3 and USART6 also provide hardware management of the CTS

and RTS signals, Smart Card mode (ISO 7816 compliant) and SPI-like communication

capability. All interfaces can be served by the DMA controller.

(1)

Table 8. USART feature comparison

Max. baud

Max. baud

USART

Standard

Modem

SPI

Smartcard

rate in Mbit/s

rate in Mbit/s

APB

LIN

irDA

name

features

(RTS/CTS)

master

(ISO 7816)

(oversampling

(oversampling

mapping

by 16)

by 8)

APB2

USART1

X

X

X

X

X

X

5.62

11.25

(max.

90 MHz)

APB1

USART2

X

X

X

X

X

X

2.81

5.62

(max.

45 MHz)

APB1

USART3

X

X

X

X

X

X

2.81

5.62

(max.

45 MHz)

APB1

UART4

X

-

X

-

X

-

2.81

5.62

(max.

45 MHz)

APB1

UART5

X

-

X

-

X

-

2.81

5.62

(max.

45 MHz)

APB2

USART6

X

X

X

X

X

X

5.62

11.25

(max.

90 MHz)

APB1

UART7

X

-

X

-

X

-

2.81

5.62

(max.

45 MHz)

APB1

UART8

X

-

X

-

X

-

2.81

5.62

(max.

45 MHz)

1.

X = feature supported.

3.25

Serial peripheral interface (SPI)

The devices feature up to six SPIs in slave and master modes in full-duplex and simplex

communication modes. SPI1, SPI4, SPI5, and SPI6 can communicate at up to 45 Mbits/s,

SPI2 and SPI3 can communicate at up to 22.5 Mbit/s. The 3-bit prescaler gives 8 master

mode frequencies and the frame is configurable to 8 bits or 16 bits. The hardware CRC

generation/verification supports basic SD Card/MMC modes. All SPIs can be served by the

DMA controller.

The SPI interface can be configured to operate in TI mode for communications in master

mode and slave mode.

38/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

2

3.26

Inter-integrated sound (I

S)

2

Two standard I

S interfaces (multiplexed with SPI2 and SPI3) are available. They can be

operated in master or slave mode, in full duplex and simplex communication modes, and

can be configured to operate with a 16-/32-bit resolution as an input or output channel.

Audio sampling frequencies from 8 kHz up to 192 kHz are supported. When either or both of

2

the I

S interfaces is/are configured in master mode, the master clock can be output to the

external DAC/CODEC at 256 times the sampling frequency.

All I2Sx can be served by the DMA controller.

Note:

For I2S2 full-duplex mode, I2S2_CK and I2S2_WS signals can be used only on GPIO Port

B and GPIO Port D.

3.27

Serial Audio interface (SAI1)

The serial audio interface (SAI1) is based on two independent audio sub-blocks which can

operate as transmitter or receiver with their FIFO. Many audio protocols are supported by

each block: I2S standards, LSB or MSB-justified, PCM/DSP, TDM, AC’97 and SPDIF

output, supporting audio sampling frequencies from 8 kHz up to 192 kHz. Both sub-blocks

can be configured in master or in slave mode.

In master mode, the master clock can be output to the external DAC/CODEC at 256 times of

the sampling frequency.

The two sub-blocks can be configured in synchronous mode when full-duplex mode is

required.

SAI1 can be served by the DMA controller.

3.28

Audio PLL (PLLI2S)

2

The devices feature an additional dedicated PLL for audio I

S and SAI applications. It allows

2

to achieve error-free I

S sampling clock accuracy without compromising on the CPU

performance, while using USB peripherals.

2

The PLLI2S configuration can be modified to manage an I

S/SAI sample rate change

without disabling the main PLL (PLL) used for CPU, USB and Ethernet interfaces.

The audio PLL can be programmed with very low error to obtain sampling rates ranging

from 8 KHz to 192 KHz.

In addition to the audio PLL, a master clock input pin can be used to synchronize the

2

I

S/SAI flow with an external PLL (or Codec output).

3.29

Audio and LCD PLL(PLLSAI)

An additional PLL dedicated to audio and LCD-TFT is used for SAI1 peripheral in case the

PLLI2S is programmed to achieve another audio sampling frequency (49.152 MHz or

11.2896 MHz) and the audio application requires both sampling frequencies simultaneously.

The PLLSAI is also used to generate the LCD-TFT clock.

DS9405 Rev 13

39/240

44

Functional overview

STM32F427xx STM32F429xx

3.30

Secure digital input/output interface (SDIO)

An SD/SDIO/MMC host interface is available, that supports MultiMediaCard System

Specification Version 4.2 in three different databus modes: 1-bit (default), 4-bit and 8-bit.

The interface allows data transfer at up to 48 MHz, and is compliant with the SD Memory

Card Specification Version 2.0.

The SDIO Card Specification Version 2.0 is also supported with two different databus

modes: 1-bit (default) and 4-bit.

The current version supports only one SD/SDIO/MMC4.2 card at any one time and a stack

of MMC4.1 or previous.

In addition to SD/SDIO/MMC, this interface is fully compliant with the CE-ATA digital

protocol Rev1.1.

3.31

Ethernet MAC interface with dedicated DMA and IEEE 1588

support

The devices provide an IEEE-802.3-2002-compliant media access controller (MAC) for

ethernet LAN communications through an industry-standard medium-independent interface

(MII) or a reduced medium-independent interface (RMII). The microcontroller requires an

external physical interface device (PHY) to connect to the physical LAN bus (twisted-pair,

fiber, etc.). The PHY is connected to the device MII port using 17 signals for MII or 9 signals

for RMII, and can be clocked using the 25 MHz (MII) from the microcontroller.

The devices include the following features:

•

Supports 10 and 100 Mbit/s rates

•

Dedicated DMA controller allowing high-speed transfers between the dedicated SRAM

and the descriptors (see the STM32F4xx reference manual for details)

•

Tagged MAC frame support (VLAN support)

•

Half-duplex (CSMA/CD) and full-duplex operation

•

MAC control sublayer (control frames) support

•

32-bit CRC generation and removal

•

Several address filtering modes for physical and multicast address (multicast and

group addresses)

•

32-bit status code for each transmitted or received frame

•

Internal FIFOs to buffer transmit and receive frames. The transmit FIFO and the

receive FIFO are both 2 Kbytes.

•

Supports hardware PTP (precision time protocol) in accordance with IEEE 1588 2008

(PTP V2) with the time stamp comparator connected to the TIM2 input

•

Triggers interrupt when system time becomes greater than target time

3.32

Controller area network (bxCAN)

The two CANs are compliant with the 2.0A and B (active) specifications with a bitrate up to 1

Mbit/s. They can receive and transmit standard frames with 11-bit identifiers as well as

extended frames with 29-bit identifiers. Each CAN has three transmit mailboxes, two receive

40/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

FIFOS with 3 stages and 28 shared scalable filter banks (all of them can be used even if one

CAN is used). 256 bytes of SRAM are allocated for each CAN.

3.33

Universal serial bus on-the-go full-speed (OTG_FS)

The devices embed an USB OTG full-speed device/host/OTG peripheral with integrated

transceivers. The USB OTG FS peripheral is compliant with the USB 2.0 specification and

with the OTG 1.0 specification. It has software-configurable endpoint setting and supports

suspend/resume. The USB OTG full-speed controller requires a dedicated 48 MHz clock

that is generated by a PLL connected to the HSE oscillator. The major features are:

•

Combined Rx and Tx FIFO size of 320 × 35 bits with dynamic FIFO sizing

•

Supports the session request protocol (SRP) and host negotiation protocol (HNP)

•

4 bidirectional endpoints

•

8 host channels with periodic OUT support

•

HNP/SNP/IP inside (no need for any external resistor)

•

For OTG/Host modes, a power switch is needed in case bus-powered devices are

connected

3.34

Universal serial bus on-the-go high-speed (OTG_HS)

The devices embed a USB OTG high-speed (up to 480 Mb/s) device/host/OTG peripheral.

The USB OTG HS supports both full-speed and high-speed operations. It integrates the

transceivers for full-speed operation (12 MB/s) and features a UTMI low-pin interface (ULPI)

for high-speed operation (480 MB/s). When using the USB OTG HS in HS mode, an

external PHY device connected to the ULPI is required.

The USB OTG HS peripheral is compliant with the USB 2.0 specification and with the OTG

1.0 specification. It has software-configurable endpoint setting and supports

suspend/resume. The USB OTG full-speed controller requires a dedicated 48 MHz clock

that is generated by a PLL connected to the HSE oscillator.

The major features are:

•

Combined Rx and Tx FIFO size of 1 Kbit × 35 with dynamic FIFO sizing

•

Supports the session request protocol (SRP) and host negotiation protocol (HNP)

•

6 bidirectional endpoints

•

12 host channels with periodic OUT support

•

Internal FS OTG PHY support

•

External HS or HS OTG operation supporting ULPI in SDR mode. The OTG PHY is

connected to the microcontroller ULPI port through 12 signals. It can be clocked using

the 60 MHz output.

•

Internal USB DMA

•

HNP/SNP/IP inside (no need for any external resistor)

•

for OTG/Host modes, a power switch is needed in case bus-powered devices are

connected

DS9405 Rev 13

41/240

44

Functional overview

STM32F427xx STM32F429xx

3.35

Digital camera interface (DCMI)

The devices embed a camera interface that can connect with camera modules and CMOS

sensors through an 8-bit to 14-bit parallel interface, to receive video data. The camera

interface can sustain a data transfer rate up to 54 Mbyte/s at 54 MHz. It features:

•

Programmable polarity for the input pixel clock and synchronization signals

•

Parallel data communication can be 8-, 10-, 12- or 14-bit

•

Supports 8-bit progressive video monochrome or raw bayer format, YCbCr 4:2:2

progressive video, RGB 565 progressive video or compressed data (like JPEG)

•

Supports continuous mode or snapshot (a single frame) mode

•

Capability to automatically crop the image

3.36

True random number generator (RNG)

The RNG is a true random number generator that provides full entropy outputs to the

application as 32-bit samples. It is composed of a live entropy source (analog) and an

internal conditioning component.

All devices embed an RNG that delivers 32-bit random numbers generated by an integrated

analog circuit.

3.37

General-purpose input/outputs (GPIOs)

Each of the GPIO pins can be configured by software as output (push-pull or open-drain,

with or without pull-up or pull-down), as input (floating, with or without pull-up or pull-down)

or as peripheral alternate function. Most of the GPIO pins are shared with digital or analog

alternate functions. All GPIOs are high-current-capable and have speed selection to better

manage internal noise, power consumption and electromagnetic emission.

The I/O configuration can be locked if needed by following a specific sequence in order to

avoid spurious writing to the I/Os registers.

Fast I/O handling allowing maximum I/O toggling up to 90 MHz.

3.38

Analog-to-digital converters (ADCs)

Three 12-bit analog-to-digital converters are embedded and each ADC shares up to 16

external channels, performing conversions in the single-shot or scan mode. In scan mode,

automatic conversion is performed on a selected group of analog inputs.

Additional logic functions embedded in the ADC interface allow:

•

Simultaneous sample and hold

•

Interleaved sample and hold

The ADC can be served by the DMA controller. An analog watchdog feature allows very

precise monitoring of the converted voltage of one, some or all selected channels. An

interrupt is generated when the converted voltage is outside the programmed thresholds.

To synchronize A/D conversion and timers, the ADCs could be triggered by any of TIM1,

TIM2, TIM3, TIM4, TIM5, or TIM8 timer.

42/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Functional overview

3.39

Temperature sensor

The temperature sensor has to generate a voltage that varies linearly with temperature. The

conversion range is between 1.7 V and 3.6 V. The temperature sensor is internally

connected to the same input channel as V

, ADC1_IN18, which is used to convert the

BAT

sensor output voltage into a digital value. When the temperature sensor and V

BAT

conversion are enabled at the same time, only V

conversion is performed.

BAT

As the offset of the temperature sensor varies from chip to chip due to process variation, the

internal temperature sensor is mainly suitable for applications that detect temperature

changes instead of absolute temperatures. If an accurate temperature reading is needed,

then an external temperature sensor part should be used.

3.40

Digital-to-analog converter (DAC)

The two 12-bit buffered DAC channels can be used to convert two digital signals into two

analog voltage signal outputs.

This dual digital Interface supports the following features:

•

two DAC converters: one for each output channel

•

8-bit or 10-bit monotonic output

•

left or right data alignment in 12-bit mode

•

synchronized update capability

•

noise-wave generation

•

triangular-wave generation

•

dual DAC channel independent or simultaneous conversions

•

DMA capability for each channel

•

external triggers for conversion

•

input voltage reference V

REF+

Eight DAC trigger inputs are used in the device. The DAC channels are triggered through

the timer update outputs that are also connected to different DMA streams.

3.41

Serial wire JTAG debug port (SWJ-DP)

The Arm SWJ-DP interface is embedded, and is a combined JTAG and serial wire debug

port that enables either a serial wire debug or a JTAG probe to be connected to the target.

Debug is performed using 2 pins only instead of 5 required by the JTAG (JTAG pins could

be re-use as GPIO with alternate function): the JTAG TMS and TCK pins are shared with

SWDIO and SWCLK, respectively, and a specific sequence on the TMS pin is used to

switch between JTAG-DP and SW-DP.

DS9405 Rev 13

43/240

44

Functional overview

STM32F427xx STM32F429xx

3.42

Embedded Trace Macrocell™

The Arm Embedded Trace Macrocell provides a greater visibility of the instruction and data

flow inside the CPU core by streaming compressed data at a very high rate from the

STM32F42x through a small number of ETM pins to an external hardware trace port

analyzer (TPA) device. The TPA is connected to a host computer using USB, Ethernet, or

any other high-speed channel. Real-time instruction and data flow activity can be recorded

and then formatted for display on the host computer that runs the debugger software. TPA

hardware is commercially available from common development tool vendors.

The Embedded Trace Macrocell operates with third party debugger software tools.

44/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

4

Pinouts and pin description

Figure 11. STM32F42x LQFP100 pinout

PB7

PB6

PB5

PB4

PB3

PD7

PD6

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

VDD

VSS

PE1

PE0

PB9

PB8

BOOT0

PE2

100

99

98

97

96

95

94

93

92

91

90

89

88

87

86

85

84

83

82

81

80

79

78

77

76

1

75

VDD

PE3

2

74

VSS

PE4

3

73

VCAP_2

PE5

4

72

PA13

PE6

5

71

PA12

VBAT

6

70

PA 11

7

69

PC13

PA10

PC14

8

68

PA9

PC15

9

67

PA8

VSS

10

66

PC9

VDD

11

65

PC8

12

64

PH0

PC7

LQFP100

13

63

PH1

PC6

NRST

14

62

PD15

PC0

15

61

PD14

PC1

16

60

PD13

PC2

17

59

PD12

PC3

18

58

PD11

VDD

19

57

PD10

VSSA

20

56

PD9

VREF+

21

55

PD8

VDDA

22

54

PB15

PA0

23

53

PB14

PA1

24

52

PB13

PA2

25

51

PB12

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

PA4

PA5

PA6

PA7

PA3

PB0

PB1

PB2

PE7

PE8

PE9

PC4

PC5

VSS

VDD

VDD

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

VCAP_1

ai18495c

1.

The above figure shows the package top view.

DS9405 Rev 13

45/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Figure 12. STM32F42x WLCSP143 ballout

11

10

9

8

7

6

5

4

3

2

1

PDR

A

PB8

PB6

PG15

PG12

PD7

PD5

PD2

PC10

VDD

PE1

_ON

B

PA14

PB9

PB7

PB3

PG11

PD4

PD3

PD0

PC11

PE0

PE4

BOOT

PB5

PB4

PG10

VDD

PD1

PC12

PA15

VDD

PE3

VBAT

C

0

VCAP

D

PE5

PE2

VDD

PG13

PG13

PA10

PA11

PA13

VSS

PC13

PC14

_2

E

PC15

PF1

PE6

VSS

VDD

PG9

PC8

PC9

PA9

PA12

VDD

PF4

PF5

PF7

PG14

VSS

PD6

PC7

PC6

PA8

PF2

PF0

F

G

PF10

PF9

VDD

PG5

PG4

PG6

PG3

PG8

VDD

VDD

PF6

PF3

H

NRST

PC0

VSS

PD12

PD13

PD10

VSS

PG7

PH1

VSS

PF8

J

PC3

VDD

VDD

VDD

VDD

PE10

PB15

PD14

PG2

PC2

PH0

PA0

PA1

PB1

PF13

PG1

PE11

PB14

PD11

PD15

PC1

VSSA

PC1

K

VREF

L

PA2

PA7

PB2

PF14

PE7

PE12

PE15

PD8

VDD

VDDA

+

M

PA5

PC4

PF11

PF15

PE8

PE14

PB10

PB12

PD9

PA4

PA3

VCAP

BYPASS_

N

PC5

PB0

PF12

PG0

PE9

PE13

PB11

PB13

PA6

_1

REG

MS31855V2

1.

The above figure shows the package bump view.

46/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Figure 13. STM32F42x LQFP144 pinout

DD

DD

SS

DD

SS

V

PDR_ON

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PG15

V

V

PG14

PG13

PG12

PG11

PG10

PG9

PD7

PD6

V

V

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

PE2

144

143

142

141

140

139

138

137

136

135

134

133

132

131

130

129

128

127

126

125

124

123

122

121

109

120

119

118

117

116

115

114

113

112

111

110

108

1

V

DD

PE3

107

2

V

SS

PE4

106

3

V

CAP_2

PE5

105

4

PA13

PE6

104

5

PA12

VBAT

103

6

PA11

PC13

102

7

PA10

PC14

101

8

PA9

PC15

100

9

PA8

PF0

99

10

PC9

PF1

98

11

PC8

PF2

97

12

PC7

PF3

96

13

PC6

PF4

95

14

V

DD

PF5

94

15

V

SS

V

93

16

PG8

SS

V

92

17

PG7

DD

PF6

91

18

PG6

LQFP144

PF7

90

19

PG5

89

PF8

20

PG4

88

PF9

21

PG3

87

PF10

22

PG2

86

23

PH0

PD15

85

24

PH1

PD14

84

25

NRST

V

DD

26

PC0

83

V

SS

27

PC1

82

PD13

28

PC2

81

PD12

29

PC3

80

PD11

V

30

79

PD10

DD

V

31

78

PD9

SSA

32

V

77

PD8

REF+

33

V

76

PB15

DDA

34

PA0

75

PB14

35

PA1

74

PB13

36

PA2

73

PB12

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

55

56

57

58

59

60

72

61

62

63

64

65

66

67

68

69

70

71

SS

DD

SS

SS

DD

DD

DD

V

V

V

V

V

V

V

PA3

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PE7

PE8

PE9

PC4

PC5

PG0

PG1

PF11

PF12

PF13

PF14

PF15

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

CAP_1

ai18496b

V

1.

The above figure shows the package top view.

DS9405 Rev 13

47/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Figure 14. STM32F42x LQFP176 pinout

DD

SS

DD

SS

DD

SS

DD

PI7

PI6

PI5

PI4

PDR_ON

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PG15

V

V

PG14

PG13

PG12

PG11

PG10

PG9

PD7

PD6

V

V

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

V

V

PI3

PI2

V

PE2

140

176

175

174

173

172

171

170

169

168

167

166

165

164

163

162

161

160

159

158

157

156

155

154

153

141

139

138

137

136

135

134

133

152

151

150

149

148

147

146

145

144

143

142

132

1

PI1

PE3

131

2

PI0

PE4

130

3

PH15

PE5

129

4

PH14

PE6

128

5

PH13

VBAT

V

127

6

DD

PI8

V

126

7

SS

PC13

V

125

8

CAP_2

PC14

124

9

PA13

PC15

123

10

PA12

PI9

122

11

PA11

121

PI10

12

PA10

120

13

PI11

PA9

119

14

VSS

PA8

118

15

VDD

PC9

117

16

PF0

PC8

116

17

PF1

PC7

115

18

PF2

PC6

114

19

V

PF3

DD

113

20

PF4

V

SS

112

21

PF5

PG8

LQFP176

111

22

VSS

PG7

110

23

VDD

PG6

109

24

PF6

PG5

108

25

PF7

PG4

26

PF8

107

PG3

27

PF9

106

PG2

28

PF10

105

PD15

29

PH0

104

PD14

30

PH1

103

V

DD

31

NRST

102

V

SS

32

PC0

101

PD13

33

PC1

100

PD12

34

PC2

PD11

99

35

PC3

PD10

98

36

VDD

97

PD9

37

PD8

VSSA

96

VREF+

38

PB15

95

VDDA

39

PB14

94

40

PB13

PA0

93

41

PB12

PA1

92

42

V

PA2

91

DD

43

V

PH2

90

SS

44

PH3

89

PH12

45

46

47

48

49

50

51

52

53

54

55

56

57

58

59

60

61

62

63

64

65

66

67

68

80

69

70

71

72

73

74

75

76

77

78

79

88

81

82

83

84

85

86

87

SS

DD

DD

DD

DD

VSS

V

V

V

V

PE7

PE8

PE9

PA3

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PC4

PC5

PH4

PH5

PH6

PH7

PH8

PH9

V

PG0

PG1

PF13

PF14

PF15

PF11

PF12

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

PH10

PH11

CAP_1

V

BYPASS_REG

MS31878V1

1.

The above figure shows the package top view.

48/240

DS9405 Rev 13

Figure 15. STM32F42x LQFP208 pinout

STM32F427xx STM32F429xx

PI7

PI6

PI5

PI4

VDD

PDR_ON

VSS

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PG15

PK7

PK6

PK5

PK4

PK3

VDD

VSS

PG14

PG13

PG12

PG11

PG10

PG9

PJ15

PJ14

PJ13

PJ12

PD7

PD6

VDD

VSS

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

VDD

PI3

165

163

162

161

160

159

158

157

208

207

206

205

204

203

202

201

200

199

198

197

196

195

194

193

192

191

190

189

188

187

186

185

184

183

182

181

180

179

178

177

176

175

174

173

172

171

170

169

168

167

166

164

PI2

156

PE2

1

PI1

155

PE3

2

PI0

154

PE4

3

PH15

153

PE5

4

PH14

152

PE6

5

PH13

151

VBAT

6

VDD

150

PI8

7

VSS

149

PC13

8

VCAP2

148

PC14

9

PA13

147

PC15

10

PA12

146

PI9

11

PA11

145

PI10

12

PA10

144

PI11

13

PA9

143

VSS

14

PA8

142

VDD

15

PC9

141

PF0

16

PC8

140

PF1

17

PC7

139

PF2

18

PC6

138

PI12

19

VDD

137

PI13

20

VSS

136

PI14

21

PG8

135

PF3

22

DS9405 Rev 13

PG7

134

PF4

23

PG6

133

PF5

24

LQFP208

PG5

132

VSS

25

PG4

131

VDD

26

PG3

130

PF6

27

PG2

129

PF7

28

PK2

128

PF8

29

PK1

127

PF9

30

PK0

126

PF10

31

VSS

125

PH0

32

VDD

124

PH1

33

PJ11

123

NRST

34

PJ10

122

PC0

35

PJ9

121

PC1

36

PJ8

120

PC2

37

PJ7

119

PC3

38

PJ6

118

VDD

39

PD15

117

VSSA

40

PD14

116

VREF+

41

Pinouts and pin description

VDD

115

VDDA

42

VSS

114

PA0

43

PD13

113

PA1

44

PD12

112

PA2

45

PD11

111

PH2

46

PD10

110

PH3

47

PD9

109

PH4

48

PD8

108

PH5

49

PB15

107

PA3

50

PB14

106

VSS

51

PB13

105

VDD

52

53

54

55

56

57

58

59

60

61

62

63

64

65

66

67

68

69

70

71

72

73

74

75

76

77

78

79

80

81

82

83

84

85

86

87

88

89

90

91

92

93

94

95

96

97

98

99

100

101

102

103

104

PJ0

PJ1

PJ2

PJ3

PJ4

PJ5

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PE7

PE8

PE9

49/240

PC4

PC5

PH6

PH7

PH8

PH9

VSS

VSS

PG0

PG1

VSS

VSS

PI15

VDD

VDD

VDD

VDD

VDD

MS30422V2

PF11

PF12

PF13

PF14

PF15

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

PB12

PH10

PH11

PH12

VCAP1

1.

The above figure shows the package top view.

Pinouts and pin description

STM32F427xx STM32F429xx

Figure 16. STM32F42x UFBGA169 ballout

1

2

3

4

5

6

7

8

9

10

11

12

13

A

PI6

PI5

PE1

BOOT0

PB4

PG12

PD7

PD3

PC12

PA14

PI3

PE2

B

PA15

PG11

PI4

PE0

PB7

PI7

PB3

PD6

PC11

PD2

PI2

PI0

C

PE3

PE4

PDR

PB9

_ON

PB6

PG15

PG10

PD5

PD1

PC10

PH15

PI1

PH14

D

PE5

PE6

PB8

PB5

VSS

VDD

PD4

PD0

VDD

VCAP

PH13

VDD

VSS

_2

E

PC14

PI9

PI10

PC13

VBAT

VDD

VSS

PA9

PA10

PA13

PA11

PA12

PA8

F

PC15

VDD

VSS

VSS

PF0

PF1

VDD

VDD

PC7

PC8

PC6

PC9

PG8

PH1

G

PH0

PF4

PF3

PF2

PC0

VDD

VSS

VSS

VDD

PG7

PG5

PG6

H

PF10

NRST

PF5

VDD

PC1

PC2

PC3

VDD

PE13

PD11

PD14

PG4

PG2

J

VSSA

VREF-

VREF+

VDDA

PA0

VSS

VSS

PE8

PE14

VSS

VDD

PD15

PD12

K

PA1

PB1

PA2

PA3

PA7

VDD

PE15

PF14

PE9

PD13

PH9

PD10

PD9

L

PH3

PH2

PH5

PC4

PB2

VDD

PF15

PE10

PD8

PB15

PB10

PH8

PH12

BYPASS

M

PH4

PA5

_REG

PF11

PC5

PF13

PG1

PE11

PH7

PB11

PH11

PB13

PB14

N

PA4

PA6

PF12

PG0

PB0

PE7

PE12

VCAP

PH6

_1

PH10

PB12

MS33732V1

1.

The above figure shows the package top view.

2.

The 4 corners balls, A1, A13, N1 and N13, are not bonded internally and should be left not connected on the PCB.

50/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Figure 17. STM32F42x UFBGA176 ballout

1

2

3

9

10

11

12

13

14

15

4

5

6

7

8

PE1

PE0

PB8

PB5

A

PE3

PE2

PG14

PG13

PB4

PB3

PD7

PC12

PA15

PA14

PA13

PB9

PB7

PE6

PB6

PG15

PG12

PG11

PG10

PD6

PD0

PC11

PC10

PA12

B

PE4

PE5

VDD

PI7

PI6

PI5

PDR_ON

VBAT

VDD

VDD

VDD

PG9

PD5

PD1

PI3

PI2

PA11

C

VSS

VSS

VSS

PD4

PD3

PD2

PH15

PI1

PA10

PC13

PI8

VSS

D

PI9

PI4

BOOT0

PF0

PI10

PI11

PH13

PH14

PI0

PA9

PC14

E

VSS

PH2

VSS

VSS

VSS

VSS

VSS

VCAP2

PC9

PA8

VDD

F

PC15

VSS

VSS

PH0

VSS

VDD

PH3

VSS

VSS

VSS

VSS

VSS

VDD

PC8

PC7

G

PF1

H

PF2

PH4

VSS

VSS

VSS

VSS

VSS

VSS

VDD

PG8

PC6

PH1

NRST

PF3

PF4

PH5

VSS

J

VSS

VSS

VSS

VSS

VDD

VDD

PG7

PG6

PF5

K

PF6

VDD

VSS

PF7

SS

VSS

VSS

VSS

PH12

PG5

PG4

PG3

V

BYPASS_

L

PF9

PF8

PH11

PH10

PD15

PG2

PF10

REG

PB2

PG1

VSS

VSS

VCAP_1

PH6

PH8

PH9

PD14

PD13

M

VSSA

PC0

PC1

PC2

PC3

PG0

VDD

VDD

VDD

PE13

PH7

PD12

PD11

PD10

N

VREF-

PA4

PC4

PF13

PA0

PA1

PF12

PF15

PE8

PE9

PE11

PE14

PB12

PB13

PD9

PD8

PA2

PA6

PA5

PC5

P

VREF+

R

VDDA

PF14

PE7

PE10

PE12

PE15

PB10

PB11

PB14

PB15

PA3

PA7

PB1

PB0

PF11

ai18497c

1.

The above figure shows the package top view.

DS9405 Rev 13

51/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Figure 18. STM32F42x TFBGA216 ballout

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

PE4

PE3

PE2

PG14

PE1

PE0

PB8

PB5

PB4

PB3

PD7

PC12

PA15

PA14

PA13

A

PG15

PG11

PJ13

PE5

PE6

PG13

PB9

PB7

PB6

PJ12

PD6

PD0

PC11

PC10

PA12

B

VBAT

PI8

PI4

PK7

PK6

PK5

PD5

PD3

PD1

PI3

PI2

PA11

PG12

PG10

PJ14

C

PK4

PK3

PG9

PC13

PF0

PI5

PI7

PI10

PI6

PJ15

PD4

PD2

PH15

PI1

PA10

D

PDR_

BOOT0 VDD

VDD

VDD

PC14

PF1

PI12

PI9

VDD

VCAP2

PH13

PH14

PI0

PA9

E

ON

F

PC15

VSS

PI11

VDD

VDD

VSS

VSS

VSS

VSS

VSS

VDD

PK1

PK2

PC9

PA8

G

PH0

PF2

PI13

PI15

VDD

VSS

VDD

PJ11

PK0

PC8

PC7

VSS

H

PH1

PF3

PI14

PH4

VDD

VSS

VSS

VDD

PJ8

PJ10

PG8

PC6

J

NRST

PF4

PH5

PH3

VDD

VSS

VSS

VDD

PJ7

PJ9

PG7

PG6

K

PF7

PF6

PF5

PH2

VDD

VSS

VSS

VDD

PJ6

PD15

PB13

PD10

VSS

VSS

VSS

L

BYPASS-

PF10

PF9

PF8

PC3

VSS

VDD

VCAP1

PD14

PB12

PD9

PD8

VDD

VDD

VDD

REG

VSSA

PC0

PC1

PC2

PB2

PF12

PG1

PF15

PJ4

PD12

PD13

PG3

PG2

PJ5

PH12

M

N

VREF-

PA1

PA0

PA4

PC4

PF13

PG0

PJ3

PE8

PD11

PG5

PG4

PH7

PH9

PH11

P

VREF+

PA2

PA6

PA5

PC5

PF14

PJ2

PF11

PE9

PE11

PE14

PB10

PH6

PH8

PH10

R

VDDA

PA3

PA7

PB1

PB0

PJ0

PJ1

PE7

PE10

PE12

PE15

PE13

PB11

PB14

PB15

MS30423V2

1.

The above figure shows the package top view.

52/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 9. Legend/abbreviations used in the pinout table

Name

Abbreviation

Definition

Unless otherwise specified in brackets below the pin name, the pin function during and after

Pin name

reset is the same as the actual pin name

S

Supply pin

Pin type

I

Input only pin

I/O

Input / output pin

FT

5 V tolerant I/O

TTa

3.3 V tolerant I/O directly connected to ADC

I/O structure

B

Dedicated BOOT0 pin

RST

Bidirectional reset pin with  weak pull-up resistor

Notes

Unless otherwise specified by a note, all I/Os are set as floating inputs during and after reset

Alternate

Functions selected through GPIOx_AFR registers

functions

Additional

Functions directly selected/enabled through peripheral registers

functions

Table 10. STM32F427xx and STM32F429xx pin and ball definitions

Pin number

Pin name

Additional

(2)

(function

Alternate functions

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TRACECLK,

SPI4_SCK,

1

1

B2

A2

1

D8

1

A3

PE2

I/O FT

-

SAI1_MCLK_A,

-

ETH_MII_TXD3,

FMC_A23, EVENTOUT

TRACED0,

2

2

C1

A1

2

C10

2

A2

PE3

I/O FT

-

SAI1_SD_B,

-

FMC_A19, EVENTOUT

TRACED1, SPI4_NSS,

SAI1_FS_A, FMC_A20,

3

3

C2

B1

3

B11

3

A1

PE4

I/O FT

-

-

DCMI_D4, LCD_B0,

EVENTOUT

DS9405 Rev 13

53/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TRACED2, TIM9_CH1,

SPI4_MISO,

4

4

D1

B2

4

D9

4

B1

PE5

I/O FT

-

SAI1_SCK_A,

-

FMC_A21, DCMI_D6,

LCD_G0, EVENTOUT

TRACED3, TIM9_CH2,

SPI4_MOSI,

5

5

D2

B3

5

E8

5

B2

PE6

I/O FT

-

SAI1_SD_A,

-

FMC_A22, DCMI_D7,

LCD_G1, EVENTOUT

-

-

-

-

-

-

-

G6

V

S

-

-

-

-

SS

-

-

-

-

-

-

-

F5

V

S

-

-

-

-

DD

S

-

-

-

-

6

6

E5

C1

6

C11

6

C1

V

BAT

(4)

NC

-

-

D2

7

-

7

C2

PI8

I/O FT

EVENTOUT

TAMP_2

(3)

(5)

(4)

7

7

E4

D1

8

D10

8

D1

PC13

I/O FT

EVENTOUT

TAMP_1

(5)

PC14-

(4)

OSC32_IN

OSC32_IN

8

8

E1

E1

9

D11

9

E1

EVENTOUT

I/O FT

(6)

(5)

(PC14)

PC15-

(4)

OSC32_

OSC32_OUT

9

9

F1

F1

10

E11

10

F1

EVENTOUT

I/O FT

(6)

(5)

OUT

(PC15)

-

-

-

-

-

-

-

G5

V

S

-

-

-

-

DD

CAN1_RX, FMC_D30,

-

-

E2

D3

11

-

11

E4

PI9

I/O FT

-

LCD_VSYNC,

-

EVENTOUT

ETH_MII_RX_ER,

FMC_D31,

-

-

E3

E3

12

-

12

D5

PI10

I/O FT

-

-

LCD_HSYNC,

EVENTOUT

OTG_HS_ULPI_DIR,

NC

E4

13

-

13

F3

PI11

I/O FT

-

-

-

-

(3)

EVENTOUT

S

-

-

-

-

-

-

F6

F2

14

E7

14

F2

V

SS

54/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

-

-

F4

F3

15

E10

15

F4

V

S

-

-

-

-

DD

I2C2_SDA, FMC_A0,

-

10

F2

E2

16

F11

16

D2

PF0

I/O FT

-

-

EVENTOUT

I2C2_SCL, FMC_A1,

-

11

F3

H3

17

E9

17

E2

PF1

I/O FT

-

-

EVENTOUT

I2C2_SMBA, FMC_A2,

-

12

G5

H2

18

F10

18

G2

PF2

I/O FT

-

-

EVENTOUT

LCD_HSYNC,

-

-

-

-

-

-

19

E3

PI12

I/O FT

-

-

EVENTOUT

LCD_VSYNC,

-

-

-

-

-

-

20

G3

PI13

I/O FT

-

-

EVENTOUT

-

-

-

-

-

-

21

H3

PI14

I/O FT

LCD_CLK, EVENTOUT

-

(6)

FMC_A3, EVENTOUT

ADC3_IN9

-

13

G4

J2

19

G11

22

H2

PF3

I/O FT

ADC3_

(6)

-

14

G3

J3

20

F9

23

J2

PF4

I/O FT

FMC_A4, EVENTOUT

IN14

ADC3_

(6)

FMC_A5, EVENTOUT

-

15

H3

K3

21

F8

24

K3

PF5

I/O FT

IN15

S

-

-

-

-

10

16

G7

G2

22

H7

25

H6

V

SS

S

-

-

-

-

11

17

G8

G3

23

-

26

H5

V

DD

TIM10_CH1,

SPI5_NSS,

SAI1_SD_B,

NC

(6)

K2

24

G10

27

K2

PF6

I/O FT

-

18

ADC3_IN4

(3)

UART7_Rx,

FMC_NIORD,

EVENTOUT

TIM11_CH1,

SPI5_SCK,

SAI1_MCLK_B,

NC

(6)

K1

25

F7

28

K1

PF7

I/O FT

-

19

ADC3_IN5

(3)

UART7_Tx,

FMC_NREG,

EVENTOUT

DS9405 Rev 13

55/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

SPI5_MISO,

SAI1_SCK_B,

NC

(6)

-

20

L3

26

H11

29

L3

PF8

I/O FT

TIM13_CH1,

ADC3_IN6

(3)

FMC_NIOWR,

EVENTOUT

SPI5_MOSI,

NC

SAI1_FS_B,

(6)

-

21

L2

27

G8

30

L2

PF9

I/O FT

ADC3_IN7

(3)

TIM14_CH1, FMC_CD,

EVENTOUT

FMC_INTR,

(6)

DCMI_D11, LCD_DE,

ADC3_IN8

-

22

H1

L1

28

G9

31

L1

PF10

I/O FT

EVENTOUT

PH0-OSC_IN

(6)

12

23

G2

G1

29

J11

32

G1

I/O FT

-

EVENTOUT

OSC_IN

(PH0)

PH1-

OSC_OUT

OSC_OUT

13

24

G1

H1

30

H10

33

H1

I/O FT

-

EVENTOUT

(6)

(PH1)

RS

14

25

H2

J1

31

H9

34

J1

NRST

I/O

-

-

-

T

OTG_HS_ULPI_STP,

ADC123_

(6)

FMC_SDNWE,

15

26

G6

M2

32

H8

35

M2

PC0

I/O FT

IN10

EVENTOUT

ETH_MDC,

ADC123_

(6)

16

27

H5

M3

33

K11

36

M3

PC1

I/O FT

EVENTOUT

IN11

SPI2_MISO,

I2S2ext_SD,

OTG_HS_ULPI_DIR,

ADC123_

(6)

17

28

H6

M4

34

J10

37

M4

PC2

I/O FT

ETH_MII_TXD2,

IN12

FMC_SDNE0,

EVENTOUT

SPI2_MOSI/I2S2_SD,

OTG_HS_ULPI_NXT,

ADC123_

(6)

18

29

H7

M5

35

J9

38

L4

PC3

I/O FT

ETH_MII_TX_CLK,

IN13

FMC_SDCKE0,

EVENTOUT

S

-

-

-

-

19

30

-

-

36

G7

39

J5

V

DD

56/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

-

-

-

-

-

-

-

J6

V

S

-

-

-

-

SS

S

-

-

-

-

20

31

J1

M1

37

K10

40

M1

V

SSA

S

-

-

-

-

-

-

J2

N1

-

-

-

N1

V

REF

–

21

32

J3

P1

38

L11

41

P1

V

S

-

-

-

-

REF+

S

-

-

-

-

22

33

J4

R1

39

L10

42

R1

V

DDA

TIM2_CH1/TIM2_ETR,

TIM5_CH1, TIM8_ETR,

ADC123_

PA0-WKUP

USART2_CTS,

(7)

23

34

J5

N3

40

K9

43

N3

I/O FT

IN0/WKUP

UART4_TX,

(PA0)

(6)

ETH_MII_CRS,

EVENTOUT

TIM2_CH2, TIM5_CH2,

USART2_RTS,

UART4_RX,

ADC123_

(6)

24

35

K1

N2

41

K8

44

N2

PA1

I/O FT

ETH_MII_RX_CLK/ET

IN1

H_RMII_REF_CLK,

EVENTOUT

TIM2_CH3, TIM5_CH3,

TIM9_CH1,

ADC123_

(6)

25

36

K2

P2

42

L9

45

P2

PA2

I/O FT

USART2_TX,

IN2

ETH_MDIO,

EVENTOUT

ETH_MII_CRS,

-

-

L2

F4

43

-

46

K4

PH2

I/O FT

-

FMC_SDCKE0,

-

LCD_R0, EVENTOUT

ETH_MII_COL,

-

-

L1

G4

44

-

47

J4

PH3

I/O FT

-

FMC_SDNE0,

-

LCD_R1, EVENTOUT

I2C2_SCL,

-

-

M2

H4

45

-

48

H4

PH4

I/O FT

-

OTG_HS_ULPI_NXT,

-

EVENTOUT

I2C2_SDA, SPI5_NSS,

-

-

L3

J4

46

-

49

J3

PH5

I/O FT

-

FMC_SDNWE,

-

EVENTOUT

DS9405 Rev 13

57/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TIM2_CH4, TIM5_CH4,

TIM9_CH2,

USART2_RX,

ADC123_

(6)

26

37

K3

R2

47

M11

50

R2

PA3

I/O FT

OTG_HS_ULPI_D0,

IN3

ETH_MII_COL,

LCD_B5, EVENTOUT

S

-

-

-

-

27

38

-

-

-

51

K6

V

SS

BYPASS_

-

-

M1

L4

48

N11

-

L5

I

FT

-

-

-

REG

S

-

-

-

-

28

39

J11

K4

49

J8

52

K5

V

DD

SPI1_NSS,

SPI3_NSS/I2S3_WS,

USART2_CK,

ADC12_

M1

(6)

29

40

N2

N4

50

53

N4

PA4

I/O TTa

OTG_HS_SOF,

IN4 /DAC_

0

DCMI_HSYNC,

OUT1

LCD_VSYNC,

EVENTOUT

TIM2_CH1/TIM2_ETR,

TIM8_CH1N,

ADC12_

(6)

SPI1_SCK,

IN5/DAC_

30

41

M3

P4

51

M9

54

P4

PA5

I/O TTa

OTG_HS_ULPI_CK,

OUT2

EVENTOUT

TIM1_BKIN,

TIM3_CH1,

TIM8_BKIN,

ADC12_

(6)

31

42

N3

P3

52

N10

55

P3

PA6

I/O FT

SPI1_MISO,

IN6

TIM13_CH1,

DCMI_PIXCLK,

LCD_G2, EVENTOUT

TIM1_CH1N,

TIM3_CH2,

TIM8_CH1N,

SPI1_MOSI,

ADC12_

(6)

32

43

K4

R3

53

L8

56

R3

PA7

I/O FT

TIM14_CH1,

IN7

ETH_MII_RX_DV/ETH

_RMII_CRS_DV,

EVENTOUT

58/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

ETH_MII_RXD0/ETH_

ADC12_

(6)

33

44

L4

N5

54

M8

57

N5

PC4

I/O FT

RMII_RXD0,

IN14

EVENTOUT

ETH_MII_RXD1/ETH_

ADC12_

(6)

RMII_RXD1,

34

45

M4

P5

55

N9

58

P5

PC5

I/O FT

IN15

EVENTOUT

S

-

-

-

-

-

-

-

-

-

J7

59

L7

V

DD

-

-

-

-

-

-

60

L6

VSS

S

-

-

-

-

TIM1_CH2N,

TIM3_CH3,

TIM8_CH2N, LCD_R3,

ADC12_

(6)

35

46

N4

R5

56

N8

61

R5

PB0

I/O FT

OTG_HS_ULPI_D1,

IN8

ETH_MII_RXD2,

EVENTOUT

TIM1_CH3N,

TIM3_CH4,

TIM8_CH3N, LCD_R6,

ADC12_

(6)

36

47

K5

R4

57

K7

62

R4

PB1

I/O FT

OTG_HS_ULPI_D2,

IN9

ETH_MII_RXD3,

EVENTOUT

PB2-BOOT1

37

48

L5

M6

58

L7

63

M5

I/O FT

-

EVENTOUT

-

(PB2)

-

-

-

-

-

-

64

G4

PI15

I/O FT

-

LCD_R0, EVENTOUT

-

-

-

-

-

-

-

65

R6

PJ0

I/O FT

-

LCD_R1, EVENTOUT

-

-

-

-

-

-

-

66

R7

PJ1

I/O FT

-

LCD_R2, EVENTOUT

-

-

-

-

-

-

-

67

P7

PJ2

I/O FT

-

LCD_R3, EVENTOUT

-

-

-

-

-

-

-

68

N8

PJ3

I/O FT

-

LCD_R4, EVENTOUT

-

-

-

-

-

-

-

69

M9

PJ4

I/O FT

-

LCD_R5, EVENTOUT

-

SPI5_MOSI,

FMC_SDNRAS,

-

49

M5

R6

59

M7

70

P8

PF11

I/O FT

-

-

DCMI_D12,

EVENTOUT

-

50

N5

P6

60

N7

71

M6

PF12

I/O FT

-

FMC_A6, EVENTOUT

-

S

-

-

-

-

51

G9

M8

61

-

72

K7

V

SS

DS9405 Rev 13

59/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

-

52

D10

N8

62

-

73

L8

V

S

-

-

-

DD

-

53

M6

N6

63

K6

74

N6

PF13

I/O FT

-

FMC_A7, EVENTOUT

-

-

54

K7

R7

64

L6

75

P6

PF14

I/O FT

-

FMC_A8, EVENTOUT

-

-

55

L7

P7

65

M6

76

M8

PF15

I/O FT

-

FMC_A9, EVENTOUT

-

-

56

N6

N7

66

N6

77

N7

PG0

I/O FT

-

FMC_A10, EVENTOUT

-

-

57

M7

M7

67

K5

78

M7

PG1

I/O FT

-

FMC_A11, EVENTOUT

-

TIM1_ETR,

38

58

N7

R8

68

L5

79

R8

PE7

I/O FT

-

UART7_Rx, FMC_D4,

-

EVENTOUT

TIM1_CH1N,

39

59

J8

P8

69

M5

80

N9

PE8

I/O FT

-

UART7_Tx, FMC_D5,

-

EVENTOUT

TIM1_CH1, FMC_D6,

40

60

K8

P9

70

N5

81

P9

PE9

I/O FT

-

-

EVENTOUT

S

-

-

-

-

61

J6

M9

71

H3

82

K8

V

SS

S

-

-

-

-

62

G10

N9

72

J5

83

L9

V

DD

TIM1_CH2N, FMC_D7,

41

63

L8

R9

73

J4

84

R9

PE10

I/O FT

-

-

EVENTOUT

TIM1_CH2, SPI4_NSS,

42

64

M8

P10

74

K4

85

P10

PE11

I/O FT

-

FMC_D8, LCD_G3,

-

EVENTOUT

TIM1_CH3N,

43

65

N8

R10

75

L4

86

R10

PE12

I/O FT

-

SPI4_SCK, FMC_D9,

-

LCD_B4, EVENTOUT

TIM1_CH3,

SPI4_MISO,

44

66

H9

N11

76

N4

87

R12

PE13

I/O FT

-

-

FMC_D10, LCD_DE,

EVENTOUT

TIM1_CH4,

45

67

J9

P11

77

M4

88

P11

PE14

I/O FT

-

SPI4_MOSI, FMC_D11,

-

LCD_CLK, EVENTOUT

TIM1_BKIN, FMC_D12,

46

68

K9

R11

78

L3

89

R11

PE15

I/O FT

-

-

LCD_R7, EVENTOUT

60/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TIM2_CH3, I2C2_SCL,

SPI2_SCK/I2S2_CK,

USART3_TX,

47

69

L9

R12

79

M3

90

P12

PB10

I/O FT

-

-

OTG_HS_ULPI_D3,

ETH_MII_RX_ER,

LCD_G4, EVENTOUT

TIM2_CH4, I2C2_SDA,

USART3_RX,

OTG_HS_ULPI_D4,

48

70

M9

R13

80

N3

91

R13

PB11

I/O FT

-

-

ETH_MII_TX_EN/ETH_

RMII_TX_EN, LCD_G5,

EVENTOUT

S

-

-

-

-

49

71

N9

M10

81

N2

92

L11

V

CAP_1

S

-

-

-

-

-

-

-

-

-

H2

93

K9

V

SS

50

72

F8

N10

82

J6

94

L10

V

S

-

-

-

-

DD

-

-

-

-

-

-

95

M14

PJ5

I/O

-

-

LCD_R6, EVENTOUT

-

I2C2_SMBA,

SPI5_SCK,

TIM12_CH1,

-

-

N10

M11

83

-

96

P13

PH6

I/O FT

-

-

ETH_MII_RXD2,

FMC_SDNE1,

DCMI_D8, EVENTOUT

I2C3_SCL,

SPI5_MISO,

-

-

M10

N12

84

-

97

N13

PH7

I/O FT

-

ETH_MII_RXD3,

-

FMC_SDCKE1,

DCMI_D9, EVENTOUT

I2C3_SDA, FMC_D16,

-

-

L10

M12

85

-

98

P14

PH8

I/O FT

-

DCMI_HSYNC,

-

LCD_R2, EVENTOUT

I2C3_SMBA,

TIM12_CH2,

-

-

K10

M13

86

-

99

N14

PH9

I/O FT

-

-

FMC_D17, DCMI_D0,

LCD_R3, EVENTOUT

TIM5_CH1, FMC_D18,

-

-

N11

L13

87

-

100

P15

PH10

I/O FT

-

DCMI_D1, LCD_R4,

-

EVENTOUT

DS9405 Rev 13

61/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TIM5_CH2, FMC_D19,

-

-

M11

L12

88

-

101

N15

PH11

I/O FT

-

DCMI_D2, LCD_R5,

-

EVENTOUT

TIM5_CH3, FMC_D20,

-

-

L11

K12

89

-

102

M15

PH12

I/O FT

-

DCMI_D3, LCD_R6,

-

EVENTOUT

-

-

E7

H12

90

-

-

K10

V

S

-

-

-

-

SS

S

-

-

-

-

-

-

H8

J12

91

-

103

K11

V

DD

TIM1_BKIN,

I2C2_SMBA,

SPI2_NSS/I2S2_WS,

USART3_CK,

CAN2_RX,

51

73

N12

P12

92

M2

104

L13

PB12

I/O FT

-

-

OTG_HS_ULPI_D5,

ETH_MII_TXD0/ETH_

RMII_TXD0,

OTG_HS_ID,

EVENTOUT

TIM1_CH1N,

SPI2_SCK/I2S2_CK,

USART3_CTS,

CAN2_TX,

OTG_HS_

52

74

M12

P13

93

N1

105

K14

PB13

I/O FT

-

OTG_HS_ULPI_D6,

VBUS

ETH_MII_TXD1/ETH_

RMII_TXD1,

EVENTOUT

TIM1_CH2N,

TIM8_CH2N,

SPI2_MISO,

I2S2ext_SD,

53

75

M13

R14

94

K3

106

R14

PB14

I/O FT

-

-

USART3_RTS,

TIM12_CH1,

OTG_HS_DM,

EVENTOUT

62/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

RTC_REFIN,

TIM1_CH3N,

TIM8_CH3N,

SPI2_MOSI/I2S2_SD,

54

76

L13

R15

95

J3

107

R15

PB15

I/O FT

-

-

TIM12_CH2,

OTG_HS_DP,

EVENTOUT

USART3_TX,

55

77

L12

P15

96

L2

108

L15

PD8

I/O FT

-

-

FMC_D13, EVENTOUT

USART3_RX,

56

78

K13

P14

97

M1

109

L14

PD9

I/O FT

-

-

FMC_D14, EVENTOUT

USART3_CK,

57

79

K11

N15

98

H4

110

K15

PD10

I/O FT

-

FMC_D15, LCD_B3,

-

EVENTOUT

USART3_CTS,

58

80

H10

N14

99

K2

111

N10

PD11

I/O FT

-

-

FMC_A16, EVENTOUT

TIM4_CH1,

59

81

J13

N13

100

H6

112

M10

PD12

I/O FT

-

USART3_RTS,

-

FMC_A17, EVENTOUT

TIM4_CH2, FMC_A18,

60

82

K12

M15

101

H5

113

M11

PD13

I/O FT

-

-

EVENTOUT

S

-

-

-

-

83

-

-

102

-

114

J10

V

SS

-

84

F7

J13

103

L1

115

J11

V

S

-

-

-

DD

TIM4_CH3, FMC_D0,

61

85

H11

M14

104

J2

116

L12

PD14

I/O FT

-

-

EVENTOUT

TIM4_CH4, FMC_D1,

62

86

J12

L14

105

K1

117

K13

PD15

I/O FT

-

-

EVENTOUT

-

-

-

-

-

-

118

K12

PJ6

I/O FT

-

LCD_R7, EVENTOUT

-

-

-

-

-

-

-

119

J12

PJ7

I/O FT

-

LCD_G0, EVENTOUT

-

-

-

-

-

-

-

120

H12

PJ8

I/O FT

-

LCD_G1, EVENTOUT

-

-

-

-

-

-

-

121

J13

PJ9

I/O FT

-

LCD_G2, EVENTOUT

-

-

-

-

-

-

-

122

H13

PJ10

I/O FT

-

LCD_G3, EVENTOUT

-

-

-

-

-

-

-

123

G12

PJ11

I/O FT

-

LCD_G4, EVENTOUT

-

DS9405 Rev 13

63/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

-

-

-

-

-

-

124

H11

VDD

I/O FT

-

-

-

-

-

-

-

-

-

125

H10

VSS

I/O FT

-

-

-

-

-

-

-

-

-

126

G13

PK0

I/O FT

-

LCD_G5, EVENTOUT

-

-

-

-

-

-

-

127

F12

PK1

I/O FT

-

LCD_G6, EVENTOUT

-

-

-

-

-

-

-

128

F13

PK2

I/O FT

-

LCD_G7, EVENTOUT

-

-

87

H13

L15

106

J1

129

M13

PG2

I/O FT

-

FMC_A12, EVENTOUT

-

NC

K15

107

G3

130

M12

PG3

I/O FT

-

FMC_A13, EVENTOUT

-

-

88

(3)

FMC_A14/FMC_BA0,

-

89

H12

K14

108

G5

131

N12

PG4

I/O FT

-

-

EVENTOUT

FMC_A15/FMC_BA1,

-

90

G13

K13

109

G6

132

N11

PG5

I/O FT

-

-

EVENTOUT

FMC_INT2,

-

91

G11

J15

110

G4

133

J15

PG6

I/O FT

-

DCMI_D12, LCD_R7,

-

EVENTOUT

USART6_CK,

FMC_INT3,

-

92

G12

J14

111

H1

134

J14

PG7

I/O FT

-

-

DCMI_D13, LCD_CLK,

EVENTOUT

SPI6_NSS,

USART6_RTS,

-

93

F13

H14

112

G2

135

H14

PG8

I/O FT

-

ETH_PPS_OUT,

-

FMC_SDCLK,

EVENTOUT

S

-

-

-

-

94

J7

G12

113

D2

136

G10

V

SS

S

-

-

-

-

95

E6

H13

114

G1

137

G11

V

DD

TIM3_CH1, TIM8_CH1,

I2S2_MCK,

USART6_TX,

63

96

F9

H15

115

F2

138

H15

PC6

I/O FT

-

-

SDIO_D6, DCMI_D0,

LCD_HSYNC,

EVENTOUT

64/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TIM3_CH2, TIM8_CH2,

I2S3_MCK,

64

97

F10

G15

116

F3

139

G15

PC7

I/O FT

-

USART6_RX,

-

SDIO_D7, DCMI_D1,

LCD_G6, EVENTOUT

TIM3_CH3, TIM8_CH3,

USART6_CK,

65

98

F11

G14

117

E4

140

G14

PC8

I/O FT

-

-

SDIO_D0, DCMI_D2,

EVENTOUT

MCO2, TIM3_CH4,

TIM8_CH4, I2C3_SDA,

66

99

F12

F14

118

E3

141

F14

PC9

I/O FT

-

-

I2S_CKIN, SDIO_D1,

DCMI_D3, EVENTOUT

MCO1, TIM1_CH1,

I2C3_SCL,

67

100

E13

F15

119

F1

142

F15

PA8

I/O FT

-

USART1_CK,

-

OTG_FS_SOF,

LCD_R6, EVENTOUT

TIM1_CH2,

I2C3_SMBA,

OTG_FS_

68

101

E8

E15

120

E2

143

E15

PA9

I/O FT

-

USART1_TX,

VBUS

DCMI_D0, EVENTOUT

TIM1_CH3,

USART1_RX,

69

102

E9

D15

121

D5

144

D15

PA10

I/O FT

-

-

OTG_FS_ID,

DCMI_D1, EVENTOUT

TIM1_CH4,

USART1_CTS,

70

103

E10

C15

122

D4

145

C15

PA11

I/O FT

-

CAN1_RX, LCD_R4,

-

OTG_FS_DM,

EVENTOUT

TIM1_ETR,

USART1_RTS,

71

104

E11

B15

123

E1

146

B15

PA12

I/O FT

-

CAN1_TX, LCD_R5,

-

OTG_FS_DP,

EVENTOUT

DS9405 Rev 13

65/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

PA13

JTMS-SWDIO,

72

105

E12

A15

124

D3

147

A15

I/O FT

-

-

(JTMS-

EVENTOUT

SWDIO)

S

-

-

-

73

106

D12

F13

125

D1

148

E11

V

CAP_2

74

107

J10

F12

126

D2

149

F10

V

S

-

-

-

SS

S

-

-

-

75

108

H4

G13

127

C1

150

F11

V

DD

TIM8_CH1N,

-

-

D13

E12

128

-

151

E12

PH13

I/O FT

-

CAN1_TX, FMC_D21,

-

LCD_G2, EVENTOUT

TIM8_CH2N,

-

-

C13

E13

129

-

152

E13

PH14

I/O FT

-

FMC_D22, DCMI_D4,

-

LCD_G3, EVENTOUT

TIM8_CH3N,

-

-

C12

D13

130

-

153

D13

PH15

I/O FT

-

FMC_D23, DCMI_D11,

-

LCD_G4, EVENTOUT

TIM5_CH4,

(8)

SPI2_NSS/I2S2_WS

,

-

-

B13

E14

131

-

154

E14

PI0

I/O FT

-

-

FMC_D24, DCMI_D13,

LCD_G5, EVENTOUT

(8)

,

SPI2_SCK/I2S2_CK

FMC_D25, DCMI_D8,

-

-

-

C11

D14

132

-

155

D14

PI1

I/O FT

-

LCD_G6, EVENTOUT

TIM8_CH4,

SPI2_MISO,

-

-

B12

C14

133

-

156

C14

PI2

I/O FT

-

I2S2ext_SD,

-

FMC_D26, DCMI_D9,

LCD_G7, EVENTOUT

TIM8_ETR,

SPI2_MOSI/I2S2_SD,

-

-

A12

C13

134

-

157

C13

PI3

I/O FT

-

-

FMC_D27, DCMI_D10,

EVENTOUT

S

-

-

-

-

-

D11

D9

135

F5

-

F9

V

SS

S

-

-

-

-

-

D3

C9

136

A1

158

E10

V

DD

66/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

PA14

JTCK-SWCLK/

76

109

A11

A14

137

B1

159

A14

I/O FT

-

-

(JTCK-

EVENTOUT

SWCLK)

JTDI,

TIM2_CH1/TIM2_ETR,

PA15

77

110

B11

A13

138

C2

160

A13

I/O FT

-

SPI1_NSS,

-

(JTDI)

SPI3_NSS/I2S3_WS,

EVENTOUT

SPI3_SCK/I2S3_CK,

USART3_TX,

78

111

C10

B14

139

A2

161

B14

PC10

I/O FT

-

UART4_TX, SDIO_D2,

-

DCMI_D8, LCD_R2,

EVENTOUT

I2S3ext_SD,

SPI3_MISO,

79

112

B10

B13

140

B2

162

B13

PC11

I/O FT

-

USART3_RX,

-

UART4_RX, SDIO_D3,

DCMI_D4, EVENTOUT

SPI3_MOSI/I2S3_SD,

USART3_CK,

80

113

A10

A12

141

C3

163

A12

PC12

I/O FT

-

-

UART5_TX, SDIO_CK,

DCMI_D9, EVENTOUT

CAN1_RX, FMC_D2,

81

114

D9

B12

142

B3

164

B12

PD0

I/O FT

-

-

EVENTOUT

CAN1_TX, FMC_D3,

82

115

C9

C12

143

C4

165

C12

PD1

I/O FT

-

-

EVENTOUT

TIM3_ETR,

UART5_RX,

83

116

B9

D12

144

A3

166

D12

PD2

I/O FT

-

SDIO_CMD,

-

DCMI_D11,

EVENTOUT

SPI2_SCK/I2S2_CK,

USART2_CTS,

84

117

A9

D11

145

B4

167

C11

PD3

I/O FT

-

-

FMC_CLK, DCMI_D5,

LCD_G7, EVENTOUT

DS9405 Rev 13

67/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

USART2_RTS,

85

118

D8

D10

146

B5

168

D11

PD4

I/O FT

-

FMC_NOE,

-

EVENTOUT

USART2_TX,

86

119

C8

C11

147

A4

169

C10

PD5

I/O FT

-

FMC_NWE,

-

EVENTOUT

-

120

-

D8

148

-

170

F8

V

S

-

-

-

SS

S

-

-

-

-

121

D6

C8

149

C5

171

E9

V

DD

SPI3_MOSI/I2S3_SD,

SAI1_SD_A,

USART2_RX,

87

122

B8

B11

150

F4

172

B11

PD6

I/O FT

-

-

FMC_NWAIT,

DCMI_D10, LCD_B2,

EVENTOUT

USART2_CK,

88

123

A8

A11

151

A5

173

A11

PD7

I/O FT

-

FMC_NE1/FMC_NCE2

-

, EVENTOUT

-

-

-

-

-

-

174

B10

PJ12

I/O FT

-

LCD_B0, EVENTOUT

-

-

-

-

-

-

-

175

B9

PJ13

I/O FT

-

LCD_B1, EVENTOUT

-

-

-

-

-

-

-

176

C9

PJ14

I/O FT

-

LCD_B2, EVENTOUT

-

-

-

-

-

-

-

177

D10

PJ15

I/O FT

-

LCD_B3, EVENTOUT

-

USART6_RX,

FMC_NE2/FMC_NCE3

NC

C10

152

E5

178

D9

PG9

I/O FT

-

-

-

124

(3)

(9)

,

, DCMI_VSYNC

EVENTOUT

LCD_G3,

FMC_NCE4_1/FMC_N

-

125

C7

B10

153

C6

179

C8

PG10

I/O FT

-

-

E3, DCMI_D2,

LCD_B2, EVENTOUT

ETH_MII_TX_EN/ETH_

RMII_TX_EN,

-

126

B7

B9

154

B6

180

B8

PG11

I/O FT

-

FMC_NCE4_2,

-

DCMI_D3, LCD_B3,

EVENTOUT

68/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

SPI6_MISO,

USART6_RTS,

-

127

A7

B8

155

A6

181

C7

PG12

I/O FT

-

-

LCD_B4, FMC_NE4,

LCD_B1, EVENTOUT

SPI6_SCK,

USART6_CTS,

NC

A8

156

D6

182

B3

PG13

I/O FT

-

ETH_MII_TXD0/ETH_

-

-

128

(3)

RMII_TXD0, FMC_A24,

EVENTOUT

SPI6_MOSI,

USART6_TX,

NC

-

129

A7

157

F6

183

A4

PG14

I/O FT

-

ETH_MII_TXD1/ETH_

-

(3)

RMII_TXD1, FMC_A25,

EVENTOUT

-

130

D7

D7

158

-

184

F7

V

S

-

-

-

SS

S

-

-

-

-

131

L6

C7

159

E6

185

E8

V

DD

-

-

-

-

-

-

186

D8

PK3

I/O FT

-

LCD_B4, EVENTOUT

-

-

-

-

-

-

-

187

D7

PK4

I/O FT

-

LCD_B5, EVENTOUT

-

-

-

-

-

-

-

188

C6

PK5

I/O FT

-

LCD_B6, EVENTOUT

-

-

-

-

-

-

-

189

C5

PK6

I/O FT

-

LCD_B7, EVENTOUT

-

-

-

-

-

-

-

190

C4

PK7

I/O FT

-

LCD_DE, EVENTOUT

-

USART6_CTS,

FMC_SDNCAS,

-

132

C6

B7

160

A7

191

B7

PG15

I/O FT

-

-

DCMI_D13,

EVENTOUT

JTDO/TRACESWO,

PB3

TIM2_CH2, SPI1_SCK,

89

133

B6

A10

161

B7

192

A10

I/O FT

-

-

(JTDO/TRACE

SPI3_SCK/I2S3_CK,

SWO)

EVENTOUT

NJTRST, TIM3_CH1,

SPI1_MISO,

PB4

90

134

A6

A9

162

C7

193

A9

I/O FT

-

SPI3_MISO,

-

(NJTRST)

I2S3ext_SD,

EVENTOUT

DS9405 Rev 13

69/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

TIM3_CH2,

I2C1_SMBA,

SPI1_MOSI,

SPI3_MOSI/I2S3_SD,

CAN2_RX,

91

135

D5

A6

163

C8

194

A8

PB5

I/O FT

-

-

OTG_HS_ULPI_D7,

ETH_PPS_OUT,

FMC_SDCKE1,

DCMI_D10,

EVENTOUT

TIM4_CH1, I2C1_SCL,

USART1_TX,

92

136

C5

B6

164

A8

195

B6

PB6

I/O FT

-

CAN2_TX,

-

FMC_SDNE1,

DCMI_D5, EVENTOUT

TIM4_CH2, I2C1_SDA,

USART1_RX,

93

137

B5

B5

165

B8

196

B5

PB7

I/O FT

-

FMC_NL,

-

DCMI_VSYNC,

EVENTOUT

94

138

A5

D6

166

C9

197

E6

BOOT0

I

B

-

-

V

PP

TIM4_CH3,

TIM10_CH1,

I2C1_SCL, CAN1_RX,

95

139

D4

A5

167

A9

198

A7

PB8

I/O FT

-

-

ETH_MII_TXD3,

SDIO_D4, DCMI_D6,

LCD_B6, EVENTOUT

TIM4_CH4,

TIM11_CH1,

I2C1_SDA,

96

140

C4

B4

168

B9

199

B4

PB9

I/O FT

-

SPI2_NSS/I2S2_WS,

-

CAN1_TX, SDIO_D5,

DCMI_D7, LCD_B7,

EVENTOUT

TIM4_ETR,

UART8_RX,

97

141

B4

A4

169

B10

200

A6

PE0

I/O FT

-

-

FMC_NBL0, DCMI_D2,

EVENTOUT

70/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 10. STM32F427xx and STM32F429xx pin and ball definitions (continued)

Pin number

Pin name

Additional

(2)

Alternate functions

(function

functions

(1)

after reset)

Notes

Pin type

I / O structure

LQFP100

LQFP144

LQFP176

LQFP208

TFBGA216

UFBGA169

WLCSP143

UFBGA176

UART8_Tx,

98

142

A4

A3

170

A10

201

A5

PE1

I/O FT

-

FMC_NBL1, DCMI_D3,

-

EVENTOUT

99

-

F5

D5

-

-

202

F6

V

S

-

-

-

SS

-

143

C3

C6

171

A11

203

E5

PDR_ON

S

-

-

-

100

144

K6

C5

172

D7

204

E7

V

S

-

-

-

DD

TIM8_BKIN,

-

-

B3

D4

173

-

205

C3

PI4

I/O FT

-

FMC_NBL2, DCMI_D5,

-

LCD_B4, EVENTOUT

TIM8_CH1,

FMC_NBL3,

-

-

A3

C4

174

-

206

D3

PI5

I/O FT

-

-

DCMI_VSYNC,

LCD_B5, EVENTOUT

TIM8_CH2, FMC_D28,

-

-

A2

C3

175

-

207

D6

PI6

I/O FT

-

DCMI_D6, LCD_B6,

-

EVENTOUT

TIM8_CH3, FMC_D29,

-

-

B1

C2

176

-

208

D4

PI7

I/O FT

-

DCMI_D7, LCD_B7,

-

EVENTOUT

1.

Function availability depends on the chosen device.

2.

On the UFBGA176 package, the balls F6, F7, F8, F9, F10, G6, G7, G8, G9, G10, H6, H7, H8, H9, H10, J6, J7, J8, J9, J10,

K6, K7, K8, K9, and K10 are connected to VSS. Their purpose is heat dissipation and package mechanical stability

3.

NC (not-connected) pins are not bonded. They must be configured by software to output push-pull and forced to 0 in the

output data register to avoid extra current consumption in low-power modes.

4.

PC13, PC14, PC15, and PI8 are supplied through the power switch. Since the switch only sinks a limited amount of current

(3 mA), the use of GPIOs PC13 to PC15 and PI8 in output mode is limited:

- The speed should not exceed 2 MHz with a maximum load of 30 pF.

- These I/Os must not be used as a current source (for example, to drive an LED).

5.

The main function after the first backup domain power-up. Later on, it depends on the contents of the RTC registers even

after reset (because these registers are not reset by the main reset). For details on how to manage these I/Os, refer to the

RTC register description sections in the STM32F4xx reference manual, available from the STMicroelectronics website:

www.st.com.

6.

FT = 5 V tolerant except when in analog mode or oscillator mode (for PC14, PC15, PH0, and PH1).

7.

If the device is delivered in a WLCSP143, UFBGA169, UFBGA176, LQFP176 or TFBGA216 package, and the

(Regulator OFF/internal reset ON mode), then PA0 is used as an internal Reset (active

BYPASS_REG pin is set to V

DD

low).

8.

PI0 and PI1 cannot be used for I2S2 full-duplex mode.

9.

The DCMI_VSYNC alternate function on PG9 is only available on silicon revision 3.

DS9405 Rev 13

71/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 11. FMC pin definition

NOR/PSRAM/

NOR/PSRAM

Pin name

CF

NAND16

SDRAM

SRAM

Mux

PF0

A0

A0

-

-

A0

PF1

A1

A1

-

-

A1

PF2

A2

A2

-

-

A2

PF3

A3

A3

-

-

A3

PF4

A4

A4

-

-

A4

PF5

A5

A5

-

-

A5

PF12

A6

A6

-

-

A6

PF13

A7

A7

-

-

A7

PF14

A8

A8

-

-

A8

PF15

A9

A9

-

-

A9

PG0

A10

A10

-

-

A10

PG1

-

A11

-

-

A11

PG2

-

A12

-

-

A12

PG3

-

A13

-

-

-

PG4

-

A14

-

-

BA0

PG5

-

A15

-

-

BA1

PD11

-

A16

A16

CLE

-

PD12

-

A17

A17

ALE

-

PD13

-

A18

A18

-

-

PE3

-

A19

A19

-

-

PE4

-

A20

A20

-

-

PE5

-

A21

A21

-

-

PE6

-

A22

A22

-

-

PE2

-

A23

A23

-

-

PG13

-

A24

A24

-

-

PG14

-

A25

A25

-

-

PD14

D0

D0

DA0

D0

D0

PD15

D1

D1

DA1

D1

D1

PD0

D2

D2

DA2

D2

D2

PD1

D3

D3

DA3

D3

D3

PE7

D4

D4

DA4

D4

D4

PE8

D5

D5

DA5

D5

D5

PE9

D6

D6

DA6

D6

D6

PE10

D7

D7

DA7

D7

D7

72/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Pinouts and pin description

Table 11. FMC pin definition (continued)

NOR/PSRAM/

NOR/PSRAM

Pin name

CF

NAND16

SDRAM

SRAM

Mux

PE11

D8

D8

DA8

D8

D8

PE12

D9

D9

DA9

D9

D9

PE13

D10

D10

DA10

D10

D10

PE14

D11

D11

DA11

D11

D11

PE15

D12

D12

DA12

D12

D12

PD8

D13

D13

DA13

D13

D13

PD9

D14

D14

DA14

D14

D14

PD10

D15

D15

DA15

D15

D15

PH8

-

D16

-

-

D16

PH9

-

D17

-

-

D17

PH10

-

D18

-

-

D18

PH11

-

D19

-

-

D19

PH12

-

D20

-

-

D20

PH13

-

D21

-

-

D21

PH14

-

D22

-

-

D22

PH15

-

D23

-

-

D23

PI0

-

D24

-

-

D24

PI1

-

D25

-

-

D25

PI2

-

D26

-

-

D26

PI3

-

D27

-

-

D27

PI6

-

D28

-

-

D28

PI7

-

D29

-

-

D29

PI9

-

D30

-

-

D30

PI10

-

D31

-

-

D31

PD7

-

NE1

NE1

NCE2

-

PG9

-

NE2

NE2

NCE3

-

PG10

NCE4_1

NE3

NE3

-

-

PG11

NCE4_2

-

-

-

-

PG12

-

NE4

NE4

-

-

PD3

-

CLK

CLK

-

-

PD4

NOE

NOE

NOE

NOE

-

PD5

NWE

NWE

NWE

NWE

-

PD6

NWAIT

NWAIT

NWAIT

NWAIT

-

PB7

-

NL(NADV)

NL(NADV)

-

-

DS9405 Rev 13

73/240

85

Pinouts and pin description

STM32F427xx STM32F429xx

Table 11. FMC pin definition (continued)

NOR/PSRAM/

NOR/PSRAM

Pin name

CF

NAND16

SDRAM

SRAM

Mux

PF6

NIORD

-

-

-

-

PF7

NREG

-

-

-

-

PF8

NIOWR

-

-

-

-

PF9

CD

-

-

-

-

PF10

INTR

-

-

-

-

PG6

-

-

-

INT2

-

PG7

-

-

-

INT3

-

PE0

-

NBL0

NBL0

-

NBL0

PE1

-

NBL1

NBL1

-

NBL1

PI4

-

NBL2

-

-

NBL2

PI5

-

NBL3

-

-

NBL3

PG8

-

-

-

-

SDCLK

PC0

-

-

-

-

SDNWE

PF11

-

-

-

-

SDNRAS

PG15

-

-

-

-

SDNCAS

PH2

-

-

-

-

SDCKE0

PH3

-

-

-

-

SDNE0

PH6

-

-

-

-

SDNE1

PH7

-

-

-

-

SDCKE1

PH5

-

-

-

-

SDNWE

PC2

-

-

-

-

SDNE0

PC3

-

-

-

-

SDCKE0

PB5

-

-

-

-

SDCKE1

PB6

-

-

-

-

SDNE1

74/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Table 12. STM32F427xx and STM32F429xx alternate function mapping

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

TIM2_

TIM5_

TIM8_

USART2_

ETH_MII_

EVEN

PA0

-

CH1/TIM2

-

-

-

UART4_TX

-

-

-

-

-

CH1

ETR

CTS

CRS

TOUT

_ETR

ETH_MII_

TIM2_

TIM5_

USART2_

RX_CLK/E

EVEN

PA1

-

-

-

-

-

UART4_RX

-

-

-

-

-

CH2

CH2

RTS

TH_RMII_

TOUT

REF_CLK

TIM2_

TIM5_

TIM9_

USART2_

ETH_

EVEN

PA2

-

-

-

-

-

-

-

-

-

-

CH3

CH3

CH1

TX

MDIO

TOUT

TIM2_

TIM5_

TIM9_

USART2_

OTG_HS_

ETH_MII_

EVEN

PA3

-

-

-

-

-

-

-

-

LCD_B5

CH4

CH4

CH2

RX

ULPI_D0

COL

TOUT

SPI3_

OTG_HS_

DCMI_

LCD_

EVEN

USART2_

SPI1_

DS9405 Rev 13

-

-

-

-

PA4

-

-

-

-

-

NSS/

SOF

HSYNC

VSYNC

TOUT

CK

NSS

I2S3_WS

TIM2_

TIM8_

SPI1_

OTG_HS_

EVEN

PA5

-

CH1/TIM2

-

-

-

-

-

-

-

-

-

-

CH1N

SCK

ULPI_CK

TOUT

_ETR

Port A

TIM1_

TIM3_

TIM8_

SPI1_

DCMI_

EVEN

PA6

-

-

-

-

-

TIM13_CH1

-

-

-

LCD_G2

BKIN

CH1

BKIN

MISO

PIXCLK

TOUT

ETH_MII_

SPI1_

RX_DV/

EVEN

TIM1_

TIM3_

TIM8_

-

-

-

TIM14_CH1

-

-

-

-

-

PA7

-

MOSI

ETH_RMII

TOUT

CH1N

CH2

CH1N

_CRS_DV

TIM1_

I2C3_

USART1_

OTG_FS_

EVEN

PA8

MCO1

-

-

-

-

-

-

-

-

-

LCD_R6

CH1

SCL

CK

SOF

TOUT

Pinouts and pin description

EVEN

DCMI_

USART1_

I2C3_

TIM1_

-

-

-

-

-

-

-

-

-

-

PA9

-

TOUT

D0

TX

SMBA

CH2

TIM1_

USART1_

OTG_FS_

DCMI_

EVEN

PA10

-

-

-

-

-

-

-

-

-

-

-

CH3

RX

ID

D1

TOUT

EVEN

OTG_FS_

USART1_

TIM1_

-

-

-

LCD_R4

-

CAN1_RX

-

-

-

-

-

PA11

-

TOUT

DM

CTS

CH4

EVEN

OTG_FS_

USART1_

TIM1_

-

-

-

LCD_R5

-

CAN1_TX

-

-

-

-

-

PA12

-

TOUT

DP

RTS

ETR

75/240

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

76/240

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

JTMS-

EVEN

PA13

SWDI

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

O

JTCK-

EVEN

Port A

PA14

SWCL

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

K

TIM2_

SPI3_

SPI1_

EVEN

PA15

JTDI

CH1/TIM2

-

-

-

NSS/

-

-

-

-

-

-

-

-

NSS

TOUT

_ETR

I2S3_WS

TIM1_

TIM3_

TIM8_

OTG_HS_

ETH_MII_

EVEN

PB0

-

-

-

-

-

-

LCD_R3

-

-

-

CH2N

CH3

CH2N

ULPI_D1

RXD2

TOUT

TIM1_

TIM3_

TIM8_

OTG_HS_

ETH_MII_

EVEN

PB1

-

-

-

-

-

-

LCD_R6

-

-

-

CH3N

CH4

CH3N

ULPI_D2

RXD3

TOUT

DS9405 Rev 13

EVEN

PB2

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

JTDO/

SPI3_

TIM2_

SPI1_

EVEN

PB3

TRAC

-

-

-

SCK/

-

-

-

-

-

-

-

-

CH2

SCK

TOUT

ESWO

I2S3_CK

NJTR

TIM3_

SPI1_

SPI3_

I2S3ext_

EVEN

PB4

-

-

-

-

-

-

-

-

-

-

ST

CH1

MISO

MISO

SD

TOUT

SPI3_

EVEN

OTG_HS_

ETH_PPS

FMC_

DCMI_

I2C1_

SPI1_

TIM3_

-

MOSI/

-

-

CAN2_RX

-

PB5

-

-

TOUT

ULPI_D7

_OUT

SDCKE1

D10

SMBA

MOSI

CH2

Port B

I2S3_SD

TIM4_

I2C1_

USART1_

FMC_

DCMI_

EVEN

PB6

-

-

-

-

-

-

CAN2_TX

-

-

-

CH1

SCL

TX

SDNE1

D5

TOUT

STM32F427xx STM32F429xx

EVEN

DCMI_

USART1_

I2C1_

TIM4_

-

-

-

-

-

FMC_NL

-

-

-

PB7

-

-

TOUT

VSYNC

RX

SDA

CH2

TIM4_

TIM10_

I2C1_

ETH_MII_

DCMI_

EVEN

PB8

-

-

-

-

-

-

CAN1_RX

-

SDIO_D4

LCD_B6

CH3

CH1

SCL

TXD3

D6

TOUT

SPI2_

TIM4_

TIM11_

I2C1_

DCMI_

EVEN

PB9

-

-

NSS/I2

-

-

-

CAN1_TX

-

-

SDIO_D5

LCD_B7

CH4

CH1

SDA

D7

TOUT

S2_WS

SPI2_

TIM2_

I2C2_

USART3_

OTG_HS_

ETH_MII_

EVEN

PB10

-

-

-

SCK/I2

-

-

-

-

-

LCD_G4

CH3

SCL

TX

ULPI_D3

RX_ER

TOUT

S2_CK

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

STM32F427xx STM32F429xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

ETH_MII_

TIM2_

I2C2_

USART3_

OTG_HS_

TX_EN/

EVEN

PB11

-

-

-

-

-

-

-

-

-

LCD_G5

CH4

SDA

RX

ULPI_D4

ETH_RMII

TOUT

_TX_EN

ETH_MII_

SPI2_

TIM1_

I2C2_

USART3_

OTG_HS_

TXD0/ETH

OTG_HS_

EVEN

PB12

-

-

-

NSS/I2

-

-

CAN2_RX

-

-

BKIN

SMBA

CK

ULPI_D5

_RMII_

ID

TOUT

S2_WS

TXD0

Port B

ETH_MII_

SPI2_

TIM1_

USART3_

OTG_HS_

TXD1/ETH

EVEN

PB13

-

-

-

-

SCK/I2

-

-

CAN2_TX

-

-

-

CH1N

CTS

ULPI_D6

_RMII_TX

TOUT

S2_CK

D1

TIM1_

TIM8_

SPI2_

I2S2ext_

USART3_

OTG_HS_

EVEN

PB14

-

-

-

-

TIM12_CH1

-

-

-

-

CH2N

CH2N

MISO

SD

RTS

DM

TOUT

DS9405 Rev 13

SPI2_

RTC_

TIM1_

TIM8_

OTG_HS_

EVEN

PB15

-

-

MOSI/I2

-

-

-

TIM12_CH2

-

-

-

-

REFIN

CH3N

CH3N

DP

TOUT

S2_SD

EVEN

FMC_SDN

OTG_HS_

-

-

-

PC0

-

-

-

-

-

-

-

-

-

-

TOUT

WE

ULPI_STP

EVEN

PC1

-

-

-

-

-

-

-

-

-

-

-

ETH_MDC

-

-

-

TOUT

EVEN

OTG_HS_

ETH_MII_

FMC_

SPI2_

I2S2ext_

-

-

-

-

-

PC2

-

-

-

-

-

TOUT

ULPI_DIR

TXD2

SDNE0

MISO

SD

SPI2_

OTG_HS_

ETH_MII_

FMC_

EVEN

PC3

-

-

-

-

-

MOSI/I2

-

-

-

-

-

-

ULPI_NXT

TX_CLK

SDCKE0

TOUT

S2_SD

Pinouts and pin description

ETH_MII_

Port

RXD0/ETH

EVEN

C

PC4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

_RMII_

TOUT

RXD0

ETH_MII_

RXD1/ETH

EVEN

PC5

-

-

-

-

-

-

-

-

-

-

-

-

-

-

_RMII_

TOUT

RXD1

TIM3_

TIM8_

I2S2_

USART6_

DCMI_

LCD_

EVEN

PC6

-

-

-

-

-

-

-

-

SDIO_D6

CH1

CH1

MCK

TX

D0

HSYNC

TOUT

77/240

TIM3_

TIM8_

I2S3_

USART6_

DCMI_

EVEN

PC7

-

-

-

-

-

-

-

-

SDIO_D7

LCD_G6

CH2

CH2

MCK

RX

D1

TOUT

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

78/240

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

TIM3_

TIM8_

USART6_

DCMI_

EVEN

PC8

-

-

-

-

-

-

-

-

-

SDIO_D0

-

CH3

CH3

CK

D2

TOUT

TIM3_

TIM8_

I2C3_

I2S_

DCMI_

EVEN

PC9

MCO2

-

-

-

-

-

-

-

SDIO_D1

-

CH4

CH4

SDA

CKIN

D3

TOUT

SPI3_

USART3_

DCMI_

EVEN

PC10

-

-

-

-

-

-

SCK/I2S

UART4_TX

-

-

-

SDIO_D2

LCD_R2

TX

D8

TOUT

3_CK

I2S3ext

SPI3_

USART3_

DCMI_

EVEN

PC11

-

-

-

-

-

UART4_RX

-

-

-

SDIO_D3

-

_SD

MISO

RX

D4

TOUT

Port

C

SPI3_

USART3_

DCMI_

EVEN

PC12

-

-

-

-

-

-

MOSI/I2

UART5_TX

-

-

-

SDIO_CK

-

CK

D9

TOUT

S3_SD

DS9405 Rev 13

EVEN

PC13

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

EVEN

PC14

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

EVEN

PC15

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

EVEN

PD0

-

-

-

-

-

-

-

-

-

CAN1_RX

-

-

FMC_D2

-

-

TOUT

EVEN

PD1

-

-

-

-

-

-

-

-

-

CAN1_TX

-

-

FMC_D3

-

-

TOUT

EVEN

SDIO_

DCMI_

TIM3_

-

-

-

-

-

-

UART5_RX

-

-

-

PD2

-

-

TOUT

CMD

D11

ETR

STM32F427xx STM32F429xx

SPI2_S

EVEN

DCMI_

USART2_

PD3

-

-

-

-

-

CK/I

-

LCD_G7

-

-

-

-

FMC_CLK

Port

TOUT

D5

CTS

2S2_CK

D

USART2_

EVEN

PD4

-

-

-

-

-

-

-

-

-

-

-

FMC_NOE

-

-

RTS

TOUT

USART2_

EVEN

PD5

-

-

-

-

-

-

-

-

-

-

-

FMC_NWE

-

-

TX

TOUT

SPI3_

SAI1_

USART2_

FMC_

DCMI_

EVEN

PD6

-

-

-

-

-

MOSI/I2

-

-

-

-

LCD_B2

SD_A

RX

NWAIT

D10

TOUT

S3_SD

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

STM32F427xx STM32F429xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

FMC_NE1/

USART2_

EVEN

PD7

-

-

-

-

-

-

-

-

-

-

-

FMC_

-

-

CK

TOUT

NCE2

USART3_

EVEN

PD8

-

-

-

-

-

-

-

-

-

-

-

FMC_D13

-

-

TX

TOUT

USART3_

EVEN

PD9

-

-

-

-

-

-

-

-

-

-

-

FMC_D14

-

-

RX

TOUT

USART3_

EVEN

PD10

-

-

-

-

-

-

-

-

-

-

-

FMC_D15

-

LCD_B3

CK

TOUT

Port

USART3_

EVEN

D

PD11

-

-

-

-

-

-

-

-

-

-

-

FMC_A16

-

-

CTS

TOUT

TIM4_

USART3_

EVEN

PD12

-

-

-

-

-

-

-

-

-

-

FMC_A17

-

-

DS9405 Rev 13

CH1

RTS

TOUT

TIM4_

EVEN

PD13

-

-

-

-

-

-

-

-

-

-

-

FMC_A18

-

-

CH2

TOUT

TIM4_

EVEN

PD14

-

-

-

-

-

-

-

-

-

-

-

FMC_D0

-

-

CH3

TOUT

EVEN

TIM4_

-

-

-

-

-

-

-

-

-

FMC_D1

-

-

PD15

-

-

TOUT

CH4

TIM4_

FMC_

DCMI_

EVEN

PE0

-

-

-

-

-

-

-

UART8_Rx

-

-

-

-

ETR

NBL0

D2

TOUT

FMC_

DCMI_

EVEN

PE1

-

-

-

-

-

-

-

-

UART8_Tx

-

-

-

-

NBL1

D3

TOUT

Pinouts and pin description

EVEN

ETH_MII_

SPI4_

SAI1_

TRAC

FMC_A23

-

-

-

-

-

-

-

-

-

-

PE2

TOUT

TXD3

SCK

MCLK_A

ECLK

TRAC

SAI1_

EVEN

PE3

-

-

-

-

-

Port E

-

-

-

-

-

FMC_A19

-

-

ED0

SD_B

TOUT

TRAC

SPI4_

SAI1_

DCMI_

EVEN

PE4

-

-

-

-

-

-

-

-

-

FMC_A20

LCD_B0

ED1

NSS

FS_A

D4

TOUT

TRAC

TIM9_

SPI4_M

SAI1_

DCMI_

EVEN

PE5

-

-

-

-

-

-

-

-

FMC_A21

LCD_G0

ED2

CH1

ISO

SCK_A

D6

TOUT

TRAC

TIM9_

SPI4_

SAI1_

DCMI_

EVEN

PE6

-

-

-

-

-

-

-

-

FMC_A22

LCD_G1

ED3

CH2

MOSI

SD_A

D7

TOUT

79/240

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

80/240

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

TIM1_

EVEN

PE7

-

-

-

-

-

-

-

UART7_Rx

-

-

-

FMC_D4

-

-

ETR

TOUT

TIM1_

EVEN

PE8

-

-

-

-

-

-

-

UART7_Tx

-

-

-

FMC_D5

-

-

CH1N

TOUT

TIM1_

EVEN

PE9

-

-

-

-

-

-

-

-

-

-

-

FMC_D6

-

-

CH1

TOUT

TIM1_

EVEN

PE10

-

-

-

-

-

-

-

-

-

-

-

FMC_D7

-

-

CH2N

TOUT

EVEN

SPI4_

TIM1_

Port E

-

-

-

-

-

-

FMC_D8

-

LCD_G3

-

-

-

PE11

-

TOUT

NSS

CH2

TIM1_

SPI4_

EVEN

PE12

-

-

-

-

-

-

-

-

-

-

FMC_D9

-

LCD_B4

CH3N

SCK

TOUT

DS9405 Rev 13

TIM1_

SPI4_

EVEN

PE13

-

-

-

-

-

-

-

-

-

-

FMC_D10

-

LCD_DE

CH3

MISO

TOUT

TIM1_

SPI4_

LCD_

EVEN

PE14

-

-

-

-

-

-

-

-

-

-

FMC_D11

-

CH4

MOSI

CLK

TOUT

TIM1_

EVEN

PE15

-

-

-

-

-

-

-

-

-

-

FMC_D12

-

LCD_R7

BKIN

TOUT

EVEN

I2C2_

-

-

-

-

-

-

-

FMC_A0

-

-

PF0

-

-

-

-

TOUT

SDA

I2C2_

EVEN

PF1

-

-

-

-

-

-

-

-

FMC_A1

-

-

SCL

TOUT

I2C2_

EVEN

PF2

-

-

-

-

-

-

-

-

-

-

-

FMC_A2

-

-

STM32F427xx STM32F429xx

SMBA

TOUT

EVEN

PF3

-

-

-

-

-

-

-

-

-

-

-

FMC_A3

-

-

TOUT

Port F

EVEN

PF4

-

-

-

-

-

-

-

-

-

-

-

FMC_A4

-

-

TOUT

EVEN

PF5

-

-

-

-

-

-

-

-

-

-

-

FMC_A5

-

-

TOUT

EVEN

FMC_

SPI5_

SAI1_

TIM10_

-

-

-

UART7_Rx

-

-

-

-

PF6

-

-

-

TOUT

NIORD

NSS

SD_B

CH1

TIM11_

SPI5_

SAI1_

FMC_

EVEN

PF7

-

-

-

-

-

UART7_Tx

-

-

-

-

-

CH1

SCK

MCLK_B

NREG

TOUT

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

STM32F427xx STM32F429xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

SPI5_

SAI1_

FMC_

EVEN

PF8

-

-

-

-

-

-

-

TIM13_CH1

-

-

-

-

MISO

SCK_B

NIOWR

TOUT

SPI5_

SAI1_

EVEN

PF9

-

-

-

-

-

-

-

TIM14_CH1

-

-

FMC_CD

-

-

MOSI

FS_B

TOUT

DCMI_

EVEN

PF10

-

-

-

-

-

-

-

-

-

-

-

-

FMC_INTR

LCD_DE

D11

TOUT

SPI5_

FMC_

DCMI_

EVEN

PF11

-

-

-

-

-

-

-

-

-

-

-

-

MOSI

SDNRAS

D12

TOUT

Port F

EVEN

PF12

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A6

-

-

TOUT

EVEN

PF13

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A7

-

-

TOUT

DS9405 Rev 13

EVEN

PF14

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A8

-

-

TOUT

EVEN

PF15

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A9

-

-

TOUT

EVEN

PG0

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A10

-

-

TOUT

EVEN

PG1

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A11

-

-

TOUT

EVEN

PG2

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A12

-

-

TOUT

EVEN

PG3

-

-

-

-

-

-

-

-

-

-

-

-

FMC_A13

-

-

Pinouts and pin description

TOUT

Port

FMC_A14/

EVEN

PG4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

G

FMC_BA0

TOUT

FMC_A15/

EVEN

PG5

-

-

-

-

-

-

-

-

-

-

-

-

-

-

FMC_BA1

TOUT

DCMI_

EVEN

PG6

-

-

-

-

-

-

-

-

-

-

-

-

FMC_INT2

LCD_R7

D12

TOUT

DCMI_

LCD_

EVEN

USART6_

-

-

-

FMC_INT3

PG7

-

-

-

-

-

-

-

-

D13

CLK

TOUT

CK

81/240

EVEN

SPI6_

USART6_

ETH_PPS

FMC_SDC

-

-

PG8

-

-

-

-

-

-

-

-

-

LK

TOUT

NSS

RTS

_OUT

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

82/240

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

FMC_NE2/

DCMI_

USART6_

EVEN

PG9

-

-

-

-

-

-

-

-

-

-

-

FMC_

-

VSYNC

RX

TOUT

(1)

NCE3

FMC_

DCMI_

EVEN

PG10

-

-

-

-

-

-

-

-

-

LCD_G3

-

-

NCE4_1/

LCD_B2

D2

TOUT

FMC_NE3

ETH_MII_

TX_EN/

FMC_

DCMI_

EVEN

PG11

-

-

-

-

-

-

-

-

-

-

-

LCD_B3

ETH_RMII

NCE4_2

D3

TOUT

_TX_EN

Port

EVEN

USART6_

SPI6_

LCD_B4

-

-

FMC_NE4

-

LCD_B1

-

-

PG12

-

-

-

-

-

G

TOUT

RTS

MISO

ETH_MII_

DS9405 Rev 13

SPI6_

USART6_

TXD0/

EVEN

PG13

-

-

-

-

-

-

-

-

-

FMC_A24

-

-

SCK

CTS

ETH_RMII

TOUT

_TXD0

ETH_MII_

USART6_

TXD1/

EVEN

SPI6_

-

-

-

-

FMC_A25

-

-

PG14

-

-

-

-

-

TX

ETH_RMII

TOUT

MOSI

_TXD1

USART6_

FMC_

DCMI_

EVEN

PG15

-

-

-

-

-

-

-

-

-

-

-

-

CTS

SDNCAS

D13

TOUT

EVEN

PH0

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

EVEN

PH1

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

STM32F427xx STM32F429xx

ETH_MII_

FMC_

EVEN

PH2

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R0

CRS

SDCKE0

TOUT

Port

ETH_MII_

FMC_SDN

EVEN

PH3

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R1

H

COL

E0

TOUT

EVEN

I2C2_

OTG_HS_

-

-

-

-

PH4

-

-

-

-

-

-

-

-

-

ULPI_NXT

TOUT

SCL

I2C2_

SPI5_N

FMC_SDN

EVEN

PH5

-

-

-

-

-

-

-

-

-

-

-

-

SDA

SS

WE

TOUT

ETH_MII_

FMC_

DCMI_

I2C2_

SPI5_

-

-

-

-

-

TIM12_CH1

-

PH6

-

-

-

-

RXD2

SDNE1

D8

SMBA

SCK

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

STM32F427xx STM32F429xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

DCMI_

I2C3_

SPI5_

ETH_MII_

FMC_

PH7

-

-

-

-

-

-

-

-

-

-

-

SCL

MISO

RXD3

SDCKE1

D9

I2C3_

DCMI_

EVEN

PH8

-

-

-

-

-

-

-

-

-

-

-

FMC_D16

LCD_R2

SDA

HSYNC

TOUT

I2C3_

DCMI_

EVEN

PH9

-

-

-

-

-

-

-

-

TIM12_CH2

-

-

FMC_D17

LCD_R3

SMBA

D0

TOUT

TIM5_

DCMI_

EVEN

PH10

-

-

-

-

-

-

-

-

-

-

-

FMC_D18

LCD_R4

CH1

D1

TOUT

Port

TIM5_

DCMI_

EVEN

PH11

-

-

-

-

-

-

-

-

-

-

-

FMC_D19

LCD_R5

H

CH2

D2

TOUT

TIM5_

DCMI_

EVEN

PH12

-

-

-

-

-

-

-

-

-

-

-

FMC_D20

LCD_R6

CH3

D3

TOUT

DS9405 Rev 13

EVEN

TIM8_

-

-

-

-

-

CAN1_TX

-

-

FMC_D21

-

LCD_G2

PH13

-

-

-

TOUT

CH1N

EVEN

DCMI_

TIM8_

LCD_G3

-

-

-

-

-

-

-

-

FMC_D22

PH14

-

-

-

TOUT

D4

CH2N

TIM8_

DCMI_

EVEN

PH15

-

-

-

-

-

-

-

-

-

-

-

FMC_D23

LCD_G4

CH3N

D11

TOUT

SPI2_

TIM5_

DCMI_

EVEN

PI0

-

-

-

-

NSS/I2

-

-

-

-

-

-

FMC_D24

LCD_G5

CH4

D13

TOUT

S2_WS

SPI2_

EVEN

DCMI_

LCD_G6

PI1

-

-

-

-

-

SCK/I2

-

-

-

-

-

-

FMC_D25

TOUT

D8

S2_CK

Pinouts and pin description

EVEN

DCMI_

SPI2_

I2S2ext_

TIM8_

LCD_G7

-

-

-

-

-

FMC_D26

-

PI2

-

-

-

TOUT

D9

MISO

SD

CH4

SPI2_M

Port I

TIM8_

DCMI_D

EVEN

PI3

-

-

-

-

OSI/I2S

FMC_D27

ETR

10

TOUT

2_SD

TIM8_

FMC_

DCMI_D

EVEN

PI4

-

-

-

-

-

-

-

-

-

-

-

LCD_B4

BKIN

NBL2

5

TOUT

TIM8_

FMC_

DCMI_

EVEN

PI5

-

-

-

-

-

-

-

-

-

-

-

LCD_B5

CH1

NBL3

VSYNC

TOUT

83/240

TIM8_

DCMI_

EVEN

PI6

-

-

-

-

-

-

-

-

-

-

-

FMC_D28

LCD_B6

CH2

D6

TOUT

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

84/240

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

TIM8_

DCMI_

EVEN

PI7

-

-

-

-

-

-

-

-

-

-

-

FMC_D29

LCD_B7

CH3

D7

TOUT

EVEN

PI8

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

TOUT

LCD_

EVEN

PI9

-

-

-

-

-

-

-

-

-

CAN1_RX

-

-

FMC_D30

-

VSYNC

TOUT

ETH_MII_

LCD_

EVEN

PI10

-

-

-

-

-

-

-

-

-

-

-

FMC_D31

-

RX_ER

HSYNC

TOUT

EVEN

OTG_HS_

Port I

-

-

-

-

PI11

-

-

-

-

-

-

-

-

-

-

TOUT

ULPI_DIR

LCD_

EVEN

PI12

-

-

-

-

-

-

-

-

-

-

-

-

-

-

HSYNC

TOUT

DS9405 Rev 13

LCD_

EVEN

PI13

-

-

-

-

-

-

-

-

-

-

-

-

-

-

VSYNC

TOUT

LCD_

EVEN

PI14

-

-

-

-

-

-

-

-

-

-

-

-

-

-

CLK

TOUT

EVEN

PI15

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R0

TOUT

EVEN

PJ0

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R1

TOUT

EVEN

PJ1

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R2

TOUT

EVEN

PJ2

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R3

STM32F427xx STM32F429xx

TOUT

EVEN

PJ3

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R4

TOUT

Port J

EVEN

PJ4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R5

TOUT

EVEN

PJ5

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R6

TOUT

EVEN

PJ6

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_R7

TOUT

EVEN

PJ7

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G0

TOUT

Table 12. STM32F427xx and STM32F429xx alternate function mapping (continued)

STM32F427xx STM32F429xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

AF14

AF15

Port

SPI3/

USART6/

CAN1/2/

OTG2_HS

TIM8/9/

I2C1/

SPI1/2/

SPI2/3/

FMC/SDIO

SYS

TIM1/2

TIM3/4/5

USART1/

UART4/5/7

TIM12/13/14

/OTG1_

ETH

DCMI

LCD

SYS

10/11

2/3

3/4/5/6

SAI1

/OTG2_FS

2/3

/8

/LCD

FS

EVEN

PJ8

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G1

TOUT

EVEN

PJ9

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G2

TOUT

EVEN

PJ10

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G3

TOUT

EVEN

PJ11

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G4

TOUT

Port J

EVEN

PJ12

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B0

TOUT

EVEN

PJ13

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B1

TOUT

DS9405 Rev 13

EVEN

PJ14

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B2

TOUT

EVEN

PJ15

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B3

TOUT

EVEN

PK0

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G5

TOUT

EVEN

PK1

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G6

TOUT

EVEN

PK2

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_G7

TOUT

EVEN

PK3

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B4

Pinouts and pin description

TOUT

Port K

EVEN

PK4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B5

TOUT

EVEN

PK5

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B6

TOUT

EVEN

PK6

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_B7

TOUT

EVEN

PK7

-

-

-

-

-

-

-

-

-

-

-

-

-

-

LCD_DE

TOUT

85/240

1.

The DCMI_VSYNC alternate function on PG9 is only available on silicon revision 3.

Memory mapping

STM32F427xx STM32F429xx

5

Memory mapping

The memory map is shown in Figure 19.

Figure 19. Memory map

Reserved

0xE010 0000 - 0xFFFF FFFF

Cortex-M4 internal

0xE000 0000 - 0xE00F FFFF

peripherals

0x6000 0000 - 0xDFFF FFFF

AHB3

Reserved

0x5006 0C00 - 0x5FFF FFFF

0x5006 0BFF

AHB2

0xFFFF FFFF

512-Mbyte

Block 7

Cortex-M4

0x5000 0000

Internal

0x4008 0000 - 0x4FFF FFFF

Reserved

peripherals

0x4007 FFFF

0xE000 0000

0xDFFF FFFF

512-Mbyte

Block 6

FMC

0xD000 0000

0xCFFF FFFF

AHB1

512-Mbyte

Block 5

FMC

0xA000 0000

0x9FFF FFFF

Reserved

0xE010 0000 - 0xFFFF FFFF

Cortex-M4 internal

0xE000 0000 - 0xE00F FFFF

peripherals

0x6000 0000 - 0xDFFF FFFF

AHB3

0x5006 0C00 - 0x5FFF FFFF

512-Mbyte

0x4002 0000

Block 4

Reserved

0x4001 6C00 - 0x4001 FFFF

FMC bank 3 to

bank 4

0x4001 6BFF

0x8000 0000

0x7FFF FFFF

512-Mbyte

Block 3

FMC bank 1 to

bank 2

0x6000 0000

0x5FFF FFFF

APB2

512-Mbyte

Block 2

Peripherals

0x4000 0000

0x3FFF FFFF

512-Mbyte

Block 1

SRAM

0x2003 0000 - 0x3FFF FFFF

Reserved

0x4001 0000

SRAM (64 KB aliased

0x2000 0000

0x2002 0000 - 0x2002 FFFF

0x4000 8000 - 0x4000 FFFF

Reserved

By bit-banding

0x1FFF FFFF

512-Mbyte

Block 0

0x4000 7FFF

SRAM (16 KB aliased

0x2001 C000 - 0x2001 FFFF

By bit-banding

SRAM (112 KB aliased

SRAM

0x2000 0000 - 0x2001 BFFF

By bit-banding

0x0000 0000

0x1FFF C010 - 0x1FFF FFFF

Reserved

Option Bytes

0x1FFF C000 - 0x1FFF C00F

Reserved

0x1FFF 7A10 - 0x1FFF 7FFF

0x1FFF 0000 - 0x1FFF 7A0F

System memory

APB1

0x1FFE C010 - 0x1FFE FFFF

Reserved

0x1FFE C000 - 0x1FFF C00F

Option bytes

Reserved

0x1001 0000 - 0x1FFE BFFF

CCM data RAM

0x1000 0000 - 0x1000 FFFF

(64 KB data SRAM)

0x0820 0000 - 0x0FFF FFFF

Reserved

0x0800 0000 - 0x081F FFFF

Flash memory

0x0020 0000 - 0x07FF FFFF

Reserved

Aliased to Flash, system

0x4000 0000

0x0000 0000 - 0x001F FFFF

memory or SRAM depending

on the BOOT pins

MS30424V5

86/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Memory mapping

Table 13. STM32F427xx and STM32F429xx register boundary addresses

Bus

Boundary address

Peripheral

0xE00F FFFF - 0xFFFF FFFF

Reserved

Cortex-M4

0xE000 0000 - 0xE00F FFFF

Cortex-M4 internal peripherals

0xD000 0000 - 0xDFFF FFFF

FMC bank 6

0xC000 0000 - 0xCFFF FFFF

FMC bank 5

0xA000 1000 - 0xBFFF FFFF

Reserved

0xA000 0000- 0xA000 0FFF

FMC control register

AHB3

0x9000 0000 - 0x9FFF FFFF

FMC bank 4

0x8000 0000 - 0x8FFF FFFF

FMC bank 3

0x7000 0000 - 0x7FFF FFFF

FMC bank 2

0x6000 0000 - 0x6FFF FFFF

FMC bank 1

0x5006 0C00- 0x5FFF FFFF

Reserved

0x5006 0800 - 0X5006 0BFF

RNG

0x5005 0400 - X5006 07FF

Reserved

0x5005 0000 - 0X5005 03FF

DCMI

AHB2

0x5004 0000- 0x5004 FFFF

Reserved

0x5000 0000 - 0X5003 FFFF

USB OTG FS

DS9405 Rev 13

87/240

90

Memory mapping

STM32F427xx STM32F429xx

Table 13. STM32F427xx and STM32F429xx register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4008 0000- 0x4FFF FFFF

Reserved

0x4004 0000 - 0x4007 FFFF

USB OTG HS

0x4002 BC00- 0x4003 FFFF

Reserved

0x4002 B000 - 0x4002 BBFF

DMA2D

0x4002 9400 - 0x4002 AFFF

Reserved

0x4002 9000 - 0x4002 93FF

0x4002 8C00 - 0x4002 8FFF

0x4002 8800 - 0x4002 8BFF

ETHERNET MAC

0x4002 8400 - 0x4002 87FF

0x4002 8000 - 0x4002 83FF

0x4002 6800 - 0x4002 7FFF

Reserved

0x4002 6400 - 0x4002 67FF

DMA2

0x4002 6000 - 0x4002 63FF

DMA1

0X4002 5000 - 0X4002 5FFF

Reserved

0x4002 4000 - 0x4002 4FFF

BKPSRAM

0x4002 3C00 - 0x4002 3FFF

Flash interface register

AHB1

0x4002 3800 - 0x4002 3BFF

RCC

0X4002 3400 - 0X4002 37FF

Reserved

0x4002 3000 - 0x4002 33FF

CRC

0x4002 2C00 - 0x4002 2FFF

Reserved

0x4002 2800 - 0x4002 2BFF

GPIOK

0x4002 2400 - 0x4002 27FF

GPIOJ

0x4002 2000 - 0x4002 23FF

GPIOI

0x4002 1C00 - 0x4002 1FFF

GPIOH

0x4002 1800 - 0x4002 1BFF

GPIOG

0x4002 1400 - 0x4002 17FF

GPIOF

0x4002 1000 - 0x4002 13FF

GPIOE

0X4002 0C00 - 0x4002 0FFF

GPIOD

0x4002 0800 - 0x4002 0BFF

GPIOC

0x4002 0400 - 0x4002 07FF

GPIOB

0x4002 0000 - 0x4002 03FF

GPIOA

88/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Memory mapping

Table 13. STM32F427xx and STM32F429xx register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4001 6C00- 0x4001 FFFF

Reserved

0x4001 6800 - 0x4001 6BFF

LCD-TFT

0x4001 5C00 - 0x4001 67FF

Reserved

0x4001 5800 - 0x4001 5BFF

SAI1

0x4001 5400 - 0x4001 57FF

SPI6

0x4001 5000 - 0x4001 53FF

SPI5

0x4001 5400 - 0x4001 57FF

SPI6

0x4001 5000 - 0x4001 53FF

SPI5

0x4001 4C00 - 0x4001 4FFF

Reserved

0x4001 4800 - 0x4001 4BFF

TIM11

0x4001 4400 - 0x4001 47FF

TIM10

0x4001 4000 - 0x4001 43FF

TIM9

0x4001 3C00 - 0x4001 3FFF

EXTI

APB2

0x4001 3800 - 0x4001 3BFF

SYSCFG

0x4001 3400 - 0x4001 37FF

SPI4

0x4001 3000 - 0x4001 33FF

SPI1

0x4001 2C00 - 0x4001 2FFF

SDIO

0x4001 2400 - 0x4001 2BFF

Reserved

0x4001 2000 - 0x4001 23FF

ADC1 - ADC2 - ADC3

0x4001 1800 - 0x4001 1FFF

Reserved

0x4001 1400 - 0x4001 17FF

USART6

0x4001 1000 - 0x4001 13FF

USART1

0x4001 0800 - 0x4001 0FFF

Reserved

0x4001 0400 - 0x4001 07FF

TIM8

0x4001 0000 - 0x4001 03FF

TIM1

DS9405 Rev 13

89/240

90

Memory mapping

STM32F427xx STM32F429xx

Table 13. STM32F427xx and STM32F429xx register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4000 8000- 0x4000 FFFF

Reserved

0x4000 7C00 - 0x4000 7FFF

UART8

0x4000 7800 - 0x4000 7BFF

UART7

0x4000 7400 - 0x4000 77FF

DAC

0x4000 7000 - 0x4000 73FF

PWR

0x4000 6C00 - 0x4000 6FFF

Reserved

0x4000 6800 - 0x4000 6BFF

CAN2

0x4000 6400 - 0x4000 67FF

CAN1

0x4000 6000 - 0x4000 63FF

Reserved

0x4000 5C00 - 0x4000 5FFF

I2C3

0x4000 5800 - 0x4000 5BFF

I2C2

0x4000 5400 - 0x4000 57FF

I2C1

0x4000 5000 - 0x4000 53FF

UART5

0x4000 4C00 - 0x4000 4FFF

UART4

0x4000 4800 - 0x4000 4BFF

USART3

0x4000 4400 - 0x4000 47FF

USART2

0x4000 4000 - 0x4000 43FF

I2S3ext

APB1

0x4000 3C00 - 0x4000 3FFF

SPI3 / I2S3

0x4000 3800 - 0x4000 3BFF

SPI2 / I2S2

0x4000 3400 - 0x4000 37FF

I2S2ext

0x4000 3000 - 0x4000 33FF

IWDG

0x4000 2C00 - 0x4000 2FFF

WWDG

0x4000 2800 - 0x4000 2BFF

RTC & BKP Registers

0x4000 2400 - 0x4000 27FF

Reserved

0x4000 2000 - 0x4000 23FF

TIM14

0x4000 1C00 - 0x4000 1FFF

TIM13

0x4000 1800 - 0x4000 1BFF

TIM12

0x4000 1400 - 0x4000 17FF

TIM7

0x4000 1000 - 0x4000 13FF

TIM6

0x4000 0C00 - 0x4000 0FFF

TIM5

0x4000 0800 - 0x4000 0BFF

TIM4

0x4000 0400 - 0x4000 07FF

TIM3

0x4000 0000 - 0x4000 03FF

TIM2

90/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6

Electrical characteristics

6.1

Parameter conditions

Unless otherwise specified, all voltages are referenced to V

.

SS

6.1.1

Minimum and maximum values

Unless otherwise specified the minimum and maximum values are guaranteed in the worst

conditions of ambient temperature, supply voltage and frequencies by tests in production on

100% of the devices with an ambient temperature at T

= 25 °C and T

= T

max (given by

A

A

A

the selected temperature range).

Data based on characterization results, design simulation and/or technology characteristics

are indicated in the table footnotes and are not tested in production. Based on

characterization, the minimum and maximum values refer to sample tests and represent the

mean value plus or minus three times the standard deviation (mean±3σ).

6.1.2

Typical values

Unless otherwise specified, typical data are based on T

= 25 °C, V

= 3.3 V (for the

A

DD

1.7 V ≤V

≤ 3.6 V voltage range). They are given only as design guidelines and are not

DD

tested.

Typical ADC accuracy values are determined by characterization of a batch of samples from

a standard diffusion lot over the full temperature range, where 95% of the devices have an

error less than or equal to the value indicated

(mean±2σ)

.

6.1.3

Typical curves

Unless otherwise specified, all typical curves are given only as design guidelines and are

not tested.

6.1.4

Loading capacitor

The loading conditions used for pin parameter measurement are shown in Figure 20.

6.1.5

Pin input voltage

The input voltage measurement on a pin of the device is described in Figure 21.

Figure 20. Pin loading conditions

Figure 21. Pin input voltage

MCU pin

MCU pin

C = 50 pF

V

IN

MS19011V2

MS19010V2

DS9405 Rev 13

91/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.1.6

Power supply scheme

Figure 22. Power supply scheme

VBAT

VBAT =

Power

1.65 to 3.6V

switch

Backup circuitry

(OSC32K,RTC,

Wakeup logic

Backup registers,

backup RAM)

OUT

GPIOs

Logic

IN

VCAP_1

VCAP_2

2 × 2.2 μF

Kernel logic

(CPU, digital

& RAM)

VDD

VDD

1/2/...14/15

15 × 100 nF

VSS

regulator

+ 1 × 4.7 μF

1/2/...14/15

Voltage

IO

Level shifter

BYPASS_REG

Reset

Flash memory

PDR_ON

VDD

controller

VDDA

VREF

VREF+

Analog:

100 nF

100 nF

RCs,

ADC

VREF-

+ 1 μF

+ 1 μF

PLL,..

VSSA

MS19911V3

1.

To connect BYPASS_REG and PDR_ON pins, refer to Section 3.17: Power supply supervisor and Section 3.18: Voltage

regulator

2.

The two 2.2 µF ceramic capacitors should be replaced by two 100 nF decoupling capacitors when the voltage regulator is

OFF.

3.

The 4.7 µF ceramic capacitor must be connected to one of the VDD pins.

=V

and V

=V

.

4.

V

DDA

DD

SSA

SS

Caution:

Each power supply pair (V

/V

, V

/V

...) must be decoupled with filtering ceramic

DD

SS

DDA

SSA

capacitors as shown above. These capacitors must be placed as close as possible to, or

below, the appropriate pins on the underside of the PCB to ensure good operation of the

device. It is not recommended to remove filtering capacitors to reduce PCB size or cost.

This might cause incorrect operation of the device.

92/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.1.7

Current consumption measurement

Figure 23. Current consumption measurement scheme

I

_V

DD

BAT

V

BAT

I

DD

V

DD

V

DDA

ai14126

6.2

Absolute maximum ratings

Stresses above the absolute maximum ratings listed in Table 14: Voltage characteristics,

Table 15: Current characteristics, and Table 16: Thermal characteristics may cause

permanent damage to the device. These are stress ratings only and functional operation of

the device at these conditions is not implied. Exposure to maximum rating conditions for

extended periods may affect device reliability.

Device mission profile (application conditions) is compliant with JEDEC JESD47

Qualification Standard, extended mission profiles are available on demand.

Table 14. Voltage characteristics

Symbol

Ratings

Min

Max

Unit

External main supply voltage (including V

,

V

DDA

DD

–V

− 0.3

4.0

V

(1)

DD

SS

VBAT)

and

(2)

Input voltage on FT pins

V

− 0.3

V

+ 4.0

SS

DD

V

− 0.3

4.0

Input voltage on TTa pins

V

SS

V

IN

− 0.3

4.0

Input voltage on any other pin

V

SS

Input voltage on BOOT0 pin

V

9.0

SS

|

Variations between different V

power pins

-

50

|∆V

DDx

DD

mV

Variations between all the different ground pins

|V

-V

|

-

50

SSX

SS

including V

REF-

see Section 6.3.15:

Absolute maximum

V

Electrostatic discharge voltage (human body model)

ESD(HBM)

ratings (electrical

sensitivity)

1.

All main power (V

, V

) and ground (V

, V

) pins must always be connected to the external

DD

DDA

SS

SSA

power supply, in the permitted range.

2.

V

maximum value must always be respected. Refer to Table 15 for the values of the maximum allowed

IN

injected current.

DS9405 Rev 13

93/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 15. Current characteristics

Symbol

Ratings

Max.

Unit

(1)

∑I

Total current into sum of all V

power lines (source)

270

VDD

DD_x

(1)

∑I

Total current out of sum of all V

ground lines (sink)

− 270

VSS

SS_x

(1)

Maximum current into each V

power line (source)

100

I

VDD

DD_x

(1)

I

Maximum current out of each V

ground line (sink)

− 100

VSS

SS_x

Output current sunk by any I/O and control pin

25

I

IO

Output current sourced by any I/Os and control pin

− 25

mA

(2)

Total output current sunk by sum of all I/O and control pins

120

∑I

IO

(2)

− 120

Total output current sourced by sum of all I/Os and control pins

(4)

Injected current on FT pins

− 5/+0

(4)

(3)

Injected current on NRST and BOOT0 pins

I

INJ(PIN)

(5)

±5

Injected current on TTa pins

(6)

(5)

Total injected current (sum of all I/O and control pins)

±25

∑I

INJ(PIN)

1.

All main power (V

, V

) and ground (V

, V

) pins must always be connected to the external power supply, in the

DD

DDA

SS

SSA

permitted range.

2.

This current consumption must be correctly distributed over all I/Os and control pins. The total output current must not be

sunk/sourced between two consecutive power supply pins referring to high pin count LQFP packages.

3.

Negative injection disturbs the analog performance of the device. See note in Section 6.3.21: 12-bit ADC characteristics.

4.

Positive injection is not possible on these I/Os and does not occur for input voltages lower than the specified maximum

value.

>V

while a negative injection is induced by V

<V

. I

must never be

5.

A positive injection is induced by V

IN

DDA

IN

SS

INJ(PIN)

exceeded. Refer to Table 14 for the values of the maximum allowed input voltage.

Σ

I

is the absolute sum of the positive and

6.

When several inputs are submitted to a current injection, the maximum

INJ(PIN)

negative injected currents (instantaneous values).

Table 16. Thermal characteristics

Symbol

Ratings

Value

Unit

T

Storage temperature range

− 65 to +150

°C

STG

Maximum junction temperature

125

°C

T

J

94/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3

Operating conditions

6.3.1

General operating conditions

Table 17. General operating conditions

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Power Scale 3 (VOS[1:0] bits in

PWR_CR register = 0x01), Regulator

0

-

120

ON, over-drive OFF

Over-

-

144

Power Scale 2 (VOS[1:0] bits

drive OFF

in PWR_CR register = 0x10),

0

f

Internal AHB clock frequency

Over-

Regulator ON

HCLK

-

168

drive ON

Over-

-

168

MHz

Power Scale 1 (VOS[1:0] bits

drive OFF

in PWR_CR register= 0x11),

0

Over-

Regulator ON

-

180

drive ON

Over-drive OFF

0

-

42

Internal APB1 clock frequency

f

PCLK1

Over-drive ON

0

-

45

Over-drive OFF

0

-

84

f

Internal APB2 clock frequency

PCLK2

Over-drive ON

0

-

90

(2)

V

Standard operating voltage

1.7

-

3.6

DD

Analog operating voltage

(2)

1.7

-

2.4

(ADC limited to 1.2 M samples)

V

(5)

DDA

V

Must be the same potential as V

(3)(4)

DD

Analog operating voltage

2.4

-

3.6

(ADC limited to 2.4 M samples)

Backup operating voltage

1.65

-

3.6

V

BAT

Power Scale 3 ((VOS[1:0] bits in

PWR_CR register = 0x01), 120 MHz

1.08

1.14

1.20

HCLK max frequency

Power Scale 2 ((VOS[1:0] bits in

PWR_CR register = 0x10), 144 MHz

Regulator ON: 1.2 V internal

1.20

1.26

1.32

HCLK max frequency with over-drive

voltage on V

/V

pins

CAP_1

CAP_2

OFF or 168 MHz with over-drive ON

V

V

Power Scale 1 ((VOS[1:0] bits in

12

PWR_CR register = 0x11), 168 MHz

1.26

1.32

1.40

HCLK max frequency with over-drive

OFF or 180 MHz with over-drive ON

Max frequency 120 MHz

1.10

1.14

1.20

Regulator OFF: 1.2 V external

voltage must be supplied from

Max frequency 144 MHz

1.20

1.26

1.32

external regulator on

(6)

/V

pins

V

Max frequency 168 MHz

1.26

1.32

1.38

CAP_1

CAP_2

DS9405 Rev 13

95/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 17. General operating conditions (continued)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

2 V  ≤  V

≤  3.6 V

− 0.3

-

5.5

Input voltage on RST and FT

DD

(7)

pins

≤  2 V

− 0.3

-

5.2

V

DD

V

V

+

V

IN

DDA

Input voltage on TTa pins

− 0.3

-

0.3

Input voltage on BOOT0 pin

0

-

9

LQFP100

-

-

465

WLCSP143

-

-

641

LQFP144

-

-

500

Power dissipation at T

= 85 °C

UFBGA169

-

-

385

A

P

mW

= 105 °C for

for suffix 6 or T

D

A

LQFP176

-

-

526

(8)

suffix 7

UFBGA176

-

-

513

LQFP208

-

-

1053

TFBGA216

-

-

690

Maximum power dissipation

− 40

-

85

Ambient temperature for 6 suffix

°C

version

(9)

− 40

-

105

Low power dissipation

T

A

Maximum power dissipation

− 40

-

105

Ambient temperature for 7 suffix

°C

version

(9)

− 40

-

125

Low power dissipation

6 suffix version

− 40

-

105

T

Junction temperature range

°C

J

7 suffix version

− 40

-

125

1.

The overdrive mode is not supported at the voltage ranges from 1.7 to 2.1 V.

/V

minimum value of 1.7 V is obtained with the use of an external power supply supervisor (refer to Section 3.17.2:

2.

V

DD

DDA

Internal reset OFF).

3.

When the ADC is used, refer to Table 75: ADC characteristics.

pin is present, it must respect the following condition: V

-V

< 1.2 V.

4.

If a V

REF+

DDA

REF+

and V

from the same source. A maximum difference of 300 mV between V

and V

5.

It is recommended to power V

DD

DDA

DD

DDA

can be tolerated during power-up and power-down operation.

6.

The overdrive mode is not supported when the internal regulator is OFF.

7.

To sustain a voltage higher than VDD+0.3, the internal pull-up and pull-down resistors must be disabled

is lower, higher P

values are allowed as long as T

does not exceed T

.

8.

If T

A

D

J

Jmax

can be extended to this range as long as T

does not exceed T

.

9.

In low-power dissipation state, T

A

J

Jmax

96/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 18. Limitations depending on the operating power supply range

Maximum Flash

Maximum HCLK

Possible Flash

Operating

memory access

frequency vs Flash

I/O operation

memory

power supply

ADC operation

frequency with

memory wait states

operations

range

no wait states

(1)(2)

)

(f

Flashmax

168 MHz with 8 wait

8-bit erase and

=1.7 to

Conversion time

V

No I/O

(4)

DD

states and over-drive

20 MHz

program

(3)

2.1 V

up to 1.2 Msps

compensation

OFF

operations only

180 MHz with 8 wait

16-bit erase and

= 2.1 to

Conversion time

No I/O

V

DD

22 MHz

states and over-drive

program

2.4 V

up to 1.2 Msps

compensation

ON

operations

180 MHz with 7 wait

16-bit erase and

= 2.4 to

Conversion time

I/O compensation

V

DD

24 MHz

states and over-drive

program

2.7 V

up to 2.4 Msps

works

ON

operations

180 MHz with 5 wait

32-bit erase and

= 2.7 to

Conversion time

I/O compensation

V

DD

30 MHz

states and over-drive

program

(5)

3.6 V

up to 2.4 Msps

works

ON

operations

1.

Applicable only when the code is executed from flash memory. When the code is executed from RAM, no wait state is

required.

2.

Thanks to the ART accelerator and the 128-bit flash memory, the number of wait states given here does not impact the

execution speed from flash memory since the ART accelerator allows to achieve a performance equivalent to 0 wait state

program execution.

V

minimum value of 1.7 V is obtained with the use of an external power supply supervisor (refer to

3.

The V

DD/

DDA

Section 3.17.2: Internal reset OFF).

4.

Prefetch is not available.

5.

The voltage range for USB full speed  PHYs can drop down to 2.7 V. However, the electrical characteristics of D- and D+

pins are degraded between 2.7 and 3 V.

6.3.2

VCAP1/VCAP2 external capacitor

Stabilization for the main regulator is achieved by connecting an external capacitor C

to

EXT

the VCAP1/VCAP2 pins. C

is specified in Table 19.

EXT

Figure 24. External capacitor C

EXT

C

ESR

R

Leak

MS19044V2

1.

Legend: ESR is the equivalent series resistance.

(1)

Table 19. VCAP1/VCAP2 operating conditions

Symbol

Parameter

Conditions

CEXT

Capacitance of external capacitor

2.2 µF

ESR

ESR of external capacitor

< 2 Ω

capacitors are not required and should be

1.

When bypassing the voltage regulator, the two 2.2 µF V

CAP

replaced by two 100 nF decoupling capacitors.

DS9405 Rev 13

97/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.3

Operating conditions at power-up / power-down (regulator ON)

Subject to general operating conditions for T

.

A

Table 20. Operating conditions at power-up / power-down (regulator ON)

Symbol

Parameter

Min

Max

Unit

V

rise time rate

20

∞

DD

t

µs/V

VDD

fall time rate

20

∞

V

DD

6.3.4

Operating conditions at power-up / power-down (regulator OFF)

Subject to general operating conditions for T

.

A

(1)

Table 21. Operating conditions at power-up / power-down (regulator OFF)

Symbol

Parameter

Conditions

Min

Max

Unit

V

rise time rate

Power-up

20

∞

DD

t

VDD

V

fall time rate

Power-down

20

∞

DD

µs/V

V

and V

rise time rate

Power-up

20

∞

CAP_1

CAP_2

t

VCAP

and V

fall time rate

Power-down

20

∞

V

CAP_1

CAP_2

1.

To reset the internal logic at power-down, a reset must be applied on pin PA0 when V

reaches below

DD

1.08 V.

98/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.5

Reset and power control block characteristics

The parameters given in Table 22 are derived from tests performed under ambient

temperature and V

supply voltage conditions summarized in Table 17.

DD

Table 22.  Reset and power control block characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

PLS[2:0]=000 (rising edge)

2.09

2.14

2.19

V

PLS[2:0]=000 (falling edge)

1.98

2.04

2.08

V

PLS[2:0]=001 (rising edge)

2.23

2.30

2.37

V

PLS[2:0]=001 (falling edge)

2.13

2.19

2.25

V

PLS[2:0]=010 (rising edge)

2.39

2.45

2.51

V

PLS[2:0]=010 (falling edge)

2.29

2.35

2.39

V

PLS[2:0]=011 (rising edge)

2.54

2.60

2.65

V

PLS[2:0]=011 (falling edge)

2.44

2.51

2.56

V

Programmable voltage

V

PVD

detector level selection

PLS[2:0]=100 (rising edge)

2.70

2.76

2.82

V

PLS[2:0]=100 (falling edge)

2.59

2.66

2.71

V

PLS[2:0]=101 (rising edge)

2.86

2.93

2.99

V

PLS[2:0]=101 (falling edge)

2.75

2.84

2.92

V

PLS[2:0]=110 (rising edge)

2.96

3.03

3.10

V

PLS[2:0]=110 (falling edge)

2.85

2.93

2.99

V

PLS[2:0]=111 (rising edge)

3.07

3.14

3.21

V

PLS[2:0]=111 (falling edge)

2.95

3.03

3.09

V

(1)

PVD hysteresis

-

-

100

-

mV

V

PVDhyst

Falling edge

1.60

1.68

1.76

V

Power-on/power-down

V

POR/PDR

reset threshold

Rising edge

1.64

1.72

1.80

V

(1)

PDR hysteresis

-

-

40

-

mV

V

PDRhyst

Falling edge

2.13

2.19

2.24

V

Brownout level 1

V

BOR1

threshold

Rising edge

2.23

2.29

2.33

V

Falling edge

2.44

2.50

2.56

V

Brownout level 2

V

BOR2

threshold

Rising edge

2.53

2.59

2.63

V

Falling edge

2.75

2.83

2.88

V

Brownout level 3

V

BOR3

threshold

Rising edge

2.85

2.92

2.97

V

(1)

BOR hysteresis

-

-

100

-

mV

V

BORhyst

T

RSTTEMPO

POR reset temporization -

0.5

1.5

3.0

ms

(1)(2)

DS9405 Rev 13

99/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 22.  Reset and power control block characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

InRush current on

voltage regulator power-

(1)

I

-

-

160

200

mA

RUSH

on (POR or wakeup

from Standby)

InRush energy on

V

= 1.7 V, T

= 105 °C,

voltage regulator power-

DD

A

(1)

-

-

5.4

µC

E

RUSH

on (POR or wakeup

I

= 171 mA for 31 µs

RUSH

from Standby)

1.

Specified by design.

) to the instant

2.

The reset temporization is measured from the power-on (POR reset or wake-up from V

BAT

when the first instruction is read by the user application code.

6.3.6

Overdrive switching characteristics

When the overdrive mode switches from enabled to disabled or disabled to enabled, the

system clock is stalled during the internal voltage set-up.

The overdrive switching characteristics are given in Table 23. They are subject to general

operating conditions for T

.

A

(1)

Table 23. Over-drive switching characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

HSI

-

45

-

Over_drive switch

Tod_swen

enable time

HSE max for 4 MHz

45

-

100

and min for 26 MHz

External HSE

-

40

50 MHz

-

µs

HSI

-

20

-

HSE max for 4 MHz

Over_drive switch

20

-

Tod_swdis

and min for 26 MHz.

disable time

80

External HSE

-

15

-

50 MHz

1.

Specified by design.

100/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.7

Supply current characteristics

The current consumption is a function of several parameters and factors such as the

operating voltage, ambient temperature, I/O pin loading, device software configuration,

operating frequencies, I/O pin switching rate, program location in memory and executed

binary code.

The current consumption is measured as described in Figure 23: Current consumption

measurement scheme.

All the run-mode current consumption measurements given in this section are performed

with a reduced code that gives a consumption equivalent to CoreMark code.

Typical and maximum current consumption

The MCU is placed under the following conditions:

•

All I/O pins are in input mode with a static value at V

or V

(no load).

DD

SS

•

All peripherals are disabled except if it is explicitly mentioned.

•

The flash memory access time is adjusted both to f

frequency and V

range (see

HCLK

DD

Table 18: Limitations depending on the operating power supply range).

•

Regulator ON

•

The voltage scaling and overdrive mode are adjusted to f

frequency as follows:

HCLK

–

Scale 3 for f

≤  120 MHz

HCLK

–

Scale 2 for 120 MHz < f

≤  144 MHz

HCLK

–

Scale 1 for 144 MHz < f

≤  180 MHz. The overdrive is only ON at 180 MHz.

HCLK

•

The system clock is HCLK, f

= f

/4, and f

= f

/2.

PCLK1

HCLK

PCLK2

HCLK

•

The external clock frequency is 4 MHz and PLL is ON when f

is higher than

HCLK

25 MHz.

•

The maximum values are obtained for V

= 3.6 V and a maximum ambient

DD

temperature (T

), and the typical values for T

= 25 °C and V

= 3.3 V unless

A

A

DD

otherwise specified.

DS9405 Rev 13

101/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 24. Typical and maximum current consumption in Run mode, code with data processing

(1)

running from Flash memory (ART accelerator enabled except prefetch) or RAM

(2)

Max

Symbol

Parameter

Conditions

f

(MHz)

Typ

Unit

HCLK

T

=

T

=

T

=

A

A

A

25 °C

85 °C

105 °C

(5)

(5)

180

98

104

123

141

(5)

(5)

168

89

98

116

133

150

75

84

100

115

144

72

81

96

112

120

54

58

72

85

90

43

45

56

66

All

60

29

30

52

62

Peripherals

(3)(4)

enabled

30

16

20

34

46

25

13

16

30

43

16

11

13

27

39

8

5

9

23

36

4

4

8

21

34

Supply

2

2

7

20

33

mA

I

current in

DD

(5)

(5)

180

44

47

69

87

RUN mode

(5)

(5)

168

41

45

66

83

150

36

39

57

73

144

33

37

56

72

120

25

29

43

56

90

20

23

41

53

All

Peripherals

60

14

16

34

45

(3)

disabled

30

8

12

26

39

25

7

10

24

37

16

7

9

22

35

8

3

7

21

34

4

3

6

20

33

2

2

6

20

33

1.

Code and data processing running from SRAM1 using boot pins.

2.

Evaluated by characterization.

3.

When analog peripheral blocks such as ADCs, DACs, HSE, LSE, HSI, or LSI are ON, an additional power consumption

should be considered.

4.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of 1.6 mA per ADC

for the analog part.

5.

Evaluated by test in production.

102/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 25. Typical and maximum current consumption in Run mode, code with data processing

running from Flash memory (ART accelerator disabled)

(1)

Max

Symbol

Parameter

Conditions

f

(MHz)

Typ

Unit

HCLK

TA=

TA=85 °C TA=105 °C

25 °C

180

103

112

140

151

168

98

107

126

144

150

87

95

112

128

144

85

92

108

124

120

66

71

85

99

90

54

58

69

80

All Peripherals

60

37

39

47

55

(2)(3)

enabled

30

20

24

39

51

25

17

21

35

48

16

12

16

30

42

8

7

11

24

37

4

5

8

22

35

Supply

2

3

7

21

34

I

mA

current in

DD

180

57

62

87

106

RUN mode

168

50

54

76

93

150

46

50

70

86

144

45

49

68

84

120

36

41

56

69

90

29

34

46

57

All Peripherals

60

21

24

33

41

(3)

disabled

30

13

17

31

44

25

11

15

28

41

16

8

12

25

38

8

5

9

23

35

4

4

7

21

34

2

3

6.5

20

33

1.

Evaluated by characterization unless otherwise specified.

2.

When analog peripheral blocks such as ADCs, DACs, HSE, LSE, HSI, or LSI are ON, an additional power consumption

should be considered.

3.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of 1.6 mA per ADC for

the analog part.

DS9405 Rev 13

103/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 26. Typical and maximum current consumption in Sleep mode

(1)

Max

Symbol

Parameter

Conditions

f

(MHz)

Typ

Unit

HCLK

T

=

T

=

T

=

A

A

A

25 °C

85 °C

105 °C

(3)

(3)

180

78

89

110

130

(3)

(3)

168

66

75

93

110

150

56

61

80

96

144

54

58

78

94

120

40

44

59

72

90

32

34

46

56

All

Peripherals

60

22

23

31

45

(2)

enabled

30

10

16

30

43

25

9

14

28

40

16

5

12

25

40

8

3

8

22

35

4

3

7

21

34

Supply

2

2

6.5

20

33

current in

mA

I

DD

(3)

(3)

54

76

180

21

26

Sleep mode

(3)

(3)

168

16

20

41

58

150

14

17

36

52

144

13

16.5

35

51

120

10

14

28

41

90

8

13

26

37

All

60

6

9

24

37

Peripherals

disabled

30

5

8

22

35

25

3

7

21

34

16

3

7

21

34

8

2

6

20

33

4

2

6

20

33

2

2

6

20

33

1.

Evaluated by characterization unless otherwise specified.

2.

When analog peripheral blocks such as ADCs, DACs, HSE, LSE, HSI, or LSI are ON, an additional power consumption

should be considered.

3.

Based on characterization, tested in production.

104/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 27. Typical and maximum current consumptions in Stop mode

(1)

Max

Typ

V

= 3.6 V

DD

Symbol

Parameter

Conditions

Unit

=

T

=

T

=

T

=

T

A

A

A

A

25 °C

25 °C

85 °C

105 °C

Flash memory in Stop mode, all

oscillators OFF, no independent

0.40

1.50

14.00

Supply current in Stop

watchdog

mode with voltage

25.00

regulator in main

Flash memory in Deep power

regulator mode

0.35

down mode, all oscillators OFF, no

independent watchdog

I

DD_STOP_NM

(normal mode)

1.50

14.00

25.00

Flash memory in Stop mode, all

oscillators OFF, no independent

0.29

1.10

Supply current in Stop

watchdog

mode with voltage

10.00

18.00

regulator in Low Power

Flash memory in Deep power

regulator mode

down mode, all oscillators OFF, no

0.23

1.10

10.00

18.00

mA

independent watchdog

Supply current in Stop

Flash memory in Deep power

mode with voltage

down mode, main regulator in

regulator in main

0.19

0.50

6.00

9.00

under-drive mode, all oscillators

regulator and under-

OFF, no independent  watchdog

I

DD_STOP_UDM

drive mode

(under-drive

Supply current in Stop

mode)

Flash memory in Deep power

mode with voltage

down mode, Low Power regulator

regulator in Low Power

0.10

0.40

4.00

7.00

in under-drive mode, all oscillators

regulator and under-

OFF, no independent  watchdog

drive mode

1.

Data based on characterization, tested in production.

DS9405 Rev 13

105/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 28. Typical and maximum current consumptions in Standby mode

(1)

(2)

Typ

Max

=

T

=

T

=

T

A

A

A

= 25 °C

T

A

Symbol

Parameter

Conditions

Unit

25 °C

85 °C

105 °C

=

V

=

V

=

V

DD

DD

DD

V

= 3.6 V

DD

1.7 V

2.4 V

3.3 V

Backup SRAM ON, low-speed

2.80

3.00

oscillator (LSE) and RTC ON

3.60

7.00

19.00

36.00

Backup SRAM OFF, low-

speed oscillator (LSE) and

2.30

Supply current

RTC ON

I

in Standby

DD_STBY

2.60

3.10

6.00

16.00

31.00

µA

mode

Backup SRAM ON, RTC and

(3)

(3)

(3)

2.30

2.50

LSE OFF

2.90

6.00

18.00

35.00

Backup SRAM OFF, RTC and

(3)

1.70

1.90

2.20

LSE OFF

5.00

(3)

(3)

15.00

30.00

1.

The typical current consumption values are given with PDR OFF (internal reset OFF). When the PDR is OFF (internal reset

OFF), the typical current consumption is reduced by an additional 1.2 µA.

2.

Evaluated by characterization, not tested in production unless otherwise specified.

3.

Based on characterization, tested in production.

Table 29. Typical and maximum current consumptions in V

mode

BAT

(2)

Typ

Max

=

T

A

= 25 °C

T

= 85 °C

T

(1)

A

A

Symbol

Parameter

Conditions

Unit

105 °C

=

V

=

V

=

V

BAT

BAT

BAT

V

= 3.6 V

BAT

1.7 V

2.4 V

3.3 V

Backup SRAM ON, low-speed

1.28

1.40

1.62

6

11

oscillator (LSE) and RTC ON

Backup SRAM OFF, low-speed

0.66

0.76

0.97

3

5

Backup

oscillator (LSE) and RTC ON

domain supply

I

µA

DD_VBAT

Backup SRAM ON, RTC and

current

0.70

0.72

0.74

5

10

LSE OFF

Backup SRAM OFF, RTC and

0.10

0.10

0.10

2

4

LSE OFF

1.

Crystal used: Abracon ABS07-120-32.768 kHz-T with a C

of 6 pF for typical values.

L

2.

Evaluated by characterization.

106/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 25. Typical V

current consumption (LSE and RTC ON/backup RAM OFF)

BAT

3

2.5

2

1.65V

1.7V

1.5

1.8V

2V

IDD_VBAT (μA)

1

2.4V

2.7V

3V

0.5

3.3V

3.6V

0

0°C

25°C

55°C

85°C

105°C

Temperature

MS30490V1

Figure 26. Typical V

current consumption (LSE and RTC ON/backup RAM ON)

BAT

6

5

4

IDD_VBAT (μA)

1.65V

1.7V

3

1.8V

2V

2.4V

2

2.7V

3V

1

3.3V

3.6V

0

0°C

25°C

55°C

85°C

105°C

Temperature

MS30491V1

DS9405 Rev 13

107/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Additional current consumption

The MCU is placed under the following conditions:

•

All I/O pins are configured in analog mode.

•

The flash memory access time is adjusted to fHCLK frequency.

•

The voltage scaling is adjusted to fHCLK frequency as follows:

–

Scale 3 for f

≤ 120 MHz,

HCLK

–

Scale 2 for 120 MHz < f

≤ 144 MHz

HCLK

–

Scale 1 for 144 MHz < f

≤ 180 MHz. The overdrive is only ON at 180 MHz.

HCLK

•

The system clock is HCLK, f

= f

/4, and f

= f

/2.

PCLK1

HCLK

PCLK2

HCLK

•

HSE crystal clock frequency is 25 MHz.

•

When the regulator is OFF, V12 is provided externally as described in Table 17:

General operating conditions

•

T

= 25 °C.

A

Table 30. Typical current consumption in Run mode, code with data processing running from

Flash memory or RAM, regulator ON (ART accelerator enabled except prefetch),

(1)

V

=1.7 V

DD

Symbol

Parameter

Conditions

f

(MHz)

Typ

Unit

HCLK

168

88.2

150

74.3

144

71.3

120

52.9

All Peripheral

enabled

90

42.6

60

28.6

30

15.7

Supply current in

25

12.3

mA

I

RUN mode from

DD

168

40.6

supply

V

DD

150

30.6

144

32.6

120

24.7

All Peripheral

disabled

90

19.7

60

13.6

30

7.7

25

6.7

1.

When peripherals are enabled, the power consumption corresponding to the analog part of the peripherals (such as ADC,

or DAC) is not included.

108/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 31.

Typical current consumption in Run mode, code with data processing running

(1)

from Flash memory, regulator OFF (ART accelerator enabled except prefetch)

VDD=3.3 V

VDD=1.7 V

f

HCLK

Symbol

Parameter

Conditions

Unit

(MHz)

I

I

I

I

DD12

DD

DD12

DD

168

77.8

1.3

76.8

1.0

150

70.8

1.3

69.8

1.0

144

64.5

1.3

63.6

1.0

120

49.9

1.2

49.3

0.9

All Peripherals

enabled

90

39.2

1.3

38.7

1.0

60

27.2

1.2

26.8

0.9

30

15.6

1.2

15.4

0.9

Supply current in

25

13.6

1.2

13.5

0.9

RUN mode from

I

/ I

mA

DD12

DD

V

and V

168

38.2

1.3

37.0

1.0

12

DD

supply

150

34.6

1.3

33.4

1.0

144

31.3

1.3

30.3

1.0

120

24.0

1.2

23.2

0.9

All Peripherals

disabled

90

18.1

1.4

18.0

1.0

60

12.9

1.2

12.5

0.9

30

7.2

1.2

6.9

0.9

25

6.3

1.2

6.1

0.9

1.

When peripherals are enabled, the power consumption corresponding to the analog part of the peripherals (such as ADC,

or DAC) is not included.

DS9405 Rev 13

109/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)

Table 32. Typical current consumption in Sleep mode, regulator ON, V

=1.7 V

DD

Symbol

Parameter

Conditions

f

(MHz)

Typ

Unit

HCLK

168

65.5

150

55.5

144

53.5

120

39.0

All Peripherals enabled

90

31.6

60

21.7

30

9.8

25

8.8

Supply current in Sleep

mA

I

DD

mode from V

supply

168

15.7

DD

150

13.7

144

12.7

120

9.7

All Peripherals disabled

90

7.7

60

5.7

30

4.7

25

2.8

1.

When peripherals are enabled, the power consumption corresponding to the analog part of the peripherals (such as ADC,

or DAC) is not included.

110/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)

Table 33. Tyical current consumption in Sleep mode, regulator OFF

VDD=3.3 V

VDD=1.7 V

Unit

Symbol

Parameter

Conditions

f

(MHz)

HCLK

I

I

I

I

DD12

DD

DD12

DD

180

61.5

1.4

-

-

168

59.4

1.3

59.4

1.0

150

53.9

1.3

53.9

1.0

144

49.0

1.3

49.0

1.0

All Peripherals

120

38.0

1.2

38.0

0.9

enabled

90

29.3

1.4

29.3

1.1

60

20.2

1.2

20.2

0.9

30

11.9

1.2

11.9

0.9

Supply current

25

10.4

1.2

10.4

0.9

in Sleep mode

mA

I

/I

DD12

DD

from V

and

180

14.9

1.4

-

-

12

supply

V

DD

168

14.0

1.3

14.0

1.0

150

12.6

1.3

12.6

1.0

144

11.5

1.3

11.5

1.0

All Peripherals

120

8.7

1.2

8.7

0.9

disabled

90

7.1

1.4

7.1

1.1

60

5.0

1.2

5.0

0.9

30

3.1

1.2

3.1

0.9

25

2.8

1.2

2.8

0.9

1.

When peripherals are enabled, the power consumption corresponding to the analog part of the peripherals (such as ADC,

or DAC) is not included.

DS9405 Rev 13

111/240

197

Electrical characteristics

STM32F427xx STM32F429xx

I/O system current consumption

The current consumption of the I/O system has two components: static and dynamic.

I/O static current consumption

All the I/Os used as inputs with pull resistors generate current consumption when the pin is

externally held to the opposite level. The value of this current consumption can be simply

computed by using the pull-up/pull-down resistor values given in Table 57: I/O static

characteristics.

For the output pins, any internal or external pull-up or pull-down and external load must also

be considered to estimate the current consumption.

Additional I/O current consumption is due to I/Os configured as inputs if an intermediate

voltage level is externally applied. This current consumption is caused by the input Schmitt

trigger circuits used to discriminate the input value. Unless this specific configuration is

required by the application, this supply current consumption can be avoided by configuring

these I/Os in analog mode. This is notably the case of ADC input pins, which should be

configured as analog inputs.

Caution:

Any floating input pin can also settle to an intermediate voltage level or switch inadvertently,

as a result of external electromagnetic noise. To avoid current consumption related to

floating pins, they must either be configured in analog mode, or forced internally to a definite

digital value. This can be done either by using pull-up/down resistors or by configuring the

pins in output mode.

I/O dynamic current consumption

In addition to the internal peripheral current consumption (see Table 35: Peripheral current

consumption), the I/Os used by an application also contribute to the current consumption.

When an I/O pin switches, it uses the current from the MCU supply voltage to supply the I/O

pin circuitry and to charge/discharge the capacitive load internal or external connected to

the pin:

×

×

I

V

f

C

=

SW

DD

SW

where

I

is the current sunk by a switching I/O to charge/discharge the capacitive load

SW

V

is the MCU supply voltage

DD

f

is the I/O switching frequency

SW

C is the total capacitance seen by the I/O pin: C = C

+ C

INT

EXT

The test pin is configured in push-pull output mode and is toggled by software at a fixed

frequency.

112/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)

Table 34. Switching output I/O current consumption

I/O toggling

Symbol

Parameter

Conditions

Typ

Unit

frequency

(fsw)

2 MHz

0.0

8 MHz

0.2

25 MHz

0.6

= 3.3 V

V

DD

50 MHz

1.1

(2)

C= C

INT

60 MHz

1.3

84 MHz

1.8

90 MHz

1.9

I/O switching

I

mA

DDIO

Current

2 MHz

0.1

8 MHz

0.4

= 3.3 V

V

25 MHz

1.23

DD

C

= 0 pF

EXT

50 MHz

2.43

+ C

C = C

INT

EXT

60 MHz

2.93

+ C

S

84 MHz

3.86

90 MHz

4.07

2 MHz

0.18

8 MHz

0.67

= 3.3 V

V

25 MHz

2.09

DD

= 10 pF

C

EXT

50 MHz

3.6

C = C

+ C

INT

EXT

60 MHz

4.5

+ C

S

84 MHz

7.8

90 MHz

9.8

2 MHz

0.26

I/O switching

I

mA

DDIO

Current

= 3.3 V

V

8 MHz

1.01

DD

= 22 pF

C

EXT

25 MHz

3.14

C = C

+ C

INT

EXT

50 MHz

6.39

+ C

S

60 MHz

10.68

2 MHz

0.33

= 3.3 V

V

DD

8 MHz

1.29

= 33 pF

C

EXT

C = C

+ Cext

25 MHz

4.23

INT

+ C

S

50 MHz

11.02

is the PCB board capacitance including the pad pin. C

= 7 pF (estimated value).

1.

C

S

S

2.

This test is performed by cutting the LQFP176 package pin (pad removal).

DS9405 Rev 13

113/240

197

Electrical characteristics

STM32F427xx STM32F429xx

On-chip peripheral current consumption

The MCU is placed under the following conditions:

•

At startup, all I/O pins are in analog input configuration.

•

All peripherals are disabled unless otherwise mentioned.

•

I/O compensation cell enabled.

•

The ART accelerator is ON.

•

Scale 1 mode selected, internal digital voltage V12 = 1.32 V.

•

HCLK is the system clock. f

= f

/4, and f

= f

/2.

PCLK1

HCLK

PCLK2

HCLK

The given value is calculated by measuring the difference of current consumption

–

with all the peripherals clocked off

–

with only one peripheral clocked on

–

f

= 180 MHz (scale1 + overdrive ON), f

= 144 MHz (scale 2),

HCLK

HCLK

f

= 120 MHz (scale 3)"

HCLK

•

Ambient operating temperature is 25 °C and V

=3.3 V.

DD

Table 35. Peripheral current consumption

(1)

I

( Typ)

DD

Peripheral

Unit

Scale 1

Scale 2

Scale 3

GPIOA

2.50

2.36

2.08

GPIOB

2.56

2.36

2.08

GPIOC

2.44

2.29

2.00

GPIOD

2.50

2.36

2.08

GPIOE

2.44

2.29

2.00

GPIOF

2.44

2.29

2.00

GPIOG

2.39

2.22

2.00

GPIOH

2.33

2.15

1.92

GPIOI

2.39

2.22

2.00

AHB1

µA/MHz

(up to

GPIOJ

2.33

2.15

1.92

180 MHz)

GPIOK

2.33

2.15

1.92

OTG_HS+ULPI

27.00

24.86

21.92

CRC

0.44

0.42

0.33

BKPSRAM

0.78

0.69

0.58

DMA1

25.33

23.26

20.50

DMA2

24.72

22.71

20.00

DMA2D

28.50

26.32

23.33

ETH_MAC

ETH_MAC_TX

21.56

20.07

17.75

ETH_MAC_RX

ETH_MAC_PTP

114/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 35. Peripheral current consumption (continued)

(1)

I

( Typ)

DD

Peripheral

Unit

Scale 1

Scale 2

Scale 3

OTG_FS

25.67

26.67

23.58

AHB2

DCMI

3.72

3.40

3.00

µA/MHz

(up to

180 MHz)

RNG

2.28

2.36

2.17

AHB3

FMC

21.39

19.79

17.50

µA/MHz

(up to

180 MHz)

(2)

14.06

13.19

11.75

µA/MHz

Bus matrix

TIM2

17.56

16.42

14.47

TIM3

14.22

13.36

11.80

TIM4

14.89

13.64

12.13

TIM5

17.33

16.42

14.47

TIM6

2.89

2.53

2.47

TIM7

3.11

2.81

2.47

TIM12

7.33

6.97

6.13

TIM13

4.89

4.47

4.13

TIM14

5.56

5.31

4.80

PWR

11.11

10.31

9.13

USART2

4.22

3.92

3.47

USART3

4.44

4.19

3.80

APB1

UART4

4.00

3.92

3.47

UART5

4.00

3.92

3.47

µA/MHz

(up to

45 MHz)

UART7

4.00

3.92

3.47

UART8

3.78

3.92

3.47

I2C1

4.00

3.92

3.47

I2C2

4.00

3.92

3.47

I2C3

4.00

3.92

3.47

(3)

SPI2

3.11

3.08

2.80

(3)

SPI3

3.56

3.36

3.13

I2S2

2.89

2.81

2.47

I2S3

3.33

3.08

2.80

CAN1

6.89

6.42

5.80

CAN2

6.67

6.14

5.47

(4)

DAC

2.89

2.25

2.13

WWDG

0.89

0.86

0.80

DS9405 Rev 13

115/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 35. Peripheral current consumption (continued)

(1)

I

( Typ)

DD

Peripheral

Unit

Scale 1

Scale 2

Scale 3

SDIO

8.11

8.75

7.83

TIM1

17.11

15.97

14.17

TIM8

17.33

16.11

14.33

TIM9

7.22

6.67

6.00

TIM10

4.56

4.31

3.83

TIM11

4.78

4.44

4.00

(5)

ADC1

4.67

4.31

3.83

APB2

(5)

ADC2

4.78

4.44

4.00

(5)

ADC3

4.56

4.17

3.67

µA/MHz

(up to

90 MHz)

SPI1

1.44

1.39

1.17

USART1

4.00

3.75

3.33

USART6

4.00

3.75

3.33

SPI4

1.44

1.39

1.17

SPI5

1.44

1.39

1.17

SPI6

1.44

1.39

1.17

SYSCFG

0.78

0.69

0.67

LCD_TFT

39.89

37.22

33.17

SAI1

3.78

3.47

3.17

1.

When the I/O compensation cell is ON, I

typical value increases by 0.22 mA.

DD

2.

The BusMatrix is automatically active when at least one master is ON.

3.

To enable an I2S peripheral, first set the I2SMOD bit and then the I2SE bit in the SPI_I2SCFGR register.

4.

When the DAC is ON and EN1/2 bits are set in the DAC_CR register, add an additional power

consumption of 0.8 mA per DAC channel for the analog part.

5.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of

1.6 mA per ADC for the analog part.

116/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.8

Wake-up time from low-power modes

The wake-up times given in Table 36 are measured starting from the wake-up event trigger

up to the first instruction executed by the CPU:

•

For Stop or Sleep modes: the wake-up event is WFE.

•

WKUP (PA0) pin is used to wake up from Standby, Stop, and Sleep modes.

All timings are derived from tests performed under ambient temperature and V

=3.3 V.

DD

Table 36. Low-power mode wakeup timings

(1)

(1)

Symbol

Parameter

Conditions

Unit

Typ

Max

CPU

(2)

Wakeup from Sleep

-

6

-

clock

t

WUSLEEP

cycle

Main regulator is ON

13.6

-

Main regulator is ON and Flash

93

111

memory in Deep power down mode

Wakeup from Stop mode

(2)

t

with MR/LP regulator in

WUSTOP

normal mode

Low power regulator is ON

22

32

Low power regulator is ON and Flash

103

126

memory in Deep power down mode

µs

Main regulator in under-drive mode

(Flash memory in Deep power-down

105

128

mode)

Wakeup from Stop mode

(2)

with MR/LP regulator in

t

Low power regulator in under-drive

WUSTOP

Under-drive mode

mode

125

155

(Flash memory in Deep power-down

mode )

Wakeup from Standby

tWUSTDBY

-

318

412

(2)(3)

mode

1.

Evaluated by characterization.

2.

The wake-up times are measured from the wake-up event to the point in which the application code reads the first

maximum value is given at –40 °C.

3.

t

WUSTDBY

DS9405 Rev 13

117/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.9

External clock source characteristics

High-speed external user clock generated from an external source

In bypass mode the HSE oscillator is switched off and the input pin is a standard I/O. The

external clock signal has to respect the Table 57: I/O static characteristics. However, the

recommended clock input waveform is shown in Figure 27.

The characteristics given in Table 37 result from tests performed using a high-speed

external clock source, and under ambient temperature and supply voltage conditions

summarized in Table 17.

Table 37. High-speed external user clock characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

External user clock source

1

-

50

MHz

f

(1)

HSE_ext

frequency

V

OSC_IN input pin high level voltage

0.7V

-

V

HSEH

DD

DD

V

OSC_IN input pin low level voltage

V

-

0.3V

V

HSEL

SS

DD

-

t

(1)

w(HSE)

OSC_IN high or low time

5

-

-

t

w(HSE)

ns

t

(1)

r(HSE)

OSC_IN rise or fall time

-

-

10

t

f(HSE)

(1)

C

OSC_IN input capacitance

-

-

5

-

pF

in(HSE)

Duty cycle

-

45

-

55

%

DuCy

(HSE)

OSC_IN Input leakage current

V

≤ V

≤ V

-

-

±1

µA

I

L

SS

IN

DD

1.

Specified by design.

118/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Low-speed external user clock generated from an external source

In bypass mode the LSE oscillator is switched off and the input pin is a standard I/O. The

external clock signal has to respect the Table 57: I/O static characteristics. However, the

recommended clock input waveform is shown in Figure 28.

The characteristics given in Table 38 result from tests performed using a low-speed external

clock source, and under ambient temperature and supply voltage conditions summarized in

Table 17.

Table 38. Low-speed external user clock characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

User External clock source

f

-

32.768

1000

kHz

(1)

LSE_ext

frequency

OSC32_IN input pin high level

V

-

V

0.7V

LSEH

DD

DD

voltage

V

V

OSC32_IN input pin low level voltage

-

V

-

0.3V

LSEL

SS

DD

t

(1)

w(LSE)

OSC32_IN high or low time

450

-

-

t

f(LSE)

ns

t

(1)

r(LSE)

OSC32_IN rise or fall time

-

-

50

t

f(LSE)

(1)

C

OSC32_IN input capacitance

-

-

5

-

pF

in(LSE)

Duty cycle

-

30

-

70

%

DuCy

(LSE)

I

OSC32_IN Input leakage current

V

≤ V

≤ V

-

-

±1

µA

L

SS

IN

DD

1.

Specified by design.

Figure 27. High-speed external clock source AC timing diagram

V

HSEH

90 %

10

%

V

HSEL

t

t

f(HSE)

W(HSE)

t

t

t

W(HSE)

r(HSE)

T

HSE

External

f

HSE_ext

I

L

clock source

OSC_IN

STM32F

ai17528

DS9405 Rev 13

119/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 28. Low-speed external clock source AC timing diagram

V

LSEH

90%

10%

V

LSEL

t

t

f(LSE)

W(LSE)

t

t

t

W(LSE)

r(LSE)

T

LSE

External

f

LSE_ext

I

L

OSC32_IN

clock source

STM32F

ai17529

High-speed external clock generated from a crystal/ceramic resonator

The high-speed external (HSE) clock can be supplied with a 4 to 26 MHz crystal/ceramic

resonator oscillator. All the information in this paragraph is based on the characterization

results obtained with typical external components specified in Table 39. In the application,

the resonator and the load capacitors have to be placed as close as possible to the

oscillator pins to minimize output distortion and startup stabilization time. Refer to the crystal

resonator manufacturer for more details on the resonator characteristics (frequency,

package, accuracy).

(1)

Table 39. HSE 4-26 MHz oscillator characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Oscillator frequency

-

4

-

26

MHz

OSC_IN

Feedback resistor

-

-

200

-

kΩ

R

F

V

=3.3 V,

DD

ESR= 30

,

-

450

-

Ω

C

=5 pF@25 MHz

L

I

HSE current consumption

µA

DD

=3.3 V,

V

DD

ESR= 30

,

-

530

-

Ω

C

=10 pF@25 MHz

L

(2)

ACC

HSE accuracy

-

− 500

-

500

ppm

HSE

_crit_max

Maximum critical crystal g

Startup

-

-

1

mA/V

G

m

m

(3)

Startup time

V

is stabilized

-

2

-

ms

t

SU(HSE

DD

1.

Specified by design.

2.

This parameter depends on the crystal used in the application. The minimum and maximum values must

be respected to comply with USB standard specifications.

3.

t

is the startup time measured from the moment that it is enabled (by software) until a stabilized 8

SU(HSE)

MHz oscillation is reached. This value is based on characterization and not tested in production. It is

measured for a standard crystal resonator and it can vary significantly with the crystal manufacturer.

120/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

For C

and C

, it is recommended to use high-quality external ceramic capacitors in the

L1

L2

5 pF to 25 pF range (typ.), designed for high-frequency applications, and selected to match

the requirements of the crystal or resonator (see Figure 29). C

and C

are usually the

L1

L2

same size. The crystal manufacturer typically specifies a load capacitance, which is the

series combination of C

and C

. PCB and MCU pin capacitance must be included (10 pF

L1

L2

can be used as a rough estimate of the combined pin and board capacitance) when sizing

C

and C

.

L1

L2

Note:

For information on selecting the crystal, refer to the application note AN2867 “Oscillator

design guide for ST microcontrollers” available from the ST website www.st.com.

Figure 29. Typical application with an 8 MHz crystal

Resonator with

integrated capacitors

C

L1

f

OSC_IN

HSE

Bias

8 MHz

R

controlled

F

resonator

gain

OSC_OUT

(1)

STM32F

R

EXT

C

L2

ai17530

1.

R

value depends on the crystal characteristics.

EXT

Low-speed external clock generated from a crystal/ceramic resonator

The low-speed external (LSE) clock can be supplied with a 32.768 kHz crystal/ceramic

resonator oscillator. All the information given in this paragraph are based on

characterization results obtained with typical external components specified in Table 40. In

the application, the resonator and the load capacitors have to be placed as close as

possible to the oscillator pins to minimize output distortion and startup stabilization time.

Refer to the crystal resonator manufacturer for more details on the resonator characteristics

(frequency, package, accuracy).

(1)

Table 40. LSE oscillator characteristics (f

= 32.768 kHz)

LSE

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

R

Feedback resistor

-

-

18.4

-

MΩ

F

LSE current consumption

-

-

-

1

µA

I

DD

(2)

LSE accuracy

-

− 500

-

500

ppm

ACC

LSE

G

_crit_max

Maximum critical crystal g

Startup

-

-

0.56

µA/V

m

m

(3)

startup time

V

is stabilized

-

2

-

s

t

SU(LSE)

DD

1.

Specified by design.

2.

This parameter depends on the crystal used in the application. Refer to application note AN2867.

3.

t

is the startup time measured from the moment that it is enabled (by software) to a stabilized

SU(LSE)

32.768 kHz oscillation is reached. This value is based on characterization and not tested in production. It is

measured for a standard crystal resonator and it can vary significantly with the crystal manufacturer.

Note:

For information on selecting the crystal, refer to the application note AN2867 “Oscillator

design guide for ST microcontrollers” available from the ST website www.st.com.

DS9405 Rev 13

121/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 30. Typical application with a 32.768 kHz crystal

Resonator with

integrated capacitors

C

L1

f

OSC32_IN

LSE

Bias

32.768 kHz

R

controlled

F

resonator

gain

OSC32_OUT

STM32F

C

L2

ai17531

6.3.10

Internal clock source characteristics

The parameters given in Table 41 and Table 42 are derived from tests performed under

ambient temperature and V

supply voltage conditions summarized in Table 17.

DD

High-speed internal (HSI) RC oscillator

(1)

Table 41. HSI oscillator characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Frequency

-

-

16

-

MHz

HSI

(2)

HSI user-trimming step

-

-

-

1

%

(3)

= –40 to 105 °C

− 8

-

4.5

%

T

A

ACC

HSI

(3)

T

Accuracy of the HSI oscillator

= –10 to 85 °C

− 4

-

4

%

A

(4)

= 25 °C

− 1

-

1

%

T

A

(2)

HSI oscillator startup time

-

-

2.2

4

µs

t

su(HSI)

HSI oscillator power

(2)

I

-

-

60

80

µA

DD(HSI)

consumption

= –40 to 125 °C unless otherwise specified.

1.

VDD = 3.3 V, PLL OFF, T

A

2.

Specified by design.

3.

Evaluated by characterization results.

4.

Factory calibrated, parts not soldered.

122/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 31. ACCHSI accuracy versus temperature

6

4

2

0

(%)

-40

0

25

55

85

105

125

TA (°C)

HSI

-2

ACC

-4

Min

Max

Typical

-6

-8

MSv41925V1

1.

Evaluated by characterization results.

Low-speed internal (LSI) RC oscillator

(1)

Table 42. LSI oscillator characteristics

Symbol

Parameter

Min

Typ

Max

Unit

(2)

f

Frequency

17

32

47

kHz

LSI

(3)

LSI oscillator startup time

-

15

40

µs

t

su(LSI)

(3)

I

LSI oscillator power consumption

-

0.4

0.6

µA

DD(LSI)

1.

V

= 3 V, T

= –40 to 105 °C unless otherwise specified.

DD

A

2.

Evaluated by characterization results.

3.

Specified by design.

DS9405 Rev 13

123/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 32. ACC

versus temperature

LSI

50

max

40

avg

min

30

20

10

0

-10

Normalized deviation (%)

-20

-30

-40

-45

-35

-25

-15

-5

5

15

25

35

45

55

65

75

85

95

105

Temperature (°C)

MS19013V1

6.3.11

PLL characteristics

The parameters given in Table 43 and Table 44 are derived from tests performed under

temperature and V

supply voltage conditions summarized in Table 17.

DD

Table 43. Main PLL characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

(2)

f

PLL input clock

-

0.95

1

2.10

MHz

PLL_IN

PLL multiplier output clock

-

24

-

180

MHz

f

PLL_OUT

48 MHz PLL multiplier output

f

-

-

48

75

MHz

PLL48_OUT

clock

PLL VCO output

-

100

-

432

MHz

f

VCO_OUT

VCO freq = 100 MHz

75

-

200

t

PLL lock time

µs

LOCK

VCO freq = 432 MHz

100

-

300

124/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 43. Main PLL characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

RMS

-

25

-

peak

Cycle-to-cycle jitter

to

-

±150

-

peak

System clock

120 MHz

RMS

-

15

-

peak

Period Jitter

to

-

±200

-

(3)

Jitter

ps

peak

Main clock output (MCO) for

Cycle to cycle at 50 MHz

-

32

-

RMII Ethernet

on 1000 samples

Main clock output (MCO) for MII

Cycle to cycle at 25 MHz

-

40

-

Ethernet

on 1000 samples

Cycle to cycle at 1 MHz

Bit Time CAN jitter

-

330

-

on 1000 samples

VCO freq = 100 MHz

0.15

0.40

(4)

PLL power consumption on VDD

-

mA

I

DD(PLL)

VCO freq = 432 MHz

0.45

0.75

VCO freq = 100 MHz

0.30

0.40

PLL power consumption on

(4)

I

-

mA

DDA(PLL)

VDDA

VCO freq = 432 MHz

0.55

0.85

1.

Use the appropriate division factor M to obtain the specified PLL input clock values. The M factor is shared between PLL

and PLLI2S.

2.

Specified by design.

3.

The use of 2 PLLs in parallel could degrade the Jitter up to +30%.

4.

Evaluated by characterization.

Table 44. PLLI2S (audio PLL) characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

(2)

f

PLLI2S input clock

-

0.95

1

2.10

MHz

PLLI2S_IN

PLLI2S multiplier output clock

-

-

-

216

MHz

f

PLLI2S_OUT

f

PLLI2S VCO output

-

100

-

432

MHz

VCO_OUT

VCO freq = 100 MHz

75

-

200

t

PLLI2S lock time

µs

LOCK

VCO freq = 432 MHz

100

-

300

RMS

-

90

-

-

Cycle to cycle at

12.288 MHz on

peak

48KHz period,

to

-

±280

-

ps

N=432, R=5

peak

Master I2S clock jitter

Average frequency of

(3)

Jitter

12.288 MHz

-

90

-

ps

N = 432, R = 5

on 1000 samples

Cycle to cycle at 48 KHz

WS I2S clock jitter

-

400

-

ps

on 1000 samples

DS9405 Rev 13

125/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 44. PLLI2S (audio PLL) characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

VCO freq = 100 MHz

0.15

0.40

PLLI2S power consumption on

(4)

I

-

mA

DD(PLLI2S)

V

VCO freq = 432 MHz

0.45

0.75

DD

VCO freq = 100 MHz

0.30

0.40

PLLI2S power consumption on

(4)

I

-

mA

DDA(PLLI2S)

V

VCO freq = 432 MHz

0.55

0.85

DDA

1.

Use the appropriate division factor M to have the specified PLL input clock values.

2.

Specified by design.

3.

Value given with the main PLL running.

4.

Evaluated by characterization.

Table 45. PLLISAI (audio and LCD-TFT PLL) characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

(2)

f

PLLSAI input clock

-

0.95

1

2.10

MHz

PLLSAI_IN

PLLSAI multiplier output clock

-

-

-

216

MHz

f

PLLSAI_OUT

PLLSAI VCO output

-

100

-

432

MHz

f

VCO_OUT

VCO freq = 100 MHz

75

-

200

t

PLLSAI lock time

µs

LOCK

VCO freq = 432 MHz

100

-

300

RMS

-

90

-

-

Cycle to cycle at

12.288 MHz on

peak

48KHz period,

to

-

±280

-

ps

N=432, R=5

peak

Main SAI clock jitter

Average frequency of

(3)

Jitter

12.288 MHz

-

90

-

ps

N = 432, R = 5

on 1000 samples

Cycle to cycle at 48 KHz

FS clock jitter

-

400

-

ps

on 1000 samples

VCO freq = 100 MHz

0.15

0.40

PLLSAI power consumption on

(4)

-

mA

I

DD(PLLSAI)

V

VCO freq = 432 MHz

0.45

0.75

DD

VCO freq = 100 MHz

0.30

0.40

PLLSAI power consumption on

(4)

I

-

mA

DDA(PLLSAI)

V

VCO freq = 432 MHz

0.55

0.85

DDA

1.

Use the appropriate division factor M to have the specified PLL input clock values.

2.

Specified by design.

3.

Value given with the main PLL running.

4.

Evaluated by characterization.

126/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.12

PLL spread spectrum clock generation (SSCG) characteristics

The spread spectrum clock generation (SSCG) feature allows the decrease of

electromagnetic interferences (see Table 52: EMI characteristics for fHSE= 25 MHz and

fCPU= 168 MHz). It is available only on the main PLL.

Table 46. SSCG parameters constraint

(1)

Symbol

Parameter

Min

Typ

Max

Unit

Modulation frequency

-

-

10

KHz

f

Mod

md

Peak modulation depth

0.25

-

2

%

15

MODEPER * INCSTEP

-

-

-

2

− 1

-

1.

Specified by design.

Equation 1

The frequency modulation period (MODEPER) is given by the equation below:

[

⁄

(

×

)

]

MODEPER

round f

4

f

=

PLL_IN

Mod

f

and f

must be expressed in Hz.

PLL_IN

Mod

As an example:

If f

= 1 MHz, and f

= 1 kHz, the modulation depth (MODEPER) is given by

PLL_IN

MOD

equation 1:

6

3

[

⁄

(

×

)

]

MODEPER

round 10

4

10

250

=

=

Equation 2

Equation 2 allows to calculate the increment step (INCSTEP):

15

[

(

(

)

×

×

)

⁄

(

×

×

)

]

INCSTEP

round

2

1

md

PLLN

100

5

MODEPER

=

–

f

must be expressed in MHz.

VCO_OUT

With a modulation depth (md) = ±2 % (4 % peak to peak), and PLLN = 240 (in MHz):

15

[

(

(

)

×

×

)

⁄

(

×

×

)

]

INCSTEP

round

2

1

2

240

100

5

250

126md(quantitazed)%

=

–

=

An amplitude quantization error may be generated because the linear modulation profile is

obtained by taking the quantized values (rounded to the nearest integer) of MODPER and

INCSTEP. As a result, the achieved modulation depth is quantized. The percentage

quantized modulation depth is given by the following formula:

15

(

×

×

×

)

⁄

(

(

)

×

)

md

%

MODEPER

INCSTEP

100

5

2

1

PLLN

=

–

quantized

As a result:

15

(

×

×

×

)

⁄

(

(

)

×

)

md

%

250

126

100

5

2

1

240

2.002%(peak)

=

–

=

quantized

DS9405 Rev 13

127/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 33 and Figure 34 show the main PLL output clock waveforms in center spread and

down spread modes, where:

F0 is f

nominal.

PLL_OUT

T

is the modulation period.

mode

md is the modulation depth.

Figure 33. PLL output clock waveforms in center spread mode

Frequency (PLL_OUT)

md

F0

md

Time

tmode

2xtmode

ai17291

Figure 34. PLL output clock waveforms in down spread mode

Frequency (PLL_OUT)

F0

2xmd

Time

tmode

2xtmode

ai17292b

128/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.13

Memory characteristics

Flash memory

The characteristics are given at TA = –40 to 105 °C unless otherwise specified.

The devices are shipped to customers with the flash memory erased.

Table 47. Flash memory characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Write / Erase 8-bit mode, V

= 1.7 V

-

5

-

DD

I

Supply current

= 2.1 V

-

8

-

Write / Erase 16-bit mode, V

mA

DD

DD

= 3.3 V

-

12

-

Write / Erase 32-bit mode, V

DD

Table 48. Flash memory programming

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Program/erase parallelism

(2)

t

Word programming time

-

16

100

µs

prog

(PSIZE) = x 8/16/32

Program/erase parallelism

-

400

800

(PSIZE) = x 8

Program/erase parallelism

Sector (16 KB) erase time

-

300

600

ms

t

ERASE16KB

(PSIZE) = x 16

Program/erase parallelism

-

250

500

(PSIZE) = x 32

Program/erase parallelism

-

1200

2400

(PSIZE) = x 8

Program/erase parallelism

Sector (64 KB) erase time

-

700

1400

ms

t

ERASE64KB

(PSIZE) = x 16

Program/erase parallelism

-

550

1100

(PSIZE) = x 32

Program/erase parallelism

-

2

4

(PSIZE) = x 8

Program/erase parallelism

Sector (128 KB) erase time

-

1.3

2.6

s

t

ERASE128KB

(PSIZE) = x 16

Program/erase parallelism

-

1

2

(PSIZE) = x 32

Program/erase parallelism

-

16

32

(PSIZE) = x 8

Program/erase parallelism

Mass erase time

-

11

22

s

t

ME

(PSIZE) = x 16

Program/erase parallelism

-

8

16

(PSIZE) = x 32

DS9405 Rev 13

129/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 48. Flash memory programming (continued)

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Program/erase parallelism

-

16

32

(PSIZE) = x 8

Program/erase parallelism

t

Bank erase time

-

11

22

s

BE

(PSIZE) = x 16

Program/erase parallelism

-

8

16

(PSIZE) = x 32

32-bit program operation

2.7

-

3.6

V

V

Programming voltage

16-bit program operation

2.1

-

3.6

V

prog

8-bit program operation

1.7

-

3.6

V

1.

Evaluated by characterization.

2.

The maximum programming time is measured after 100 K erase operations.

Table 49. Flash memory programming with V

PP

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(2)

t

-

16

100

Double word programming

µs

prog

t

Sector (16 KB) erase time

-

230

-

ERASE16KB

T

= 0 to +40 °C

A

Sector (64 KB) erase time

= 3.3 V

-

490

-

t

V

ms

ERASE64KB

DD

= 8.5 V

V

PP

Sector (128 KB) erase time

-

875

-

t

ERASE128KB

t

Mass erase time

-

6.9

-

s

ME

Bank erase time

-

-

6.9

-

s

t

BE

Programming voltage

-

2.7

-

3.6

V

V

prog

V

V

voltage range

-

7

-

9

V

PP

PP

Minimum current sunk on

-

10

-

-

mA

I

PP

the V

pin

PP

Cumulative time during

(3)

t

-

-

-

1

hour

VPP

which V

is applied

PP

1.

Specified by design.

2.

The maximum programming time is measured after 100 K erase operations.

3.

V

should only be connected during programming/erasing.

PP

130/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 50. Flash memory endurance and data retention

Value

Symbol

Parameter

Conditions

Unit

(1)

Min

T

= –40 to +85 °C (6 suffix versions)

A

N

Endurance

kcycles

10

END

T

= –40 to +105 °C (7 suffix versions)

A

(2)

1 kcycle

at T

= 85 °C

30

A

(2)

t

Data retention

at T

= 105 °C

10

1 kcycle

Years

RET

A

(2)

at T

= 55 °C

20

10 kcycles

A

1.

Evaluated by characterization results.

2.

Cycling performed over the whole temperature range.

6.3.14

EMC characteristics

Susceptibility tests are performed on a sample basis during device characterization.

Functional EMS (electromagnetic susceptibility)

While a simple application is executed on the device (toggling 2 LEDs through I/O ports).

Two electromagnetic events stress the device until a failure occurs. The LEDs indicate the

failure:

•

Electrostatic discharge (ESD) (positive and negative) is applied to all device pins until

a functional disturbance occurs. This test is compliant with the IEC 61000-4-2 standard.

•

FTB: A burst of fast transient voltage (positive and negative) is applied to V

and V

DD

SS

through a 100 pF capacitor, until a functional disturbance occurs. This test is compliant

with the IEC 61000-4-4 standard.

A device reset allows normal operations to be resumed.

The test results are given in Table 51. They are based on the EMS levels and classes

defined in application note AN1709.

Table 51. EMS characteristics

Level/

Symbol

Parameter

Conditions

Class

= 3.3 V, LQFP176, T

=

V

DD

A

Voltage limits to be applied on any I/O pin to

V

+25 °C, f

= 168 MHz, conforms

2B

FESD

HCLK

induce a functional disturbance

to IEC 61000-4-2

Fast transient voltage burst limits to be

V

= 3.3 V, LQFP176, T

=+25 °C,

DD

A

V

applied through 100 pF on V

and V

f

= 168 MHz, conforms to

4A

EFTB

DD

SS

HCLK

pins to induce a functional disturbance

IEC 61000-4-2

When the application is exposed to a noisy environment, it is recommended to avoid pin

exposition to disturbances. The pins showing a middle range robustness are: PA0, PA1,

PA2, PH2, PH3, PH4, PH5, PA3, PA4, PA5, PA6, PA7, PC4, and PC5.

As a consequence, it is recommended to add a serial resistor (1 kΏ) located as close as

possible to the MCU to the pins exposed to noise (connected to tracks longer than 50 mm

on PCB).

DS9405 Rev 13

131/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Designing hardened software to avoid noise problems

EMC characterization and optimization are performed at component level with a typical

application environment and simplified MCU software. It should be noted that good EMC

performance is highly dependent on the user application and the software in particular.

Therefore, it is recommended that the user applies EMC software optimization and

prequalification tests in relation with the EMC level requested for his application.

Software recommendations

The software flowchart must include the management of runaway conditions such as:

•

Corrupted program counter

•

Unexpected reset

•

Critical data corruption (control registers...)

Prequalification trials

Most of the common failures (unexpected reset and program counter corruption) can be

reproduced by manually forcing a low state on the NRST pin or the oscillator pins for 1

second.

To complete these trials, ESD stress can be applied directly on the device, over the range of

specification values. When unexpected behavior is detected, the software can be hardened

to prevent unrecoverable errors occurring (see application note AN1015).

Electromagnetic Interference (EMI)

The electromagnetic field emitted by the device is monitored while a simple application,

?

executing EEMBC

code, is running. This emission test is compliant with SAE IEC61967-2

standard, which specifies the test board and the pin loading.

Table 52. EMI characteristics for

= 25 MHz and

= 168 MHz

f

f

HSE

CPU

Max vs.

Monitored

/f

]

[f

HSE

CPU

Unit

Symbol

Parameter

Conditions

frequency band

25/168 MHz

0.1 to 30 MHz

16

V

= 3.3 V, T

= 25 °C, LQFP176

DD

A

dBµV

30 to 130 MHz

23

package, conforming to SAE J1752/3

(1)

Peak

EEMBC, ART ON, all peripheral

130 MHz to

25

clocks enabled, clock dithering

1GHz

disabled.

0.1 MHz to

(2)

4

-

Level

1GHz

S

EMI

0.1 to 30 MHz

17

VDD = 3.3 V, TA = 25 °C, LQFP176

dBµV

30 to 130 MHz

8

package, conforming to SAE J1752/3

(1)

Peak

EEMBC, ART ON, all peripheral

130 MHz to

11

clocks enabled, clock dithering

1GHz

enabled

0.1 MHz to

(2)

3.5

-

Level

1GHz

1.

Refer to chapter “EMI radiated test” in AN1709.

2.

Refer to chapter “EMI level classification” in AN1709.

132/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 53. EMI characteristics for

= 25 MHz and

= 180 MHz

f

HSE

CPU

Max vs.

Monitored

/f

]

[f

HSE

CPU

Unit

Symbol

Parameter

Conditions

frequency band

25/180 MHz

0.1 to 30 MHz

19

V

= 3.3 V, T

= 25 °C, LQFP176

DD

A

dBµV

30 to 130 MHz

23

package, conforming to SAE J1752/3

(1)

Peak

EEMBC, ART ON, all peripheral

130 MHz to

22

clocks enabled, clock dithering

1GHz

disabled.

0.1 MHz to

(2)

4

-

Level

1GHz

S

EMI

0.1 to 30 MHz

16

VDD = 3.3 V, TA = 25 °C, LQFP176

dBµV

30 to 130 MHz

10

package, conforming to SAE J1752/3

(1)

Peak

EEMBC, ART ON, all peripheral

130 MHz to

16

clocks enabled, clock dithering

1GHz

enabled

0.1 MHz to

(2)

3.5

-

Level

1GHz

1.

Refer to chapter “EMI radiated test” in AN1709.

2.

Refer to chapter “EMI level classification” in AN1709.

6.3.15

Absolute maximum ratings (electrical sensitivity)

Based on three different tests (ESD, LU) using specific measurement methods, the device is

stressed to determine its performance in terms of electrical sensitivity.

Electrostatic discharge (ESD)

Electrostatic discharges (a positive then a negative pulse separated by 1 second) are

applied to the pins of each sample according to each pin combination. The sample size

depends on the number of supply pins in the device (3 parts × (n+1) supply pins). This test

conforms to the ANSI/ESDA/JEDEC JS-001 and ANSI/ESD S5.3.1 standards.

Table 54. ESD absolute maximum ratings

Maximum

Symbol

Ratings

Conditions

Class

Unit

(1)

value

Electrostatic discharge

= +25 °C conforming to

T

A

2

2000

voltage (human body

V

ESD(HBM)

ANSI/ESDA/JEDEC JS-001

model)

T

= +25 °C conforming to ANSI/ESD S5.3.1,

A

V

C3

250

LQFP100/144/176, UFBGA169/176,

Electrostatic discharge

TFBGA176 and WLCSP143 packages

V

voltage (charge device

ESD(CDM)

model)

= +25 °C conforming to ANSI/ESD S5.3.1,

T

A

C3

250

LQFP208 package

1.

Evaluated by characterization.

DS9405 Rev 13

133/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Static latchup

Two complementary static tests are required on six parts to assess the latchup

performance:

•

A supply overvoltage is applied to each power supply pin

•

A current injection is applied to each input, output, and configurable I/O pin

These tests are compliant with the EIA/JESD 78A IC latchup standard.

Table 55. Electrical sensitivities

Symbol

Parameter

Conditions

Class

LU

Static latch-up class

T

= +105 °C conforming to JESD78A

II level A

A

6.3.16

I/O current injection characteristics

As a general rule, current injection to the I/O pins, due to external voltage below V

or

SS

above V

(for standard, 3 V-capable I/O pins) should be avoided during normal product

DD

operation. However, to give an indication of the robustness of the microcontroller in cases

when abnormal injection accidentally happens, susceptibility tests are performed on a

sample basis during device characterization.

Functional susceptibility to I/O current injection

While a simple application is executed on the device, the device is stressed by injecting

current into the I/O pins programmed in floating input mode. While current is injected into

the I/O pin, one at a time, the device is checked for functional failures.

An out of range parameter indicates the failure: ADC error above a certain limit (>5 LSB

TUE), out of conventional limits of induced leakage current on adjacent pins (out of –

5 µA/+0 µA range), or other functional failure (for example reset, oscillator frequency

deviation).

Negative induced leakage current is caused by negative injection and positive induced

leakage current by positive injection.

The test results are given in Table 56.

(1)

Table 56. I/O current injection susceptibility

Functional susceptibility

Symbol

Description

Unit

Negative

Positive

injection

injection

Injected current on BOOT0 pin

− 0

NA

Injected current on NRST pin

− 0

NA

Injected current on PA0, PA1, PA2, PA3, PA6, PA7, PB0,

I

− 0

NA

mA

INJ

PC0, PC1, PC2, PC3, PC4, PC5, PH1, PH2, PH3, PH4, PH5

Injected current on TTa pins: PA4 and PA5

− 0

+5

Injected current on any other FT pin

− 5

NA

.

1.

NA = not applicable

134/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Note:

It is recommended to add a Schottky diode (pin to ground) to analog pins, which may

potentially inject negative currents.

6.3.17

I/O port characteristics

General input/output characteristics

Unless otherwise specified, the parameters given in Table 57: I/O static characteristics are

derived from tests performed under the conditions summarized in Table 17. All I/Os are

CMOS and TTL compliant.

Note:

For information on GPIO configuration, refer to the application note AN4899 “STM32 GPIO

configuration for hardware settings and low-power consumption” available from

www.st.com.

Table 57. I/O static characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

0.35V

−0.04

DD

FT, TTa and NRST I/O input

(1)

1.7 V

V

3.6 V

-

-

≤

≤

DD

low level voltage

(2)

0.3V

DD

V

V

1.75 V

3.6 V,

IL

≤

DD

-

-

T

105 °C

–40 °C

≤

≤

BOOT0 I/O input low level

A

(1)

0.1V

+0.1

DD

voltage

1.7 V

V

3.6 V,

≤

≤

DD

-

-

0 °C

T

105 °C

≤

≤

A

(1)

0.45V

+0.3

FT, TTa and NRST I/O input

DD

1.7 V

V

3.6 V

-

-

≤

≤

(5)

DD

high level voltage

(2)

0.7V

DD

1.75 V

V

3.6 V,

≤

≤

V

V

DD

IH

T

105 °C

–40 °C

≤

≤

BOOT0 I/O input high level

A

(1)

0.17V

+0.7

-

-

DD

voltage

V

3.6 V,

1.7 V

≤

≤

DD

0 °C

T

105 °C

≤

≤

A

FT, TTa and NRST I/O input

(3)

V

3.6 V

10%V

-

-

1.7 V

≤

≤

DD

DD

hysteresis

1.75 V

V

3.6 V,

≤

≤

DD

V

V

HYS

T

105 °C

–40 °C

≤

≤

A

BOOT0 I/O input hysteresis

0.1

-

-

V

3.6 V,

1.7 V

≤

≤

DD

T

105 °C

0 °C

≤

≤

A

(4)

I/O input leakage current

V

V

V

-

-

±1

≤

≤

SS

IN

DD

I

µA

lkg

(5)

V

5 V

-

-

3

I/O FT input leakage current

=

IN

DS9405 Rev 13

135/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 57. I/O static characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

All pins

except for

30

40

50

PA10/PB12

Weak pull-up

(OTG_FS_ID,

R

equivalent

V

V

=

OTG_HS_ID)

PU

IN

SS

(6)

resistor

PA10/PB12

(OTG_FS_ID,

7

10

14

OTG_HS_ID)

kΩ

All pins

except for

30

40

50

PA10/PB12

Weak pull-

(OTG_FS_ID,

down

R

V

V

=

OTG_HS_ID)

PD

IN

DD

equivalent

(7)

resistor

PA10/PB12

(OTG_FS_ID,

7

10

14

OTG_HS_ID)

(8)

C

I/O pin capacitance

-

-

5

-

pF

IO

1.

Specified by design.

2.

Tested in production.

3.

With a minimum of 200 mV.

4.

Leakage could be higher than the maximum value, if negative current is injected on adjacent pins, refer to Table 56: I/O

current injection susceptibility

5.

To sustain a voltage higher than VDD +0.3 V, the internal pull-up/pull-down resistors must be disabled. Leakage could be

higher than the maximum value, if a negative current is injected on adjacent pins. Refer to Table 56: I/O current injection

susceptibility

6.

Pull-up resistors are designed with a true resistance in series with a switchable PMOS. This PMOS contribution to the series

resistance is minimum (~10% order).

7.

Pull-down resistors are designed with a true resistance in series with a switchable NMOS. This NMOS contribution to the

series resistance is minimum (~10% order).

8.

Hysteresis voltage between Schmitt trigger switching levels. Evaluated by characterization.

All I/Os are CMOS and TTL compliant (no software configuration required). Their

characteristics cover more than the strict CMOS-technology or TTL parameters. The

coverage of these requirements for FT I/Os is shown in Figure 35.

136/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 35. FT I/O input characteristics

VIL/VIH (V)

2.52

TTL requirement

VIHmin = 2V

2.0

1.7

1.92

Tested in production  - CMOS requirement VIHmin = 0.7VDD

Area not

1.22

1.19

determined

Based on Design simulations, VIHmin= 0.45VDD+0.3

1.065

0.8

0.55

TTL requirement VILmax

Based on Design simulations, VILmax= 0.35VDD-0.04

= 0.8V

0.51

Tested in production  - CMOS requirement VILmax = 0.3VDD

VDD (V)

1.7

2.0

2.4

2.7

3.3

3.6

MS33746V1

Output driving current

The GPIOs (general-purpose input/outputs) can sink or source up to ±8 mA, and sink or

source up to ±20 mA (with a relaxed V

/V

) except PC13, PC14, PC15, and PI8, which

OL

OH

can sink or source up to ±3mA. When using the PC13 to PC15 and PI8 GPIOs in output

mode, the speed should not exceed 2 MHz with a maximum load of 30 pF.

In the user application, the number of I/O pins, which can drive current must be limited to

respect the absolute maximum rating specified in Section 6.2. In particular:

•

The sum of the currents sourced by all the I/Os on V

plus the maximum Run

DD,

consumption of the MCU sourced on V

cannot exceed the absolute maximum rating

DD,

∑I

(see Table 15).

VDD

•

The sum of the currents sunk by all the I/Os on V

plus the maximum Run

SS

consumption of the MCU sunk on V

cannot exceed the absolute maximum rating

SS

∑I

(see Table 15).

VSS

DS9405 Rev 13

137/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Output voltage levels

Unless otherwise specified, the parameters given in Table 58 are derived from tests

performed under ambient temperature and V

supply voltage conditions summarized in

DD

Table 17. All I/Os are CMOS and TTL compliant.

Table 58. Output voltage characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

(1)

V

Output low level voltage for an I/O pin

CMOS port

-

0.4

OL

I

= +8 mA

V

IO

(3)

Output high level voltage for an I/O pin

V

− 0.4

-

V

OH

DD

V

3.6 V

2.7 V

≤

≤

DD

(1)

(2)

Output low level voltage for an I/O pin

TTL port

V

-

0.4

OL

I

=+ 8mA

V

IO

(3)

Output high level voltage for an I/O pin

2.4

-

V

OH

V

3.6 V

2.7 V

≤

≤

DD

(1)

(4)

-

1.3

Output low level voltage for an I/O pin

V

I

= +20 mA

OL

IO

V

V

3.6 V

2.7 V

(3)

(4)

≤

≤

V

Output high level voltage for an I/O pin

V

−1.3

-

DD

OH

DD

(4)

(1)

V

-

0.4

Output low level voltage for an I/O pin

I

= +6 mA

OL

IO

V

V

3.6 V

1.8 V

(3)

(4)

≤

≤

V

Output high level voltage for an I/O pin

V

−0.4

-

DD

OH

DD

(5)

(1)

V

-

0.4

Output low level voltage for an I/O pin

I

= +4 mA

OL

IO

V

V

3.6V

1.7 V

(5)

(3)

≤

≤

Output high level voltage for an I/O pin

V

−0.4

-

V

DD

OH

DD

1.

The I

current sunk by the device must always respect the absolute maximum rating specified in Table 15.

IO

and the sum of I

(I/O ports and control pins) must not exceed I

.

IO

VSS

2.

TTL and CMOS outputs are compatible with JEDEC standards JESD36 and JESD52.

3.

The I

current sourced by the device must always respect the absolute maximum rating specified in

IO

Table 15 and the sum of I

(I/O ports and control pins) must not exceed I

.

IO

VDD

4.

Based on characterization data.

5.

Specified by design.

138/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Input/output AC characteristics

The definition and values of input/output AC characteristics are given in Figure 36 and

Table 59, respectively.

Unless otherwise specified, the parameters given in Table 59 are derived from tests

performed under the ambient temperature and V

supply voltage conditions summarized

DD

in Table 17.

(1)(2)

Table 59. I/O AC characteristics

OSPEEDRy

[1:0] bit

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

value

C

= 50 pF, V

≥

2.7 V

-

-

4

L

DD

= 50 pF, V

≥ 1.7 V

-

-

2

C

L

DD

(3)

f

C

Maximum frequency

= 10 pF, V

≥ 2.7 V

-

-

8

MHz

max(IO)out

L

DD

= 10 pF, V

≥ 1.8 V

-

-

4

C

00

L

DD

C

= 10 pF, V

≥ 1.7 V

-

-

3

L

DD

Output high to low level fall

t

/

= 50 pF, V

= 1.7 V to

C

f(IO)out

L

DD

time and output low to high

-

-

100

ns

t

3.6 V

r(IO)out

level rise time

C

= 50 pF, V

≥ 2.7 V

-

-

25

L

DD

= 50 pF, V

≥ 1.8 V

-

-

12.5

C

L

DD

C

= 50 pF, V

≥ 1.7 V

-

-

10

L

DD

(3)

f

Maximum frequency

MHz

max(IO)out

= 10 pF, V

≥ 2.7 V

-

-

50

C

L

DD

= 10 pF, V

≥ 1.8 V

-

-

20

C

L

DD

01

C

= 10 pF, V

≥ 1.7 V

-

-

12.5

L

DD

= 50 pF, V

≥ 2.7 V

-

-

10

C

L

DD

Output high to low level fall

C

= 10 pF, V

≥ 2.7 V

-

-

6

/

t

L

DD

f(IO)out

ns

time and output low to high

t

C

= 50 pF, V

≥ 1.7 V

-

-

20

r(IO)out

level rise time

L

DD

= 10 pF, V

≥ 1.7 V

-

-

10

C

L

DD

(4)

C

= 40 pF, V

≥ 2.7 V

-

-

50

L

DD

(4)

C

= 10 pF, V

≥ 2.7 V

-

-

100

L

DD

(3)

Maximum frequency

C

= 40 pF, V

≥ 1.7 V

-

-

25

MHz

f

max(IO)out

L

DD

= 10 pF, V

≥ 1.8 V

-

-

50

C

L

DD

C

= 10 pF, V

≥ 1.7 V

-

-

42.5

10

L

DD

= 40 pF, V

≥2.7 V

-

-

6

C

L

DD

Output high to low level fall

C

= 10 pF, V

≥ 2.7 V

-

-

4

/

t

L

DD

f(IO)out

ns

time and output low to high

t

C

= 40 pF, V

≥ 1.7 V

-

-

10

r(IO)out

level rise time

L

DD

= 10 pF, V

≥ 1.7 V

-

-

6

C

L

DD

DS9405 Rev 13

139/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 59. I/O AC characteristics

(continued)

OSPEEDRy

[1:0] bit

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

value

(4)

C

= 30 pF, V

≥ 2.7 V

-

-

100

L

DD

= 30 pF, V

≥ 1.8 V

-

-

50

C

L

DD

C

= 30 pF, V

≥ 1.7 V

-

-

42.5

L

DD

(3)

f

Maximum frequency

MHz

max(IO)out

(4)

= 10 pF, V

≥ 2.7 V

-

-

180

C

L

DD

C

= 10 pF, V

≥ 1.8 V

-

-

100

L

DD

C

= 10 pF, V

≥ 1.7 V

-

-

72.5

L

DD

11

C

= 30 pF, V

≥ 2.7 V

-

-

4

L

DD

= 30 pF, V

≥1.8 V

-

-

6

C

L

DD

Output high to low level fall

C

= 30 pF, V

≥1.7 V

-

-

7

t

/

L

DD

f(IO)out

time and output low to high

ns

t

= 10 pF, V

≥ 2.7 V

-

-

2.5

C

r(IO)out

level rise time

L

DD

C

= 10 pF, V

≥1.8 V

-

-

3.5

L

DD

= 10 pF, V

≥1.7 V

-

-

4

C

L

DD

Pulse width of external signals

-

tEXTIpw

detected by the EXTI

-

10

-

-

ns

controller

1.

Specified by design.

2.

The I/O speed is configured using the OSPEEDRy[1:0] bits. Refer to the STM32F4xx reference manual for a description of

the GPIOx_SPEEDR GPIO port output speed register.

3.

The maximum frequency is defined in Figure 36.

V

> 2.4 V, the compensation cell should be used.

4.

For maximum frequencies above 50 MHz and

DD

Figure 36. I/O AC characteristics definition

10%

90%

50%

50%

10%

90%

t

t

r(IO)out

f(IO)out

T

Maximum frequency is achieved with a duty cycle at (45 - 55%) when loaded by the

specified capacitance.

MS32132V4

140/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.18

NRST pin characteristics

The NRST pin input driver uses CMOS technology. It is connected to a permanent pull-up

resistor, R

(see Table 57: I/O static characteristics).

PU

Unless otherwise specified, the parameters given in Table 60 are derived from tests

performed under the ambient temperature and V

supply voltage conditions summarized

DD

in Table 17.

Table 60. NRST pin characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

Weak pull-up equivalent resistor

V

V

30

40

50

kΩ

R

=

PU

IN

SS

(2)

T

NRST Input filtered pulse

-

-

-

100

ns

F(NRST)

(2)

NRST Input not filtered pulse

V

> 2.7 V

300

-

-

ns

T

NF(NRST)

DD

Generated reset pulse duration

Internal Reset source

20

-

-

µs

T

NRST_OUT

1.

The pull-up is designed with a true resistance in series with a switchable PMOS. This PMOS contribution

to the series

.

(~10% order)

resistance must be minimum

2.

Specified by design.

Figure 37. Recommended NRST pin protection

V

DD

External

(1)

reset circuit

R

PU

(2)

Internal Reset

NRST

Filter

0.1 μF

STM32F

ai14132c

1.

The reset network protects the device against parasitic resets.

2.

The external capacitor must be placed as close as possible to the device.

max level specified in

3.

The user must ensure that the level on the NRST pin can go below the V

IL(NRST)

Table 60. Otherwise, the reset is not considered by the device.

DS9405 Rev 13

141/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.19

TIM timer characteristics

The parameters given in Table 61 are specified by design.

Refer to Section 6.3.17: I/O port characteristics for details on the input/output alternate

function characteristics (output compare, input capture, external clock, PWM output).

(1)(2)

Table 61. TIMx characteristics

(3)

Symbol

Parameter

Min

Max

Unit

Conditions

AHB/APBx prescaler=1

or 2 or 4,

=

f

t

1

-

TIMxCLK

TIMxCLK

180 MHz

t

Timer resolution time

res(TIM)

AHB/APBx prescaler>4,

t

1

-

TIMxCLK

= 90 MHz

f

TIMxCLK

Timer external clock

f

/2

f

MHz

0

EXT

TIMxCLK

frequency on CH1 to CH4

f

= 180 MHz

TIMxCLK

Res

Timer resolution

-

16/32

bit

TIM

Maximum possible count

65536 ×

t

t

-

-

MAX_COUNT

TIMxCLK

with 32-bit counter

65536

1.

TIMx is used as a general term to refer to the TIM1 to TIM12 timers.

2.

Specified by design.

3.

The maximum timer frequency on APB1 or APB2 is up to 180 MHz, by setting the TIMPRE bit in the

RCC_DCKCFGR register, if APBx prescaler is 1 or 2 or 4, then TIMxCLK = HCKL, otherwise TIMxCLK =

4x PCLKx.

6.3.20

Communications interfaces

2

I

C interface

characteristics

2

2

The I

C interface meets the timing requirements of the I

C-bus specification and user

manual rev. 03 for:

•

Standard mode (Sm): with a bit rate up to 100 kbit/s

•

Fast mode (Fm): with a bit rate up to 400 kbit/s.

2

The I

C timings requirements are specified by design when the I2C peripheral is properly

configured (refer to RM0090 reference manual).

The SDA and SCL I/O requirements are met with the following restrictions: the SDA and

SCL I/O pins are not “true” open-drain. When configured as open-drain, the PMOS

connected between the I/O pin and V

is disabled, but is still present. Refer to

DD

2

C I/O characteristics

for more details on the I

.

Section 6.3.17: I/O port characteristics

2

All I

C SDA and SCL I/Os embed an analog filter. Refer to the table below for the analog

filter characteristics:

142/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)

Table 62. I2C analog filter characteristics

Symbol

Parameter

Min

Max

Unit

Maximum pulse width of spikes that

(2)

(3)

t

260

ns

50

AF

are suppressed by the analog filter

1.

Specified by design.

2.

Spikes with widths below t

are filtered.

AF(min)

3.

Spikes with widths above t

are not filtered

AF(max)

SPI interface characteristics

Unless otherwise specified, the parameters given in Table 63 for the SPI interface are

derived from tests performed under the ambient temperature, f

frequency, and V

PCLKx

DD

supply voltage conditions summarized in Table 17, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output alternate

function characteristics (NSS, SCK, MOSI, MISO for SPI).

(1)

Table 63. SPI dynamic characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Master mode, SPI1/4/5/6,

45

2.7 V≤V

≤3.6 V

DD

Receiver

45

-

-

Slave mode,

SPI1/4/5/6,

Transmitter/

(2)

f

38

≤3.6 V

2.7 V≤V

SCK

full-duplex

SPI clock frequency

MHz

DD

1/t

c(SCK)

Master mode, SPI1/2/3/4/5/6,

22.5

≤3.6 V

1.7 V≤V

DD

-

-

Slave mode, SPI1/2/3/4/5/6,

22.5

≤3.6 V

1.7 V≤V

DD

Duty cycle of SPI clock

Duty(SCK)

Slave mode

30

50

70

%

frequency

DS9405 Rev 13

143/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)

Table 63. SPI dynamic characteristics

(continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Master mode, SPI presc = 2,

t

− 0.5

+0.5

T

T

T

w(SCKH)

PCLK

PCLK

PCLK

2.7 V≤V

≤3.6 V

DD

SCK high and low time

Master mode, SPI presc = 2,

t

− 2

+2

T

T

T

w(SCKL)

PCLK

PCLK

PCLK

1.7 V≤V

≤3.6 V

DD

t

NSS setup time

Slave mode, SPI presc = 2

4

T

su(NSS)

PCLK

-

-

NSS hold time

Slave mode, SPI presc = 2

2

t

T

h(NSS)

PCLK

Master mode

3

-

-

t

su(MI)

Data input setup time

t

Slave mode

0

-

-

ns

su(SI)

Master mode

0.5

-

-

t

h(MI)

Data input hold time

t

Slave mode

2

-

-

h(SI)

t

)

Data output access time Slave mode, SPI presc = 2

0

-

4

T

a(SO

PCLK

Slave mode, SPI1/4/5/6,

0

-

8.5

2.7 V≤V

≤3.6 V

DD

t

Data output disable time

dis(SO)

Slave mode, SPI1/2/3/4/5/6 and

0

-

16.5

≤3.6 V

1.7 V≤V

DD

Slave mode (after enable edge),

-

11

13

SPI1/4/5/6 and 2.7V ≤ V

≤ 3.6V

DD

Slave mode (after enable edge),

-

14

15

≤3.6 V

SPI2/3, 2.7 V≤V

t

Data output valid/hold

DD

v(SO)

time

t

Slave mode (after enable edge),

h(SO)

-

15.5

19

≤3.6 V

SPI1/4/5/6, 1.7 V≤V

DD

Slave mode (after enable edge),

ns

-

15.5

17.5

SPI2/3, 1.7 V≤V

≤3.6 V

DD

Master mode (after enable edge),

-

-

2.5

SPI1/4/5/6, 2.7 V≤V

≤3.6 V

DD

Data output valid time

t

v(MO)

Master mode (after enable edge),

-

-

4.5

≤3.6 V

SPI1/2/3/4/5/6, 1.7 V≤V

DD

t

Master mode (after enable edge)

0

-

-

Data output hold time

h(MO)

1.

Evaluated by characterization.

and t

which has to fit into SCK

2.

The maximum frequency in Slave transmitter mode is determined by the sum of t

v(SO)

su(MI),

low or high phase preceding the SCK sampling edge. This value can be achieved when the SPI communicates with a

master having t

= 0 while Duty(SCK) = 50%.

su(MI)

144/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 38. SPI timing diagram - slave mode and CPHA = 0

NSS input

t

su(NSS)

t

t

h(NSS)

c(SCK)

t

w(SCKH)

CPHA=0

CPOL=0

CPHA=0

CPOL=1

SCK input

t

t

t

t

t

a(SO)

w(SCKL)

h(SO)

v(SO)

dis(SO)

MISO output

First bit OUT

Next bits OUT

Last bit OUT

t

t

su(SI)

h(SI)

First bit IN

Next bits IN

Last bit IN

MOSI input

MSv41658V2

Figure 39. SPI timing diagram - slave mode and CPHA = 1

NSS input

t

c(SCK)

t

h(NSS)

t

t

su(NSS)

w(SCKH)

CPHA=1

CPOL=0

CPHA=1

SCK input

CPOL=1

t

t

t

t

w(SCKL)

v(SO)

a(SO)

h(SO)

t

dis(SO)

MISO output

Last bit OUT

Next bits OUT

First bit OUT

t

t

su(SI)

h(SI)

First bit IN

Next bits IN

Last bit IN

MOSI input

MSv41659V2

DS9405 Rev 13

145/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 40. SPI timing diagram - master mode

High

NSS input

CPHA=0

t

c(SCK)

t

w(SCKH)

CPOL=0

CPHA=0

CPOL=1

SCK output

t

w(SCKL

)

CPHA=1

CPOL=0

CPHA=1

CPOL=1

SCK output

t

t

su(MI)

h(MI)

MISO input

First bit IN

Next bits IN

Last bit IN

Next bits OUT

Last bit OUT

First bit OUT

MOSI output

t

t

v(MO)

h(MO)

MSv72626V1

2

I

S interface characteristics

2

Unless otherwise specified, the parameters given in Table 64 for the I

S interface are

derived from tests performed under the ambient temperature, f

frequency, and V

PCLKx

DD

supply voltage conditions summarized in Table 17, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output alternate

function characteristics (CK, SD, WS).

2

(1)

Table 64. I

S dynamic characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

f

I2S Main clock output

-

256x8K

256xFs

MHz

MCK

Master data: 32 bits

-

64xFs

f

I2S clock frequency

MHz

CK

Slave data: 32 bits

-

64xFs

D

I2S clock frequency duty cycle

Slave receiver

30

70

%

CK

146/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

2

(1)

Table 64. I

S dynamic characteristics

(continued)

Symbol

Parameter

Conditions

Min

Max

Unit

t

WS valid time

Master mode

0

6

v(WS)

t

WS hold time

Master mode

0

-

h(WS)

WS setup time

Slave mode

1

-

t

su(WS)

WS hold time

Slave mode

0

-

t

h(WS)

t

Master receiver

7.5

-

su(SD_MR)

Data input setup time

Slave receiver

2

-

t

su(SD_SR)

ns

Master receiver

0

-

t

h(SD_MR)

Data input hold time

t

Slave receiver

0

-

h(SD_SR)

t

v(SD_ST)

Slave transmitter (after enable edge)

-

27

t

Data output valid time

h(SD_ST)

t

Master transmitter (after enable edge)

-

20

v(SD_MT)

-

t

Data output hold time

Master transmitter (after enable edge)

2.5

h(SD_MT)

1.

Evaluated by characterization.

2.

The maximum value of 256xFs is 45 MHz (APB1 maximum frequency).

Note:

Refer to the I2S section of the RM0090 reference manual for more details on the sampling

frequency (F

).

S

f

, f

, and D

values reflect only the digital peripheral behavior. The values of these

MCK

CK

CK

parameters might be slightly impacted by the source clock precision. D

depends mainly

CK

on the value of ODD bit. The digital contribution leads to a minimum value of

(I2SDIV/(2*I2SDIV+ODD) and a maximum value of (I2SDIV+ODD)/(2*I2SDIV+ODD). F

S

maximum value is supported for each mode/condition.

DS9405 Rev 13

147/240

197

Electrical characteristics

STM32F427xx STM32F429xx

2

(1)

Figure 41. I

S slave timing diagram (Philips protocol)

1.

.LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first

byte.

2

(1)

Figure 42. I

S master timing diagram (Philips protocol)

1.

LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first

byte.

148/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

SAI characteristics

Unless otherwise specified, the parameters given in Table 65 for SAI are derived from tests

performed under the ambient temperature, f

frequency, and VDD supply voltage

PCLKx

conditions summarized in Table 17, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C=30 pF

•

Measurement points are performed at CMOS levels: 0.5V

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output alternate

function characteristics (SCK,SD,WS).

(1)

Table 65. SAI characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

f

SAI Main clock output

-

256 x 8K

256xFs

MHz

MCKL

Master data: 32 bits

-

64xFs

F

SAI clock frequency

MHz

SCK

Slave  data: 32 bits

-

64xFs

SAI clock frequency duty

Slave receiver

30

70

%

D

SCK

cycle

t

FS valid time

Master mode

8

22

v(FS)

FS setup time

Slave mode

2

-

t

su(FS)

Master mode

8

-

t

FS hold time

h(FS)

Slave mode

0

-

t

Master receiver

5

-

su(SD_MR)

Data input setup time

t

Slave receiver

3

-

su(SD_SR)

t

Master receiver

0

-

ns

h(SD_MR)

Data input hold time

Slave receiver

0

-

t

h(SD_SR)

t

Slave transmitter (after enable

v(SD_ST)

-

22

edge)

t

h(SD_ST)

Data output valid time

Master transmitter (after enable

t

-

20

v(SD_MT)

edge)

Master transmitter (after enable

Data output hold time

8

-

t

h(SD_MT)

edge)

1.

Evaluated by characterization.

2.

256xFs maximum corresponds to 45 MHz (APB2 maximum frequency)

DS9405 Rev 13

149/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 43. SAI master timing waveforms

1/f

SCK

SAI_SCK_X

t

h(FS)

SAI_FS_X

(output)

t

t

t

v(FS)

v(SD_MT)

h(SD_MT)

SAI_SD_X

Slot n

Slot n+2

(transmit)

t

t

su(SD_MR)

h(SD_MR)

SAI_SD_X

Slot n

(receive)

MS32771V1

Figure 44. SAI slave timing waveforms

1/f

SCK

SAI_SCK_X

t

t

t

w(CKH_X)

w(CKL_X)

h(FS)

SAI_FS_X

(input)

t

t

t

su(FS)

v(SD_ST)

h(SD_ST)

SAI_SD_X

Slot n

Slot n+2

(transmit)

t

t

su(SD_SR)

h(SD_SR)

SAI_SD_X

(receive)

Slot n

MS32772V1

150/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

USB OTG full speed (FS) characteristics

This interface is present in both the USB OTG HS and USB OTG FS controllers.

Table 66. USB OTG full speed startup time

Symbol

Parameter

Max

Unit

(1)

t

USB OTG full speed transceiver startup time

1

µs

STARTUP

1.

Specified by design.

Table 67. USB OTG full speed DC electrical characteristics

(1)

(1)

Symbol

Parameter

Conditions

Min.

Typ.

Max.

Unit

USB OTG full speed

(2)

transceiver operating

3.0

-

3.6

V

V

DD

voltage

I(USB_FS_DP/DM,

(3)

V

Differential input sensitivity

0.2

-

-

Input

DI

USB_HS_DP/DM)

levels

Differential common mode

(3)

V

V

range

0.8

-

2.5

Includes V

CM

DI

range

Single ended receiver

(3)

V

1.3

-

2.0

SE

threshold

(4)

Ω

V

Static output level low

R

of 1.5 k

to 3.6 V

-

-

0.3

Output

OL

L

V

levels

(4)

Ω

Static output level high

R

of 15 k

to V

2.8

-

3.6

V

OH

L

SS

PA11, PA12, PB14, PB15

17

21

24

(USB_FS_DP/DM,

USB_HS_DP/DM)

R

= V

V

PD

IN

DD

PA9, PB13

(OTG_FS_VBUS,

0.65

1.1

2.0

OTG_HS_VBUS)

kΩ

PA12, PB15 (USB_FS_DP,

= V

1.5

1.8

2.1

V

IN

SS

USB_HS_DP)

R

PA9, PB13

PU

(OTG_FS_VBUS,

V

= V

0.25

0.37

0.55

IN

SS

OTG_HS_VBUS)

1.

All the voltages are measured from the local ground potential.

2.

The USB OTG full speed transceiver functionality is ensured down to 2.7 V but not the full USB full speed

electrical characteristics, which are degraded in the 2.7-to-3.0 V V

voltage range.

DD

3.

Specified by design.

is the load connected on the USB OTG full speed drivers.

R

4.

L

When the VBUS sensing feature is enabled, PA9 and PB13 should be left at their default

Note:

state (floating input), not as alternate function. A typical 200 µA current consumption of the

sensing block (current-to-voltage conversion to determine the different sessions) can be

observed on PA9 and PB13 when the feature is enabled.

DS9405 Rev 13

151/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 45. USB OTG full speed timings: definition of data signal rise and fall time

Cross over

points

Differential

data lines

V

CRS

V

SS

t

t

f

r

ai14137b

(1)

USB OTG full speed electrical characteristics

Table 68.

Driver characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

t

Rise time

C

= 50 pF

4

20

ns

r

L

(2)

t

Fall time

C

= 50 pF

4

20

ns

f

L

t

Rise/ fall time matching

t

/t

90

110

%

rfm

r

f

V

Output signal crossover voltage

-

1.3

2.0

V

CRS

Driving high or

(3)

Z

Output driver impedance

28

44

Ω

DRV

low

1.

Specified by design.

Measured from 10% to 90% of the data signal. For more detailed information, refer to USB Specification -

2.

Chapter 7 (version 2.0).

3.

No external termination series resistors are required on DP (D+) and DM (D-) pins since the matching

impedance is included in the embedded driver.

USB high speed (HS) characteristics

Unless otherwise specified, the parameters given in Table 71 for ULPI are derived from

tests performed under the ambient temperature,

frequency summarized in Table 70

f

HCLK

and V

supply voltage conditions summarized in Table 69, with the following configuration:

DD

•

Output speed is set to OSPEEDRy[1:0] = 10, unless otherwise specified

•

Capacitive load C = 30 pF, unless otherwise specified

•

Measurement points are done at CMOS levels: 0.5V

.

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output

characteristics.

Table 69. USB HS DC electrical characteristics

(1)

(1)

Symbol

Parameter

Min.

Max.

Unit

USB OTG HS operating voltage

1.7

3.6

V

Input level

V

DD

1.

All the voltages are measured from the local ground potential.

152/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)

Table 70. USB HS clock timing parameters

Symbol

Parameter

Min

Typ

Max

Unit

f

value to guarantee proper operation of

HCLK

30

-

-

MHz

USB HS interface

Frequency (first transition)

8-bit ±10%

54

60

66

MHz

F

START_8BIT

F

Frequency (steady state) ±500 ppm

59.97

60

60.03

MHz

STEADY

Duty cycle (first transition)

8-bit ±10%

40

50

60

%

D

START_8BIT

Duty cycle (steady state) ±500 ppm

49.975

50

50.025

%

D

STEADY

Time to reach the steady state frequency and

t

-

-

1.4

ms

STEADY

duty cycle after the first transition

t

Peripheral

-

-

5.6

Clock startup time after the

START_DEV

ms

de-assertion of SuspendM

Host

-

-

-

t

START_HOST

PHY preparation time after the first transition

-

-

-

µs

t

PREP

of the input clock

1.

Specified by design.

Figure 46. ULPI timing diagram

Clock

t

t

HC

SC

Control In

(ULPI_DIR,

t

t

ULPI_NXT)

SD

HD

data In

(8-bit)

t

t

DC

DC

Control out

(ULPI_STP)

t

DD

data out

(8-bit)

ai17361c

DS9405 Rev 13

153/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)

Table 71. Dynamic characteristics: USB ULPI

Symbol

Parameter

Conditions

Min.

Typ.

Max.

Unit

Control in (ULPI_DIR, ULPI_NXT) setup time

-

2

-

-

t

SC

t

Control in (ULPI_DIR, ULPI_NXT) hold time

-

0.5

-

-

HC

Data in setup time

-

1.5

-

-

t

SD

Data in hold time

-

2

-

-

t

HD

< 3.6 V,

2.7 V < V

DD

C

= 15 pF and

-

9

9.5

L

ns

OSPEEDRy[1:0] = 11

2.7 V < V

< 3.6 V,

DD

-

/t

Data/control output delay

C

= 20 pF and

t

DC

DD

L

OSPEEDRy[1:0] = 10

12

15

< 3.6 V,

1.7 V < V

DD

-

= 15 pF and

C

L

OSPEEDRy[1:0] = 11

1.

Specified by characterization.

154/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Ethernet characteristics

Unless otherwise specified, the parameters given in Table 72, Table 73 and Table 74 for

SMI, RMII and MII are derived from tests performed under the ambient temperature,

f

HCLK

frequency summarized in Table 17 with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF for 2.7 V < V

< 3.6 V

DD

•

Capacitive load C = 20 pF for 1.71 V < V

< 3.6 V

DD

•

Measurement points are done at CMOS levels: 0.5V

.

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output

characteristics.

Table 72 gives the list of Ethernet MAC signals for the SMI (station management interface)

and Figure 47 shows the corresponding timing diagram.

Figure 47. Ethernet SMI timing diagram

tMDC

ETH_MDC

td(MDIO)

ETH_MDIO(O)

tsu(MDIO)

th(MDIO)

ETH_MDIO(I)

MS31384V1

(1)

Table 72. Dynamics characteristics: Ethernet MAC signals for SMI

Symbol

Parameter

Min

Typ

Max

Unit

t

MDC cycle time(2.38 MHz)

411

420

425

MDC

T

Write data valid time

6

10

13

d(MDIO)

ns

t

Read data setup time

12

-

-

su(MDIO)

Read data hold time

0

-

-

t

h(MDIO)

1.

Evaluated by characterization.

DS9405 Rev 13

155/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 73 gives the list of Ethernet MAC signals for the RMII and Figure 48 shows the

corresponding timing diagram.

Figure 48. Ethernet RMII timing diagram

RMII_REF_CLK

t

d(TXEN)

t

d(TXD)

RMII_TX_EN

RMII_TXD[1:0]

t

t

su(RXD)

ih(RXD)

t

t

su(CRS)

ih(CRS)

RMII_RXD[1:0]

RMII_CRS_DV

ai15667b

(1)

Table 73. Dynamics characteristics: Ethernet MAC signals for RMII

Symbol

Parameter

Condition

Min

Typ

Max

Unit

t

Receive data setup time

1.5

-

-

su(RXD)

t

Receive data hold time

0

-

-

ih(RXD)

1.71 V < V

< 3.6 V

DD

t

Carrier sense setup time

1

-

-

su(CRS)

Carrier sense hold time

1

-

-

t

ih(CRS)

ns

< 3.6 V

8

10.5

12

2.7 V < V

Transmit enable valid delay

DD

t

d(TXEN)

time

1.71 V < V

< 3.6 V

8

10.5

14

DD

2.7 V < V

< 3.6 V

8

11

12.5

DD

t

Transmit data valid delay time

d(TXD)

< 3.6 V

8

11

14.5

1.71 V < V

DD

1.

Evaluated by characterization results.

Table 74 gives the list of Ethernet MAC signals for MII and Figure 48 shows the

corresponding timing diagram.

156/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 49. Ethernet MII timing diagram

MII_RX_CLK

tsu(RXD)

tih(RXD)

tsu(ER)

tih(ER)

tsu(DV)

tih(DV)

MII_RXD[3:0]

MII_RX_DV

MII_RX_ER

MII_TX_CLK

td(TXEN)

td(TXD)

MII_TX_EN

MII_TXD[3:0]

ai15668b

(1)

Table 74. Dynamics characteristics: Ethernet MAC signals for MII

Symbol

Parameter

Condition

Min

Typ

Max

Unit

t

Receive data setup time

9

-

-

su(RXD)

Receive data hold time

10

-

-

t

ih(RXD)

t

Data valid setup time

9

-

-

su(DV)

1.71 V < V

< 3.6 V

DD

Data valid hold time

8

-

-

t

ih(DV)

Error setup time

6

-

-

t

su(ER)

ns

t

Error hold time

8

-

-

ih(ER)

2.7 V < V

< 3.6 V

8

10

14

DD

t

Transmit enable valid delay time

d(TXEN)

< 3.6 V

8

10

16

1.71 V < V

DD

2.7 V < V

< 3.6 V

7.5

10

15

DD

t

Transmit data valid delay time

d(TXD)

< 3.6 V

7.5

10

17

1.71 V < V

DD

1.

Evaluated by characterization.

CAN (controller area network) interface

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output alternate

function characteristics (CANx_TX and CANx_RX).

DS9405 Rev 13

157/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.21

12-bit ADC characteristics

Unless otherwise specified, the parameters given in Table 75 are derived from tests

performed under the ambient temperature, f

frequency and V

supply voltage

PCLK2

DDA

conditions summarized in Table 17.

Table 75. ADC characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

V

Power supply

-

3.6

1.7

DDA

V

− V

< 1.2 V

DDA

REF+

(1)

V

V

Positive reference voltage

1.7

-

V

REF+

DDA

V

Negative reference voltage

-

-

0

-

REF-

(1)

= 1.7

to 2.4 V

0.6

15

18

MHz

V

DDA

ADC clock frequency

f

ADC

V

= 2.4 to 3.6 V

0.6

30

36

MHz

DDA

f

= 30 MHz,

ADC

-

-

1764

kHz

12-bit resolution

(2)

f

External trigger frequency

TRIG

-

-

-

17

1/f

ADC

0

(3)

V

Conversion voltage range

-

V

-

V

or V

(V

AIN

REF+

SSA

REF-

tied to ground)

See Equation 1 for

(2)

R

External input impedance

-

-

50

kΩ

AIN

details

(2)(4)

Sampling switch resistance

-

1.5

-

6

kΩ

R

ADC

Internal sample and hold

(2)

C

-

-

4

7

pF

ADC

capacitor

f

= 30 MHz

-

-

0.100

µs

Injection trigger conversion

ADC

(2)

t

lat

latency

(5)

-

-

-

3

1/f

ADC

f

= 30 MHz

-

-

0.067

µs

Regular trigger conversion

ADC

(2)

t

latr

latency

(5)

-

-

-

2

1/f

ADC

= 30 MHz

0.100

-

16

µs

f

ADC

(2)

t

Sampling time

S

-

3

-

480

1/f

ADC

(2)

t

Power-up time

-

-

2

3

µs

STAB

= 30 MHz

f

ADC

0.50

-

16.40

µs

12-bit resolution

f

= 30 MHz

ADC

0.43

-

16.34

µs

10-bit resolution

f

Total conversion time (including

= 30 MHz

(2)

ADC

t

0.37

-

16.27

µs

CONV

sampling time)

8-bit resolution

f

= 30 MHz

ADC

0.30

-

16.20

µs

6-bit resolution

9 to 492 (t

for sampling +n-bit resolution for successive

S

1/f

ADC

approximation)

158/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 75. ADC characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

12-bit resolution

-

-

2

Msps

Single ADC

12-bit resolution

Sampling rate

-

-

3.75

Msps

Interleave Dual ADC

(2)

f

= 30 MHz, and

(f

S

ADC

mode

= 3 ADC cycles)

t

S

12-bit resolution

-

-

6

Msps

Interleave Triple ADC

mode

ADC V

DC current

REF

(2)

consumption in conversion

-

-

300

500

µA

I

VREF+

mode

ADC V

DC current

DDA

(2)

I

consumption in conversion

-

-

1.6

1.8

mA

VDDA

mode

1.

V

minimum value of 1.7 V is obtained with the use of an external power supply supervisor (refer to Section 3.17.2:

DDA

Internal reset OFF).

2.

Evaluated by characterization results.

is internally connected to V

and V

is internally connected to V

.

3.

V

REF+

DDA

REF-

SSA

maximum value is given for V

=1.7 V, and a minimum value for V

=3.3 V.

4.

R

ADC

DD

DD

must be added to the latency specified in Table 75.

5.

For external triggers, a delay of 1/f

PCLK2

Equation 1: R

max formula

AIN

(

)

k

0.5

–

=

----------------------------------------------------------------

R

R

–

AIN

ADC

2

N

+

×

×

(

)

f

C

ln

2

ADC

ADC

The formula above (Equation 1) is used to determine the maximum external impedance

allowed for an error below 1/4 of LSB. N = 12 (from 12-bit resolution) and k is the number of

sampling periods defined in the ADC_SMPR1 register.

Table 76. ADC static accuracy at f

= 18 MHz

ADC

(1)

Symbol

Parameter

Test conditions

Typ

Max

Unit

ET

Total unadjusted error

±3

±4

f

=18 MHz

ADC

EO

Offset error

±2

±3

= 1.7 to 3.6 V

V

DDA

LSB

EG

Gain error

±1

±3

= 1.7 to 3.6 V

V

REF

V

− V

< 1.2 V

ED

Differential linearity error

±1

±2

DDA

REF

EL

Integral linearity error

±2

±3

1.

Evaluated by characterization - not tested in production results.

DS9405 Rev 13

159/240

197

Electrical characteristics

STM32F427xx STM32F429xx

a

Table 77. ADC static accuracy at f

= 30 MHz

ADC

(1)

Symbol

Parameter

Test conditions

Typ

Max

Unit

ET

Total unadjusted error

±2

±5

f

= 30 MHz,

ADC

EO

Offset error

±1.5

±2.5

R

< 10 kΩ

AIN

EG

Gain error

±1.5

±3

LSB

V

= 2.4 to 3.6 V,

DDA

= 1.7 to 3.6 V,

V

REF

ED

Differential linearity error

±1

±2

V

− V

< 1.2 V

DDA

REF

EL

Integral linearity error

±1.5

±3

1.

Evaluated by characterization.

Table 78. ADC static accuracy at f

= 36 MHz

ADC

(1)

Symbol

Parameter

Test conditions

Typ

Max

Unit

ET

Total unadjusted error

±4

±7

f

=36 MHz,

ADC

EO

Offset error

±2

±3

= 2.4 to 3.6 V,

V

DDA

LSB

EG

Gain error

±3

±6

= 1.7 to 3.6 V

V

REF

ED

Differential linearity error

±2

±3

V

− V

< 1.2 V

DDA

REF

EL

Integral linearity error

±3

±6

1.

Evaluated by characterization.

(1)

Table 79. ADC dynamic accuracy at f

= 18 MHz - limited test conditions

ADC

Symbol

Parameter

Test conditions

Min

Typ

Max

Unit

ENOB

Effective number of bits

10.3

10.4

-

bits

f

=18 MHz

ADC

SINAD

Signal-to-noise and distortion ratio

64

64.2

-

V

= V

= 1.7 V

DDA

REF+

Input Frequency = 20 KHz

SNR

Signal-to-noise ratio

64

65

-

dB

Temperature = 25 °C

THD

Total harmonic distortion

− 67

− 72

-

1.

Evaluated by characterization.

(1)

Table 80. ADC dynamic accuracy at f

= 36 MHz - limited test conditions

ADC

Symbol

Parameter

Test conditions

Min

Typ

Max

Unit

ENOB

Effective number of bits

10.6

10.8

-

bits

f

=36 MHz

ADC

SINAD

Signal-to noise and distortion ratio

66

67

-

V

= V

= 3.3 V

DDA

REF+

Input Frequency = 20 KHz

SNR

Signal-to noise ratio

64

68

-

dB

Temperature = 25 °C

THD

Total harmonic distortion

− 70

− 72

-

1.

Evaluated by characterization.

160/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Note:

ADC accuracy vs. negative injection current: injecting a negative current on any analog

input pins should be avoided as this significantly reduces the accuracy of the conversion

being performed on another analog input. It is recommended to add a Schottky diode (pin to

ground) to analog pins, which may potentially inject negative currents.

Any positive injection current within the limits specified for I

and ∑I

in

INJ(PIN)

INJ(PIN)

Section 6.3.17 does not affect the ADC accuracy.

Figure 50. ADC accuracy characteristics

V

V

DDA

REF+

[1LSB

=

(or              depending on package)]

IDEAL

4096

4096

E

G

4095

4094

4093

(2)

E

T

(3)

7

(1)

6

5

E

E

O

L

4

3

E

D

2

1L SB

IDEAL

1

0

1

2

3

456

7

4093 4094 4095 4096

V

V

SSA

DDA

ai14395c

1.

See also Table 77.

2.

Example of an actual transfer curve.

3.

Ideal transfer curve.

4.

End-point correlation line.

= Total Unadjusted Error: maximum deviation between the actual and the ideal transfer curves.

5.

E

T

EO = Offset Error: deviation between the first actual transition and the first ideal one.

EG = Gain Error: deviation between the last ideal transition and the last actual one.

ED = Differential Linearity Error: maximum deviation between actual steps and the ideal one.

EL = Integral Linearity Error: maximum deviation between any actual transition and the end-point

correlation line.

DS9405 Rev 13

161/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 51. Typical connection diagram when using the ADC with FT/TT pins

featuring the analog switch function

(4)

(4)

V

V

REF+

DDA

Sample-and-hold ADC converter

I/O

analog

(1)

switch

R

R

ADC

AIN

(2)

Sampling

V

AIN

switch with

multiplexing

(3)

C

I

parasitic

ADC

lkg

Converter

C

V

V

V

SS

SS

SSA

MSv67871V3

1.

Refer to Table 75: ADC characteristics for the values of R

, RADC, and C

.

AIN

ADC

represents the capacitance of the PCB (dependent on soldering and PCB layout quality) plus the

2.

C

parasitic

pad capacitance (refer to Table 57: I/O static characteristics). A high C

value downgrades

parasitic

should be reduced.

conversion accuracy. To remedy this, f

ADC

3.

Refer to Table 57: I/O static characteristics for the value of I

Ikg.

4.

Refer to Figure 22: Power supply scheme.

162/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

General PCB design guidelines

Power supply decoupling should be performed as shown in Figure 52 or Figure 53,

depending on whether V

is connected to V

or not. The 10 nF capacitors should be

REF+

DDA

ceramic (good quality). They should be placed them as close as possible to the chip.

Figure 52. Power supply and reference decoupling (V

V

not connected to

)

REF+

DDA

STM32F

(1)

V

REF+

1 μF // 10 nF

V

DDA

1 μF // 10 nF

(1)

V

/V

SSA

REF-

ai17535b

1.

V

and V

inputs are both available on UFBGA176. V

is also available on LQFP100, LQFP144,

REF+

REF–

REF+

and LQFP176. When V

and V

are not available, they are internally connected to V

and V

.

REF+

REF–

DDA

SSA

DS9405 Rev 13

163/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 53. Power supply and reference decoupling

(V

V

)

connected to

REF+

DDA

STM32F

(1)

V

/V

REF+

DDA

1 μF // 10 nF

(1)

V

/V

REF-

SSA

ai17536c

1.

V

and V

inputs are both available on UFBGA176. V

is also available on LQFP100, LQFP144,

REF+

REF–

REF+

and LQFP176. When V

and V

are not available, they are internally connected to V

and V

.

REF+

REF–

DDA

SSA

6.3.22

Temperature sensor characteristics

Table 81. Temperature sensor characteristics

Symbol

Parameter

Min

Typ

Max

Unit

(1)

T

V

linearity with temperature

-

±1

±2

°C

L

SENSE

(1)

Average slope

-

2.5

-

mV/°C

Avg_Slope

(1)

Voltage at 25 °C

-

0.76

-

V

V

25

(2)

t

Startup time

-

6

10

µs

START

(2)

ADC sampling time when reading the temperature (1 °C accuracy)

10

-

-

µs

T

S_temp

1.

Evaluated by characterization.

2.

Specified by design.

Table 82. Temperature sensor calibration values

Symbol

Parameter

Memory address

TS_CAL1

TS ADC raw data acquired at temperature of 30 °C, V

= 3.3 V

0x1FFF 7A2C - 0x1FFF 7A2D

DDA

= 3.3 V

0x1FFF 7A2E - 0x1FFF 7A2F

TS_CAL2

TS ADC raw data acquired at temperature of 110 °C, V

DDA

164/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.23

V

monitoring characteristics

BAT

Table 83. V

monitoring characteristics

BAT

Symbol

Parameter

Min

Typ

Max

Unit

-

50

-

KΩ

R

Resistor bridge for V

BAT

Q

Ratio on V

measurement

-

4

-

-

BAT

(1)

Error on Q

–1

-

+1

%

Er

ADC sampling time when reading the V

(2)(2)

BAT

5

-

-

µs

T

S_vbat

1 mV accuracy

1.

Specified by design.

2.

The shortest sampling time can be determined in the application by multiple iterations.

6.3.24

Reference voltage

The parameters given in Table 84 are derived from tests performed under ambient

temperature and V

supply voltage conditions summarized in Table 17.

DD

Table 84.  internal reference voltage

Symbol

Parameter

Conditions

Min

Max

Unit

Typ

V

Internal reference voltage

–40 °C < T

< +105 °C

1.18

1.21

1.24

V

REFINT

A

ADC sampling time when reading the

(1)

T

-

10

-

-

µs

S_vrefint

internal reference voltage

Internal reference voltage spread over the

(2)

V

= 3V ± 10mV

-

3

5

mV

V

RERINT_s

DD

temperature range

(2)

T

Temperature coefficient

-

-

30

50

ppm/°C

Coeff

(2)

Startup time

-

-

6

10

µs

t

START

1.

The shortest sampling time can be determined in the application by multiple iterations.

2.

Specified by design, not tested in production

Table 85. Internal reference voltage calibration values

Symbol

Parameter

Memory address

Raw data acquired at temperature of 30 °C

= 3.3 V

0x1FFF 7A2A - 0x1FFF 7A2B

V

REFIN_CAL

VDDA

DS9405 Rev 13

165/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.25

DAC electrical characteristics

Table 86. DAC characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Comments

(1)

V

Analog supply voltage

-

1.7

-

3.6

V

-

DDA

Reference supply

(1)

-

1.7

V

-

3.6

V

V

≤ V

REF+

REF+

DDA

voltage

V

Ground

-

0

-

0

V

-

SSA

R

LOAD

connected

5

-

-

-

to V

DAC output

SSA

(2)

R

Resistive load

kΩ

LOAD

buffer ON

R

LOAD

connected

25

-

to V

DDA

When the buffer is OFF, the

Minimum resistive load

Impedance output with

(2)

R

-

-

-

15

kΩ

between DAC_OUT and V

O

SS

buffer OFF

to have a 1% accuracy is

1.5 MΩ

Maximum capacitive load at

(2)

Capacitive load

-

-

-

50

pF

DAC_OUT pin (when the

C

LOAD

buffer is ON).

DAC_O

It gives the maximum output

Lower DAC_OUT

UT

-

0.2

-

-

V

excursion of the DAC.

voltage with buffer ON

(2)

min

It corresponds to 12-bit input

code (0x0E0) to (0xF1C) at

DAC_O

Higher DAC_OUT

V

= 3.6 V and (0x1C7) to

V

DDA

REF+

-

-

-

V

UT

voltage with buffer ON

− 0.2

(0xE38) at V

= 1.7 V

(2)

REF+

max

DAC_O

Lower DAC_OUT

UT

voltage with buffer

-

-

0.5

-

mV

(2)

min

OFF

It gives the maximum output

excursion of the DAC.

DAC_O

Higher DAC_OUT

V

REF+

UT

voltage with buffer

-

-

-

−

V

(2)

max

OFF

1LSB

With no load, worst code

= 3.6 V in

(0x800) at V

REF+

-

-

170

240

terms of DC consumption on

DAC DC V

current

REF

the inputs

consumption in

(4)

I

µA

VREF+

quiescent mode

With no load, worst code

(Standby mode)

= 3.6 V in

(0xF1C) at V

REF+

-

-

50

75

terms of DC consumption on

the inputs

166/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 86. DAC characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Comments

With no load, middle code

-

-

280

380

µA

(0x800) on the inputs

DAC DC VDDA

With no load, worst code

(4)

I

current consumption in

DDA

= 3.6 V in

(0xF1C) at V

(3)

quiescent mode

REF+

-

-

475

625

µA

terms of DC consumption on

the inputs

Differential non

Given for the DAC in 10-bit

-

-

-

±0.5

LSB

linearity Difference

configuration.

(4)

between two

DNL

consecutive code-

Given for the DAC in 12-bit

-

-

-

±2

LSB

1LSB)

configuration.

Integral non linearity

Given for the DAC in 10-bit

-

-

-

±1

LSB

(difference between

configuration.

measured value at

(4)

Code i and the value

INL

Given for the DAC in 12-bit

at Code i on a line

-

-

-

±4

LSB

configuration.

drawn between Code

0 and last Code 1023)

Given for the DAC in 12-bit

-

-

-

±10

mV

Offset error

configuration

(difference between

Given for the DAC in 10-bit at

(4)

-

-

-

±3

LSB

Offset

measured value at

V

= 3.6 V

REF+

Code (0x800) and the

Given for the DAC in 12-bit at

/2)

ideal value = V

REF+

-

-

-

±12

LSB

= 3.6 V

V

REF+

Gain

Given for the DAC in 12-bit

Gain error

-

-

-

±0.5

%

(4)

error

configuration

Settling time (full

scale: for a 10-bit input

code transition

t

≤ 50 pF,

between the lowest

C

SETTLIN

LOAD

-

-

3

6

µs

(4)

≥ 5 kΩ

R

and the highest input

G

LOAD

codes when

DAC_OUT reaches

final value ±4LSB

Total Harmonic

≤ 50 pF,

C

(4)

LOAD

Distortion

THD

-

-

-

-

dB

≥ 5 kΩ

R

LOAD

Buffer ON

Max frequency for a

correct DAC_OUT

Update

≤ 50 pF,

change when small

MS/

C

LOAD

-

-

-

1

(2)

rate

R

≥ 5 kΩ

variation in the input

s

LOAD

code (from code i to

i+1LSB)

DS9405 Rev 13

167/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Table 86. DAC characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Comments

Wakeup time from off

≤ 50 pF, R

≥ 5 kΩ

C

(

LOAD

LOAD

t

state (Setting the ENx

WAKEUP

input code between lowest and

-

-

6.5

10

µs

4)

bit in the DAC Control

highest possible ones.

register)

Power supply rejection

PSRR+

ratio (to V

) (static

-

-

–67

–40

dB No R

, C

= 50 pF

(2)

DDA

LOAD

LOAD

DC measurement)

1.

V

minimum value of 1.7 V is obtained with the use of an external power supply supervisor (refer to Section 3.17.2:

DDA

Internal reset OFF).

2.

Specified by design.

3.

The quiescent mode corresponds to a state where the DAC maintains a stable output level to ensure that no dynamic

consumption occurs.

4.

Evaluated by characterization - not tested in production.

Figure 54. 12-bit buffered /non-buffered DAC

Buffered/non-buffered DAC

(1)

Buffer

R

LOAD

12-bit

DACx_OUT

digital to

analog

converter

C

LOAD

ai17157a

1.

The DAC integrates an output buffer that can be used to reduce the output impedance and to drive external loads directly

without the use of an external operational amplifier. The buffer can be bypassed by configuring the BOFFx bit in the

DAC_CR register.

168/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.26

FMC characteristics

Unless otherwise specified, the parameters given in Table 87 to Table 102 for the FMC

interface are derived from tests performed under the ambient temperature, f

frequency,

HCLK

and V

supply voltage conditions summarized in Table 17, with the following configuration:

DD

•

Output speed is set to OSPEEDRy[1:0] = 10 except at V

range 1.7 to 2.1 V where

DD

OSPEEDRy[1:0] = 11

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output

characteristics.

Asynchronous waveforms and timings

Figure 55 through Figure 58 represent asynchronous waveforms and Table 87 through

Table 94 provide the corresponding timings. The results shown in these tables are obtained

with the following FMC configuration:

•

AddressSetupTime = 0x1

•

AddressHoldTime = 0x1

•

DataSetupTime = 0x1 (except for asynchronous NWAIT mode, DataSetupTime = 0x5)

•

BusTurnAroundDuration = 0x0

•

For SDRAM memories, V

ranges from 2.7 to 3.6 V and maximum frequency

DD

FMC_SDCLK = 90 MHz

•

For Mobile LPSDR SDRAM memories, V

ranges from 1.7 to 1.95 V and maximum

DD

frequency FMC_SDCLK = 84 MHz

DS9405 Rev 13

169/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 55. Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms

t

w(NE)

FMC_NE

t

t

t

w(NOE)

h(NE_NOE)

v(NOE_NE)

FMC_NOE

FMC_NWE

t

t

v(A_NE)

h(A_NOE)

FMC_A[25:0]

Address

t

v(BL_NE)

t

h(BL_NOE)

FMC_NBL[1:0]

t

h(Data_NE)

t

t

su(Data_NOE)

h(Data_NOE)

t

su(Data_NE)

Data

FMC_D[15:0]

t

v(NADV_NE)

t

w(NADV)

(1)

FMC_NADV

FMC_NWAIT

th(NE_NWAIT)

tsu(NWAIT_NE)

MS32753V1

1.

Mode 2/B, C, and D only. In Mode 1, FMC_NADV is not used.

Table 87. Asynchronous non-multiplexed SRAM/PSRAM/NOR -

(1)(2)

read timings

Symbol

Parameter

Min

Max

Unit

t

FMC_NE low time

2T

− 0.5

2 T

+0.5

ns

w(NE)

HCLK

HCLK

FMC_NEx low to FMC_NOE low

0

1

ns

t

v(NOE_NE)

FMC_NOE low time

2T

2T

+ 0.5

ns

t

w(NOE)

HCLK

HCLK

t

FMC_NOE high to FMC_NE high hold time

0

-

ns

h(NE_NOE)

FMC_NEx low to FMC_A valid

-

2

ns

t

v(A_NE)

Address hold time after FMC_NOE high

0

-

ns

t

h(A_NOE)

t

FMC_NEx low to FMC_BL valid

-

2

ns

v(BL_NE)

FMC_BL hold time after FMC_NOE high

0

-

ns

t

h(BL_NOE)

t

Data to FMC_NEx high setup time

T

+ 2.5

-

ns

su(Data_NE)

HCLK

Data to FMC_NOEx high setup time

T

+2

-

ns

t

su(Data_NOE)

HCLK

170/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 87. Asynchronous non-multiplexed SRAM/PSRAM/NOR -

(1)(2)

read timings

(continued)

Symbol

Parameter

Min

Max

Unit

t

Data hold time after FMC_NOE high

0

-

ns

h(Data_NOE)

t

Data hold time after FMC_NEx high

0

-

ns

h(Data_NE)

FMC_NEx low to FMC_NADV low

-

0

ns

t

v(NADV_NE)

FMC_NADV low time

-

T

+1

ns

t

w(NADV)

HCLK

1.

C

= 30 pF.

L

2.

Evaluated by characterization.

Table 88. Asynchronous non-multiplexed SRAM/PSRAM/NOR read -

(1)(2)

NWAIT timings

Symbol

Parameter

Min

Max

Unit

FMC_NE low time

7T

+0.5

7T

+1

t

w(NE)

HCLK

HCLK

FMC_NWE low time

5T

− 1.5

5T

+2

t

w(NOE)

HCLK

HCLK

ns

t

FMC_NWAIT valid before FMC_NEx high

5T

+1.5

-

su(NWAIT_NE)

HCLK

FMC_NEx hold time after FMC_NWAIT

4T

t

+1

-

h(NE_NWAIT)

HCLK

invalid

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

DS9405 Rev 13

171/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 56. Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms

t

w(NE)

FMC_NEx

FMC_NOE

t

t

t

h(NE_NWE)

v(NWE_NE)

w(NWE)

FMC_NWE

t

th(A_NWE)

v(A_NE)

FMC_A[25:0]

Address

t

t

v(BL_NE)

h(BL_NWE)

FMC_NBL[1:0]

NBL

t

t

v(Data_NE)

h(Data_NWE)

Data

FMC_D[15:0]

t

v(NADV_NE)

t

w(NADV)

(1)

FMC_NADV

FMC_NWAIT

th(NE_NWAIT)

tsu(NWAIT_NE)

MS32754V1

1.

Mode 2/B, C, and D only. In Mode 1, FMC_NADV is not used.

(1)(2)

Table 89. Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_NE low time

3T

3T

+1

ns

w(NE)

HCLK

HCLK

t

FMC_NEx low to FMC_NWE low

T

− 0.5

T

+ 0.5

ns

v(NWE_NE)

HCLK

HCLK

FMC_NWE low time

T

T

+ 0.5

ns

t

w(NWE)

HCLK

HCLK

FMC_NWE high to FMC_NE high hold time

T

+1.5

-

ns

t

h(NE_NWE)

HCLK

t

FMC_NEx low to FMC_A valid

-

0

ns

v(A_NE)

Address hold time after FMC_NWE high

T

+0.5

-

ns

t

h(A_NWE)

HCLK

FMC_NEx low to FMC_BL valid

-

1.5

ns

t

v(BL_NE)

t

FMC_BL hold time after FMC_NWE high

T

+0.5

-

ns

h(BL_NWE)

HCLK

Data to FMC_NEx low to Data valid

-

T

+ 2

ns

t

v(Data_NE)

HCLK

Data hold time after FMC_NWE high

T

+0.5

-

ns

t

h(Data_NWE)

HCLK

t

FMC_NEx low to FMC_NADV low

-

0.5

ns

v(NADV_NE)

FMC_NADV low time

-

T

+ 0.5

ns

t

w(NADV)

HCLK

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

172/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 90. Asynchronous non-multiplexed SRAM/PSRAM/NOR write -

(1)(2)

NWAIT timings

Symbol

Parameter

Min

Max

Unit

FMC_NE low time

8T

+1

8T

+2

ns

t

w(NE)

HCLK

HCLK

t

FMC_NWE low time

6T

− 1

6T

+2

ns

w(NWE)

HCLK

HCLK

t

FMC_NWAIT valid before FMC_NEx high

6T

+1.5

-

ns

su(NWAIT_NE)

HCLK

FMC_NEx hold time after FMC_NWAIT

4T

t

+1

-

ns

h(NE_NWAIT)

HCLK

invalid

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

Figure 57. Asynchronous multiplexed PSRAM/NOR read waveforms

t

w(NE)

FMC_

NE

t

t

h(NE_NOE)

v(NOE_NE)

FMC_NOE

t

w(NOE)

FMC_NWE

t

t

h(A_NOE)

v(A_NE)

FMC_

A[25:16]

Address

t

t

v(BL_NE)

h(BL_NOE)

FMC_

NBL[1:0]

NBL

t

h(Data_NE)

t

su(Data_NE)

t

t

t

v(A_NE)

su(Data_NOE)

h(Data_NOE)

AD[15:0]

Address

Data

FMC_

t

t

h(AD_NADV)

v(NADV_NE)

t

w(NADV)

FMC_NADV

FMC_NWAIT

th(NE_NWAIT)

tsu(NWAIT_NE)

MS32755V1

DS9405 Rev 13

173/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 91. Asynchronous multiplexed PSRAM/NOR read timings

Symbol

Parameter

Min

Max

Unit

t

FMC_NE low time

3T

− 1

3T

+0.5

ns

w(NE)

HCLK

HCLK

FMC_NEx low to FMC_NOE low

2T

− 0.5

2T

ns

t

v(NOE_NE)

HCLK

HCLK

FMC_NOE low time

T

− 1

T

+1

ns

t

tw(NOE)

HCLK

HCLK

t

FMC_NOE high to FMC_NE high hold time

1

-

ns

h(NE_NOE)

FMC_NEx low to FMC_A valid

-

2

ns

t

v(A_NE)

FMC_NEx low to FMC_NADV low

0

2

ns

t

v(NADV_NE)

t

FMC_NADV low time

T

− 0.5

T

+0.5

ns

w(NADV)

HCLK

HCLK

FMC_AD(address) valid hold time after

0

-

ns

t

h(AD_NADV)

FMC_NADV high)

t

Address hold time after FMC_NOE high

T

− 0.5

-

ns

h(A_NOE)

HCLK

FMC_BL time after FMC_NOE high

0

-

ns

t

h(BL_NOE)

t

FMC_NEx low to FMC_BL valid

-

2

ns

v(BL_NE)

Data to FMC_NEx high setup time

T

+1.5

-

ns

t

su(Data_NE)

HCLK

Data to FMC_NOE high setup time

T

+1

-

ns

t

su(Data_NOE)

HCLK

t

Data hold time after FMC_NEx high

0

-

ns

h(Data_NE)

Data hold time after FMC_NOE high

0

-

ns

t

h(Data_NOE)

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

(1)(2)

Table 92. Asynchronous multiplexed PSRAM/NOR read-NWAIT timings

Symbol

Parameter

Min

Max

Unit

FMC_NE low time

8T

+0.5

8T

+2

ns

t

w(NE)

HCLK

HCLK

FMC_NWE low time

5T

− 1

5T

+1.5

ns

t

w(NOE)

HCLK

HCLK

t

FMC_NWAIT valid before FMC_NEx high

5T

+1.5

-

ns

su(NWAIT_NE)

HCLK

FMC_NEx hold time after FMC_NWAIT

4T

t

+1

-

ns

h(NE_NWAIT)

HCLK

invalid

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

174/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 58. Asynchronous multiplexed PSRAM/NOR write waveforms

t

w(NE)

FMC_

NEx

FMC_NOE

t

t

t

h(NE_NWE)

v(NWE_NE)

w(NWE)

FMC_NWE

t

t

h(A_NWE)

v(A_NE)

FMC_

A[25:16]

Address

t

t

v(BL_NE)

h(BL_NWE)

FMC_

NBL[1:0]

NBL

t

t

t

v(A_NE)

v(Data_NADV)

h(Data_NWE)

Address

Data

FMC_

AD[15:0]

t

t

h(AD_NADV)

v(NADV_NE)

t

w(NADV)

FMC_NADV

FMC_NWAIT

th(NE_NWAIT)

tsu(NWAIT_NE)

MS32756V1

(1)(2)

Table 93. Asynchronous multiplexed PSRAM/NOR write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_NE low time

4T

4T

+0.5

ns

w(NE)

HCLK

HCLK

FMC_NEx low to FMC_NWE low

T

− 1

T

+0.5

ns

t

v(NWE_NE)

HCLK

HCLK

t

FMC_NWE low time

2T

2T

+0.5

ns

w(NWE)

HCLK

HCLK

FMC_NWE high to FMC_NE high hold time

T

-

ns

t

h(NE_NWE)

HCLK

FMC_NEx low to FMC_A valid

-

0

ns

t

v(A_NE)

t

FMC_NEx low to FMC_NADV low

0.5

1

ns

v(NADV_NE)

FMC_NADV low time

T

− 0.5

T

+ 0.5

ns

t

w(NADV)

HCLK

HCLK

FMC_AD(adress) valid hold time after FMC_NADV high)

T

− 2

-

ns

t

h(AD_NADV)

HCLK

t

Address hold time after FMC_NWE high

T

-

ns

h(A_NWE)

HCLK

FMC_BL hold time after FMC_NWE high

T

− 2

-

ns

t

h(BL_NWE)

HCLK

FMC_NEx low to FMC_BL valid

-

2

ns

t

v(BL_NE)

t

FMC_NADV high to Data valid

-

T

+1.5

ns

v(Data_NADV)

HCLK

Data hold time after FMC_NWE high

T

+0.5

-

ns

t

h(Data_NWE)

HCLK

1.

C

= 30 pF.

L

2.

Evaluated by characterization.

DS9405 Rev 13

175/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 94. Asynchronous multiplexed PSRAM/NOR write-NWAIT timings

Symbol

Parameter

Min

Max

Unit

FMC_NE low time

9T

9T

+0.5

ns

t

w(NE)

HCLK

HCLK

FMC_NWE low time

7T

7T

+2

ns

t

w(NWE)

HCLK

HCLK

t

FMC_NWAIT valid before FMC_NEx high

6T

+1.5

-

ns

su(NWAIT_NE)

HCLK

FMC_NEx hold time after FMC_NWAIT

4T

t

–1

-

ns

h(NE_NWAIT)

HCLK

invalid

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

Synchronous waveforms and timings

Figure 59 through Figure 62 represent synchronous waveforms and Table 95 through

Table 98 provide the corresponding timings. The results shown in these tables are obtained

with the following FMC configuration:

•

BurstAccessMode = FMC_BurstAccessMode_Enable;

•

MemoryType = FMC_MemoryType_CRAM;

•

WriteBurst = FMC_WriteBurst_Enable;

•

CLKDivision = 1; (0 is not supported. See the STM32F4xx reference manual: RM0090)

•

DataLatency = 1 for NOR flash; DataLatency = 0 for PSRAM

In all timing tables, the

is the HCLK clock period (with maximum

T

HCLK

FMC_CLK = 90 MHz).

176/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 59. Synchronous multiplexed NOR/PSRAM read timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FMC_CLK

Data latency = 0

t

d(CLKL-NExL)

td(CLKH-NExH)

FMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FMC_NADV

t

td(CLKH-AIV)

d(CLKL-AV)

FMC_A[25:16]

t

td(CLKH-NOEH)

d(CLKL-NOEL)

FMC_NOE

t

t

d(CLKL-ADIV)

h(CLKH-ADV)

t

t

t

t

su(ADV-CLKH)

su(ADV-CLKH)

d(CLKL-ADV)

h(CLKH-ADV)

FMC_AD[15:0]

AD[15:0]

D1

D2

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FMC_NWAIT

(WAITCFG = 1b,

WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FMC_NWAIT

(WAITCFG = 0b,

WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

MS32757V1

(1)(2)

Table 95. Synchronous multiplexed NOR/PSRAM read timings

Symbol

Parameter

Min

Max

Unit

t

FMC_CLK period

2T

− 1

-

ns

w(CLK)

HCLK

FMC_CLK low to FMC_NEx low (x=0..2)

-

0

ns

t

d(CLKL-NExL)

FMC_CLK high to FMC_NEx high (x= 0…2)

T

-

ns

t

d(CLKH_NExH)

HCLK

t

FMC_CLK low to FMC_NADV low

-

0

ns

d(CLKL-NADVL)

FMC_CLK low to FMC_NADV high

0

-

ns

t

d(CLKL-NADVH)

FMC_CLK low to FMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

t

FMC_CLK high to FMC_Ax invalid (x=16…25)

0

-

ns

d(CLKH-AIV)

FMC_CLK low to FMC_NOE low

-

T

+0.5

ns

t

d(CLKL-NOEL)

HCLK

FMC_CLK high to FMC_NOE high

T

− 0.5

-

ns

t

d(CLKH-NOEH)

HCLK

t

FMC_CLK low to FMC_AD[15:0] valid

-

0.5

ns

d(CLKL-ADV)

FMC_CLK low to FMC_AD[15:0] invalid

0

-

ns

t

d(CLKL-ADIV)

DS9405 Rev 13

177/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 95. Synchronous multiplexed NOR/PSRAM read timings

(continued)

Symbol

Parameter

Min

Max

Unit

FMC_A/D[15:0] valid data before FMC_CLK

t

5

-

ns

su(ADV-CLKH)

high

FMC_A/D[15:0] valid data after FMC_CLK high

0

-

ns

t

h(CLKH-ADV)

FMC_NWAIT valid before FMC_CLK high

4

-

ns

t

su(NWAIT-CLKH)

t

FMC_NWAIT valid after FMC_CLK high

0

-

ns

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization.

Figure 60. Synchronous multiplexed PSRAM write timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FMC_CLK

Data latency = 0

t

t

d(CLKL-NExL)

d(CLKH-NExH)

FMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FMC_NADV

t

d(CLKH-AIV)

t

d(CLKL-AV)

FMC_A[25:16]

t

t

d(CLKH-NWEH)

d(CLKL-NWEL)

FMC_NWE

t

t

d(CLKL-ADIV)

d(CLKL-Data)

t

t

d(CLKL-Data)

d(CLKL-ADV)

AD[15:0]

D1

D2

FMC_AD[15:0]

FMC_NWAIT

(WAITCFG = 0b,

WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

t

d(CLKH-NBLH)

FMC_NBL

MS32758V1

178/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)(2)

Table 96. Synchronous multiplexed PSRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_CLK period,  VDD range= 2.7 to 3.6 V

2T

− 1

-

ns

w(CLK)

HCLK

FMC_CLK low to FMC_NEx low (x=0..2)

-

1.5

ns

t

d(CLKL-NExL)

FMC_CLK high to FMC_NEx high (x= 0…2)

T

-

ns

t

d(CLKH-NExH)

HCLK

t

FMC_CLK low to FMC_NADV low

-

0

ns

d(CLKL-NADVL)

FMC_CLK low to FMC_NADV high

0

-

ns

t

d(CLKL-NADVH)

FMC_CLK low to FMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

t

FMC_CLK high to FMC_Ax invalid (x=16…25)

T

-

ns

d(CLKH-AIV)

HCLK

FMC_CLK low to FMC_NWE low

-

0

ns

t

d(CLKL-NWEL)

FMC_CLK high to FMC_NWE high

T

−0.5

-

ns

t

(CLKH-NWEH)

HCLK

t

FMC_CLK low to FMC_AD[15:0] valid

-

3

ns

d(CLKL-ADV)

FMC_CLK low to FMC_AD[15:0] invalid

0

-

ns

t

d(CLKL-ADIV)

t

FMC_A/D[15:0] valid data after FMC_CLK low

-

3

ns

d(CLKL-DATA)

FMC_CLK low to FMC_NBL low

0

-

ns

t

d(CLKL-NBLL)

FMC_CLK high to FMC_NBL high

T

−0.5

-

ns

t

d(CLKH-NBLH)

HCLK

t

FMC_NWAIT valid before FMC_CLK high

4

-

ns

su(NWAIT-CLKH)

FMC_NWAIT valid after FMC_CLK high

0

-

ns

t

h(CLKH-NWAIT)

= 30 pF.

1.

C

L

2.

Evaluated by characterization.

DS9405 Rev 13

179/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 61. Synchronous non-multiplexed NOR/PSRAM read timings

t

t

w(CLK)

w(CLK)

FMC_CLK

t

t

d(CLKL-NExL)

d(CLKH-NExH)

Data latency = 0

FMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FMC_NADV

t

t

d(CLKH-AIV)

d(CLKL-AV)

FMC_A[25:0]

t

t

d(CLKL-NOEL)

d(CLKH-NOEH)

FMC_NOE

t

t

su(DV-CLKH)

h(CLKH-DV)

t

t

su(DV-CLKH)

h(CLKH-DV)

D1

D2

FMC_D[15:0]

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FMC_NWAIT

(WAITCFG = 1b,

WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FMC_NWAIT

(WAITCFG = 0b,

WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

MS32759V1

(1)(2)

Table 97. Synchronous non-multiplexed NOR/PSRAM read timings

Symbol

Parameter

Min

Max

Unit

t

FMC_CLK period

2T

− 1

-

ns

w(CLK)

HCLK

FMC_CLK low to FMC_NEx low (x=0..2)

-

0.5

ns

t

(CLKL-NExL)

t

d(CLKH-

FMC_CLK high to FMC_NEx high (x= 0…2)

T

-

ns

HCLK

NExH)

t

d(CLKL-

FMC_CLK low to FMC_NADV low

-

0

ns

NADVL)

t

d(CLKL-

FMC_CLK low to FMC_NADV high

0

-

ns

NADVH)

t

FMC_CLK low to FMC_Ax valid (x=16…25)

-

0

ns

d(CLKL-AV)

FMC_CLK high to FMC_Ax invalid (x=16…25)

T

− 0.5

-

ns

t

d(CLKH-AIV)

HCLK

t

FMC_CLK low to FMC_NOE low

-

T

+2

ns

d(CLKL-NOEL)

HCLK

t

d(CLKH-

FMC_CLK high to FMC_NOE high

T

− 0.5

-

ns

HCLK

NOEH)

t

FMC_D[15:0] valid data before FMC_CLK high

5

-

ns

su(DV-CLKH)

180/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)(2)

Table 97. Synchronous non-multiplexed NOR/PSRAM read timings

(continued)

Symbol

Parameter

Min

Max

Unit

t

FMC_D[15:0] valid data after FMC_CLK high

0

-

ns

h(CLKH-DV)

t

FMC_NWAIT valid before FMC_CLK high

4

-

-

(NWAIT-CLKH)

t

h(CLKH-

FMC_NWAIT valid after FMC_CLK high

0

-

-

NWAIT)

1.

C

= 30 pF.

L

2.

Guaranteed by characterization results.

Figure 62. Synchronous non-multiplexed PSRAM write timings

t

t

w(CLK)

w(CLK)

FMC_CLK

t

t

d(CLKL-NExL)

d(CLKH-NExH)

Data latency = 0

FMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FMC_NADV

t

d(CLKH-AIV)

t

d(CLKL-AV)

FMC_A[25:0]

t

t

d(CLKH-NWEH)

d(CLKL-NWEL)

FMC_NWE

t

t

d(CLKL-Data)

d(CLKL-Data)

FMC_D[15:0]

D1

D2

FMC_NWAIT

(WAITCFG = 0b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

d(CLKH-NBLH)

t

h(CLKH-NWAITV)

FMC_NBL

MS32760V1

(1)(2)

Table 98. Synchronous non-multiplexed PSRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_CLK period

2T

− 1

-

ns

(CLK)

HCLK

FMC_CLK low to FMC_NEx low (x=0..2)

-

0.5

ns

t

d(CLKL-NExL)

FMC_CLK high to FMC_NEx high (x= 0…2)

T

-

ns

t

(CLKH-NExH)

HCLK

t

FMC_CLK low to FMC_NADV low

-

0

ns

d(CLKL-NADVL)

FMC_CLK low to FMC_NADV high

0

-

ns

t

d(CLKL-NADVH)

FMC_CLK low to FMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

DS9405 Rev 13

181/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 98. Synchronous non-multiplexed PSRAM write timings

(continued)

Symbol

Parameter

Min

Max

Unit

t

FMC_CLK high to FMC_Ax invalid (x=16…25)

0

-

ns

d(CLKH-AIV)

t

FMC_CLK low to FMC_NWE low

-

0

ns

d(CLKL-NWEL)

FMC_CLK high to FMC_NWE high

T

−0.5

-

ns

t

d(CLKH-NWEH)

HCLK

FMC_D[15:0] valid data after FMC_CLK low

-

2.5

ns

t

d(CLKL-Data)

t

FMC_CLK low to FMC_NBL low

0

-

ns

d(CLKL-NBLL)

FMC_CLK high to FMC_NBL high

T

−0.5

-

ns

t

d(CLKH-NBLH)

HCLK

FMC_NWAIT valid before FMC_CLK high

4

-

-

t

su(NWAIT-CLKH)

t

FMC_NWAIT valid after FMC_CLK high

0

-

-

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization.

PC Card/CompactFlash controller waveforms and timings

Figure 63 through Figure 68 represent synchronous waveforms, and Table 99 and

Table 100 provide the corresponding timings. The results shown in this table are obtained

with the following FMC configuration:

•

COM.FMC_SetupTime = 0x04;

•

COM.FMC_WaitSetupTime = 0x07;

•

COM.FMC_HoldSetupTime = 0x04;

•

COM.FMC_HiZSetupTime = 0x00;

•

ATT.FMC_SetupTime = 0x04;

•

ATT.FMC_WaitSetupTime = 0x07;

•

ATT.FMC_HoldSetupTime = 0x04;

•

ATT.FMC_HiZSetupTime = 0x00;

•

IO.FMC_SetupTime = 0x04;

•

IO.FMC_WaitSetupTime = 0x07;

•

IO.FMC_HoldSetupTime = 0x04;

•

IO.FMC_HiZSetupTime = 0x00;

•

TCLRSetupTime = 0;

•

TARSetupTime = 0.

In all timing tables, the

is the HCLK clock period.

T

HCLK

182/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 63. PC Card/CompactFlash controller waveforms for common memory read

access

(1)

FMC_NCE4_2

FMC_NCE4_1

t

h(NCEx-AI)

t

v(NCEx-A)

FMC_A[10:0]

t

h(NCEx-NREG)

t

d(NREG-NCEx)

t

h(NCEx-NIORD)

t

d(NIORD-NCEx)

t

h(NCEx-

NIOWR

)

FMC_NREG

FMC_NIOWR

FMC_NIORD

FMC_NWE

t

t

d(NCE4_1-NOE)

w(NOE)

FMC_N

OE

t

t

su(D-NOE)

h(NOE-D)

FMC_D[15:0]

MS32761V1

1.

FMC_NCE4_2 remains high (inactive during 8-bit access.

Figure 64. PC Card/CompactFlash controller waveforms for common memory write

access

FMC_NCE4_1

FMC_NCE4_2

High

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FMC_A[10:0]

t

h(NCE4_1-NREG)

t

d(NREG-NCE4_1)

t

h(NCE4_1-NIORD)

t

d(NIORD-NCE4_1)

t

h(NCE4_1-NIOWR)

FMC_NREG

FMC_NIOWR

FMC_NIORD

t

t

t

d(NCE4_1-NWE)

w(NWE)

d(NWE-NCE4_1)

FMC_NWE

FMC_N

OE

MEMxHIZ =1

t

d(D-NWE)

t

t

v(NWE-D)

h(NWE-D)

FMC_D[15:0]

MS32762V1

DS9405 Rev 13

183/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 65. PC Card/CompactFlash controller waveforms for attribute memory

read access

FMC_NCE4_1

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FMC_NCE4_2

High

FMC_A[10:0]

FMC_NIOWR

FMC_NIORD

t

t

d(NREG-NCE4_1)

h(NCE4_1-NREG)

FMC_NREG

FMC_NWE

t

t

t

d(NCE4_1-NOE)

w(NOE)

d(NOE-NCE4_1)

FMC_NOE

t

t

su(D-NOE)

h(NOE-D)

(1)

FMC_D[15:0]

MS32763V1

1.

Only data bits 0...7 are read (bits 8...15 are disregarded).

184/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 66. PC Card/CompactFlash controller waveforms for attribute memory

write access

FMC_NCE4_1

High

FMC_NCE4_2

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FMC_A[10:0]

FMC_NIOWR

FMC_NIORD

t

t

d(NREG-NCE4_1)

h(NCE4_1-NREG)

FMC_NREG

t

t

d(NCE4_1-NWE)

w(NWE)

FMC_NWE

t

d(NWE-NCE4_1)

FMC_NOE

t

v(NWE-D)

FMC_D[7:0](1)

MS32764V1

1.

Only data bits 0...7 are driven (bits 8...15 remains Hi-Z).

Figure 67. PC Card/CompactFlash controller waveforms for I/O space read access

FMC_NCE4_1

FMC_NCE4_2

t

t

h(NCE4_1-AI)

v(NCEx-A)

FMC_A[10:0]

FMC_NREG

FMC_NWE

FMC_NOE

FMC_NIOWR

t

t

w(NIORD)

d(NIORD-NCE4_1)

FMC_NIORD

t

t

su(D-NIORD)

d(NIORD-D)

FMC_D[15:0]

MS32765V1

DS9405 Rev 13

185/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 68. PC Card/CompactFlash controller waveforms for I/O space write access

FMC_NCE4_1

FMC_NCE4_2

t

t

v(NCEx-A)

h(NCE4_1-AI)

FMC_A[10:0]

FMC_NREG

FMC_NWE

FMC_NOE

FMC_NIORD

t

t

d(NCE4_1-NIOWR)

w(NIOWR)

FMC_NIOWR

ATTxHIZ =1

t

h(NIOWR-D)

t

v(NIOWR-D)

FMC_D[15:0]

MS32766V1

Table 99. Switching characteristics for PC Card/CF read and write cycles

(1)(2)

in attribute/common space

Symbol

Parameter

Min

Max

Unit

t

FMC_Ncex low to FMC_Ay valid

-

0

ns

v(NCEx-A)

FMC_NCEx high to FMC_Ax invalid

0

-

ns

t

h(NCEx_AI)

FMC_NCEx low to FMC_NREG valid

-

1

ns

t

d(NREG-NCEx)

t

FMC_NCEx high to FMC_NREG invalid

T

− 2

-

ns

h(NCEx-NREG)

HCLK

FMC_NCEx low to FMC_NWE low

-

5T

ns

t

d(NCEx-NWE)

HCLK

FMC_NWE low width

8T

− 0.5

8T

+0.5

ns

t

w(NWE)

HCLK

HCLK

t

FMC_NWE high to FMC_NCEx high

5T

+1

-

ns

d(NWE_NCEx)

HCLK

FMC_NWE low to FMC_D[15:0] valid

-

0

ns

t

V(NWE-D)

t

FMC_NWE high to FMC_D[15:0] invalid

9T

− 0.5

-

ns

h(NWE-D)

HCLK

FMC_D[15:0] valid before FMC_NWE high

13T

− 3

-

ns

t

d(D-NWE)

HCLK

t

FMC_NCEx low to FMC_NOE low

-

5T

ns

d(NCEx-NOE)

HCLK

t

FMC_NOE low width

8 T

− 0.5

8 T

+0.5

ns

w(NOE)

HCLK

HCLK

t

FMC_NOE high to FMC_NCEx high

5T

− 1

-

ns

d(NOE_NCEx)

HCLK

FMC_D[15:0] valid data before FMC_NOE high

T

-

ns

t

su (D-NOE)

HCLK

FMC_NOE high to FMC_D[15:0] invalid

0

-

ns

t

h(NOE-D)

1.

C

= 30 pF.

L

2.

Guaranteed by characterization results.

186/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Table 100. Switching characteristics for PC Card/CF read and write cycles

(1)(2)

in I/O space

Symbol

Parameter

Min

Max

Unit

tw(NIOWR)

FMC_NIOWR low width

8T

− 0.5

-

ns

HCLK

tv(NIOWR-D)

FMC_NIOWR low to FMC_D[15:0] valid

-

0

ns

− 2

-

ns

th(NIOWR-D)

FMC_NIOWR high to FMC_D[15:0] invalid

9T

HCLK

td(NCE4_1-NIOWR)

FMC_NCE4_1 low to FMC_NIOWR valid

-

5T

ns

HCLK

-

ns

th(NCEx-NIOWR)

FMC_NCEx high to FMC_NIOWR invalid

5T

HCLK

ns

td(NIORD-NCEx)

FMC_NCEx low to FMC_NIORD valid

-

5T

HCLK

th(NCEx-NIORD)

FMC_NCEx high to FMC_NIORD) valid

6T

+2

-

ns

HCLK

− 0.5

8T

+0.5

ns

tw(NIORD)

FMC_NIORD low width

8T

HCLK

HCLK

-

ns

tsu(D-NIORD)

FMC_D[15:0] valid before FMC_NIORD high

T

HCLK

td(NIORD-D)

FMC_D[15:0] valid after FMC_NIORD high

0

-

ns

1.

C

= 30 pF.

L

2.

Evaluated by characterization.

NAND controller waveforms and timings

Figure 69 and Figure 70 represent synchronous waveforms, and Table 101 and Table 102

provide the corresponding timings. The results shown in this table are obtained with the

following FMC configuration:

•

COM.FMC_SetupTime = 0x01;

•

COM.FMC_WaitSetupTime = 0x03;

•

COM.FMC_HoldSetupTime = 0x02;

•

COM.FMC_HiZSetupTime = 0x01;

•

ATT.FMC_SetupTime = 0x01;

•

ATT.FMC_WaitSetupTime = 0x03;

•

ATT.FMC_HoldSetupTime = 0x02;

•

ATT.FMC_HiZSetupTime = 0x01;

•

Bank = FMC_Bank_NAND;

•

MemoryDataWidth = FMC_MemoryDataWidth_16b;

•

ECC = FMC_ECC_Enable;

•

ECCPageSize = FMC_ECCPageSize_512Bytes;

•

TCLRSetupTime = 0;

•

TARSetupTime = 0.

In all timing tables, the

is the HCLK clock period.

T

HCLK

DS9405 Rev 13

187/240

197

Electrical characteristics

STM32F427xx STM32F429xx

Figure 69. NAND controller waveforms for read access

FMC_NCEx

ALE (FMC_A17)

CLE (FMC_A16)

t

t

h(NOE-ALE)

d(ALE-NOE)

FMC_NWE

t

w(NOE)

FMC_NOE (NRE)

t

t

su(D-NOE)

h(NOE-D)

FMC_D[y:0]

MSv73150V1

1.

y= 7 or 15 depending on the NAND flash memory interface.

Figure 70. NAND controller waveforms for write access

FMC_NCEx

ALE (FMC_A17)

CLE (FMC_A16)

t

t

t

d(ALE-NWE)

w(NWE)

h(NWE-ALE)

FMC_NWE

FMC_NOE (NRE)

t

d(D-NWE)

t

t

v(NWE-D)

h(NWE-D)

FMC_D[y:0]

MSv73151V1

2.

y= 7 or 15 depending on the NAND flash memory interface.

(1)

N

Table 101. Switching characteristics for NAND Flash read cycles

Symbol

Parameter

Min

Max

Unit

t

FMC_NOE low width

4T

− 0.5

4T

+0.5

ns

w(N0E)

HCLK

HCLK

FMC_D[15-0] valid data before FMC_NOE high

9

-

ns

t

su(D-NOE)

t

FMC_D[15-0] valid data after FMC_NOE high

0

-

ns

h(NOE-D)

FMC_ALE valid before FMC_NOE low

-

3T

− 0.5

ns

t

d(ALE-NOE)

HCLK

FMC_NWE high to FMC_ALE invalid

3T

− 2

-

ns

t

h(NOE-ALE)

HCLK

= 30 pF.

1.

C

L

188/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)

Table 102. Switching characteristics for NAND Flash write cycles

Symbol

Parameter

Min

Max

Unit

t

FMC_NWE low width

4T

4T

+1

ns

w(NWE)

HCLK

HCLK

FMC_NWE low to FMC_D[15-0] valid

0

-

ns

t

v(NWE-D)

FMC_NWE high to FMC_D[15-0] invalid

3T

− 1

-

ns

t

h(NWE-D)

HCLK

t

FMC_D[15-0] valid before FMC_NWE high

5T

− 3

-

ns

d(D-NWE)

HCLK

FMC_ALE valid before FMC_NWE low

-

3T

−0.5

ns

t

d(ALE-NWE)

HCLK

FMC_NWE high to FMC_ALE invalid

3T

− 1

-

ns

t

h(NWE-ALE)

HCLK

1.

C

= 30 pF.

L

SDRAM waveforms and timings

Figure 71. SDRAM read access waveforms (CL = 1)

FMC_SDCLK

td(SDCLKL_AddC)

td(SDCLKL_AddR)

th(SDCLKL_AddR)

Row n

Col1

Col2

Coli

FMC_A[12:0]

Coln

th(SDCLKL_AddC)

td(SDCLKL_SNDE)

FMC_SDNE[1:0]

th(SDCLKL_SNDE)

th(SDCLKL_NRAS)

td(SDCLKL_NRAS)

FMC_SDNRAS

th(SDCLKL_NCAS)

td(SDCLKL_NCAS)

FMC_SDNCAS

FMC_SDNWE

tsu(SDCLKH_Data)

th(SDCLKH_Data)

Data1

Data2

FMC_D[31:0]

Datai

Datan

MS32751V2

DS9405 Rev 13

189/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 103. SDRAM read timings

Symbol

Parameter

Min

Max

Unit

FMC_SDCLK period

2T

− 0.5

2T

+0.5

t

w(SDCLK)

HCLK

HCLK

t

Data input setup time

2

-

su(SDCLKH _Data)

t

Data input hold time

0

-

h(SDCLKH_Data)

Address valid time

-

1.5

t

d(SDCLKL_Add)

Chip select valid time

-

0.5

t

d(SDCLKL- SDNE)

ns

t

Chip select hold time

0

-

h(SDCLKL_SDNE)

SDNRAS valid time

-

0.5

t

d(SDCLKL_SDNRAS)

t

SDNRAS hold time

0

-

h(SDCLKL_SDNRAS)

SDNCAS valid time

-

0.5

t

d(SDCLKL_SDNCAS)

t

SDNCAS hold time

0

-

h(SDCLKL_SDNCAS)

1.

CL = 30 pF on data and address lines. CL=15pF on FMC_SDCLK.

2.

Evaluated by characterization.

(1)(2)

Table 104. LPSDR SDRAM read timings

Symbol

Parameter

Min

Max

Unit

t

FMC_SDCLK period

2T

− 0.5

2T

+0.5

W(SDCLK)

HCLK

HCLK

t

Data input setup time

2.5

-

su(SDCLKH_Data)

t

Data input hold time

0

-

h(SDCLKH_Data)

t

Address valid time

-

1

d(SDCLKL_Add)

t

Chip select valid time

-

1

d(SDCLKL_SDNE)

ns

Chip select hold time

1

-

t

h(SDCLKL_SDNE)

t

SDNRAS valid time

-

1

d(SDCLKL_SDNRAS

SDNRAS hold time

1

-

t

h(SDCLKL_SDNRAS)

t

SDNCAS valid time

-

1

d(SDCLKL_SDNCAS)

SDNCAS hold time

1

-

t

h(SDCLKL_SDNCAS)

1.

CL = 10 pF.

2.

Evaluated by characterization.

190/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 72. SDRAM write access waveforms

FMC_SDCLK

td(SDCLKL_AddC)

td(SDCLKL_AddR)

th(SDCLKL_AddR)

Row n

Col2

Coli

FMC_A[12:0]

Col1

Coln

th(SDCLKL_AddC)

td(SDCLKL_SNDE)

FMC_SDNE[1:0]

th(SDCLKL_NRAS)

th(SDCLKL_SNDE)

td(SDCLKL_NRAS)

FMC_SDNRAS

th(SDCLKL_NCAS)

td(SDCLKL_NCAS)

FMC_SDNCAS

td(SDCLKL_NWE)

th(SDCLKL_NWE)

FMC_SDNWE

td(SDCLKL_Data)

Data1

Data2

FMC_D[31:0]

Datai

Datan

th(SDCLKL_Data)

td(SDCLKL_NBL)

FMC_NBL[3:0]

MS32752V2

DS9405 Rev 13

191/240

197

Electrical characteristics

STM32F427xx STM32F429xx

(1)(2)

Table 105. SDRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_SDCLK  period

2T

− 0.5

2T

+0.5

w(SDCLK)

HCLK

HCLK

t

)

Data output valid time

-

3.5

d(SDCLKL _Data

Data output hold time

0

-

t

h(SDCLKL _Data)

t

Address valid time

-

1.5

d(SDCLKL_Add)

SDNWE valid time

-

1

t

d(SDCLKL_SDNWE)

SDNWE hold time

0

-

t

h(SDCLKL_SDNWE)

t

Chip  select valid time

-

0.5

d(SDCLKL_ SDNE)

ns

Chip  select hold time

0

-

t

h(SDCLKL-_SDNE)

SDNRAS valid time

-

2

t

d(SDCLKL_SDNRAS)

t

SDNRAS hold time

0

-

h(SDCLKL_SDNRAS)

SDNCAS valid time

-

0.5

t

d(SDCLKL_SDNCAS)

t

SDNCAS hold time

0

-

d(SDCLKL_SDNCAS)

NBL valid time

-

0.5

t

d(SDCLKL_NBL)

NBLoutput time

0

-

t

h(SDCLKL_NBL)

1.

CL = 30 pF on data and address lines. CL=15 pF on FMC_SDCLK.

2.

Evaluated by characterization.

(1)(2)

Table 106. LPSDR SDRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FMC_SDCLK  period

2T

− 0.5

2T

+0.5

w(SDCLK)

HCLK

HCLK

)

Data output valid time

-

5

t

d(SDCLKL _Data

t

Data output hold time

2

-

h(SDCLKL _Data)

Address valid time

-

2.8

t

d(SDCLKL_Add)

SDNWE valid time

-

2

t

d(SDCLKL-SDNWE)

t

SDNWE hold time

1

-

h(SDCLKL-SDNWE)

Chip  select valid time

-

1.5

t

d(SDCLKL- SDNE)

ns

Chip  select hold time

1

-

t

h(SDCLKL- SDNE)

t

SDNRAS valid time

-

1.5

d(SDCLKL-SDNRAS)

SDNRAS hold time

1.5

-

t

h(SDCLKL-SDNRAS)

t

SDNCAS valid time

-

1.5

d(SDCLKL-SDNCAS)

SDNCAS hold time

1.5

-

t

d(SDCLKL-SDNCAS)

t

NBL valid time

-

1.5

d(SDCLKL_NBL)

NBL output time

1.5

-

t

h(SDCLKL-NBL)

1.

CL = 10 pF.

2.

Evaluated by characterization.

192/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

6.3.27

Camera interface (DCMI) timing specifications

Unless otherwise specified, the parameters given in Table 107 for DCMI are derived

frequency, and V

supply

from tests performed under the ambient temperature, f

DD

HCLK

voltage summarized in Table 17, with the following configuration:

•

DCMI_PIXCLK polarity: falling

•

DCMI_VSYNC and DCMI_HSYNC polarity: high

•

Data formats: 14 bits

Table 107. DCMI characteristics

Symbol

Parameter

Min

Max

Unit

-

0.4

Frequency ratio DCMI_PIXCLK/f

HCLK

DCMI_PIXCLK Pixel clock input

-

54

MHz

D

Pixel clock input duty cycle

30

70

%

Pixel

Data input setup time

2

-

t

su(DATA)

t

Data input hold time

2.5

-

h(DATA)

t

su(HSYNC)

ns

DCMI_HSYNC/DCMI_VSYNC input setup time

0.5

-

t

su(VSYNC)

t

h(HSYNC)

DCMI_HSYNC/DCMI_VSYNC input hold time

1

-

t

h(VSYNC)

Figure 73. DCMI timing diagram

1/DCMI_PIXCLK

DCMI_PIXCLK

t

t

h(HSYNC)

su(HSYNC)

DCMI_HSYNC

t

t

h(HSYNC)

su(VSYNC)

DCMI_VSYNC

t

t

su(DATA)

h(DATA)

DATA[0:13]

MS32414V2

DS9405 Rev 13

193/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.28

LCD-TFT controller (LTDC) characteristics

Table 108

Unless otherwise specified, the parameters given in

for LCD-TFT are derived

performed under the ambient temperature, f

frequency, and VDD supply

from tests

HCLK

voltage summarized in Table 17

, with the following configuration:

•

LCD_CLK polarity: high

•

LCD_DE polarity: low

•

LCD_VSYNC and LCD_HSYNC polarity: high

•

Pixel formats: 24 bits

Table 108. LTDC characteristics

Symbol

Parameter

Min

Max

Unit

LTDC clock output frequency

-

83

MHz

f

CLK

D

LTDC clock output duty cycle

45

55

%

CLK

t

w(CLKH)

Clock High time, low time

tw(CLK)/2 − 0.5

tw(CLK)/2+0.5

t

w(CLKL)

t

Data output valid time

-

3.5

v(DATA)

Data output hold time

1.5

-

t

h(DATA)

t

v(HSYNC)

ns

HSYNC/VSYNC/DE output valid

t

-

2.5

v(VSYNC)

time

t

v(DE)

t

h(HSYNC)

HSYNC/VSYNC/DE output hold

t

2

-

h(VSYNC)

time

th(DE)

194/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

Figure 74. LCD-TFT horizontal timing diagram

tCLK

LCD_CLK

LCD_VSYNC

tv(HSYNC)

tv(HSYNC)

LCD_HSYNC

th(DE)

tv(DE)

LCD_DE

tv(DATA)

LCD_R[0:7]

LCD_G[0:7]

Pixel

Pixel

LCD_B[0:7]

Pixel

1

2

N

th(DATA)

HSYNC

Horizontal

Active width

Horizontal

width

back porch

back porch

One line

MS32749V1

Figure 75. LCD-TFT vertical timing diagram

tCLK

LCD_CLK

tv(VSYNC)

tv(VSYNC)

LCD_VSYNC

LCD_R[0:7]

LCD_G[0:7]

M lines data

LCD_B[0:7]

VSYNC

Vertical

Active width

Vertical

width

back porch

back porch

One frame

MS32750V1

DS9405 Rev 13

195/240

197

Electrical characteristics

STM32F427xx STM32F429xx

6.3.29

SD/SDIO MMC card host interface (SDIO) characteristics

Unless otherwise specified, the parameters given in Table 109 for the SDIO/MMC interface

are derived from tests performed under the ambient temperature, f

frequency and V

PCLK2

DD

supply voltage conditions summarized in Table 17, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section 6.3.17: I/O port characteristics for more details on the input/output

characteristics.

Figure 76. SDIO high-speed mode

Figure 77. SD default mode

CK

t

t

OVD

OHD

D, CMD

(output)

ai14888

196/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Electrical characteristics

(1)(2)

Table 109. Dynamic characteristics: SD / MMC characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Clock frequency in data transfer mode

-

0

-

48

MHz

PP

-

SDIO_CK/fPCLK2 frequency ratio

-

-

-

8/3

-

Clock low time

fpp =48 MHz

8.5

9

-

t

W(CKL)

ns

Clock high time

fpp =48 MHz

8.3

10

-

t

W(CKH)

CMD, D inputs (referenced to CK) in MMC and SD HS mode

t

Input setup time HS

fpp =48 MHz

3.5

-

-

ISU

ns

Input hold time HS

fpp =48 MHz

0

-

-

t

IH

CMD, D outputs (referenced to CK) in MMC and SD HS mode

t

Output valid time HS

fpp =48 MHz

-

4.5

7

OV

ns

Output hold time HS

fpp =48 MHz

3

-

-

t

OH

CMD, D inputs (referenced to CK) in SD default mode

1.5

-

-

fpp =24 MHz

tISUD

Input setup time SD

ns

0.5

-

-

fpp =24 MHz

tIHD

Input hold time SD

CMD, D outputs (referenced to CK) in SD default mode

-

4.5

6.5

fpp =24 MHz

tOVD

Output valid default time SD

ns

3.5

-

-

fpp =24 MHz

tOHD

Output hold default time SD

1.

Evaluated by characterization.

= 2.7 to 3.6 V.

2.

V

DD

6.3.30

RTC characteristics

Table 110. RTC characteristics

Symbol

Parameter

Conditions

Min

Max

Any read/write operation

-

f

/RTCCLK

frequency ratio

4

-

PCLK1

from/to an RTC register

DS9405 Rev 13

197/240

197

Package information

STM32F427xx STM32F429xx

7

Package information

In order to meet environmental requirements, ST offers these devices in different grades of

®

®

ECOPACK

packages, depending on their level of environmental compliance. ECOPACK

specifications, grade definitions and product status are available at: www.st.com.

®

ECOPACK

is an ST trademark.

7.1

Device marking

Refer to technical note “Reference device marking schematics for STM32 microcontrollers

and microprocessors” (TN1433 ) available on www.st.com, for the location of pin 1 / ball A1

as well as the location and orientation of the marking areas versus pin 1 / ball A1.

Parts marked as “ES”, “E” or accompanied by an engineering sample notification letter, are

not yet qualified and therefore not approved for use in production. ST is not responsible for

any consequences resulting from such use. In no event will ST be liable for the customer

using any of these engineering samples in production. ST’s Quality department must be

contacted prior to any decision to use these engineering samples to run a qualification

activity.

A WLCSP simplified marking example (if any) is provided in the corresponding package

information subsection.

198/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.2

LQFP100 package information (1L)

This LQFP is 100 lead, 14 x 14 mm low-profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 78. LQFP100 - Outline

ș

ș

2

(2)

R1

R2

H

(6)

D1/4

B

GAUGE PLANE

SECTION B-B

S

ș

B

E1/4

ș

L

4x N/4 TIPS

4x

(L1)

A-B

aaa

C

D

D

bbb H A-B

(1)

(11)

SECTION A-A

BOTTOM VIEW

(N-4) x e

(13)

C

A

(9)

(11)

b

0.05

WITH PLATING

ccc

b

C

A2 A1

aaa

C A-BD

(12)

SIDE VIEW

D

(4)

c

c1

(11)

(11)

D1

(2)

(5)

D

(3)

(4)

(10)

N

b1

BASE METAL

(11)

1

2

E1/4

3

SECTION B-B

D1/4

(2)

(6)

B

A

(5)

E1

E

SECTION A-A

A

A

TOP VIEW

1L_LQFP100_ME_V3

DS9405 Rev 13

199/240

231

Package information

STM32F427xx STM32F429xx

Table 111. LQFP100 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

1.50

1.60

-

0.0590

0.0630

(12)

A1

0.05

-

0.15

0.0019

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0570

(9)(11)

b

0.17

0.22

0.27

0.0067

0.0087

0.0106

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0090

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

c1

0.09

-

0.16

0.0035

-

0.0063

(4)

16.00 BSC

0.6299 BSC

D

(2)(5)

14.00 BSC

0.5512 BSC

D1

(4)

E

16.00 BSC

0.6299 BSC

(2)(5)

14.00 BSC

0.5512 BSC

E1

e

0.50 BSC

0.0197 BSC

L

0.45

0.60

0.75

0.177

0.0236

0.0295

(1)(11)

1.00

-

0.0394

-

L1

(13)

N

100

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

(1)

0.20

0.0079

aaa

(1)

0.20

0.0079

bbb

(1)

ccc

0.08

0.0031

(1)

0.08

0.0031

ddd

200/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The Top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is “0.25 mm” per side. D1 and E1 are Maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All Dimensions are in millimeters.

8.

No intrusion allowed inwards the leads.

9.

Dimension “b” does not include dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum “b” dimension by more than 0.08 mm.

Dambar cannot be located on the lower radius or the foot. Minimum space between

protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch packages.

10.

Exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead between 0.10 mm and 0.25 mm

from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

“N” is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to 4 decimal digits.

15.

Drawing is not to scale.

Figure 79. LQFP100 - Footprint example

75

51

76

50

0.5

0.3

16.7

14.3

100

26

1.2

1

25

12.3

16.7

1L_LQFP100_FP_V1

1.

Dimensions are expressed in millimeters.

DS9405 Rev 13

201/240

231

Package information

STM32F427xx STM32F429xx

7.3

WLCSP143 package information

Figure 80. WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package outline

A1 ball location

e1

F

G

bbb

Detail A

e2

e

A3

e

A2

Bottom view

Bump side

A

Side view

D

Bump

A3

eee

A1

E

b

A1 orientation

ccc

Z

Y

X

Seating

ddd

Z

reference

plane

Detail A

Rotated 90°

aaa

Top view

Wafer back side

A0WE_ME_V2

1.

Drawing is not to scale.

Table 112. WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package mechanical data

(1)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

0.525

0.555

0.585

0.0207

0.0219

0.0230

A1

-

0.175

-

-

0.0069

-

A2

-

0.380

-

-

0.0150

-

202/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Table 112. WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package mechanical data (continued)

(1)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

(2)

A3

-

0.025

-

-

0.0010

-

(3)

0.220

0.250

0.280

0.0087

0.0098

0.0110

b

D

4.486

4.521

4.556

0.1766

0.1780

0.1794

E

5.512

5.547

5.582

0.2170

0.2184

0.2198

e

-

0.400

-

-

0.0157

-

e1

-

4.000

-

-

0.1575

-

e2

-

4.800

-

-

0.1890

-

F

-

0.2605

-

-

0.0103

-

G

-

0.3735

-

-

0.0147

-

aaa

-

-

0.100

-

-

0.0039

bbb

-

-

0.100

-

-

0.0039

ccc

-

-

0.100

-

-

0.0039

ddd

-

-

0.050

-

-

0.0020

eee

-

-

0.050

-

-

0.0020

1.

Values in inches are converted from mm and rounded to 4 decimal digits.

2.

Back side coating.

3.

Dimension is measured at the maximum bump diameter parallel to primary datum Z.

Figure 81. WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package recommended footprint

Dpad

Dsm

A0WE_FP_V1

DS9405 Rev 13

203/240

231

Package information

STM32F427xx STM32F429xx

Table 113. WLCSP143 recommended PCB design rules

Dimension

Recommended values

Pitch

0.4

Dpad

0.225 mm

0.290 mm typ. (depends on the soldermask

Dsm

registration tolerance)

Stencil opening

0.250 mm

Stencil thickness

0.100 mm

7.3.1

Device marking for WLCSP143

The following figure gives an example of topside marking orientation versus ball A 1

identifier location.

Other optional marking or inset/upset marks, which depend on assembly location, are not

indicated below.

Figure 82. WLCSP143 marking example (package top view)

ball A1

Product

identification(1)

Date code = Year+Week

Revision code

WW

Y

MSv37234V3

204/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.4

LQFP144 package information (1A)

This LQFP is a 144-pin, 20 x 20 mm low-profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 83. LQFP144 - Outline

BOTTOM VIEW

2

1

(2)

R1

R2

H

B

GAUGE PLANE

SECTION B-B

(6)

D 1/4

S

0.25

B

L

3

E 1/4

(L1)

(1)

(11)

4x N/4 TIPS

aaa C A-B D

SECTION A-A

bbb

4x

H A-B D

(N-4)x e

C

A

(12)

0.05

ddd

C A-B D

b

A2

A1

ccc

C

D

(4)

D1

(2)

(5)

(3)

D

(10)

(9)

(11)

(4)

N

b

WITH PLATING

1

2

E 1/4

3

(11)

(11)

c

c1

(6)

D 1/4

(2)

(3)

A

B

(3)

(5)

E1

E

b1

BASE METAL

(11)

SECTION B-B

A

A

(Section A-A)

TOP VIEW

1A_LQFP144_ME_V2

DS9405 Rev 13

205/240

231

Package information

STM32F427xx STM32F429xx

Table 114. LQFP144 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.60

-

-

0.0630

(12)

A1

0.05

-

0.15

0.0020

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0571

(9)(11)

b

0.17

0.22

0.27

0.0067

0.0087

0.0106

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0090

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

c1

0.09

-

0.16

0.0035

-

0.0063

(4)

22.00 BSC

0.8661 BSC

D

(2)(5)

20.00 BSC

0.7874 BSC

D1

(4)

E

22.00 BSC

0.8661 BSC

(2)(5)

20.00 BSC

0.7874 BSC

E1

e

0.50 BSC

0.0197 BSC

L

0.45

0.60

0.75

0.0177

0.0236

0.0295

L1

1.00 REF

0.0394 REF

(13)

N

144

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

aaa

0.20

0.0079

bbb

0.20

0.0079

ccc

0.08

0.0031

ddd

0.08

0.0031

206/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The Top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is “0.25 mm” per side. D1 and E1 are Maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All Dimensions are in millimeters.

8.

No intrusion allowed inwards the leads.

9.

Dimension “b” does not include dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum “b” dimension by more than 0.08 mm.

Dambar cannot be located on the lower radius or the foot. Minimum space between

protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch packages.

10.

Exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead between 0.10 mm and 0.25 mm

from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

“N” is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to 4 decimal digits.

15.

Drawing is not to scale.

DS9405 Rev 13

207/240

231

Package information

STM32F427xx STM32F429xx

Figure 84. LQFP144 - Footprint example

1.35

108

73

109

72

0.35

0.50

19.90

17.85

22.60

144

37

1

36

19.90

22.60

1A_LQFP144_FP

1.

Dimensions are expressed in millimeters.

208/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.5

LQFP176 package information (1T)

This LQFP is a 176-pin, 24 x 24 mm, 0.5 mm pitch, low profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 85. LQFP176 - Outline

1

2

ș

ș

R1

(2)

R2

H

B

(See SECTION B-B)

GAUGE PLANE

(6)

0.25

D1/4

ș

S

B

L

ș

E1/4

(L1)

4x

4x N/4 TIPS

(1)

(11)

A-B

aaa

C

D

D

bbb H A-B

SECTION A-A

BOTTOM VIEW

A2

0.05

(N-4) x e

C

A

(12)

ccc

C

A1

ddd

C A-BD

b

SIDE VIEW

(4)

D

D1

(2)

(5)

(9)

(11)

D

(10)

N

(4)

b

WITH PLATING

E1/4

c

c1

D1/4

(11)

(11)

(6)

(5)

(2)

A

B

E

E1

b1

BASE METAL

(11)

SECTION A-A

A

A

SECTION B-B

TOP VIEW

1T_LQFP176_ME_V2

DS9405 Rev 13

209/240

231

Package information

STM32F427xx STM32F429xx

Table 115. LQFP176 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.600

-

-

0.0630

(12)

A1

0.050

-

0.150

0.0020

-

0.0059

A2

1.350

1.400

1.450

0.0531

0.0551

0.0571

(9)(11)

0.170

0.220

0.270

0.0067

0.0087

0.0106

b

(11)

0.170

0.200

0.230

0.0067

0.0079

0.0091

b1

(11)

0.090

-

0.200

0.0035

-

0.0079

c

(11)

0.090

-

0.160

0.0035

-

0.063

c1

(4)

26.000

1.0236

D

(2)(5)

24.000

0.9449

D1

(4)

26.000

0.0197

E

(2)(5)

24.000

0.9449

E1

e

0.500

0.1970

L

0.450

0.600

0.750

0.0177

0.0236

0.0295

(1)(11)

1

0.0394 REF

L1

(13)

176

N

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.080

-

-

0.0031

-

-

R2

0.080

-

0.200

0.0031

-

0.0079

S

0.200

-

-

0.0079

-

-

(1)

0.200

0.0079

aaa

(1)

0.200

0.0079

bbb

(1)

0.080

0.0031

ccc

(1)

0.080

0.0031

ddd

210/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The Top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is “0.25 mm” per side. D1 and E1 are Maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All Dimensions are in millimeters.

8.

No intrusion allowed inwards the leads.

9.

Dimension “b” does not include dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum “b” dimension by more than 0.08 mm.

Dambar cannot be located on the lower radius or the foot. Minimum space between

protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch packages.

10.

Exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead between 0.10 mm and 0.25 mm

from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

“N” is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to 4 decimal digits.

15.

Drawing is not to scale.

DS9405 Rev 13

211/240

231

Package information

STM32F427xx STM32F429xx

Figure 86. LQFP176 - Footprint example

176

0.5

1

26.7

21.8

1.2

44

89

133

132

0.3

45

88

1.2

21.8

26.7

1T_FP_V1

1.

Dimensions are expressed in millimeters.

212/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.6

LQFP208 package information

This LQFP is a 208-pin, 28 x 28 mm low-profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 87. LQFP208 - Outline

BOTTOM VIEW

2

1

(2)

R1

R2

H

D 1/4

(6)

SECTION B-B

B

GAUGE PLANE

S

0.25

B

L

3

(L1)

(1)

(11)

E 1/4

SECTION A-A

4x N/4 TIPS

aaa C A-B D

4x

bbb H A-B D

e

(N – 4)x

(13)

C

A

A2

b

ddd

C A-B D

A1

0.05

(12)

ccc C

D

(4)

D1

(2) (5)

D (3)

(10)

N

(9)

(11)

(4)

WITH

b

PLATING

1

2

3

E 1/4

(11)

(11)

c

c1

D 1/4

(6)

b1

BASE METAL

(3)

(11)

(3)

A

B

E

SECTION B-B

E1

(2)

(5)

A

A

(Section A-A)

TOP VIEW

UH_LQFP208_ME_V2

DS9405 Rev 13

213/240

231

Package information

STM32F427xx STM32F429xx

Table 116. LQFP208 - Mechanical data

(15)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.60

-

-

0.0630

(12)

A1

0.05

-

0.15

0.0020

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0571

(9)(11)

b

0.17

0.22

0.27

0.0067

0.0087

0.0106

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0091

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

c1

0.09

-

0.16

0.0035

-

0.0063

(4)

30.00 BSC

1.1732 BSC

D

(2)(5)

28.00 BSC

1.0945 BSC

D1

(4)

E

30.00 BSC

1.1732 BSC

(2)(5)

28.00 BSC

1.0945 BSC

E1

e

0.50 BSC

0.0197 BSC

L

0.45

0.60

0.75

0.0177

0.0236

0.0295

L1

1.00 REF

0.0394 REF

(13)

N

208

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

(1)(7)

0.20

0.0079

aaa

(1)(7)

0.20

0.0079

bbb

(1)(7)

ccc

0.08

0.0031

(1)(7)

0.08

0.0031

ddd

214/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The Top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is “0.25 mm” per side. D1 and E1 are Maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All Dimensions are in millimeters.

8.

No intrusion allowed inwards the leads.

9.

Dimension “b” does not include dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum “b” dimension by more than 0.08 mm.

Dambar cannot be located on the lower radius or the foot. Minimum space between

protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch packages.

10.

Exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead between 0.10 mm and 0.25 mm

from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

“N” is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to 4 decimal digits.

15.

Drawing is not to scale.

Figure 88. LQFP208 - footprint example

208

157

1

156

0.50

1.25

0.30

28.3

30.7

105

52

53

104

1.2

25.8

30.7

UH_LQFP208_FP_V3

1.

Dimensions are expressed in millimeters.

DS9405 Rev 13

215/240

231

Package information

STM32F427xx STM32F429xx

7.7

UFBGA169 package information (A0YV)

This UFBGA is a 169-ball, 7 x 7 mm, 0.50 mm pitch, ultra fine pitch ball grid array package.

Figure 89. UFBGA169 - Outline

E1

SE

e

N

M

L

K

J

e

H

D1

G

F

SD

E

D

C

B

A

1

2 3

4 5 6 7 8 9 1011 12

13

b (169 balls)

Ø

A1 ball pad

A

corner

eee

C

B

Ø

M

C

fff

M

Ø

BOTTOM VIEW

Detail A

A3

A

C

ccc

A2

Mold resin

Seating plane

A1

C

SIDE VIEW

A5

Solder balls

2

C

Substrate

C

ddd

DETAIL A

B

E

A

A1 ball pad

3

corner

(DATUM A)

D

(DATUM B)

aaa

C

(4x)

TOP VIEW

A0YV_UFBGA169_ME_V2

1.

Drawing is not to scale.

2.

Primary datum C is defined by the plane established by the contact points of three or more solder balls that

support the device when it is placed on top of a planar surface.

3.

The terminal (ball) A1 corner must be identified on the top surface of the package by using a corner

chamfer, ink or metallized markings, or other feature of package body or integral heat slug. A distinguish

feature is allowable on the bottom surface of the package to identify the terminal A1 corner. Exact shape of

each corner is optional.

216/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Table 117. UFBGA169 - Mechanical data

(1)

millimeters

inches

Symbol

Min.

Typ.

Max.

Min.

Typ.

Max.

(2)

A

-

-

0.60

-

-

0.0236

(3)

0.05

-

-

0.0020

-

-

A1

A2

-

0.43

-

-

0.0169

-

(4)

b

0.23

0.28

0.33

0.0091

0.0110

0.0130

(5)

7.00 BSC

0.2756 BSC

D

(5)

6.00 BSC

0.2362 BSC

D1

(5)

E

7.00 BSC

0.2756 BSC

(5)

6.00 BSC

0.2362 BSC

E1

(5)(6)

0.50 BSC

0.0197 BSC

e

(7)

N

169

(5)(8)

0.50 BSC

0.0197 BSC

SD

(5)(8)

SE

0.50 BSC

0.0197 BSC

(9)

0.15

0.0059

aaa

(9)

0.20

0.0079

ccc

(9)

ddd

0.08

0.0031

(9)

0.15

0.0059

eee

(9)

0.05

0.0020

fff

1.

Values in inches are converted from mm and rounded to 4 decimal digits.

2.

The profile height, A, is the distance from the seating plane to the highest point on the package. It is

measured perpendicular to the seating plane.

3.

A1 is defined as the distance from the seating plane to the lowest point on the package body.

4.

Dimension b is measured at the maximum diameter of the terminal (ball) in a plane parallel to primary

datum C.

5.

BSC stands for BASIC dimensions. It corresponds to the nominal value and has no tolerance. For

tolerances refer to form and position table.

6.

e represents the solder ball grid pitch.

7.

N represents the total number of balls on the BGA.

8.

Basic dimensions SD and SE are defined with respect to datums A and B. It defines the position of the

centre ball(s) in the outer row or column of a fully populated matrix.

9.

Tolerance of form and position drawing

DS9405 Rev 13

217/240

231

Package information

STM32F427xx STM32F429xx

Figure 90. UFBGA169 - Footprint example

Dpad

Dsm

BGA_WLCSP_FT_V1

Table 118. UFBGA169 - Example of PCB design rules (0.5 mm pitch BGA)

Dimension

Values

Pitch

0.5 mm

Dpad

0.27 mm

0.35 mm typ. (depends on the soldermask

Dsm

registration tolerance)

Solder paste

0.27 mm aperture diameter.

Note:

Non-solder mask defined (NSMD) pads are recommended.

Note:

4 to 6 mils solder paste screen printing process.

218/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.8

UFBGA(176+25) package information (A0E7)

This UFBGA is a 176+25-ball, 10 x 10 mm, 0.65 mm pitch, ultra fine pitch ball grid array

package.

Figure 91. UFBGA(176+25) - Outline

Seating plane

A4

C

ddd

C

A

A2

A1

A3

b

A1 ball

A

A1 ball

index

E

identifier

area

E1

e

F

A

F

D

D1

e

B

R

15

1

Øb (176 + 25  balls)

TOP VIEW

BOTTOM VIEW

Ø

eee

M

C

A

B

C

M

Ø

fff

A0E7_ME_V10

1.

Drawing is not to scale.

Table 119. UFBGA(176+25) - Mechanical data

(1)

millimeters

inches

Symbol

Min.

Typ.

Max.

Min.

Typ.

Max.

A

-

-

0.600

-

-

0.0236

A1

0.050

0.080

0.110

0.0020

0.0031

0.0043

A2

-

0.450

-

-

0.0177

-

A3

-

0.130

-

-

0.0051

-

A4

-

0.320

-

-

0.0126

-

b

0.240

0.290

0.340

0.0094

0.0114

0.0134

D

9.850

10.000

10.150

0.3878

0.3937

0.3996

D1

-

9.100

-

-

0.3583

-

E

9.850

10.000

10.150

0.3878

0.3937

0.3996

E1

-

9.100

-

-

0.3583

-

e

-

0.650

-

-

0.0256

-

F

-

0.450

-

-

0.0177

-

ddd

-

-

0.080

-

-

0.0031

DS9405 Rev 13

219/240

231

Package information

STM32F427xx STM32F429xx

Table 119. UFBGA(176+25) - Mechanical data (continued)

(1)

millimeters

inches

Symbol

Min.

Typ.

Max.

Min.

Typ.

Max.

eee

-

-

0.150

-

-

0.0059

fff

-

-

0.050

-

-

0.0020

1.

Values in inches are converted from mm and rounded to 4 decimal digits.

Figure 92. UFBGA(176+25) - Footprint example

Dpad

Dsm

BGA_WLCSP_FT_V1

Table 120. UFBGA(176+25) - Example of PCB design rules (0.65 mm pitch BGA)

Dimension

Values

Pitch

0.65 mm

Dpad

0.300 mm

0.400 mm typ. (depends on the soldermask

Dsm

registration tolerance)

Stencil opening

0.300 mm

Stencil thickness

Between 0.100 mm and 0.125 mm

Pad trace width

0.100 mm

220/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

7.9

TFBGA216 package information (A0L2)

This TFBGA is a 216-ball, 13 x 13 mm, 0.8 mm pitch, fine pitch ball grid array package.

Figure 93. TFBGA216 - Outline

Z

Seating plane

ddd Z

A2

A1

A

D1

X

A1 ball

A1 ball

D

identifier

index area

e

F

A

G

E1

E

e

Y

R

15

1

Øb (216 balls)

BOTTOM VIEW

TOP VIEW

Ø

eee

Z Y X

M

Z

Ø

fff

M

A0L2_ME_V3

1.

Drawing is not to scale.

2.

• The terminal A1 corner must be identified on the top surface by using a corner chamfer, ink or metalized

markings, or other feature of package body or integral heat slug.

• A distinguishing feature is allowable on the bottom surface of the package to identify the terminal A1

corner. Exact shape of each corner is optional

DS9405 Rev 13

221/240

231

Package information

STM32F427xx STM32F429xx

Table 121. TFBGA216 - Mechanical data

(1)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.200

-

-

0.0472

(2)

A1

0.150

-

-

0.0059

-

-

A2

-

0.760

-

-

0.0299

-

(3)

0.350

0.400

0.450

0.0138

0.0157

0.0177

b

D

12.850

13.000

13.150

0.5059

0.5118

0.5177

D1

-

11.200

-

-

0.4409

-

E

12.850

13.000

13.150

0.5059

0.5118

0.5177

E1

-

11.200

-

-

0.4409

-

e

-

0.800

-

-

0.0315

-

F

-

0.900

-

-

0.0354

-

G

-

0.900

-

-

0.0354

-

ddd

-

-

0.100

-

-

0.0039

(4)

-

-

0.150

-

-

0.0059

eee

(5)

-

-

0.080

-

-

0.0031

fff

1.

Values in inches are converted from mm and rounded to four decimal digits.

2.

• The terminal A1 corner must be identified on the top surface by using a corner chamfer, ink or metallized

markings, or other feature of package body or integral heat slug.

• A distinguishing feature is allowable on the bottom surface of the package to identify the terminal A1

corner. Exact shape of each corner is optional.

3.

Initial ball equal 0.350 mm.

4.

The tolerance of position that controls the location of the pattern of balls with respect to datums A and B.

For each ball there is a cylindrical tolerance zone eee perpendicular to datum C and located on true

position with respect to datums A and B as defined by e. The axis perpendicular to datum C of each ball

must lie within this tolerance zone.

5.

The tolerance of position that controls the location of the balls within the matrix with respect to each other.

For each ball there is a cylindrical tolerance zone fff perpendicular to datum C and located on true position

as defined by e. The axis perpendicular to datum C of each ball must lie within this tolerance zone. Each

tolerance zone fff in the array is contained entirely in the respective zone eee above The axis of each ball

must lie simultaneously in both tolerance zones.

222/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Package information

Figure 94. TFBGA216 - Footprint example

Dpad

Dsm

BGA_WLCSP_FT_V1

Table 122. TFBGA216 - Example of PCB design rules (0.8 mm pitch)

Dimension

Values

Pitch

0.8 mm

Dpad

0.400 mm

0.470 mm typ. (depends on the soldermask

Dsm

registration tolerance)

Stencil opening

0.400 mm

Stencil thickness

Between 0.100 mm and 0.125 mm

Pad trace width

0.120 mm

DS9405 Rev 13

223/240

231

Package information

STM32F427xx STM32F429xx

7.10

Thermal characteristics

The maximum chip-junction temperature, T

max, in degrees Celsius, may be calculated

J

using the following equation:

T

max = T

max + (P

max x θ

)

J

A

D

JA

Where:

•

T

max is the maximum ambient temperature in °C,

A

•

θ

is the package junction-to-ambient thermal resistance, in °C/W,

JA

•

P

max is the sum of P

max and P

max (P

max = P

max + P

max),

D

INT

I/O

D

INT

I/O

•

P

max is the product of I

and

V

, expressed in Watts. This is the maximum chip

INT

DD

DD

internal power.

P

max represents the maximum power dissipation on output pins where:

I/O

P

max

= ∑ (V

× I

) + ∑((V

– V

) × I

),

I/O

OL

OL

DD

OH

OH

taking into account the actual V

/ I

and V

/ I

of the I/Os at low and high level in the

OL

OL

OH

OH

application.

Table 123. Package thermal characteristics

Symbol

Parameter

Value

Unit

Thermal resistance junction-ambient

43

LQFP100 - 14 × 14 mm / 0.5 mm pitch

Thermal resistance junction-ambient

31.2

WLCSP143

Thermal resistance junction-ambient

40

LQFP144 - 20 × 20 mm / 0.5 mm pitch

Thermal resistance junction-ambient

38

LQFP176 - 24 × 24 mm / 0.5 mm pitch

Θ

°C/W

JA

Thermal resistance junction-ambient

19

LQFP208 - 28 × 28 mm / 0.5 mm pitch

Thermal resistance junction-ambient

52

UFBGA169 - 7 × 7mm / 0.5 mm pitch

Thermal resistance junction-ambient

39

UFBGA176 - 10× 10 mm / 0.5 mm pitch

Thermal resistance junction-ambient

29

TFBGA216 - 13 × 13 mm / 0.8 mm pitch

Reference document

JESD51-2 Integrated Circuits Thermal Test Method Environment Conditions - Natural

Convection (Still Air). Available from www.jedec.org.

224/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Ordering information

8

Ordering information

Example:

STM32

F

429 V

I

T

6

xxx

Device family

STM32 = Arm-based 32-bit microcontroller

Product type

F = general-purpose

Device subfamily

427= STM32F427xx, USB OTG FS/HS, camera interface,

Ethernet

429= STM32F429xx, USB OTG FS/HS, camera interface,

Ethernet, LCD-TFT

Pin count

V = 100 pins

Z = 143 and 144 pins

A = 169 pins

I = 176 pins

B = 208 pins

N = 216 pins

Flash memory size

E = 512 Kbytes of Flash memory

G = 1024 Kbytes of Flash memory

I = 2048 Kbytes of Flash memory

Package

T = LQFP

H = BGA

Y = WLCSP

Temperature range

6 = Industrial temperature range, –40 to 85 °C.

7 = Industrial temperature range, –40 to 105 °C.

Options

xxx = programmed parts

TR = tape and reel

For a list of available options (speed, package, etc.) or for further information on any aspect

of this device, please contact your nearest ST sales office.

DS9405 Rev 13

225/240

231

Recommendations when using internal reset OFF

STM32F427xx STM32F429xx

Appendix A

Recommendations when using internal reset

OFF

When the internal reset is OFF, the following integrated features are no longer supported:

•

The integrated power-on reset (POR) / power-down reset (PDR) circuitry is disabled.

•

The brownout reset (BOR) circuitry must be disabled.

•

The embedded programmable voltage detector (PVD) is disabled.

•

V

functionality is no more available and VBAT pin should be connected to V

.

BAT

DD

•

The over-drive mode is not supported.

A.1

Operating conditions

Table 124. Limitations depending on the operating power supply range

Maximum

Flash

Maximum Flash

Operating

memory

Possible Flash

memory access

power

ADC

access

I/O operation

memory

frequency with

supply

operation

frequency

operations

(1)(2)

wait states

range

with no wait

states

)

(f

Flashmax

Conversion

168 MHz with 8

8-bit erase and

V

=1.7 to

– No I/O

(4)

DD

time up to

20 MHz

wait states and

program

(3)

2.1 V

compensation

1.2 Msps

over-drive OFF

operations only

1.

Applicable only when the code is executed from Flash memory. When the code is executed from RAM, no

wait state is required.

2.

Thanks to the ART accelerator and the 128-bit Flash memory, the number of wait states given here does

not impact the execution speed from Flash memory since the ART accelerator allows to achieve a

performance equivalent to 0 wait state program execution.

3.

V

/V

minimum value of 1.7 V, with the use of an external power supply supervisor (refer to

DD

DDA

Section 3.17.1: Internal reset ON).

4.

Prefetch is not available. Refer to AN3430 application note for details on how to adjust performance and

power.

226/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Application block diagrams

Appendix B

Application block diagrams

B.1

USB OTG full speed (FS) interface solutions

Figure 95. USB controller configured as peripheral-only and used

in Full speed mode

V

DD

5V to V

DD

(1)

Volatge regulator

STM32F4xx

VBUS

DM

PA11//PB14

OSC_IN

DP

PA12/PB15

V

SS

OSC_OUT

USB Std-B connector

MS19000V5

1.

External voltage regulator only needed when building a V

powered device.

BUS

2.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

Figure 96. USB controller configured as host-only and used in full speed mode

V

DD

EN

GPIO

Current limiter

5 V Pwr

(1)

power switch

Overcurrent

GPIO+IRQ

STM32F4xx

VBUS

DM

PA11//PB14

OSC_IN

DP

PA12/PB15

V

SS

OSC_OUT

USB Std-A connector

MS19001V4

1.

The current limiter is required only if the application has to support a V

powered device. A basic power

BUS

switch can be used if 5 V are available on the application board.

2.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

DS9405 Rev 13

227/240

231

Application block diagrams

STM32F427xx STM32F429xx

Figure 97. USB controller configured in dual mode and used in full speed mode

V

DD

5 V to V

DD

(1)

voltage regulator

V

DD

EN

GPIO

Current limiter

5 V Pwr

(2)

power switch

Overcurrent

GPIO+IRQ

STM32F4xx

VBUS

PA9/PB13

DM

PA11/PB14

OSC_IN

DP

PA12/PB15

(3)

ID

PA10/PB12

OSC_OUT

V

micro-AB connector

SS

USB

MS19002V3

1.

External voltage regulator only needed when building a V

powered device.

BUS

powered device. A basic power

2.

The current limiter is required only if the application has to support a V

BUS

switch can be used if 5 V are available on the application board.

3.

The ID pin is required in dual role only.

4.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

228/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Application block diagrams

B.2

USB OTG high speed (HS) interface solutions

Figure 98. USB controller configured as peripheral, host, or dual-mode

and used in high speed mode

STM32F4xx

DP

FS PHY

not connected

DM

USB HS

OTG Ctrl

DP

ULPI_CLK

DM

ULPI_D[7:0]

(2)

ID

USB

ULPI_DIR

connector

V

ULPI

BUS

ULPI_STP

V

SS

ULPI_NXT

High speed

OTG PHY

XT1

PLL

(1)

24 or 26 MHz XT

MCO1 or MCO2

XI

MS19005V2

1.

It is possible to use MCO1 or MCO2 to save a crystal. It is however not mandatory to clock the STM32F42x

with a 24 or 26 MHz crystal when using USB HS. The above figure only shows an example of a possible

connection.

2.

The ID pin is required in dual role only.

DS9405 Rev 13

229/240

231

Application block diagrams

STM32F427xx STM32F429xx

B.3

Ethernet interface solutions

Figure 99. MII mode using a 25 MHz crystal

STM32

MII_TX_CLK

MCU

MII_TX_EN

Ethernet

Ethernet

MII_TXD[3:0]

MAC 10/100

MII_CRS

MII

MII_COL

= 15 pins

(1)

HCLK

MII_RX_CLK

MII + MDC

= 17 pins

MII_RXD[3:0]

MII_RX_DV

IEEE1588 PTP

Timer

MII_RX_ER

input

PHY 10/100

trigger

Timestamp

MDIO

TIM2

comparator

MDC

(2)

PPS_OUT

HCLK

PLL

XTAL

OSC

25 MHz

PHY_CLK 25 MHz

MCO1/MCO2

XT1

MS19968V1

1.

f

must be greater than 25 MHz.

HCLK

2.

Pulse per second when using IEEE1588 PTP optional signal.

Figure 100. RMII with a 50 MHz oscillator

STM32

Ethernet

PHY 10/100

MCU

RMII_TX_EN

Ethernet

RMII_TXD[1:0]

MAC 10/100

RMII

RMII_RXD[1:0]

(1)

HCLK

= 7 pins

RMII_CRX_DV

RMII + MDC

RMII_REF_CLK

= 9 pins

IEEE1588 PTP

MDIO

Timer

input

MDC

trigger

Timestamp

TIM2

comparator

/2 or /20

2.5 or 25 MHz

50 MHz

synchronous

HCLK

OSC

PLL

50 MHz

50 MHz

PHY_CLK    50 MHz

XT1

MS19969V1

1.

f

must be greater than 25 MHz.

HCLK

230/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Application block diagrams

Figure 101. RMII with a 25 MHz crystal and PHY with PLL

STM32F

Ethernet

PHY 10/100

MCU

RMII_TX_EN

Ethernet

RMII_TXD[1:0]

MAC 10/100

RMII_RXD[1:0]

RMII

(1)

HCLK

= 7 pins

RMII_CRX_DV

REF_CLK

RMII + MDC

RMII_REF_CLK

IEEE1588 PTP

= 9 pins

MDIO

Timer

input

MDC

trigger

Timestamp

TIM2

comparator

/2 or /20

2.5 or 25 MHz

50 MHz

synchronous

PLL

HCLK

XTAL

PLL

25 MHz

XT1

PHY_CLK   25 MHz

OSC

MCO1/MCO2

MS19970V1

1.

f

must be greater than 25 MHz.

HCLK

2.

The 25 MHz (PHY_CLK) must be derived directly from the HSE oscillator, before the PLL block.

DS9405 Rev 13

231/240

231

Important security notice

STM32F427xx STM32F429xx

9

Important security notice

The STMicroelectronics group of companies (ST) places a high value on product security,

which is why the ST product(s) identified in this documentation may be certified by various

security certification bodies and/or may implement our own security measures as set forth

herein. However, no level of security certification and/or built-in security measures can

guarantee that ST products are resistant to all forms of attacks. As such, it is the

responsibility of each of ST's customers to determine if the level of security provided in an

ST product meets the customer needs both in relation to the ST product alone, as well as

when combined with other components and/or software for the customer end product or

application. In particular, take note that:

•

ST products may have been certified by one or more security certification bodies, such

as Platform Security Architecture (www.psacertified.org) and/or Security Evaluation

standard for IoT Platforms (www.trustcb.com). For details concerning whether the ST

product(s) referenced herein have received security certification along with the level

and current status of such certification, either visit the relevant certification standards

website or go to the relevant product page on www.st.com for the most up to date

information. As the status and/or level of security certification for an ST product can

change from time to time, customers should re-check security certification status/level

as needed. If an ST product is not shown to be certified under a particular security

standard, customers should not assume it is certified.

•

Certification bodies have the right to evaluate, grant and revoke security certification in

relation to ST products. These certification bodies are therefore independently

responsible for granting or revoking security certification for an ST product, and ST

does not take any responsibility for mistakes, evaluations, assessments, testing, or

other activity carried out by the certification body with respect to any ST product.

•

Industry-based cryptographic algorithms (such as AES, DES, or MD5) and other open

standard technologies which may be used in conjunction with an ST product are based

on standards which were not developed by ST. ST does not take responsibility for any

flaws in such cryptographic algorithms or open technologies or for any methods which

have been or may be developed to bypass, decrypt or crack such algorithms or

technologies.

•

While robust security testing may be done, no level of certification can absolutely

guarantee protections against all attacks, including, for example, against advanced

attacks which have not been tested for, against new or unidentified forms of attack, or

against any form of attack when using an ST product outside of its specification or

intended use, or in conjunction with other components or software which are used by

customer to create their end product or application. ST is not responsible for resistance

against such attacks. As such, regardless of the incorporated security features and/or

any information or support that may be provided by ST, each customer is solely

responsible for determining if the level of attacks tested for meets their needs, both in

relation to the ST product alone and when incorporated into a customer end product or

application.

•

All security features of ST products (inclusive of any hardware, software,

documentation, and the like), including but not limited to any enhanced security

features added by ST, are provided on an "AS IS" BASIS. AS SUCH, TO THE EXTENT

PERMITTED BY APPLICABLE LAW, ST DISCLAIMS ALL WARRANTIES, EXPRESS

OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, unless the

applicable written and signed contract terms specifically provide otherwise.

232/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Revision history

10

Revision history

Table 125. Document revision history

Date

Revision

Changes

19-Mar-2013

1

Initial release.

Added STM32F429xx part numbers and related informations.

STM32F427xx part numbers:

Replaced FSMC by FMC added Chrom-ART Accelerator and SAI

interface.

Increased core, timer, GPIOs, SPI maximum frequencies

Updated Figure 8.Updated Figure 9.

Removed note in Section ·: Standby mode.

Updated Figure 18.

Updated Table 10: STM32F437xx and STM32F439xx pin and ball

definitions and Table 12: STM32F437xx and STM32F439xx alternate

function mapping..

Modified Figure 19: Memory map.

Updated Table 17: General operating conditions, Table 18: Limitations

depending on the operating power supply range. Removed note 1 in

Table 22: reset and power control block characteristics. Added Table

23: Over-drive switching characteristics.

Updated Section : Typical and maximum current consumption, Table

10-Sep-2013

2

34: Switching output I/O current consumption, Table 35: Peripheral

current consumption and Section : On-chip peripheral current

consumption.

Updated Table 36: Low-power mode wakeup timings.

Modified Section : High-speed external user clock generated from an

external source, Section : Low-speed external user clock generated

from an external source, and Section 6.3.10: Internal clock source

characteristics.

Updated Table 43: Main PLL characteristics and Table 45: PLLISAI

(audio and LCD-TFT PLL) characteristics.

Updated Table 52: EMI characteristics.

Updated Table 57: Output voltage characteristics and Table 58: I/O AC

characteristics.

Updated Table 60: TIMx characteristics, Table 61: I2C characteristics,

Table 62: SPI dynamic characteristics, Section : SAI characteristics.

Updated Table 102: SDRAM read timings and Table 104: SDRAM write

timings.

DS9405 Rev 13

233/240

239

Revision history

STM32F427xx STM32F429xx

Table 125. Document revision history

Date

Revision

Changes

.Added STM32F429xE part numbers featuring 512 Mbytes of Flash

memory and UFBGA169 package.

Added LPSDR SDRAM.

Changed INTN into INTR in Figure 4: STM32F437xx and

STM32F439xx block diagram.

Added note 4 in Table 2: STM32F427xx and STM32F429xx features

and peripheral counts.

Updated Section 3.15: Boot modes.

Updated for PA4 and PA5 in Table 10: STM32F437xx and

STM32F439xx pin and ball definitions.

Added VIN for BOOT0 pins in Table 14: Voltage characteristics.

Updated Note 6., added Note 1.,and updated maximum VIN for B pins

in Table 17: General operating conditions.

Updated maximum Flash memory access frequency with wait states

for VDD =1.8 to 2.1 V in Table 18: Limitations depending on the

operating power supply range.

Updated Table 24: Typical and maximum current consumption in Run

mode, code with data processing   running from Flash memory (ART

accelerator enabled except prefetch) or RAM and Table 25: Typical and

maximum current consumption in Run mode, code with data

24-Jan-2014

3

processing   running from Flash memory (ART accelerator disabled).

Updated Table 30: Typical current consumption in Run mode, code

with data processing running from   Flash memory or RAM, regulator

ON (ART accelerator enabled except prefetch),  VDD=1.7 V, Table 31:

Typical current consumption in Run mode, code with data processing

running   from Flash memory, regulator OFF (ART accelerator enabled

except prefetch), and Table 32: Typical current consumption in Sleep

mode, regulator ON, VDD=1.7 V.

Updated Table 57: Output voltage characteristics.

Updated Table 58: I/O AC characteristics. Added Figure 35.

Updated th(SDA), tr(SDA) and tr(SCL) and added tSP in Table 61: I2C

characteristics.

Updated fSCK in Table 62: SPI dynamic characteristics.

Updated Table 70: Dynamic characteristics: USB ULPI.

Updated Section 6.3.26: FMC characteristics conditions. Updated

Figure 73: SDRAM read access waveforms (CL = 1) and Figure 74:

SDRAM write access waveforms. Added Table 103: LPSDR SDRAM

read timings and Table 105: LPSDR SDRAM write timings. Updated

Table 102: SDRAM read timings and Table 104: SDRAM write timings

and added note 2.Table 108: Dynamic characteristics: SD / MMC

characteristics

234/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Revision history

Table 125. Document revision history

Date

Revision

Changes

In the whole document, minimum supply voltage changed to 1.7 V

when external power supply supervisor is used.

Added DCMI_VSYNC alternate function on PG9 and updated note 6.

in Table 10: STM32F437xx and STM32F439xx pin and ball definitions

and Table 12: STM32F437xx and STM32F439xx alternate function

mapping. Added note 2.belowFigure 16: STM32F43x UFBGA169

ballout.

Changed SVGA (800x600) into XGA1024x768) on cover page and in

Section 3.10: LCD-TFT controller (available only on STM32F439xx).

Updated Section 3.18.2: Regulator OFF.

Updated signal corresponding to pin L5 in Figure 12: STM32F43x

WLCSP143 ballout.

Added ACCHSE in Table 39: HSE 4-26 MHz oscillator characteristics

and ACCLSE in Table 40: LSE oscillator characteristics (fLSE = 32.768

kHz).

Updated Table 53: ESD absolute maximum ratings.

24-Apr-2014

4

Updated VIH in Table 56: I/O static characteristics. Added condition

VDD>1.7 V in Table 58: I/O AC characteristics.

Updated conditions in Table 62: SPI dynamic characteristics.

Added ZDRV in Table 67: USB OTG full speed electrical characteristics

Removed note 3 in Table 80: Temperature sensor characteristics.

Added Figure 82: LQFP100 marking example (package top view),

Figure 85: WLCSP143 marking example (package top view), Figure

88: LQFP144 marking example (package top view), Figure 91:

LQFP176 marking (package top view), Figure 94: LQFP208 marking

example (package top view), Figure 97: UFBGA169 marking example

(package top view) and Figure 100: UFBGA176+25 marking example

(package top view).

Added Appendix A: Recommendations when using internal reset OFF.

Removed Internal reset OFF hardware connection appendix.

DS9405 Rev 13

235/240

239

Revision history

STM32F427xx STM32F429xx

Table 125. Document revision history

Date

Revision

Changes

Update SPI/IS2 in Table 2: STM32F427xx and STM32F429xx features

and peripheral counts.

Updated LQFP208 in Table 4: Regulator ON/OFF and internal reset

ON/OFF availability.

Updated Figure 19: Memory map.

Changed PLS[2:0]=101 (falling edge) maximum value in Table 22:

reset and power control block characteristics.

Updated current consumption with all peripherals disabled in Table 24:

Typical and maximum current consumption in Run mode, code with

data processing   running from Flash memory (ART accelerator

enabled except prefetch) or RAM. Updated note 1. in Table 28: Typical

and maximum current consumptions in Standby mode.

Updated tWUSTOP in Table 36: Low-power mode wakeup timings.

Updated ESD standards and Table 53: ESD absolute maximum

ratings.

Updated Table 56: I/O static characteristics.

Section : I2C interface characteristics: updated section introduction,

removed Table I2C characteristics, Figure I2C bus AC waveforms and

measurement circuit and Table SCL frequency; added Table 61: I2C

analog filter characteristics.

Updated measurement conditions in Table 62: SPI dynamic

characteristics.

Updated Figure 51: Typical connection diagram using the ADC.

19-Feb-2015

5

Updated Section : Device marking for LQFP100.

Updated Figure 83: WLCSP143 - 143-ball, 4.521x 5.547 mm, 0.4 mm

pitch wafer level chip scale package outline and Table 111: WLCSP143

- 143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

package mechanical data; added Figure 84: WLCSP143 - 143-ball,

4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale recommended

footprint and Table 112: WLCSP143 recommended PCB design rules

(0.4 mm pitch). Updated Figure 85: WLCSP143 marking example

(package top view) and related note. Updated Section : Device

marking for WLCSP143.

Updated Section : Device marking for LQFP144.

Updated Section : Device marking for LQFP176.

Updated Figure 92: LQFP208 - 208-pin, 28 x 28 mm low-profile quad

flat package outline; Updated Section : Device marking for LQFP208.

Modified UFBGA169 pitch, updated Figure 95: UFBGA169 - 169-ball 7

x 7 mm 0.50 mm pitch, ultra fine pitch ball grid array package outline

and Table 116: UFBGA169 - 169-ball 7 x 7 mm 0.50 mm pitch, ultra

fine pitch ball grid array package mechanical data; updated Section :

Device marking for LQFP208.

updated Section : Device marking for UFBGA169, Section : Device

marking for UFBGA176+25 and Section : Device marking for

TFBGA176.

Updated Z pin count in Table  : .

?ǣ

236/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Revision history

Table 125. Document revision history

Date

Revision

Changes

Updated notes related to the minimum and maximum values

guaranteed by design, characterization or test in production.

Updated IDD_STOP_UDM in Table 27: Typical and maximum current

consumptions in Stop mode.

Removed note related to tests in production in Table 24: Typical and

maximum current consumption in Run mode, code with data

processing   running from Flash memory (ART accelerator enabled

except prefetch) or RAM and Table 26: Typical and maximum current

consumption in Sleep mode.

Updated Table 41: HSI oscillator characteristics. Figure 31 renamed

ACCHSI accuracy versus temperature and updated.

Updated Figure 38: SPI timing diagram - slave mode and CPHA = 0.

Updated Section : Ethernet characteristics.

Updated Table 43: Main PLL characteristics, Table 44: PLLI2S (audio

17-Sep-2015

6

PLL) characteristics and Table 45: PLLISAI (audio and LCD-TFT PLL)

characteristics.

Removed note 1 in Table 75: ADC static accuracy at fADC = 18 MHz,

Table 76: ADC static accuracy at fADC = 30 MHz and Table 77: ADC

static accuracy at fADC = 36 MHz.

Updated td(SDCLKL _Data) and th(SDCLKL _Data) in Table 104:

SDRAM write timings.

Added Figure 96: UFBGA169 - 169-ball, 7 x 7 mm, 0.50 mm pitch, ultra

fine pitch ball grid array recommended footprint and Table 117:

UFBGA169 recommended PCB design rules (0.5 mm pitch BGA).

Added Figure 99: UFBGA176+25-ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package recommended footprint and

Table 119: UFBGA176+25 recommended PCB design rules (0.65 mm

pitch BGA).

Updated |VSSX -VSS| in Table 14: Voltage characteristics to add

VREF-.

Updated td(TXEN) and td(TXD) minimum value in Table 72: Dynamics

characteristics: Ethernet MAC signals for RMII and Table 73: Dynamics

characteristics: Ethernet MAC signals for MII.

Added VREF- in Table 74: ADC characteristics.

Added A1 minimum and maximum values in Table 111: WLCSP143 -

143-ball, 4.521x 5.547 mm, 0.4 mm pitch wafer level chip scale

30-Nov-2015

7

package mechanical data. Updated Figure 86: LQFP144-144-pin, 20 x

20 mm low-profile quad flat package outline.

Updated Figure 98: UFBGA176+25 - ball 10 x 10 mm, 0.65 mm pitch

ultra thin fine pitch  ball grid array package outline and Table 118:

UFBGA176+25 - ball, 10 x 10 mm, 0.65 mm pitch, ultra fine pitch ball

grid array package mechanical data. Updated Figure 101: TFBGA216 -

216 ball 13 × 13 mm 0.8 mm pitch thin fine pitch ball grid array package

outline and Table 120: TFBGA216 - 216 ball 13 × 13 mm 0.8 mm pitch

thin fine pitch ball grid array package mechanical data.

Updated Figure 22: Power supply scheme.

Added td(TXD) values corresponding to 1.71 V < VDD < 3.6 V in Table

21-Jan-2016

8

72: Dynamics characteristics: Ethernet MAC signals for RMII.

DS9405 Rev 13

237/240

239

Revision history

STM32F427xx STM32F429xx

Table 125. Document revision history

Date

Revision

Changes

Updated Figure 1: Compatible board design

STM32F10xx/STM32F2xx/STM32F4xx for LQFP100 package.

Added mission profile compliance with JEDEC JESD47 in Section 6.2:

Absolute maximum ratings.

Changed Figure 31 HSI deviation versus temperature to ACCHSI

versus temperature.

Updated RLOAD in Table 85: DAC characteristics.

Added note 2. related to the position of the 0.1 µF capacitor below

Figure 37: Recommended NRST pin protection.

18-Jul-2016

9

Updated Figure 40: SPI timing diagram - master mode.

Added reference to optional marking or inset/upset marks in all

package device marking sections. Updated Figure 85: WLCSP143

marking example (package top view), Figure 88: LQFP144 marking

example (package top view), Figure 91: LQFP176 marking (package

top view), Figure 94: LQFP208 marking example (package top view).

Updated Figure 98: UFBGA176+25 - ball 10 x 10 mm, 0.65 mm pitch

ultra thin fine pitch  ball grid array package outline and Table 118:

UFBGA176+25 - ball, 10 x 10 mm, 0.65 mm pitch, ultra fine pitch ball

grid array package mechanical data.

Updated Arm wordmark and added Arm logo in Section 2: Description.

Updated LDC-TFT feature on cover page.

Updated Table 24: Typical and maximum current consumption in Run

mode, code with data processing   running from Flash memory (ART

19-Jan-2018

10

accelerator enabled except prefetch) or RAM and Table 26: Typical and

maximum current consumption in Sleep mode.

RADC minimum value added in Table 74: ADC characteristics.

LTDC clock output frequency changed to 83 MHz in Table 107: LTDC

characteristics.

238/240

DS9405 Rev 13

STM32F427xx STM32F429xx

Revision history

Table 125. Document revision history

Date

Revision

Changes

General datasheet update to include minor terminology updates.

Updated the datasheet cover page.

General update of the Section 7: Package information.

Updated the figure Figure 19: Memory map.

Edited the footnote in Table 41: HSI oscillator characteristics.

Updated the Table 22: Reset and power control block characteristics.

Updated the Section 6.2: Absolute maximum ratings.

Updated the Section : I/O system current consumption.

Updated the Section 3.36: True random number generator (RNG).

24-Oct-2024

11

Updated the Figure 36: I/O AC characteristics definition.

Updated the Figure 51: Typical connection diagram when using the

ADC with FT/TT pins featuring the analog switch function.

Updated the Section : Electromagnetic Interference (EMI).

Updated the Figure 38: SPI timing diagram - slave mode and CPHA =

0, the Figure 39: SPI timing diagram - slave mode and CPHA = 1, and

the Figure 40: SPI timing diagram - master mode.

Updated the Figure 69: NAND controller waveforms for read access

and Figure 70: NAND controller waveforms for write access.

Updated the Table 14: Voltage characteristics.

05-May-2025

12

Corrected Table 57: I/O static characteristics.

Updated Arm legal notice in Section 1: Introduction.

max. in Table 14: Voltage characteristics.

23-Feb-2026

13

Updated V

IN

Updated terminology in Table 60: NRST pin characteristics.

DS9405 Rev 13

239/240

239

STM32F427xx STM32F429xx

IMPORTANT NOTICE – READ CAREFULLY

STMicroelectronics NV and its subsidiaries (“ST”) reserve the right to make changes, corrections, enhancements, modifications, and

improvements to ST products and/or to this document at any time without notice.

In the event of any conflict between the provisions of this document and the provisions of any contractual arrangement in force between the

purchasers and ST, the provisions of such contractual arrangement shall prevail.

The purchasers should obtain the latest relevant information on ST products before placing orders. ST products are sold pursuant to ST’s

terms and conditions of sale in place at the time of order acknowledgment.

The purchasers are solely responsible for the choice, selection, and use of ST products and ST assumes no liability for application

assistance or the design of the purchasers’ products.

No license, express or implied, to any intellectual property right is granted by ST herein.

Resale of ST products with provisions different from the information set forth herein shall void any warranty granted by ST for such product.

If the purchasers identify an ST product that meets their functional and performance requirements but that is not designated for the

purchasers' market segment, the purchasers shall contact ST for more information.

ST and the ST logo are trademarks of ST. For additional information about ST trademarks, refer to www.st.com/trademarks. All other product

or service names are the property of their respective owners.

Information in this document supersedes and replaces information previously supplied in any prior versions of this document.

© 2026 STMicroelectronics – All rights reserved

240/240

DS9405 Rev 13