STM32F405xx STM32F407xx

®

®

Arm

Cortex

-M4 32b MCU+FPU, 210DMIPS, up to 1MB flash/192+4KB RAM, USB

OTG HS/FS, Ethernet, 17 TIMs, 3 ADCs, 15 comm. interfaces, and camera

-

Datasheet

production data

Features

•

Includes ST state-of-the-art patented

technology

®

®

•

Core: Arm

32-bit Cortex

-M4 CPU with FPU,

Adaptive real-time accelerator (ART

WLCSP90

Accelerator) allowing 0-wait state execution

UFBGA176

LQFP64 (10 × 10 mm)

(4.223x3.969 mm)

(10 × 10 mm)

LQFP100 (14 × 14 mm)

from flash memory, frequency up to 168 MHz,

LQFP144 (20 × 20 mm)

memory protection unit, 210 DMIPS/

LQFP176 (24 × 24 mm)

1.25 DMIPS/MHz (Dhrystone 2.1), and DSP

instructions

•

Up to 17 timers: up to twelve 16-bit and two 32-

•

Memories

bit timers up to 168 MHz, each with up to 4

IC/OC/PWM or pulse counter and quadrature

–

Up to 1 Mbyte of flash memory

(incremental) encoder input

–

Up to 192+4 Kbytes of SRAM including 64-

Kbyte of CCM (core coupled memory) data

•

Debug mode

RAM

–

Serial wire debug (SWD) & JTAG

–

512 bytes of OTP memory

interfaces

–

Flexible static memory controller

–

Cortex-M4 Embedded Trace Macrocell™

supporting Compact Flash, SRAM,

•

Up to 140 I/O ports with interrupt capability

PSRAM, NOR and NAND memories

–

Up to 136 fast I/Os up to 84 MHz

•

LCD parallel interface, 8080/6800 modes

–

Up to 138 5 V-tolerant I/Os

•

Clock, reset, and supply management

•

Up to 15 communication interfaces

–

1.8 V to 3.6 V application supply and I/Os

2

–

Up to 3 × I

C interfaces (SMBus/PMBus)

–

POR, PDR, PVD and BOR

–

Up to 4 USARTs/2 UARTs (10.5 Mbit/s, ISO

–

4-to-26 MHz crystal oscillator

7816 interface, LIN, IrDA, modem control)

–

Internal 16 MHz factory-trimmed RC (1%

–

Up to 3 SPIs (42 Mbits/s), 2 with muxed

accuracy)

2

full-duplex I

S to achieve audio class

–

32 kHz oscillator for RTC with calibration

accuracy via internal audio PLL or external

clock

–

Internal 32 kHz RC with calibration

–

2 × CAN interfaces (2.0B Active)

•

Low-power operation

–

SDIO interface

–

Sleep, Stop, and Standby modes

•

Advanced connectivity

–

V

supply for RTC, 20×32-bit backup

BAT

registers + optional 4 KB backup SRAM

–

USB 2.0 full-speed device/host/OTG

controller with on-chip PHY

•

3×12-bit, 2.4 MSPS A/D converters: up to 24

–

USB 2.0 high-speed/full-speed

channels and 7.2 MSPS in triple interleaved

device/host/OTG controller with dedicated

mode

DMA, on-chip full-speed PHY and ULPI

•

2×12-bit D/A converters

–

10/100 Ethernet MAC with dedicated DMA:

•

General-purpose DMA: 16-stream DMA

supports IEEE 1588v2 hardware, MII/RMII

controller with FIFOs and burst support

February 2026

DS8626 Rev 11

1/205

This is information on a product in full production.

www.st.com

STM32F405xx, STM32F407xx

•

•

96-bit unique ID

8- to 14-bit parallel camera interface up to

54 Mbytes/s

•

RTC: subsecond accuracy, hardware calendar

•

True random number generator

•

All packages are ECOPACK2 compliant

•

CRC calculation unit

Table 1. Device summary

Reference

Part number

STM32F405xx

STM32F405RG, STM32F405VG, STM32F405ZG, STM32F405OG, STM32F405OE

STM32F407VG, STM32F407IG, STM32F407ZG, STM32F407VE, STM32F407ZE,

STM32F407xx

STM32F407IE

2/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Contents

Contents

1

Introduction  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 13

2

Description . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 14

2.1

Full compatibility throughout the family   . . . . . . . . . . . . . . . . . . . . . . . . . .  17

3

Functional overview  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

®

®

3.0.1

Arm

Cortex

-M4 core with FPU and embedded Flash and SRAM  . . . 21

3.0.2

Adaptive real-time memory accelerator (ART Accelerator)  . . . . . . . . . . 21

3.0.3

Memory protection unit   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

3.0.4

Embedded flash memory . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

3.0.5

CRC (cyclic redundancy check) calculation unit  . . . . . . . . . . . . . . . . . . 22

3.0.6

Embedded SRAM   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

3.0.7

Multi-AHB bus matrix  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 22

3.0.8

DMA controller (DMA)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

3.0.9

Flexible static memory controller (FSMC)  . . . . . . . . . . . . . . . . . . . . . . . 24

3.0.10

Nested vectored interrupt controller (NVIC) . . . . . . . . . . . . . . . . . . . . . . 24

3.0.11

External interrupt/event controller (EXTI)   . . . . . . . . . . . . . . . . . . . . . . . 24

3.0.12

Clocks and startup  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 24

3.0.13

Boot modes  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

3.0.14

Power supply schemes  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

3.0.15

Power supply supervisor  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 25

3.0.16

Voltage regulator  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

3.0.17

Regulator ON/OFF and internal reset ON/OFF availability  . . . . . . . . . . 30

3.0.18

Real-time clock (RTC), backup SRAM and backup registers   . . . . . . . . 30

3.0.19

Low-power modes  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 31

3.0.20

V

operation  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

BAT

3.0.21

Timers and watchdogs . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

3.0.22

Inter-integrated circuit interface (I²C)  . . . . . . . . . . . . . . . . . . . . . . . . . . . 35

3.0.23

Universal synchronous/asynchronous receiver transmitters (USART)  . 35

3.0.24

Serial peripheral interface (SPI) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

3.0.25

Inter-integrated sound (I2S) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

3.0.26

Audio PLL (PLLI2S)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 37

3.0.27

Secure digital input/output interface (SDIO)   . . . . . . . . . . . . . . . . . . . . . 37

3.0.28

Ethernet MAC interface with dedicated DMA and IEEE 1588 support   . 37

DS8626 Rev 11

3/205

6

Contents

STM32F405xx, STM32F407xx

3.0.29

Controller area network (bxCAN) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 38

3.0.30

Universal serial bus on-the-go full-speed (OTG_FS) . . . . . . . . . . . . . . . 38

3.0.31

Universal serial bus on-the-go high-speed (OTG_HS)   . . . . . . . . . . . . . 39

3.0.32

Digital camera interface (DCMI)   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 39

3.0.33

True random number generator (RNG)  . . . . . . . . . . . . . . . . . . . . . . . . . 39

3.0.34

General-purpose input/outputs (GPIOs)   . . . . . . . . . . . . . . . . . . . . . . . . 39

3.0.35

Analog-to-digital converters (ADCs)   . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

3.0.36

Temperature sensor . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

3.0.37

Digital-to-analog converter (DAC)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 40

3.0.38

Serial wire JTAG debug port (SWJ-DP) . . . . . . . . . . . . . . . . . . . . . . . . . 41

3.0.39

Embedded Trace Macrocell™  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 41

4

Pinouts and pin description . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

5

Memory mapping . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 72

6

Electrical characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1

Parameter conditions  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  77

6.1.1

Minimum and maximum values  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1.2

Typical values   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1.3

Typical curves   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1.4

Loading capacitor   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1.5

Pin input voltage   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

6.1.6

Power supply scheme  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 78

6.1.7

Current consumption measurement   . . . . . . . . . . . . . . . . . . . . . . . . . . . 79

6.2

Absolute maximum ratings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  79

6.3

Operating conditions   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  80

6.3.1

General operating conditions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 80

6.3.2

VCAP_1/VCAP_2 external capacitor . . . . . . . . . . . . . . . . . . . . . . . . . . . 83

6.3.3

Operating conditions at power-up / power-down (regulator ON)  . . . . . . 83

6.3.4

Operating conditions at power-up / power-down (regulator OFF)  . . . . . 83

6.3.5

Embedded reset and power control block characteristics  . . . . . . . . . . . 84

6.3.6

Supply current characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 85

6.3.7

Wakeup time from low-power mode   . . . . . . . . . . . . . . . . . . . . . . . . . . . 99

6.3.8

External clock source characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . 100

6.3.9

Internal clock source characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . 104

6.3.10

PLL characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 105

4/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Contents

6.3.11

PLL spread spectrum clock generation (SSCG) characteristics   . . . . . 107

6.3.12

Memory characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 109

6.3.13

EMC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  111

6.3.14

Absolute maximum ratings (electrical sensitivity)   . . . . . . . . . . . . . . . . 113

6.3.15

I/O current injection characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . 114

6.3.16

I/O port characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 115

6.3.17

NRST pin characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

6.3.18

TIM timer characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

6.3.19

Communications interfaces  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 122

6.3.20

CAN (controller area network) interface   . . . . . . . . . . . . . . . . . . . . . . . 134

6.3.21

12-bit ADC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 134

6.3.22

Temperature sensor characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

6.3.23

V

monitoring characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

BAT

6.3.24

Embedded reference voltage . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

6.3.25

DAC electrical characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

6.3.26

FSMC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143

6.3.27

Camera interface (DCMI) timing specifications  . . . . . . . . . . . . . . . . . . 161

6.3.28

SD/SDIO MMC card host interface (SDIO) characteristics  . . . . . . . . . 162

6.3.29

RTC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163

7

Package information . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 164

7.1

Device marking . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  164

7.2

WLCSP90 package information  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  165

7.3

LQFP64 package information (5W)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  168

7.4

LQFP100 package information (1L)  . . . . . . . . . . . . . . . . . . . . . . . . . . . .  171

7.5

LQFP144 package information (1A) . . . . . . . . . . . . . . . . . . . . . . . . . . . .  174

7.6

UFBGA(176+25) package information (A0E7) . . . . . . . . . . . . . . . . . . . .  178

7.7

LQFP176 package information (1T) . . . . . . . . . . . . . . . . . . . . . . . . . . . .  180

7.8

Thermal characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  184

8

Ordering information   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 185

Appendix A

Application block diagrams . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 186

A.1

USB OTG full speed (FS) interface solutions . . . . . . . . . . . . . . . . . . . . .  186

A.2

USB OTG high speed (HS) interface solutions . . . . . . . . . . . . . . . . . . . .  188

A.3

Ethernet interface solutions. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  189

DS8626 Rev 11

5/205

6

Contents

STM32F405xx, STM32F407xx

9

Important security notice  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 191

10

Revision history   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 192

6/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

List of tables

List of tables

Table 1.

Device summary . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2

Table 2.

STM32F405xx and STM32F407xx: features and peripheral counts. . . . . . . . . . . . . . . . . . 15

Table 3.

Regulator ON/OFF and internal reset ON/OFF availability. . . . . . . . . . . . . . . . . . . . . . . . . 30

Table 4.

Timer feature comparison. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 32

Table 5.

USART feature comparison . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 36

Table 6.

Legend/abbreviations used in the pinout table . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

Table 7.

STM32F40xxx pin and ball definitions  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 48

Table 8.

FSMC pin definition  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 60

Table 9.

Alternate function mapping . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 63

Table 10.

Register boundary addresses. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 73

Table 11.

Voltage characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 79

Table 12.

Current characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 80

Table 13.

Thermal characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 80

Table 14.

General operating conditions . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 80

Table 15.

Limitations depending on the operating power supply range . . . . . . . . . . . . . . . . . . . . . . . 82

Table 16.

VCAP_1/VCAP_2 operating conditions  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 83

Table 17.

Operating conditions at power-up / power-down (regulator ON)  . . . . . . . . . . . . . . . . . . . . 83

Table 18.

Operating conditions at power-up / power-down (regulator OFF). . . . . . . . . . . . . . . . . . . . 83

Table 19.

Embedded reset and power control block characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . 84

Table 20.

Typical and maximum current consumption in Run mode, code with data processing

running from flash memory (ART accelerator enabled) or RAM . . . . . . . . . . . . . . . . . . . . 86

Table 21.

Typical and maximum current consumption in Run mode, code with data processing

running from flash memory (ART accelerator disabled)  . . . . . . . . . . . . . . . . . . . . . . . . . . 87

Table 22.

Typical and maximum current consumption in Sleep mode . . . . . . . . . . . . . . . . . . . . . . . . 90

Table 23.

Typical and maximum current consumptions in Stop mode . . . . . . . . . . . . . . . . . . . . . . . . 91

Table 24.

Typical and maximum current consumptions in Standby mode . . . . . . . . . . . . . . . . . . . . . 91

Table 25.

Typical and maximum current consumptions in V

mode. . . . . . . . . . . . . . . . . . . . . . . . 92

BAT

Table 26.

Typical current consumption in Run mode, code with data processing

running from flash memory, regulator ON (ART accelerator enabled

except prefetch), V

= 1.8 V . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 94

DD

Table 27.

Switching output I/O current consumption  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 96

Table 28.

Peripheral current consumption . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 97

Table 29.

Low-power mode wakeup timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 100

Table 30.

High-speed external user clock characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 100

Table 31.

Low-speed external user clock characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 101

Table 32.

HSE 4-26 MHz oscillator characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 102

Table 33.

LSE oscillator characteristics (f

= 32.768 kHz) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 103

LSE

Table 34.

HSI oscillator characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 104

Table 35.

LSI oscillator characteristics   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 104

Table 36.

Main PLL characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 105

Table 37.

PLLI2S (audio PLL) characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 106

Table 38.

SSCG parameters constraint . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 107

Table 39.

Flash memory characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 109

Table 40.

Flash memory programming. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 109

Table 41.

Flash memory programming with VPP . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 111

Table 42.

Flash memory endurance and data retention . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 111

Table 43.

EMS characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 112

Table 44.

EMI characteristics for fHSE = 25 MH and fCPU = 168 MHz . . . . . . . . . . . . . . . . . . . . . . 113

DS8626 Rev 11

7/205

9

List of tables

STM32F405xx, STM32F407xx

Table 45.

ESD absolute maximum ratings . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 113

Table 46.

Electrical sensitivities . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 114

Table 47.

I/O current injection susceptibility . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 115

Table 48.

I/O static characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 115

Table 49.

Output voltage characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 117

Table 50.

I/O AC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 118

Table 51.

NRST pin characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

Table 52.

Characteristics of TIMx connected to the APB1 domain  . . . . . . . . . . . . . . . . . . . . . . . . . 121

Table 53.

Characteristics of TIMx connected to the APB2 domain  . . . . . . . . . . . . . . . . . . . . . . . . . 122

Table 54.

I2C analog filter characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 122

Table 55.

SPI dynamic characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 123

Table 56.

I2S dynamic characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 127

Table 57.

USB OTG FS startup time  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 129

Table 58.

USB OTG FS DC electrical characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 129

Table 59.

USB OTG FS electrical characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 130

Table 60.

USB HS DC electrical characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 130

Table 61.

USB HS clock timing parameters . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 130

Table 62.

ULPI timing . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 131

Table 63.

Ethernet DC electrical characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 132

Table 64.

Dynamic characteristics: Ethernet MAC signals for SMI. . . . . . . . . . . . . . . . . . . . . . . . . . 132

Table 65.

Dynamic characteristics: Ethernet MAC signals for RMII . . . . . . . . . . . . . . . . . . . . . . . . . 133

Table 66.

Dynamic characteristics: Ethernet MAC signals for MII . . . . . . . . . . . . . . . . . . . . . . . . . . 134

Table 67.

ADC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 134

Table 68.

ADC accuracy at f

= 30 MHz  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 136

ADC

Table 69.

Temperature sensor characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

Table 70.

Temperature sensor calibration values. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 139

Table 71.

V

monitoring characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

BAT

Table 72.

Embedded internal reference voltage. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

Table 73.

Internal reference voltage calibration values  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

Table 74.

DAC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 140

Table 75.

Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings . . . . . . . . . . . . . . . . . 144

Table 76.

Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings . . . . . . . . . . . . . . . . . 145

Table 77.

Asynchronous multiplexed PSRAM/NOR read timings. . . . . . . . . . . . . . . . . . . . . . . . . . . 146

Table 78.

Asynchronous multiplexed PSRAM/NOR write timings  . . . . . . . . . . . . . . . . . . . . . . . . . . 147

Table 79.

Synchronous multiplexed NOR/PSRAM read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . 149

Table 80.

Synchronous multiplexed PSRAM write timings. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 150

Table 81.

Synchronous non-multiplexed NOR/PSRAM read timings . . . . . . . . . . . . . . . . . . . . . . . . 152

Table 82.

Synchronous non-multiplexed PSRAM write timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . 153

Table 83.

Switching characteristics for PC Card/CF read and write cycles

in attribute/common space. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 158

Table 84.

Switching characteristics for PC Card/CF read and write cycles

in I/O space . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 159

Table 85.

Switching characteristics for NAND Flash read cycles . . . . . . . . . . . . . . . . . . . . . . . . . . . 160

Table 86.

Switching characteristics for NAND Flash write cycles. . . . . . . . . . . . . . . . . . . . . . . . . . . 161

Table 87.

DCMI characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 161

Table 88.

Dynamic characteristics: SD/MMC characteristics . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163

Table 89.

RTC characteristics  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163

Table 90.

WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 166

Table 91.

WLCSP90 recommended PCB design rules  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 167

Table 92.

LQFP64 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 169

Table 93.

LQFP100 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 172

8/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

List of tables

Table 94.

LQFP144 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 175

Table 95.

UFBGA(176+25) - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 178

Table 96.

UFBGA(176+25) - Example of PCB design rules (0.65 mm pitch BGA)  . . . . . . . . . . . . . 179

Table 97.

LQFP176 - Mechanical data . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 181

Table 98.

Package thermal characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 184

Table 99.

Document revision history  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 192

DS8626 Rev 11

9/205

9

List of figures

STM32F405xx, STM32F407xx

List of figures

Figure 1.

Compatible board design between STM32F10xx/STM32F40xxx for LQFP64 . . . . . . . . . . 17

Figure 2.

Compatible board design STM32F10xx/STM32F2/STM32F40xxx

for LQFP100 package. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

Figure 3.

Compatible board design between STM32F10xx/STM32F2/STM32F40xxx

for LQFP144 package. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 18

Figure 4.

Compatible board design between STM32F2 and STM32F40xxx

for LQFP176 and BGA176 packages  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 19

Figure 5.

STM32F40xxx block diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 20

Figure 6.

Multi-AHB matrix. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 23

Figure 7.

Power supply supervisor interconnection with internal reset OFF . . . . . . . . . . . . . . . . . . . 26

Figure 8.

PDR_ON and NRST control with internal reset OFF . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 27

Figure 9.

Regulator OFF  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 28

Figure 10.

Startup in regulator OFF mode: slow V

slope

DD

- power-down reset risen after V

/V

stabilization . . . . . . . . . . . . . . . . . . . . . . . . 29

CAP_1

CAP_2

Figure 11.

Startup in regulator OFF mode: fast V

slope

DD

- power-down reset risen before V

/V

stabilization  . . . . . . . . . . . . . . . . . . . . . . 30

CAP_1

CAP_2

Figure 12.

STM32F40xxx LQFP64 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 42

Figure 13.

STM32F40xxx LQFP100 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 43

Figure 14.

STM32F40xxx LQFP144 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 44

Figure 15.

STM32F40xxx LQFP176 pinout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 45

Figure 16.

STM32F40xxx UFBGA176 ballout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 46

Figure 17.

STM32F40xxx WLCSP90 ballout . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 47

Figure 18.

STM32F40xxx memory map. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 72

Figure 19.

Pin loading conditions. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

Figure 20.

Pin input voltage . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 77

Figure 21.

Power supply scheme  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 78

Figure 22.

Current consumption measurement scheme . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 79

Figure 23.

External capacitor C

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 83

EXT

Figure 24.

Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator ON) or RAM, and peripherals OFF . . . . 88

Figure 25.

Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator ON) or RAM, and peripherals ON . . . . . 88

Figure 26.

Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator OFF) or RAM, and peripherals OFF . . . 89

Figure 27.

Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator OFF) or RAM, and peripherals ON . . . . 89

Figure 28.

Typical V

current consumption (LSE and RTC ON/backup RAM OFF)  . . . . . . . . . . . . 92

BAT

Figure 29.

Typical V

current consumption (LSE and RTC ON/backup RAM ON) . . . . . . . . . . . . . 93

BAT

Figure 30.

High-speed external clock source AC timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . 101

Figure 31.

Low-speed external clock source AC timing diagram. . . . . . . . . . . . . . . . . . . . . . . . . . . . 102

Figure 32.

Typical application with an 8 MHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 103

Figure 33.

Typical application with a 32.768 kHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 104

Figure 34.

ACC

versus temperature . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 105

LSI

Figure 35.

PLL output clock waveforms in center spread mode  . . . . . . . . . . . . . . . . . . . . . . . . . . . . 108

Figure 36.

PLL output clock waveforms in down spread mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 109

Figure 37.

I/O AC characteristics definition . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 119

Figure 38.

Recommended NRST pin protection  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 120

Figure 39.

SPI timing diagram - slave mode and CPHA = 0 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 125

10/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

List of figures

Figure 40.

SPI timing diagram - slave mode and CPHA = 1 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 125

Figure 41.

SPI timing diagram - master mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 126

Figure 42.

I2S slave timing diagram (Philips protocol)  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 128

(1)

Figure 43.

I2S master timing diagram (Philips protocol)

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 128

Figure 44.

USB OTG FS timings: definition of data signal rise and fall time . . . . . . . . . . . . . . . . . . . 130

Figure 45.

ULPI timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 131

Figure 46.

Ethernet SMI timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 132

Figure 47.

Ethernet RMII timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 133

Figure 48.

Ethernet MII timing diagram . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 133

Figure 49.

ADC accuracy characteristics. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 136

Figure 50.

Typical connection diagram when using the ADC with FT/TT pins featuring

analog switch function  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 137

Figure 51.

Power supply and reference decoupling (V

not connected to V

). . . . . . . . . . . . . 138

REF+

DDA

Figure 52.

Power supply and reference decoupling (V

connected to V

). . . . . . . . . . . . . . . . 139

REF+

DDA

Figure 53.

12-bit buffered /non-buffered DAC . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 143

Figure 54.

Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms . . . . . . . . . . . . . . 144

Figure 55.

Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms . . . . . . . . . . . . . . 145

Figure 56.

Asynchronous multiplexed PSRAM/NOR read waveforms. . . . . . . . . . . . . . . . . . . . . . . . 146

Figure 57.

Asynchronous multiplexed PSRAM/NOR write waveforms  . . . . . . . . . . . . . . . . . . . . . . . 147

Figure 58.

Synchronous multiplexed NOR/PSRAM read timings  . . . . . . . . . . . . . . . . . . . . . . . . . . . 148

Figure 59.

Synchronous multiplexed PSRAM write timings. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 150

Figure 60.

Synchronous non-multiplexed NOR/PSRAM read timings . . . . . . . . . . . . . . . . . . . . . . . . 151

Figure 61.

Synchronous non-multiplexed PSRAM write timings . . . . . . . . . . . . . . . . . . . . . . . . . . . . 153

Figure 62.

PC Card/CompactFlash controller waveforms for common memory read access . . . . . . 154

Figure 63.

PC Card/CompactFlash controller waveforms for common memory write access. . . . . . 155

Figure 64.

PC Card/CompactFlash controller waveforms for attribute memory read

access. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 156

Figure 65.

PC Card/CompactFlash controller waveforms for attribute memory write

access. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 157

Figure 66.

PC Card/CompactFlash controller waveforms for I/O space read access . . . . . . . . . . . . 157

Figure 67.

PC Card/CompactFlash controller waveforms for I/O space write access . . . . . . . . . . . . 158

Figure 68.

NAND controller waveforms for read access . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 160

Figure 69.

NAND controller waveforms for write access . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 160

Figure 70.

DCMI timing diagram  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 161

Figure 71.

SDIO high-speed mode  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 162

Figure 72.

SD default mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 163

Figure 73.

WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package outline. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 165

Figure 74.

WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package recommended footprint  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 166

Figure 75.

WLCSP90 marking example (package top view) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 167

(15)

Figure 76.

LQFP64 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 168

(15)

Figure 77.

LQFP100 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 171

Figure 78.

LQFP100 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 173

(15)

Figure 79.

LQFP144 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 174

Figure 80.

LQFP144 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 177

Figure 81.

UFBGA(176+25) - Outline  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 178

Figure 82.

UFBGA(176+25) - Footprint example. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 179

(15)

Figure 83.

LQFP176 - Outline

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 180

Figure 84.

LQFP176 - Footprint example  . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 183

Figure 85.

USB controller configured as peripheral-only and used

in Full speed mode . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 186

DS8626 Rev 11

11/205

12

List of figures

STM32F405xx, STM32F407xx

Figure 86.

USB controller configured as host-only and used in full speed mode. . . . . . . . . . . . . . . . 186

Figure 87.

USB controller configured in dual mode and used in full speed mode . . . . . . . . . . . . . . . 187

Figure 88.

USB controller configured as peripheral, host, or dual-mode

and used in high speed mode. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 188

Figure 89.

MII mode using a 25 MHz crystal . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 189

Figure 90.

RMII with a 50 MHz oscillator . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 189

Figure 91.

RMII with a 25 MHz crystal and PHY with PLL. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . 190

12/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Introduction

1

Introduction

This datasheet provides the description of the STM32F405xx and STM32F407xx lines of

microcontrollers. For more details on the whole STMicroelectronics STM32™ family, refer to

Section 2.1: Full compatibility throughout the family.

The STM32F405xx and STM32F407xx datasheet should be read in conjunction with the

STM32F4xx reference manual which is available from the STMicroelectronics website

www.st.com.

For information on the device errata with respect to the datasheet and reference manual,

refer to the STM32F405xx and STM32F407xx errata sheet (ES0182), which is available

from the STMicroelectronics website www.st.com.

®

®

®

For information on the Arm

Cortex

-M4 core, refer to the Cortex

-M4 programming

manual (PM0214) available from www.st.com.

Note:

Arm and Cortex are registered trademarks of Arm Limited (or its subsidiaries or affiliates) in

the US and/or elsewhere.

The Arm word and logo are trademarks of Arm Limited (or its subsidiaries) in the US and/or

elsewhere. All rights reserved.

DS8626 Rev 11

13/205

190

Description

STM32F405xx, STM32F407xx

2

Description

®

The STM32F405xx and STM32F407xx family is based on the high-performance Arm

®

®

Cortex

-M4 32-bit RISC core operating at a frequency of up to 168 MHz. The Cortex

-M4

core features a floating-point unit (FPU) single precision which supports all Arm single-

precision data-processing instructions and data types. It also implements a full set of DSP

instructions and a memory protection unit (MPU) which enhances application security.

The STM32F405xx and STM32F407xx family incorporates high-speed embedded

memories (flash memory up to 1 Mbyte, up to 192 Kbytes of SRAM), up to 4 Kbytes of

backup SRAM, and an extensive range of enhanced I/Os and peripherals connected to two

APB buses, three AHB buses and a 32-bit multi-AHB bus matrix.

All devices offer three 12-bit ADCs, two DACs, a low-power RTC, 12 general-purpose 16-bit

timers including two PWM timers for motor control, two general-purpose 32-bit timers. a true

random number generator (RNG). They also feature standard and advanced

communication interfaces.

2

•

Up to three I

Cs

2

•

Three SPIs, two I

Ss full duplex. To achieve audio class accuracy, the I2S peripherals

can be clocked via a dedicated internal audio PLL or via an external clock to allow

synchronization

•

Four USARTs plus two UARTs

•

A USB OTG full speed and a USB OTG high speed with full-speed capability (with the

ULPI)

•

Two CANs

•

An SDIO/MMC interface

•

Ethernet and the camera interface available on STM32F407xx devices only

New advanced peripherals include an SDIO, an enhanced flexible static memory control

(FSMC) interface (for devices offered in packages of 100 pins and more), a camera

interface for CMOS sensors. Refer to Table 2: STM32F405xx and STM32F407xx: features

and peripheral counts for the list of peripherals available on each part number.

The STM32F405xx and STM32F407xx family operates in the –40 to +105 °C temperature

range from a 1.8 to 3.6 V power supply. The supply voltage can drop to 1.7 V when the

device operates in the 0 to 70 °C temperature range using an external power supply

supervisor: refer to Section : Internal reset OFF. A comprehensive set of power-saving

mode allows the design of low-power applications.

The STM32F405xx and STM32F407xx family offers devices in various packages ranging

from 64 pins to 176 pins. The set of included peripherals changes with the device chosen.

These features make the STM32F405xx and STM32F407xx microcontroller family suitable

for a wide range of applications:

•

Motor drive and application control

•

Medical equipment

•

Industrial applications: PLC, inverters, circuit breakers

•

Printers, and scanners

•

Alarm systems, video intercom, and HVAC

•

Home audio appliances

14/205

DS8626 Rev 11

Figure 5 shows the general block diagram of the device family.

STM32F405xx, STM32F407xx

Table 2. STM32F405xx and STM32F407xx: features and peripheral counts

Peripherals

STM32F405RG STM32F405OG STM32F405VG STM32F405ZG STM32F405OE STM32F407Vx STM32F407Zx STM32F407Ix

Flash memory in

1024

512

512

1024

512

1024

512

1024

Kbytes

System

192(112+16+64)

SRAM in

Kbytes

Backup

4

FSMC memory

(1)

No

Yes

controller

Ethernet

No

Yes

General-

10

purpose

DS8626 Rev 11

Advanced

2

-control

Timers

Basic

2

IWDG

Yes

WWDG

Yes

RTC

Yes

True random number

Yes

generator

Description

15/205

Table 2. STM32F405xx and STM32F407xx: features and peripheral counts (continued)

16/205

Description

Peripherals

STM32F405RG STM32F405OG STM32F405VG STM32F405ZG STM32F405OE STM32F407Vx STM32F407Zx STM32F407Ix

(2)

SPI / I2S

3/2 (full duplex)

2

I

C

3

USART/

4/2

UART

Communi

USB

cation

Yes

OTG FS

interfaces

USB

Yes

OTG HS

CAN

2

SDIO

Yes

Camera interface

No

Yes

DS8626 Rev 11

GPIOs

51

72

82

114

72

82

114

140

3

12-bit ADC

Number of channels

16

13

16

24

13

16

24

24

12-bit DAC

Yes

Number of channels

2

Maximum CPU

168 MHz

frequency

(3)

Operating voltage

1.8 to 3.6 V

STM32F405xx, STM32F407xx

Ambient temperatures: –40 to +85 °C /–40 to +105 °C

Operating

temperatures

Junction temperature: –40 to + 125 °C

UFBGA176

Package

LQFP64

WLCSP90

LQFP100

LQFP144

WLCSP90

LQFP100

LQFP144

LQFP176

1.

For the LQFP100 and WLCSP90 packages, only FSMC Bank1 or Bank2 are available. Bank1 can only support a multiplexed NOR/PSRAM memory using the NE1 Chip

Select. Bank2 can only support a 16- or 8-bit NAND flash memory using the NCE2 Chip Select. The interrupt line cannot be used since Port G is not available in this

package.

2

S audio mode.

2.

The SPI2 and SPI3 interfaces give the flexibility to work in an exclusive way in either the SPI mode or the I

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use of an external power supply supervisor (refer to

3.

V

DD

DDA

Section : Internal reset OFF).

STM32F405xx, STM32F407xx

Description

2.1

Full compatibility throughout the family

The STM32F405xx and STM32F407xx are part of the STM32F4 family. They are fully pin-

to-pin, software and feature compatible with the STM32F2xx devices, allowing the user to

try different memory densities, peripherals, and performances (FPU, higher frequency) for a

greater degree of freedom during the development cycle.

The STM32F405xx and STM32F407xx devices maintain a close compatibility with the

whole STM32F10xxx family. All functional pins are pin-to-pin compatible. The

STM32F405xx and STM32F407xx, however, are not drop-in replacements for the

STM32F10xxx devices: the two families do not have the same power scheme, and so their

power pins are different. Nonetheless, transition from the STM32F10xxx to the

STM32F40xxx family remains simple as only a few pins are impacted.

Figure 4, Figure 3, Figure 2, and Figure 1 give compatible board designs between the

STM32F40xxx, STM32F2, and STM32F10xxx families.

Figure 1. Compatible board design between STM32F10xx/STM32F40xxx for LQFP64

V

SS

V

SS

48

33

32

47

49

31

V

SS

V

SS

Ω

0

resistor or soldering bridge

present for the STM32F10xx

configuration, not present in the

STM32F4xx configuration

17

64

1

16

ai18489

DS8626 Rev 11

17/205

190

Description

STM32F405xx, STM32F407xx

Figure 2. Compatible board design STM32F10xx/STM32F2/STM32F40xxx

for LQFP100 package

75

V

SS

51

50

76

73

49

V

SS

V

SS

UHVLVWRURUVROGHULQJEULGJH

SUHVHQWIRUWKH670

32F10xxx

FRQILJXUDWLRQQRWSUHVHQWLQWKH

99 (VSS)

100

26

19

20

670)[[FRQILJXUDWLRQ

1

25

V

SS

V

SS

966IRU670)[[

7ZRUHVLVWRUVFRQQHFWHGWR

9''IRU670)[[

V

V

DD

SS

966IRUWKH670)[[

-

966IRUWKH670)[[

-

966RU1&IRUWKH670)[[

-

ai18488d

Figure 3. Compatible board design between STM32F10xx/STM32F2/STM32F40xxx

for LQFP144 package

UHVLVWRURUVROGHULQJEULGJH

SUHVHQWIRUWKH670

32F10xx

V

FRQILJXUDWLRQQRWSUHVHQWLQWKH

SS

73

108

670)[[FRQILJXUDWLRQ

106

109

72

71

V

SS

1RWSRSXODWHGZKHQ

V

SS

UHVLVWRURUVROGHULQJ

EULGJHSUHVHQW

6LJQDOIURP

H[WHUQDOSRZHU

3'5B21

VXSSO\

144

VXSHUYLVRU

37

30

31

1

36

V

V

V

SS

DD

SS

1RWSRSXODWHGIRU670

32F10xx

7ZRUHVLVWRUVFRQQHFWHGWR

IRU670)[[

V

SS

IRUWKH670)[[

- V

V

V

SS

IRU670)[[

V

DD

SS

DD

RU1&IRUWKH670)[[

, V

- V

DD

SS

RUVLJQDOIURPH[WHUQDOSRZHUVXSSO\VXSHUYLVRUIRUWKH670)[[

- V

DD

ai18487d

18/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Description

Figure 4. Compatible board design between STM32F2 and STM32F40xxx

for LQFP176 and BGA176 packages

132

89

6LJQDOIURPH[WHUQDO

SRZHUVXSSO\

88

133

3'5B21

VXSHUYLV

U

o

176

45

1

44

V

V

DD

SS

7ZRUHVLVWRUVFRQQHFWHGWR

RU1&IRUWKH670)[[

, V

- V

DD

SS

RUVLJQDOIURPH[WHUQDOSRZHUVXSSO\VXSHUYLV

UIRUWKH670)[[

o

- V

DD

MS19919V3

DS8626 Rev 11

19/205

190

Functional overview

STM32F405xx, STM32F407xx

3

Functional overview

Figure 5. STM32F40xxx block diagram

External memory

CLK, NE [3:0], A[23:0],

controller (FSMC)

D[31:0], OEN, WEN,

NBL[3:0], NL, NREG,

JTAG & SW

MPU

NWAIT/IORDY, CD

JTDO/SWD, JTDO

CCM data RAM 64 KB

NJTRST, JTDI,

AHB3

SRAM, PSRAM, NOR Flash,

JTCK/SWCLK

PC Card (ATA), NAND Flash

NVIC

ETM

NIORD, IOWR, INT[2:3]

TRACECLK

INTN, NIIS16 as AF

D-BUS

TRACED[3:0]

Arm Cortex-M4

168 MHz

I-BUS

FPU

Flash

S-BUS

up to

1 MB

Ethernet MAC

CACHE

DMA/

MII or RMII as AF

RNG

ART ACCEL/

10/100

FIFO

MDIO as AF

SRAM 112 KB

PUIXCLK, D[13:0]

USB

DMA/

DP, DM

HSYNC, VSYNC

Camera

interface

OTG HS

ULPI:CK, D[7:0], DIR, STP, NXT

DP

USB

PHY

ID, VBUS, SOF

DM

AHB bus-matrix 8S7M

FIFO

SRAM 16 KB

FIFO

OTG FS

8 Streams

ID, VBUS, SOF

DMA2

PHY

AHB2 168 MHz

FIFO

FIFO

DMA1

AHB1 168 MHz

8 Streams

FIFO

Power managmt

VDD

Voltage

VDD = 1.8 to 3.6 V

regulator

VSS

3.3 to 1.2 V

VCAP1, VCPA2

@V

DD

@V

DDA

PA[15:0]

GPIO PORT A

VDDA, VSSA

GPIO PORT B

PB[15:0]

RC HS

reset

RC L S

P L L1&2

NRST

GPIO PORT C

PC[15:0]

Supply

POR

supervision

Int

POR/PDR

BOR

PVD

@V

DD

@V

DDA

GPIO PORT D

PD[15:0]

OSC_IN

XTAL OSC

OSC_OUT

4- 16MHz

PE[15:0]

Reset &

GPIO PORT E

clock

control

PF[15:0]

IWDG

M AN A G T

GPIO PORT F

PWR

VBAT = 1.65 to 3.6 V

interface

PG[15:0]

GPIO PORT G

@V

BAT

OSC32_IN

PH[15:0]

XTAL 32 kHz

FCLK

GPIO PORT H

PCLKx

HCLKx

RTC

LS

PI[11:0]

GPIO PORT I

AWU

OSC32_OUT

RTC_AF1

RTC_AF1

Backup register

LS

4 KB BKPSRAM

TIM2

4 channels, ETR as AF

32b

TIM3

140 AF

4 channels, ETR as AF

16b

EXT IT. WKUP

DMA2

TIM4

16b

4 channels, ETR as AF

DMA1

D[7:0]

SDIO / MMC

CMD, CK as AF

TIM5

4 channels

32b

AHB/APB2

AHB/APB1

FIFO

4 compl. channels (TIM1_CH1[1:4]N,

TIM12

2 channels as AF

16b

TIM1 / PWM

4 channels (TIM1_CH1[1:4]ETR,

16b

BKIN as AF

TIM13

1 channel as AF

16b

4 compl. channels (TIM1_CH1[1:4]N,

TIM14

1 channel as AF

16b

TIM8 / PWM

4 channels (TIM1_CH1[1:4]ETR,

16b

RX, TX as AF

BKIN as AF

smcard

2 channels as AF

USART2

CTS, RTS as AF

irDA

TIM9

16b

RX, TX as AF

smcard

USART3

CTS, RTS as AF

irDA

1 channel as AF

TIM10

16b

WWDG

UART4

RX, TX as AF

1 channel as AF

TIM11

16b

A P B 1 3 0 M Hz

RX, TX, CK,

CTS, RTS as AF

UART5

RX, TX as AF

smcard

USART1

APB2 84 MHz (max)

MOSI/SD, MISO/SD_ext, SCK/CK

irDA

smcard

USART6

SP2/I2S2

NSS/WS, MCK as AF

RX, TX, CK,

TIM6

16b

MOSI/SD, MISO/SD_ext, SCK/CK

irDA

CTS, RTS as AF

APB1 42  MHz (max)

NSS/WS, MCK as AF

MOSI, MISO,

SCK, NSS as AF

SPI1

SP3/I2S3

TIM7

@V

DDA

I2C1/SMBUS

SCL, SDA, SMBA as AF

16b

VDDREF_ADC

8 analog inputs common

@V

DDA

I2C2/SMBUS

SCL, SDA, SMBA as AF

Temperature sensor

DAC1

ADC1

I2C3/SMBUS

SCL, SDA, SMBA as AF

to the 3 ADCs

ITF

DAC2

8 analog inputs common

ADC2

IF

bxCAN1

TX, RX

to the ADC1 & 2

TX, RX

8 analog inputs for ADC3

ADC3

bxCAN2

FIFO

DAC1_OUT

DAC2_OUT

as AF

as AF

MS19920V5

1.

The camera interface and ethernet are available only on STM32F407xx devices.

20/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

®

®

3.0.1

Arm

Cortex

-M4 core with FPU and embedded Flash and SRAM

The Arm Cortex-M4 processor with FPU is the latest generation of Arm processors for

embedded systems. It was developed to provide a low-cost platform that meets the needs of

MCU implementation, with a reduced pin count and low-power consumption, while

delivering outstanding computational performance and an advanced response to interrupts.

The Arm Cortex-M4 32-bit RISC processor with FPU features exceptional code-efficiency,

delivering the high-performance expected from an Arm core in the memory size usually

associated with 8- and 16-bit devices.

The processor supports a set of DSP instructions which allow efficient signal processing and

complex algorithm execution.

Its single precision FPU (floating point unit) speeds up software development by using

metalanguage development tools, while avoiding saturation.

The STM32F405xx and STM32F407xx family is compatible with all Arm tools and software.

Figure 5 shows the general block diagram of the STM32F40xxx family.

Note:

Cortex-M4 with FPU is binary compatible with Cortex-M3.

3.0.2

Adaptive real-time memory accelerator (ART Accelerator)

The ART Accelerator is a memory accelerator which is optimized for STM32 industry-

®

®

standard Arm

Cortex

-M4 with FPU processors. It balances the inherent performance

advantage of the Arm Cortex-M4 with FPU over flash memory technologies, which normally

requires the processor to wait for the flash memory at higher frequencies.

To release the processor full 210 DMIPS performance at this frequency, the accelerator

implements an instruction prefetch queue and branch cache, which increases program

execution speed from the 128-bit flash memory. Based on CoreMark benchmark, the

performance achieved thanks to the ART accelerator is equivalent to 0 wait state program

execution from flash memory at a CPU frequency up to 168 MHz.

3.0.3

Memory protection unit

The memory protection unit (MPU) is used to manage the CPU accesses to memory to

prevent one task to accidentally corrupt the memory or resources used by any other active

task. This memory area is organized into up to 8 protected areas that can in turn be divided

up into 8 subareas. The protection area sizes are between 32 bytes and the whole 4

gigabytes of addressable memory.

The MPU is especially helpful for applications where some critical or certified code has to be

protected against the misbehavior of other tasks. It is usually managed by an RTOS (real-

time operating system). If a program accesses a memory location that is prohibited by the

MPU, the RTOS can detect it and take action. In an RTOS environment, the kernel can

dynamically update the MPU area setting, based on the process to be executed.

The MPU is optional and can be bypassed for applications that do not need it.

3.0.4

Embedded flash memory

The STM32F40xxx devices embed a flash memory of 512 Kbytes or 1 Mbytes available for

storing programs and data, plus 512 bytes of OTP memory.

DS8626 Rev 11

21/205

190

Functional overview

STM32F405xx, STM32F407xx

3.0.5

CRC (cyclic redundancy check) calculation unit

The CRC (cyclic redundancy check) calculation unit is used to get a CRC code from a 32-bit

data word and a fixed generator polynomial.

Among other applications, CRC-based techniques are used to verify data transmission or

storage integrity. In the scope of the EN/IEC 60335-1 standard, they offer a means of

verifying the flash memory integrity. The CRC calculation unit helps compute a software

signature during runtime, to be compared with a reference signature generated at link-time

and stored at a given memory location.

3.0.6

Embedded SRAM

All STM32F40xxx products embed:

•

Up to 192 Kbytes of system SRAM including 64 Kbytes of CCM (core coupled memory)

data RAM

RAM memory is accessed (read/write) at CPU clock speed with 0 wait states.

•

4 Kbytes of backup SRAM

This area is accessible only from the CPU. Its content is protected against possible

unwanted write accesses, and is retained in Standby or V

mode.

BAT

3.0.7

Multi-AHB bus matrix

The 32-bit multi-AHB bus matrix interconnects all the masters (CPU, DMAs, Ethernet, USB

HS) and the slaves (flash memory, RAM, FSMC, AHB, and APB peripherals) and ensures a

seamless and efficient operation even when several high-speed peripherals work

simultaneously.

22/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

Figure 6. Multi-AHB matrix

64-Kbyte

CCM data RAM

ARM

GP

GP

MAC

USB OTG

Cortex-M4

DMA1

DMA2

Ethernet

HS

I-bus

S-bus

D-bus

DMA_P1

DMA_P2

USB_HS_M

DMA_MEM1

DMA_MEM2

ETHERNET_M

ICODE

Flash

memory

DCODE

ACCEL

SRAM1

112 Kbyte

SRAM2

16 Kbyte

AHB1

APB1

peripherals

AHB2

peripherals

FSMC

APB2

Static MemCtl

Bus matrix-S

ai18490e

3.0.8

DMA controller (DMA)

The devices feature two general-purpose dual-port DMAs (DMA1 and DMA2) with 8

streams each. They are able to manage memory-to-memory, peripheral-to-memory and

memory-to-peripheral transfers. They feature dedicated FIFOs for APB/AHB peripherals,

support burst transfer and are designed to provide the maximum peripheral bandwidth

(AHB/APB).

The two DMA controllers support circular buffer management, so that no specific code is

needed when the controller reaches the end of the buffer. The two DMA controllers also

have a double buffering feature, which automates the use and switching of two memory

buffers without requiring any special code.

Each stream is connected to dedicated hardware DMA requests, with support for software

trigger on each stream. Configuration is made by software and transfer sizes between

source and destination are independent.

The DMA can be used with the main peripherals:

2

•

SPI and I

S

2

•

I

C

•

USART

•

General-purpose, basic and advanced-control timers TIMx

•

DAC

•

SDIO

•

Camera interface (DCMI)

•

ADC.

DS8626 Rev 11

23/205

190

Functional overview

STM32F405xx, STM32F407xx

3.0.9

Flexible static memory controller (FSMC)

The FSMC is embedded in the STM32F405xx and STM32F407xx family. It has four Chip

Select outputs supporting the following modes: PCCard/Compact Flash, SRAM, PSRAM,

NOR Flash and NAND Flash.

Functionality overview:

•

Write FIFO

•

Maximum FSMC_CLK frequency for synchronous accesses is 60 MHz.

LCD parallel interface

The FSMC can be configured to interface seamlessly with most graphic LCD controllers. It

supports the Intel 8080 and Motorola 6800 modes, and is flexible enough to adapt to

specific LCD interfaces. This LCD parallel interface capability makes it easy to build cost-

effective graphic applications using LCD modules with embedded controllers or high

performance solutions using external controllers with dedicated acceleration.

3.0.10

Nested vectored interrupt controller (NVIC)

The STM32F405xx and STM32F407xx embed a nested vectored interrupt controller able to

manage 16 priority levels, and handle up to 82 maskable interrupt channels plus the 16

®

interrupt lines of the Cortex

-M4 with FPU core.

•

Closely coupled NVIC gives low-latency interrupt processing

•

Interrupt entry vector table address passed directly to the core

•

Allows early processing of interrupts

•

Processing of late arriving, higher-priority interrupts

•

Support tail chaining

•

Processor state automatically saved

•

Interrupt entry restored on interrupt exit with no instruction overhead

This hardware block provides flexible interrupt management features with minimum interrupt

latency.

3.0.11

External interrupt/event controller (EXTI)

The external interrupt/event controller consists of 23 edge-detector lines used to generate

interrupt/event requests. Each line can be independently configured to select the trigger

event (rising edge, falling edge, both) and can be masked independently. A pending register

maintains the status of the interrupt requests. The EXTI can detect an external line with a

pulse width shorter than the Internal APB2 clock period. Up to 140 GPIOs can be connected

to the 16 external interrupt lines.

3.0.12

Clocks and startup

On reset the 16 MHz internal RC oscillator is selected as the default CPU clock. The

16 MHz internal RC oscillator is factory-trimmed to offer 1% accuracy over the full

temperature range. The application can then select as system clock either the RC oscillator

or an external 4-26 MHz clock source. This clock can be monitored for failure. If a failure is

detected, the system automatically switches back to the internal RC oscillator and a

software interrupt is generated (if enabled). This clock source is input to a PLL thus allowing

to increase the frequency up to 168 MHz. Similarly, full interrupt management of the PLL

24/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

clock entry is available when necessary (for example if an indirectly used external oscillator

fails).

Several prescalers allow the configuration of the three AHB buses, the high-speed APB

(APB2) and the low-speed APB (APB1) domains. The maximum frequency of the three AHB

buses is 168 MHz while the maximum frequency of the high-speed APB domains is

84 MHz. The maximum allowed frequency of the low-speed APB domain is 42 MHz.

The devices embed a dedicated PLL (PLLI2S) which allows to achieve audio class

2

performance. In this case, the I

S master clock can generate all standard sampling

frequencies from 8 kHz to 192 kHz.

3.0.13

Boot modes

At startup, boot pins are used to select one out of three boot options:

•

Boot from user Flash

•

Boot from system memory

•

Boot from embedded SRAM

The boot loader is located in system memory. It is used to reprogram the flash memory by

using USART1 (PA9/PA10), USART3 (PC10/PC11 or PB10/PB11), CAN2 (PB5/PB13), USB

OTG FS in Device mode (PA11/PA12) through DFU (device firmware upgrade).

3.0.14

Power supply schemes

•

V

= 1.8 to 3.6 V: external power supply for I/Os and the internal regulator (when

DD

enabled), provided externally through V

pins.

DD

•

V

, V

= 1.8 to 3.6 V: external analog power supplies for ADC, DAC, Reset

SSA

DDA

blocks, RCs and PLL. V

and V

must be connected to V

and V

, respectively.

DDA

SSA

DD

SS

•

V

= 1.65 to 3.6 V: power supply for RTC, external clock 32 kHz oscillator and

BAT

backup registers (through power switch) when V

is not present.

DD

Refer to Figure 21: Power supply scheme for more details.

Note:

V

/V

minimum value of 1.7 V is obtained when the device operates in reduced

DD

DDA

temperature range, and with the use of an external power supply supervisor (refer to

Section : Internal reset OFF).

Refer to Table 2 in order to identify the packages supporting this option.

3.0.15

Power supply supervisor

Internal reset ON

On packages embedding the PDR_ON pin, the power supply supervisor is enabled by

holding PDR_ON high. On all other packages, the power supply supervisor is always

enabled.

The device has an integrated power-on reset (POR) / power-down reset (PDR) circuitry

coupled with a Brownout reset (BOR) circuitry. At power-on, POR/PDR is always active and

ensures proper operation starting from 1.8 V. After the 1.8 V POR threshold level is

reached, the option byte loading process starts, either to confirm or modify default BOR

threshold levels, or to disable BOR permanently. Three BOR thresholds are available

through option bytes. The device remains in reset mode when V

is below a specified

DD

threshold, V

or V

, without the need for an external reset circuit.

POR/PDR

BOR

DS8626 Rev 11

25/205

190

Functional overview

STM32F405xx, STM32F407xx

The device also features an embedded programmable voltage detector (PVD) that monitors

the V

/V

power supply and compares it to the V

threshold. An interrupt can be

DD

DDA

PVD

generated when V

/V

drops below the V

threshold and/or when V

/V

is

DD

DDA

PVD

DD

DDA

higher than the V

threshold. The interrupt service routine can then generate a warning

PVD

message and/or put the MCU into a safe state. The PVD is enabled by software.

Internal reset OFF

This feature is available only on packages featuring the PDR_ON pin. The internal power-on

reset (POR) / power-down reset (PDR) circuitry is disabled with the PDR_ON pin.

An external power supply supervisor should monitor V

and should maintain the device in

DD

reset mode as long as V

is below a specified threshold. PDR_ON should be connected to

DD

this external power supply supervisor. Refer to Figure 7: Power supply supervisor

interconnection with internal reset OFF.

Figure 7. Power supply supervisor interconnection with internal reset OFF

V

DD

power supply supervisor

External V

DD

Ext. reset controller active when

< 1.7 V

V

DD

Application reset

NRST

signal

PDR_ON

V

DD

V

SS

MS31383V4

1.

PDR = 1.7 V for reduce temperature range; PDR = 1.8 V for all temperature range.

The V

specified threshold, below which the device must be maintained under reset, is

DD

1.8 V (see Figure 7). This supply voltage can drop to 1.7 V when the device operates in the

0 to 70 °C temperature range.

A comprehensive set of power-saving mode allows to design low-power applications.

When the internal reset is OFF, the following integrated features are no more supported:

•

The integrated power-on reset (POR) / power-down reset (PDR) circuitry is disabled

•

The brownout reset (BOR) circuitry is disabled

•

The embedded programmable voltage detector (PVD) is disabled

•

V

functionality is no more available and V

pin should be connected to V

BAT

BAT

DD

All packages, except for the LQFP64 and LQFP100, allow to disable the internal reset

through the PDR_ON signal.

26/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

Figure 8. PDR_ON and NRST control with internal reset OFF

V

DD

PDR = 1.7 V

time

Reset by a source different from

power supply supervisor

NRST

PDR_ON

PDR_ON

time

MS19009V8

1.

PDR = 1.7 V for reduce temperature range; PDR = 1.8 V for all temperature range.

3.0.16

Voltage regulator

The regulator has four operating modes:

•

Regulator ON

–

Main regulator mode (MR)

–

Low-power regulator (LPR)

–

Power-down

•

Regulator OFF

Regulator ON

On packages embedding the BYPASS_REG pin, the regulator is enabled by holding

BYPASS_REG low. On all other packages, the regulator is always enabled.

There are three power modes configured by software when regulator is ON:

•

MR is used in the nominal regulation mode (With different voltage scaling in Run)

In Main regulator mode (MR mode), different voltage scaling are provided to reach the

best compromise between maximum frequency and dynamic power consumption.

Refer to Table 14: General operating conditions.

•

LPR is used in the Stop modes

The LP regulator mode is configured by software when entering Stop mode.

•

Power-down is used in Standby mode.

The Power-down mode is activated only when entering in Standby mode. The regulator

output is in high impedance and the kernel circuitry is powered down, inducing zero

consumption. The contents of the registers and SRAM are lost)

DS8626 Rev 11

27/205

190

Functional overview

STM32F405xx, STM32F407xx

Two external ceramic capacitors should be connected on V

& V

pin. Refer to

CAP_1

CAP_2

Figure 21: Power supply scheme and Figure 16: VCAP_1/VCAP_2 operating conditions.

All packages have regulator ON feature.

Regulator OFF

This feature is available only on packages featuring the BYPASS_REG pin. The regulator is

disabled by holding BYPASS_REG high. The regulator OFF mode allows to supply

externally a V

voltage source through V

and V

pins.

12

CAP_1

CAP_2

Since the internal voltage scaling is not manage internally, the external voltage value must

be aligned with the targeted maximum frequency. Refer to Table 14: General operating

conditions.

The two 2.2 µF ceramic capacitors should be replaced by two 100 nF decoupling

capacitors.

Refer to Figure 21: Power supply scheme

When the regulator is OFF, there is no more internal monitoring on V

. An external power

12

supply supervisor should be used to monitor the V

of the logic power domain. PA0 pin

12

should be used for this purpose, and act as power-on reset on V

power domain.

12

In regulator OFF mode the following features are no more supported:

•

PA0 cannot be used as a GPIO pin since it allows to reset a part of the V

logic power

12

domain which is not reset by the NRST pin.

•

As long as PA0 is kept low, the debug mode cannot be used under power-on reset. As

a consequence, PA0 and NRST pins must be managed separately if the debug

connection under reset or pre-reset is required.

•

The standby mode is not available

Figure 9. Regulator OFF

V

12

power

External V

CAP_1/2

supply supervisor

Ext. reset controller active

Application reset

< Min V

when V

12

CAP_1/2

signal (optional)

PA0

NRST

BYPASS_REG

V

12

(note 1)

(1 x 100 nF)

V

CAP1

V

2 x 100 nF

CAP2

V

DD

V

DD

1/2/...N

N ×

100 nF

+ 1 × 4.7 μF

V

SS

1/2/...N

ai18498V5

28/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

The following conditions must be respected:

•

V

should always be higher than V

and V

to avoid current injection

DD

CAP_1

CAP_2

between power domains.

•

If the time for V

and V

to reach V

minimum value is faster than the time for

CAP_1

CAP_2

12

V

to reach 1.8 V, then PA0 should be kept low to cover both conditions: until V

DD

CAP_1

and V

reach V

minimum value and until V

reaches 1.8 V (see Figure 10).

CAP_2

12

DD

•

Otherwise, if the time for V

and V

to reach V

minimum value

is slower

CAP_1

CAP_2

12

than the time for V

to reach 1.8 V, then PA0 could be asserted low externally (see

DD

Figure 11).

•

If V

and V

go below V

minimum value and V

is higher than 1.8 V, then

CAP_1

CAP_2

12

DD

a reset must be asserted on PA0 pin.

Note:

The minimum value of V

depends on the maximum frequency targeted in the application

12

(see Table 14: General operating conditions).

Figure 10. Startup in regulator OFF mode: slow V

slope

DD

- power-down reset risen after V

/V

stabilization

CAP_1

CAP_2

V

DD

(2)

PDR = 1.7 V or 1.8 V

/V

V

CAP_2

CAP_1

V

12

Min V

12

time

NRST

time

ai18491e

1.

This figure is valid both whatever the internal reset mode (ON or OFF).

2.

PDR = 1.7 V for reduced temperature range; PDR = 1.8 V for all temperature ranges.

DS8626 Rev 11

29/205

190

Functional overview

STM32F405xx, STM32F407xx

Figure 11. Startup in regulator OFF mode: fast V

slope

DD

- power-down reset risen before V

/V

stabilization

CAP_1

CAP_2

V

DD

(2)

PDR = 1.7 V or 1.8 V

/V

V

CAP_2

CAP_1

V

12

Min V

12

time

NRST

PA0 asserted externally

time

ai18492d

1.

This figure is valid both whatever the internal reset mode (ON or OFF).

2.

PDR = 1.7 V for a reduced temperature range; PDR = 1.8 V for all temperature ranges.

3.0.17

Regulator ON/OFF and internal reset ON/OFF availability

Table 3. Regulator ON/OFF and internal reset ON/OFF availability

Internal reset

Regulator ON

Regulator OFF

Internal reset ON

OFF

LQFP64

Yes

No

LQFP100

Yes

No

LQFP144

Yes

Yes

PDR_ON

WLCSP90

Yes

Yes

PDR_ON set to

connected to an

UFBGA176

BYPASS_REG set

BYPASS_REG set

V

external power

DD

to V

to V

LQFP176

supply supervisor

SS

DD

3.0.18

Real-time clock (RTC), backup SRAM and backup registers

The backup domain of the STM32F405xx and STM32F407xx includes:

•

The real-time clock (RTC)

•

4 Kbytes of backup SRAM

•

20 backup registers

The real-time clock (RTC) is an independent BCD timer/counter. Dedicated registers contain

the second, minute, hour (in 12/24 hour), week day, date, month, year, in BCD (binary-

coded decimal) format. Correction for 28, 29 (leap year), 30, and 31 day of the month are

performed automatically. The RTC provides a programmable alarm and programmable

periodic interrupts with wakeup from Stop and Standby modes. The sub-seconds value is

also available in binary format.

It is clocked by a 32.768 kHz external crystal, resonator or oscillator, the internal low-power

RC oscillator or the high-speed external clock divided by 2 to 31. The internal low-speed RC

30/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

has a typical frequency of 32 kHz. The RTC can be calibrated using an external 512 Hz

output to compensate for any natural quartz deviation.

Two alarm registers are used to generate an alarm at a specific time and calendar fields can

be independently masked for alarm comparison. To generate a periodic interrupt, a 16-bit

programmable binary auto-reload downcounter with programmable resolution is available

and allows automatic wakeup and periodic alarms from every 120 µs to every 36 hours.

A 20-bit prescaler is used for the time base clock. It is by default configured to generate a

time base of 1 second from a clock at 32.768 kHz.

The 4-Kbyte backup SRAM is an EEPROM-like memory area. It can be used to store data

which need to be retained in V

and standby mode. This memory area is disabled by

BAT

default to minimize power consumption (see Section 3.0.19: Low-power modes). It can be

enabled by software.

The backup registers are 32-bit registers used to store 80 bytes of user application data

when V

power is not present. Backup registers are not reset by a system, a power reset,

DD

or when the device wakes up from the Standby mode (see Section 3.0.19: Low-power

modes).

Additional 32-bit registers contain the programmable alarm subseconds, seconds, minutes,

hours, day, and date.

Like backup SRAM, the RTC and backup registers are supplied through a switch that is

powered either from the V

supply when present or from the V

pin.

DD

BAT

3.0.19

Low-power modes

The STM32F405xx and STM32F407xx support three low-power modes to achieve the best

compromise between low-power consumption, short startup time and available wakeup

sources:

•

Sleep mode

In Sleep mode, only the CPU is stopped. All peripherals continue to operate and can

wake up the CPU when an interrupt/event occurs.

•

Stop mode

The Stop mode achieves the lowest power consumption while retaining the contents of

SRAM and registers. All clocks in the V

domain are stopped, the PLL, the HSI RC

12

and the HSE crystal oscillators are disabled. The voltage regulator can also be put

either in normal or in low-power mode.

The device can be woken up from the Stop mode by any of the EXTI line (the EXTI line

source can be one of the 16 external lines, the PVD output, the RTC alarm / wakeup /

tamper / time stamp events, the USB OTG FS/HS wakeup or the Ethernet wakeup).

•

Standby mode

The Standby mode is used to achieve the lowest power consumption. The internal

voltage regulator is switched off so that the entire V

domain is powered off. The PLL,

12

the HSI RC and the HSE crystal oscillators are also switched off. After entering

DS8626 Rev 11

31/205

190

Functional overview

STM32F405xx, STM32F407xx

Standby mode, the SRAM and register contents are lost except for registers in the

backup domain and the backup SRAM when selected.

The device exits the Standby mode when an external reset (NRST pin), an IWDG reset,

a rising edge on the WKUP pin, or an RTC alarm / wakeup / tamper /time stamp event

occurs.

The standby mode is not supported when the embedded voltage regulator is bypassed

and the V

domain is controlled by an external power.

12

3.0.20

V

operation

BAT

The V

pin allows to power the device V

domain from an external battery, an external

BAT

BAT

supercapacitor, or from V

when no external battery and an external supercapacitor are

DD

present.

V

operation is activated when V

is not present.

BAT

DD

The V

pin supplies the RTC, the backup registers and the backup SRAM.

BAT

Note:

When the microcontroller is supplied from V

, external interrupts and RTC alarm/events

BAT

do not exit it from V

operation.

BAT

When PDR_ON pin is not connected to V

(internal reset OFF), the V

functionality is no

DD

BAT

more available and V

pin should be connected to V

.

BAT

DD

3.0.21

Timers and watchdogs

The STM32F405xx and STM32F407xx devices include two advanced-control timers, eight

general-purpose timers, two basic timers and two watchdog timers.

All timer counters can be frozen in debug mode.

Table 4 compares the features of the advanced-control, general-purpose and basic timers.

Table 4. Timer feature comparison

Max

Max

DMA

Capture/

Timer

Counter

Counter

Prescaler

Complemen-

interface

timer

Timer

request

compare

type

resolution

type

factor

tary output

clock

clock

generation

channels

(MHz)

(MHz)

Up,

Any integer

Advanced

TIM1,

16-bit

Down,

between 1

Yes

4

Yes

84

168

-control

TIM8

Up/down

and 65536

32/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

Table 4. Timer feature comparison (continued)

Max

Max

DMA

Capture/

Timer

Counter

Counter

Prescaler

Complemen-

interface

timer

Timer

request

compare

type

resolution

type

factor

tary output

clock

clock

generation

channels

(MHz)

(MHz)

Up,

Any integer

TIM2,

32-bit

Down,

between 1

Yes

4

No

42

84

TIM5

Up/down

and 65536

Up,

Any integer

TIM3,

16-bit

Down,

between 1

Yes

4

No

42

84

TIM4

Up/down

and 65536

Any integer

TIM9

16-bit

Up

between 1

No

2

No

84

168

and 65536

General

purpose

TIM10

Any integer

,

16-bit

Up

between 1

No

1

No

84

168

TIM11

and 65536

Any integer

TIM12

16-bit

Up

between 1

No

2

No

42

84

and 65536

TIM13

Any integer

,

16-bit

Up

between 1

No

1

No

42

84

TIM14

and 65536

Any integer

TIM6,

Basic

16-bit

Up

between 1

Yes

0

No

42

84

TIM7

and 65536

Advanced-control timers (TIM1, TIM8)

The advanced-control timers (TIM1, TIM8) can be seen as three-phase PWM generators

multiplexed on 6 channels. They have complementary PWM outputs with programmable

inserted dead times. They can also be considered as complete general-purpose timers.

Their 4 independent channels can be used for:

•

Input capture

•

Output compare

•

PWM generation (edge- or center-aligned modes)

•

One-pulse mode output

If configured as standard 16-bit timers, they have the same features as the general-purpose

TIMx timers. If configured as 16-bit PWM generators, they have full modulation capability (0-

100%).

The advanced-control timer can work together with the TIMx timers via the Timer Link

feature for synchronization or event chaining.

TIM1 and TIM8 support independent DMA request generation.

DS8626 Rev 11

33/205

190

Functional overview

STM32F405xx, STM32F407xx

General-purpose timers (TIMx)

There are ten synchronizable general-purpose timers embedded in the STM32F40xxx

devices (see Table 4 for differences).

•

TIM2, TIM3, TIM4, TIM5

The STM32F40xxx include 4 full-featured general-purpose timers: TIM2, TIM5, TIM3,

and TIM4.The TIM2 and TIM5 timers are based on a 32-bit auto-reload

up/downcounter and a 16-bit prescaler. The TIM3 and TIM4 timers are based on a 16-

bit auto-reload up/downcounter and a 16-bit prescaler. They all feature 4 independent

channels for input capture/output compare, PWM or one-pulse mode output. This gives

up to 16 input capture/output compare/PWMs on the largest packages.

The TIM2, TIM3, TIM4, TIM5 general-purpose timers can work together, or with the

other general-purpose timers and the advanced-control timers TIM1 and TIM8 via the

Timer Link feature for synchronization or event chaining.

Any of these general-purpose timers can be used to generate PWM outputs.

TIM2, TIM3, TIM4, TIM5 all have independent DMA request generation. They are

capable of handling quadrature (incremental) encoder signals and the digital outputs

from 1 to 4 hall-effect sensors.

•

TIM9, TIM10, TIM11, TIM12, TIM13, and TIM14

These timers are based on a 16-bit auto-reload upcounter and a 16-bit prescaler.

TIM10, TIM11, TIM13, and TIM14 feature one independent channel, whereas TIM9

and TIM12 have two independent channels for input capture/output compare, PWM or

one-pulse mode output. They can be synchronized with the TIM2, TIM3, TIM4, TIM5

full-featured general-purpose timers. They can also be used as simple time bases.

Basic timers TIM6 and TIM7

These timers are mainly used for DAC trigger and waveform generation. They can also be

used as a generic 16-bit time base.

TIM6 and TIM7 support independent DMA request generation.

Independent watchdog

The independent watchdog is based on a 12-bit downcounter and 8-bit prescaler. It is

clocked from an independent 32 kHz internal RC and as it operates independently from the

main clock, it can operate in Stop and Standby modes. It can be used either as a watchdog

to reset the device when a problem occurs, or as a free-running timer for application timeout

management. It is hardware- or software-configurable through the option bytes.

Window watchdog

The window watchdog is based on a 7-bit downcounter that can be set as free-running. It

can be used as a watchdog to reset the device when a problem occurs. It is clocked from

the main clock. It has an early warning interrupt capability and the counter can be frozen in

debug mode.

34/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

SysTick timer

This timer is dedicated to real-time operating systems, but could also be used as a standard

downcounter. It features:

•

A 24-bit downcounter

•

Autoreload capability

•

Maskable system interrupt generation when the counter reaches 0

•

Programmable clock source.

3.0.22

Inter-integrated circuit interface (I²C)

Up to three I²C bus interfaces can operate in multimaster and slave modes. They can

support the Standard-mode (up to 100 kHz) and Fast-mode (up to 400 kHz). They support

the 7/10-bit addressing mode and the 7-bit dual addressing mode (as slave). A hardware

CRC generation/verification is embedded.

They can be served by DMA and they support SMBus 2.0/PMBus.

3.0.23

Universal synchronous/asynchronous receiver transmitters (USART)

The STM32F405xx and STM32F407xx embed four universal synchronous/asynchronous

receiver transmitters (USART1, USART2, USART3 and USART6) and two universal

asynchronous receiver transmitters (UART4 and UART5).

These six interfaces provide asynchronous communication, IrDA SIR ENDEC support,

multiprocessor communication mode, single-wire half-duplex communication mode and

have LIN Master/Slave capability. The USART1 and USART6 interfaces are able to

communicate at speeds of up to 10.5 Mbit/s. The other available interfaces communicate at

up to 5.25 Mbit/s.

USART1, USART2, USART3 and USART6 also provide hardware management of the CTS

and RTS signals, Smart Card mode (ISO 7816 compliant) and SPI-like communication

capability. All interfaces can be served by the DMA controller.

DS8626 Rev 11

35/205

190

Functional overview

STM32F405xx, STM32F407xx

Table 5. USART feature comparison

Max. baud rate

Max. baud rate

Modem

USART

Standard

SPI

Smartcard

in Mbit/s

in Mbit/s

APB

(RTS/

LIN

irDA

name

features

master

(ISO 7816)

(oversampling

(oversampling

mapping

CTS)

by 16)

by 8)

APB2

USART1

X

X

X

X

X

X

5.25

10.5

(max.

84 MHz)

APB1

USART2

X

X

X

X

X

X

2.62

5.25

(max.

42 MHz)

APB1

USART3

X

X

X

X

X

X

2.62

5.25

(max.

42 MHz)

APB1

UART4

X

-

X

-

X

-

2.62

5.25

(max.

42 MHz)

APB1

UART5

X

-

X

-

X

-

2.62

5.25

(max.

42 MHz)

APB2

USART6

X

X

X

X

X

X

5.25

10.5

(max.

84 MHz)

3.0.24

Serial peripheral interface (SPI)

The STM32F40xxx feature up to three SPIs in slave and master modes in full-duplex and

simplex communication modes. SPI1 can communicate at up to 42 Mbits/s, SPI2 and SPI3

can communicate at up to 21 Mbit/s. The 3-bit prescaler gives 8 master mode frequencies

and the frame is configurable to 8 bits or 16 bits. The hardware CRC generation/verification

supports basic SD Card/MMC modes. All SPIs can be served by the DMA controller.

The SPI interface can be configured to operate in TI mode for communications in master

mode and slave mode.

2

3.0.25

Inter-integrated sound (I

S)

2

Two standard I

S interfaces (multiplexed with SPI2 and SPI3) are available. They can be

operated in master or slave mode, in full duplex and half-duplex communication modes, and

can be configured to operate with a 16-/32-bit resolution as an input or output channel.

Audio sampling frequencies from 8 kHz up to 192 kHz are supported. When either or both of

2

the I

S interfaces is/are configured in master mode, the master clock can be output to the

external DAC/CODEC at 256 times the sampling frequency.

2

All I

Sx can be served by the DMA controller.

36/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

3.0.26

Audio PLL (PLLI2S)

2

The devices feature an additional dedicated PLL for audio I

S application. It allows to

2

achieve error-free I

S sampling clock accuracy without compromising on the CPU

performance, while using USB peripherals.

2

The PLLI2S configuration can be modified to manage an I

S sample rate change without

disabling the main PLL (PLL) used for CPU, USB and Ethernet interfaces.

The audio PLL can be programmed with very low error to obtain sampling rates ranging

from 8 KHz to 192 KHz.

2

In addition to the audio PLL, a master clock input pin can be used to synchronize the I

S

flow with an external PLL (or Codec output).

3.0.27

Secure digital input/output interface (SDIO)

An SD/SDIO/MMC host interface is available, that supports MultiMediaCard System

Specification Version 4.2 in three different databus modes: 1-bit (default), 4-bit and 8-bit.

The interface allows data transfer at up to 48 MHz, and is compliant with the SD Memory

Card Specification Version 2.0.

The SDIO Card Specification Version 2.0 is also supported with two different databus

modes: 1-bit (default) and 4-bit.

The current version supports only one SD/SDIO/MMC4.2 card at any one time and a stack

of MMC4.1 or previous.

In addition to SD/SDIO/MMC, this interface is fully compliant with the CE-ATA digital

protocol Rev1.1.

3.0.28

Ethernet MAC interface with dedicated DMA and IEEE 1588 support

Peripheral available only on the STM32F407xx devices.

The STM32F407xx devices provide an IEEE-802.3-2002-compliant media access controller

(MAC) for ethernet LAN communications through an industry-standard medium-

independent interface (MII) or a reduced medium-independent interface (RMII). The

STM32F407xx requires an external physical interface device (PHY) to connect to the

physical LAN bus (twisted-pair, fiber, etc.). the PHY is connected to the STM32F407xx MII

port using 17 signals for MII or 9 signals for RMII, and can be clocked using the 25 MHz

(MII) from the STM32F407xx.

DS8626 Rev 11

37/205

190

Functional overview

STM32F405xx, STM32F407xx

The STM32F407xx includes the following features:

•

Supports 10 and 100 Mbit/s rates

•

Dedicated DMA controller allowing high-speed transfers between the dedicated SRAM

and the descriptors (see the STM32F40xxx/41xxx reference manual for details)

•

Tagged MAC frame support (VLAN support)

•

Half-duplex (CSMA/CD) and full-duplex operation

•

MAC control sublayer (control frames) support

•

32-bit CRC generation and removal

•

Several address filtering modes for physical and multicast address (multicast and

group addresses)

•

32-bit status code for each transmitted or received frame

•

Internal FIFOs to buffer transmit and receive frames. The transmit FIFO and the

receive FIFO are both 2 Kbytes.

•

Supports hardware PTP (precision time protocol) in accordance with IEEE 1588 2008

(PTP V2) with the time stamp comparator connected to the TIM2 input

•

Triggers interrupt when system time becomes greater than target time

3.0.29

Controller area network (bxCAN)

The two CANs are compliant with the 2.0A and B (active) specifications with a bitrate up to 1

Mbit/s. They can receive and transmit standard frames with 11-bit identifiers as well as

extended frames with 29-bit identifiers. Each CAN has three transmit mailboxes, two receive

FIFOS with 3 stages and 28 shared scalable filter banks (all of them can be used even if one

CAN is used). 256 bytes of SRAM are allocated for each CAN.

3.0.30

Universal serial bus on-the-go full-speed (OTG_FS)

The STM32F405xx and STM32F407xx embed an USB OTG full-speed device/host/OTG

peripheral with integrated transceivers. The USB OTG FS peripheral is compliant with the

USB 2.0 specification and with the OTG 1.0 specification. It has software-configurable

endpoint setting and supports suspend/resume. The USB OTG full-speed controller

requires a dedicated 48 MHz clock that is generated by a PLL connected to the HSE

oscillator. The major features are:

•

Combined Rx and Tx FIFO size of 320 × 35 bits with dynamic FIFO sizing

•

Supports the session request protocol (SRP) and host negotiation protocol (HNP)

•

4 bidirectional endpoints

•

8 host channels with periodic OUT support

•

HNP/SNP/IP inside (no need for any external resistor)

•

For OTG/Host modes, a power switch is needed in case bus-powered devices are

connected

38/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

3.0.31

Universal serial bus on-the-go high-speed (OTG_HS)

The STM32F405xx and STM32F407xx devices embed a USB OTG high-speed (up to

480 Mb/s) device/host/OTG peripheral. The USB OTG HS supports both full-speed and

high-speed operations. It integrates the transceivers for full-speed operation (12 MB/s) and

features a UTMI low-pin interface (ULPI) for high-speed operation (480 MB/s). When using

the USB OTG HS in HS mode, an external PHY device connected to the ULPI is required.

The USB OTG HS peripheral is compliant with the USB 2.0 specification and with the OTG

1.0 specification. It has software-configurable endpoint setting and supports

suspend/resume. The USB OTG full-speed controller requires a dedicated 48 MHz clock

that is generated by a PLL connected to the HSE oscillator.

The major features are:

•

Combined Rx and Tx FIFO size of 1 Kbit × 35 with dynamic FIFO sizing

•

Supports the session request protocol (SRP) and host negotiation protocol (HNP)

•

6 bidirectional endpoints

•

12 host channels with periodic OUT support

•

Internal FS OTG PHY support

•

External HS or HS OTG operation supporting ULPI in SDR mode. The OTG PHY is

connected to the microcontroller ULPI port through 12 signals. It can be clocked using

the 60 MHz output.

•

Internal USB DMA

•

HNP/SNP/IP inside (no need for any external resistor)

•

for OTG/Host modes, a power switch is needed in case bus-powered devices are

connected

3.0.32

Digital camera interface (DCMI)

The camera interface is not available in STM32F405xx devices.

STM32F407xx products embed a camera interface that can connect with camera modules

and CMOS sensors through an 8-bit to 14-bit parallel interface, to receive video data. The

camera interface can sustain a data transfer rate up to 54 Mbyte/s at 54 MHz. It features:

•

Programmable polarity for the input pixel clock and synchronization signals

•

Parallel data communication can be 8-, 10-, 12- or 14-bit

•

Supports 8-bit progressive video monochrome or raw bayer format, YCbCr 4:2:2

progressive video, RGB 565 progressive video or compressed data (like JPEG)

•

Supports continuous mode or snapshot (a single frame) mode

•

Capability to automatically crop the image

3.0.33

True random number generator (RNG)

All STM32F405xx and STM32F407xx products embed a true random number generator

(RNG) that provides full entropy outputs to the application as 32-bit samples. It is composed

of a live entropy source (analog) and an internal conditioning component.

3.0.34

General-purpose input/outputs (GPIOs)

Each of the GPIO pins can be configured by software as output (push-pull or open-drain,

with or without pull-up or pull-down), as input (floating, with or without pull-up or pull-down)

DS8626 Rev 11

39/205

190

Functional overview

STM32F405xx, STM32F407xx

or as peripheral alternate function. Most of the GPIO pins are shared with digital or analog

alternate functions. All GPIOs are high-current-capable and have speed selection to better

manage internal noise, power consumption and electromagnetic emission.

The I/O configuration can be locked if needed by following a specific sequence in order to

avoid spurious writing to the I/Os registers.

Fast I/O handling allowing maximum I/O toggling up to 84 MHz.

3.0.35

Analog-to-digital converters (ADCs)

Three 12-bit analog-to-digital converters are embedded and each ADC shares up to 16

external channels, performing conversions in the single-shot or scan mode. In scan mode,

automatic conversion is performed on a selected group of analog inputs.

Additional logic functions embedded in the ADC interface allow:

•

Simultaneous sample and hold

•

Interleaved sample and hold

The ADC can be served by the DMA controller. An analog watchdog feature allows very

precise monitoring of the converted voltage of one, some or all selected channels. An

interrupt is generated when the converted voltage is outside the programmed thresholds.

To synchronize A/D conversion and timers, the ADCs could be triggered by any of TIM1,

TIM2, TIM3, TIM4, TIM5, or TIM8 timer.

3.0.36

Temperature sensor

The temperature sensor has to generate a voltage that varies linearly with temperature. The

conversion range is between 1.8 V and 3.6 V. The temperature sensor is internally

connected to the ADC1_IN16 input channel which is used to convert the sensor output

voltage into a digital value.

As the offset of the temperature sensor varies from chip to chip due to process variation, the

internal temperature sensor is mainly suitable for applications that detect temperature

changes instead of absolute temperatures. If an accurate temperature reading is needed,

then an external temperature sensor part should be used.

3.0.37

Digital-to-analog converter (DAC)

The two 12-bit buffered DAC channels can be used to convert two digital signals into two

analog voltage signal outputs.

40/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Functional overview

This dual digital Interface supports the following features:

•

two DAC converters: one for each output channel

•

8-bit or 12-bit monotonic output

•

left or right data alignment in 12-bit mode

•

synchronized update capability

•

noise-wave generation

•

triangular-wave generation

•

dual DAC channel independent or simultaneous conversions

•

DMA capability for each channel

•

external triggers for conversion

•

input voltage reference V

REF+

Eight DAC trigger inputs are used in the device. The DAC channels are triggered through

the timer update outputs that are also connected to different DMA streams.

3.0.38

Serial wire JTAG debug port (SWJ-DP)

The Arm SWJ-DP interface is embedded, and is a combined JTAG and serial wire debug

port that enables either a serial wire debug or a JTAG probe to be connected to the target.

Debug is performed using 2 pins only instead of 5 required by the JTAG (JTAG pins could

be re-use as GPIO with alternate function): the JTAG TMS and TCK pins are shared with

SWDIO and SWCLK, respectively, and a specific sequence on the TMS pin is used to

switch between JTAG-DP and SW-DP.

3.0.39

Embedded Trace Macrocell™

The Arm Embedded Trace Macrocell provides a greater visibility of the instruction and data

flow inside the CPU core by streaming compressed data at a very high rate from the

STM32F40xxx through a small number of ETM pins to an external hardware trace port

analyser (TPA) device. The TPA is connected to a host computer using USB, Ethernet, or

any other high-speed channel. Real-time instruction and data flow activity can be recorded

and then formatted for display on the host computer that runs the debugger software. TPA

hardware is commercially available from common development tool vendors.

The Embedded Trace Macrocell operates with third party debugger software tools.

DS8626 Rev 11

41/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

4

Pinouts and pin description

Figure 12. STM32F40xxx LQFP64 pinout

PB7

PB6

PB5

PB4

PB3

PD2

PC12

PC11

PC10

PA15

PA14

VDD

PB9

PB8

BOOT0

VSS

64 63 62 61 60 59 58 57 56 55 54 53 52 51 50 49

VDD

VBAT

48

1

VCAP_2

PC13

47

2

PA13

PC14

46

3

PA12

PC15

45

4

PA11

PH0

44

5

PA10

PH1

43

6

PA9

NRST

42

7

PA8

PC0

41

8

LQFP64

PC9

PC1

40

9

PC8

PC2

39

10

PC7

PC3

38

11

PC6

VSSA

37

PB15

VDDA

36

PB14

PA0_WKUP

35

PB13

PA1

34

PB12

PA2

12

13

14

15

33

16

17 18 19 20 21 22 23 24

25 26 27 28

29 30 31 32

PA3

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PC4

PC5

VSS

VDD

VDD

PB10

PB11

VCAP_1

ai18493b

1.

The above figure shows the package top view.

42/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

Figure 13. STM32F40xxx LQFP100 pinout

PB7

PB6

PB5

PB4

PB3

PD7

PD6

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

VDD

VSS

PE1

PE0

PB9

PB8

BOOT0

PE2

100

99

98

97

96

95

94

93

92

91

90

89

88

87

86

85

84

83

82

81

80

79

78

77

76

1

75

VDD

PE3

2

74

VSS

PE4

3

73

VCAP_2

PE5

4

72

PA13

PE6

5

71

PA12

VBAT

6

70

PA 11

7

69

PC13

PA10

PC14

8

68

PA9

PC15

9

67

PA8

VSS

10

66

PC9

VDD

11

65

PC8

12

64

PH0

PC7

LQFP100

13

63

PH1

PC6

NRST

14

62

PD15

PC0

15

61

PD14

PC1

16

60

PD13

PC2

17

59

PD12

PC3

18

58

PD11

VDD

19

57

PD10

VSSA

20

56

PD9

VREF+

21

55

PD8

VDDA

22

54

PB15

PA0

23

53

PB14

PA1

24

52

PB13

PA2

25

51

PB12

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50

PA4

PA5

PA6

PA7

PA3

PB0

PB1

PB2

PE7

PE8

PE9

PC4

PC5

VSS

VDD

VDD

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

VCAP_1

ai18495c

1.

The above figure shows the package top view.

DS8626 Rev 11

43/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

Figure 14. STM32F40xxx LQFP144 pinout

DD

DD

SS

DD

SS

V

PDR_ON

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PG15

V

V

PG14

PG13

PG12

PG11

PG10

PG9

PD7

PD6

V

V

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

PE2

144

143

142

141

140

139

138

137

136

135

134

133

132

131

130

129

128

127

126

125

124

123

122

121

109

120

119

118

117

116

115

114

113

112

111

110

108

1

V

DD

PE3

107

2

V

SS

PE4

106

3

V

CAP_2

PE5

105

4

PA13

PE6

104

5

PA12

VBAT

103

6

PA11

PC13

102

7

PA10

PC14

101

8

PA9

PC15

100

9

PA8

PF0

99

10

PC9

PF1

98

11

PC8

PF2

97

12

PC7

PF3

96

13

PC6

PF4

95

14

V

DD

PF5

94

15

V

SS

V

93

16

PG8

SS

V

92

17

PG7

DD

PF6

91

18

PG6

LQFP144

PF7

90

19

PG5

89

PF8

20

PG4

88

PF9

21

PG3

87

PF10

22

PG2

86

23

PH0

PD15

85

24

PH1

PD14

84

25

NRST

V

DD

26

PC0

83

V

SS

27

PC1

82

PD13

28

PC2

81

PD12

29

PC3

80

PD11

V

30

79

PD10

DD

V

31

78

PD9

SSA

32

V

77

PD8

REF+

33

V

76

PB15

DDA

34

PA0

75

PB14

35

PA1

74

PB13

36

PA2

73

PB12

37

38

39

40

41

42

43

44

45

46

47

48

49

50

51

52

53

54

55

56

57

58

59

60

72

61

62

63

64

65

66

67

68

69

70

71

SS

DD

SS

SS

DD

DD

DD

V

V

V

V

V

V

V

PA3

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PE7

PE8

PE9

PC4

PC5

PG0

PG1

PF11

PF12

PF13

PF14

PF15

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

CAP_1

ai18496b

V

1.

The above figure shows the package top view.

44/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

Figure 15. STM32F40xxx LQFP176 pinout

DD

DD

SS

DD

SS

DD

SS

PI7

PI6

PI5

PI4

V

PDR_ON

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PG15

V

V

PG14

PG13

PG12

PG11

PG10

PG9

PD7

PD6

V

V

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

V

V

PI3

PI2

140

152

151

150

149

148

147

146

145

144

143

142

139

138

137

136

135

134

133

PE2

PI1

PE3

PE4

PE5

PE6

V

BAT

PI8

PC14

176

175

174

173

172

171

170

169

168

167

166

165

164

163

162

161

160

159

158

157

156

155

154

153

141

1

132

2

PI0

131

3

PH15

130

4

PH14

129

5

128

PH13

6

V

127

DD

7

V

126

SS

8

PC13

125

V

CAP_2

9

124

PA13

PC15

10

PA12

123

11

PI9

122

PA11

12

PI10

121

PA10

13

PI11

120

PA9

14

V

119

PA8

SS

V

15

118

PC9

DD

PF0

16

117

PC8

PF1

17

116

PC7

PF2

18

115

PC6

PF3

19

114

V

DD

PF4

20

113

V

SS

PF5

21

112

PG8

LQFP176

V

22

111

PG7

SS

V

23

110

PG6

DD

PF6

24

109

PG5

PF7

25

108

PG4

PF8

26

PG3

107

PF9

27

PG2

106

PF10

28

PD15

105

PH0

29

PD14

104

PH1

30

V

103

V

DD

NRST

31

V

102

V

SS

PC0

32

PD13

101

PC1

33

PD12

100

PC2

34

PD11

99

PC3

35

PD10

98

V

36

PD9

DD

97

V

37

PD8

SSA

96

V

38

PB15

REF+

95

V

39

PB14

DDA

94

PA0

40

PB13

93

PA1

41

PB12

92

PA2

42

V

91

DD

PH2

43

V

90

SS

PH3

44

PH12

89

69

70

71

72

73

74

75

76

77

78

79

45

46

47

48

49

50

51

52

53

54

55

56

57

58

59

60

61

62

63

64

65

66

67

68

80

81

82

83

84

85

86

87

88

SS

SS

DD

DD

DD

DD

V

V

V

V

V

V

PA3

PA4

PA5

PA6

PA7

PB0

PB1

PB2

PE7

PE8

PE9

PH4

PH5

PH6

PH7

PH8

PH9

PC4

PC5

PG0

PG1

CAP_1

PF11

PF12

PF13

PF14

PF15

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

PH10

PH11

V

MS19916V4

BYPASS_REG

1.

The above figure shows the package top view.

DS8626 Rev 11

45/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

Figure 16. STM32F40xxx UFBGA176 ballout

1

2

3

9

10

11

12

13

14

15

3

4

5

6

7

8

A

PE3

PE2

PE1

PE0

PB8

PB5

PG14

PG13

PB4

PB3

PD7

PC12

PA15

PA14

PA13

B

PE4

PE5

PE6

PB9

PB7

PB6

PG15

PG12

PG11

PG10

PD6

PD0

PC11

PC10

PA12

VDD

C

VBAT

PI7

PI6

PI5

VDD

VDD

VDD

PG9

PD5

PD1

PI3

PI2

PA11

PDR_ON

PC13

PI8

VSS

D

PI9

PI4

BOOT0

VSS

VSS

VSS

PD4

PD3

PD2

PH15

PI1

PA10

PC14

E

PF0

PI10

PI11

PH13

PH14

PI0

PA9

F

PC15

VSS

VDD

PH2

VSS

VSS

VSS

VSS

VSS

VSS

VCAP_2

PC9

PA8

G

VSS

VDD

PH3

VSS

VSS

VSS

VSS

VSS

VSS

VDD

PC8

PC7

PH0

PH1

H

PF2

PF1

PH4

VSS

VSS

VSS

VSS

VSS

VSS

VDD

PG8

PC6

J

NRST

PF3

PF4

PH5

VSS

VSS

VSS

VSS

VSS

VDD

VDD

PG7

PG6

6

PF5

VSS

VSS

VSS

VSS

VSS

PH12

PG5

PG4

PG3

K

PF7

PF

VDD

BYPASS_

L

PF10

PF9

PF8

PH11

PH10

PD15

PG2

REG

M

VSSA

PC0

PC1

PC2

PC3

PB2

PG1

VSS

VSS

VCAP_1

PH6

PH8

PH9

PD14

PD13

PA0

N

VREF-

PA1

PA4

PC4

PF13

PG0

VDD

VDD

VDD

PE13

PH7

PD12

PD11

PD10

P

VREF+

PA2

PA6

PA5

PC5

PF12

PF15

PE8

PE9

PE11

PE14

PB12

PB13

PD9

PD8

R

VDDA

PA3

PA7

PB1

PB0

PF11

PF14

PE7

PE10

PE12

PE15

PB10

PB11

PB14

PB15

ai18497b

1.

This figure shows the package top view.

46/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

Figure 17. STM32F40xxx WLCSP90 ballout

10

9

8

7

6

5

4

3

2

1

PA14

BOOT0

A

VDD

VBAT

PC13

PDR_ON

PB4

PD7

PD4

PC12

PC14

PI1

PC15

VDD

PB7

PB3

PD6

PD2

PA15

VCAP_2

B

PA0

VSS

PC11

PI0

PB9

PB6

PD5

PD1

PA11

PA12

C

BYPASS_

PA13

D

PC2

PB5

PD0

PC10

PA8

PA10

PA9

PB8

REG

PC3

E

VSS

VDD

VSS

VDD

PC7

PC0

PC9

PC8

VSS

PH1

VDD

PE10

PE14

VCAP_1

PD15

F

PH0

PC6

PD14

PA1

G

VDDA

PA5

PB0

PE7

PE13

PE15

PD10

PD11

PD12

NRST

PA3

PA6

PB1

PB10

PB15

PE8

PE12

PD9

PD8

H

VSSA

PE9

PB14

PE11

PB11

PB12

PA2

PA4

PB13

PB2

J

PA7

MS30402V1

1.

This figure shows the package bump view.

Table 6. Legend/abbreviations used in the pinout table

Name

Abbreviation

Definition

Unless otherwise specified in brackets below the pin name, the pin function during and after

Pin name

reset is the same as the actual pin name

S

Supply pin

Pin type

I

Input only pin

I/O

Input / output pin

FT

5 V tolerant I/O

TTa

3.3 V tolerant I/O directly connected to ADC

I/O structure

B

Dedicated BOOT0 pin

RST

Bidirectional reset pin with embedded weak pull-up resistor

Notes

Unless otherwise specified by a note, all I/Os are set as floating inputs during and after reset

Alternate

Functions selected through GPIOx_AFR registers

functions

Additional

Functions directly selected/enabled through peripheral registers

functions

DS8626 Rev 11

47/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

TRACECLK/ FSMC_A23 /

-

-

1

1

A2

1

PE2

I/O

FT

-

ETH_MII_TXD3 /

-

EVENTOUT

TRACED0/FSMC_A19 /

-

-

2

2

A1

2

PE3

I/O

FT

-

-

EVENTOUT

TRACED1/FSMC_A20 /

-

-

3

3

B1

3

PE4

I/O

FT

-

-

DCMI_D4/ EVENTOUT

TRACED2 / FSMC_A21 /

-

-

4

4

B2

4

PE5

I/O

FT

-

TIM9_CH1 / DCMI_D6 /

-

EVENTOUT

TRACED3 / FSMC_A22 /

-

-

5

5

B3

5

PE6

I/O

FT

-

TIM9_CH2 / DCMI_D7 /

-

EVENTOUT

1

A10

6

6

C1

6

V

S

-

-

-

-

BAT

RTC_TAMP1,

(3)(

-

RTC_TAMP2,

-

-

-

-

D2

7

PI8

I/O

FT

4)

RTC_TS

RTC_OUT,

(3)

-

RTC_TAMP1,

2

A9

7

7

D1

8

PC13

I/O

FT

(4)

RTC_TS

(3)(

PC14/OSC32_IN

(5)

-

OSC32_IN

3

B10

8

8

E1

9

I/O

FT

4)

(PC14)

PC15/

(3)(

(5)

OSC32_OUT

4

B9

9

9

F1

10

-

OSC32_OUT

I/O

FT

4)

(PC15)

-

-

-

-

D3

11

PI9

I/O

FT

-

CAN1_RX  / EVENTOUT

-

ETH_MII_RX_ER /

-

-

-

-

E3

12

PI10

I/O

FT

-

-

EVENTOUT

OTG_HS_ULPI_DIR /

-

-

-

-

E4

13

PI11

I/O

FT

-

-

EVENTOUT

S

-

-

-

-

-

-

-

-

F2

14

V

SS

S

-

-

-

-

-

-

-

-

F3

15

V

DD

FSMC_A0 / I2C2_SDA /

-

-

-

10

E2

16

PF0

I/O

FT

-

-

EVENTOUT

48/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

FSMC_A1 / I2C2_SCL /

-

-

-

11

H3

17

PF1

I/O

FT

-

-

EVENTOUT

FSMC_A2 / I2C2_SMBA /

-

-

-

12

H2

18

PF2

I/O

FT

-

-

EVENTOUT

(5)

FSMC_A3/EVENTOUT

ADC3_IN9

-

-

-

13

J2

19

PF3

I/O

FT

(5)

-

-

-

14

J3

20

PF4

I/O

FT

FSMC_A4/EVENTOUT

ADC3_IN14

(5)

FSMC_A5/EVENTOUT

ADC3_IN15

-

-

-

15

K3

21

PF5

I/O

FT

S

-

-

-

-

-

C9

10

16

G2

22

V

SS

-

B8

11

17

G3

23

V

S

-

-

-

-

DD

TIM10_CH1 /

(5)

FSMC_NIORD/

ADC3_IN4

-

-

-

18

K2

24

PF6

I/O

FT

EVENTOUT

TIM11_CH1/FSMC_NREG/

(5)

ADC3_IN5

-

-

-

19

K1

25

PF7

I/O

FT

EVENTOUT

TIM13_CH1 /

(5)

-

-

-

20

L3

26

PF8

I/O

FT

FSMC_NIOWR/

ADC3_IN6

EVENTOUT

TIM14_CH1 / FSMC_CD/

(5)

-

-

-

21

L2

27

PF9

I/O

FT

ADC3_IN7

EVENTOUT

(5)

FSMC_INTR/ EVENTOUT

ADC3_IN8

-

-

-

22

L1

28

PF10

I/O

FT

PH0/OSC_IN

(5)

5

F10

12

23

G1

29

I/O

FT

-

EVENTOUT

OSC_IN

(PH0)

PH1/OSC_OUT

(5)

6

F9

13

24

H1

30

I/O

FT

-

EVENTOUT

OSC_OUT

(PH1)

7

G10

14

25

J1

31

NRST

I/O RST

-

-

-

OTG_HS_ULPI_STP/

(5)

ADC123_IN10

8

E10

15

26

M2

32

PC0

I/O

FT

EVENTOUT

(5)

9

-

16

27

M3

33

PC1

I/O

FT

ETH_MDC/ EVENTOUT

ADC123_IN11

SPI2_MISO /

OTG_HS_ULPI_DIR /

(5)

10

D10

17

28

M4

34

PC2

I/O

FT

ADC123_IN12

ETH_MII_TXD2

/I2S2ext_SD/ EVENTOUT

DS8626 Rev 11

49/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

SPI2_MOSI / I2S2_SD /

OTG_HS_ULPI_NXT /

(5)

11

E9

18

29

M5

35

PC3

I/O

FT

ADC123_IN13

ETH_MII_TX_CLK/

EVENTOUT

S

-

-

-

-

-

-

19

30

-

36

V

DD

12

H10

20

31

M1

37

V

S

-

-

-

-

SSA

S

-

-

-

-

-

-

-

-

N1

-

V

REF

–

S

-

-

-

-

-

-

21

32

P1

38

V

REF+

13

G9

22

33

R1

39

V

S

-

-

-

-

DDA

USART2_CTS/

UART4_TX/

PA0/WKUP

ETH_MII_CRS /

ADC123_IN0/WK

(6)

14

C10

23

34

N3

40

I/O

FT

(5)

TIM2_CH1_ETR/

UP

(PA0)

TIM5_CH1 / TIM8_ETR/

EVENTOUT

USART2_RTS /

UART4_RX/

ETH_RMII_REF_CLK /

(5)

15

F8

24

35

N2

41

PA1

I/O

FT

ADC123_IN1

ETH_MII_RX_CLK /

TIM5_CH2 / TIM2_CH2/

EVENTOUT

USART2_TX/TIM5_CH3 /

(5)

TIM9_CH1 / TIM2_CH3 /

ADC123_IN2

16

J10

25

36

P2

42

PA2

I/O

FT

ETH_MDIO/ EVENTOUT

-

-

-

-

F4

43

PH2

I/O

FT

-

ETH_MII_CRS/EVENTOUT

-

-

-

-

-

G4

44

PH3

I/O

FT

-

ETH_MII_COL/EVENTOUT

-

I2C2_SCL /

-

-

-

-

H4

45

PH4

I/O

FT

-

OTG_HS_ULPI_NXT/

-

EVENTOUT

-

-

-

-

J4

46

PH5

I/O

FT

-

I2C2_SDA/ EVENTOUT

-

USART2_RX/TIM5_CH4 /

TIM9_CH2 / TIM2_CH4 /

(5)

17

H9

26

37

R2

47

PA3

I/O

FT

OTG_HS_ULPI_D0 /

ADC123_IN3

ETH_MII_COL/

EVENTOUT

18

E5

27

38

-

-

V

S

-

-

-

-

SS

50/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

-

D9

-

-

L4

48

BYPASS_REG

I

FT

-

-

-

S

-

-

-

-

19

E4

28

39

K4

49

V

DD

SPI1_NSS / SPI3_NSS /

USART2_CK /

ADC12_IN4

(5)

20

J9

29

40

N4

50

PA4

I/O TTa

DCMI_HSYNC /

/DAC_OUT1

OTG_HS_SOF/ I2S3_WS/

EVENTOUT

SPI1_SCK/

OTG_HS_ULPI_CK /

ADC12_IN5/DAC

(5)

21

G8

30

41

P4

51

PA5

I/O TTa

TIM2_CH1_ETR/

_OUT2

TIM8_CH1N/ EVENTOUT

SPI1_MISO /

TIM8_BKIN/TIM13_CH1 /

(5)

22

H8

31

42

P3

52

PA6

I/O

FT

ADC12_IN6

DCMI_PIXCLK / TIM3_CH1

/ TIM1_BKIN/ EVENTOUT

SPI1_MOSI/ TIM8_CH1N /

TIM14_CH1/TIM3_CH2/

ETH_MII_RX_DV /

(5)

23

J8

32

43

R3

53

PA7

I/O

FT

ADC12_IN7

TIM1_CH1N /

ETH_RMII_CRS_DV/

EVENTOUT

ETH_RMII_RX_D0 /

(5)

ETH_MII_RX_D0/

ADC12_IN14

24

-

33

44

N5

54

PC4

I/O

FT

EVENTOUT

ETH_RMII_RX_D1 /

(5)

25

-

34

45

P5

55

PC5

I/O

FT

ETH_MII_RX_D1/

ADC12_IN15

EVENTOUT

TIM3_CH3 / TIM8_CH2N/

OTG_HS_ULPI_D1/

(5)

26

G7

35

46

R5

56

PB0

I/O

FT

ADC12_IN8

ETH_MII_RXD2 /

TIM1_CH2N/ EVENTOUT

TIM3_CH4 / TIM8_CH3N/

OTG_HS_ULPI_D2/

(5)

27

H7

36

47

R4

57

PB1

I/O

FT

ADC12_IN9

ETH_MII_RXD3 /

TIM1_CH3N/ EVENTOUT

PB2/BOOT1

28

J7

37

48

M6

58

I/O

FT

-

EVENTOUT

-

(PB2)

DS8626 Rev 11

51/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

-

-

-

49

R6

59

PF11

I/O

FT

-

DCMI_D12/ EVENTOUT

-

-

-

-

50

P6

60

PF12

I/O

FT

-

FSMC_A6/ EVENTOUT

-

S

-

-

-

-

-

-

-

51

M8

61

V

SS

-

-

-

52

N8

62

V

S

-

-

-

-

DD

-

-

-

53

N6

63

PF13

I/O

FT

-

FSMC_A7/ EVENTOUT

-

-

-

-

54

R7

64

PF14

I/O

FT

-

FSMC_A8/ EVENTOUT

-

-

-

-

55

P7

65

PF15

I/O

FT

-

FSMC_A9/ EVENTOUT

-

-

-

-

56

N7

66

PG0

I/O

FT

-

FSMC_A10/ EVENTOUT

-

-

-

-

57

M7

67

PG1

I/O

FT

-

FSMC_A11/ EVENTOUT

-

FSMC_D4/TIM1_ETR/

-

G6

38

58

R8

68

PE7

I/O

FT

-

-

EVENTOUT

FSMC_D5/ TIM1_CH1N/

-

H6

39

59

P8

69

PE8

I/O

FT

-

-

EVENTOUT

FSMC_D6/TIM1_CH1/

-

J6

40

60

P9

70

PE9

I/O

FT

-

-

EVENTOUT

S

-

-

-

-

-

-

-

61

M9

71

V

SS

S

-

-

-

-

-

-

-

62

N9

72

V

DD

FSMC_D7/TIM1_CH2N/

-

F6

41

63

R9

73

PE10

I/O

FT

-

-

EVENTOUT

FSMC_D8/TIM1_CH2/

-

J5

42

64

P10

74

PE11

I/O

FT

-

-

EVENTOUT

FSMC_D9/TIM1_CH3N/

-

H5

43

65

R10

75

PE12

I/O

FT

-

-

EVENTOUT

FSMC_D10/TIM1_CH3/

-

G5

44

66

N11

76

PE13

I/O

FT

-

-

EVENTOUT

FSMC_D11/TIM1_CH4/

-

F5

45

67

P11

77

PE14

I/O

FT

-

-

EVENTOUT

FSMC_D12/TIM1_BKIN/

-

G4

46

68

R11

78

PE15

I/O

FT

-

-

EVENTOUT

52/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

SPI2_SCK / I2S2_CK /

I2C2_SCL/ USART3_TX /

29

H4

47

69

R12

79

PB10

I/O

FT

-

OTG_HS_ULPI_D3 /

-

ETH_MII_RX_ER /

TIM2_CH3/ EVENTOUT

I2C2_SDA/USART3_RX/

OTG_HS_ULPI_D4 /

30

J4

48

70

R13

80

PB11

I/O

FT

-

ETH_RMII_TX_EN/

-

ETH_MII_TX_EN /

TIM2_CH4/ EVENTOUT

31

F4

49

71

M10

81

V

S

-

-

-

-

CAP_1

32

-

50

72

N10

82

V

S

-

-

-

-

DD

I2C2_SMBA / TIM12_CH1 /

-

-

-

-

M11

83

PH6

I/O

FT

-

ETH_MII_RXD2/

-

EVENTOUT

I2C3_SCL /

-

-

-

-

N12

84

PH7

I/O

FT

-

ETH_MII_RXD3/

-

EVENTOUT

I2C3_SDA /

-

-

-

-

M12

85

PH8

I/O

FT

-

DCMI_HSYNC/

-

EVENTOUT

I2C3_SMBA / TIM12_CH2/

-

-

-

-

M13

86

PH9

I/O

FT

-

-

DCMI_D0/ EVENTOUT

TIM5_CH1 / DCMI_D1/

-

-

-

-

L13

87

PH10

I/O

FT

-

-

EVENTOUT

TIM5_CH2 / DCMI_D2/

-

-

-

-

L12

88

PH11

I/O

FT

-

-

EVENTOUT

TIM5_CH3 / DCMI_D3/

-

-

-

-

K12

89

PH12

I/O

FT

-

-

EVENTOUT

S

-

-

-

-

-

-

-

-

H12

90

V

SS

S

-

-

-

-

-

-

-

-

J12

91

V

DD

DS8626 Rev 11

53/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

SPI2_NSS / I2S2_WS /

I2C2_SMBA/

USART3_CK/ TIM1_BKIN /

CAN2_RX /

33

J3

51

73

P12

92

PB12

I/O

FT

-

-

OTG_HS_ULPI_D5/

ETH_RMII_TXD0 /

ETH_MII_TXD0/

OTG_HS_ID/ EVENTOUT

SPI2_SCK / I2S2_CK /

USART3_CTS/

TIM1_CH1N /CAN2_TX /

34

J1

52

74

P13

93

PB13

I/O

FT

-

OTG_HS_ULPI_D6 /

OTG_HS_VBUS

ETH_RMII_TXD1 /

ETH_MII_TXD1/

EVENTOUT

SPI2_MISO/ TIM1_CH2N /

TIM12_CH1 /

OTG_HS_DM/

35

J2

53

75

R14

94

PB14

I/O

FT

-

-

USART3_RTS /

TIM8_CH2N/I2S2ext_SD/

EVENTOUT

SPI2_MOSI / I2S2_SD/

TIM1_CH3N / TIM8_CH3N

36

H1

54

76

R15

95

PB15

I/O

FT

-

RTC_REFIN

/ TIM12_CH2 /

OTG_HS_DP/ EVENTOUT

FSMC_D13 / USART3_TX/

-

H2

55

77

P15

96

PD8

I/O

FT

-

-

EVENTOUT

FSMC_D14 / USART3_RX/

-

H3

56

78

P14

97

PD9

I/O

FT

-

-

EVENTOUT

FSMC_D15 / USART3_CK/

-

G3

57

79

N15

98

PD10

I/O

FT

-

-

EVENTOUT

FSMC_CLE /

-

G1

58

80

N14

99

PD11

I/O

FT

-

FSMC_A16/USART3_CTS/

-

EVENTOUT

FSMC_ALE/

FSMC_A17/TIM4_CH1 /

-

G2

59

81

N13

100

PD12

I/O

FT

-

-

USART3_RTS/

EVENTOUT

54/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

FSMC_A18/TIM4_CH2/

-

-

60

82

M15

101

PD13

I/O

FT

-

-

EVENTOUT

-

-

-

83

-

102

V

S

-

-

-

-

SS

-

-

-

84

J13

103

V

S

-

-

-

-

DD

FSMC_D0/TIM4_CH3/

-

F2

61

85

M14

104

PD14

I/O

FT

-

-

EVENTOUT/ EVENTOUT

FSMC_D1/TIM4_CH4/

-

F1

62

86

L14

105

PD15

I/O

FT

-

-

EVENTOUT

-

-

-

87

L15

106

PG2

I/O

FT

-

FSMC_A12/ EVENTOUT

-

-

-

-

88

K15

107

PG3

I/O

FT

-

FSMC_A13/ EVENTOUT

-

-

-

-

89

K14

108

PG4

I/O

FT

-

FSMC_A14/ EVENTOUT

-

-

-

-

90

K13

109

PG5

I/O

FT

-

FSMC_A15/ EVENTOUT

-

-

-

-

91

J15

110

PG6

I/O

FT

-

FSMC_INT2/ EVENTOUT

-

FSMC_INT3 /USART6_CK/

-

-

-

92

J14

111

PG7

I/O

FT

-

-

EVENTOUT

USART6_RTS /

-

-

-

93

H14

112

PG8

I/O

FT

-

ETH_PPS_OUT/

-

EVENTOUT

S

-

-

-

-

-

-

94

G12

113

V

SS

S

-

-

-

-

-

-

95

H13

114

V

DD

I2S2_MCK /

TIM8_CH1/SDIO_D6 /

37

F3

63

96

H15

115

PC6

I/O

FT

-

USART6_TX /

-

DCMI_D0/TIM3_CH1/

EVENTOUT

I2S3_MCK /

TIM8_CH2/SDIO_D7 /

38

E1

64

97

G15

116

PC7

I/O

FT

-

USART6_RX /

-

DCMI_D1/TIM3_CH2/

EVENTOUT

TIM8_CH3/SDIO_D0

39

E2

65

98

G14

117

PC8

I/O

FT

-

/TIM3_CH3/ USART6_CK /

-

DCMI_D2/ EVENTOUT

DS8626 Rev 11

55/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

I2S_CKIN/ MCO2 /

TIM8_CH4/SDIO_D1 /

40

E3

66

99

F14

118

PC9

I/O

FT

-

-

/I2C3_SDA / DCMI_D3 /

TIM3_CH4/ EVENTOUT

MCO1 / USART1_CK/

TIM1_CH1/ I2C3_SCL/

41

D1

67

100

F15

119

PA8

I/O

FT

-

-

OTG_FS_SOF/

EVENTOUT

USART1_TX/ TIM1_CH2 /

42

D2

68

101

E15

120

PA9

I/O

FT

-

I2C3_SMBA / DCMI_D0/

OTG_FS_VBUS

EVENTOUT

USART1_RX/ TIM1_CH3/

43

D3

69

102

D15

121

PA10

I/O

FT

-

OTG_FS_ID/DCMI_D1/

-

EVENTOUT

USART1_CTS / CAN1_RX

44

C1

70

103

C15

122

PA11

I/O

FT

-

/ TIM1_CH4 /

-

OTG_FS_DM/ EVENTOUT

USART1_RTS / CAN1_TX/

45

C2

71

104

B15

123

PA12

I/O

FT

-

TIM1_ETR/ OTG_FS_DP/

-

EVENTOUT

PA13

46

D4

72

105

A15

124

I/O

FT

-

JTMS-SWDIO/ EVENTOUT

-

(JTMS-SWDIO)

S

-

-

-

-

47

B1

73

106

F13

125

V

CAP_2

S

-

-

-

-

-

E7

74

107

F12

126

V

SS

S

-

-

-

-

48

E6

75

108

G13

127

V

DD

TIM8_CH1N / CAN1_TX/

-

-

-

-

E12

128

PH13

I/O

FT

-

-

EVENTOUT

TIM8_CH2N / DCMI_D4/

-

-

-

-

E13

129

PH14

I/O

FT

-

-

EVENTOUT

TIM8_CH3N / DCMI_D11/

-

-

-

-

D13

130

PH15

I/O

FT

-

-

EVENTOUT

TIM5_CH4 / SPI2_NSS /

-

C3

-

-

E14

131

PI0

I/O

FT

-

I2S2_WS / DCMI_D13/

-

EVENTOUT

SPI2_SCK / I2S2_CK /

-

B2

-

-

D14

132

PI1

I/O

FT

-

-

DCMI_D8/ EVENTOUT

56/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

TIM8_CH4 /SPI2_MISO /

-

-

-

-

C14

133

PI2

I/O

FT

-

DCMI_D9 / I2S2ext_SD/

-

EVENTOUT

TIM8_ETR / SPI2_MOSI /

-

-

-

-

C13

134

PI3

I/O

FT

-

I2S2_SD / DCMI_D10/

-

EVENTOUT

-

-

-

-

D9

135

V

S

-

-

-

-

SS

S

-

-

-

-

-

-

-

-

C9

136

V

DD

PA14

49

A2

76

109

A14

137

I/O

FT

-

JTCK-SWCLK/ EVENTOUT

-

(JTCK/SWCLK)

JTDI/ SPI3_NSS/

PA15

50

B3

77

110

A13

138

I/O

FT

-

I2S3_WS/TIM2_CH1_ETR

-

(JTDI)

/ SPI1_NSS / EVENTOUT

SPI3_SCK / I2S3_CK/

UART4_TX/SDIO_D2 /

51

D5

78

111

B14

139

PC10

I/O

FT

-

-

DCMI_D8 / USART3_TX/

EVENTOUT

UART4_RX/ SPI3_MISO /

SDIO_D3 /

52

C4

79

112

B13

140

PC11

I/O

FT

-

-

DCMI_D4/USART3_RX /

I2S3ext_SD/ EVENTOUT

UART5_TX/SDIO_CK /

DCMI_D9 / SPI3_MOSI

53

A3

80

113

A12

141

PC12

I/O

FT

-

-

/I2S3_SD / USART3_CK/

EVENTOUT

FSMC_D2/CAN1_RX/

-

D6

81

114

B12

142

PD0

I/O

FT

-

-

EVENTOUT

FSMC_D3 / CAN1_TX/

-

C5

82

115

C12

143

PD1

I/O

FT

-

-

EVENTOUT

TIM3_ETR/UART5_RX/

54

B4

83

116

D12

144

PD2

I/O

FT

-

SDIO_CMD / DCMI_D11/

-

EVENTOUT

FSMC_CLK/

-

-

84

117

D11

145

PD3

I/O

FT

-

USART2_CTS/

-

EVENTOUT

DS8626 Rev 11

57/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

FSMC_NOE/

-

A4

85

118

D10

146

PD4

I/O

FT

-

USART2_RTS/

-

EVENTOUT

FSMC_NWE/USART2_TX/

-

C6

86

119

C11

147

PD5

I/O

FT

-

-

EVENTOUT

S

-

-

-

-

-

-

-

120

D8

148

V

SS

S

-

-

-

-

-

-

-

121

C8

149

V

DD

FSMC_NWAIT/

-

B5

87

122

B11

150

PD6

I/O

FT

-

-

USART2_RX/ EVENTOUT

USART2_CK/FSMC_NE1/

-

A5

88

123

A11

151

PD7

I/O

FT

-

-

FSMC_NCE2/ EVENTOUT

USART6_RX /

-

-

-

124

C10

152

PG9

I/O

FT

-

FSMC_NE2/FSMC_NCE3/

-

EVENTOUT

FSMC_NCE4_1/

-

-

-

125

B10

153

PG10

I/O

FT

-

-

FSMC_NE3/ EVENTOUT

FSMC_NCE4_2 /

ETH_MII_TX_EN/

-

-

-

126

B9

154

PG11

I/O

FT

-

-

ETH _RMII_TX_EN/

EVENTOUT

FSMC_NE4 /

-

-

-

127

B8

155

PG12

I/O

FT

-

USART6_RTS/

-

EVENTOUT

FSMC_A24 /

USART6_CTS

-

-

-

128

A8

156

PG13

I/O

FT

-

/ETH_MII_TXD0/

-

ETH_RMII_TXD0/

EVENTOUT

FSMC_A25 / USART6_TX

/ETH_MII_TXD1/

-

-

-

129

A7

157

PG14

I/O

FT

-

-

ETH_RMII_TXD1/

EVENTOUT

S

-

-

-

-

-

E8

-

130

D7

158

V

SS

S

-

-

-

-

-

F7

-

131

C7

159

V

DD

USART6_CTS /

-

-

-

132

B7

160

PG15

I/O

FT

-

-

DCMI_D13/ EVENTOUT

58/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

JTDO/ TRACESWO/

PB3

SPI3_SCK / I2S3_CK /

55

B6

89

133

A10

161

I/O

FT

-

-

(JTDO/

TIM2_CH2 / SPI1_SCK/

TRACESWO)

EVENTOUT

NJTRST/ SPI3_MISO /

PB4

56

A6

90

134

A9

162

I/O

FT

-

TIM3_CH1 / SPI1_MISO /

-

(NJTRST)

I2S3ext_SD/ EVENTOUT

I2C1_SMBA/ CAN2_RX /

OTG_HS_ULPI_D7 /

ETH_PPS_OUT/TIM3_CH2

57

D7

91

135

A6

163

PB5

I/O

FT

-

-

/ SPI1_MOSI/ SPI3_MOSI /

DCMI_D10 / I2S3_SD/

EVENTOUT

I2C1_SCL/ TIM4_CH1 /

CAN2_TX /

58

C7

92

136

B6

164

PB6

I/O

FT

-

-

DCMI_D5/USART1_TX/

EVENTOUT

I2C1_SDA / FSMC_NL /

DCMI_VSYNC /

59

B7

93

137

B5

165

PB7

I/O

FT

-

-

USART1_RX/ TIM4_CH2/

EVENTOUT

60

A7

94

138

D6

166

BOOT0

I

B

-

-

V

PP

TIM4_CH3/SDIO_D4/

TIM10_CH1 / DCMI_D6 /

61

D8

95

139

A5

167

PB8

I/O

FT

-

ETH_MII_TXD3 /

-

I2C1_SCL/ CAN1_RX/

EVENTOUT

SPI2_NSS/ I2S2_WS

/

TIM4_CH4/ TIM11_CH1/

62

C8

96

140

B4

168

PB9

I/O

FT

-

SDIO_D5 / DCMI_D7 /

-

I2C1_SDA / CAN1_TX/

EVENTOUT

TIM4_ETR / FSMC_NBL0 /

-

-

97

141

A4

169

PE0

I/O

FT

-

-

DCMI_D2/ EVENTOUT

FSMC_NBL1 / DCMI_D3/

-

-

98

142

A3

170

PE1

I/O

FT

-

-

EVENTOUT

S

-

-

-

-

63

-

99

-

D5

-

V

SS

DS8626 Rev 11

59/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

(1)

Table 7. STM32F40xxx pin and ball definitions

(continued)

Pin number

Pin name

Additional

Alternate functions

(function after

functions

(2)

reset)

Notes

Pin type

LQFP64

I / O structure

LQFP100

LQFP144

LQFP176

WLCSP90

UFBGA176

-

A8

-

143

C6

171

PDR_ON

I

FT

-

-

-

10

64

A1

144

C5

172

V

S

-

-

-

-

DD

0

TIM8_BKIN / DCMI_D5/

-

-

-

-

D4

173

PI4

I/O

FT

-

-

EVENTOUT

TIM8_CH1 /

-

-

-

-

C4

174

PI5

I/O

FT

-

DCMI_VSYNC/

-

EVENTOUT

TIM8_CH2 / DCMI_D6/

-

-

-

-

C3

175

PI6

I/O

FT

-

-

EVENTOUT

TIM8_CH3 / DCMI_D7/

-

-

-

-

C2

176

PI7

I/O

FT

-

-

EVENTOUT

1.

UFBGA176 F6, F7, F8, F9, F10, G6, G7, G8, G9, G10, H6, H7, H8, H9, H10, J6, J7, J8, J9, J10, K6, K7, K8, K9 and K10

balls are connected to V

for heat dissipation and package mechanical stability.

SS

2.

Function availability depends on the chosen device.

3.

PC13, PC14, PC15 and PI8 are supplied through the power switch. Since the switch only sinks a limited amount of current

(3 mA), the use of GPIOs PC13 to PC15 and PI8 in output mode is limited:

- The speed should not exceed 2 MHz with a maximum load of 30 pF.

- These I/Os must not be used as a current source (e.g. to drive an LED).

4.

Main function after the first backup domain power-up. Later on, it depends on the contents of the RTC registers even after

reset (because these registers are not reset by the main reset). For details on how to manage these I/Os, refer to the RTC

register description sections in the STM32F4xx reference manual, available from the STMicroelectronics website:

www.st.com.

5.

FT = 5 V tolerant except when in analog mode or oscillator mode (for PC14, PC15, PH0 and PH1).

6.

If the device is delivered in an UFBGA176 or WLCSP90 and the BYPASS_REG pin is set to VDD (Regulator off/internal

reset ON mode), then PA0 is used as an internal Reset (active low).

Table 8. FSMC pin definition

FSMC

WLCSP90

(1)

(2)

Pins

LQFP100

(2)

NOR/PSRAM/

CF

NOR/PSRAM Mux

NAND 16 bit

SRAM

PE2

-

A23

A23

-

Yes

-

PE3

-

A19

A19

-

Yes

-

PE4

-

A20

A20

-

Yes

-

PE5

-

A21

A21

-

Yes

-

PE6

-

A22

A22

-

Yes

-

60/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Pinouts and pin description

Table 8. FSMC pin definition (continued)

FSMC

WLCSP90

(1)

(2)

LQFP100

Pins

(2)

NOR/PSRAM/

CF

NOR/PSRAM Mux

NAND 16 bit

SRAM

PF0

A0

A0

-

-

-

-

PF1

A1

A1

-

-

-

-

PF2

A2

A2

-

-

-

-

PF3

A3

A3

-

-

-

-

PF4

A4

A4

-

-

-

-

PF5

A5

A5

-

-

-

-

PF6

NIORD

-

-

-

-

-

PF7

NREG

-

-

-

-

-

PF8

NIOWR

-

-

-

-

-

PF9

CD

-

-

-

-

-

PF10

INTR

-

-

-

-

-

PF12

A6

A6

-

-

-

-

PF13

A7

A7

-

-

-

-

PF14

A8

A8

-

-

-

-

PF15

A9

A9

-

-

-

-

PG0

A10

A10

-

-

-

-

PG1

-

A11

-

-

-

-

PE7

D4

D4

DA4

D4

Yes

Yes

PE8

D5

D5

DA5

D5

Yes

Yes

PE9

D6

D6

DA6

D6

Yes

Yes

PE10

D7

D7

DA7

D7

Yes

Yes

PE11

D8

D8

DA8

D8

Yes

Yes

PE12

D9

D9

DA9

D9

Yes

Yes

PE13

D10

D10

DA10

D10

Yes

Yes

PE14

D11

D11

DA11

D11

Yes

Yes

PE15

D12

D12

DA12

D12

Yes

Yes

PD8

D13

D13

DA13

D13

Yes

Yes

PD9

D14

D14

DA14

D14

Yes

Yes

PD10

D15

D15

DA15

D15

Yes

Yes

PD11

-

A16

A16

CLE

Yes

Yes

PD12

-

A17

A17

ALE

Yes

Yes

PD13

-

A18

A18

-

Yes

-

PD14

D0

D0

DA0

D0

Yes

Yes

DS8626 Rev 11

61/205

190

Pinouts and pin description

STM32F405xx, STM32F407xx

Table 8. FSMC pin definition (continued)

FSMC

WLCSP90

(1)

(2)

LQFP100

Pins

(2)

NOR/PSRAM/

CF

NOR/PSRAM Mux

NAND 16 bit

SRAM

PD15

D1

D1

DA1

D1

Yes

Yes

PG2

-

A12

-

-

-

-

PG3

-

A13

-

-

-

-

PG4

-

A14

-

-

-

-

PG5

-

A15

-

-

-

-

PG6

-

-

-

INT2

-

-

PG7

-

-

-

INT3

-

-

PD0

D2

D2

DA2

D2

Yes

Yes

PD1

D3

D3

DA3

D3

Yes

Yes

PD3

-

CLK

CLK

-

Yes

-

PD4

NOE

NOE

NOE

NOE

Yes

Yes

PD5

NWE

NWE

NWE

NWE

Yes

Yes

PD6

NWAIT

NWAIT

NWAIT

NWAIT

Yes

Yes

PD7

-

NE1

NE1

NCE2

Yes

Yes

PG9

-

NE2

NE2

NCE3

-

-

PG10

NCE4_1

NE3

NE3

-

-

-

PG11

NCE4_2

-

-

-

-

-

PG12

-

NE4

NE4

-

-

-

PG13

-

A24

A24

-

-

-

PG14

-

A25

A25

-

-

-

PB7

-

NADV

NADV

-

Yes

Yes

PE0

-

NBL0

NBL0

-

Yes

-

PE1

-

NBL1

NBL1

-

Yes

-

1.

Full FSMC features are available on LQFP144, LQFP176, and UFBGA176. The features available on

smaller packages are given in the dedicated package column.

2.

Ports F and G are not available in devices delivered in 100-pin packages.

62/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Table 9. Alternate function mapping

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

TIM2_CH1_

PA0

-

TIM 5_CH1

TIM8_ETR

-

-

-

USART2_CTS

UART4_TX

-

-

ETH_MII_CRS

-

-

-

EVENTOUT

ETR

ETH_MII

_RX_CLK

-

-

-

EVENTOUT

PA1

-

TIM2_CH2

TIM5_CH2

-

-

-

-

USART2_RTS

UART4_RX

-

-

ETH_RMII__REF

_CLK

PA2

-

TIM2_CH3

TIM5_CH3

TIM9_CH1

-

-

-

USART2_TX

-

-

-

ETH_MDIO

-

-

-

EVENTOUT

OTG_HS_ULPI_

PA3

-

TIM2_CH4

TIM5_CH4

TIM9_CH2

-

-

-

USART2_RX

-

-

ETH _MII_COL

-

-

-

EVENTOUT

D0

SPI3_NSS

DCMI_

PA4

-

-

-

-

-

SPI1_NSS

USART2_CK

-

-

-

-

OTG_HS_SOF

-

EVENTOUT

I2S3_WS

HSYNC

TIM2_CH1_

OTG_HS_ULPI_

PA5

-

-

TIM8_CH1N

-

SPI1_SCK

-

-

-

-

-

-

-

-

EVENTOUT

ETR

CK

DCMI_

DS8626 Rev 11

PA6

-

TIM1_BKIN

TIM3_CH1

TIM8_BKIN

-

SPI1_MISO

-

-

-

TIM13_CH1

-

-

-

-

EVENTOUT

PIXCLK

ETH_MII _RX_DV

Port A

PA7

-

TIM1_CH1N

TIM3_CH2

TIM8_CH1N

-

SPI1_MOSI

-

-

-

TIM14_CH1

-

ETH_RMII

-

-

-

EVENTOUT

_CRS_DV

PA8

MCO1

TIM1_CH1

-

-

I2C3_SCL

-

-

USART1_CK

-

-

OTG_FS_SOF

-

-

-

-

EVENTOUT

I2C3_

PA9

-

TIM1_CH2

-

-

-

-

USART1_TX

-

-

-

-

-

DCMI_D0

-

EVENTOUT

SMBA

PA10

-

TIM1_CH3

-

-

-

-

-

USART1_RX

-

-

OTG_FS_ID

-

-

DCMI_D1

-

EVENTOUT

PA11

-

TIM1_CH4

-

-

-

-

-

USART1_CTS

-

CAN1_RX

OTG_FS_DM

-

-

-

-

EVENTOUT

PA12

-

TIM1_ETR

-

-

-

-

-

USART1_RTS

-

CAN1_TX

OTG_FS_DP

-

-

-

-

EVENTOUT

JTMS-

PA13

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

SWDIO

Pinouts and pin description

JTCK-

PA14

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

SWCLK

TIM 2_CH1

SPI3_NSS/

PA15

JTDI

-

-

-

SPI1_NSS

-

-

-

-

-

-

-

-

EVENTOUT

TIM 2_ETR

I2S3_WS

63/205

Table 9. Alternate function mapping (continued)

64/205

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

OTG_HS_ULPI_

PB0

-

TIM1_CH2N

TIM3_CH3

TIM8_CH2N

-

-

-

-

-

-

ETH _MII_RXD2

-

-

-

EVENTOUT

D1

OTG_HS_ULPI_

PB1

-

TIM1_CH3N

TIM3_CH4

TIM8_CH3N

-

-

-

-

-

ETH _MII_RXD3

-

-

-

EVENTOUT

D2

PB2

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

JTDO/

SPI3_SCK

PB3

TRACES

TIM2_CH2

-

-

-

SPI1_SCK

-

-

-

-

-

-

-

-

EVENTOUT

I2S3_CK

WO

PB4

NJTRST

-

TIM3_CH1

-

SPI1_MISO

SPI3_MISO

I2S3ext_SD

-

-

-

-

-

-

-

EVENTOUT

I2C1_SMB

SPI3_MOSI

OTG_HS_ULPI_

PB5

-

-

TIM3_CH2

SPI1_MOSI

-

-

CAN2_RX

ETH _PPS_OUT

-

DCMI_D10

-

EVENTOUT

A

I2S3_SD

D7

PB6

-

-

TIM4_CH1

I2C1_SCL

-

-

USART1_TX

-

CAN2_TX

-

-

-

DCMI_D5

-

EVENTOUT

DCMI_VSYN

PB7

-

-

TIM4_CH2

I2C1_SDA

-

-

USART1_RX

-

-

-

-

FSMC_NL

-

EVENTOUT

C

DS8626 Rev 11

Port B

PB8

-

-

TIM4_CH3

TIM10_CH1

I2C1_SCL

-

-

-

-

CAN1_RX

-

ETH _MII_TXD3

SDIO_D4

DCMI_D6

-

EVENTOUT

SPI2_NSS

PB9

-

-

TIM4_CH4

TIM11_CH1

I2C1_SDA

-

-

-

CAN1_TX

-

-

SDIO_D5

DCMI_D7

-

EVENTOUT

I2S2_WS

SPI2_SCK

OTG_HS_ULPI_

PB10

-

TIM2_CH3

-

-

I2C2_SCL

-

USART3_TX

-

-

ETH_ MII_RX_ER

-

-

-

EVENTOUT

I2S2_CK

D3

ETH _MII_TX_EN

OTG_HS_ULPI_

PB11

-

TIM2_CH4

-

-

I2C2_SDA

-

-

USART3_RX

-

-

ETH

-

-

-

EVENTOUT

D4

_RMII_TX_EN

I2C2_

SPI2_NSS

OTG_HS_ULPI_

ETH _MII_TXD0

PB12

-

TIM1_BKIN

-

-

-

USART3_CK

-

CAN2_RX

OTG_HS_ID

-

-

EVENTOUT

SMBA

I2S2_WS

D5

ETH _RMII_TXD0

ETH _MII_TXD1

SPI2_SCK

OTG_HS_ULPI_

PB13

-

TIM1_CH1N

-

-

-

-

USART3_CTS

-

CAN2_TX

-

-

-

EVENTOUT

I2S2_CK

D6

ETH _RMII_TXD1

PB14

-

TIM1_CH2N

-

TIM8_CH2N

-

SPI2_MISO

I2S2ext_SD

USART3_RTS

-

TIM12_CH1

-

-

OTG_HS_DM

-

-

EVENTOUT

STM32F405xx, STM32F407xx

RTC_

SPI2_MOSI

PB15

TIM1_CH3N

-

TIM8_CH3N

-

-

-

-

TIM12_CH2

-

-

OTG_HS_DP

-

-

EVENTOUT

REFIN

I2S2_SD

Table 9. Alternate function mapping (continued)

STM32F405xx, STM32F407xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

OTG_HS_ULPI_

PC0

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

STP

PC1

-

-

-

-

-

-

-

-

-

-

-

ETH_MDC

-

-

-

EVENTOUT

OTG_HS_ULPI_

PC2

-

-

-

-

-

SPI2_MISO

I2S2ext_SD

-

-

-

ETH _MII_TXD2

-

-

-

EVENTOUT

DIR

SPI2_MOSI

OTG_HS_ULPI_

ETH

PC3

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

I2S2_SD

NXT

_MII_TX_CLK

ETH_MII_RXD0

PC4

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

ETH_RMII_RXD0

ETH _MII_RXD1

PC5

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

ETH _RMII_RXD1

PC6

-

-

TIM3_CH1

TIM8_CH1

I2S2_MCK

-

USART6_TX

-

-

-

SDIO_D6

DCMI_D0

-

EVENTOUT

Port C

PC7

-

-

TIM3_CH2

TIM8_CH2

-

-

I2S3_MCK

-

USART6_RX

-

-

-

SDIO_D7

DCMI_D1

-

EVENTOUT

DS8626 Rev 11

PC8

-

-

TIM3_CH3

TIM8_CH3

-

-

-

-

USART6_CK

-

-

-

SDIO_D0

DCMI_D2

-

EVENTOUT

PC9

MCO2

-

TIM3_CH4

TIM8_CH4

I2C3_SDA

I2S_CKIN

-

-

-

-

-

-

SDIO_D1

DCMI_D3

-

EVENTOUT

SPI3_SCK/

PC10

-

-

-

-

-

-

USART3_TX/

UART4_TX

-

-

-

SDIO_D2

DCMI_D8

-

EVENTOUT

I2S3_CK

PC11

-

-

-

-

-

I2S3ext_SD

SPI3_MISO/

USART3_RX

UART4_RX

-

-

-

SDIO_D3

DCMI_D4

-

EVENTOUT

SPI3_MOSI

PC12

-

-

-

-

-

-

USART3_CK

UART5_TX

-

-

-

SDIO_CK

DCMI_D9

-

EVENTOUT

I2S3_SD

PC13

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

PC14

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

PC15

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

Pinouts and pin description

65/205

Table 9. Alternate function mapping (continued)

66/205

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

PD0

-

-

-

-

-

-

-

-

-

CAN1_RX

-

-

FSMC_D2

-

-

EVENTOUT

PD1

-

-

-

-

-

-

-

-

-

CAN1_TX

-

-

FSMC_D3

-

-

EVENTOUT

PD2

-

-

TIM3_ETR

-

-

-

-

-

UART5_RX

-

-

-

SDIO_CMD

DCMI_D11

-

EVENTOUT

PD3

-

-

-

-

-

-

-

USART2_CTS

-

-

-

-

FSMC_CLK

-

-

EVENTOUT

PD4

-

-

-

-

-

-

-

USART2_RTS

-

-

-

-

FSMC_NOE

-

-

EVENTOUT

PD5

-

-

-

-

-

-

-

USART2_TX

-

-

-

-

FSMC_NWE

-

-

EVENTOUT

PD6

-

-

-

-

-

-

-

USART2_RX

-

-

-

-

FSMC_NWAIT

-

-

EVENTOUT

FSMC_NE1/

PD7

-

-

-

-

-

-

-

USART2_CK

-

-

-

-

-

-

EVENTOUT

FSMC_NCE2

Port D

PD8

-

-

-

-

-

-

-

USART3_TX

-

-

-

-

FSMC_D13

-

-

EVENTOUT

PD9

-

-

-

-

-

-

-

USART3_RX

-

-

-

-

FSMC_D14

-

-

EVENTOUT

DS8626 Rev 11

PD10

-

-

-

-

-

-

-

USART3_CK

-

-

-

-

FSMC_D15

-

-

EVENTOUT

PD11

-

-

-

-

-

-

-

USART3_CTS

-

-

-

-

FSMC_A16

-

-

EVENTOUT

PD12

-

-

TIM4_CH1

-

-

-

-

USART3_RTS

-

-

-

-

FSMC_A17

-

-

EVENTOUT

PD13

-

-

TIM4_CH2

-

-

-

-

-

-

-

-

-

FSMC_A18

-

-

EVENTOUT

PD14

-

-

TIM4_CH3

-

-

-

-

-

-

-

-

-

FSMC_D0

-

-

EVENTOUT

PD15

-

-

TIM4_CH4

-

-

-

-

-

-

-

-

-

FSMC_D1

-

-

EVENTOUT

STM32F405xx, STM32F407xx

Table 9. Alternate function mapping (continued)

STM32F405xx, STM32F407xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

PE0

-

-

TIM4_ETR

-

-

-

-

-

-

-

-

-

FSMC_NBL0

DCMI_D2

-

EVENTOUT

PE1

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_NBL1

DCMI_D3

-

EVENTOUT

TRACECL

PE2

-

-

-

-

-

-

-

-

-

-

ETH _MII_TXD3

FSMC_A23

-

-

EVENTOUT

K

PE3

TRACED0

-

-

-

-

-

-

-

-

-

-

-

FSMC_A19

-

-

EVENTOUT

PE4

TRACED1

-

-

-

-

-

-

-

-

-

-

-

FSMC_A20

DCMI_D4

-

EVENTOUT

PE5

TRACED2

-

-

TIM9_CH1

-

-

-

-

-

-

-

-

FSMC_A21

DCMI_D6

-

EVENTOUT

PE6

TRACED3

-

-

TIM9_CH2

-

-

-

-

-

-

-

-

FSMC_A22

DCMI_D7

-

EVENTOUT

PE7

-

TIM1_ETR

-

-

-

-

-

-

-

-

-

-

FSMC_D4

-

-

EVENTOUT

Port E

PE8

-

TIM1_CH1N

-

-

-

-

-

-

-

-

-

-

FSMC_D5

-

-

EVENTOUT

PE9

-

TIM1_CH1

-

-

-

-

-

-

-

-

-

-

FSMC_D6

-

-

EVENTOUT

DS8626 Rev 11

PE10

-

TIM1_CH2N

-

-

-

-

-

-

-

-

-

-

FSMC_D7

-

-

EVENTOUT

PE11

-

TIM1_CH2

-

-

-

-

-

-

-

-

-

-

FSMC_D8

-

-

EVENTOUT

PE12

-

TIM1_CH3N

-

-

-

-

-

-

-

-

-

-

FSMC_D9

-

-

EVENTOUT

PE13

-

TIM1_CH3

-

-

-

-

-

-

-

-

-

-

FSMC_D10

-

-

EVENTOUT

PE14

-

TIM1_CH4

-

-

-

-

-

-

-

-

-

-

FSMC_D11

-

-

EVENTOUT

PE15

-

TIM1_BKIN

-

-

-

-

-

-

-

-

-

-

FSMC_D12

-

-

EVENTOUT

Pinouts and pin description

67/205

Table 9. Alternate function mapping (continued)

68/205

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

PF0

-

-

-

-

I2C2_SDA

-

-

-

-

-

-

-

FSMC_A0

-

-

EVENTOUT

PF1

-

-

-

-

I2C2_SCL

-

-

-

-

-

-

-

FSMC_A1

-

-

EVENTOUT

I2C2_

PF2

-

-

-

-

-

-

-

-

-

-

-

FSMC_A2

-

-

EVENTOUT

SMBA

PF3

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A3

-

-

EVENTOUT

PF4

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A4

-

-

EVENTOUT

PF5

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A5

-

-

EVENTOUT

PF6

-

-

-

TIM10_CH1

-

-

-

-

-

-

-

-

FSMC_NIORD

-

-

EVENTOUT

PF7

-

-

-

TIM11_CH1

-

-

-

-

-

-

-

-

FSMC_NREG

-

-

EVENTOUT

Port F

FSMC_

PF8

-

-

-

-

-

-

-

-

-

TIM13_CH1

-

-

-

-

EVENTOUT

NIOWR

PF9

-

-

-

-

-

-

-

-

-

TIM14_CH1

-

-

FSMC_CD

-

-

EVENTOUT

DS8626 Rev 11

PF10

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_INTR

-

-

EVENTOUT

PF11

-

-

-

-

-

-

-

-

-

-

-

-

DCMI_D12

-

EVENTOUT

PF12

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A6

-

-

EVENTOUT

PF13

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A7

-

-

EVENTOUT

PF14

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A8

-

-

EVENTOUT

PF15

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A9

-

-

EVENTOUT

STM32F405xx, STM32F407xx

Table 9. Alternate function mapping (continued)

STM32F405xx, STM32F407xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

PG0

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A10

-

-

EVENTOUT

PG1

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A11

-

-

EVENTOUT

PG2

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A12

-

-

EVENTOUT

PG3

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A13

-

-

EVENTOUT

PG4

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A14

-

-

EVENTOUT

PG5

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_A15

-

-

EVENTOUT

PG6

-

-

-

-

-

-

-

-

-

-

-

-

FSMC_INT2

-

-

EVENTOUT

PG7

-

-

-

-

-

-

-

-

USART6_CK

-

-

-

FSMC_INT3

-

-

EVENTOUT

USART6_

PG8

-

-

-

-

-

-

-

-

-

-

ETH _PPS_OUT

-

-

-

EVENTOUT

RTS

FSMC_NE2/

PG9

-

-

-

-

-

-

-

-

USART6_RX

-

-

-

-

-

EVENTOUT

Port G

FSMC_NCE3

DS8626 Rev 11

FSMC_

PG10

-

-

-

-

-

-

-

-

-

-

-

-

NCE4_1/

-

-

EVENTOUT

FSMC_NE3

ETH _MII_TX_EN

FSMC_NCE4_

PG11

-

-

-

-

-

-

-

-

-

-

-

ETH _RMII_

-

-

EVENTOUT

2

TX_EN

USART6_

PG12

-

-

-

-

-

-

-

-

-

-

-

FSMC_NE4

-

-

EVENTOUT

RTS

ETH _MII_TXD0

PG13

-

-

-

-

-

-

-

-

UART6_CTS

-

-

FSMC_A24

-

-

EVENTOUT

ETH _RMII_TXD0

ETH _MII_TXD1

PG14

-

-

-

-

-

-

-

-

USART6_TX

-

-

FSMC_A25

-

-

EVENTOUT

ETH _RMII_TXD1

USART6_

PG15

-

-

-

-

-

-

-

-

-

-

-

-

DCMI_D13

-

EVENTOUT

CTS

Pinouts and pin description

69/205

Table 9. Alternate function mapping (continued)

70/205

Pinouts and pin description

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

PH0

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

PH1

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

PH2

-

-

-

-

-

-

-

-

-

-

-

ETH _MII_CRS

-

-

-

EVENTOUT

PH3

-

-

-

-

-

-

-

-

-

-

-

ETH _MII_COL

-

-

-

EVENTOUT

OTG_HS_ULPI_

PH4

-

-

-

-

I2C2_SCL

-

-

-

-

-

-

-

-

-

EVENTOUT

NXT

PH5

-

-

-

-

I2C2_SDA

-

-

-

-

-

-

-

-

-

-

EVENTOUT

I2C2_

PH6

-

-

-

-

-

-

-

-

TIM12_CH1

-

ETH _MII_RXD2

-

-

-

EVENTOUT

SMBA

PH7

-

-

-

-

I2C3_SCL

-

-

-

-

-

-

ETH _MII_RXD3

-

-

-

EVENTOUT

Port H

DCMI_

PH8

-

-

-

-

I2C3_SDA

-

-

-

-

-

-

-

-

-

EVENTOUT

HSYNC

DS8626 Rev 11

I2C3_

PH9

-

-

-

-

-

-

-

-

TIM12_CH2

-

-

-

DCMI_D0

-

EVENTOUT

SMBA

PH10

-

-

TIM5_CH1

-

-

-

-

-

-

-

-

-

-

DCMI_D1

-

EVENTOUT

PH11

-

-

TIM5_CH2

-

-

-

-

-

-

-

-

-

-

DCMI_D2

-

EVENTOUT

PH12

-

-

TIM5_CH3

-

-

-

-

-

-

-

-

-

-

DCMI_D3

-

EVENTOUT

PH13

-

-

-

TIM8_CH1N

-

-

-

-

-

CAN1_TX

-

-

-

-

-

EVENTOUT

PH14

-

-

-

TIM8_CH2N

-

-

-

-

-

-

-

-

-

DCMI_D4

-

EVENTOUT

PH15

-

-

-

TIM8_CH3N

-

-

-

-

-

-

-

-

-

DCMI_D11

-

EVENTOUT

STM32F405xx, STM32F407xx

Table 9. Alternate function mapping (continued)

STM32F405xx, STM32F407xx

AF0

AF1

AF2

AF3

AF4

AF5

AF6

AF7

AF8

AF9

AF10

AF11

AF12

AF13

Port

AF14

AF15

SPI1/SPI2/

CAN1/2

TIM8/9/10

SPI3/I2Sext

USART1/2/3/

UART4/5/

OTG_FS/

FSMC/SDIO

SYS

TIM1/2

TIM3/4/5

I2C1/2/3

TIM12/13/

ETH

DCMI

I2S2/I2S2e

/11

/I2S3

I2S3ext

USART6

OTG_HS

/OTG_FS

14

xt

SPI2_NSS

PI0

-

-

TIM5_CH4

-

-

-

-

-

-

-

-

-

DCMI_D13

-

EVENTOUT

I2S2_WS

SPI2_SCK

PI1

-

-

-

-

-

-

-

-

-

-

-

-

DCMI_D8

-

EVENTOUT

I2S2_CK

PI2

-

-

-

TIM8_CH4

-

SPI2_MISO

I2S2ext_SD

-

-

-

-

-

-

DCMI_D9

-

EVENTOUT

SPI2_MOSI

PI3

-

-

-

TIM8_ETR

-

-

-

-

-

-

-

-

DCMI_D10

-

EVENTOUT

I2S2_SD

PI4

-

-

-

TIM8_BKIN

-

-

-

-

-

-

-

-

-

DCMI_D5

-

EVENTOUT

DCMI_

PI5

-

-

-

TIM8_CH1

-

-

-

-

-

-

-

-

-

-

EVENTOUT

Port I

VSYNC

PI6

-

-

-

TIM8_CH2

-

-

-

-

-

-

-

-

-

DCMI_D6

-

EVENTOUT

PI7

-

-

-

TIM8_CH3

-

-

-

-

-

-

-

-

-

DCMI_D7

-

EVENTOUT

PI8

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

DS8626 Rev 11

PI9

-

-

-

-

-

-

-

-

-

CAN1_RX

-

-

-

-

-

EVENTOUT

PI10

-

-

-

-

-

-

-

-

-

-

-

ETH _MII_RX_ER

-

-

-

EVENTOUT

OTG_HS_ULPI_

PI11

-

-

-

-

-

-

-

-

-

-

-

-

-

-

EVENTOUT

DIR

Pinouts and pin description

71/205

Memory mapping

STM32F405xx, STM32F407xx

5

Memory mapping

The memory map is shown in Figure 18.

Figure 18. STM32F40xxx memory map

0xE010 0000 - 0xFFFF FFFF

Reserved

0xE000 0000 - 0xE00F FFFF

CORTEX-M4 internal peripherals

0xA000 1000 - 0xDFFF FFFF

Reserved

0xA000 0FFF

AHB3

0x6000 0000

0xFFFF FFFF

0xE000 0000

0xDFFF FFFF

0x5006 0C00 - 0x5FFF FFFF

Reserved

0x5006 0BFF

AHB2

512-Mbyte

block 7

0x5000 0000

Cortex-M4's

0x4008 0000 - 0x4FFF FFFF

Reserved

internal

0x4007 FFFF

peripherals

512-Mbyte

block 6

Not used

0xC000 0000

0xBFFF FFFF

AHB1

512-Mbyte

block 5

FSMC registers

0xA000 0000

0x9FFF FFFF

512-Mbyte

0x4002 0000

block 4

0x4001 5800 - 0x4001 FFFF

FSMC bank 3

Reserved

0x4001 57FF

& bank4

0x8000 0000

0x7FFF FFFF

512-Mbyte

block 3

FSMC bank1

& bank2

0x6000 0000

0x5FFF FFFF

512-Mbyte

APB2

block 2

Peripherals

0x4000 0000

0x3FFF FFFF

512-Mbyte

block 1

SRAM

0x2000 0000

0x1FFF FFFF

512-Mbyte

block 0

Code

Reserved

0x4001 0000

0x2002 0000 - 0x3FFF FFFF

0x4000 7800 - 0x4000 FFFF

Reserved

SRAM (16 KB aliased

0x2001 C000 - 0x2001 FFFF

0x4000 7FFF

by bit-banding)

SRAM (112 KB aliased

0x2000 0000 - 0x2001 BFFF

by bit-banding)

0x0000 0000

Reserved

0x1FFF C008 - 0x1FFF FFFF

Option Bytes

0x1FFF C000 - 0x1FFF C007

Reserved

0x1FFF 7A10 - 0x1FFF 7FFF

System memory + OTP

0x1FFF 0000 - 0x1FFF 7A0F

Reserved

0x1001 0000 - 0x1FFE FFFF

APB1

CCM data RAM

0x1000 0000 - 0x1000 FFFF

(64 KB data SRAM)

Reserved

0x0810 0000 - 0x0FFF FFFF

Flash

0x080F FFFF

0x0800 0000

-

Reserved

0x0010 0000 - 0x07FF FFFF

Aliased to Flash, system

0x0000 0000 - 0x000F FFFF

memory or SRAM depending

on the BOOT pins

0x4000 0000

ai18513g

72/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Memory mapping

Table 10. Register boundary addresses

Bus

Boundary address

Peripheral

0xE00F FFFF - 0xFFFF FFFF

Reserved

Cortex-M4

0xE000 0000 - 0xE00F FFFF

Cortex-M4 internal peripherals

0xA000 1000 - 0xDFFF FFFF

Reserved

0xA000 0000 - 0xA000 0FFF

FSMC control register

0x9000 0000 - 0x9FFF FFFF

FSMC bank 4

AHB3

0x8000 0000 - 0x8FFF FFFF

FSMC bank 3

0x7000 0000 - 0x7FFF FFFF

FSMC bank 2

0x6000 0000 - 0x6FFF FFFF

FSMC bank 1

0x5006 0C00- 0x5FFF FFFF

Reserved

0x5006 0800 - 0x5006 0BFF

RNG

AHB2

0x5005 0400 - 0x5006 07FF

Reserved

0x5005 0000 - 0x5005 03FF

DCMI

0x5004 0000- 0x5004 FFFF

Reserved

0x5000 0000 - 0x5003 FFFF

USB OTG FS

0x4008 0000- 0x4FFF FFFF

Reserved

DS8626 Rev 11

73/205

190

Memory mapping

STM32F405xx, STM32F407xx

Table 10. Register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4004 0000 - 0x4007 FFFF

USB OTG HS

0x4002 9400 - 0x4003 FFFF

Reserved

0x4002 9000 - 0x4002 93FF

0x4002 8C00 - 0x4002 8FFF

0x4002 8800 - 0x4002 8BFF

ETHERNET MAC

0x4002 8400 - 0x4002 87FF

0x4002 8000 - 0x4002 83FF

0x4002 6800 - 0x4002 7FFF

Reserved

0x4002 6400 - 0x4002 67FF

DMA2

0x4002 6000 - 0x4002 63FF

DMA1

0x4002 5000 - 0x4002 5FFF

Reserved

0x4002 4000 - 0x4002 4FFF

BKPSRAM

0x4002 3C00 - 0x4002 3FFF

Flash interface register

AHB1

0x4002 3800 - 0x4002 3BFF

RCC

0x4002 3400 - 0x4002 37FF

Reserved

0x4002 3000 - 0x4002 33FF

CRC

0x4002 2400 - 0x4002 2FFF

Reserved

0x4002 2000 - 0x4002 23FF

GPIOI

0x4002 1C00 - 0x4002 1FFF

GPIOH

0x4002 1800 - 0x4002 1BFF

GPIOG

0x4002 1400 - 0x4002 17FF

GPIOF

0x4002 1000 - 0x4002 13FF

GPIOE

0x4002 0C00 - 0x4002 0FFF

GPIOD

0x4002 0800 - 0x4002 0BFF

GPIOC

0x4002 0400 - 0x4002 07FF

GPIOB

0x4002 0000 - 0x4002 03FF

GPIOA

0x4001 5800- 0x4001 FFFF

Reserved

74/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Memory mapping

Table 10. Register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4001 4C00 - 0x4001 57FF

Reserved

0x4001 4800 - 0x4001 4BFF

TIM11

0x4001 4400 - 0x4001 47FF

TIM10

0x4001 4000 - 0x4001 43FF

TIM9

0x4001 3C00 - 0x4001 3FFF

EXTI

0x4001 3800 - 0x4001 3BFF

SYSCFG

0x4001 3400 - 0x4001 37FF

Reserved

0x4001 3000 - 0x4001 33FF

SPI1

APB2

0x4001 2C00 - 0x4001 2FFF

SDIO

0x4001 2400 - 0x4001 2BFF

Reserved

0x4001 2000 - 0x4001 23FF

ADC1 - ADC2 - ADC3

0x4001 1800 - 0x4001 1FFF

Reserved

0x4001 1400 - 0x4001 17FF

USART6

0x4001 1000 - 0x4001 13FF

USART1

0x4001 0800 - 0x4001 0FFF

Reserved

0x4001 0400 - 0x4001 07FF

TIM8

0x4001 0000 - 0x4001 03FF

TIM1

0x4000 7800- 0x4000 FFFF

Reserved

DS8626 Rev 11

75/205

190

Memory mapping

STM32F405xx, STM32F407xx

Table 10. Register boundary addresses (continued)

Bus

Boundary address

Peripheral

0x4000 7800 - 0x4000 7FFF

Reserved

0x4000 7400 - 0x4000 77FF

DAC

0x4000 7000 - 0x4000 73FF

PWR

0x4000 6C00 - 0x4000 6FFF

Reserved

0x4000 6800 - 0x4000 6BFF

CAN2

0x4000 6400 - 0x4000 67FF

CAN1

0x4000 6000 - 0x4000 63FF

Reserved

0x4000 5C00 - 0x4000 5FFF

I2C3

0x4000 5800 - 0x4000 5BFF

I2C2

0x4000 5400 - 0x4000 57FF

I2C1

0x4000 5000 - 0x4000 53FF

UART5

0x4000 4C00 - 0x4000 4FFF

UART4

0x4000 4800 - 0x4000 4BFF

USART3

0x4000 4400 - 0x4000 47FF

USART2

0x4000 4000 - 0x4000 43FF

I2S3ext

APB1

0x4000 3C00 - 0x4000 3FFF

SPI3 / I2S3

0x4000 3800 - 0x4000 3BFF

SPI2 / I2S2

0x4000 3400 - 0x4000 37FF

I2S2ext

0x4000 3000 - 0x4000 33FF

IWDG

0x4000 2C00 - 0x4000 2FFF

WWDG

0x4000 2800 - 0x4000 2BFF

RTC & BKP Registers

0x4000 2400 - 0x4000 27FF

Reserved

0x4000 2000 - 0x4000 23FF

TIM14

0x4000 1C00 - 0x4000 1FFF

TIM13

0x4000 1800 - 0x4000 1BFF

TIM12

0x4000 1400 - 0x4000 17FF

TIM7

0x4000 1000 - 0x4000 13FF

TIM6

0x4000 0C00 - 0x4000 0FFF

TIM5

0x4000 0800 - 0x4000 0BFF

TIM4

0x4000 0400 - 0x4000 07FF

TIM3

0x4000 0000 - 0x4000 03FF

TIM2

76/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

6

Electrical characteristics

6.1

Parameter conditions

Unless otherwise specified, all voltages are referenced to V

.

SS

6.1.1

Minimum and maximum values

Unless otherwise specified the minimum and maximum values are evaluated in the worst

conditions of ambient temperature, supply voltage, and frequencies by tests in production

on 100% of the devices with an ambient temperature at T

= 25 °C and T

= T

max (given

A

A

A

by the selected temperature range).

Data based on characterization results, design simulation and/or technology characteristics

are indicated in the table footnotes and are not tested in production. Based on

characterization, the minimum and maximum values refer to sample tests and represent the

mean value plus or minus three times the standard deviation (mean±3Σ).

6.1.2

Typical values

Unless otherwise specified, typical data are based on T

= 25 °C, V

= 3.3 V (for the

A

DD

≤

≤

1.8 V

V

3.6 V voltage range). They are given only as design guidelines and are not

DD

tested.

Typical ADC accuracy values are determined by characterization of a batch of samples from

a standard diffusion lot over the full temperature range, where 95% of the devices have an

error less than or equal to the value indicated

(mean±2Σ)

.

6.1.3

Typical curves

Unless otherwise specified, all typical curves are given only as design guidelines and are

not tested.

6.1.4

Loading capacitor

The loading conditions used for pin parameter measurement are shown in Figure 19.

6.1.5

Pin input voltage

The input voltage measurement on a pin of the device is described in Figure 20.

Figure 19. Pin loading conditions

Figure 20. Pin input voltage

MCU pin

MCU pin

C = 50 pF

V

IN

MS19011V2

MS19010V2

DS8626 Rev 11

77/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

6.1.6

Power supply scheme

Figure 21. Power supply scheme

VBAT

VBAT =

1.65 to 3.6V

Backup circuitry

(OSC32K,RTC,

Power

Wakeup logic

switch

Backup registers,

backup RAM)

OUT

GPIOs

Logic

IN

VCAP_1

VCAP_2

2 × 2.2 μF

Kernel logic

(CPU, digital

& RAM)

VDD

VDD

1/2/...14/15

15 × 100 nF

VSS

regulator

+ 1 × 4.7 μF

1/2/...14/15

Voltage

IO

Level shifter

BYPASS_REG

Flash memory

VDD

Reset

PDR_ON

controller

VDDA

VREF

VREF+

Analog:

100 nF

100 nF

ADC

VREF-

+ 1 μF

+ 1 μF

RCs,

PLL,..

VSSA

MS19911V3

1.

Each power supply pair must be decoupled with filtering ceramic capacitors as shown above. These capacitors must be

placed as close as possible to, or below, the appropriate pins on the underside of the PCB to ensure the good functionality

of the device.

2.

To connect BYPASS_REG and PDR_ON pins, refer to Section 3.0.16: Voltage regulator and Table 3.0.15: Power supply

supervisor.

3.

The two 2.2 µF ceramic capacitors should be replaced by two 100 nF decoupling capacitors when the voltage regulator is

OFF.

pin.

4.

The 4.7 µF ceramic capacitor must be connected to one of the V

DD

=V

and V

=V

.

5.

V

DDA

DD

SSA

SS

78/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

6.1.7

Current consumption measurement

Figure 22. Current consumption measurement scheme

I

_V

DD

BAT

V

BAT

I

DD

V

DD

V

DDA

ai14126

6.2

Absolute maximum ratings

Stresses above the absolute maximum ratings listed in Table 11: Voltage characteristics,

Table 12: Current characteristics, and Table 13: Thermal characteristics may cause

permanent damage to the device. These are stress ratings only and functional operation of

the device at these conditions is not implied. Exposure to maximum rating conditions for

extended periods may affect device reliability. Device mission profile (application conditions)

is compliant with JEDEC JESD47 Qualification Standard, extended mission profiles are

available on demand.

Table 11. Voltage characteristics

Symbol

Ratings

Min

Max

Unit

(1)

V

–V

External main supply voltage (including V

,

V

)

–0.3

4.0

DD

SS

DDA

DD

(2)

Input voltage on five-volt tolerant pin

V

–0.3

V

+4

V

SS

DD

V

IN

Input voltage on any other pin

V

–0.3

4.0

SS

Δ

|

Variations between different V

power pins

-

50

|

V

DDx

DD

mV

Variations between all the different ground pins

−

|V

V

|

-

50

SSX

SS

including V

−

REF

see Section 6.3.14:

Absolute maximum

V

Electrostatic discharge voltage (human body model)

ESD(HBM)

ratings (electrical

sensitivity)

1.

All main power (V

, V

) and ground (V

, V

) pins must always be connected to the external power

DD

DDA

SS

SSA

supply, in the permitted range.

2.

V

maximum value must always be respected. Refer to Table 12 for the values of the maximum allowed

IN

injected current.

DS8626 Rev 11

79/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 12. Current characteristics

Symbol

Ratings

Max.

Unit

(1)

I

Total current into V

power lines (source)

240

VDD

DD

(1)

Total current out of V

ground lines (sink)

240

I

VSS

SS

Output current sunk by any I/O and control pin

25

I

IO

Output current source by any I/Os and control pin

25

mA

(3)

Injected current on five-volt tolerant I/O

–5/+0

(2)

I

INJ(PIN)

(4)

±5

Injected current on any other pin

(5)

(4)

Σ

I

Total injected current (sum of all I/O and control pins)

±25

INJ(PIN)

1.

All main power (V

, V

) and ground (V

, V

) pins must always be connected to the external power

DD

DDA

SS

SSA

supply, in the permitted range.

2.

Negative injection disturbs the analog performance of the device. See note in Section 6.3.21: 12-bit ADC

characteristics.

3.

Positive injection is not possible on these I/Os. A negative injection is induced by V

<V

. I

must

IN

SS

INJ(PIN)

never be exceeded. Refer to Table 11 for the values of the maximum allowed input voltage.

4.

A positive injection is induced by V

>V

while a negative injection is induced by V

<V

. I

must

IN

DD

IN

SS

INJ(PIN)

never be exceeded. Refer to Table 11 for the values of the maximum allowed input voltage.

5.

When several inputs are submitted to a current injection, the maximum ΣI

is the absolute sum of the

INJ(PIN)

positive and negative injected currents (instantaneous values).

Table 13. Thermal characteristics

Symbol

Ratings

Value

Unit

T

Storage temperature range

–65 to +150

°C

STG

Maximum junction temperature

125

°C

T

J

6.3

Operating conditions

6.3.1

General operating conditions

Table 14. General operating conditions

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

VOS bit in PWR_CR register = 0

0

-

144

f

Internal AHB clock frequency

HCLK

VOS bit in PWR_CR register= 1

0

-

168

MHz

f

Internal APB1 clock frequency

-

0

-

42

PCLK1

Internal APB2 clock frequency

-

0

-

84

f

PCLK2

(2)

V

Standard operating voltage

-

1.8

-

3.6

V

DD

Analog operating voltage

(2)

1.8

-

2.4

(ADC limited to 1.2 M samples)

Must be the same potential as

(3)(4)

V

V

(5)

DDA

V

Analog operating voltage

DD

2.4

-

3.6

(ADC limited to 1.4 M samples)

V

Backup operating voltage

-

1.65

-

3.6

V

BAT

80/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 14. General operating conditions (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

VOS bit in PWR_CR register = 0

1.08

1.14

1.20

V

Regulator ON:

Max frequency 144MHz

1.2 V internal voltage on

VOS bit in PWR_CR register= 1

/V

pins

V

1.20

1.26

1.32

V

CAP_1

CAP_2

Max frequency 168MHz

V

12

Max frequency 144MHz

1.10

1.14

1.20

V

Regulator OFF:

1.2 V external voltage must be

supplied from external regulator

Max frequency 168MHz

1.20

1.26

1.30

V

/V

pins

on V

CAP_1

CAP_2

2 V ≤ V

≤ 3.6 V

–0.3

-

5.5

Input voltage on RST and FT

DD

(6)

pins

≤ 2 V

–0.3

-

5.2

V

DD

V

V

V

+

IN

DDA

Input voltage on TTa pins

-

–0.3

-

0.3

Input voltage on B pin

-

-

-

5.5

LQFP64

-

-

435

LQFP100

-

-

465

Power dissipation at T

= 85 °C

LQFP144

-

-

500

A

P

for suffix 6 or T

= 105 °C for

mW

D

A

LQFP176

-

-

526

(7)

suffix 7

UFBGA176

-

-

513

WLCSP90

-

-

543

Maximum power dissipation

–40

-

85

Ambient temperature for 6 suffix

°C

version

(8)

–40

-

105

Low-power dissipation

T

A

Maximum power dissipation

–40

-

105

Ambient temperature for 7 suffix

°C

version

(8)

–40

-

125

Low-power dissipation

6 suffix version

–40

-

105

T

Junction temperature range

°C

J

7 suffix version

–40

-

125

1.

The average expected gain in power consumption when VOS = 0 compared to VOS = 1 is around 10% for the whole

temperature range, when the system clock frequency is between 30 and 144 MHz.

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use of

2.

V

DD

DDA

an external power supply supervisor (refer to Section : Internal reset OFF).

3.

When the ADC is used, refer to Table 67: ADC characteristics.

pin is present, it must respect the following condition: V

-V

< 1.2 V.

4.

If V

REF+

DDA

REF+

and V

from the same source. A maximum difference of 300 mV between V

and

5.

It is recommended to power V

DD

DDA

DD

V

can be tolerated during power-up and power-down operation.

DDA

+0.3, the internal pull-up and pull-down resistors must be disabled.

6.

To sustain a voltage higher than V

DD

is lower, higher P

values are allowed as long as T

does not exceed T

.

7.

If T

A

D

J

Jmax

can be extended to this range as long as T

does not exceed T

.

8.

In low-power dissipation state, T

A

J

Jmax

DS8626 Rev 11

81/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 15. Limitations depending on the operating power supply range

Maximum

Maximum flash

flash memory

memory access

Operating

Possible

Clock output

access

frequency

power

ADC

flash

I/O operation

Frequency on

frequency

with wait

supply

operation

memory

(1)

(2)

I/O pins

with no wait

states

range

operations

state

)

(f

Flashmax

– Degraded

8-bit erase

speed

Conversion

=1.8 to

160 MHz with 7

V

and program

(4)

DD

performance

time up to

20 MHz

up to 30 MHz

(3)

2.1 V

wait states

operations

1.2 Msps

– No I/O

only

compensation

– Degraded

speed

Conversion

16-bit erase

= 2.1 to

V

168 MHz with 7

DD

performance

time up to

22 MHz

up to 30 MHz

and program

2.4 V

wait states

1.2 Msps

operations

– No I/O

compensation

– Degraded

speed

Conversion

16-bit erase

performance

= 2.4 to

168 MHz with 6

V

DD

time up to

24 MHz

up to 48 MHz

and program

2.7 V

wait states

– I/O

2.4 Msps

operations

compensation

works

– up to

60 MHz

– Full-speed

=

when V

DD

operation

Conversion

32-bit erase

3.0 to 3.6 V

= 2.7 to

168 MHz with 5

V

DD

time up to

30 MHz

and program

– I/O

(5)

3.6 V

wait states

– up to

2.4 Msps

operations

compensation

48 MHz

works

=

when V

DD

2.7 to 3.0 V

1.

It applies only when code executed from flash memory access, when code executed from RAM, no wait state is required.

2.

Thanks to the ART accelerator and the 128-bit flash memory, the number of wait states given here does not impact the

execution speed from flash memory since the ART accelerator allows to achieve a performance equivalent to 0 wait state

program execution.

/VDDA minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use

3.

V

DD

of an external power supply supervisor (refer to Section : Internal reset OFF).

4.

Prefetch is not available. Refer to AN3430 application note for details on how to adjust performance and power.

5.

The voltage range for OTG USB FS can drop down to 2.7 V. However it is degraded between 2.7 and 3 V.

82/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

6.3.2

V

/V

external capacitor

CAP_1

CAP_2

Stabilization for the main regulator is achieved by connecting an external capacitor C

to

EXT

the V

/V

pins. C

is specified in Table 16.

CAP_1

CAP_2

EXT

Figure 23. External capacitor C

EXT

C

ESR

R

Leak

MS19044V2

1.

Legend: ESR is the equivalent series resistance.

(1)

Table 16. V

/V

operating conditions

CAP_1

CAP_2

Symbol

Parameter

Conditions

CEXT

Capacitance of external capacitor

2.2 µF

Ω

ESR

ESR of external capacitor

< 2

1.

When bypassing the voltage regulator, the two 2.2 µF V

capacitors are not required and should be

CAP

replaced by two 100 nF decoupling capacitors.

6.3.3

Operating conditions at power-up / power-down (regulator ON)

Subject to general operating conditions for T

.

A

Table 17. Operating conditions at power-up / power-down (regulator ON)

Symbol

Parameter

Min

Max

Unit

V

rise time rate

20

∞

DD

t

µs/V

VDD

fall time rate

20

∞

V

DD

6.3.4

Operating conditions at power-up / power-down (regulator OFF)

Subject to general operating conditions for T

.

A

(1)

Table 18. Operating conditions at power-up / power-down (regulator OFF)

Symbol

Parameter

Conditions

Min

Max

Unit

V

rise time rate

Power-up

20

∞

DD

t

VDD

V

fall time rate

Power-down

20

∞

DD

V

and V

rise time

µs/V

CAP_1

CAP_2

Power-up

20

∞

rate

t

VCAP

V

and V

fall time

CAP_1

CAP_2

Power-down

20

∞

rate

1.

To reset the internal logic at power-down, a reset must be applied on pin PA0 when V

reach below

DD

minimum value of V

.

12

DS8626 Rev 11

83/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

6.3.5

Embedded reset and power control block characteristics

The parameters given in Table 19 are derived from tests performed under ambient

temperature and V

supply voltage conditions summarized in Table 14.

DD

Table 19. Embedded reset and power control block characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

PLS[2:0]=000 (rising

2.09

2.14

2.19

V

edge)

PLS[2:0]=000 (falling

1.98

2.04

2.08

V

edge)

PLS[2:0]=001 (rising

2.23

2.30

2.37

V

edge)

PLS[2:0]=001 (falling

2.13

2.19

2.25

V

edge)

PLS[2:0]=010 (rising

2.39

2.45

2.51

V

edge)

PLS[2:0]=010 (falling

2.29

2.35

2.39

V

edge)

PLS[2:0]=011 (rising edge)

2.54

2.60

2.65

V

PLS[2:0]=011 (falling

2.44

2.51

2.56

V

Programmable voltage

edge)

V

PVD

detector level selection

PLS[2:0]=100 (rising

2.70

2.76

2.82

V

edge)

PLS[2:0]=100 (falling

2.59

2.66

2.71

V

edge)

PLS[2:0]=101 (rising

2.86

2.93

2.99

V

edge)

PLS[2:0]=101 (falling

2.75

2.84

2.92

V

edge)

PLS[2:0]=110 (rising edge)

2.96

3.03

3.10

V

PLS[2:0]=110 (falling

2.85

2.93

2.99

V

edge)

PLS[2:0]=111 (rising edge)

3.07

3.14

3.21

V

PLS[2:0]=111 (falling

2.95

3.03

3.09

V

edge)

(1)

PVD hysteresis

-

-

100

-

mV

V

PVDhyst

Falling edge

1.60

1.68

1.76

V

Power-on/power-down

V

POR/PDR

reset threshold

Rising edge

1.64

1.72

1.80

V

(1)

PDR hysteresis

-

-

40

-

mV

V

PDRhyst

Falling edge

2.13

2.19

2.24

V

Brownout level 1

V

BOR1

threshold

Rising edge

2.23

2.29

2.33

V

84/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 19. Embedded reset and power control block characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Falling edge

2.44

2.50

2.56

V

Brownout level 2

V

BOR2

threshold

Rising edge

2.53

2.59

2.63

V

Falling edge

2.75

2.83

2.88

V

Brownout level 3

V

BOR3

threshold

Rising edge

2.85

2.92

2.97

V

(1)

BOR hysteresis

-

-

100

-

mV

V

BORhyst

(1)(2)

Reset temporization

-

0.5

1.5

3.0

ms

T

RSTTEMPO

InRush current on

voltage regulator

(1)

I

-

-

160

200

mA

RUSH

power-on (POR or

wakeup from Standby)

InRush energy on

voltage regulator

V

= 1.8 V, T

= 105 °C,

(1)

DD

A

-

-

5.4

µC

E

RUSH

= 171 mA for 31 µs

power-on (POR or

I

RUSH

wakeup from Standby)

1.

Specified by design.

) to the instant

2.

The reset temporization is measured from the power-on (POR reset or wakeup from V

BAT

when first instruction is read by the user application code.

6.3.6

Supply current characteristics

The current consumption is a function of several parameters and factors such as the

operating voltage, ambient temperature, I/O pin loading, device software configuration,

operating frequencies, I/O pin switching rate, program location in memory and executed

binary code.

The current consumption is measured as described in Figure 22: Current consumption

measurement scheme.

All Run mode current consumption measurements given in this section are performed using

a CoreMark-compliant code.

Typical and maximum current consumption

The MCU is placed under the following conditions:

•

At startup, all I/O pins are configured as analog inputs by firmware.

•

All peripherals are disabled except if it is explicitly mentioned.

•

The flash memory access time is adjusted to f

frequency (0 wait state from 0 to

HCLK

30 MHz, 1 wait state from 30 to 60 MHz, 2 wait states from 60 to 90 MHz, 3 wait states

from 90 to 120 MHz, 4 wait states from 120 to 150 MHz, and 5 wait states from 150 to

168 MHz).

•

When the peripherals are enabled HCLK is the system clock, f

= f

/4, and

PCLK1

HCLK

f

= f

/2, except is explicitly mentioned.

PCLK2

HCLK

•

The maximum values are obtained for V

= 3.6 V and maximum ambient temperature

DD

(T

), and the typical values for T

= 25 °C and V

= 3.3 V unless otherwise specified.

A

A

DD

DS8626 Rev 11

85/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 20. Typical and maximum current consumption in Run mode, code with data processing

(1)

running from flash memory (ART accelerator enabled) or RAM

(2)

Typ

Max

Symbol

Parameter

Conditions

f

Unit

HCLK

T

=

T

=

T

=

A

A

A

25 °C

85 °C

105 °C

168 MHz

87

102

109

144 MHz

67

80

86

120 MHz

56

69

75

90 MHz

44

56

62

60 MHz

30

42

49

(3)

, all

External clock

30 MHz

16

28

35

(4)(5)

peripherals enabled

25 MHz

12

24

31

(6)

16 MHz

9

20

28

8 MHz

5

17

24

4 MHz

3

15

22

2 MHz

2

14

21

Supply current in

mA

I

DD

Run mode

168 MHz

40

54

61

144 MHz

31

43

50

120 MHz

26

38

45

90 MHz

20

32

39

60 MHz

14

26

33

(3)

External clock

, all

30 MHz

8

20

27

(4)(5)

peripherals disabled

25 MHz

6

18

25

(6)

5

16

24

16 MHz

8 MHz

3

15

22

4 MHz

2

14

21

2 MHz

2

14

21

1.

Code and data processing running from SRAM1 using boot pins.

max and f

max with peripherals enabled.

2.

Evaluated by characterization, tested in production at V

DD

HCLK

> 25 MHz.

3.

External clock is 4 MHz and PLL is on when f

HCLK

4.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of 1.6 mA per ADC for

the analog part.

5.

When analog peripheral blocks such as ADCs, DACs, HSE, LSE, HSI, or LSI are ON, an additional power consumption

should be considered.

6.

In this case HCLK = system clock/2.

86/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 21. Typical and maximum current consumption in Run mode, code with data processing

running from flash memory (ART accelerator disabled)

(1)

Typ

Max

Symbol

Parameter

Conditions

f

Unit

HCLK

= 25 °C T

= 85 °C T

= 105 °C

T

A

A

A

168 MHz

93

109

117

144 MHz

76

89

96

120 MHz

67

79

86

90 MHz

53

65

73

60 MHz

37

49

56

(2)

External clock

,

all peripherals

30 MHz

20

32

39

(3)(4)

enabled

25 MHz

16

27

35

16 MHz

11

23

30

8 MHz

6

18

25

4 MHz

4

16

23

2 MHz

3

15

22

Supply current

mA

I

DD

in Run mode

168 MHz

46

61

69

144 MHz

40

52

60

120 MHz

37

48

56

90 MHz

30

42

50

60 MHz

22

33

41

(2)

,

External clock

all peripherals

30 MHz

12

24

31

(3)(4)

disabled

25 MHz

10

21

29

16 MHz

7

19

26

8 MHz

4

16

23

4 MHz

3

15

22

2 MHz

2

14

21

1.

Evaluated by characterization, tested in production at V

max and f

max with peripherals enabled.

DD

HCLK

> 25 MHz.

2.

External clock is 4 MHz and PLL is on when f

HCLK

3.

When analog peripheral blocks such as (ADCs, DACs, HSE, LSE, HSI,LSI) are on, an additional power consumption

should be considered.

4.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of 1.6 mA per ADC

for the analog part.

DS8626 Rev 11

87/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 24. Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator ON) or RAM, and peripherals OFF

50

45

40

35

-45 °C

30

0 °C

25

(mA)

25 °C

20

55 °C

DD RUN

I

85 °C

15

105 °C

10

5

0

0

20

40

60

80

100

120

140

160

180

CPU Frequency (MHz

MS19974V1

Figure 25. Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator ON) or RAM, and peripherals ON

100

90

80

70

-45°C

60

0°C

50

(mA)

25°C

40

55°C

DD RUN

I

85°C

30

105°C

20

10

0

0

20

40

60

80

100

120

140

160

180

CPU Frequency (MHz

MS19975V1

88/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 26. Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator OFF) or RAM, and peripherals OFF

60

50

40

-45°C

0°C

30

(mA)

25°C

55°C

DD RUN

I

20

85°C

105°C

10

0

0

20

40

60

80

100

120

140

160

180

CPU Frequency (MHz

MS19976V1

Figure 27. Typical current consumption versus temperature, Run mode, code with data

processing running from Flash (ART accelerator OFF) or RAM, and peripherals ON

120

100

80

-45°C

0°C

60

(mA)

25°C

55°C

DD RUN

I

40

85°C

105°C

20

0

0

20

40

60

80

100

120

140

160

180

CPU Frequency (MHz

MS19977V1

DS8626 Rev 11

89/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 22. Typical and maximum current consumption in Sleep mode

(1)

Typ

Max

Symbol

Parameter

Conditions

f

Unit

HCLK

T

=

T

=

T

=

A

A

A

25 °C

85 °C

105 °C

168 MHz

59

77

84

144 MHz

46

61

67

120 MHz

38

53

60

90 MHz

30

44

51

60 MHz

20

34

41

(2)

,

External clock

30 MHz

11

24

31

(3)

all peripherals enabled

25 MHz

8

21

28

16 MHz

6

18

25

8 MHz

3

16

23

4 MHz

2

15

22

2 MHz

2

14

21

Supply current in

mA

I

DD

Sleep mode

168 MHz

12

27

35

144 MHz

9

22

29

120 MHz

8

20

28

90 MHz

7

19

26

60 MHz

5

17

24

(2)

External clock

, all

30 MHz

3

16

23

peripherals disabled

25 MHz

2

15

22

16 MHz

2

14

21

8 MHz

1

14

21

4 MHz

1

13

21

2 MHz

1

13

21

1.

Evaluated by characterization, tested in production at V

max and f

max with peripherals enabled.

DD

HCLK

> 25 MHz.

2.

External clock is 4 MHz and PLL is on when f

HCLK

3.

Add an additional power consumption of 1.6 mA per ADC for the analog part. In applications, this consumption occurs only

while the ADC is ON (ADON bit is set in the ADC_CR2 register).

90/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 23. Typical and maximum current consumptions in Stop mode

Typ

Max

Symbol

Parameter

Conditions

Unit

T

=

T

=

T

=

T

=

A

A

A

A

25 °C

25 °C

85 °C

105 °C

Flash in Stop mode, low-speed and high-

Supply

speed internal RC oscillators and high-speed

0.45

1.5

11.00

20.00

current in

oscillator OFF (no independent watchdog)

Stop mode

Flash in Deep power-down mode, low-speed

with main

and high-speed internal RC oscillators and

regulator in

0.40

1.5

11.00

20.00

high-speed oscillator OFF (no independent

Run mode

watchdog)

I

mA

DD_STOP

Flash in Stop mode, low-speed and high-

Supply

speed internal RC oscillators and high-speed

0.31

1.1

8.00

15.00

current in

oscillator OFF (no independent watchdog)

Stop mode

with main

Flash in Deep power-down mode, low-speed

regulator in

and high-speed internal RC oscillators and

0.28

1.1

8.00

15.00

Low-power

high-speed oscillator OFF (no independent

mode

watchdog)

Table 24. Typical and maximum current consumptions in Standby mode

(1)

Typ

Max

T

=

T

=

A

A

= 25 °C

T

A

Symbol

Parameter

Conditions

85 °C

105 °C

Unit

V

=

V

=

V

=

DD

DD

DD

V

= 3.6 V

DD

1.8 V

2.4 V

3.3 V

Backup SRAM ON, low-

3.0

3.4

4.0

20

36

speed oscillator and RTC ON

Backup SRAM OFF, low-

2.4

2.7

3.3

16

32

Supply current

speed oscillator and RTC ON

I

in Standby

µA

DD_STBY

Backup SRAM ON, RTC

mode

2.4

2.6

3.0

12.5

24.8

OFF

Backup SRAM OFF, RTC

1.7

1.9

2.2

9.8

19.2

OFF

1.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

91/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 25. Typical and maximum current consumptions in V

mode

BAT

(1)

Typ

Max

=

T

=

T

A

A

T

= 25 °C

A

85 °C

105 °C

Symbol

Parameter

Conditions

Unit

V

V

BAT

BAT

=

V

BAT

= 3.6 V

=

=

V

BAT

2.4 V

1.8 V

3.3 V

Backup SRAM ON, low-speed

1.29

1.42

1.68

6

11

oscillator and RTC ON

Backup

Backup SRAM OFF, low-speed

I

domain

0.62

0.73

0.96

3

5

DD_VBA

µA

oscillator and RTC ON

supply

T

current

Backup SRAM ON, RTC OFF

0.79

0.81

0.86

5

10

Backup SRAM OFF, RTC OFF

0.10

0.10

0.10

2

4

1.

Evaluated by characterization - not tested in production.

Figure 28. Typical V

current consumption (LSE and RTC ON/backup RAM OFF)

BAT

3.5

3

2.5

1.65V

1.8V

2

2V

2.4V

1.5

2.7V

IVBAT in (μA)

3V

3.3V

1

3.6V

0.5

0

0

10

20

30

40

50

60

70

80

90

100

Temperature in (°C)

MS19990V1

92/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 29. Typical V

current consumption (LSE and RTC ON/backup RAM ON)

BAT

6

5

4

1.65V

1.8V

2V

3

2.4V

2.7V

IVBAT in (μA)

3V

2

3.3V

3.6V

1

0

0

10

20

30

40

50

60

70

80

90

100

Temperature in (°C)

MS19991V1

Additional current consumption

The MCU is placed under the following conditions:

•

All I/O pins are configured in analog mode.

•

The flash memory access time is adjusted to f

frequency.

HCLK

•

The voltage scaling is adjusted to f

frequency as follows:

HCLK

–

Scale 2 for f

≤

144 MHz

HCLK

–

Scale 1 for 144 MHz < f

≤ 168 MHz.

HCLK

•

The system clock is HCLK, f

= f

/4, and f

= f

/2.

PCLK1

HCLK

PCLK2

HCLK

•

The HSE crystal clock frequency is 25 MHz.

•

T

= 25 °C.

A

DS8626 Rev 11

93/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 26. Typical current consumption in Run mode, code with data processing

running from flash memory, regulator ON (ART accelerator enabled

(1)

except prefetch), V

= 1.8 V

DD

=

Typ. at T

A

Unit

Symbol

Parameter

Conditions

f

(MHz)

HCLK

25 °C

160

36.2

144

29.3

120

24.7

Supply current in

All peripheral

IDD

90

19.3

mA

Run mode

disabled

60

13.4

30

7.7

25

6.0

1.

When peripherals are enabled, the power consumption corresponding to the analog part of the peripherals

(such as ADC or DAC) is not included.

I/O system current consumption

The current consumption of the I/O system has two components: static and dynamic.

I/O static current consumption

All the I/Os used as input with pull-up or pull-down generate current consumption when the

pin is externally held to the opposite level. The value of this current consumption can be

simply computed by using the pull-up/pull-down resistors values given in Table 48: I/O static

characteristics.

For the output pins, any internal or external pull-up or pull-down and external load must also

be considered to estimate the current consumption.

Additional I/O current consumption is due to I/Os configured as inputs if an intermediate

voltage level is externally applied. This current consumption is caused by the input Schmitt

trigger circuits used to discriminate the input value. Unless this specific configuration is

required by the application, this supply current consumption can be avoided by configuring

these I/Os in analog mode. This is notably the case of ADC input pins which should be

configured as analog inputs.

Caution:

Any floating input pin can also settle to an intermediate voltage level or switch inadvertently,

as a result of external electromagnetic noise. To avoid current consumption related to

floating pins, they must either be configured in analog mode, or forced internally to a definite

digital value. This can be done either by using pull-up/down resistors or by configuring the

pins in output mode.

I/O dynamic current consumption

In addition to the internal peripheral current consumption measured previously (see

Table 28: Peripheral current consumption), the I/Os used by an application also contribute

to the current consumption. When an I/O pin switches, it uses the current from the MCU

supply voltage to supply the I/O pin circuitry and to charge/discharge the capacitive load

internal and external connected to the pin:

×

×

I

V

f

C

=

SW

DD

SW

94/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

where

I

is the current sunk by a switching I/O to charge/discharge the capacitive load

SW

V

is the MCU supply voltage

DD

f

is the I/O switching frequency

SW

C is the total capacitance seen by the I/O pin: C = C

+ C

INT

EXT

The test pin is configured in push-pull output mode and is toggled by software at a fixed

frequency.

DS8626 Rev 11

95/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 27. Switching output I/O current consumption

I/O toggling

(1)

Symbol

Parameter

Conditions

Typ

Unit

frequency (f

)

SW

2 MHz

0.02

8 MHz

0.14

(2)

V

= 3.3 V

DD

25 MHz

0.51

C = C

INT

50 MHz

0.86

60 MHz

1.30

2 MHz

0.10

8 MHz

0.38

= 3.3 V

V

DD

= 0 pF

25 MHz

1.18

C

EXT

C = C

+ C

+ C

INT

EXT

S

50 MHz

2.47

60 MHz

2.86

2 MHz

0.17

8 MHz

0.66

V

= 3.3 V

I/O switching

DD

mA

I

DDIO

= 10 pF

25 MHz

1.70

C

current

EXT

C = C

+ C

+ C

INT

EXT

S

50 MHz

2.65

60 MHz

3.48

2 MHz

0.23

8 MHz

0.95

= 3.3 V

V

DD

C

= 22 pF

25 MHz

3.20

EXT

+ C

+ C

C = C

INT

EXT

S

50 MHz

4.69

60 MHz

8.06

2 MHz

0.30

8 MHz

1.22

= 3.3 V

V

DD

25 MHz

3.90

= 33 pF

C

EXT

+ C

+ C

C = C

INT

EXT

S

50 MHz

8.82

(3)

60 MHz

-

1.

C

is the PCB board capacitance including the pad pin. C

= 7 pF (estimated value).

S

S

2.

This test is performed by cutting the LQFP package pin (pad removal).

3.

At 60 MHz, C maximum load is specified 30 pF.

96/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

On-chip peripheral current consumption

The current consumption of the on-chip peripherals is given in Table 28. The MCU is placed

under the following conditions:

•

At startup, all I/O pins are configured as analog pins by firmware.

•

All peripherals are disabled unless otherwise mentioned

•

The code is running from flash memory and the flash memory access time is equal to 5

wait states at 168 MHz.

•

The code is running from flash memory and the flash memory access time is equal to 4

wait states at 144 MHz, and the power scale mode is set to 2.

•

The ART accelerator is ON.

•

The given value is calculated by measuring the difference of current consumption

–

with all peripherals clocked off

–

with one peripheral clocked on (with only the clock applied)

•

When the peripherals are enabled: HCLK is the system clock, f

= f

/4, and

PCLK1

HCLK

f

= f

/2.

PCLK2

HCLK

•

The typical values are obtained for V

= 3.3 V and T

= 25 °C, unless otherwise

DD

A

specified.

Table 28. Peripheral current consumption

(1)

I

(Typ)

DD

Peripheral

Unit

Scale1

Scale2

(up t 168 MHz)

(up to 144 MHz)

GPIOA

2.70

2.40

GPIOB

2.50

2.22

GPIOC

2.54

2.28

GPIOD

2.55

2.28

GPIOE

2.68

2.40

GPIOF

2.53

2.28

GPIOG

2.51

2.22

GPIOH

2.51

2.22

AHB1

GPIOI

2.50

2.22

µA/MHz

(up to 168 MHz)

OTG_HS+ULPI

28.33

25.38

CRC

0.41

0.40

BKPSRAM

0.63

0.58

DMA1

37.44

33.58

DMA2

37.69

33.93

ETH_MAC

ETH_MAC_TX

20.43

18.39

ETH_MAC_RX

ETH_MAC_PTP

DS8626 Rev 11

97/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 28. Peripheral current consumption (continued)

(1)

I

(Typ)

DD

Peripheral

Unit

Scale1

Scale2

(up t 168 MHz)

(up to 144 MHz)

OTG_FS

26.45

26.67

AHB2

DCMI

5.87

5.35

µA/MHz

(up to 168 MHz)

RNG

1.50

1.67

AHB3

FSMC

12.46

11.31

µA/MHz

(up to 168 MHz)

(2)

13.10

11.81

µA/MHz

Bus matrix

TIM2

16.71

16.50

TIM3

12.33

11.94

TIM4

13.45

12.92

TIM5

17.14

16.58

TIM6

2.43

3.06

TIM7

2.43

2.22

TIM12

6.62

6.83

TIM13

5.05

5.47

TIM14

5.26

5.61

PWR

1.00

0.56

USART2

2.69

2.78

USART3

2.74

2.78

APB1

µA/MHz

UART4

3.24

3.33

(up to 42 MHz)

UART5

2.69

2.78

I2C1

2.67

2.50

I2C2

2.83

2.78

I2C3

2.81

2.78

SPI2

2.43

2.22

SPI3

2.43

2.22

(3)

2.43

2.22

I2S2

(3)

2.26

2.22

I2S3

CAN1

5.12

5.56

CAN2

4.81

5.28

(4)

1.67

1.67

DAC

WWDG

1.00

0.83

98/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 28. Peripheral current consumption (continued)

(1)

I

(Typ)

DD

Peripheral

Unit

Scale1

Scale2

(up t 168 MHz)

(up to 144 MHz)

SDIO

7.08

7.92

TIM1

16.79

15.51

TIM8

17.88

16.53

TIM9

7.64

7.28

TIM10

4.89

4.82

TIM11

5.19

4.82

APB2

(5)

4.67

4.58

ADC1

µA/MHz

(up to 84 MHz)

(5)

4.67

4.58

ADC2

(5)

4.43

4.44

ADC3

SPI1

1.32

1.39

USART1

3.51

3.72

USART6

3.55

3.75

SYSCFG

0.74

0.56

1.

When the I/O compensation cell is ON, I

typical value increases by 0.22 mA.

DD

2.

The BusMatrix is automatically active when at least one master is ON.

3.

To enable an I2S peripheral, first set the I2SMOD bit and then the I2SE bit in the SPI_I2SCFGR register.

4.

When the DAC is ON and EN1/2 bits are set in DAC_CR register, add an additional power consumption of

0.8 mA per DAC channel for the analog part.

5.

When the ADC is ON (ADON bit set in the ADC_CR2 register), add an additional power consumption of

1.6 mA per ADC for the analog part.

6.3.7

Wakeup time from low-power mode

The wakeup times given in Table 29 is measured on a wakeup phase with a 16 MHz HSI

RC oscillator. The clock source used to wake up the device depends from the current

operating mode:

•

Stop or Standby mode: the clock source is the RC oscillator

•

Sleep mode: the clock source is the clock that was set before entering Sleep mode.

All timings are derived from tests performed under ambient temperature and V

supply

DD

voltage conditions summarized in Table 14.

DS8626 Rev 11

99/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 29. Low-power mode wakeup timings

(1)

(1)

(1)

Symbol

Parameter

Min

Typ

Max

Unit

CPU

(2)

t

Wakeup from Sleep mode

-

5

-

clock

WUSLEEP

cycle

Wakeup from Stop mode (regulator in Run mode and

-

13

-

flash memory in Stop mode)

Wakeup from Stop mode (regulator in low-power mode

-

17

40

and flash memory in Stop mode)

(2)

t

µs

WUSTOP

Wakeup from Stop mode (regulator in Run mode and

-

105

-

flash memory in Deep power-down mode)

Wakeup from Stop mode (regulator in low-power mode

-

110

-

and flash memory in Deep power-down mode)

(2)(3)

Wakeup from Standby mode

260

375

480

µs

t

WUSTDBY

1.

Evaluated by characterization - not tested in production.

2.

The wakeup times are measured from the wakeup event to the point in which the application code reads the first instruction.

minimum and maximum values are given at 105 °C and –45 °C, respectively.

3.

t

WUSTDBY

6.3.8

External clock source characteristics

High-speed external user clock generated from an external source

The characteristics given in Table 30 result from tests performed using an high-speed

external clock source, and under ambient temperature and supply voltage conditions

summarized in Table 14.

Table 30. High-speed external user clock characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

External user clock source

f

1

-

50

MHz

(1)

HSE_ext

frequency

V

OSC_IN input pin high level voltage

0.7V

-

V

HSEH

DD

DD

V

V

OSC_IN input pin low level voltage

V

-

0.3V

HSEL

SS

DD

-

t

(1)

w(HSE)

OSC_IN high or low time

5

-

-

t

w(HSE)

ns

t

(1)

r(HSE)

OSC_IN rise or fall time

-

-

10

t

f(HSE)

(1)

C

OSC_IN input capacitance

-

-

5

-

pF

in(HSE)

DuCy

Duty cycle

-

45

-

55

%

(HSE)

≤

≤

OSC_IN Input leakage current

V

V

V

-

-

±1

µA

I

L

SS

IN

DD

1.

Specified by design.

100/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Low-speed external user clock generated from an external source

The characteristics given in Table 31 result from tests performed using an low-speed

external clock source, and under ambient temperature and supply voltage conditions

summarized in Table 14.

Table 31. Low-speed external user clock characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

User External clock source

f

-

32.768

1000

kHz

(1)

LSE_ext

frequency

OSC32_IN input pin high level

V

-

V

0.7V

LSEH

DD

DD

voltage

V

V

OSC32_IN input pin low level voltage

-

V

-

0.3V

LSEL

SS

DD

t

(1)

w(LSE)

OSC32_IN high or low time

450

-

-

t

f(LSE)

ns

t

(1)

r(LSE)

OSC32_IN rise or fall time

-

-

50

t

f(LSE)

(1)

C

OSC32_IN input capacitance

-

-

5

-

pF

in(LSE)

Duty cycle

-

30

-

70

%

DuCy

(LSE)

≤

≤

I

OSC32_IN Input leakage current

V

V

V

-

-

±1

µA

L

SS

IN

DD

1.

Specified by design.

Figure 30. High-speed external clock source AC timing diagram

V

HSEH

90 %

10

%

V

HSEL

t

t

f(HSE)

W(HSE)

t

t

t

W(HSE)

r(HSE)

T

HSE

External

f

HSE_ext

I

L

clock source

OSC_IN

STM32F

ai17528

DS8626 Rev 11

101/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 31. Low-speed external clock source AC timing diagram

V

LSEH

90%

10%

V

LSEL

t

t

f(LSE)

W(LSE)

t

t

t

W(LSE)

r(LSE)

T

LSE

External

f

LSE_ext

I

L

OSC32_IN

clock source

STM32F

ai17529

High-speed external clock generated from a crystal/ceramic resonator

The high-speed external (HSE) clock can be supplied with a 4 to 26 MHz crystal/ceramic

resonator oscillator. All the information given in this paragraph are based on

characterization results obtained with typical external components specified in Table 32. In

the application, the resonator and the load capacitors have to be placed as close as

possible to the oscillator pins in order to minimize output distortion and startup stabilization

time. Refer to the crystal resonator manufacturer for more details on the resonator

characteristics (frequency, package, accuracy).

(1)

Table 32. HSE 4-26 MHz oscillator characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Oscillator frequency

-

4

-

26

MHz

OSC_IN

Ω

Feedback resistor

-

-

200

-

k

R

F

G

Oscillator transconductance

5

-

-

m

Startup

mA/V

Maximum critical crystal G

-

-

1

G

mcritmax

m

(2)

Startup time

V

is stabilized

-

2

-

ms

t

SU(HSE)

DD

1.

Specified by design.

2.

Evaluated by characterization - not tested in production. t

is the startup time measured from the

SU(HSE)

moment it is enabled (by software) to a stabilized 8 MHz oscillation is reached. This value is measured for

a standard crystal resonator and can vary significantly with the crystal manufacturer

For C

and C

, it is recommended to use high-quality external ceramic capacitors in the

L1

L2

5 pF to 25 pF range (typ.), designed for high-frequency applications, and selected to match

the requirements of the crystal or resonator (see Figure 32). C

and C

are usually the

L1

L2

same size. The crystal manufacturer typically specifies a load capacitance which is the

series combination of C

and C

. PCB and MCU pin capacitance must be included (10 pF

L1

L2

can be used as a rough estimate of the combined pin and board capacitance) when sizing

C

and C

.

L1

L2

102/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Note:

For information on electing the crystal, refer to the application note AN2867 “Oscillator

design guide for ST microcontrollers” available from the ST website www.st.com.

Figure 32. Typical application with an 8 MHz crystal

Resonator with

integrated capacitors

C

L1

f

OSC_IN

HSE

Bias

8 MHz

R

controlled

F

resonator

gain

OSC_OUT

(1)

STM32F

R

EXT

C

L2

ai17530

1.

R

value depends on the crystal characteristics.

EXT

Low-speed external clock generated from a crystal/ceramic resonator

The low-speed external (LSE) clock can be supplied with a 32.768 kHz crystal/ceramic

resonator oscillator. All the information given in this paragraph are based on

characterization results obtained with typical external components specified in Table 33. In

the application, the resonator and the load capacitors have to be placed as close as

possible to the oscillator pins in order to minimize output distortion and startup stabilization

time. Refer to the crystal resonator manufacturer for more details on the resonator

characteristics (frequency, package, accuracy).

(1)

Table 33. LSE oscillator characteristics (f

= 32.768 kHz)

LSE

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Oscillator frequency

-

-

32.768

-

kHz

f

OSC_IN

Ω

R

Feedback resistor

-

-

18.4

-

M

F

LSE current consumption

-

-

-

1

µA

I

DD

Oscillator transconductance

2.8

-

-

G

m

Startup

µA/V

Maximum critical crystal G

-

-

0.56

G

mcritmax

m

(2)

Startup time

V

is stabilized

-

2

-

s

t

SU(LSE)

DD

1.

Specified by design.

2.

Evaluated by characterization - not tested in production. t

is the startup time measured from the

SU(LSE)

moment it is enabled (by software) to a stabilized 32.768 kHz oscillation is reached. This value is

measured for a standard crystal resonator and it can vary significantly with the crystal manufacturer

Note:

For information on electing the crystal, refer to the application note AN2867 “Oscillator

design guide for ST microcontrollers” available from the ST website www.st.com.

DS8626 Rev 11

103/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 33. Typical application with a 32.768 kHz crystal

Resonator with

integrated capacitors

C

L1

32.768 kHz

L2

f

OSC32_IN

LSE

R

F

resonator

OSC32_OUT

Bias

controlled

gain

STM32F

C

ai17531

6.3.9

Internal clock source characteristics

The parameters given in Table 34 and Table 35 are  derived from tests performed under

ambient temperature and V

supply voltage conditions summarized in Table 14.

DD

High-speed internal (HSI) RC oscillator

(1)

Table 34. HSI oscillator characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Frequency

-

-

16

-

MHz

HSI

(2)

HSI user trimming step

-

-

-

1

%

(3)

= –40 to 105 °C

–8

-

4.5

%

T

A

ACC

HSI

(3)

T

Accuracy of the HSI oscillator

= –10 to 85 °C

–4

-

4

%

A

(4)

= 25 °C

–1

-

1

%

T

A

(2)

HSI oscillator startup time

-

-

2.2

4

µs

t

su(HSI)

HSI oscillator power

(2)

I

-

-

60

80

µA

DD(HSI)

consumption

= 3.3 V, PLL OFF, T

= –40 to 125 °C unless otherwise specified.

1.

V

DD

A

2.

Specified by design.

3.

Evaluated by characterization - not tested in production.

4.

Factory calibrated, parts not soldered.

Low-speed internal (LSI) RC oscillator

(1)

Table 35. LSI oscillator characteristics

Symbol

Parameter

Min

Typ

Max

Unit

(2)

f

Frequency

17

32

47

kHz

LSI

(3)

LSI oscillator startup time

-

15

40

µs

t

su(LSI)

(3)

I

LSI oscillator power consumption

-

0.4

0.6

µA

DD(LSI)

1.

V

= 3 V, T

= –40 to 105 °C unless otherwise specified.

DD

A

2.

Evaluated by characterization - not tested in production.

3.

Specified by design.

104/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 34. ACC

versus temperature

LSI

50

max

40

avg

min

30

20

10

0

-10

Normalized deviation (%)

-20

-30

-40

-45

-35

-25

-15

-5

5

15

25

35

45

55

65

75

85

95

105

Temperature (°C)

MS19013V1

6.3.10

PLL characteristics

The parameters given in Table 36 and Table 37 are derived from tests performed under

temperature and V

supply voltage conditions summarized in Table 14.

DD

Table 36. Main PLL characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

(2)

f

PLL input clock

-

0.95

1

2.10

MHz

PLL_IN

PLL multiplier output clock

-

24

-

168

MHz

f

PLL_OUT

48 MHz PLL multiplier output

f

-

-

48

75

MHz

PLL48_OUT

clock

PLL VCO output

-

100

-

432

MHz

f

VCO_OUT

VCO freq = 100 MHz

75

-

200

t

PLL lock time

µs

LOCK

VCO freq = 432 MHz

100

-

300

DS8626 Rev 11

105/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 36. Main PLL characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

RMS

-

25

-

peak

Cycle-to-cycle jitter

±

to

-

150

-

peak

System clock

120 MHz

RMS

-

15

-

peak

Period Jitter

±

to

-

200

-

(3)

Jitter

ps

peak

Main clock output (MCO) for

Cycle to cycle at 50 MHz

-

32

-

RMII Ethernet

on 1000 samples

Main clock output (MCO) for MII

Cycle to cycle at 25 MHz

-

40

-

Ethernet

on 1000 samples

Cycle to cycle at 1 MHz

Bit Time CAN jitter

-

330

-

on 1000 samples

VCO freq = 100 MHz

0.15

0.40

(4)

PLL power consumption on VDD

-

mA

I

DD(PLL)

VCO freq = 432 MHz

0.45

0.75

VCO freq = 100 MHz

0.30

0.40

PLL power consumption on

(4)

I

-

mA

DDA(PLL)

VDDA

VCO freq = 432 MHz

0.55

0.85

1.

Take care of using the appropriate division factor M to obtain the specified PLL input clock values. The M factor is shared

between PLL and PLLI2S.

2.

Specified by design.

3.

The use of 2 PLLs in parallel could degraded the Jitter up to +30%.

4.

Evaluated by characterization - not tested in production.

Table 37. PLLI2S (audio PLL) characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

(2)

f

PLLI2S input clock

-

0.95

1

2.10

MHz

PLLI2S_IN

PLLI2S multiplier output clock

-

-

-

216

MHz

f

PLLI2S_OUT

f

PLLI2S VCO output

-

100

-

432

MHz

VCO_OUT

VCO freq = 100 MHz

75

-

200

t

PLLI2S lock time

µs

LOCK

VCO freq = 432 MHz

100

-

300

RMS

-

90

-

Cycle to cycle at

12.288 MHz on

peak

48KHz period,

±

to

-

280

-

ps

N=432, R=5

peak

2

Master I

S clock jitter

Average frequency of

(3)

Jitter

12.288 MHz

-

90

-

ps

N = 432, R = 5

on 1000 samples

Cycle to cycle at 48 KHz

2

S clock jitter

-

400

-

ps

WS I

on 1000 samples

106/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 37. PLLI2S (audio PLL) characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

VCO freq = 100 MHz

0.15

0.40

PLLI2S power consumption on

(4)

I

-

mA

DD(PLLI2S)

V

VCO freq = 432 MHz

0.45

0.75

DD

VCO freq = 100 MHz

0.30

0.40

PLLI2S power consumption on

(4)

I

-

mA

DDA(PLLI2S)

V

VCO freq = 432 MHz

0.55

0.85

DDA

1.

Take care of using the appropriate division factor M to have the specified PLL input clock values.

2.

Specified by design.

3.

Value given with main PLL running.

4.

Evaluated by characterization - not tested in production.

6.3.11

PLL spread spectrum clock generation (SSCG) characteristics

The spread spectrum clock generation (SSCG) feature allows to reduce electromagnetic

interferences (see Table 44: EMI characteristics for fHSE = 25 MH and fCPU = 168 MHz). It

is available only on the main PLL.

Table 38. SSCG parameters constraint

(1)

Symbol

Parameter

Min

Typ

Max

Unit

Modulation frequency

-

-

10

KHz

f

Mod

md

Peak modulation depth

0.25

-

2

%

15

−

MODEPER * INCSTEP

-

-

-

2

1

-

1.

Specified by design.

Equation 1

The frequency modulation period (MODEPER) is given by the equation below:

[

⁄

(

×

)

]

MODEPER

round f

4

f

=

PLL_IN

Mod

f

and f

must be expressed in Hz.

PLL_IN

Mod

As an example:

If f

= 1 MHz, and f

= 1 kHz, the modulation depth (MODEPER) is given by

PLL_IN

MOD

equation 1:

6

3

[

⁄

(

×

)

]

MODEPER

round 10

4

10

250

=

=

Equation 2

Equation 2 allows to calculate the increment step (INCSTEP):

15

[

(

(

)

×

×

)

⁄

(

×

×

)

]

INCSTEP

round

2

1

md

PLLN

100

5

MODEPER

=

–

f

must be expressed in MHz.

VCO_OUT

DS8626 Rev 11

107/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

With a modulation depth (md) = ±2 % (4 % peak to peak), and PLLN = 240 (in MHz):

15

[

(

(

)

×

×

)

⁄

(

×

×

)

]

INCSTEP

round

2

1

2

240

100

5

250

126md(quantitazed)%

=

–

=

An amplitude quantization error may be generated because the linear modulation profile is

obtained by taking the quantized values (rounded to the nearest integer) of MODPER and

INCSTEP. As a result, the achieved modulation depth is quantized. The percentage

quantized modulation depth is given by the following formula:

15

(

×

×

×

)

⁄

(

(

)

×

)

md

%

MODEPER

INCSTEP

100

5

2

1

PLLN

=

–

quantized

As a result:

15

(

×

×

×

)

⁄

(

(

)

×

)

md

%

250

126

100

5

2

1

240

2.002%(peak)

=

–

=

quantized

Figure 35 and Figure 36 show the main PLL output clock waveforms in center spread and

down spread modes, where:

F0 is f

nominal.

PLL_OUT

T

is the modulation period.

mode

md is the modulation depth.

Figure 35. PLL output clock waveforms in center spread mode

Frequency (PLL_OUT)

F0

md

md

Time

tmode

2xtmode

ai17291

108/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 36. PLL output clock waveforms in down spread mode

Frequency (PLL_OUT)

F0

2xmd

Time

tmode

2xtmode

ai17292b

6.3.12

Memory characteristics

Flash memory

The characteristics are given at T

=

40 to 105 °C unless otherwise specified.

–

A

The devices are shipped to customers with the flash memory erased.

Table 39. Flash memory characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

= 1.8 V

-

5

-

Write / Erase 8-bit mode, V

DD

Write / Erase 16-bit mode, V

mA

I

Supply current

= 2.1 V

-

8

-

DD

DD

= 3.3 V

-

12

-

Write / Erase 32-bit mode, V

DD

Table 40. Flash memory programming

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Program/erase parallelism

(2)

t

Word programming time

µs

-

16

100

prog

(PSIZE) = x 8/16/32

Program/erase parallelism

-

400

800

(PSIZE) = x 8

Program/erase parallelism

t

Sector (16 KB) erase time

-

300

600

ms

ERASE16KB

(PSIZE) = x 16

Program/erase parallelism

-

250

500

(PSIZE) = x 32

Program/erase parallelism

-

1200

2400

(PSIZE) = x 8

Program/erase parallelism

Sector (64 KB) erase time

-

700

1400

ms

t

ERASE64KB

(PSIZE) = x 16

Program/erase parallelism

-

550

1100

(PSIZE) = x 32

DS8626 Rev 11

109/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 40. Flash memory programming (continued)

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Program/erase parallelism

-

2

4

(PSIZE) = x 8

Program/erase parallelism

t

Sector (128 KB) erase time

-

1.3

2.6

s

ERASE128KB

(PSIZE) = x 16

Program/erase parallelism

-

1

2

(PSIZE) = x 32

Program/erase parallelism

-

16

32

(PSIZE) = x 8

Program/erase parallelism

Mass erase time

-

11

22

s

t

ME

(PSIZE) = x 16

Program/erase parallelism

-

8

16

(PSIZE) = x 32

32-bit program operation

2.7

-

3.6

V

V

Programming voltage

16-bit program operation

2.1

-

3.6

V

prog

8-bit program operation

1.8

-

3.6

V

1.

Evaluated by characterization - not tested in production.

2.

The maximum programming time is measured after 100K erase operations.

110/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 41. Flash memory programming with V

PP

(1)

(1)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(2)

t

Double word programming

-

16

100

µs

prog

Sector (16 KB) erase time

-

230

-

t

=

ERASE16KB

T

0 to +40 °C

A

Sector (64 KB) erase time

V

= 3.3 V

-

490

-

t

ms

ERASE64KB

DD

= 8.5 V

V

PP

t

Sector (128 KB) erase time

-

875

-

ERASE128KB

Mass erase time

-

6.9

-

s

t

ME

Programming voltage

-

2.7

-

3.6

V

V

prog

V

V

voltage range

-

7

-

9

V

PP

PP

Minimum current sunk on

-

10

-

-

mA

I

PP

the V

pin

PP

Cumulative time during

(3)

t

-

-

-

1

hour

VPP

which V

is applied

PP

1.

Specified by design.

2.

The maximum programming time is measured after 100K erase operations.

3.

V

should only be connected during programming/erasing.

PP

Table 42. Flash memory endurance and data retention

Value

Symbol

Parameter

Conditions

Unit

(1)

Min

T

= –40 to +85 °C (6 suffix versions)

A

N

Endurance

kcycles

10

END

T

= –40 to +105 °C (7 suffix versions)

A

(2)

1 kcycle

at T

= 85 °C

30

A

(2)

t

Data retention

at T

= 105 °C

10

1 kcycle

Years

RET

A

(2)

at T

= 55 °C

20

10 kcycles

A

1.

Evaluated by characterization - not tested in production.

2.

Cycling performed over the whole temperature range.

6.3.13

EMC characteristics

Susceptibility tests are performed on a sample basis during device characterization.

Functional EMS (electromagnetic susceptibility)

While a simple application is executed on the device (toggling 2 LEDs through I/O ports).

the device is stressed by two electromagnetic events until a failure occurs. The failure is

indicated by the LEDs:

•

Electrostatic discharge (ESD) (positive and negative) is applied to all device pins until

a functional disturbance occurs. This test is compliant with the IEC 61000-4-2 standard.

•

FTB: A burst of fast transient voltage (positive and negative) is applied to V

and V

DD

SS

through a 100 pF capacitor, until a functional disturbance occurs. This test is compliant

with the IEC 61000-4-4 standard.

DS8626 Rev 11

111/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

A device reset allows normal operations to be resumed.

The test results are given in Table 43. They are based on the EMS levels and classes

defined in application note AN1709.

Table 43. EMS characteristics

Level/

Symbol

Parameter

Conditions

Class

=

3.3 V, LQFP176, T

= +25 °C,

V

DD

A

Voltage limits to be applied on any I/O pin to

V

f

= 168 MHz, conforms to

2B

FESD

HCLK

induce a functional disturbance

IEC 61000-4-2

=

Fast transient voltage burst limits to be

V

3.3 V, LQFP176, T

= +25 °C,

DD

A

V

applied through 100 pF on V

and V

f

= 168 MHz, conforms to

4A

EFTB

DD

SS

HCLK

pins to induce a functional disturbance

IEC 61000-4-2

Designing hardened software to avoid noise problems

EMC characterization and optimization are performed at component level with a typical

application environment and simplified MCU software. It should be noted that good EMC

performance is highly dependent on the user application and the software in particular.

Therefore it is recommended that the user applies EMC software optimization and

prequalification tests in relation with the EMC level requested for his application.

Software recommendations

The software flowchart must include the management of runaway conditions such as:

•

Corrupted program counter

•

Unexpected reset

•

Critical Data corruption (control registers...)

Prequalification trials

Most of the common failures (unexpected reset and program counter corruption) can be

reproduced by manually forcing a low state on the NRST pin or the Oscillator pins for 1

second.

To complete these trials, ESD stress can be applied directly on the device, over the range of

specification values. When unexpected behavior is detected, the software can be hardened

to prevent unrecoverable errors occurring (see application note AN1015).

112/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Electromagnetic Interference (EMI)

The electromagnetic field emitted by the device are monitored while a simple application,

executing EEMBC code, is running. This emission test is compliant with SAE IEC61967-2

standard which specifies the test board and the pin loading.

Table 44. EMI characteristics for

= 25 MH and

= 168 MHz

f

f

HSE

CPU

Monitored

Symbol

Parameter

Conditions

Value

Unit

frequency band

0.1 to 30 MHz

32

V

= 3.3 V, T

= 25 °C, LQFP176

DD

A

(1)

Peak

30 to 130 MHz

25

dBµV

package, conforming to SAE J1752/3

EEMBC, code running from flash

130 MHz to 1GHz

29

memory with ART accelerator enabled

(2)

0.1 MHz to 2 GHz

4

-

Level

S

EMI

0.1 to 30 MHz

19

=

=

V

3.3 V, T

25 °C, LQFP176

DD

A

package, conforming to SAE J1752/3

(1)

Peak

30 to 130 MHz

16

dBµV

EEMBC, code running from flash

130 MHz to 1GHz

18

memory with ART accelerator and PLL

spread spectrum enabled

(2)

Level

0.1 MHz to 2 GHz

3.5

-

1.

Refer to AN1709 "EMI radiated test" chapter.

2.

Refer to AN1709 "EMI level classification" chapter.

6.3.14

Absolute maximum ratings (electrical sensitivity)

Stresses above the absolute maximum ratings listed in Table 11: Voltage characteristics,

Table 12: Current characteristics, and Table 13: Thermal characteristics may cause

permanent damage to the device. These are stress ratings only and the functional operation

of the device at these conditions is not implied. Exposure to maximum rating conditions for

extended periods may affect device reliability. Device mission profile (application conditions)

is compliant with JEDEC JESD47 Qualification Standard, extended mission profiles are

available on demand.

Electrostatic discharge (ESD)

Electrostatic discharges (a positive then a negative pulse separated by 1 second) are

applied to the pins of each sample according to each pin combination. The sample size

depends on the number of supply pins in the device (3 parts × (n+1) supply pins). This test

conforms to the JESD22-A114/C101 standard.

Table 45. ESD absolute maximum ratings

Maximum

Symbol

Ratings

Conditions

Class

Unit

(1)

value

Electrostatic discharge

(2)

=

voltage (human body

T

+25 °C conforming to JESD22-A114

2

2000

V

ESD(HBM)

A

model)

V

Electrostatic discharge

=

+25 °C conforming to

T

A

voltage (charge device

V

II

250

ESD(CDM)

ANSI/ESDA/JEDEC JS-002

model)

1.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

113/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

2.

On V

pin, V

is limited to 1000 V.

BAT

ESD(HBM)

Static latchup

Two complementary static tests are required on six parts to assess the latchup

performance:

•

A supply overvoltage is applied to each power supply pin

•

A current injection is applied to each input, output and configurable I/O pin

These tests are compliant with EIA/JESD 78A IC latchup standard.

Table 46. Electrical sensitivities

Symbol

Parameter

Conditions

Class

=

LU

Static latch-up class

T

+105 °C conforming to JESD78A

II level A

A

6.3.15

I/O current injection characteristics

As a general rule, current injection to the I/O pins, due to external voltage below V

or

SS

above V

(for standard, 3 V-capable I/O pins) should be avoided during normal product

DD

operation. However, in order to give an indication of the robustness of the microcontroller in

cases when abnormal injection accidentally happens, susceptibility tests are performed on a

sample basis during device characterization.

Functional susceptibilty to I/O current injection

While a simple application is executed on the device, the device is stressed by injecting

current into the I/O pins programmed in floating input mode. While current is injected into

the I/O pin, one at a time, the device is checked for functional failures.

The failure is indicated by an out of range parameter: ADC error above a certain limit (>5

LSB TUE), out of conventional limits of induced leakage current on adjacent pins (out of

5 μA/+0 μA range), or other functional failure (for example reset, oscillator frequency

deviation).

Negative induced leakage current is caused by negative injection and positive induced

leakage current by positive injection.

The test results are given in Table 47.

114/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 47. I/O current injection susceptibility

Functional susceptibility

Symbol

Description

Unit

Negative

Positive

injection

injection

Injected current on BOOT0 pin

−

NA

0

Injected current on NRST pin

−

NA

0

Injected current on PE2, PE3, PE4, PE5, PE6,

PI8, PC13, PC14, PC15, PI9, PI10, PI11, PF0,

PF1, PF2, PF3, PF4, PF5, PF10, PH0/OSC_IN,

(1)

−

NA

mA

I

0

INJ

PH1/OSC_OUT, PC0, PC1, PC2, PC3, PB6,

PB7, PB8, PB9, PE0, PE1, PI4, PI5, PI6, PI7,

PDR_ON, BYPASS_REG

Injected current on all FT pins

− 5

NA

Injected current on any other pin

− 5

+5

1.

It is recommended to add a Schottky diode (pin to ground) to analog pins which may potentially inject negative currents.

6.3.16

I/O port characteristics

General input/output characteristics

Unless otherwise specified, the parameters given in Table 48 are derived from tests

performed under the conditions summarized in Table 14. All I/Os are CMOS and TTL

compliant.

Note:

For information on GPIO configuration, refer to application note AN4899 “STM32 GPIO

configuration for hardware settings and low-power consumption” available from the ST

website www.st.com.

Table 48. I/O static characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

-

-

0.3V

-0.04

FT, TTa and NRST I/O input low

DD

V

3.6 V

1.7 V

≤

≤

DD

level voltage

(2)

-

-

0.3V

DD

1.75 V

V

3.6 V

≤

≤

DD

V

-

-

IL

T

105 °C

-40 °C

≤

≤

BOOT0 I/O input low level

A

(1)

-+0.1

0.1V

DD

voltage

1.7 V

V

3.6 V

≤

≤

DD

-

-

T

105 °C

0 °C

≤

≤

A

V

(1)

0.45V

+0.3

-

-

FT, TTa and NRST I/O input low

DD

V

3.6 V

1.7 V

≤

≤

DD

level voltage

(2)

0.7V

-

-

DD

V

3.6 V

1.75 V

≤

≤

DD

V

-

-

IH

T

105 °C

-40 °C

≤

≤

BOOT0 I/O input low level

A

(1)

0.17V

+0.7

DD

voltage

1.7 V

V

3.6 V

≤

≤

DD

-

-

T

105 °C

0 °C

≤

≤

A

DS8626 Rev 11

115/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 48. I/O static characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

FT, TTa and NRST I/O input

(3)

V

3.6 V

10%V

-

-

1.7 V

≤

≤

DD

DD

hysteresis

1.75 V

V

3.6 V

≤

≤

DD

V

V

HYS

T

105 °C

-40 °C

≤

≤

A

BOOT0 I/O input hysteresis

0.1

-

-

V

3.6 V

1.7 V

≤

≤

DD

T

105 °C

0 °C

≤

≤

A

(4)

±

I/O input leakage current

V

V

V

-

-

1

≤

≤

SS

IN

DD

I

µA

lkg

(5)

V

5 V

-

-

3

I/O FT input leakage current

=

IN

All pins

except for

PA10 and

V

30

40

50

V

=

IN

SS

PB12

Weak pull-up

(OTG_FS_ID,

R

equivalent

OTG_HS_ID)

PU

(6)

resistor

PA10 and

PB12

-

7

10

14

Ω

k

(OTG_FS_ID,

OTG_HS_ID)

All pins

except for

V

30

40

50

V

=

Weak pull-down

IN

DD

PA10 and

R

equivalent

PB12

PD

(7)

resistor

PA10 and

-

7

10

14

PB12

I/O pin

(8)

C

-

-

-

5

-

pF

IO

capacitance

1.

Specified by design.

2.

Tested in production.

3.

With a minimum of 200 mV.

4.

Leakage could be higher than the maximum value, if negative current is injected on adjacent pins.Refer to Table 47: I/O

current injection susceptibility

+ 0.3 V, the internal pull-up/pull-down resistors must be disabled. Leakage could be

5.

To sustain a voltage higher than V

DD

higher than the maximum value, if negative current is injected on adjacent pins. Refer to Table 47: I/O current injection

susceptibility.

6.

Pull-up and pull-down resistors are designed with a true resistance in series with a switchable PMOS. This PMOS

.

(~10% order)

contribution

to the series resistance is minimum

7.

Pull-up and pull-down resistors are designed with a true resistance in series with a switchable NMOS. This NMOS

.

contribution

(~10% order)

to the series resistance is minimum

8.

Hysteresis voltage between Schmitt trigger switching levels. Evaluated by characterization - not tested in production.

All I/Os are CMOS and TTL compliant (no software configuration required). Their

characteristics cover more than the strict CMOS-technology or TTL parameters.

116/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Output driving current

±

The GPIOs (general purpose input/outputs) can sink or source up to

8 mA, and sink or

±

source up to

20 mA (with a relaxed V

/V

) except PC13, PC14 and PC15 which can

OL

OH

±

sink or source up to

3mA. When using the PC13 to PC15 GPIOs in output mode, the speed

should not exceed 2 MHz with a maximum load of 30 pF.

In the user application, the number of I/O pins which can drive current must be limited to

respect the absolute maximum rating specified in Section 6.2. In particular:

•

The sum of the currents sourced by all the I/Os on V

plus the maximum Run

DD,

consumption of the MCU sourced on V

cannot exceed the absolute maximum rating

DD,

I

(see Table 12).

VDD

•

The sum of the currents sunk by all the I/Os on V

plus the maximum Run

SS

consumption of the MCU sunk on V

cannot exceed the absolute maximum rating

SS

I

(see Table 12).

VSS

Output voltage levels

Unless otherwise specified, the parameters given in Table 49 are derived from tests

performed under ambient temperature and V

supply voltage conditions summarized in

DD

Table 14. All I/Os are CMOS and TTL compliant.

(1)

Table 49. Output voltage characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

V

-

0.4

Output low level voltage

CMOS port

OL

= +8 mA

V

I

IO

(3)

V

Output high level voltage

V

–0.4

-

OH

DD

2.7 V < V

< 3.6 V

DD

(2)

V

-

0.4

Output low level voltage

TTL port

OL

V

I

=+ 8mA

IO

(3)

V

Output high level voltage

2.4

-

OH

< 3.6 V

2.7 V < V

DD

(2)(4)

-

1.3

Output low level voltage

V

= +20 mA

I

OL

IO

V

2.7 V < V

< 3.6 V

(3)(4)

Output high level voltage

V

–1.3

-

V

DD

OH

DD

(2)(4)

-

0.4

Output low level voltage

V

= +6 mA

I

OL

IO

V

< 2.7 V

2 V < V

(3)(4)

Output high level voltage

V

–0.4

-

V

DD

OH

DD

1.

PC13, PC14, PC15 and PI8 are supplied through the power switch. Since the switch only sinks a limited

amount of current (3 mA), the use of GPIOs PC13 to PC15 and PI8 in output mode is limited: the speed

should not exceed 2 MHz with a maximum load of 30 pF and these I/Os must not be used as a current

source (e.g. to drive an LED).

2.

The I

current sunk by the device must always respect the absolute maximum rating specified in Table 12

IO

(I/O ports and control pins) must not exceed I

.

and the sum of I

IO

VSS

3.

The I

current sourced by the device must always respect the absolute maximum rating specified in

IO

Table 12 and the sum of I

(I/O ports and control pins) must not exceed I

.

IO

VDD

4.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

117/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Input/output AC characteristics

The definition and values of input/output AC characteristics are given in Figure 37 and

Table 50, respectively.

Unless otherwise specified, the parameters given in Table 50 are derived from tests

performed under the ambient temperature and V

supply voltage conditions summarized

DD

in Table 14.

(1)(2)

Table 50. I/O AC characteristics

OSPEEDRy

[1:0] bit

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

value

C

= 50

pF, V

2.70 V

-

-

4

L

DD >

C

= 50

pF, V

1.8 V

-

-

2

L

DD >

(3)

f

MHz

Maximum frequency

max(IO)out

= 10

pF, V

2.70 V

-

-

8

C

L

DD >

00

= 10

pF, V

1.8 V

-

-

4

C

L

DD >

Output high to low level fall

/

= 50

pF, V

= 1.8 V to

t

C

f(IO)out

L

DD

time and output low to high

-

-

100

ns

t

3.6 V

r(IO)out

level rise time

C

= 50

pF, V

2.70 V

-

-

25

L

DD >

C

= 50

pF, V

1.8 V

-

-

12.5

L

DD >

(3)

MHz

Maximum frequency

f

max(IO)out

(4)

C

= 10

pF, V

2.70 V

-

-

50

L

DD >

C

= 10

pF, V

1.8 V

-

-

20

L

DD >

01

= 50

pF, V

>2.7 V

-

-

10

C

L

DD

Output high to low level fall

C

= 50

pF, V

> 1.8 V

-

-

20

t

/

L

DD

f(IO)out

ns

time and output low to high

t

= 10

pF, V

2.70 V

-

-

6

C

r(IO)out

level rise time

L

DD >

= 10

pF, V

1.8 V

-

-

10

C

L

DD >

(4)

C

= 40

pF, V

2.70 V

-

-

50

L

DD >

C

= 40

pF, V

1.8 V

-

-

25

L

DD >

(3)

Maximum frequency

MHz

f

max(IO)out

(4)

= 10

pF, V

2.70 V

-

-

100

C

L

DD >

(4)

C

= 10

pF, V

1.8 V

-

-

50

L

DD >

10

= 40

pF, V

2.70 V

-

-

6

C

L

DD >

Output high to low level fall

C

= 40

pF, V

1.8 V

-

-

10

t

/

L

DD >

f(IO)out

ns

time and output low to high

t

C

= 10

pF, V

2.70 V

-

-

4

r(IO)out

level rise time

L

DD >

= 10

pF, V

1.8 V

-

-

6

C

L

DD >

118/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)(2)

Table 50. I/O AC characteristics

(continued)

OSPEEDRy

[1:0] bit

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

value

(4)

C

= 30

pF, V

2.70 V

-

-

100

L

DD >

(4)

C

= 30

pF, V

1.8 V

-

-

50

L

DD >

(3)

F

Maximum frequency

MHz

max(IO)out

(4)

C

= 10

pF, V

2.70 V

-

-

180

L

DD >

(4)

C

= 10

pF, V

1.8 V

-

-

100

L

DD >

11

= 30

pF, V

2.70 V

-

-

4

C

L

DD >

Output high to low level fall

C

= 30

pF, V

1.8 V

-

-

6

t

/

L

DD >

f(IO)out

ns

time and output low to high

t

= 10

pF, V

2.70 V

-

-

2.5

C

r(IO)out

level rise time

L

DD >

= 10

pF, V

1.8 V

-

-

4

C

L

DD >

Pulse width of external signals

-

t

detected by the EXTI

-

10

-

-

ns

EXTIpw

controller

1.

Evaluated by characterization - not tested in production.

2.

The I/O speed is configured using the OSPEEDRy[1:0] bits. Refer to the STM32F4xx reference manual for a description of

the GPIOx_SPEEDR GPIO port output speed register.

3.

The maximum frequency is defined in Figure 37.

4.

For maximum frequencies above 50 MHz, the compensation cell should be used.

Figure 37. I/O AC characteristics definition

90%

10%

50%

10%

50%

90%

t

t

EXTERNAL

r(IO)out

f(IO)out

OUTPUT

T

ON CL

Maximum frequency is achieved with a duty cycle at

45-55% when loaded by the specified capacitance

MSv75936V1

DS8626 Rev 11

119/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

6.3.17

NRST pin characteristics

The NRST pin input driver uses CMOS technology. It is connected to a permanent pull-up

resistor, R

(see Table 48).

PU

Unless otherwise specified, the parameters given in Table 51 are derived from tests

performed under the ambient temperature and V

supply voltage conditions summarized

DD

in Table 14.

Table 51. NRST pin characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

NRST Input low level voltage

-

-

0.8

V

TTL ports

IL(NRST)

2.7 V ≤ V

≤ 3.6 V

(1)

V

NRST Input high level voltage

2

-

-

DD

IH(NRST)

V

(1)

NRST Input low level voltage

V

-

-

0.3V

CMOS ports

IL(NRST)

DD

1.8 V ≤ V

≤ 3.6 V

(1)

V

NRST Input high level voltage

0.7V

-

-

DD

IH(NRST)

DD

NRST Schmitt trigger voltage

V

-

-

200

-

mV

hys(NRST)

hysteresis

(2)

Ω

Weak pull-up equivalent resistor

V

V

30

40

50

k

R

=

PU

IN

SS

(1)

NRST Input filtered pulse

-

-

-

100

ns

V

F(NRST)

(1)

NRST Input not filtered pulse

V

> 2.7 V

300

-

-

ns

V

NF(NRST)

DD

Internal reset

T

Generated reset pulse duration

20

-

-

µs

NRST_OUT

source

1.

Specified by design.

2.

The pull-up is designed with a true resistance in series with a switchable PMOS. This PMOS contribution

to

.

(~10% order)

the series resistance must be minimum

Figure 38. Recommended NRST pin protection

V

DD

External

(1)

reset circuit

R

PU

(2)

Internal Reset

NRST

0.1 μF

Filter

STM32F

ai14132c

1.

The reset network protects the device against parasitic resets.

max level specified in

2.

The user must ensure that the level on the NRST pin can go below the V

IL(NRST)

Table 51. Otherwise the reset is not taken into account by the device.

6.3.18

TIM timer characteristics

The parameters given in Table 52 and Table 53 are specified by design.

Refer to Section 6.3.16: I/O port characteristics for details on the input/output alternate

function characteristics (output compare, input capture, external clock, PWM output).

120/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)

Table 52. Characteristics of TIMx connected to the APB1 domain

Symbol

Parameter

Conditions

Min

Max

Unit

AHB/APB1

1

-

t

TIMxCLK

prescaler distinct

from 1, f

=

11.9

-

ns

TIMxCLK

84 MHz

t

Timer resolution time

res(TIM)

1

-

t

AHB/APB1

TIMxCLK

prescaler = 1,

23.8

-

ns

= 42 MHz

f

TIMxCLK

/2

MHz

0

f

Timer external clock

TIMxCLK

f

EXT

frequency on CH1 to CH4

0

42

MHz

Res

Timer resolution

-

16/32

bit

TIM

16-bit counter clock

1

65536

t

TIMxCLK

period when internal clock

f

= 84 MHz

TIMxCLK

0.0119

780

µs

is selected

APB1= 42 MHz

t

COUNTER

32-bit counter clock

1

-

t

TIMxCLK

period when internal clock

0.0119

51130563

µs

is selected

-

65536 × 65536

t

TIMxCLK

t

Maximum possible count

MAX_COUNT

-

51.1

s

1.

TIMx is used as a general term to refer to the TIM2, TIM3, TIM4, TIM5, TIM6, TIM7, and TIM12 timers.

DS8626 Rev 11

121/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

(1)

Table 53. Characteristics of TIMx connected to the APB2 domain

Symbol

Parameter

Conditions

Min

Max

Unit

AHB/APB2

1

-

t

TIMxCLK

prescaler distinct

from 1, f

=

5.95

-

ns

TIMxCLK

168 MHz

t

Timer resolution time

res(TIM)

1

-

t

AHB/APB2

TIMxCLK

prescaler = 1,

11.9

-

ns

= 84 MHz

f

TIMxCLK

Timer external clock

/2

MHz

0

f

TIMxCLK

frequency on CH1 to

f

EXT

0

84

MHz

CH4

=

f

Timer resolution

-

16

bit

Res

TIMxCLK

TIM

168 MHz

16-bit counter clock

APB2 = 84 MHz

t

period when internal

1

65536

t

COUNTER

TIMxCLK

clock is selected

t

Maximum possible count

-

32768

t

MAX_COUNT

TIMxCLK

1.

TIMx is used as a general term to refer to the TIM1, TIM8, TIM9, TIM10, and TIM11 timers.

6.3.19

Communications interfaces

2

I

C interface

characteristics

2

2

The I

C interface meets the timings requirements of the I

C-bus specification and user

manual rev. 03 for:

•

Standard-mode (Sm): with a bit rate up to 100 kbit/s

•

Fast-mode (Fm): with a bit rate up to 400 kbit/s.

2

The I

C timings requirements are specified by design when the I2C peripheral is properly

configured (refer to RM0090 reference manual).

The SDA and SCL I/O requirements are met with the following restrictions: the SDA and

SCL I/O pins are not “true” open-drain. When configured as open-drain, the PMOS

connected between the I/O pin and V

is disabled, but is still present. Refer to

DD

2

C I/O characteristics

for more details on the I

.

Section 6.3.16: I/O port characteristics

2

All I

C SDA and SCL I/Os embed an analog filter. Refer to the table below for the analog

filter characteristics:

(1)

Table 54. I2C analog filter characteristics

Symbol

Parameter

Min

Max

Unit

Maximum pulse width of spikes

(2)

(3)

t

that are suppressed by the analog

50

260

ns

AF

filter

1.

Specified by design.

2.

Spikes with widths below t

are filtered.

AF(min)

3.

Spikes with widths above t

are not filtered

AF(max)

122/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

SPI interface characteristics

Unless otherwise specified, the parameters given in Table 55 for SPI are derived from tests

performed under the ambient temperature, f

frequency and V

supply voltage

PCLKx

DD

conditions summarized in Table 14 with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5 V

DD

Refer to Section 6.3.16: I/O port characteristics for more details on the input/output alternate

function characteristics (NSS, SCK, MOSI, MISO).

(1)

Table 55. SPI dynamic characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Master mode, SPI1,

42

2.7V < V

< 3.6V

DD

f

-

-

SCK

Slave mode, SPI1,

42

< 3.6V

2.7V < V

DD

SPI clock frequency

MHz

Master mode, SPI1/2/3,

21

< 3.6V

1.7V < V

DD

-

-

1/t

c(SCK)

Slave mode, SPI1/2/3,

21

< 3.6V

1.7V < V

DD

Duty cycle of SPI clock

Duty(SCK)

Slave mode

30

50

70

%

frequency

DS8626 Rev 11

123/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

(1)

Table 55. SPI dynamic characteristics

(continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

Master mode, SPI presc = 2,

t

-0.5

T

T

+0.5

T

w(SCKH)

PCLK

PCLK

PCLK

2.7V < V

< 3.6V

DD

SCK high and low time

Master mode, SPI presc = 2,

T

-2

T

T

+2

t

w(SCKL)

PCLK

PCLK

PCLK

1.7V < V

< 3.6V

DD

t

NSS setup time

Slave mode, SPI presc = 2

4 x T

su(NSS)

PCLK

-

-

NSS hold time

Slave mode, SPI presc = 2

2 x T

t

h(NSS)

PCLK

Master mode

6.5

-

-

t

su(MI)

Data input setup time

t

Slave mode

2.5

-

-

su(SI)

Master mode

2.5

-

-

t

h(MI)

Data input hold time

t

Slave mode

4

-

-

h(SI)

(2)

t

Data output access time

Slave mode, SPI presc = 2

0

-

4 x T

a(SO)

PCLK

Slave mode, SPI1,

0

-

7.5

2.7V < V

< 3.6V

DD

(3)

t

Data output disable time

dis(SO)

Slave mode, SPI1/2/3

ns

0

-

16.5

< 3.6V

1.7V < V

DD

Slave mode (after enable edge),

-

11

13

< 3.6V

SPI1, 2.7V < V

DD

Slave mode (after enable edge),

-

12

16.5

t

v(SO)

< 3.6V

SPI2/3, 2.7V < V

DD

Data output valid/hold time

Slave mode (after enable edge),

t

-

15.5

19

h(SO)

< 3.6V

SPI1, 1.7V < V

DD

Slave mode (after enable edge),

-

18

20.5

SPI2/3, 1.7V < V

< 3.6V

DD

Master mode (after enable edge),

-

-

2.5

SPI1, 2.7V < V

< 3.6V

DD

Data output valid time

t

v(MO)

Master mode (after enable edge),

-

-

4.5

< 3.6V

SPI1/2/3, 1.7V < V

DD

t

Data output hold time

Master mode (after enable edge)

0

-

-

h(MO)

1.

Evaluated by characterization - not tested in production.

2.

Min time is for the minimum time to drive the output and the max time is for the maximum time to validate the data.

3.

Min time is for the minimum time to invalidate the output and the max time is for the maximum time to put the data in Hi-Z.

124/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 39. SPI timing diagram - slave mode and CPHA = 0

NSS input

t

su(NSS)

t

t

h(NSS)

c(SCK)

t

w(SCKH)

CPHA=0

CPOL=0

CPHA=0

CPOL=1

SCK input

t

t

t

t

t

a(SO)

w(SCKL)

h(SO)

v(SO)

dis(SO)

MISO output

First bit OUT

Next bits OUT

Last bit OUT

t

t

su(SI)

h(SI)

MOSI input

First bit IN

Next bits IN

Last bit IN

MSv41658V2

Figure 40. SPI timing diagram - slave mode and CPHA = 1

NSS input

t

t

c(SCK)

h(NSS)

t

t

su(NSS)

CPHA=1

w(SCKH)

CPOL=0

CPHA=1

SCK input

CPOL=1

t

w(SCKL)

t

t

t

t

v(SO)

a(SO)

(SO)

h

dis(SO)

MISO output

Last bit OUT

Next bits OUT

First bit OUT

t

t

su(SI)

h(SI)

First bit IN

Next bits IN

Last bit IN

MOSI input

MSv41659V2

DS8626 Rev 11

125/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 41. SPI timing diagram - master mode

High

NSS input

t

c(SCK)

t

w(SCKH)

CPHA=0

CPOL=0

CPHA=0

CPOL=1

SCK output

CPHA=1

t

w(SCKL

)

CPOL=0

CPHA=1

CPOL=1

SCK output

t

t

su(MI)

h(MI)

MISO input

First bit IN

Next bits IN

Last bit IN

Last bit OUT

First bit OUT

Next bits OUT

MOSI output

t

t

v(MO)

h(MO)

MSv72626V1

126/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

2

I

S interface characteristics

2

Unless otherwise specified, the parameters given in Table 56 for the i

S interface are

derived from tests performed under the ambient temperature, f

frequency and V

PCLKx

DD

supply voltage conditions summarized in Table 14, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5 V

DD

Refer to Section 6.3.16: I/O port characteristics for more details on the input/output alternate

function characteristics (CK, SD, WS).

2

(1)

Table 56. I

S dynamic characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

256 x

(2)

2

f

I

S main clock output

-

256 x F

MHz

MCK

S

8K

Master data: 32 bits

-

64 x F

S

2

I

S clock frequency

MHz

f

CK

Slave data: 32 bits

-

64 x F

S

2

D

I

S clock frequency duty cycle

Slave receiver

30

70

%

CK

t

WS valid time

Master mode

0

6

v(WS)

t

WS hold time

Master mode

0

-

h(WS)

WS setup time

Slave mode

1

-

t

su(WS)

t

WS hold time

Slave mode

0

-

h(WS)

Master receiver

7.5

-

t

su(SD_MR)

Data input setup time

t

Slave receiver

2

-

su(SD_SR)

ns

t

Master receiver

0

-

h(SD_MR)

Data input hold time

Slave receiver

0

-

t

h(SD_SR)

t

v(SD_ST)

Slave transmitter (after enable edge)

-

27

t

Data output valid time

h(SD_ST)

t

Master transmitter (after enable edge)

-

20

v(SD_MT)

Data output hold time

Master transmitter (after enable edge)

2.5

-

t

h(SD_MT)

1.

Evaluated by characterization - not tested in production.

is 42 MHz (APB1 maximum frequency).

2.

The maximum value of 256 x F

S

2

Note:

Refer to the I

S section of RM0090 reference manual for more details on the sampling

frequency (F

). f

, f

, and D

values reflect only the digital peripheral behavior. The

S

MCK

CK

CK

value of these parameters might be slightly impacted by the source clock accuracy. D

CK

depends mainly on the value of ODD bit. The digital contribution leads to a minimum value

of I2SDIV / (2 x I2SDIV + ODD) and a maximum value of (I2SDIV + ODD) / (2 x I2SDIV +

ODD). F

maximum value is supported for each mode/condition.

S

DS8626 Rev 11

127/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

2

Figure 42. I

S slave timing diagram (Philips protocol)

1.

LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first

byte.

2

(1)

Figure 43. I

S master timing diagram (Philips protocol)

1.

Evaluated by characterization - not tested in production.

2.

LSB transmit/receive of the previously transmitted byte. No LSB transmit/receive is sent before the first

byte.

128/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

USB OTG FS characteristics

This interface is present in both the USB OTG HS and USB OTG FS controllers.

Table 57. USB OTG FS startup time

Symbol

Parameter

Max

Unit

(1)

t

USB OTG FS transceiver startup time

1

µs

STARTUP

1.

Specified by design.

Table 58. USB OTG FS DC electrical characteristics

(1)

(1)

Symbol

Parameter

Conditions

Min.

Typ.

Max.

Unit

USB OTG FS operating

(2)

V

-

3.6

V

-

3.0

DD

voltage

I(USB_FS_DP/DM,

(3)

V

Differential input sensitivity

0.2

-

-

DI

USB_HS_DP/DM)

Input

levels

Differential common mode

(3)

Includes V

range

0.8

-

2.5

V

V

CM

DI

range

Single ended receiver

(3)

V

-

1.3

-

2.0

SE

threshold

(4)

Ω

Static output level low

R

of 1.5

k

to 3.6 V

-

-

0.3

V

Output

OL

L

V

levels

(4)

Ω

Static output level high

R

of 15

k

to V

2.8

-

3.6

V

OH

L

SS

PA11, PA12, PB14, PB15

(USB_FS_DP/DM,

17

21

24

USB_HS_DP/DM)

R

= V

V

PD

IN

DD

PA9, PB13

(OTG_FS_VBUS,

0.65

1.1

2.0

OTG_HS_VBUS)

Ω

k

PA12, PB15 (USB_FS_DP,

V

= V

1.5

1.8

2.1

IN

SS

USB_HS_DP)

R

PA9, PB13

PU

(OTG_FS_VBUS,

V

= V

0.25

0.37

0.55

IN

SS

OTG_HS_VBUS)

1.

All the voltages are measured from the local ground potential.

2.

The STM32F405xx and STM32F407xx USB OTG FS functionality is ensured down to 2.7 V but not the full

USB OTG FS electrical characteristics which are degraded in the 2.7-to-3.0 V V

voltage range.

DD

3.

Specified by design.

is the load connected on the USB OTG FS drivers

R

4.

L

DS8626 Rev 11

129/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 44. USB OTG FS timings: definition of data signal rise and fall time

Cross over

points

Differential

data lines

V

CRS

V

SS

t

t

f

r

ai14137b

(1)

Table 59. USB OTG FS electrical characteristics

Driver characteristics

Symbol

Parameter

Conditions

Min

Max

Unit

(2)

t

Rise time

C

= 50 pF

4

20

ns

r

L

(2)

t

Fall time

C

= 50 pF

4

20

ns

f

L

Rise/ fall time matching

t

/t

90

110

%

t

rfm

r

f

Output signal crossover voltage

-

1.3

2.0

V

V

CRS

1.

Specified by design.

Measured from 10% to 90% of the data signal. For more detailed informations, please refer to USB

2.

Specification - Chapter 7 (version 2.0).

USB HS characteristics

Unless otherwise specified, the parameters given in Table 62 for ULPI are derived from

tests performed under the ambient temperature, f

frequency summarized in Table 61

HCLK

and V

supply voltage conditions summarized in Table 60, with the following configuration:

DD

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

.

DD

Refer to Section Section 6.3.16: I/O port characteristics for more details on the input/output

characteristics.

Table 60. USB HS DC electrical characteristics

(1)

(1)

Symbol

Parameter

Min.

Max.

Unit

USB OTG HS operating voltage

2.7

3.6

V

Input level

V

DD

1.

All the voltages are measured from the local ground potential.

(1)

Table 61. USB HS clock timing parameters

Parameter

Symbol

Min

Nominal

Max

Unit

value to guarantee proper operation of

f

HCLK

-

30

-

-

MHz

USB HS interface

54

60

66

MHz

Frequency (first transition)

8-bit ±10%

F

START_8BIT

130/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)

Table 61. USB HS clock timing parameters

Parameter

Symbol

Min

Nominal

Max

Unit

Frequency (steady state) ±500 ppm

F

59.97

60

60.03

MHz

STEADY

Duty cycle (first transition)

8-bit ±10%

D

40

50

60

%

START_8BIT

49.975

50

50.025

%

Duty cycle (steady state) ±500 ppm

D

STEADY

Time to reach the steady state frequency and

-

-

1.4

ms

T

STEADY

duty cycle after the first transition

Peripheral

T

-

-

5.6

Clock startup time after the

START_DEV

ms

de-assertion of SuspendM

-

-

-

Host

T

START_HOST

PHY preparation time after the first transition

T

-

-

-

µs

PREP

of the input clock

1.

Specified by design.

Table 62. ULPI timing

(1)

Value

Parameter

Symbol

Unit

Min.

Max.

-

2.0

Control in (ULPI_DIR) setup time

t

SC

Control in (ULPI_NXT) setup time

-

1.5

0

-

Control in (ULPI_DIR, ULPI_NXT) hold time

t

HC

-

2.0

Data in setup time

t

ns

SD

Data in hold time

t

0

-

HD

-

9.2

Control out (ULPI_STP) setup time and hold time

t

DC

-

10.7

Data out available from clock rising edge

t

DD

1.

V

= 2.7 V to 3.6 V and T

= –40 to 85 °C.

DD

A

Figure 45. ULPI timing diagram

Clock

t

t

HC

SC

Control In

(ULPI_DIR,

t

t

ULPI_NXT)

SD

HD

data In

(8-bit)

t

t

DC

DC

Control out

(ULPI_STP)

t

DD

data out

(8-bit)

ai17361c

DS8626 Rev 11

131/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Ethernet characteristics

Unless otherwise specified, the parameters given in Table 64, Table 65 and Table 66 for

SMI, RMII and MII are derived from tests performed under the ambient temperature, f

HCLK

frequency summarized in Table 14 and VDD supply voltage conditions summarized in

Table 63, with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

.

DD

Refer to Section 6.3.16: I/O port characteristics for more details on the input/output

characteristics.

Table 63. Ethernet DC electrical characteristics

(1)

(1)

Symbol

Parameter

Min.

Max.

Unit

Input level

V

Ethernet operating voltage

2.7

3.6

V

DD

1.

All the voltages are measured from the local ground potential.

Table 64 gives the list of Ethernet MAC signals for the SMI (station management interface)

and Figure 46 shows the corresponding timing diagram.

Figure 46. Ethernet SMI timing diagram

tMDC

ETH_MDC

td(MDIO)

ETH_MDIO(O)

tsu(MDIO)

th(MDIO)

ETH_MDIO(I)

MS31384V1

(1)

Table 64. Dynamic characteristics: Ethernet MAC signals for SMI

Symbol

Parameter

Min

Typ

Max

Unit

t

MDC cycle time(2.38 MHz)

411

420

425

MDC

T

Write data valid time

6

10

13

d(MDIO)

ns

Read data setup time

12

-

-

t

su(MDIO)

t

Read data hold time

0

-

-

h(MDIO)

1.

Evaluated by characterization - not tested in production.

Table 65 gives the list of Ethernet MAC signals for the RMII and Figure 47 shows the

corresponding timing diagram.

132/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 47. Ethernet RMII timing diagram

RMII_REF_CLK

t

d(TXEN)

t

d(TXD)

RMII_TX_EN

RMII_TXD[1:0]

t

t

su(RXD)

ih(RXD)

t

t

su(CRS)

ih(CRS)

RMII_RXD[1:0]

RMII_CRS_DV

ai15667

Table 65. Dynamic characteristics: Ethernet MAC signals for RMII

Symbol

Rating

Min

Typ

Max

Unit

t

Receive data setup time

2

-

-

ns

su(RXD)

Receive data hold time

1

-

-

ns

t

ih(RXD)

Carrier sense set-up time

0.5

-

-

ns

t

su(CRS)

t

Carrier sense hold time

2

-

-

ns

ih(CRS)

Transmit enable valid delay time

8

9.5

11

ns

t

d(TXEN)

Transmit data valid delay time

8.5

10

11.5

ns

t

d(TXD)

Table 66 gives the list of Ethernet MAC signals for MII and Figure 47 shows the

corresponding timing diagram.

Figure 48. Ethernet MII timing diagram

DS8626 Rev 11

133/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

(1)

Table 66. Dynamic characteristics: Ethernet MAC signals for MII

Symbol

Parameter

Min

Typ

Max

Unit

t

Receive data setup time

9

-

-

su(RXD)

Receive data hold time

10

-

-

t

ih(RXD)

Data valid setup time

9

-

-

t

su(DV)

t

Data valid hold time

8

-

-

ih(DV)

ns

Error setup time

6

-

-

t

su(ER)

Error hold time

8

-

-

t

ih(ER)

t

Transmit enable valid delay time

0

10

14

d(TXEN)

Transmit data valid delay time

0

10

15

t

d(TXD)

1.

Evaluated by characterization - not tested in production.

6.3.20

CAN (controller area network) interface

Refer to Section 6.3.16: I/O port characteristics for more details on the input/output alternate

function characteristics (CANTX and CANRX).

6.3.21

12-bit ADC characteristics

Unless otherwise specified, the parameters given in Table 67 are derived from tests

performed under the ambient temperature, f

frequency and V

supply voltage

PCLK2

DDA

conditions summarized in Table 14.

Table 67. ADC characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

(1)

Power supply

-

1.8

-

3.6

V

DDA

(1)(2)(3)

V

V

Positive reference voltage

-

1.8

-

V

REF+

DDA

V

Negative reference voltage

-

-

0

-

−

REF

(1)(3)

V

= 1.8

to

DDA

0.6

15

18

MHz

2.4 V

ADC clock frequency

f

ADC

(3)

= 2.4 to 3.6 V

0.6

30

36

MHz

V

DDA

f

= 30 MHz,

ADC

-

-

1764

kHz

12-bit resolution

(4)

External trigger frequency

f

TRIG

-

-

-

17

1/f

ADC

0 (V

or V

(5)

SSA

REF-

V

Conversion voltage range

-

V

-

V

AIN

REF+

tied to ground)

See Equation 1 for

(4)

κΩ

R

External input impedance

-

-

50

AIN

details

(4)(6)

κΩ

Sampling switch resistance

-

-

-

6

R

ADC

Internal sample and hold

(4)

C

-

-

4

-

pF

ADC

capacitor

134/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 67. ADC characteristics (continued)

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

= 30 MHz

-

-

0.100

µs

f

Injection trigger conversion

ADC

(4)

t

lat

latency

(7)

-

-

3

1/f

ADC

= 30 MHz

-

-

0.067

µs

f

Regular trigger conversion

ADC

(4)

t

latr

latency

(7)

1/f

-

-

2

ADC

f

= 30 MHz

0.100

-

16

µs

ADC

(4)

t

Sampling time

S

-

3

-

480

1/f

ADC

(4)

t

Power-up time

-

-

2

3

µs

STAB

f

= 30 MHz

ADC

0.50

-

16.40

µs

12-bit resolution

f

= 30 MHz

ADC

0.43

-

16.34

µs

10-bit resolution

f

Total conversion time (including

= 30 MHz

(4)

ADC

t

0.37

-

16.27

µs

CONV

sampling time)

8-bit resolution

f

= 30 MHz

ADC

0.30

-

16.20

µs

6-bit resolution

9 to 492 (t

for sampling +n-bit resolution for successive

S

1/f

ADC

approximation)

12-bit resolution

-

-

2

Msps

Single ADC

12-bit resolution

Sampling rate

-

-

3.75

Msps

Interleave Dual ADC

(4)

f

= 30 MHz, and

(f

S

ADC

mode

t

= 3 ADC cycles)

S

12-bit resolution

-

-

6

Msps

Interleave Triple ADC

mode

ADC V

DC current

REF

(4)

consumption in conversion

-

-

300

500

µA

I

VREF+

mode

ADC V

DC current

DDA

(4)

I

consumption in conversion

-

-

1.6

1.8

mA

VDDA

mode

1.

V

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use of

DD

DDA

an external power supply supervisor (refer to Section : Internal reset OFF).

2.

It is recommended to maintain the voltage difference between V

and V

below 1.8 V.

REF+

DDA

-V

< 1.2 V.

3.

V

DDA

REF+

4.

Evaluated by characterization - not tested in production.

is internally connected to V

and V

is internally connected to V

.

5.

V

REF+

DDA

REF-

SSA

maximum value is given for V

=1.8 V, and minimum value for V

=3.3 V.

6.

R

ADC

DD

DD

must be added to the latency specified in Table 67.

7.

For external triggers, a delay of 1/f

PCLK2

DS8626 Rev 11

135/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Equation 1: R

max formula

AIN

(

)

k

0.5

–

=

R

R

–

----------------------------------------------------------------

AIN

ADC

2

N

+

×

×

(

)

f

C

ln

2

ADC

ADC

The formula above (Equation 1) is used to determine the maximum external impedance

allowed for an error below 1/4 of LSB. N = 12 (from 12-bit resolution) and k is the number of

sampling periods defined in the ADC_SMPR1 register.

a

= 30 MHz

Table 68. ADC accuracy at f

ADC

(1)

Symbol

Parameter

Test conditions

Typ

Max

Unit

ET

Total unadjusted error

±2

±5

EO

Offset error

±1.5

±2.5

f

= 60 MHz,

PCLK2

Ω

EG

Gain error

= 30 MHz, R

< 10 k

,

±1.5

±3

LSB

f

ADC

AIN

(2)

V

= 1.8

to 3.6 V

DDA

ED

Differential linearity error

±1

±2

EL

Integral linearity error

±1.5

±3

1.

Evaluated by characterization - not tested in production.

2.

V

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range,

DD

DDA

and with the use of an external power supply supervisor (refer to Section : Internal reset OFF).

Note:

ADC accuracy vs. negative injection current: injecting a negative current on any analog

input pins should be avoided as this significantly reduces the accuracy of the conversion

being performed on another analog input. It is recommended to add a Schottky diode (pin to

ground) to analog pins which may potentially inject negative currents.

Any positive injection current within the limits specified for I

and SI

in

INJ(PIN)

INJ(PIN)

Section 6.3.16 does not affect the ADC accuracy.

Figure 49. ADC accuracy characteristics

V

V

REF+

DDA

[1LSB =

(or                 )]

n

n

2

2

Output code

E

G

(1) Example of an actual transfer curve

n

2

-1

(2) Ideal transfer curve

n

2

-2

(3) End-point correlation line

n

-3

2

n = ADC resolution

= total unadjusted error: maximum deviation

E

T

between the actual and ideal transfer curves

= offset error: maximum deviation between the first

E

O

7

actual transition and the first ideal one

6

= gain error: deviation between the last ideal

E

(2)

(3)

E

T

(1)

E

G

E

L

5

O

transition and the last actual one

4

= differential linearity error: maximum deviation

E

D

between actual steps and the ideal one

3

= integral linearity error: maximum deviation between

E

2

L

any actual transition and the end point correlation line

1

0

E

D

1 LSB ideal

)

(V

V

DDA

REF+

V

SSA

REF+

REF+

REF+

REF+

REF+

REF+

REF+

REF+

REF+

REF+

REF+

)*V

)*V

)*V

)*V

)*V

)*V

)*V

)*V

)*V

)*V

)*V

n

n

n

n

n

n

n

n

n

n

n

/2

/2

/2

/2

n

(1/2

(2/2

(3/2

(4/2

(5/2

(6/2

(7/2

n-3

n-2

n-1

(2

(2

(2

(2

MSv19880V6

136/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 50. Typical connection diagram when using the ADC with FT/TT pins featuring

analog switch function

(4)

(4)

V

V

REF+

DDA

Sample-and-hold ADC converter

I/O

analog

(1)

switch

R

R

ADC

AIN

(2)

Sampling

V

AIN

switch with

multiplexing

(3)

C

C

I

parasitic

ADC

lkg

Converter

V

V

V

SS

SS

SSA

MSv67871V3

1.

Refer to Table 67: ADC characteristics for the values of R

, R

and C

.

AIN

ADC

ADC

represents the capacitance of the PCB (dependent on soldering and PCB layout quality) plus the

2.

C

parasitic

pad capacitance (refer to Table 48: I/O static characteristics) A high C

value downgrades conversion

parasitic

accuracy. To remedy this, f

should be reduced.

ADC

3.

Refer to Table 48: I/O static characteristics.

4.

Refer to Figure 21: Power supply scheme.

DS8626 Rev 11

137/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

General PCB design guidelines

Power supply decoupling should be performed as shown in Figure 51 or Figure 52,

depending on whether V

is connected to V

or not. The 10 nF capacitors should be

REF+

DDA

ceramic (good quality). They should be placed them as close as possible to the chip.

Figure 51. Power supply and reference decoupling (V

V

not connected to

)

REF+

DDA

STM32F

(1)

V

REF+

1 μF // 10 nF

V

DDA

1 μF // 10 nF

(1)

V

/V

SSA

REF-

ai17535b

1.

V

and V

inputs are both available on UFBGA176. V

is also available on LQFP100, LQFP144,

REF+

REF–

REF+

and LQFP176. When V

and V

are not available, they are internally connected to V

and V

.

REF+

REF–

DDA

SSA

138/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 52. Power supply and reference decoupling

(V

V

)

connected to

REF+

DDA

STM32F

(1)

V

/V

REF+

DDA

1 μF // 10 nF

(1)

V

/V

REF-

SSA

ai17536c

1.

V

and V

inputs are both available on UFBGA176. V

is also available on LQFP100, LQFP144,

REF+

REF–

REF+

and LQFP176. When V

and V

are not available, they are internally connected to V

and V

.

REF+

REF–

DDA

SSA

6.3.22

Temperature sensor characteristics

Table 69. Temperature sensor characteristics

Symbol

Parameter

Min

Typ

Max

Unit

(1)

±

±

V

linearity with temperature

-

1

2

°C

T

L

SENSE

(1)

Avg_Slope

Average slope

-

2.5

-

mV/°C

(1)

Voltage at 25 °C

-

0.76

-

V

V

25

(2)

Startup time

-

6

10

µs

t

START

(2)

T

ADC sampling time when reading the temperature (1 °C accuracy)

10

-

-

µs

S_temp

1.

Evaluated by characterization - not tested in production.

2.

Specified by design.

Table 70. Temperature sensor calibration values

Symbol

Parameter

Memory address

TS_CAL1

TS ADC raw data acquired at temperature of 30 °C, V

=3.3 V

0x1FFF 7A2C - 0x1FFF 7A2D

DDA

=3.3 V

0x1FFF 7A2E - 0x1FFF 7A2F

TS_CAL2

TS ADC raw data acquired at temperature of 110 °C, V

DDA

DS8626 Rev 11

139/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

6.3.23

V

monitoring characteristics

BAT

Table 71. V

monitoring characteristics

BAT

Symbol

Parameter

Min

Typ

Max

Unit

Ω

-

50

-

K

R

Resistor bridge for V

BAT

Q

Ratio on V

measurement

-

2

-

-

BAT

(1)

Error on Q

–1

-

+1

%

Er

ADC sampling time when reading the V

(2)(2)

BAT

T

5

-

-

µs

S_vbat

1 mV accuracy

1.

Specified by design.

2.

Shortest sampling time can be determined in the application by multiple iterations.

6.3.24

Embedded reference voltage

The parameters given in Table 72 are derived from tests performed under ambient

temperature and V

supply voltage conditions summarized in Table 14.

DD

Table 72. Embedded internal reference voltage

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

V

Internal reference voltage

–40 °C < T

< +105 °C

1.18

1.21

1.24

V

REFINT

A

ADC sampling time when reading the

(1)

-

10

-

-

µs

T

S_vrefint

internal reference voltage

Internal reference voltage spread over the

(2)

V

= 3 V

-

3

5

mV

V

RERINT_s

DD

temperature range

(2)

T

Temperature coefficient

-

-

30

50

ppm/°C

Coeff

(2)

t

Startup time

-

-

6

10

µs

START

1.

Shortest sampling time can be determined in the application by multiple iterations.

2.

Specified by design.

Table 73. Internal reference voltage calibration values

Symbol

Parameter

Memory address

V

Raw data acquired at temperature of 30 °C, V

=3.3 V

0x1FFF 7A2A - 0x1FFF 7A2B

REFIN_CAL

DDA

6.3.25

DAC electrical characteristics

Table 74. DAC characteristics

Symbol

Parameter

Min

Typ

Max

Unit

Comments

(1)

V

Analog supply voltage

1.8

-

3.6

V

-

DDA

(1)

≤

Reference supply voltage

1.8

-

3.6

V

V

V

V

REF+

REF+

DDA

V

Ground

0

-

0

V

-

SSA

140/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 74. DAC characteristics (continued)

Symbol

Parameter

Min

Typ

Max

Unit

Comments

Resistive load with buffer

(2)

Ω

R

5

-

-

k

-

LOAD

ON

When the buffer is OFF, the

Impedance output with

Minimum resistive load between

(2)

Ω

R

-

-

15

k

O

to have a 1%

buffer OFF

DAC_OUT and V

SS

Ω

accuracy is 1.5 M

Maximum capacitive load at

(2)

C

Capacitive load

-

-

50

pF

DAC_OUT pin (when the buffer is

LOAD

ON).

It gives the maximum output

DAC_OUT

Lower DAC_OUT voltage

excursion of the DAC.

0.2

-

-

V

(2)

min

with buffer ON

It corresponds to 12-bit input code

=

(0x0E0) to (0xF1C) at V

REF+

DAC_OUT

Higher DAC_OUT voltage

3.6 V and (0x1C7) to (0xE38) at

– 0.2

V

-

-

V

(2)

DDA

max

with buffer ON

= 1.8 V

V

REF+

DAC_OUT

Lower DAC_OUT voltage

-

0.5

-

mV

(2)

min

with buffer OFF

It gives the maximum output

excursion of the DAC.

DAC_OUT

Higher DAC_OUT voltage

– 1LSB

V

-

-

V

(2)

REF+

max

with buffer OFF

With no load, worst code (0x800)

-

170

240

= 3.6 V in terms of DC

at V

REF+

DAC DC V

current

consumption on the inputs

REF

(4)

I

consumption in quiescent

µA

VREF+

With no load, worst code (0xF1C)

mode (Standby mode)

-

50

75

at V

= 3.6 V in terms of DC

REF+

consumption on the inputs

With no load, middle code (0x800)

-

280

380

µA

on the inputs

DAC DC VDDA current

(4)

consumption in quiescent

I

With no load, worst code (0xF1C)

DDA

(3)

mode

= 3.6 V in terms of DC

-

475

625

µA

at V

REF+

consumption on the inputs

Given for the DAC in 10-bit

-

-

±0.5

LSB

Differential non linearity

configuration.

(4)

Difference between two

DNL

consecutive code-1LSB)

Given for the DAC in 12-bit

-

-

±2

LSB

configuration.

Integral non linearity

Given for the DAC in 10-bit

-

-

±1

LSB

(difference between

configuration.

measured value at Code i

(4)

INL

and the value at Code i on a

Given for the DAC in 12-bit

-

-

±4

LSB

line drawn between Code 0

configuration.

and last Code 1023)

DS8626 Rev 11

141/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Table 74. DAC characteristics (continued)

Symbol

Parameter

Min

Typ

Max

Unit

Comments

Given for the DAC in 12-bit

-

-

±10

mV

Offset error

configuration

(difference between

Given for the DAC in 10-bit at

(4)

Offset

-

-

±3

LSB

measured value at Code

= 3.6 V

V

REF+

(0x800) and the ideal value

Given for the DAC in 12-bit at

/2)

= V

REF+

-

-

±12

LSB

V

= 3.6 V

REF+

Gain

Given for the DAC in 12-bit

Gain error

-

-

±0.5

%

(4)

error

configuration

Settling time (full scale: for a

10-bit input code transition

≤

50 pF,

between the lowest and the

C

(4)

LOAD

t

-

3

6

µs

SETTLING

≥

Ω

R

5 k

highest input codes when

LOAD

DAC_OUT reaches final

value ±4LSB

Total Harmonic Distortion

≤

50 pF,

C

(4)

LOAD

THD

-

-

-

dB

≥

Ω

R

5 k

Buffer ON

LOAD

Max frequency for a correct

≤

50 pF,

Update

DAC_OUT change when

C

LOAD

-

-

1

MS/s

(2)

≥

Ω

R

5 k

rate

small variation in the input

LOAD

code (from code i to i+1LSB)

≤

≥

Ω

50 pF, R

5 k

C

Wakeup time from off state

LOAD

LOAD

(4)

t

(Setting the ENx bit in the

-

6.5

10

µs

input code between lowest and

WAKEUP

DAC Control register)

highest possible ones.

Power supply rejection ratio

(2)

(to V

) (static DC

-

–67

–40

dB

No R

, C

= 50 pF

PSRR+

DDA

LOAD

LOAD

measurement)

1.

V

/V

minimum value of 1.7 V is obtained when the device operates in reduced temperature range, and with the use of

DD

DDA

an external power supply supervisor (refer to Section : Internal reset OFF).

2.

Specified by design.

3.

The quiescent mode corresponds to a state where the DAC maintains a stable output level to ensure that no dynamic

consumption occurs.

4.

Evaluated by characterization - not tested in production.

142/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 53. 12-bit buffered /non-buffered DAC

Buffered/Non-buffered DAC

Buffer(1)

R

L

DAC_OUTx

12-bit

digital to

analog

converter

C

L

ai17157V3

1.

The DAC integrates an output buffer that can be used to reduce the output impedance and to drive external

loads directly without the use of an external operational amplifier. The buffer can be bypassed by

configuring the BOFFx bit in the DAC_CR register.

6.3.26

FSMC characteristics

Unless otherwise specified, the parameters given in Table 75 to Table 86 for the FSMC

interface are derived from tests performed under the ambient temperature, f

frequency

HCLK

and V

supply voltage conditions summarized in Table 14, with the following configuration:

DD

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section Section 6.3.16: I/O port characteristics for more details on the input/output

characteristics.

Asynchronous waveforms and timings

Figure 54 through Figure 57 represent asynchronous waveforms and Table 75 through

Table 78 provide the corresponding timings. The results shown in these tables are obtained

with the following FSMC configuration:

•

AddressSetupTime = 1

•

AddressHoldTime = 0x1

•

DataSetupTime = 0x1

•

BusTurnAroundDuration = 0x0

In all timing tables, the

is the HCLK clock period.

T

HCLK

DS8626 Rev 11

143/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 54. Asynchronous non-multiplexed SRAM/PSRAM/NOR read waveforms

t

w(NE)

FSMC_NE

t

t

t

w(NOE)

h(NE_NOE)

v(NOE_NE)

FSMC_NOE

FSMC_NWE

t

t

v(A_NE)

h(A_NOE)

FSMC_A[25:0]

Address

t

v(BL_NE)

t

h(BL_NOE)

FSMC_NBL[1:0]

t

h(Data_NE)

t

t

su(Data_NOE)

h(Data_NOE)

t

su(Data_NE)

Data

FSMC_D[15:0]

t

v(NADV_NE)

t

w(NADV)

(1)

FSMC_NADV

ai14991c

1.

Mode 2/B, C and D only. In Mode 1, FSMC_NADV is not used.

(1)(2)

Table 75. Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings

Symbol

Parameter

Min

Max

Unit

FSMC_NE low time

2T

–0.5

2 T

+1

ns

t

w(NE)

HCLK

HCLK

t

FSMC_NEx low to FSMC_NOE low

0.5

3

ns

v(NOE_NE)

t

FSMC_NOE low time

2T

–2

2T

+ 2

ns

w(NOE)

HCLK

HCLK

t

FSMC_NOE high to FSMC_NE high hold time

0

-

ns

h(NE_NOE)

t

FSMC_NEx low to FSMC_A valid

-

4.5

ns

v(A_NE)

t

Address hold time after FSMC_NOE high

4

-

ns

h(A_NOE)

t

FSMC_NEx low to FSMC_BL valid

-

1.5

ns

v(BL_NE)

t

FSMC_BL hold time after FSMC_NOE high

0

-

ns

h(BL_NOE)

t

Data to FSMC_NEx high setup time

T

+4

-

ns

su(Data_NE)

HCLK

t

Data to FSMC_NOEx high setup time

T

+4

-

ns

su(Data_NOE)

HCLK

t

Data hold time after FSMC_NOE high

0

-

ns

h(Data_NOE)

t

Data hold time after FSMC_NEx high

0

-

ns

h(Data_NE)

t

FSMC_NEx low to FSMC_NADV low

-

2

ns

v(NADV_NE)

t

FSMC_NADV low time

-

T

ns

w(NADV)

HCLK

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

144/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 55. Asynchronous non-multiplexed SRAM/PSRAM/NOR write waveforms

t

w(NE)

FSMC_NEx

FSMC_NOE

t

t

t

h(NE_NWE)

v(NWE_NE)

w(NWE)

FSMC_NWE

t

t

v(A_NE)

h(A_NWE)

FSMC_A[25:0]

Address

t

t

v(BL_NE)

h(BL_NWE)

FSMC_NBL[1:0]

NBL

t

t

v(Data_NE)

h(Data_NWE)

Data

FSMC_D[15:0]

t

v(NADV_NE)

t

w(NADV)

(1)

FSMC_NADV

ai14990

1.

Mode 2/B, C and D only. In Mode 1, FSMC_NADV is not used.

(1)(2)

Table 76. Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_NE low time

3T

3T

+ 4

ns

w(NE)

HCLK

HCLK

FSMC_NEx low to FSMC_NWE low

T

–0.5

T

+0.5

ns

t

v(NWE_NE)

HCLK

HCLK

t

FSMC_NWE low time

T

–1

T

+2

ns

w(NWE)

HCLK

HCLK

FSMC_NWE high to FSMC_NE high hold time

T

–1

-

ns

t

h(NE_NWE)

HCLK

FSMC_NEx low to FSMC_A valid

-

0

ns

t

v(A_NE)

t

Address hold time after FSMC_NWE high

T

– 2

-

ns

h(A_NWE)

HCLK

FSMC_NEx low to FSMC_BL valid

-

1.5

ns

t

v(BL_NE)

FSMC_BL hold time after FSMC_NWE high

T

– 1

-

ns

t

h(BL_NWE)

HCLK

t

Data to FSMC_NEx low to Data valid

-

T

+3

ns

v(Data_NE)

HCLK

Data hold time after FSMC_NWE high

T

–1

-

ns

t

h(Data_NWE)

HCLK

FSMC_NEx low to FSMC_NADV low

-

2

ns

t

v(NADV_NE)

t

FSMC_NADV low time

-

T

+0.5

ns

w(NADV)

HCLK

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

145/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 56. Asynchronous multiplexed PSRAM/NOR read waveforms

t

w(NE)

FSMC_NE

t

t

h(NE_NOE)

v(NOE_NE)

FSMC_NOE

t

w(NOE)

FSMC_NWE

t

t

v(A_NE)

h(A_NOE)

FSMC_A[25:16]

Address

t

t

v(BL_NE)

h(BL_NOE)

FSMC_NBL[1:0]

NBL

t

h(Data_NE)

t

su(Data_NE)

t

t

t

v(A_NE)

su(Data_NOE)

h(Data_NOE)

Address

Data

FSMC_

AD[15:0]

t

t

h(AD_NADV)

v(NADV_NE)

t

w(NADV)

FSMC_NADV

ai14892b

(1)(2)

Table 77. Asynchronous multiplexed PSRAM/NOR read timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_NE low time

3T

–1

3T

+1

ns

w(NE)

HCLK

HCLK

t

FSMC_NEx low to FSMC_NOE low

2T

–0.5

2T

+0.5

ns

v(NOE_NE)

HCLK

HCLK

t

FSMC_NOE low time

T

–1

T

+1

ns

w(NOE)

HCLK

HCLK

FSMC_NOE high to FSMC_NE high hold time

0

-

ns

t

h(NE_NOE)

t

FSMC_NEx low to FSMC_A valid

-

3

ns

v(A_NE)

t

FSMC_NEx low to FSMC_NADV low

1

2

ns

v(NADV_NE)

FSMC_NADV low time

T

– 2

T

+1

ns

t

w(NADV)

HCLK

HCLK

t

FSMC_AD(adress) valid hold time after FSMC_NADV high)

T

-

ns

h(AD_NADV)

HCLK

t

Address hold time after FSMC_NOE high

T

–1

-

ns

h(A_NOE)

HCLK

t

FSMC_BL time after FSMC_NOE high

0

-

ns

h(BL_NOE)

t

FSMC_NEx low to FSMC_BL valid

-

2

ns

v(BL_NE)

t

Data to FSMC_NEx high setup time

T

+4

-

ns

su(Data_NE)

HCLK

t

Data to FSMC_NOE high setup time

T

+4

-

ns

su(Data_NOE)

HCLK

t

Data hold time after FSMC_NEx high

0

-

ns

h(Data_NE)

t

Data hold time after FSMC_NOE high

0

-

ns

h(Data_NOE)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

146/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 57. Asynchronous multiplexed PSRAM/NOR write waveforms

t

w(NE)

FSMC_NEx

FSMC_NOE

t

t

t

h(NE_NWE)

v(NWE_NE)

w(NWE)

FSMC_NWE

t

t

h(A_NWE)

v(A_NE)

FSMC_A[25:16]

Address

t

t

v(BL_NE)

h(BL_NWE)

FSMC_NBL[1:0]

NBL

t

t

t

v(A_NE)

h(Data_NWE)

v(Data_NADV)

Address

Data

FSMC_AD[15:0]

t

t

v(NADV_NE)

h(AD_NADV)

t

w(NADV)

FSMC_NADV

ai14891B

(1)(2)

Table 78. Asynchronous multiplexed PSRAM/NOR write timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_NE low time

4T

–0.5

4T

+3

ns

w(NE)

HCLK

HCLK

FSMC_NEx low to FSMC_NWE low

T

–0.5

T

-0.5

ns

t

v(NWE_NE)

HCLK

HCLK

t

FSMC_NWE low tim e

2T

–0.5

2T

+3

ns

w(NWE)

HCLK

HCLK

FSMC_NWE high to FSMC_NE high hold time

T

-

ns

t

h(NE_NWE)

HCLK

t

FSMC_NEx low to FSMC_A valid

-

0

ns

v(A_NE)

FSMC_NEx low to FSMC_NADV low

1

2

ns

t

v(NADV_NE)

FSMC_NADV low time

T

– 2

T

+ 1

ns

t

w(NADV)

HCLK

HCLK

FSMC_AD(address) valid hold time after

t

–2

-

ns

T

h(AD_NADV)

HCLK

FSMC_NADV high)

t

Address hold time after FSMC_NWE high

T

-

ns

h(A_NWE)

HCLK

t

FSMC_BL hold time after FSMC_NWE high

T

–2

-

ns

h(BL_NWE)

HCLK

FSMC_NEx low to FSMC_BL valid

-

1.5

ns

t

v(BL_NE)

FSMC_NADV high to Data valid

-

T

–0.5

ns

t

v(Data_NADV)

HCLK

t

Data hold time after FSMC_NWE high

T

-

ns

h(Data_NWE)

HCLK

1.

C

= 30 pF.

L

DS8626 Rev 11

147/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

2.

Evaluated by characterization - not tested in production.

Synchronous waveforms and timings

Figure 58 through Figure 61 represent synchronous waveforms and Table 80 through

Table 82 provide the corresponding timings. The results shown in these tables are obtained

with the following FSMC configuration:

•

BurstAccessMode = FSMC_BurstAccessMode_Enable;

•

MemoryType = FSMC_MemoryType_CRAM;

•

WriteBurst = FSMC_WriteBurst_Enable;

•

CLKDivision = 1; (0 is not supported, see the STM32F40xxx/41xxx reference manual)

•

DataLatency = 1 for NOR Flash; DataLatency = 0 for PSRAM

In all timing tables, the

is the HCLK clock period (with maximum

T

HCLK

FSMC_CLK = 60 MHz).

Figure 58. Synchronous multiplexed NOR/PSRAM read timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FSMC_CLK

Data latency = 0

t

t

d(CLKL-NExL)

d(CLKL-NExH)

FSMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FSMC_NADV

t

t

d(CLKL-AIV)

d(CLKL-AV)

FSMC_A[25:16]

t

t

d(CLKL-NOEH)

d(CLKL-NOEL)

FSMC_NOE

t

t

d(CLKL-ADIV)

h(CLKH-ADV)

t

t

t

t

su(ADV-CLKH)

su(ADV-CLKH)

d(CLKL-ADV)

h(CLKH-ADV)

AD[15:0]

D1

D2

FSMC_AD[15:0]

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FSMC_NWAIT

(WAITCFG = 1b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FSMC_NWAIT

(WAITCFG = 0b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

ai14893g

148/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)(2)

Table 79. Synchronous multiplexed NOR/PSRAM read timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_CLK period

2T

-

ns

w(CLK)

HCLK

FSMC_CLK low to FSMC_NEx low (x=0..2)

-

0

ns

t

d(CLKL-NExL)

FSMC_CLK low to FSMC_NEx high (x= 0…2)

2

-

ns

t

d(CLKL-NExH)

t

FSMC_CLK low to FSMC_NADV low

-

2

ns

d(CLKL-NADVL)

FSMC_CLK low to FSMC_NADV high

2

-

ns

t

d(CLKL-NADVH)

FSMC_CLK low to FSMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

t

FSMC_CLK low to FSMC_Ax invalid (x=16…25)

0

-

ns

d(CLKL-AIV)

FSMC_CLK low to FSMC_NOE low

-

0

ns

t

d(CLKL-NOEL)

FSMC_CLK low to FSMC_NOE high

2

-

ns

t

d(CLKL-NOEH)

t

FSMC_CLK low to FSMC_AD[15:0] valid

-

4.5

ns

d(CLKL-ADV)

FSMC_CLK low to FSMC_AD[15:0] invalid

0

-

ns

t

d(CLKL-ADIV)

t

FSMC_A/D[15:0] valid data before FSMC_CLK high

6

-

ns

su(ADV-CLKH)

FSMC_A/D[15:0] valid data after FSMC_CLK high

0

-

ns

t

h(CLKH-ADV)

FSMC_NWAIT valid before FSMC_CLK high

4

-

ns

t

su(NWAIT-CLKH)

t

FSMC_NWAIT valid after FSMC_CLK high

0

-

ns

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

149/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 59. Synchronous multiplexed PSRAM write timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FSMC_CLK

Data latency = 0

t

t

d(CLKL-NExL)

d(CLKL-NExH)

FSMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FSMC_NADV

t

t

d(CLKL-AV)

d(CLKL-AIV)

FSMC_A[25:16]

t

t

d(CLKL-NWEL)

d(CLKL-NWEH)

FSMC_NWE

t

t

d(CLKL-ADIV)

d(CLKL-Data)

t

t

d(CLKL-Data)

d(CLKL-ADV)

AD[15:0]

D1

D2

FSMC_AD[15:0]

FSMC_NWAIT

(WAITCFG = 0b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

t

d(CLKL-NBLH)

FSMC_NBL

ai14992g

(1)(2)

Table 80. Synchronous multiplexed PSRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_CLK period

2T

-

ns

w(CLK)

HCLK

FSMC_CLK low to FSMC_NEx low (x=0..2)

-

1

ns

t

d(CLKL-NExL)

t

FSMC_CLK low to FSMC_NEx high (x= 0…2)

1

-

ns

d(CLKL-NExH)

FSMC_CLK low to FSMC_NADV low

-

0

ns

t

d(CLKL-NADVL)

t

d(CLKL-

FSMC_CLK low to FSMC_NADV high

0

-

ns

NADVH)

t

FSMC_CLK low to FSMC_Ax valid (x=16…25)

-

0

ns

d(CLKL-AV)

t

FSMC_CLK low to FSMC_Ax invalid (x=16…25)

8

-

ns

d(CLKL-AIV)

FSMC_CLK low to FSMC_NWE low

-

0.5

ns

t

d(CLKL-NWEL)

FSMC_CLK low to FSMC_NWE high

0

-

ns

t

d(CLKL-NWEH)

t

FSMC_CLK low to FSMC_AD[15:0] invalid

0

-

ns

d(CLKL-ADIV)

FSMC_A/D[15:0] valid data after FSMC_CLK

-

3

ns

t

d(CLKL-DATA)

low

150/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)(2)

Table 80. Synchronous multiplexed PSRAM write timings

(continued)

Symbol

Parameter

Min

Max

Unit

t

FSMC_CLK low to FSMC_NBL high

0

-

ns

d(CLKL-NBLH)

t

su(NWAIT-

FSMC_NWAIT valid before FSMC_CLK high

4

-

ns

CLKH)

t

FSMC_NWAIT valid after FSMC_CLK high

0

-

ns

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

Figure 60. Synchronous non-multiplexed NOR/PSRAM read timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FSMC_CLK

t

t

d(CLKL-NExL)

d(CLKL-NExH)

Data latency = 0

FSMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FSMC_NADV

t

t

d(CLKL-AIV)

d(CLKL-AV)

FSMC_A[25:0]

t

t

d(CLKL-NOEL)

d(CLKL-NOEH)

FSMC_NOE

t

t

su(DV-CLKH)

h(CLKH-DV)

t

t

su(DV-CLKH)

h(CLKH-DV)

FSMC_D[15:0]

D1

D2

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FSMC_NWAIT

(WAITCFG = 1b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

FSMC_NWAIT

(WAITCFG = 0b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

h(CLKH-NWAITV)

ai14894f

DS8626 Rev 11

151/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

(1)(2)

Table 81. Synchronous non-multiplexed NOR/PSRAM read timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_CLK period

2T

–0.5

-

ns

w(CLK)

HCLK

FSMC_CLK low to FSMC_NEx low (x=0..2)

-

0.5

ns

t

d(CLKL-NExL)

FSMC_CLK low to FSMC_NEx high (x= 0…2)

0

-

ns

t

d(CLKL-NExH)

t

FSMC_CLK low to FSMC_NADV low

-

2

ns

d(CLKL-NADVL)

FSMC_CLK low to FSMC_NADV high

3

-

ns

t

d(CLKL-NADVH)

FSMC_CLK low to FSMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

t

FSMC_CLK low to FSMC_Ax invalid (x=16…25)

2

-

ns

d(CLKL-AIV)

FSMC_CLK low to FSMC_NOE low

-

0.5

ns

t

d(CLKL-NOEL)

FSMC_CLK low to FSMC_NOE high

1.5

-

ns

t

d(CLKL-NOEH)

t

FSMC_D[15:0] valid data before FSMC_CLK high

6

-

ns

su(DV-CLKH)

FSMC_D[15:0] valid data after FSMC_CLK high

3

-

ns

t

h(CLKH-DV)

t

FSMC_NWAIT valid before FSMC_CLK high

4

-

ns

su(NWAIT-CLKH)

FSMC_NWAIT valid after FSMC_CLK high

0

-

ns

t

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

152/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 61. Synchronous non-multiplexed PSRAM write timings

BUSTURN = 0

t

t

w(CLK)

w(CLK)

FSMC_CLK

t

t

d(CLKL-NExL)

d(CLKL-NExH)

Data latency = 0

FSMC_NEx

t

t

d(CLKL-NADVL)

d(CLKL-NADVH)

FSMC_NADV

t

t

d(CLKL-AV)

d(CLKL-AIV)

FSMC_A[25:0]

t

t

d(CLKL-NWEL)

d(CLKL-NWEH)

FSMC_NWE

t

t

d(CLKL-Data)

d(CLKL-Data)

D1

D2

FSMC_D[15:0]

FSMC_NWAIT

(WAITCFG = 0b, WAITPOL + 0b)

t

t

su(NWAITV-CLKH)

d(CLKL-NBLH)

t

h(CLKH-NWAITV)

FSMC_NBL

ai14993g

(1)(2)

Table 82. Synchronous non-multiplexed PSRAM write timings

Symbol

Parameter

Min

Max

Unit

t

FSMC_CLK period

2T

-

ns

w(CLK)

HCLK

d(CLKL-NExL)

t

FSMC_CLK low to FSMC_NEx low (x=0..2)

-

1

ns

FSMC_CLK low to FSMC_NEx high (x= 0…2)

1

-

ns

t

d(CLKL-NExH)

FSMC_CLK low to FSMC_NADV low

-

7

ns

t

d(CLKL-NADVL)

t

FSMC_CLK low to FSMC_NADV high

6

-

ns

d(CLKL-NADVH)

FSMC_CLK low to FSMC_Ax valid (x=16…25)

-

0

ns

t

d(CLKL-AV)

FSMC_CLK low to FSMC_Ax invalid (x=16…25)

6

-

ns

t

d(CLKL-AIV)

t

FSMC_CLK low to FSMC_NWE low

-

1

ns

d(CLKL-NWEL)

FSMC_CLK low to FSMC_NWE high

2

-

ns

t

d(CLKL-NWEH)

FSMC_D[15:0] valid data after FSMC_CLK low

-

3

ns

t

d(CLKL-Data)

t

FSMC_CLK low to FSMC_NBL high

3

-

ns

d(CLKL-NBLH)

FSMC_NWAIT valid before FSMC_CLK high

4

-

ns

t

su(NWAIT-CLKH)

FSMC_NWAIT valid after FSMC_CLK high

0

-

ns

t

h(CLKH-NWAIT)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

DS8626 Rev 11

153/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

PC Card/CompactFlash controller waveforms and timings

Figure 62 through Figure 67 represent synchronous waveforms, and Table 83 and Table 84

provide the corresponding timings. The results shown in this table are obtained with the

following FSMC configuration:

•

COM.FSMC_SetupTime = 0x04;

•

COM.FSMC_WaitSetupTime = 0x07;

•

COM.FSMC_HoldSetupTime = 0x04;

•

COM.FSMC_HiZSetupTime = 0x00;

•

ATT.FSMC_SetupTime = 0x04;

•

ATT.FSMC_WaitSetupTime = 0x07;

•

ATT.FSMC_HoldSetupTime = 0x04;

•

ATT.FSMC_HiZSetupTime = 0x00;

•

IO.FSMC_SetupTime = 0x04;

•

IO.FSMC_WaitSetupTime = 0x07;

•

IO.FSMC_HoldSetupTime = 0x04;

•

IO.FSMC_HiZSetupTime = 0x00;

•

TCLRSetupTime = 0;

•

TARSetupTime = 0.

In all timing tables, the

is the HCLK clock period.

T

HCLK

Figure 62. PC Card/CompactFlash controller waveforms for common memory read

access

(1)

FSMC_NCE4_2

FSMC_NCE4_1

t

h(NCEx-AI)

t

v(NCEx-A)

FSMC_A[10:0]

t

h(NCEx-NREG)

t

d(NREG-NCEx)

t

h(NCEx-NIORD)

t

d(NIORD-NCEx)

t

h(NCEx-

NIOWR

)

FSMC_NREG

FSMC_NIOWR

FSMC_NIORD

FSMC_NWE

t

t

d(NCE4_1-NOE)

w(NOE)

OE

FSMC_N

t

t

su(D-NOE)

h(NOE-D)

FSMC_D[15:0]

ai14895b

1.

FSMC_NCE4_2 remains high (inactive during 8-bit access.

154/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 63. PC Card/CompactFlash controller waveforms for common memory write

access

FSMC_NCE4_1

FSMC_NCE4_2

High

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FSMC_A[10:0]

t

h(NCE4_1-NREG)

t

d(NREG-NCE4_1)

t

h(NCE4_1-NIORD)

t

d(NIORD-NCE4_1)

t

h(NCE4_1-NIOWR)

FSMC_NREG

FSMC_NIOWR

FSMC_NIORD

t

t

t

d(NCE4_1-NWE)

w(NWE)

d(NWE-NCE4_1)

FSMC_NWE

FSMC_N

OE

MEMxHIZ =1

t

d(D-NWE)

t

t

v(NWE-D)

h(NWE-D)

FSMC_D[15:0]

ai14896

DS8626 Rev 11

155/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 64. PC Card/CompactFlash controller waveforms for attribute memory read

access

FSMC_NCE4_1

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FSMC_NCE4_2

High

FSMC_A[10:0]

FSMC_NIOWR

FSMC_NIORD

t

t

d(NREG-NCE4_1)

h(NCE4_1-NREG)

FSMC_NREG

FSMC_NWE

t

t

t

d(NCE4_1-NOE)

w(NOE)

d(NOE-NCE4_1)

FSMC_NOE

t

t

su(D-NOE)

h(NOE-D)

(1)

FSMC_D[15:0]

ai14897b

1.

Only data bits 0...7 are read (bits 8...15 are disregarded).

156/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 65. PC Card/CompactFlash controller waveforms for attribute memory write

access

FSMC_NCE4_1

High

FSMC_NCE4_2

t

t

v(NCE4_1-A)

h(NCE4_1-AI)

FSMC_A[10:0]

FSMC_NIOWR

FSMC_NIORD

t

t

d(NREG-NCE4_1)

h(NCE4_1-NREG)

FSMC_NREG

t

t

d(NCE4_1-NWE)

w(NWE)

FSMC_NWE

t

d(NWE-NCE4_1)

FSMC_NOE

t

v(NWE-D)

FSMC_D[7:0](1)

ai14898b

1.

Only data bits 0...7 are driven (bits 8...15 remains Hi-Z).

Figure 66. PC Card/CompactFlash controller waveforms for I/O space read access

FSMC_NCE4_1

FSMC_NCE4_2

t

t

h(NCE4_1-AI)

v(NCEx-A)

FSMC_A[10:0]

FSMC_NREG

FSMC_NWE

FSMC_NOE

FSMC_NIOWR

t

t

w(NIORD)

d(NIORD-NCE4_1)

FSMC_NIORD

t

t

su(D-NIORD)

d(NIORD-D)

FSMC_D[15:0]

ai14899B

DS8626 Rev 11

157/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 67. PC Card/CompactFlash controller waveforms for I/O space write access

FSMC_NCE4_1

FSMC_NCE4_2

t

t

v(NCEx-A)

h(NCE4_1-AI)

FSMC_A[10:0]

FSMC_NREG

FSMC_NWE

FSMC_NOE

FSMC_NIORD

t

t

d(NCE4_1-NIOWR)

w(NIOWR)

FSMC_NIOWR

ATTxHIZ =1

t

h(NIOWR-D)

t

v(NIOWR-D)

FSMC_D[15:0]

ai14900c

Table 83. Switching characteristics for PC Card/CF read and write cycles

(1)(2)

in attribute/common space

Symbol

Parameter

Min

Max

Unit

t

FSMC_Ncex low to FSMC_Ay valid

-

0

ns

v(NCEx-A)

FSMC_NCEx high to FSMC_Ax invalid

4

-

ns

t

h(NCEx_AI)

FSMC_NCEx low to FSMC_NREG valid

-

3.5

ns

t

d(NREG-NCEx)

t

FSMC_NCEx high to FSMC_NREG invalid

T

+4

-

ns

h(NCEx-NREG)

HCLK

FSMC_NCEx low to FSMC_NWE low

-

5T

+0.5

ns

t

d(NCEx-NWE)

HCLK

FSMC_NCEx low to FSMC_NOE low

-

5T

+0.5

ns

t

d(NCEx-NOE)

HCLK

t

FSMC_NOE low width

8T

–1

8T

+1

ns

w(NOE)

HCLK

HCLK

FSMC_NOE high to FSMC_NCEx high

5T

+2.5

-

ns

t

d(NOE_NCEx)

HCLK

FSMC_D[15:0] valid data before FSMC_NOE high

4.5

-

ns

t

su (D-NOE)

t

FSMC_N0E high to FSMC_D[15:0] invalid

3

-

ns

h(N0E-D)

FSMC_NWE low width

8T

–0.5

8T

+ 3

ns

t

w(NWE)

HCLK

HCLK

t

FSMC_NWE high to FSMC_NCEx high

-

ns

5T

–1

d(NWE_NCEx)

HCLK

FSMC_NCEx low to FSMC_NWE low

-

5T

+ 1

ns

t

d(NCEx-NWE)

HCLK

t

FSMC_NWE low to FSMC_D[15:0] valid

-

0

ns

v(NWE-D)

(NWE-D)

FSMC_NWE high to FSMC_D[15:0] invalid

8T

–1

-

ns

t

h

HCLK

(D-NWE)

FSMC_D[15:0] valid before FSMC_NWE high

13T

–1

-

ns

t

d

HCLK

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

158/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Table 84. Switching characteristics for PC Card/CF read and write cycles

(1)(2)

in I/O space

Symbol

Parameter

Min

Max

Unit

t

FSMC_NIOWR low width

8T

–1

-

ns

w(NIOWR)

HCLK

FSMC_NIOWR low to FSMC_D[15:0] valid

-

5T

– 1

ns

t

v(NIOWR-D)

HCLK

FSMC_NIOWR high to FSMC_D[15:0] invalid

8T

– 2

-

ns

t

h(NIOWR-D)

HCLK

t

FSMC_NCE4_1 low to FSMC_NIOWR valid

-

5T

+ 2.5

ns

d(NCE4_1-NIOWR)

HCLK

FSMC_NCEx high to FSMC_NIOWR invalid

5T

–1.5

-

ns

t

h(NCEx-NIOWR)

HCLK

FSMC_NCEx low to FSMC_NIORD valid

-

5T

+ 2

ns

t

d(NIORD-NCEx)

HCLK

t

FSMC_NCEx high to FSMC_NIORD) valid

5T

– 1.5

-

ns

h(NCEx-NIORD)

HCLK

FSMC_NIORD low width

8T

–0.5

-

ns

t

w(NIORD)

HCLK

FSMC_D[15:0] valid before FSMC_NIORD high

9

-

ns

t

su(D-NIORD)

t

FSMC_D[15:0] valid after FSMC_NIORD high

0

-

ns

d(NIORD-D)

1.

C

= 30 pF.

L

2.

Evaluated by characterization - not tested in production.

NAND controller waveforms and timings

Figure 68 and Figure 69 represent synchronous waveforms, and Table 85 and Table 86

provide the corresponding timings. The results shown in this table are obtained with the

following FSMC configuration:

•

COM.FSMC_SetupTime = 0x01;

•

COM.FSMC_WaitSetupTime = 0x03;

•

COM.FSMC_HoldSetupTime = 0x02;

•

COM.FSMC_HiZSetupTime = 0x01;

•

ATT.FSMC_SetupTime = 0x01;

•

ATT.FSMC_WaitSetupTime = 0x03;

•

ATT.FSMC_HoldSetupTime = 0x02;

•

ATT.FSMC_HiZSetupTime = 0x01;

•

Bank = FSMC_Bank_NAND;

•

MemoryDataWidth = FSMC_MemoryDataWidth_16b;

•

ECC = FSMC_ECC_Enable;

•

ECCPageSize = FSMC_ECCPageSize_512Bytes;

•

TCLRSetupTime = 0;

•

TARSetupTime = 0.

In all timing tables, the

is the HCLK clock period.

T

HCLK

DS8626 Rev 11

159/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

Figure 68. NAND controller waveforms for read access

FMC_NCEx

ALE (FMC_A17)

CLE (FMC_A16)

t

t

h(NOE-ALE)

d(ALE-NOE)

FMC_NWE

t

w(NOE)

FMC_NOE (NRE)

t

t

su(D-NOE)

h(NOE-D)

FMC_D[y:0]

MSv73150V1

1.

y = 7 or 15 depending on the NAND flash memory interface.

Figure 69. NAND controller waveforms for write access

FMC_NCEx

ALE (FMC_A17)

CLE (FMC_A16)

t

t

t

d(ALE-NWE)

w(NWE)

h(NWE-ALE)

FMC_NWE

FMC_NOE (NRE)

t

d(D-NWE)

t

t

v(NWE-D)

h(NWE-D)

FMC_D[y:0]

MSv73151V1

1.

y = 7 or 15 depending on the NAND flash memory interface.

(1)

Table 85. Switching characteristics for NAND Flash read cycles

Symbol

Parameter

Min

Max

Unit

–

4T

HCLK

t

FSMC_NOE low width

+ 3

ns

4T

w(N0E)

HCLK

0.5

t

FSMC_D[15-0] valid data before FSMC_NOE high

10

-

ns

su(D-NOE)

FSMC_D[15-0] valid data after FSMC_NOE high

0

-

ns

t

h(NOE-D)

t

FSMC_ALE valid before FSMC_NOE low

-

3T

ns

d(ALE-NOE)

HCLK

FSMC_NWE high to FSMC_ALE invalid

3T

– 2

-

ns

t

h(NOE-ALE)

HCLK

1.

C

= 30 pF.

L

160/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

(1)

Table 86. Switching characteristics for NAND Flash write cycles

Symbol

Parameter

Min

Max

Unit

t

FSMC_NWE low width

4T

–1

4T

+ 3

ns

w(NWE)

HCLK

HCLK

FSMC_NWE low to FSMC_D[15-0] valid

-

0

ns

t

v(NWE-D)

FSMC_NWE high to FSMC_D[15-0] invalid

3T

–2

-

ns

t

h(NWE-D)

HCLK

t

FSMC_D[15-0] valid before FSMC_NWE high

5T

–3

-

ns

d(D-NWE)

HCLK

FSMC_ALE valid before FSMC_NWE low

-

3T

ns

t

d(ALE-NWE)

HCLK

FSMC_NWE high to FSMC_ALE invalid

3T

–2

-

ns

t

h(NWE-ALE)

HCLK

1.

C

= 30 pF.

L

6.3.27

Camera interface (DCMI) timing specifications

Unless otherwise specified, the parameters given in Table 87 for DCMI are derived from

tests performed under the ambient temperature, f

frequency and V

supply voltage

HCLK

DD

summarized in Table 13, with the following configuration:

•

PCK polarity: falling

•

VSYNC and HSYNC polarity: high

•

Data format: 14 bits

Figure 70. DCMI timing diagram

1/DCMI_PIXCLK

DCMI_PIXCLK

t

t

h(HSYNC)

su(HSYNC)

DCMI_HSYNC

t

t

su(VSYNC)

h(HSYNC)

DCMI_VSYNC

t

t

su(DATA)

h(DATA)

DATA[0:13]

MS32414V2

(1)

Table 87. DCMI characteristics

Symbol

Parameter

Min

Max

Unit

-

Frequency ratio DCMI_PIXCLK/f

-

0.4

-

HCLK

DCMI_PIXCLK

Pixel clock input

-

54

MHz

D

Pixel clock input duty cycle

30

70

%

pixel

DS8626 Rev 11

161/205

190

Electrical characteristics

STM32F405xx, STM32F407xx

(1)

Table 87. DCMI characteristics

(continued)

Symbol

Parameter

Min

Max

Unit

t

Data input setup time

2.5

-

su(DATA)

Data hold time

1

-

t

h(DATA)

t

,

ns

su(HSYNC)

HSYNC/VSYNC input setup time

2

-

t

su(VSYNC)

,

t

h(HSYNC)

HSYNC/VSYNC input hold time

0.5

-

t

h(VSYNC)

1.

Evaluated by characterization - not tested in production.

6.3.28

SD/SDIO MMC card host interface (SDIO) characteristics

Unless otherwise specified, the parameters given in Table 88 are derived from tests

performed under ambient temperature, f

frequency and V

supply voltage conditions

PCLKx

DD

summarized in Table 14 with the following configuration:

•

Output speed is set to OSPEEDRy[1:0] = 10

•

Capacitive load C = 30 pF

•

Measurement points are done at CMOS levels: 0.5V

DD

Refer to Section 6.3.16: I/O port characteristics for more details on the input/output

characteristics.

Figure 71. SDIO high-speed mode

162/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Electrical characteristics

Figure 72. SD default mode

CK

t

t

OVD

OHD

D, CMD

(output)

ai14888

(1)

Table 88. Dynamic characteristics: SD/MMC characteristics

Symbol

Parameter

Conditions

Min

Typ

Max

Unit

f

Clock frequency in data transfer mode

-

0

-

48

MHz

PP

frequency ratio

-

-

-

8/3

-

SDIO_CK/f

PCLK2

Clock low time

f

= 48 MHz

8.5

9

-

t

W(CKL)

PP

ns

Clock high time

f

= 48 MHz

8.3

10

-

t

W(CKH)

PP

CMD, D inputs (referenced to CK) in MMC and SD HS mode

Input setup time HS

f

= 48 MHz

3

-

-

t

ISU

PP

ns

Input hold time HS

f

= 48 MHz

0

-

-

t

IH

PP

CMD, D outputs (referenced to CK) in MMC and SD HS mode

Output valid time HS

f

= 48 MHz

-

4.5

6

t

OV

PP

ns

Output hold time HS

f

= 48 MHz

1

-

-

t

OH

PP

CMD, D inputs (referenced to CK) in SD default mode

Input setup time SD

f

= 24 MHz

1.5

-

-

t

ISUD

PP

ns

Input hold time SD

f

= 24 MHz

0.5

-

-

t

IHD

PP

CMD, D outputs (referenced to CK) in SD default mode

Output valid default time SD

f

= 24 MHz

-

4.5

7

t

OVD

PP

ns

Output hold default time SD

f

= 24 MHz

0.5

-

-

t

OHD

PP

1.

Evaluated by characterization - not tested in production.

6.3.29

RTC characteristics

Table 89. RTC characteristics

Symbol

Parameter

Conditions

Min

Max

Any read/write operation

/RTCCLK

frequency ratio

4

-

-

f

PCLK1

from/to an RTC register

DS8626 Rev 11

163/205

190

Package information

STM32F405xx, STM32F407xx

7

Package information

In order to meet environmental requirements, ST offers these devices in different grades of

ECOPACK

packages, depending on their level of environmental compliance. ECOPACK

specifications, grade definitions and product status are available at: www.st.com.

ECOPACK is an ST trademark.

7.1

Device marking

Refer to technical note “Reference device marking schematics for STM32 microcontrollers

and microprocessors” (TN1433), available on www.st.com, for the location of pin 1 / ball A1

as well as the location and orientation of the marking areas versus pin 1 / ball A1.

Parts marked as “ES”, “E”, or accompanied by an engineering sample notification letter, are

not yet qualified and therefore not approved for use in production. ST is not responsible for

any consequences resulting from such use. In no event will ST be liable for the customer

using any of these engineering samples in production. ST’s Quality department must be

contacted prior to any decision to use these engineering samples to run a qualification

activity.

A WLCSP simplified marking example (if any) is provided in the corresponding package

information subsection.

164/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

7.2

WLCSP90 package information

Figure 73. WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package outline

bbb

e1

A1 ball location

Z

e

10

1

A

e2

e

Detail A

J

G

A3

F

A2

BOTTOM VIEW

BUMP SIDE

A

SIDE VIEW

A3

A2

A

FRONT VIEW

D

Bump

eee

A1

E

A1 orientation

b

Seating plane

Z

reference

Detail A

aaa

(4X)

ccc

Z X Y

ddd

Z

(Rotated 90°)

TOP VIEW

WAFER BACK SIDE

A0JW_ME_V4

1.

Drawing is not to scale.

DS8626 Rev 11

165/205

190

Package information

STM32F405xx, STM32F407xx

Table 90. WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package mechanical data

(1)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

0.540

0.570

0.600

0.0213

0.0224

0.0236

A1

-

0.190

-

-

0.0075

-

A2

-

0.380

-

-

0.0150

-

(2)

A3

-

0.025

-

-

0.0010

-

(3)

0.240

0.270

0.300

0.0094

0.0106

0.0118

b

D

4.188

4.223

4.258

0.1649

0.1663

0.1676

E

3.934

3.969

4.004

0.1549

0.1563

0.1576

e

-

0.400

-

-

0.0157

-

e1

-

3.600

-

-

0.1417

-

e2

-

3.200

-

-

0.1260

-

F

-

0.3115

-

-

0.0123

-

G

-

0.3845

-

-

0.0151

-

aaa

-

0.100

-

-

0.0039

-

bbb

-

0.100

-

-

0.0039

-

ccc

-

0.100

-

-

0.0039

-

ddd

-

0.050

-

-

0.0020

-

eee

-

0.050

-

-

0.0020

-

1.

Values in inches are converted from mm and rounded to 4 decimal digits.

2.

Back side coating.

3.

Dimension is measured at the maximum bump diameter parallel to primary datum Z.

Figure 74. WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch wafer level chip scale

package recommended footprint

Dpad

Dsm

MS18965V2

166/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Table 91. WLCSP90 recommended PCB design rules

Dimension

Recommended values

Pitch

0.4 mm

260 µm max. (circular)

Dpad

220 µm recommended

Dsm

300 µm min. (for 260 µm diameter pad)

PCB pad design

Non-solder mask defined via underbump allowed

Device marking for WLCSP90

The following figure gives an example of topside marking and ball A1 position identifier

location.

The printed markings may differ depending on the supply chain.

Other optional marking or inset/upset marks, which depend on supply chain operations, are

not indicated below.

Figure 75. WLCSP90 marking example (package top view)

Product identification(1)

F405OEB

Revision code

Date code

R

Y

WW

Ball A1

indentifer

MSv36120V1

1.

Parts marked as “ES”, “E” or accompanied by an Engineering Sample notification letter, are not yet

qualified and therefore not yet ready to be used in production and any consequences deriving from such

usage will not be at ST charge. In no event, ST will be liable for any customer usage of these engineering

samples in production. ST Quality has to be contacted prior to any decision to use these Engineering

Samples to run qualification activity.

DS8626 Rev 11

167/205

190

Package information

STM32F405xx, STM32F407xx

7.3

LQFP64 package information (5W)

This LQFP is 64-pin, 10 x 10 mm low-profile quad flat package.

(15)

Figure 76. LQFP64 - Outline

BOTTOM VIEW

2

1

(2)

R1

R2

H

B

GAUGE PLANE

SECTION B-B

D 1/4

(6)

S

0.25

B

L

E 1/4

3

4x N/4 TIPS

(L1)

aaa C A-B D

(1)

(11)

bbb H A-B D 4x

SECTION A-A

(N – 4)x e

(13)

C

A

0.05

A2 A1

(12)

ddd

C A-B D

b

ccc

C

D

(4)

D1

(5) (2)

(9)

(11)

b

(3)

D

WITH PLATING

(10)

(4)

N

E 1/4

1

2

3

A

B

(3)

(3)

(5)

(6)

D 1/4

(2)

(11)

(11)

c

c1

E1

E

b1

BASE METAL

(11)

A

A

SECTION B-B

(Section A-A)

5W_LQFP64_ME_V1

TOP VIEW

168/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Table 92. LQFP64 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.60

-

-

0.0630

(12)

A1

0.05

-

0.15

0.0020

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0570

(9)(11)

0.17

0.22

0.27

0.0067

0.0087

0.0106

b

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0091

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

0.09

-

0.16

0.0035

-

0.0063

c1

(4)

12.00 BSC

0.4724 BSC

D

(2)(5)

10.00 BSC

0.3937 BSC

D1

(4)

12.00 BSC

0.4724 BSC

E

(2)(5)

10.00 BSC

0.3937 BSC

E1

e

0.50 BSC

0.1970 BSC

L

0.45

0.60

0.75

0.0177

0.0236

0.0295

L1

1.00 REF

0.0394 REF

(13)

64

N

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

(1)

0.20

0.0079

aaa

(1)

0.20

0.0079

bbb

(1)

0.08

0.0031

ccc

(1)

0.08

0.0031

ddd

DS8626 Rev 11

169/205

190

Package information

STM32F405xx, STM32F407xx

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The Top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is “0.25 mm” per side. D1 and E1 are Maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All Dimensions are in millimeters.

8.

No intrusion allowed inwards the leads.

9.

Dimension “b” does not include dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum “b” dimension by more than 0.08 mm.

Dambar cannot be located on the lower radius or the foot. Minimum space between

protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch packages.

10.

Exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead between 0.10 mm and 0.25 mm

from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

“N” is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to 4 decimal digits.

15.

Drawing is not to scale.

170/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

7.4

LQFP100 package information (1L)

This LQFP is a 100-pin, 14 x 14 mm low-profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 77. LQFP100 - Outline

ș

ș

2

(2)

R1

R2

H

(6)

D1/4

B

GAUGE PLANE

SECTION B-B

S

ș

B

E1/4

ș

L

4x N/4 TIPS

4x

(L1)

A-B

aaa

C

D

D

bbb H A-B

(1)

(11)

SECTION A-A

BOTTOM VIEW

(N-4) x e

(13)

C

A

(9)

(11)

b

0.05

WITH PLATING

ccc

b

C

A2 A1

aaa

C A-BD

(12)

SIDE VIEW

D

(4)

c

c1

(11)

(11)

D1

(2)

(5)

D

(3)

(4)

(10)

N

b1

BASE METAL

(11)

1

2

E1/4

3

SECTION B-B

D1/4

(2)

(6)

B

A

(5)

E1

E

SECTION A-A

A

A

TOP VIEW

1L_LQFP100_ME_V3

DS8626 Rev 11

171/205

190

Package information

STM32F405xx, STM32F407xx

Table 93. LQFP100 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

1.50

1.60

-

0.0590

0.0630

(12)

A1

0.05

-

0.15

0.0019

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0570

(9)(11)

b

0.17

0.22

0.27

0.0067

0.0087

0.0106

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0090

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

c1

0.09

-

0.16

0.0035

-

0.0063

(4)

16.00 BSC

0.6299 BSC

D

(2)(5)

14.00 BSC

0.5512 BSC

D1

(4)

E

16.00 BSC

0.6299 BSC

(2)(5)

14.00 BSC

0.5512 BSC

E1

e

0.50 BSC

0.0197 BSC

L

0.45

0.60

0.75

0.177

0.0236

0.0295

(1)(11)

1.00

-

0.0394

-

L1

(13)

N

100

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

(1)

0.20

0.0079

aaa

(1)

0.20

0.0079

bbb

(1)

ccc

0.08

0.0031

(1)

0.08

0.0031

ddd

172/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at the seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is 0.25 mm per side. D1 and E1 are maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All dimensions are in millimeters.

8.

No intrusion is allowed inwards the leads.

9.

Dimension b does not include a dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum b dimension by more than 0.08 mm.

The dambar cannot be located on the lower radius or the foot. The minimum space

between the protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch

packages.

10.

The exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead that is between 0.10 mm and

0.25 mm from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

N is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to four decimal digits.

15.

Drawing is not to scale.

Figure 78. LQFP100 - Footprint example

75

51

76

0.5

0.3

16.7

14.3

50

100

26

1.2

1

25

12.3

16.7

1L_LQFP100_FP_V1

1.

Dimensions are expressed in millimeters.

DS8626 Rev 11

173/205

190

Package information

STM32F405xx, STM32F407xx

7.5

LQFP144 package information (1A)

This LQFP is a 144-pin, 20 x 20 mm low-profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 79. LQFP144 - Outline

BOTTOM VIEW

2

1

(2)

R1

R2

H

B

GAUGE PLANE

SECTION B-B

(6)

D 1/4

S

0.25

B

L

3

E 1/4

(L1)

(1)

(11)

4x N/4 TIPS

aaa C A-B D

SECTION A-A

bbb

4x

H A-B D

(N-4)x e

C

A

(12)

0.05

ddd

C A-B D

b

A2

A1

ccc

C

D

(4)

D1

(2)

(5)

(3)

D

(10)

(9)

(11)

(4)

N

b

WITH PLATING

1

2

E 1/4

3

(11)

(11)

c

c1

(6)

D 1/4

(2)

(3)

A

B

(3)

(5)

E1

E

b1

BASE METAL

(11)

SECTION B-B

A

A

(Section A-A)

TOP VIEW

1A_LQFP144_ME_V2

174/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Table 94. LQFP144 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.60

-

-

0.0630

(12)

A1

0.05

-

0.15

0.0020

-

0.0059

A2

1.35

1.40

1.45

0.0531

0.0551

0.0571

(9)(11)

b

0.17

0.22

0.27

0.0067

0.0087

0.0106

(11)

0.17

0.20

0.23

0.0067

0.0079

0.0090

b1

(11)

0.09

-

0.20

0.0035

-

0.0079

c

(11)

c1

0.09

-

0.16

0.0035

-

0.0063

(4)

22.00 BSC

0.8661 BSC

D

(2)(5)

20.00 BSC

0.7874 BSC

D1

(4)

E

22.00 BSC

0.8661 BSC

(2)(5)

20.00 BSC

0.7874 BSC

E1

e

0.50 BSC

0.0197 BSC

L

0.45

0.60

0.75

0.0177

0.0236

0.0295

L1

1.00 REF

0.0394 REF

(13)

N

144

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.08

-

-

0.0031

-

-

R2

0.08

-

0.20

0.0031

-

0.0079

S

0.20

-

-

0.0079

-

-

aaa

0.20

0.0079

bbb

0.20

0.0079

ccc

0.08

0.0031

ddd

0.08

0.0031

DS8626 Rev 11

175/205

190

Package information

STM32F405xx, STM32F407xx

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at the seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is 0.25 mm per side. D1 and E1 are maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All dimensions are in millimeters.

8.

No intrusion is allowed inwards the leads.

9.

Dimension b does not include a dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum b dimension by more than 0.08 mm.

The dambar cannot be located on the lower radius or the foot. The minimum space

between the protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch

packages.

10.

The exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead that is between 0.10 mm and

0.25 mm from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

N is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to four decimal digits.

15.

Drawing is not to scale.

176/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Figure 80. LQFP144 - Footprint example

1.35

108

73

109

72

0.35

0.50

19.90

17.85

22.60

144

37

1

36

19.90

22.60

1A_LQFP144_FP

1.

Dimensions are expressed in millimeters.

DS8626 Rev 11

177/205

190

Package information

STM32F405xx, STM32F407xx

7.6

UFBGA(176+25) package information (A0E7)

This UFBGA is a 176+25-ball, 10 x 10 mm, 0.65 mm pitch, ultra fine pitch ball grid array

package.

Figure 81. UFBGA(176+25) - Outline

Seating plane

A4

C

ddd

C

A

A2

A1

A3

b

A1 ball

A

A1 ball

index

E

identifier

area

E1

e

F

A

F

D

D1

e

B

R

15

1

Øb (176 + 25  balls)

TOP VIEW

BOTTOM VIEW

Ø

eee

M

C

A

B

C

M

Ø

fff

A0E7_ME_V10

1.

Drawing is not to scale.

Table 95. UFBGA(176+25) - Mechanical data

(1)

millimeters

inches

Symbol

Min.

Typ.

Max.

Min.

Typ.

Max.

A

-

-

0.600

-

-

0.0236

A1

0.050

0.080

0.110

0.0020

0.0031

0.0043

A2

-

0.450

-

-

0.0177

-

A3

-

0.130

-

-

0.0051

-

A4

-

0.320

-

-

0.0126

-

b

0.240

0.290

0.340

0.0094

0.0114

0.0134

D

9.850

10.000

10.150

0.3878

0.3937

0.3996

D1

-

9.100

-

-

0.3583

-

E

9.850

10.000

10.150

0.3878

0.3937

0.3996

E1

-

9.100

-

-

0.3583

-

e

-

0.650

-

-

0.0256

-

F

-

0.450

-

-

0.0177

-

ddd

-

-

0.080

-

-

0.0031

178/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Table 95. UFBGA(176+25) - Mechanical data (continued)

(1)

millimeters

inches

Symbol

Min.

Typ.

Max.

Min.

Typ.

Max.

eee

-

-

0.150

-

-

0.0059

fff

-

-

0.050

-

-

0.0020

1.

Values in inches are converted from mm and rounded to 4 decimal digits.

Figure 82. UFBGA(176+25) - Footprint example

Dpad

Dsm

BGA_WLCSP_FT_V1

Table 96. UFBGA(176+25) - Example of PCB design rules (0.65 mm pitch BGA)

Dimension

Values

Pitch

0.65 mm

Dpad

0.300 mm

0.400 mm typ. (depends on the soldermask

Dsm

registration tolerance)

Stencil opening

0.300 mm

Stencil thickness

Between 0.100 mm and 0.125 mm

Pad trace width

0.100 mm

DS8626 Rev 11

179/205

190

Package information

STM32F405xx, STM32F407xx

7.7

LQFP176 package information (1T)

This LQFP is a 176-pin, 24 x 24 mm, 0.5 mm pitch, low profile quad flat package.

Note:

See list of notes in the notes section.

(15)

Figure 83. LQFP176 - Outline

1

2

ș

ș

R1

(2)

R2

H

B

(See SECTION B-B)

GAUGE PLANE

(6)

0.25

D1/4

ș

S

B

L

ș

E1/4

(L1)

4x

4x N/4 TIPS

(1)

(11)

A-B

aaa

C

D

D

bbb H A-B

SECTION A-A

BOTTOM VIEW

A2

0.05

(N-4) x e

C

A

(12)

ccc

C

A1

ddd

C A-BD

b

SIDE VIEW

(4)

D

D1

(2)

(5)

(9)

(11)

D

(10)

N

(4)

b

WITH PLATING

E1/4

c

c1

D1/4

(11)

(11)

(6)

(5)

(2)

A

B

E

E1

b1

BASE METAL

(11)

SECTION A-A

A

A

SECTION B-B

TOP VIEW

1T_LQFP176_ME_V2

180/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Table 97. LQFP176 - Mechanical data

(14)

millimeters

inches

Symbol

Min

Typ

Max

Min

Typ

Max

A

-

-

1.600

-

-

0.0630

(12)

A1

0.050

-

0.150

0.0020

-

0.0059

A2

1.350

1.400

1.450

0.0531

0.0551

0.0571

(9)(11)

0.170

0.220

0.270

0.0067

0.0087

0.0106

b

(11)

0.170

0.200

0.230

0.0067

0.0079

0.0091

b1

(11)

0.090

-

0.200

0.0035

-

0.0079

c

(11)

0.090

-

0.160

0.0035

-

0.063

c1

(4)

26.000

1.0236

D

(2)(5)

24.000

0.9449

D1

(4)

26.000

0.0197

E

(2)(5)

24.000

0.9449

E1

e

0.500

0.1970

L

0.450

0.600

0.750

0.0177

0.0236

0.0295

(1)(11)

1

0.0394 REF

L1

(13)

176

N

θ

0°

3.5°

7°

0°

3.5°

7°

θ1

0°

-

-

0°

-

-

θ2

10°

12°

14°

10°

12°

14°

θ3

10°

12°

14°

10°

12°

14°

R1

0.080

-

-

0.0031

-

-

R2

0.080

-

0.200

0.0031

-

0.0079

S

0.200

-

-

0.0079

-

-

(1)

0.200

0.0079

aaa

(1)

0.200

0.0079

bbb

(1)

0.080

0.0031

ccc

(1)

0.080

0.0031

ddd

DS8626 Rev 11

181/205

190

Package information

STM32F405xx, STM32F407xx

Notes:

1.

Dimensioning and tolerancing schemes conform to ASME Y14.5M-1994.

2.

The top package body size may be smaller than the bottom package size by as much

as 0.15 mm.

3.

Datums A-B and D to be determined at datum plane H.

4.

To be determined at the seating datum plane C.

5.

Dimensions D1 and E1 do not include mold flash or protrusions. Allowable mold flash

or protrusions is 0.25 mm per side. D1 and E1 are maximum plastic body size

dimensions including mold mismatch.

6.

Details of pin 1 identifier are optional but must be located within the zone indicated.

7.

All dimensions are in millimeters.

8.

No intrusion is allowed inwards the leads.

9.

Dimension b does not include a dambar protrusion. Allowable dambar protrusion shall

not cause the lead width to exceed the maximum b dimension by more than 0.08 mm.

The dambar cannot be located on the lower radius or the foot. The minimum space

between the protrusion and an adjacent lead is 0.07 mm for 0.4 mm and 0.5 mm pitch

packages.

10.

The exact shape of each corner is optional.

11.

These dimensions apply to the flat section of the lead that is between 0.10 mm and

0.25 mm from the lead tip.

12.

A1 is defined as the distance from the seating plane to the lowest point on the package

body.

13.

N is the number of terminal positions for the specified body size.

14.

Values in inches are converted from mm and rounded to four decimal digits.

15.

Drawing is not to scale.

182/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Package information

Figure 84. LQFP176 - Footprint example

176

0.5

1

0.3

26.7

21.8

44

89

1.2

133

132

45

88

1.2

21.8

26.7

1T_FP_V1

1.

Dimensions are expressed in millimeters.

DS8626 Rev 11

183/205

190

Package information

STM32F405xx, STM32F407xx

7.8

Thermal characteristics

The maximum chip-junction temperature, T

max, in degrees Celsius, may be calculated

J

using the following equation:

Θ

T

max = T

max + (P

max x

)

J

A

D

JA

Where:

•

°

T

max is the maximum ambient temperature in

C,

A

•

Θ

°

is the package junction-to-ambient thermal resistance, in

C/W,

JA

•

P

max is the sum of P

max and P

max (P

max = P

max + P

max),

D

INT

I/O

D

INT

I/O

•

P

max is the product of I

and

V

, expressed in Watts. This is the maximum chip

INT

DD

DD

internal power.

P

max represents the maximum power dissipation on output pins where:

I/O

Σ

Σ

P

max

=

(V

× I

) +

((V

– V

) × I

),

I/O

OL

OL

DD

OH

OH

taking into account the actual V

/ I

and V

/ I

of the I/Os at low and high level in the

OL

OL

OH

OH

application.

Table 98. Package thermal characteristics

Symbol

Parameter

Value

Unit

Thermal resistance junction-ambient

46

LQFP64 - 10 × 10 mm / 0.5 mm pitch

Thermal resistance junction-ambient

43

LQFP100 - 14 × 14 mm / 0.5 mm pitch

Thermal resistance junction-ambient

40

LQFP144 - 20 × 20 mm / 0.5 mm pitch

Θ

°C/W

JA

Thermal resistance junction-ambient

38

LQFP176 - 24 × 24 mm / 0.5 mm pitch

Thermal resistance junction-ambient

39

UFBGA176 - 10× 10 mm / 0.65 mm pitch

Thermal resistance junction-ambient

38.1

WLCSP90 - 0.400 mm pitch

Reference document

JESD51-2 Integrated Circuits Thermal Test Method Environment Conditions - Natural

Convection (Still Air). Available from www.jedec.org.

184/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Ordering information

8

Ordering information

Example:

STM32

F

405 R

E

T

6

xxx

Device family

STM32 = Arm-based 32-bit microcontroller

Product type

F = general-purpose

Device subfamily

405 = STM32F40xxx, connectivity

407= STM32F40xxx, connectivity, camera interface, Ethernet

Pin count

R = 64 pins

O = 90 pins

V = 100 pins

Z = 144 pins

I = 176 pins

Flash memory size

E = 512 Kbytes of flash memory

G = 1024 Kbytes of flash memory

Package

T = LQFP

H = UFBGA

Y = WLCSP

Temperature range

6 = Industrial temperature range, –40 to 85 °C.

7 = Industrial temperature range, –40 to 105 °C.

Options

xxx = programmed parts

TR = tape and reel

For a list of available options (speed, package, etc.) or for further information on any aspect

of this device, please contact your nearest ST sales office.

DS8626 Rev 11

185/205

190

Application block diagrams

STM32F405xx, STM32F407xx

Appendix A

Application block diagrams

A.1

USB OTG full speed (FS) interface solutions

Figure 85. USB controller configured as peripheral-only and used

in Full speed mode

V

DD

5V to V

DD

(1)

Volatge regulator

STM32F4xx

VBUS

DM

PA11//PB14

OSC_IN

DP

PA12/PB15

V

SS

OSC_OUT

USB Std-B connector

MS19000V5

1.

External voltage regulator only needed when building a V

powered device.

BUS

2.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

Figure 86. USB controller configured as host-only and used in full speed mode

V

DD

EN

GPIO

Current limiter

5 V Pwr

(1)

power switch

Overcurrent

GPIO+IRQ

STM32F4xx

VBUS

DM

PA11//PB14

OSC_IN

DP

PA12/PB15

V

SS

OSC_OUT

USB Std-A connector

MS19001V4

1.

The current limiter is required only if the application has to support a V

powered device. A basic power

BUS

switch can be used if 5 V are available on the application board.

2.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

186/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Application block diagrams

Figure 87. USB controller configured in dual mode and used in full speed mode

V

DD

5 V to V

DD

(1)

voltage regulator

V

DD

EN

GPIO

Current limiter

5 V Pwr

(2)

power switch

Overcurrent

GPIO+IRQ

STM32F4xx

VBUS

PA9/PB13

DM

PA11/PB14

OSC_IN

DP

PA12/PB15

(3)

ID

PA10/PB12

OSC_OUT

V

micro-AB connector

SS

USB

MS19002V3

1.

External voltage regulator only needed when building a V

powered device.

BUS

powered device. A basic power

2.

The current limiter is required only if the application has to support a V

BUS

switch can be used if 5 V are available on the application board.

3.

The ID pin is required in dual role only.

4.

The same application can be developed using the OTG HS in FS mode to achieve enhanced performance

thanks to the large Rx/Tx FIFO and to a dedicated DMA controller.

DS8626 Rev 11

187/205

190

Application block diagrams

STM32F405xx, STM32F407xx

A.2

USB OTG high speed (HS) interface solutions

Figure 88. USB controller configured as peripheral, host, or dual-mode

and used in high speed mode

STM32F4xx

DP

FS PHY

not connected

DM

USB HS

OTG Ctrl

DP

ULPI_CLK

DM

ULPI_D[7:0]

(2)

ID

USB

ULPI_DIR

connector

V

ULPI

BUS

ULPI_STP

V

SS

ULPI_NXT

High speed

OTG PHY

(1)

24 or 26 MHz XT

MCO1 or MCO2

XI

XT1

PLL

MS19005V2

1.

It is possible to use MCO1 or MCO2 to save a crystal. It is however not mandatory to clock the

STM32F40xxx with a 24 or 26 MHz crystal when using USB HS. The above figure only shows an example

of a possible connection.

2.

The ID pin is required in dual role only.

188/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Application block diagrams

A.3

Ethernet interface solutions

Figure 89. MII mode using a 25 MHz crystal

STM32

MII_TX_CLK

MCU

MII_TX_EN

Ethernet

Ethernet

MII_TXD[3:0]

MAC 10/100

MII_CRS

MII

MII_COL

= 15 pins

(1)

HCLK

MII_RX_CLK

MII + MDC

= 17 pins

MII_RXD[3:0]

MII_RX_DV

IEEE1588 PTP

Timer

MII_RX_ER

input

PHY 10/100

trigger

Timestamp

MDIO

TIM2

comparator

MDC

(2)

PPS_OUT

HCLK

PLL

XTAL

OSC

25 MHz

PHY_CLK 25 MHz

MCO1/MCO2

XT1

MS19968V1

1.

f

must be greater than 25 MHz.

HCLK

2.

Pulse per second when using IEEE1588 PTP optional signal.

Figure 90. RMII with a 50 MHz oscillator

STM32

Ethernet

PHY 10/100

MCU

RMII_TX_EN

Ethernet

RMII_TXD[1:0]

MAC 10/100

RMII

RMII_RXD[1:0]

(1)

HCLK

= 7 pins

RMII_CRX_DV

RMII + MDC

RMII_REF_CLK

= 9 pins

IEEE1588 PTP

MDIO

Timer

input

MDC

trigger

Timestamp

TIM2

comparator

/2 or /20

2.5 or 25 MHz

50 MHz

synchronous

HCLK

OSC

PLL

50 MHz

50 MHz

PHY_CLK    50 MHz

XT1

MS19969V1

1.

f

must be greater than 25 MHz.

HCLK

DS8626 Rev 11

189/205

190

Application block diagrams

STM32F405xx, STM32F407xx

Figure 91. RMII with a 25 MHz crystal and PHY with PLL

STM32F

Ethernet

PHY 10/100

MCU

RMII_TX_EN

Ethernet

RMII_TXD[1:0]

MAC 10/100

RMII_RXD[1:0]

RMII

(1)

HCLK

= 7 pins

RMII_CRX_DV

REF_CLK

RMII + MDC

RMII_REF_CLK

IEEE1588 PTP

= 9 pins

MDIO

Timer

input

MDC

trigger

Timestamp

TIM2

comparator

/2 or /20

2.5 or 25 MHz

50 MHz

synchronous

PLL

HCLK

XTAL

PLL

25 MHz

XT1

PHY_CLK   25 MHz

OSC

MCO1/MCO2

MS19970V1

1.

f

must be greater than 25 MHz.

HCLK

2.

The 25 MHz (PHY_CLK) must be derived directly from the HSE oscillator, before the PLL block.

190/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Important security notice

9

Important security notice

The STMicroelectronics group of companies (ST) places a high value on product security,

which is why the ST product(s) identified in this documentation may be certified by various

security certification bodies and/or may implement our own security measures as set forth

herein. However, no level of security certification and/or built-in security measures can

guarantee that ST products are resistant to all forms of attacks. As such, it is the

responsibility of each of ST's customers to determine if the level of security provided in an

ST product meets the customer needs both in relation to the ST product alone, as well as

when combined with other components and/or software for the customer end product or

application. In particular, take note that:

•

ST products may have been certified by one or more security certification bodies, such

as Platform Security Architecture (www.psacertified.org) and/or Security Evaluation

standard for IoT Platforms (www.trustcb.com). For details concerning whether the ST

product(s) referenced herein have received security certification along with the level

and current status of such certification, either visit the relevant certification standards

website or go to the relevant product page on www.st.com for the most up to date

information. As the status and/or level of security certification for an ST product can

change from time to time, customers should re-check security certification status/level

as needed. If an ST product is not shown to be certified under a particular security

standard, customers should not assume it is certified.

•

Certification bodies have the right to evaluate, grant and revoke security certification in

relation to ST products. These certification bodies are therefore independently

responsible for granting or revoking security certification for an ST product, and ST

does not take any responsibility for mistakes, evaluations, assessments, testing, or

other activity carried out by the certification body with respect to any ST product.

•

Industry-based cryptographic algorithms (such as AES, DES, or MD5) and other open

standard technologies which may be used in conjunction with an ST product are based

on standards which were not developed by ST. ST does not take responsibility for any

flaws in such cryptographic algorithms or open technologies or for any methods which

have been or may be developed to bypass, decrypt or crack such algorithms or

technologies.

•

While robust security testing may be done, no level of certification can absolutely

guarantee protections against all attacks, including, for example, against advanced

attacks which have not been tested for, against new or unidentified forms of attack, or

against any form of attack when using an ST product outside of its specification or

intended use, or in conjunction with other components or software which are used by

customer to create their end product or application. ST is not responsible for resistance

against such attacks. As such, regardless of the incorporated security features and/or

any information or support that may be provided by ST, each customer is solely

responsible for determining if the level of attacks tested for meets their needs, both in

relation to the ST product alone and when incorporated into a customer end product or

application.

•

All security features of ST products (inclusive of any hardware, software,

documentation, and the like), including but not limited to any enhanced security

features added by ST, are provided on an "AS IS" BASIS. AS SUCH, TO THE EXTENT

PERMITTED BY APPLICABLE LAW, ST DISCLAIMS ALL WARRANTIES, EXPRESS

OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE, unless the

applicable written and signed contract terms specifically provide otherwise.

DS8626 Rev 11

191/205

191

Revision history

STM32F405xx, STM32F407xx

10

Revision history

Table 99. Document revision history

Date

Revision

Changes

15-Sep-2011

1

Initial release.

Added WLCSP90 package on cover page.

Renamed USART4 and USART5 into UART4 and UART5,

respectively.

Updated number of USB OTG HS and FS in Table 3: STM32F405xx

and STM32F407xx: features and peripheral counts.

Updated Figure 3: Compatible board design between

STM32F10xx/STM32F2/STM32F40xxx for LQFP144 package and

Figure 4: Compatible board design between STM32F2 and

STM32F40xxx  for LQFP176 and BGA176 packages, and removed

note 1 and 2.

Updated Section 3.0.9: Flexible static memory controller (FSMC).

Modified I/Os used to reprogram the Flash memory for CAN2 and USB

OTG FS in Section 3.0.13: Boot modes.

Updated note in Section 3.0.14: Power supply schemes.

PDR_ON no more available on LQFP100 package. Updated Section

3.0.16: Voltage regulator. Updated condition to obtain a minimum

supply voltage of 1.7 V in the whole document.

Renamed USART4/5 to UART4/5 and added LIN and IrDA feature for

UART4 and UART5 in Table 6: USART feature comparison.

Removed support of I2C for OTG PHY in Section 3.0.30: Universal

serial bus on-the-go full-speed (OTG_FS).

24-Jan-2012

2

Added Table 7: Legend/abbreviations used in the pinout table.

Table 8: STM32F40xxx pin and ball definitions: replaced VSS_3,

VSS_4, and VSS_8 by VSS; reformatted Table 8: STM32F40xxx pin

and ball definitions to better highlight I/O structure, and alternate

functions versus additional functions; signal corresponding to

LQFP100 pin 99 changed from PDR_ON to VSS; EVENTOUT added

in the list of alternate functions for all I/Os; ADC3_IN8 added as

alternate function for PF10; FSMC_CLE and FSMC_ALE added as

alternate functions for PD11 and PD12, respectively; PH10 alternate

function TIM15_CH1_ETR renamed TIM5_CH1; updated PA4 and PA5

I/O structure to TTa.

Removed OTG_HS_SCL, OTG_HS_SDA, OTG_FS_INTN in Table 8:

STM32F40xxx pin and ball definitions and Table 10: Alternate function

mapping.

Changed TCM data RAM to CCM data RAM in Figure 18:

STM32F40xxx memory map.

Added IVDD and IVSS maximum values in Table 14: Current

characteristics.

Added Note 1 related to fHCLK, updated Note 2 in Table 16: General

operating conditions, and added maximum power dissipation values.

Updated Table 17: Limitations depending on the operating power

supply range.

192/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Added V12 in Table 21: Embedded reset and power control block

characteristics.

Updated Table 23: Typical and maximum current consumption in Run

mode, code with data processing   running from Flash memory (ART

accelerator disabled) and Table 22: Typical and maximum current

consumption in Run mode, code with data processing   running from

Flash memory (ART accelerator enabled) or RAM. Added Figure  ,

Figure 25, Figure 26, and Figure 27.

Updated Table 24: Typical and maximum current consumption in Sleep

mode and removed Note 1.

Updated Table 25: Typical and maximum current consumptions in Stop

mode and Table 26: Typical and maximum current consumptions in

Standby mode, Table 27: Typical and maximum current consumptions

in VBAT mode, and Table 29: Switching output I/O current

consumption.

Section : On-chip peripheral current consumption: modified conditions,

and updated Table 30: Peripheral current consumption and Note 2.

Changed fHSE_ext to 50 MHz and tr(HSE)/tf(HSE) maximum value in

Table 32: High-speed external user clock characteristics.

2

Added Cin(LSE) in Table 33: Low-speed external user clock

24-Jan-2012

characteristics.

(continued)

Updated maximum PLL input clock frequency, removed related note,

and deleted jitter for MCO for RMII Ethernet typical value in Table 38:

Main PLL characteristics. Updated maximum PLLI2S input clock

frequency and removed related note in Table 39: PLLI2S (audio PLL)

characteristics.

Updated Section : Flash memory to specify that the devices are

shipped to customers with the Flash memory erased. Updated Table

41: Flash memory characteristics, and added tME in Table 42: Flash

memory programming.

Updated Table 45: EMS characteristics, and Table 46: EMI

characteristics.

Updated Table 58: I2S dynamic characteristics

Updated Figure 45: ULPI timing diagram and Table 64: ULPI timing.

Added tCOUNTER and tMAX_COUNT in Table 54: Characteristics of

TIMx connected to the APB1 domain and Table 55: Characteristics of

TIMx connected to the APB2 domain. Updated Table 67: Dynamic

characteristics: Ethernet MAC signals for RMII.

Removed USB-IF certification in Section : USB OTG FS

characteristics.

DS8626 Rev 11

193/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated Table 63: USB HS clock timing parameters

Updated Table 69: ADC characteristics.

Updated Table 70: ADC accuracy at fADC = 30 MHz.

Updated Note 1 in Table 76: DAC characteristics.

Section 6.3.26: FSMC characteristics: updated Table 77 toTable 88,

changed CL value to 30 pF, and modified FSMC configuration for

asynchronous timings and waveforms. Updated Figure 59:

Synchronous multiplexed PSRAM write timings.

Updated Table 100: Package thermal characteristics.

Appendix A.1: USB OTG full speed (FS) interface solutions: modified

2

24-Jan-2012

Figure 93: USB controller configured as peripheral-only and used in

(continued)

Full speed mode added Note 2, updated Figure 94: USB controller

configured as host-only and used in full speed mode and added Note

2, changed Figure 95: USB controller configured in dual mode and

used in full speed mode and added Note 3.

Appendix A.2: USB OTG high speed (HS) interface solutions: removed

figures USB OTG HS device-only connection in FS mode and USB

OTG HS host-only connection in FS mode, and updated Figure 96:

USB controller configured as peripheral, host, or dual-mode and used

in high speed mode and added Note 2.

Added Appendix A.3: Ethernet interface solutions.

194/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated Figure 5: STM32F40xxx block diagram and Figure 7: Power

supply supervisor interconnection with internal reset OFF

Added SDIO, added notes related to FSMC and SPI/I2S in Table 3:

STM32F405xx and STM32F407xx: features and peripheral counts.

Starting from Silicon revision Z, USB OTG full-speed interface is now

available for all STM32F405xx devices.

Added full information on WLCSP90 package together with

corresponding part numbers.

Changed number of AHB buses to 3.

Modified available Flash memory sizes in Section 3.0.4: Embedded

Flash memory.

Modified number of maskable interrupt channels in Section 3.0.10:

Nested vectored interrupt controller (NVIC).

Updated case of Regulator ON/internal reset ON, Regulator

ON/internal reset OFF, and Regulator OFF/internal reset ON in Section

3.0.16: Voltage regulator.

Updated standby mode description in Section 3.0.19: Low-power

modes.

Added Note 1 below Figure 16: STM32F40xxx UFBGA176 ballout.

Added Note 1 below Figure 17: STM32F40xxx WLCSP90 ballout.

Updated Table 8: STM32F40xxx pin and ball definitions.

Added Table 9: FSMC pin definition.

31-May-2012

3

Removed OTG_HS_INTN alternate function in Table 8: STM32F40xxx

pin and ball definitions and Table 10: Alternate function mapping.

Removed I2S2_WS on PB6/AF5 in Table 10: Alternate function

mapping.

Replaced JTRST by NJTRST, removed ETH_RMII _TX_CLK, and

modified I2S3ext_SD on PC11 in Table 10: Alternate function

mapping.

Added Table 12: register boundary addresses.

Updated Figure 18: STM32F40xxx memory map.

Updated VDDA and VREF+ decoupling capacitor in Figure 21: Power

supply scheme.

Added power dissipation maximum value for WLCSP90 in Table 16:

General operating conditions.

Updated VPOR/PDR in Table 21: Embedded reset and power control

block characteristics.

Updated notes in Table 23: Typical and maximum current consumption

in Run mode, code with data processing   running from Flash memory

(ART accelerator disabled), Table 22: Typical and maximum current

consumption in Run mode, code with data processing   running from

Flash memory (ART accelerator enabled) or RAM, and Table 24:

Typical and maximum current consumption in Sleep mode.

Updated maximum current consumption at TA = 25 °n Table 25:

Typical and maximum current consumptions in Stop mode.

DS8626 Rev 11

195/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

Removed fHSE_ext typical value in Table 32: High-speed external user

clock characteristics. Updated Table 34: HSE 4-26 MHz oscillator

characteristics and Table 35: LSE oscillator characteristics (fLSE =

32.768 kHz).

Added fPLL48_OUT maximum value in Table 38: Main PLL

characteristics.

Modified equation 1 and 2 in Section 6.3.11: PLL spread spectrum

clock generation (SSCG) characteristics.

Updated Table 41: Flash memory characteristics, Table 42: Flash

memory programming, and Table 43: Flash memory programming with

VPP.

Updated Section : Output driving current.

Table 56: I2C characteristics: Note 4 updated and applied to th(SDA) in

Fast mode, and removed note 4 related to th(SDA) minimum value.

3

31-May-2012

(continued)

Updated Table 69: ADC characteristics. Updated note concerning ADC

accuracy vs. negative injection current below Table 70: ADC accuracy

at fADC = 30 MHz.

Added WLCSP90 thermal resistance in Table 100: Package thermal

characteristics.

Updated Table 92: WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch

wafer level chip scale package mechanical data.

Updated Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package outline and Table 97:

UFBGA176+25 ball, 10 × 10 × 0.65 mm pitch, ultra thin fine pitch ball

grid array mechanical data.

Added Figure 91: LQFP176 - 176-pin, 24 x 24 mm low profile quad flat

recommended footprint.

Removed 256 and 768 Kbyte Flash memory density from Table  : .

196/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Modified Note 1 below Table 3: STM32F405xx and STM32F407xx:

features and peripheral counts.

Updated Figure 4 title.

Updated Note 3 below Figure 21: Power supply scheme.

Changed simplex mode into half-duplex mode in Section 3.0.25: Inter-

integrated sound (I2S).

Replaced DAC1_OUT and DAC2_OUT by DAC_OUT1 and

DAC_OUT2, respectively.

Updated pin 36 signal in Figure 15: STM32F40xxx LQFP176 pinout.

Changed pin number from F8 to D4 for PA13 pin in Table 8:

STM32F40xxx pin and ball definitions.

Replaced TIM2_CH1/TIM2_ETR by TIM2_CH1_ETR for PA0 and PA5

pins in Table 10: Alternate function mapping.

Changed system memory into System memory + OTP in Figure 18:

STM32F40xxx memory map.

Added Note 1 below Table 18: VCAP_1/VCAP_2 operating conditions.

Updated IDDA description in Table 76: DAC characteristics.

Removed PA9/PB13 connection to VBUS in Figure 93: USB controller

configured as peripheral-only and used in Full speed mode and Figure

94: USB controller configured as host-only and used in full speed

mode.

Updated SPI throughput on front page and Section 3.0.24: Serial

peripheral interface (SPI)

Updated operating voltages in Table 3: STM32F405xx and

04-Jun-2013

4

STM32F407xx: features and peripheral counts.

Updated note in Section 3.0.14: Power supply schemes

Updated Section 3.0.15: Power supply supervisor

Updated “Regulator ON” paragraph in Section 3.0.16: Voltage

regulator

Removed note in Section 3.0.19: Low-power modes

Corrected wrong reference manual in Section 3.0.28: Ethernet MAC

interface with dedicated DMA and IEEE 1588 support

Updated Table 17: Limitations depending on the operating power

supply range

Updated Table 26: Typical and maximum current consumptions in

Standby mode

Updated Table 27: Typical and maximum current consumptions in

VBAT mode

Updated Table 39: PLLI2S (audio PLL) characteristics

Updated Table 46: EMI characteristics

Updated Table 51: Output voltage characteristics

Updated Table 53: NRST pin characteristics

Updated Table 57: SPI dynamic characteristics

Updated Table 58: I2S dynamic characteristics

Deleted Table 59

Updated Table 64: ULPI timing

Updated Figure 46: Ethernet SMI timing diagram

DS8626 Rev 11

197/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package outline

Updated Table 97: UFBGA176+25 ball, 10 × 10 × 0.65 mm pitch, ultra

thin fine pitch ball grid array mechanical data

Updated Figure 5: STM32F40xxx block diagram

Updated Section 2: Description

Updated footnote (3) in Table 3: STM32F405xx and STM32F407xx:

features and peripheral counts.

Updated Figure 3: Compatible board design between

STM32F10xx/STM32F2/STM32F40xxx for LQFP144 package

Updated Figure 4: Compatible board design between STM32F2 and

STM32F40xxx  for LQFP176 and BGA176 packages

Updated Section 3.0.14: Power supply schemes

Updated Section 3.0.15: Power supply supervisor

Updated Section 3.0.16: Voltage regulator, including figures.

Updated Table 16: General operating conditions, including footnote (2).

Updated Table 17: Limitations depending on the operating power

supply range, including footnote (3).

Updated footnote (1) in Table 69: ADC characteristics.

Updated footnote (2) in Table 70: ADC accuracy at fADC = 30 MHz.

Updated footnote (1) in Table 76: DAC characteristics.

Updated Figure 9: Regulator OFF.

Updated Figure 7: Power supply supervisor interconnection with

4

internal reset OFF.

04-Jun-2013

(continued)

Added Section 3.0.17: Regulator ON/OFF and internal reset ON/OFF

availability.

Updated footnote (2) of Figure 21: Power supply scheme.

Replaced respectively “I2S3S_WS" by "I2S3_WS”, “I2S3S_CK” by

“I2S3_CK” and “FSMC_BLN1” by “FSMC_NBL1” in Table 10: Alternate

function mapping.

Added “EVENTOUT” as alternate function “AF15” for pin PC13, PC14,

PC15, PH0, PH1, PI8 in Table 10: Alternate function mapping

Replaced “DCMI_12” by “DCMI_D12” in Table 8: STM32F40xxx pin

and ball definitions.

Removed the following sentence from Section : I2C interface

characteristics: ”Unless otherwise specified, the parameters

given in Table 56 are derived from tests performed under the

ambient temperature, fPCLK1 frequency and VDD supply

voltage conditions summarized in Table 16.”.

In Table 8: STM32F40xxx pin and ball definitions on page 53:

– For pin PC13, replaced “RTC_AF1” by “RTC_OUT, RTC_TAMP1,

RTC_TS”

– for pin PI8, replaced “RTC_AF2” by “RTC_TAMP1, RTC_TAMP2,

RTC_TS”.

– for pin PB15, added RTC_REFIN in Alternate functions column.

In Table 10: Alternate function mapping on page 70, for port

PB15, replaced “RTC_50Hz” by “RTC_REFIN”.

198/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated Figure 6: Multi-AHB matrix.

Updated Figure 7: Power supply supervisor interconnection with

internal reset OFF

Changed 1.2 V to V12 in Section : Regulator OFF

Updated LQFP176 pin 48.

Updated Section 1: Introduction.

Updated Section 2: Description.

Updated operating voltage in Table 3: STM32F405xx and

STM32F407xx: features and peripheral counts.

Updated Note 1.

Updated Section 3.0.15: Power supply supervisor.

Updated Section 3.0.16: Voltage regulator.

Updated Figure 9: Regulator OFF.

Updated Table 4: Regulator ON/OFF and internal reset ON/OFF

availability.

Updated Section 3.0.19: Low-power modes.

Updated Section 3.0.20: VBAT operation.

Updated Section 3.0.22: Inter-integrated circuit interface (I²C) .

Updated pin 48 in Figure 15: STM32F40xxx LQFP176 pinout.

Updated Table 7: Legend/abbreviations used in the pinout table.

Updated Table 8: STM32F40xxx pin and ball definitions.

Updated Table 16: General operating conditions.

4

Updated Table 17: Limitations depending on the operating power

04-Jun-2013

(continued)

supply range.

Updated Section 6.3.7: Wakeup time from low-power mode.

Updated Table 36: HSI oscillator characteristics.

Updated Section 6.3.15: I/O current injection characteristics.

Updated Table 50: I/O static characteristics.

Updated Table 53: NRST pin characteristics.

Updated Table 56: I2C characteristics.

Updated Figure 39: I2C bus AC waveforms and measurement circuit.

Updated Section 6.3.19: Communications interfaces.

Updated Table 69: ADC characteristics.

Added Table 72: Temperature sensor calibration values.

Added Table 75: Internal reference voltage calibration values.

Updated Section 6.3.26: FSMC characteristics.

Updated Section 6.3.28: SD/SDIO MMC card host interface (SDIO)

characteristics.

Updated Table 25: Typical and maximum current consumptions in Stop

mode.

Updated Section : SPI interface characteristics included Table 57.

Updated Section : I2S interface characteristics included Table 58.

Updated Table 66: Dynamic characteristics: Ethernet MAC signals for

SMI.

Updated Table 68: Dynamic characteristics: Ethernet MAC signals for

MII.

DS8626 Rev 11

199/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated Table 66: Dynamic characteristics: Ethernet MAC signals for

SMI.

Updated Table 68: Dynamic characteristics: Ethernet MAC signals for

MII.

Updated Table 81: Synchronous multiplexed NOR/PSRAM read

timings.

Updated Table 82: Synchronous multiplexed PSRAM write timings.

Updated Table 83: Synchronous non-multiplexed NOR/PSRAM read

4

04-Jun-2013

timings.

(continued)

Updated Table 84: Synchronous non-multiplexed PSRAM write

timings.

Updated Section 6.3.27: Camera interface (DCMI) timing specifications

including Table 89: DCMI characteristics and addition of Figure 72:

DCMI timing diagram.

Updated Section 6.3.28: SD/SDIO MMC card host interface (SDIO)

characteristics including Table 90.

Updated Chapter Figure 9.

200/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Replace Cortex-M4F by Cortex-M4 with FPU throughout the

document.

Updated Section : Regulator OFF and Table 4: Regulator ON/OFF and

internal reset ON/OFF availability for LQFP176.

Updated Figure 15: STM32F40xxx LQFP176 pinout and Table 8:

STM32F40xxx pin and ball definitions.

Updated Figure 6: Multi-AHB matrix.

Added note 1 below Figure 12: STM32F40xxx LQFP64 pinout, Figure

13: STM32F40xxx LQFP100 pinout, Figure 14: STM32F40xxx

LQFP144 pinout and Figure 15: STM32F40xxx LQFP176 pinout.

Updated IVDD and IVSS in Table 14: Current characteristics.

Updated PLS[2:0]=101 (falling edge) configuration in Table 21:

Embedded reset and power control block characteristics.

Added Section : Additional current consumption. Updated Section :

On-chip peripheral current consumption.

Updated Table 31: Low-power mode wakeup timings.

Updated Table 34: HSE 4-26 MHz oscillator characteristics and Table

35: LSE oscillator characteristics (fLSE = 32.768 kHz).

Changed condition related to VESD(CDM) in Table 47: ESD absolute

maximum ratings.

Updated Table 49: I/O current injection susceptibility, Table 50: I/O

static characteristics, Table 51: Output voltage characteristics

conditions, Table 52: I/O AC characteristics and Figure 37: I/O AC

characteristics definition.

06-Mar-2015

5

Updated Section : I2C interface characteristics.

Remove note 3 in Table 71: Temperature sensor characteristics.

Updated Figure 72: DCMI timing diagram.

Modified Figure 75: WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch

wafer level chip scale package outline and Table 92: WLCSP90 - 4.223

x 3.969 mm, 0.400 mm pitch wafer level chip scale package

mechanical data. Added Figure 76: WLCSP90 - 4.223 x 3.969 mm,

0.400 mm pitch wafer level chip scale recommended footprint and

Table 93: WLCSP90 recommended PCB design rules. /

Modified Figure 78: LQFP64 – 64-pin, 10 x 10 mm low-profile quad flat

package outline and Table 94: LQFP64 – 64-pin 10 x 10 mm low-profile

quad flat package  mechanical data.

Updated Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package outline and Table 97:

UFBGA176+25 ball, 10 × 10 × 0.65 mm pitch, ultra thin fine pitch ball

grid array mechanical data. Added Figure 88: UFBGA176+25 - 201-

ball, 10 x 10 mm, 0.65 mm pitch, ultra fine pitch ball grid array

recommended footprint and Table 98: UFBGA176+25 recommended

PCB design rules (0.65 mm pitch BGA).

Updated Figure 90: LQFP176 - 176-pin, 24 x 24 mm low profile quad

flat package outline.

Added Section : Device marking for WLCSP90, Section : Device

marking for LQFP64, Section : Device marking for LFP100, Section :

Device marking for LQFP144, Section : Device marking for

UFBGA176+25 and Section : Device marking for LQFP176.

DS8626 Rev 11

201/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

In the whole document, updated notes related to values specified by

design or by characterization.

Updated Table 36: HSI oscillator characteristics.

Changed fVCO_OUT minimum value and VCO freq to 100 MHz in

Table 38: Main PLL characteristics and Table 39: PLLI2S (audio PLL)

characteristics.

Updated Figure 39: SPI timing diagram - slave mode and CPHA = 0.

Updated Figure 53: 12-bit buffered /non-buffered DAC.

22-Oct-2015

6

Removed note 1 related to better performance using a restricted VDD

range in Table 70: ADC accuracy at fADC = 30 MHz.

Upated Figure 84: LQFP144 - 144-pin, 20 x 20 mm low-profile quad flat

package outline.

Updated Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package outline and Table 97:

UFBGA176+25 ball, 10 × 10 × 0.65 mm pitch, ultra thin fine pitch ball

grid array mechanical data.

Updated Figure 2: Compatible board design

STM32F10xx/STM32F2/STM32F40xxx for LQFP100 package.

Updated |VSSX- VSS| in Table 13: Voltage characteristics to add

VREF-.

16-Mar-2016

7

Added VREF- in Table 69: ADC characteristics.

Updated Table 92: WLCSP90 - 4.223 x 3.969 mm, 0.400 mm pitch

wafer level chip scale package mechanical data.

Removed note 1 below Figure 5: STM32F40xxx block diagram.

Updated definition of stresses above maximum ratings in Section 6.2:

Absolute maximum ratings.

Updated th(NSS) in Figure 39: SPI timing diagram - slave mode and

CPHA = 0 and Figure 40: SPI timing diagram - slave mode and CPHA

= 1.

09-Sep-2016

8

Added note related to optional marking and inset/upset marks in all

package marking sections.

Updated Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm pitch,

ultra fine pitch ball grid array package outline and Table 97:

UFBGA176+25 ball, 10 × 10 × 0.65 mm pitch, ultra thin fine pitch ball

grid array mechanical data.

202/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

Revision history

Table 99. Document revision history (continued)

Date

Revision

Changes

Renamed Section 3 and Section 8 into Functional overview and

Ordering information, respectively.

Added Arm logo and legal notice in Section 1: Introduction and

updated Arm wordmark in the whole document. Removed USB

certified logos.

Updated camera interfaces for STM32F405OE in Table 3:

STM32F405xx and STM32F407xx: features and peripheral counts.

Added OTP memory in Features and Section 3.0.4: Embedded Flash

memory.

Changed random number generator to true random number generator

in the whole document and updated Section 3.0.34: True random

number generator (RNG).

Added Note 1 related to UFBGA176 in Table 8: STM32F40xxx pin and

14-Aug-2020

9

ball definitions.

Updated Section 6.2: Absolute maximum ratings introduction.

Updated VPVD minimum value for PLS[2:0]=101 (falling edge) in Table

21: Embedded reset and power control block characteristics.

Updated Note 1 in Table 36: HSI oscillator characteristics.

Added reference to application note AN4899 in Section 6.3.16: I/O port

characteristics.

Replaced DCMI_PIXCK by DCMI_PIXCLK in Table 10: Alternate

function mapping.

Renamed Section 8 into Ordering information.

Updated D1 in Figure 87: UFBGA176+25 ball, 10 x 10 mm, 0.65 mm

pitch, ultra fine pitch ball grid array package outline.

DS8626 Rev 11

203/205

204

Revision history

STM32F405xx, STM32F407xx

Table 99. Document revision history (continued)

Date

Revision

Changes

Updated:

– Cover page

– Section 1: Introduction

– Figure 5: STM32F40xxx block diagram

– Section 3.0.18: Real-time clock (RTC), backup SRAM and backup

registers

– Table 7: STM32F40xxx pin and ball definitions

– Table 9: Alternate function mapping

– I/O system current consumption

– Note 1 in Table 34: HSI oscillator characteristics.

= 32.768 kHz)

– Table 33: LSE oscillator characteristics (f

LSE

– Figure 37: I/O AC characteristics definition

– Figure 39: SPI timing diagram - slave mode and CPHA = 0,

08-Nov-2024

10

Figure 40: SPI timing diagram - slave mode and CPHA = 1, and

Figure 41: SPI timing diagram - master mode

– Table 44: EMI characteristics for fHSE = 25 MH and fCPU =

168 MHz

– Figure 49: ADC accuracy characteristics and Figure 50: Typical

connection diagram when using the ADC with FT/TT pins featuring

analog switch function

– Figure 68: NAND controller waveforms for read access and

Figure 69: NAND controller waveforms for write access

– Table 96: UFBGA(176+25) - Example of PCB design rules (0.65 mm

pitch BGA) title

– Section 7: Package information

Added Section 9: Important security notice

Updated Figure 7: Power supply supervisor interconnection with

internal reset OFF.

Updated Figure 8: PDR_ON and NRST control with internal reset OFF.

Updated Figure 9: Regulator OFF.

04-Feb-2026

11

Updated Figure 18: STM32F40xxx memory map.

Updated Figure 20: Pin input voltage.

Updated Table 45: ESD absolute maximum ratings.

Updated Section 7.3: LQFP64 package information (5W).

204/205

DS8626 Rev 11

STM32F405xx, STM32F407xx

IMPORTANT NOTICE – READ CAREFULLY

STMicroelectronics NV and its subsidiaries (“ST”) reserve the right to make changes, corrections, enhancements, modifications, and

improvements to ST products and/or to this document at any time without notice.

In the event of any conflict between the provisions of this document and the provisions of any contractual arrangement in force between the

purchasers and ST, the provisions of such contractual arrangement shall prevail.

The purchasers should obtain the latest relevant information on ST products before placing orders. ST products are sold pursuant to ST’s

terms and conditions of sale in place at the time of order acknowledgment.

The purchasers are solely responsible for the choice, selection, and use of ST products and ST assumes no liability for application

assistance or the design of the purchasers’ products.

No license, express or implied, to any intellectual property right is granted by ST herein.

Resale of ST products with provisions different from the information set forth herein shall void any warranty granted by ST for such product.

If the purchasers identify an ST product that meets their functional and performance requirements but that is not designated for the

purchasers' market segment, the purchasers shall contact ST for more information.

ST and the ST logo are trademarks of ST. For additional information about ST trademarks, refer to www.st.com/trademarks. All other product

or service names are the property of their respective owners.

Information in this document supersedes and replaces information previously supplied in any prior versions of this document.

© 2026 STMicroelectronics – All rights reserved

DS8626 Rev 11

205/205

205