M3508

P19 Brushless DC Gear Motor

User Guide

使用说明

ユーザーガイド

V1.0

2017.08

Disclaimer

Thank you for purchasing the RoboMaster M3508 P19 Brushless DC Gear

Motor (hereinafter referred to as “product”). Read this disclaimer carefully

before using this product. By using this product, you hereby agree to

this disclaimer and signify that you have read it fully. Install and use this

product in strict accordance with the User Guide. SZ DJI TECHNOLOGY

CO., LTD. and its affiliated companies assume no liability for damage(s)

or injuries incurred directly or indirectly from using, installing or modifying

this product improperly, including but not limited to using non-designated

accessories.

TM

DJI

is a trademark of SZ DJI TECHNOLOGY CO., LTD. (abbreviated

as “DJI”) and its affiliated companies. Names of products, brands, etc.,

appearing in this document are trademarks or registered trademarks

of their respective owner companies. This product and document are

copyrighted by DJI with all rights reserved. No part of this product or

document shall be reproduced in any form without the prior written

consent or authorization of DJI.

The final interpretation rights of this disclaimer is owned by DJI.

Warning

1. DO NOT let any foreign materials come into contact with the rotors, as

it may negatively affect performance.

2. Ensure all the cables are correctly connected.

3. Ensure the motor is securely mounted.

4. Properly use the motor within the maximum allowable temperature range

of the winding. Refer to the line graph in the “Performance” section of

this guide for operating range when using RoboMaster C620 Brushless

DC Motor Speed Controller. Avoid the motor from over-heating for a long

period, or the motor may be permanently damaged when using other

DJI electronic speed controller.

5. Avoid damage to the cables or wires, which may cause the motor to

work improperly.

6. DO NOT touch the motor rotors, as doing so may cause injury.

7. The motor will heat when the output power is high. Although this is

normal, take caution to avoid scalding.

Introduction

M3508 P19 Brushless DC Gear Motor is a high-performance servo motor,

specially designed for small and medium-sized mobile platforms and

robots. Compared with traditional square wave drive, the M3508 gear

motor features sinusoidal drive used with RoboMaster C620 Brushless

2

DC Motor Speed Controller, which boosts higher efficiency, flexibility, and

stability. The gear motor’s reduction ratio is approximately 19:1.

Features

Position Feedback: The built-in positioning sensor provides location

•

information.

Temperature Detection: The built-in detecting sensor effectively

•

prevents the motor from damage due to abnormal temperature.

Information Storage: Saves the motor’s calibration parameters, enabling

•

easy changing of motors.

TM

The M3508 gear motor is also compatible with DJI TAKYON

Z650 and Takyon Z660 ESCs. Please note that the above features

are not available with these models.

In the Box

×

M3508 P19 Brushless DC

7-Pin Cable

1

×

Gear Motor

1

Connecting the Motor

Example: RoboMaster C620 Brushless DC Speed Controller.

1. Connect the 7-pin ports of the gear motor and C620 with the 7-pin cable.

2. Connect the 3-phase cable of the gear motor to that of the C620.

Ensure the cables are securely connected with the corresponding

colors correctly matched.

7-Pin Cable

3-phase Cable

3

Mounting the Motor

Refer to the dimensions below to mount the gear motor to a robot or

mobile platform.

Mounting Holes:

10

4×M4×0.7

98.40±1

M5 - 6H    10

27.30

66.70±1

20

M3

-0.10

-0.05

10

0

26

-0.10

9

10

±0.10

2

35

42

Top View

Side View

1:2

Unit: mm

The gear motor is outfitted with M4 mounting holes at a depth of

10 mm. Use appropriately-sized screws to mount properly and

avoid damage.

Performance

Load characteristics (Closed Loop Speed Control Using RoboMaster

C620 Speed Controller)

η

(%)

P(W)

I(A)

n(rpm)

250

16

80

450

n

14

400

70

200

350

12

60

η

300

10

50

150

250

8

40

200

100

6

30

150

4

20

50

100

2

10

50

I

P

0

0

0

0

1

2

3

4

5

6

T(N·m)

4

Load  characteristics  (Closed  Loop  Current  Control  Using

RoboMaster C620 Speed Controller)

η

(%)

P(W)

I(A)

n(rpm)

250

16

600

80

14

70

500

200

12

60

400

10

50

150

8

40

300

100

6

30

200

4

20

50

2

10

100

n

η

P

I

0

0

0

1

2

3

4

5

6

0

T(N·m)

Load characteristics (using DJI Z650 ESC)

η

(%)

I(A)

n(rpm)

P(W)

300

70

700

18

η

16

200

100

60

600

250

14

n

50

500

200

12

P

40

400

10

150

8

30

300

100

I

6

20

4

50

10

2

0

0

0

0

1

2

3

4

5

6

T(N·m)

Operating Range

n(rpm)

°

°

257

F (125

C)

500

400

300

200

100

0

1

2

3

4

T(N·m)

Continuous Operating Range

η – Electrical Efficiency, T – Thrust, I – Current, P – Output Power,

n – Rotational Speed

5

The data above was generated in a laboratory setting with a C620 speed

controller output of 24 V, at the temperature of 77º F (25º C), and under

normal dissipation conditions. These figures should be used for reference

only. Make sure to control running time properly in accordance with

actual working temperature, dissipation, etc.

Characteristic Parameters

Refer to the parameters below to make proper use of your motor.

Using C620 Speed Controlled at rated voltage

Rotational Speed (without payload)

482 rpm

Current (without payload)

0.78 A

Rated Rotational Speed

469 rpm

Rated Torque (max continuous torque)

3 N·m

Rated Current

10 A

Max Efficiency

70 %

Stall Torque

4.5 N·m

Stall Current

2.5 A

* Tested in a lab environment using a C620 Speed Controller.

The above current refers to the input current of speed controller and the

efficiency refers to the whole system.

Using Z650 ESC at the rated voltage

Rotational Speed (without payload)

589 rpm

Current (without payload)

0.98 A

Rated Rotational Speed

469 rpm

Rated Torque (max continuous torque)

4.6 N·m

Rated Current (max continuous line current)

15.7 A

Max Efficiency

67 %

* Tested in a laboratory setting using the DJI Z650 ESC. Please go to visit

DJI official website for detailed information of the DJI Z650 ESC.

The above current refers to the input current of speed controller and the

efficiency refers to the whole system.

6

Characteristic Parameters

Rated Voltage

24 V

Torque Constant

0.3 N·m/A

Speed Constant

24.48 rpm/V

Speed/Torque Gradient

72 rpm/N·m

Mechanical Time Constant

49 ms

Phase Resistance

0.194 Ω

Phase Inductance

0.097 mH

Operating Temperature Range

32°~122° F (0°~50° C)

Max Permissible Winding Temperature

257° F (125° C)

Number of Pole Pairs

7

KN

210 N

Weight

365 g

3591/187

Reduction Ratio

7

免责声明

TM

感谢您购买

直流无刷减速电机（以下简称

RoboMaster

M3508

电机）。在使用之前，请仔细阅读本声明，一旦使用，即被视为

对本声明全部内容的认可和接受。请严格遵守手册、产品说明和

相关的法律法规、政策、准则安装和使用该产品。在使用产品过

程中，用户承诺对自己的行为及因此而产生的所有后果负责。因

TM

用户不当使用、安装、改装造成的任何损失，

将不承担法律

DJI

责任。

TM

是深圳市大疆

创新科技有限公司及其关联公司的商标。本

DJI

文出现的产品名称、品牌等，均为其所属公司的商标。本产品及

手册为大疆创新版权所有。未经许可，不得以任何形式复制翻印。

关于免责声明的最终解释权，归大疆创新所有。

产品使用注意事项

避免杂物进入转子内部，否则会导致转子运行异常。

1.

使用前请确保接线正确。

2.

使用前请确保电机安装正确、稳固。

3.

请在绕组最大允许温度范围内使用电机。搭配

4.

RoboMaster

无刷电机调速器使用时，参照本文工作范围曲线

；配合

C620

其他电调使用时，请避免电机长时间过热，以防电机被损坏。

DJI

使用时请避免损伤线材，导致电机运行异常。

5.

使用时请勿触摸电机转子部分，避免割伤。

6.

电机大功率输出时，会出现发热的情况，请注意避免烫伤。

7.

简 介

直流无刷减速电机是专为中小型移动平台和机器人等量身

M3508

打造的高性能伺服电机，可搭配

电调实现正弦

RoboMaster C620

驱动，相比传统方波驱动具有更高的效率、机动性和稳定性。本

产品减速箱减速比约为

。

19:1

产品特性

位置反馈：电机自带位置传感器，可提供位置的反馈。

•

温度检测：电机自带温度检测传感器，可有效防止电机因温度异

•

常被损坏。

8

信息存储：存储电机校准参数，支持电机的快速更换。

•

TM

直流无刷减速电机支持搭配

电

M3508

DJI TAKYON

Z650

调和

，但只可作为一般动力电机使用，不具

Takyon Z660

备上述功能。

物品清单

电机 ×

数据线 ×

1

7-Pin

1

电机电调连线

下面以搭配

电调为例，介绍连线方式：

RoboMaster C620

用

数据线分别插入电调和电机的

数据端口，连接电

1.

7-Pin

7-Pin

调和电机。

将电机的三相输入接头与电调三相动力线接头相连接，连接时

2.

请确保电调与电机连线正确（相同颜色的接线匹配连接，并且

保证不可逆接头正确匹配连接），切勿接错。

数据线

7-Pin

三相动力接头

9

安装电机

请参考电机安装孔尺寸和位置将电机安装至移动平台或机器人等

结构上。

安装孔：

10

4×M4×0.7

98.40±1

M5 - 6H    10

27.30

66.70±1

20

M3

-0.10

-0.05

10

0

26

-0.10

9

10

±0.10

2

35

42

主视图

左视图

单位

: mm

1:2

电机安装孔为

螺纹孔，深度

，请勿使用过长的螺

M4

10mm

丝，否则可能会顶到电机的减速箱外齿圈从而损坏电机。

电机参数

负载特性曲线（使用 RoboMaster C620 电调做速度闭环控制）

η

(%)

P(W)

I(A)

n(rpm)

250

16

80

450

n

14

400

70

350

60

300

50

250

200

12

η

10

150

8

40

200

100

6

30

150

4

20

50

100

2

10

50

0

0

0

I

P

0

1

2

3

4

5

6

T(N·m)

10

负载特性曲线（使用

电调做电流闭环控制）

RoboMaster C620

η

(%)

P(W)

I(A)

n(rpm)

250

16

600

80

14

70

500

200

n

12

60

η

400

10

50

150

P

8

40

300

100

6

30

I

200

4

20

50

2

10

100

0

0

0

1

2

3

4

5

6

0

T(N·m)

负载特性曲线（使用 DJI Z650 电调）

η

(%)

I(A)

n(rpm)

P(W)

300

70

700

18

η

16

60

600

250

14

n

50

500

12

200

P

40

400

10

150

8

30

300

100

I

6

20

200

4

50

10

100

2

0

0

0

0

1

2

3

4

5

6

T(N·m)

工作范围图

n(rpm)

℃

125

500

400

300

200

100

0

1

2

3

4

T(N·m)

持续工作范围

η

效率

，

扭矩，

电流，

输出功率，

转速

-

T-

I-

P-

n-

11

以上数据均为使用

电调，输入电压

，带减速箱在

℃

C620

24V

25

室温和正常散热实验环境下测得，仅供参考。实际使用时，请根

据工作环境温度、散热条件等实际情况来控制减速电机运行时间。

特征参数

请根据以下参数合理使用电机。

搭配 C620 电调在额定电压下的电机参数

空载转速

482 rpm

空载电流

0.78 A

额定转速

469 rpm

额定转矩（最大连续转矩）

3 N·m

额定电流

10 A

最大效率

70%

堵转扭矩

4.5 N·m

堵转电流

2.5 A

以上数据是配合

电调在实验环境下测得。

*

C620

以上电流均为电调的输入端电流；效率为电机和电调整个系统的

效率。

搭配 Z650 电调在额定电压下的电机参数

空载转速

589 rpm

空载电流

0.98 A

额定转速

469 rpm

额定转矩（最大连续转矩）

4.6 N·m

额定电流（最大连续线电流）

15.7 A

最大效率

67%

以上数据是配合

电调在实验环境下测得。

*

DJI Z650

以上电流均为电调的输入端电流；效率为电机和电调整个系统的

效率。

电调参数详见

官网。

DJI Z650

DJI

12

电机特征值

额定电压

24 V

转矩常数

0.3 N·m/A

转速常数

24.48 rpm/V

转速转矩梯度

72 rpm/N·m

机械时间常数

49 ms

相电阻

Ω

0.194

相电感

0.097 mH

使用环境温度

℃至

℃

0

50

绕组最高允许温度

℃

125

极对数

7

最大径向载荷（动载荷）

210 N

减速电机重量

365 g

减速比

3591/187

13

免責事項

ブラシレス直流ギアモーター（以下、「本製品」といいます）

RoboMaster M3508 P19

をお買い上げいただき、誠にありがとうございます。本製品のご使用前に、この

免責事項をよくお読みください。本製品を使用すると、この免責事項をすべて読み、

これに同意したものとみなされます。本製品は、ユーザーガイドに記載されてい

るとおりに取り付けて使用してください。

とそ

SZ DJI TECHNOLOGY CO., LTD.

の関連会社は、本製品が不適切な使用、取り付けまたは改造（指定外のアクセサ

リーの使用などが含まれます）により、直接または間接的な原因で生じた物的損

害または人的被害についていかなる責任も負いません。

TM

は

（略して「

」）およびその関連会社

DJI

SZ DJI TECHNOLOGY CO., LTD.

DJI

の商標です。本書に記載されている製品、ブランドなどの名称は、その所有者で

ある各社の商標または登録商標です。本製品および本書は、不許複製・禁無断転

載を原則とする

の著作物のため、

から書面による事前承認または許諾を

DJI

DJI

得ることなく何らかの形で本製品または文書のいかなる部分も複製することは固

く禁じられています。

本免責事項の最終解釈権限は

が有します。

DJI

警告

ローターに異物を接触させないでください。性能に悪影響を及ぼすおそれがあ

1.

ります。

すべてのケーブルが正しく接続されていることを確認してください。

2.

モーターがしっかりと取り付けられていることを確認してください。

3.

巻線の最大許容温度範囲内で、モーターを適切に使用してください。

4.

ブラシレス直流モーター速度コントローラーを使用する際

RoboMaster C620

は、このガイドの「性能」で記載している動作範囲の線グラフを参照してくだ

さい。他の

電子速度コントローラーを使用する際は、モーターが長時間過

DJI

熱状態にならないようにしてください。モーターが修復不可能な損傷を受ける

おそれがあります。

ケーブルやワイヤーが損傷しないようにしてください。損傷するとモーターが

5.

適切に作動しなくなるおそれがあります。

モーターのローターに触れないでください。怪我をするおそれがあります。

6.

出力電力が高いと、モーターが高温になります。この現象に問題はありません

7.

が、火傷にご注意ください。

はじめに

ブラシレス直流ギアモーターは、高性能なサーボモーターです。中

M3508 P19

型・小型のモバイルプラットフォームおよびロボット専用に設計されています。

ギアモーターは、

ブラシレス直流モーター速度コント

M3508

RoboMaster C620

ローラーを備えた正弦波駆動式で、従来の矩形波駆動に比べ、効率、柔軟性、安

定性に優れています。本ギアモーターの減速比は約

です。

19:1

14

機能

ポジションフィードバック：内蔵位置センサーは位置情報を提供します。

•

温度検知：内蔵検知センサーが効果的に機能して、異常温度によるモーターの

•

損傷を防ぎます。

情報の保持：モーターのキャリブレーションパラメータが保存されるので、モ

•

ーターの変更が容易に行えます。

TM

ギアモーターは、

および

M3508

DJI TAKYON

Z650 ESC

Takyon Z660

との互換性も有しています。ただし、これらのモデルでは上記の機

ESC

能は使用できません。

同梱物

ブラシレス直流

ピンケーブル

×

M3508 P19

7

1

ギアモーター

×

1

モーターの接続

例：

ブラシレス直流速度コントローラー

RoboMaster C620

ピンケーブルで、ギアモーターの

ピンポートと

の

ピンポートを接

1.  7

7

C620

7

続します。

ギアモーターの三相ケーブルを、

の三相ケーブルに接続します。対応す

2.

C620

る色が正しく合うようにケーブルをしっかり接続してください。

ピンケーブル

7

三相ケーブル

15

モーターの取り付け

ギアモーターをロボットまたはモバイルプラットフォームに取り付ける際は、以

下の寸法を参照してください。

取り付け穴：

10

4×M4×0.7

98.40±1

M5 - 6H    10

27.30

66.70±1

20

M3

-0.10

-0.05

10

0

26

-0.10

9

10

±0.10

2

35

42

上面図

側面図

単位：

1:2

mm

ギアモーターは

の深さの

取り付け穴で外付けします。正しい

10 mm

M4

サイズのネジを使用して、損傷しないように適切に取り付けてください。

性能

負荷特性（

速度コントローラーでの閉ループ速度制御）

C620

η

(%)

P(W)

I(A)

n(rpm)

250

16

80

450

n

14

400

70

200

350

12

60

η

300

10

50

150

250

8

40

200

100

6

30

150

4

20

50

100

2

10

50

0

0

0

I

P

0

1

2

3

4

5

6

T(N·m)

16

負荷特性（

速度コントローラーでの閉ループ電流制御）

C620

η

(%)

P(W)

I(A)

n(rpm)

250

16

600

80

14

70

500

200

n

12

60

400

10

50

150

η

P

8

40

300

100

6

30

I

200

4

20

50

2

10

100

0

0

0

1

2

3

4

5

6

0

T(N·m)

負荷特性（

使用時）

DJI Z650 ESC

η

(%)

I(A)

n(rpm)

P(W)

300

70

700

18

η

16

60

600

250

14

n

50

500

12

200

P

40

400

10

150

8

30

300

100

I

6

20

200

4

50

10

100

2

0

0

0

0

1

2

3

4

5

6

T(N·m)

動作範囲

n(rpm)

℃

125

500

400

300

200

100

0

1

2

3

4

T(N·m)

連続動作範囲

η

電気効率、

推力、

電流、

出力、

回転速度

–

T –

I –

P –

n –

17

上記データは、ラボ設定（

速度コントローラーの出力

、気温

℃、

C620

24V

25

標準放熱状態）の環境下で得られた測定値です。これらの数値は参考値です。

実際の動作温度、放熱状態などに応じて、適切に運転時間をコントロールして

ください。

特性パラメータ

以下のパラメータを参考にして、モーターを適切に使用してください。

定格電圧で

速度コントローラーを使用します

C620

回転速度（ペイロードなし）

482 rpm

電流（ペイロードなし）

0.78 A

定格回転速度

469 rpm

定格トルク（最大連続トルク）

3 N·m

定格電流

10 A

最大効率

70%

ストールトルク

4.5 N·m

ストール電流

2.5 A

速度コントローラーを使用してラボ環境でテストを実施。

* C620

上記の電流は、速度コントローラーの入力電流です。効率性はシステム全体に

ついてのデータです。

定格電圧で

を使用

Z650 ESC

回転速度（ペイロードなし）

589 rpm

電流（ペイロードなし）

0.98 A

定格回転速度

469 rpm

定格トルク（最大連続トルク）

4.6 N·m

定格電流（最大連続線電流）

15.7 A

最大効率

67%

を使用してラボ設定でテストを実施。

の詳細につ

* DJI Z650 ESC

DJI Z650 ESC

いては、

公式ウェブサイトをご参照ください。

DJI

上記の電流は、速度コントローラーの入力電流です。効率性はシステム全体に

ついてのデータです。

18

特性パラメータ

定格電圧

24 V

トルク定数

0.3 N·m/A

速度定数

24.48 rpm/V

速度／トルク勾配

72 rpm/N·m

機械的時定数

49 ms

相抵抗

Ω

0.194

相インダクタンス

0.097 mH

動作環境温度

〜

℃

0

50

最大許容巻線温度

℃

125

極対数

7

KN

210 N

重量

365 g

減速比

3591/187

19

FCC Compliance

This device complies with Part 15 of the FCC Rules. Operation is subject

to the following two conditions: (1) This device may not cause harmful

interference, and (2) This device must accept any interference received,

including interference that may cause undesired operation.

Any changes or modifications not expressly approved by the party

responsible for compliance could void the user’s authority to operate the

equipment.

EU Compliance Statement:

SZ DJI TECHNOLOGY CO., LTD. hereby

declares that this device is in compliance with the essential requirements

and other relevant provisions of the Directive 2014/30/EU.

A copy of the EU Declaration of Conformity is available online at www.dji.

com/euro-compliance

EU contact address: DJI GmbH, Industriestrasse. 12, 97618, Niederlauer,

Germany

Environmentally friendly disposal

Old electrical appliances must not be disposed of together with

the residual waste, but have to be disposed of separately. The

disposal at the communal collecting point via private persons is

for free. The owner of old appliances is responsible to bring the

appliances to these collecting points or to similar collection points. With

this little personal effort, you contribute to recycle valuable raw materials

and the treatment of toxic substances.

IC Compliance

This device complies with ICES-003 standard. Operation is subject to the

following two conditions: (1) this device may not cause interference, and

(2) this device must accept any interference, including interference that

may cause undesired operation of the device.

20

WWW.ROBOMASTER.COM

and

are trademarks of DJI.

Copyright © 2017 DJI All Rights Reserved.

Printed in China.